3GPP TSG-RAN WG4 Meeting # 101-e      		R4-2117737

Electronic Meeting, November 01-12, 2021

Agenda Item:	8.12.2.1.2

Source: 	CMCC

Title: 	Discussion on CSI reporting related issues for inter-cell interference MMSE-IRC

Document for:	Discussion

- Introduction

In RAN4#100-e meeting, CSI reporting requirements for inter-cell interference MMSE-IRC receiver were discussed, a way forward was agreed in [1]. In this contribution, we further discuss the left open issues for CQI reporting.

- Discussion

  - Interference Model

Issue 2-1: CSI-IM on target cell

Agreement in RAN4#100-e meeting:

- CSI-IM overlaps with PDSCH from interference is the baseline scenario for defining CSI reporting requirement.

- Other scenarios are FFS.

  - Option 1: Only consider overlapping with PDSCH from interference

  - Option 2: Further include overlapping with NZP CSI-RS from interference

  - Option 3: Further include overlapping with Interf. cell CSI-IM

  - Option 4: Make further down selection based on simulation results and also take practical network configuration into account.

    - If using target CSI-IM overlaps with neighbor NZP CSI-RS, target CSI-IM port number (X) shall be equal to neighbor cell NZP CSI-RS port number

Since CSI-IM overlaps with PDSCH from interference is the baseline, Option 1 is the simplest and effective way.

Proposal 1: Only consider CSI-IM on target cell overlapping with PDSCH from interference.

Issue 2-2: NZP CSI-RS on target cell

Agreement in RAN4#100-e meeting:

- Option 1: Overlaps with PDSCH from interference

- Option 2: Overlaps with NZP CSI-RS from interference

- Option 3: Make further down selection based on simulation results and also take practical network configuration into account.

We support Option 2 which is common configuration in practical network. In addition, Option 1 may cause network performance degradation, because the PDSCH of neighbour cell will be interfered by NZP CSI-RS of serving cell.

Proposal 2: NZP CSI-RS on target cell overlaps with NZP CSI-RS from interference.

- Conclusion

In this contribution, we discuss the interference model of CQI related requirements for inter-cell interference suppression with MMSE-IRC receiver, and provide our proposals. The proposals are:

Proposal 1: Only consider CSI-IM on target cell overlapping with PDSCH from interference.

Proposal 2: NZP CSI-RS on target cell overlaps with NZP CSI-RS from interference.

- Reference

[1] R4-2115732, WF on CSI requirements for inter-cell interference MMSE-IRC, Ericsson.