3GPP TSG-RAN4 WG4 Meeting #101-e	R4-2117774

# Electronic meeting, 1 November – 12 November, 2021

Source: 	vivo

Title: 	Further discussion on RRM impacts for extending NR operation to 71GHz

Agenda Item:	8.16.7.1

Document for:	Discussion

Introduction

In RAN4 #100e, there were discussions on RRM impacts due to introducing of extending current NR operation to 71GHz. The agreements/conclusions are captured in [1].

Rx beam sweeping scaling factor - Rx beam sweeping scaling factor - Further study whether new scaling factor is needed for FR2-2 considering RF session conclusions on UE antenna array assumptions and UE power classes and the difference with FR2-1 assumptions - Rx beam sweeping factor from FR2-1 can be used as a starting point for analysis Scheduling restrictions - Further investigate whether to update the scheduling restriction for L1/L3 measurement considering at least the following aspects with further inputs from RAN1 and RF: - Beam switching time - Synchronization assumptions with large SCS Simulations for Cell detection, PBCH detection and SSB measurements - Discuss whether simulations are needed to study the impact of higher SCS on the following - Cell detection - PBCH detection for SSB index detection - SSB measurements - If yes, also provide inputs on the simulation assumptions.

In this contribution, we further discuss on RRM impacts for extending NR operation to 71GHz.

Discussion

Rx beam sweeping scaling factor

The Rx beam sweeping scaling factor was discussed in the last meeting. The GTW agreed that whether the new scaling factor is needed for FR2-2 considering RF group conclusion. A WF [2] from RF session related to antenna element numbers was duplicated as below:

Agreement: Have further analysis on the UE EIPR requirements taking both antenna element number and PA performance

- Include multiple antenna element numbers

- Practical form factor should be considered in the analysis

There is no conclusion regarding UE antenna array assumptions and UE power classes for FR2-2. For FR2-1, 4 antenna elements for PC3 are the typical implementation. Due to the larger propagation loss and PA performance loss for 52.6-71GHz, more antenna elements may be needed compared with FR2-1. However, the specific details and clear values need to be discussed in RF group.

Proposal 1: Rx beam sweeping scaling factor should be considered until there is specific conclusion for antenna element numbers.

We copy the requirements of time period for PSS/SSS detection for FR2-1 related to intra-frequency measurements without measurement gap from [38.133].

Table 9.2.5.1-2: Time period for PSS/SSS detection, (Frequency range FR2) DRX cycle; TPSS/SSS_sync_intra. No DRX; max(600ms, ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Mpss/sss_sync_w/o_gaps : For a UE supporting FR2 power class 1 or 5, Mpss/sss_sync_w/o_gaps =40. For a UE supporting power class 2, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2 power class 3, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2 power class 4, Mpss/sss_sync_w/o_gaps =24

For FR1, the sample number for PSS/SSS detection is 5. For FR2-1, 24 and 40 samples correspond to power class 2, 3 or 4 and power class 1 or 5 respectively. It can be seen that the current requirements for FR2-1 already are already large enough. If the Rx beam sweeping factor continues to increase for FR2-2, long measurement delay would be a concern for mobility. However, from UE implementation, the more higher beamforming gain compared with FR2-1 would be needed to meet coverage. Therefore, we propose reasonable trade-off need to discussed.

Scheduling restrictions

The scheduling restriction was also discussed in the last meeting. All companies agreed that whether to update the scheduling restriction for L1/L3 measurement based on beam switching time and synchronization assumptions which need more inputs from RAN1 and RF. There is no conclusion for UE beam switching time from Issue 1-11.2 of WF [2] in RF group. In our understanding, this may depend on UE implement. Similar to BWP switching delay, the UE beam switching time may be considered to define different types based on the UE capability. According to different types of UE beam switching time, the scheduling restriction for L1/L3 shall be considered separately.

Regarding the synchronization assumptions, there is also no conclusion related to TAE requirements according to [3]. In the current requirements, cell phase synchronization accuracy is defined as 3us which is more than twice as long as the symbol length of 960kHz. This may cause more symbols which need to be performed scheduling restrictions for L3 measurement if reusing the current synchronization assumption.

Observation 1: More symbols need to be performed scheduling restrictions for L3 measurement if reusing the current synchronization assumption.

Since the UE beam switching time and TAE requirements is still in discussion, we propose that scheduling restrictions need to continue waiting for the conclusion about beam switching time and TAE requirements from RF session.

Proposal 2: Scheduling restrictions need to continue waiting for the conclusion about beam switching time and TAE requirements from RF session.

RRM requirements impact due to higher SCS

In the last meeting, we provided the preliminary simulation results for cell detection, PBCH detection and SSB measurement. Most companies also agreed to further clarify the issue. In this paper, we give further analysis on the impact on intra-frequency measurement and inter-frequency measurement due to higher SCS of 480kHz and 960kHz. Compared with FR2-1, symbol length would become shorter for new SCS. If reusing the channel model for legacy FR2, the delay spread may occupy much percentage of CP. And this may cause imperfection on the channel estimation and affect demodulation performance. We copy the delay profiles on TDL-A 30 and TDL-C 60 for FR2-1 from [38.104].

Table B.2.1.2-1: Delay profiles for NR channel models Model; Number of channel taps; Delay spread (r.m.s.); Maximum excess tap delay (span); Delay resolution. TDLA30; 12; 30 ns; 290 ns; 5 ns. TDLC60; 12; 60 ns; 520 ns; 5 ns. Table B.2.1.2-2 TDLA30 (DS = 30 ns) Tap #; Delay [ns]; Power [dB]; Fading distribution. 1; 0; -15.5; Rayleigh. 2; 10; 0; Rayleigh. 3; 15; -5.1; Rayleigh. 4; 20; -5.1; Rayleigh. 5; 25; -9.6; Rayleigh. 6; 50; -8.2; Rayleigh. 7; 65; -13.1; Rayleigh. 8; 75; -11.5; Rayleigh. 9; 105; -11.0; Rayleigh. 10; 135; -16.2; Rayleigh. 11; 150; -16.6; Rayleigh. 12; 290; -26.2; Rayleigh. Table B.2.1.2-3 TDLC60 (DS = 60 ns) Tap #; Delay [ns]; Power [dB]; Fading distribution. 1; 0; -7.8; Rayleigh. 2; 15; -0.3; Rayleigh. 3; 40; 0; Rayleigh. 4; 50; -8.9; Rayleigh. 5; 55; -14.5; Rayleigh. 6; 75; -8.5; Rayleigh. 7; 80; -10.2; Rayleigh. 8; 130; -12.1; Rayleigh. 9; 210; -13.9; Rayleigh. 10; 300; -15.2; Rayleigh. 11; 360; -16.9; Rayleigh. 12; 520; -19.4; Rayleigh.

It can be seen that the maximum excess tap delay is around 290ns and 520ns for TDL-A30 and TDL-C60 respectively which is greater than CP length of 480kHz and 960kHz. In order to further analyse the above issue, we perform simulation according to difference SCS and the simulation assumption for PBCH detection is provided in Appendix 4.2. The result is shown in Table 1.

Table 1: PBCH detection delay

| Case |  | SCS (kHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 120 | 240 | 480 | 960 |
| Propagation Condition | SNR(dB) | Number of acquisition attempts | Number of acquisition attempts | Number of acquisition attempts | Number of acquisition attempts |
| TDL-A 30ns | -8 | 6 | 6 | 7 | 16 |
|  | -7 | 4 | 5 | 5 | 10 |
|  | -6 | 3 | 3 | 3 | 5 |
|  | -5 | 2 | 3 | 2 | 4 |
|  | -4 | 2 | 2 | 2 | 3 |
|  | -3 | 2 | 2 | 2 | 2 |
| TDL-C 60ns | -8 | 5 | 6 | 10 | 16 |
|  | -7 | 3 | 4 | 6 | 12 |
|  | -6 | 2 | 3 | 3 | 6 |
|  | -5 | 2 | 2 | 2 | 5 |
|  | -4 | 2 | 1 | 2 | 3 |
|  | -3 | 1 | 1 | 2 | 3 |

It is observed that as the SCS increases, the PBCH detection delay would also increase. Cell detection and SSB measurement may also follow the same rules. Therefore, we propose it may be necessary to perform the simulation to evaluate the requirements for new SCS of 480kHz and 960kHz.

Observation 2: The maximum excess tap delay for TDL-A 30 and TDL-C 60 is greater than CP length of 480kHz and 960kHz.

Observation 3: As the SCS increases, the PBCH detection delay would also increase.

Therefore, for intra-frequency measurement and inter-frequency measurement, further evaluations are needed. The simulation assumptions for cell detection, PBCH detection for SSB index acquisition and SSB measurement are provided in Appendix 4.1, 4.2 and 4.3 separately.

Cell detection

Our preliminary simulation results for cell detection are provided in Table 2 and Table 3 for case1 and case2, which can be found in the simulation assumptions for cell detection in Appendix, respectively.

Table 2: PSS/SSS detection delay for case1

| SCS | Propagation Condition | 90 percentiles of the PSS/SSS acquisition time |
| --- | --- | --- |
| 480kHz | AWGN | 1 |
|  | TDL-A-5 | 2 |
|  | TDL-A-10 | 2 |
|  | TDL-A-20 | 2 |
|  | TDL-A-30 | 3 |
|  | TDL-C-60 | 3 |
| 960kHz | AWGN | 1 |
|  | TDL-A-5 | 2 |
|  | TDL-A-10 | 2 |
|  | TDL-A-20 | 3 |
|  | TDL-A-30 | 3 |
|  | TDL-C-60 | 6 |

Table 3: PSS/SSS detection delay for case2

| SCS | Propagation Condition | 90 percentiles of the PSS/SSS acquisition time |
| --- | --- | --- |
| 480kHz | AWGN | 1 |
|  | TDL-A-5 | 2 |
|  | TDL-A-10 | 2 |
|  | TDL-A-20 | 2 |
|  | TDL-A-30 | 2 |
|  | TDL-C-60 | 3 |
| 960kHz | AWGN | 1 |
|  | TDL-A-5 | 2 |
|  | TDL-A-10 | 2 |
|  | TDL-A-20 | 3 |
|  | TDL-A-30 | 3 |
|  | TDL-C-60 | 6 |

From the simulation results, it can be seen that at least 6 samples are required for PSS/SSS detection for the 52.6GHz~71GHz under TDL-C 60 channel model. In existing intra-frequency measurement requirements and inter-frequency measurement requirements, 2~3 samples are assumed without considering AGC. If requirements for cell detection are still to be defined under same channel models used for legacy FR2, then cell detection delay should be extended.

However, if channel models in TS 38.808 are used to defined requirements, then cell detection requirements for legacy FR2 range can be reused. 2 samples for 480kHz SCS and 3 samples for 960kHz SCS can be assumed without considering AGC. In our view, with such higher SCS, TDL-C may not be appropriate for defining requirements. TDL-A 30ns would be used instead.

Proposal 3a: Cell detection period requirements for both intra-frequency measurement and inter-frequency measurement for FR2-1 can be reused for FR2-2 under the condition that channel model TDL-A 30ns is assumed.

Proposal 3b: Cell detection period requirements for both intra-frequency measurement and inter-frequency measurement for FR2-1 is extended by 3 samples for FR2-2.

PBCH detection for SSB index acquisition

Preliminary simulation results for SSB index acquisition are provided in Table 4.

Table 4: PBCH detection delay for case1

| Case |  | SCS(kHz) |  |
| --- | --- | --- | --- |
|  |  | 480 | 960 |
| Propagation Condition | SNR (dB) | Number of acquisition attempts | Number of acquisition attempts |
| AWGN | -8 | 1 | 1 |
|  | -7 | 1 | 1 |
|  | -6 | 1 | 1 |
|  | -5 | 1 | 1 |
|  | -4 | 1 | 1 |
|  | -3 | 1 | 1 |
| TDL-A5 | -8 | 6 | 7 |
|  | -7 | 4 | 6 |
|  | -6 | 3 | 3 |
|  | -5 | 2 | 2 |
|  | -4 | 2 | 2 |
|  | -3 | 2 | 2 |
| TDL-A10 | -8 | 6 | 7 |
|  | -7 | 5 | 4 |
|  | -6 | 3 | 4 |
|  | -5 | 2 | 2 |
|  | -4 | 2 | 2 |
|  | -3 | 2 | 2 |
| TDL-A20 | -8 | 6 | 10 |
|  | -7 | 4 | 6 |
|  | -6 | 3 | 4 |
|  | -5 | 2 | 3 |
|  | -4 | 2 | 3 |
|  | -3 | 2 | 2 |
| TDL-A30 | -8 | 7 | 16 |
|  | -7 | 5 | 10 |
|  | -6 | 3 | 5 |
|  | -5 | 2 | 4 |
|  | -4 | 2 | 3 |
|  | -3 | 2 | 2 |
| TDL-C60 | -8 | 10 | 16 |
|  | -7 | 6 | 12 |
|  | -6 | 3 | 6 |
|  | -5 | 2 | 5 |
|  | -4 | 2 | 3 |
|  | -3 | 2 | 3 |

It can be seen from the simulation results that at least 3 samples are required for PBCH detection for 480kHz SCS and 6 samples for 960kHz SCS for the 52.6GHz~71GHz under TDL-C 60 channel model. It can also be seen that PBCH detection is not very much sensitive to channel models.

In existing inter-frequency measurement requirements for FR2, 3 samples are assumed for SSB index acquisition. The requirements cannot be met for 960kHz SCS case. Therefore, the inter frequency SSB index acquisition delay should be extended.

Proposal 4: SSB index acquisition delay for FR2-2 should be extended compare to that for FR2-1.

SSB measurements

Preliminary simulation results for SSB measurements are provided in Table 5 and in section 4.4 in Appendix.

Table 5: SSB measurement results

| Channel | Samples | Delta RSRP [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS =480kHz |  |  | SCS=960kHz |  |  |
|  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -0.42 | 0.68 | 1.65 | -0.42 | 0.69 | 1.64 |
|  | 2 | -0.23 | 0.58 | 1.29 | -0.22 | 0.57 | 1.29 |
|  | 3 | -0.12 | 0.52 | 1.13 | -0.11 | 0.53 | 1.11 |
|  | 5 | -0.03 | 0.47 | 0.95 | -0.02 | 0.47 | 0.94 |
| TDL-A | 1 | -2.39 | 0.03 | 2.08 | -2.50 | 0.05 | 2.08 |
|  | 2 | -1.84 | 0.00 | 1.59 | -1.87 | 0.01 | 1.61 |
|  | 3 | -1.57 | -0.02 | 1.34 | -1.61 | -0.02 | 1.31 |
|  | 5 | -1.26 | -0.03 | 1.06 | -1.32 | -0.04 | 1.07 |
| TDL-C | 1 | -2.74 | 0.06 | 2.19 | -2.67 | 0.10 | 2.26 |
|  | 2 | -2.08 | 0.03 | 1.72 | -2.00 | 0.04 | 1.77 |
|  | 3 | -1.81 | -0.01 | 1.47 | -1.73 | 0.02 | 1.52 |
|  | 5 | -1.54 | -0.03 | 1.21 | -1.45 | -0.01 | 1.26 |

It can be seen from the simulation results that 3 samples are enough for SSB measurements for both 480kHz SCS and 960kHz SCS SSBs for FR2-2 under all simulated channel models.

In existing intra-frequency and inter-frequency measurement requirements for FR2-1, 3 samples without considering implementation margin are assumed for SSB measurements. Therefore, the existing requirements are also applicable for FR2-2.

Proposal 5: SSB measurement period requirements for FR2-1 can be reused for FR2-2 for both intra-frequency inter-frequency measurements.

Other areas

So far, regarding the following requirements has not been discussed for FR2-2.

- CGI identification requirement

- SFTD measurement requirement

RAN4 may need to study whether the feature of CGI identification and SFTD measurement needs to be supported for FR2-2. In addition, the above requirements are related to MIB decode or SIB1 decode and Rx beam scaling factor.  For the new SCS or frequency, the new requirements may need to be defined. Therefore, if the feature is supported, it is necessary to study whether the new requirements need to be defined for FR2-2 compared with FR2-1.

Proposal 6: RAN4 may need to study whether the feature of CGI identification and SFTD measurement needs to be supported for FR2-2.

Proposal 7: If the feature of CGI identification and SFTD measurement is supported for FR2-2, RAN4 need to study whether the new requirements need to be defined for FR2-2 compared with FR2-1.

Conclusion

Proposal 1: Rx beam sweeping scaling factor should be considered until there is specific conclusion for antenna element numbers.

Proposal 2: Scheduling restrictions need to continue waiting for the conclusion about beam switching time and TAE requirements from RF session.

Proposal 3a: Cell detection period requirements for both intra-frequency measurement and inter-frequency measurement for FR2-1 can be reused for FR2-2 under the condition that channel model TDL-A 30ns is assumed.

Proposal 3b: Cell detection period requirements for both intra-frequency measurement and inter-frequency measurement for FR2-1 is extended by 3 samples for FR2-2.

Proposal 4: SSB index acquisition delay for FR2-2 should be extended compare to that for FR2-1.

Proposal 5: SSB measurement period requirements for FR2-1 can be reused for FR2-2 for both intra-frequency inter-frequency measurements.

Proposal 6: RAN4 may need to study whether the feature of CGI identification and SFTD measurement needs to be supported for FR2-2.

Proposal 7: If the feature of CGI identification and SFTD measurement is supported for FR2-2, RAN4 need to study whether the new requirements need to be defined for FR2-2 compared with FR2-1.

Observation 1: More symbols need to be performed scheduling restrictions for L3 measurement if reusing the current synchronization assumption.

Observation 2: The maximum excess tap delay for TDL-A 30 and TDL-C 60 is greater than CP length of 480kHz and 960kHz.

Observation 3: As the SCS increases, the PBCH detection delay would also increase.

Appendix

The simulation assumptions for the new band are provided in Appendix.

Simulation Assumptions for cell detection

Simulation assumptions for cell detection for the new band are based on R4-1708698 with changes highlighted.

Table 1: Cell-specific Parameters for PSS/SSS Detection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| Carrier frequency |  | 60GHz | The same frequency as for Cell 1 is used |
| Subcarrier spacing | KHz | 480kHz;960kHz | The same values as for Cell 1 are used |
| RB Utilization | % | 100 | 100 |
| Power boosting for PSS and SSS | dB | 0 | 0 |
| Power boosting for Data and Control | dB | 0 | 0 |
| Data Modulation | - | QPSK | QPSK |
| CP Length | - | Normal | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 | 1 |
| SS burst set periodicity | ms | 5 | 5 |
| Relative Delay of 1st Path (synchronous) | μs | 0 | CP/2 |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| Es/Noc | dB | 6.4 | 1.25; FSS: 0.25; -0.75 (SINR: -6dB; FFS: -8dB, -7dB) |
| Number of Tx antennas | - | 1 | 1 |
| PSS Sequence ID | - | To be stated by companies together with the results (examples shows in Table 3, 4) | To be stated by companies together with the results (examples shows in Table 3, 4) |
| SSS Sequence ID | - | To be stated by companies together with the results (examples shows in Table 3, 4) | To be stated by companies together with the results (examples shows in Table 3, 4) |
| Propagation Condition [38.101-4] Propagation Condition [38.808] | - | TDL-A (30ns delay spread)/TDL-C (60ns delay spread) AWGN/TDL-A (5/10/20ns delay spread) |  |
| UE speed |  | 3km/h |  |
| Noc Model | - | AWGN noise |  |

Table 2: UE-specific Parameters for Cell Detection

| Simulation parameters | Comments/values |
| --- | --- |
| Prior knowledge of Cell 1 by the UE | Yes |
| False detect threshold | Required as in a real UE implementation |
| Performance criterion for comparison | 90th percentile acquisition time for “correct” cell detection of both PSS and SSS sequence IDs. |
| Receive antennas | 2 |

Table 3: Example Cell ID Combinations that can be simulated

| Case # | Cell 1 (Interferer Cell) |  | Cell 2 (Desired Cell) |  |
| --- | --- | --- | --- | --- |
| 1 | psc1 | ssc1 | psc2 | ssc2 |
| 2 | psc1 | ssc1 | psc1 | ssc3 |

Table 4: Example PSS, SSS indices that can be simulated simulations

| PSS Indices |  | SSS Indices |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Label | Code index | Label | Code index (m0, m1) | Cell ID | (,) |
| psc1 | 43 | ssc1 | (20,0) | 337 | (112, 1) |
| psc2 | 86 | ssc2 | (40, 0) | 674 | (224, 2) |
|  |  | ssc3 | (35,0) | 673 | (224, 1) |

Performance Metric

For NR-PSS/SSS acquisition time, the detection is claimed to be successful only when both the PSS and SSS are correctly detected and the timing offset is within half CP length.

Simulation Assumptions of PBCH detection

Simulation assumptions for PBCH detection for the new band are based on R4-1713437 with changes highlighted.

Case1: PBCH reading

Basic SI reading time = the number of SS-blocks required for 99% to successfully decode the PBCH.

Case2: PBCH-DMRS time index reading (Without decode PBCH data)

DMRS time index reading time = the number of SS-blocks required for 99% to successfully reading DMRS time index.

Table 1: Parameters for basic SI reading (PBCH acquisition)

| Parameter | Unit | Value |
| --- | --- | --- |
| Carrier frequency | GHz | 60GHz |
| Subcarrier spacing | KHz | 480kHz;960kHz |
| Number of Tx antennas | - | 1 Assuming only 1Tx port is used |
| Number of Rx antennas | - | 2 |
| CP Length | - | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 |
| SS burst set periodicity | ms | 5 |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| RB Utilization | % | 100 |
| PBCH symbols within the SS block |  | According to the RAN1 agreements |
| Data and Control Power offset with respect to PSS and SSS | dB | 0 |
| PBCH power offset with respect to PBCH-DMRS | dB | 0 |
| PBCH-DMRS power offset with respect to PSS and SSS | dB | 0 |
| PSS and SSS sequences | - | According to the RAN1 agreements |
| PBCH-DMRS sequences | - | According to the RAN1 agreements |
| PBCH-DMRS RE positions within the PBCH resource | - | According to the RAN1 agreements |
| PBCH Channel coding |  | Polar code with 512 length and 24bit DCRC |
| PBCH Modulation | - | QPSK |
| PBCH Payload (including the CRC) | bits | 56bit (CRC 24bit) |
| PBCH SNR | dB | -10 : 0 dB, at least including -6dB,-8 dB |
| Propagation Condition [38.101-4] Propagation Condition [38.808] | - | TDL-A (30ns delay spread)/TDL-C (60ns delay spread) AWGN/TDL-A (5/10/20ns delay spread) |
| UE speed |  | 3km/h |
| Detection Method |  | Baseline: on one shot detection (No combination for different PBCHs,) FFS Soft- combining |

Performance Metric

The proposed simulation assumptions are used to derive the Basic SI reading time.

Basic SI reading time = the number of SS-blocks required for 99% to successfully decode the PBCH.

- 4.3 Simulation Assumptions for SSB measurements

Simulation assumptions for SSB measurements for the new band are based on R4-1706904 with changes highlighted.

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 60GHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | No / Yes |
| DRX | No |
| BS transmit antennas for SS blocks | 1 tx or single layer transmissions |
| UE receive antennas | 1 |
| Data and control channel subcarrier spacing | The same as SS block subcarrier spacing |
| Measurement period (in number of measurement samples) | 1 2 3 5; other number of samples may also be studied upon a need |
| - Subcarrier spacing | 480 kHz; 960 kHz |
| - Number of SS blocks per SS burst set | 1 |
| - SS burst periodicity | 5ms |
| - Number of transmit antenna ports | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) |
| - SS block BW | 127 subcarriers for SS, 288 subcarriers for PBCH |
| - Actual SS block transmissions | always transmitted |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS and NR-SSS sequences | - | To be indicated by companies | To be indicated by companies |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| Relative Delay of 1st Path | µs | 0 | CP/2 |
| SNR | dB | 6 dB | 1 dB |
| Es/IoT (calculated from SNR) | dB | N/A 2.46dB | -5.97 dB |
| Propagation conditions [38.101-4] | - | AWGN/TDL-A (30ns delay spread)/TDL-B (100ns delay spread)/TDL-C (300ns delay spread) for 4GHz TDL-A (30ns delay spread)/TDL-C (60ns delay spread) for 30GHzNote |  |
| UE speed |  | 3km/h |  |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned |  |  |  |

Performance Metrics

For different estimation time periods, the CDF curves and 90th %-iles are to be provided for (without RF impairment margin):

- Delta measurement   = (estimated measurement – ideal measurement) [dB];

- 4.4 Simulation results for SSB measurements

- References

[1] R4-2115351 WF on NR_ext_to_71GHz_Part_1

[2] R4-2114989 WF 60GHz UE

[3] R4-2115643