3GPP TSG-RAN WG4 Meeting # 101-e	R4-2117872

# Electronic Meeting, November 1-12, 2021

Source: 	Huawei, HiSilicon

Title: 	overall method comparisons

Agenda Item:	9.2.3.4

Document for:	Approval

1 Introduction

A new SI [1] has been approved on efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths. There are four methods being discussed in previous meetings.

In the contribution, we provide overall comparisons on the four methods.

2 Discussion

We take 13 MHz as an example, a summary on the four methods can be found in the following table.

Table 2-1 summary on the options for irregular channel bandwidth

|  | Option1: | Option2: | Option 3: | Option 4: |
| --- | --- | --- | --- | --- |
| Solutions | Wider CBW | Overlapping CBWs from network perspective (one cell approach) | Overlapping CBWs from UE perspective (two cell/CA approach) | Overlapping CBWs from UE perspective (one cell approach) |
| # of cell per UE/gNB | 1 | 1 or 2 | 2 | 1 |
| # of SSB | 1 | 1 or 2 | 2 | 1 |
| UE Supports overall spectrum in DL | Yes | No | Yes | Yes |
| Support of legacy UE | Yes | Yes | Yes | Yes |
| Pros | UE uses legacy 15MHz filter; One Legacy SSB can be reused | UE use legacy 10MHz filter One legacy SSB can be reused for >10MHz case | UE use legacy 10MHz filter | UE use two legacy 10 MHz filter to receive 13 MHz. One Legacy SSB can be reused |
| Cons | Performance degradation since UE use 15M filter to receive 13M BW There is co-existence issue during the initial access. There may be 100 KHz channel raster issue. | A UE can only use partial spectrum, i.e., 10MHz; Two legacy SSB needed for <10MHz case | Two Legacy SSBs needed, one for cell1, one for cell 2 UE need to support NC CA | UE need to support NC CA |

For Option 1 wider channel bandwidth approach, since no dedicated channel filter is assumed, it is foreseen that there are some performance degradation due to UE ACS/blocking and BS SEM. Hence the wider channel bandwidth approach is applicable to co-location case, e.g. operator holds 10 MHz and some of the spectrum is used for other RAT. And for this option, there is no specification impact at all and can be configured in the co-location deployments or similar scenarios, with BS and UE complied with existing conformance testing.

For Option 2 overlapping CBWs from network perspective. There are two main drawbacks, one is UE cannot receive the whole spectrum, the other is that two SSB is needed for the scenarios with less than 10 MHz.

For Option 3 overlapping CA case, there is additional overhead due to the duplicated SSB transmission as well as other radio resource related to two cells. Furthermore, the irregular channel being discussed are relatively small bandwidth. Hence the two cell method(s) are not preferred.

Option 4 use single SSB and utilize the whole spectrum, and can reuse the existing regular filters. It provide a good balance of implementation complexity and spectrum utilization. Meanwhile, the solution is optional with UE capability. And the support of option 4 make the network can naturally support 2. Hence, we propose to adopt the combination of option 4 and 2 to address operator’s request on licensed spectrum that is not aligned with existing NR channel bandwidths.

Proposal: it is proposed to adopt the combination of option 4 and 2 to address operator’s request on licensed spectrum that is not aligned with existing NR channel bandwidths.

Conclusions

In the contribution, we provide discussion on overall comparisons of the four methods.

Proposal: it is proposed to adopt the combination of option 4 and 2 to address operator’s request on licensed spectrum that is not aligned with existing NR channel bandwidths.

References

RP-202103, New SID: Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths, T-Mobile USA, Ericsson

R4-2114998, WF on widerCBW Approach, Ericsson

R4-2106485, Consideration on use of larger channel bandwidth, Huawei, HiSilicon

R4-2113045, Evaluation for use of larger channel bandwidth, Huawei, HiSilicon