3GPP TSG-RAN WG4 Meeting #101-e		R4-2118006

E-meeting, 1 – 12 November, 2021

Agenda item:	8.12.3.1

Source:	Intel Corporation

Title:	Simulation results for PUSCH requirements for FR1 256QAM

Document for:	Information

# Introduction

In the previous RAN4 meeting, WF on PUSCH 256QAM performance requirements [1] was agreed and all open issues were resolved.

In this paper we provide our alignment and impairment results for FR1 BS 256QAM requirements.

# Simulation results

In Figure 1 we provide the detailed simulation results for FR1 256QAM scenario.

| Type A, 15 kHz, 5 MHz | Type B, 15 kHz, 5 MHz |
| --- | --- |
| Type A, 15 kHz, 10 MHz | Type B, 15 kHz, 10 MHz |
| Type A, 30 kHz, 10 MHz | Type B, 30 kHz, 10 MHz |
| Type A, 30 kHz, 40 MHz | Type B, 30 kHz, 40 MHz |
| Type A, 30 kHz, 100 MHz | Type B, 30 kHz, 100 MHz |
| Figure 1. FR1 256QAM simulation results. |  |

In Table 1 we provide the summary of alignment and impairment results with and without Tx EVM.

Table 1. Summary of alignment and impairment results

| Antenna config | SCS, CBW | Type A |  | Type B |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Alignment results | Impairment results | Alignment results | Impairment results |
| 1x2 | 15 kHz, 5 MHz | 16.4 | 19.2 | 16.4 | 19.2 |
|  | 15 kHz, 10 MHz | 16.8 | 19.6 | 16.8 | 19.6 |
|  | 30 kHz, 10 MHz | 16.6 | 19.5 | 16.6 | 19.4 |
|  | 30 kHz, 40 MHz | 17.3 | 20.2 | 17.3 | 20.2 |
|  | 30 kHz, 100 MHz | 18.6 | 21.5 | 18.6 | 21.5 |
| 1x4 | 15 kHz, 5 MHz | 13.0 | 15.8 | 13.0 | 15.8 |
|  | 15 kHz, 10 MHz | 13.4 | 16.2 | 13.4 | 16.2 |
|  | 30 kHz, 10 MHz | 13.3 | 16.2 | 13.3 | 16.2 |
|  | 30 kHz, 40 MHz | 13.7 | 16.6 | 13.7 | 16.6 |
|  | 30 kHz, 100 MHz | 14.5 | 17.4 | 14.6 | 17.4 |
| 1x8 | 15 kHz, 5 MHz | 10.3 | 13.1 | 10.3 | 13.1 |
|  | 15 kHz, 10 MHz | 10.5 | 13.3 | 10.4 | 13.3 |
|  | 30 kHz, 10 MHz | 10.5 | 13.3 | 10.5 | 13.3 |
|  | 30 kHz, 40 MHz | 10.7 | 13.6 | 10.7 | 13.6 |
|  | 30 kHz, 100 MHz | 11.0 | 13.8 | 11.0 | 13.8 |

# Conclusion

In this paper we provided our alignment and impairment results for FR1 BS 256QAM requirements.

# References

- R4-2115748 “Way forward for FR1 PUSCH 256QAM performance requirements”, Huawei, HiSilicon, RAN4 #100-e, August 2021