3GPP TSG-RAN WG4 Meeting #101-e		R4-2118032

E-meeting, November 01-12, 2021

Agenda item:	8.16.7.4

Source:	Intel Corporation

Title:	Discussion on BWP switching delay for NR 52.6 – 71 GHz

Document for:	Discussion

# Introduction

In this paper we discuss the requirements for cross-carrier active BWP switching delay for new numerologies defined for NR 52.6 – 71 GHz operation.

# Discussion

The requirements for DCI-based BWP switching delay for the case of cross-carrier scheduling are defined in 3GPP TS38.133 in the following way:

For DCI-based BWP switch, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay + Y which starts from the beginning of DL slot n. Where, - Y=0, if the serving cell where UE receives DCI for BWP switch request is same as the serving cell on which BWP switch occurs. - Y equals to the length of 1 slot, if the serving cell where UE receives DCI for BWP switch is different from the serving cell on which BWP switch occurs for any involved serving cell. In this scenario, TBWPswitchDelay + Y shall follow the smaller SCS of scheduling cell, scheduled cells before and scheduled cells after active BWP change.

As we can see, 1 extra slot is considered for the case of cross-carrier scheduling to address the difference in the receive timing on different carriers. However, the slot length for 480kHz and 960kHz SCS is short and relative receive timing difference between slot timing boundary of different carriers may be longer than 1 slot.

Proposal 1: MRTD value should be considered for BWP switching delay definition in cross-carrier scheduling case.

Receive timing difference may increase the overall BWP switching delay if the signal from the cell carrying DCI with BWP switch request comes later that the signal from the cell where BWP switch should occur. The example is shown in Figure 1 for clarification. The BWP switch request for CC2 is sent in slot N of CC1. Due to imperfections in frame alignment between CCs and due to propagation delay difference, the signal from CC1 comes later. When the signal is received, we need to consider 3 more symbols to read DCI in CC1. Only after that CC2 will start BWP switching. Therefore, the TBWPswitchDelay can be calculated as follows:

TBWPswitchDelay = ⌈(MRTD + 3*Tsymb,DCI + BWP switch duration ) / Tslot ⌉

Figure 1. Cross-carrier BWP switch time budget

Proposal 2: RAN4 to define BWP switching delay requirements in cross-carrier scheduling case using the following formula:

TBWPswitchDelay = ⌈ (MRTD + 3*Tsymb,DCI + BWP switch duration) / Tslot, ⌉	, where

MRTD – maximum receive timing difference between the serving cell where UE receives DCI for BWP switch and the serving cell on which BWP switch occurs

Tsymb,DCI – symbol length at the serving cell where UE receives DCI for BWP switch

BWP switch duration – time required for the UE to reconfigure its BWP

Tslot – slot length at the serving cell on which BWP switch occurs

Th value for MRTD in the formula from Proposal 2 depends on the specific deployment scenario. During RAN4 #100-e the following agreement was made [1]:

- Define the requirements for the following deployment scenarios with equal priority: - Standalone single carrier and CA in FR2-2 - FR2-2 CA and DC with anchor on FR1

Based on that agreements and on the fact that there is only one agreed band in FR2-2 for now (so no inter-band CA/DC within FR2-2) the following MRTD scenarios should be considered:

- Intra-band non-contiguous NR CA within FR2-2

- Inter-band NR CA between FR1 and FR2-2

- Inter-band NR DC between FR1 and FR2-2 (no differentiation between synchronous and asynchronous cases as it is discussed in [2])

Tables 1-3 summarize the calculation of TBWPswitchDelay for the abovementioned scenarios, for Type 1 and Type 2 UEs for different SCS of the serving cell where UE receives DCI for BWP switch (SCSDCI) and for different SCS of the serving cell on which BWP switch occurs (SCSBWPswitch).

Table 1 Summary on cross-carrier BWP switch delay for intra-band non-contiguous NR CA within FR2-2

| SCSBWPswitch (kHz) | Tslot (us) | MRTD (us) | SCSDCI (kHz) | 3*Tsymb,DCI (us) | BWP switch duration (us) | BWP switch delay TBWPswitchDelay (slots) |
| --- | --- | --- | --- | --- | --- | --- |
| 480 | 31.25 | [0.26] | 960 | 3.3482 | 600 | 20 |
|  |  |  |  |  | 2000 | 65 |
|  |  |  | 480 | 6.6964 | 600 | 20 |
|  |  |  |  |  | 2000 | 65 |
|  |  |  | 120 | 26.7857 | 600 | 21 |
|  |  |  |  |  | 2000 | 65 |
| 960 | 15.625 |  | 960 | 3.3482 | 600 | 39 |
|  |  |  |  |  | 2000 | 129 |
|  |  |  | 480 | 6.6964 | 600 | 39 |
|  |  |  |  |  | 2000 | 129 |
|  |  |  | 120 | 26.7857 | 600 | 41 |
|  |  |  |  |  | 2000 | 130 |

Table 2 Summary on cross-carrier BWP switch delay for inter-band NR CA between FR1 and FR2-2

| SCSBWPswitch (kHz) | Tslot (us) | MRTD (us) | SCSDCI (kHz) | 3*Tsymb,DCI (us) | BWP switch duration (us) | BWP switch delay TBWPswitchDelay (slots) |
| --- | --- | --- | --- | --- | --- | --- |
| 480 | 31.25 | [25] | 60 | 53.5714 | 600 | 22 |
|  |  |  |  |  | 2000 | 67 |
|  |  |  | 30 | 107.1429 | 600 | 24 |
|  |  |  |  |  | 2000 | 69 |
|  |  |  | 15 | 214.2857 | 600 | 27 |
|  |  |  |  |  | 2000 | 72 |
| 960 | 15.625 |  | 60 | 53.5714 | 600 | 44 |
|  |  |  |  |  | 2000 | 134 |
|  |  |  | 30 | 107.1429 | 600 | 47 |
|  |  |  |  |  | 2000 | 137 |
|  |  |  | 15 | 214.2857 | 600 | 54 |
|  |  |  |  |  | 2000 | 144 |

Table 3 Summary on cross-carrier BWP switch delay for inter-band NR DC between FR1 and FR2-2

| SCSBWPswitch (kHz) | Tslot (us) | MRTD (us) | SCSDCI (kHz) | 3*Tsymb,DCI (us) | BWP switch duration (us) | BWP switch delay TBWPswitchDelay (slots) |
| --- | --- | --- | --- | --- | --- | --- |
| 480 | 31.25 | [33] | 60 | 53.5714 | 600 | 22 |
|  |  |  |  |  | 2000 | 67 |
|  |  |  | 30 | 107.1429 | 600 | 24 |
|  |  |  |  |  | 2000 | 69 |
|  |  |  | 15 | 214.2857 | 600 | 28 |
|  |  |  |  |  | 2000 | 72 |
| 960 | 15.625 |  | 60 | 53.5714 | 600 | 44 |
|  |  |  |  |  | 2000 | 134 |
|  |  |  | 30 | 107.1429 | 600 | 48 |
|  |  |  |  |  | 2000 | 137 |
|  |  |  | 15 | 214.2857 | 600 | 55 |
|  |  |  |  |  | 2000 | 144 |

Proposal 3: RAN4 to define requirements for cross-carrier BWP switch delay as it is shown in Tables 4-6

Table 4: Cross-carrier BWP switch delay for intra-band non-contiguous NR CA within FR2-2

|  | NR Slot length (ms) | SCSDCI (kHz) | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Type 1 | Type 2 |
| 5 | 31.25 | 120 | 20 | 65 |
|  |  | 480 | 20 | 65 |
|  |  | 960 | 21 | 65 |
| 6 | 15.625 | 120 | 39 | 129 |
|  |  | 480 | 39 | 129 |
|  |  | 960 | 41 | 130 |

Table 5: Cross-carrier BWP switch delay for inter-band NR CA between FR1 and FR2-2

|  | NR Slot length (ms) | SCSDCI (kHz) | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Type 1 | Type 2 |
| 5 | 31.25 | 60 | 22 | 67 |
|  |  | 30 | 24 | 69 |
|  |  | 15 | 27 | 72 |
| 6 | 15.625 | 60 | 44 | 134 |
|  |  | 30 | 47 | 137 |
|  |  | 15 | 54 | 144 |

Table 6: Cross-carrier BWP switch delay for inter-band NR DC between FR1 and FR2-2

|  | NR Slot length (ms) | SCSDCI (kHz) | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Type 1 | Type 2 |
| 5 | 31.25 | 60 | 22 | 67 |
|  |  | 30 | 24 | 69 |
|  |  | 15 | 28 | 72 |
| 6 | 15.625 | 60 | 44 | 134 |
|  |  | 30 | 48 | 137 |
|  |  | 15 | 55 | 144 |

Note: If the MRTD requirements different from the used in calculation will be agreed, tables 4-6 will be changed accordingly

# Conclusion

In this paper we discussed the requirements for cross-carrier active BWP switching delay for new numerologies defined for NR 52.6 – 71 GHz operation. The following proposals were made:

Proposal 1: MRTD value should be considered for BWP switching delay definition in cross-carrier scheduling case.

Proposal 2: RAN4 to define BWP switching delay requirements in cross-carrier scheduling case using the following formula:

TBWPswitchDelay = ⌈ (MRTD + 3*Tsymb,DCI + BWP switch duration ) / Tslot ⌉	, where

MRTD – maximum receive timing difference between the serving cell where UE receives DCI for BWP switch and the serving cell on which BWP switch occurs

Tsymb,DCI – symbol length at the serving cell where UE receives DCI for BWP switch

BWP switch duration – time required for the UE to reconfigure its BWP

Tslot – slot length at the serving cell on which BWP switch occurs

Proposal 3: RAN4 to define requirements for cross-carrier BWP switch delay as it is shown in Tables 4-6

Table 4: Cross-carrier BWP switch delay for intra-band non-contiguous NR CA within FR2-2

|  | NR Slot length (ms) | SCSDCI (kHz) | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Type 1 | Type 2 |
| 5 | 31.25 | 120 | 20 | 65 |
|  |  | 480 | 20 | 65 |
|  |  | 960 | 21 | 65 |
| 6 | 15.625 | 120 | 39 | 129 |
|  |  | 480 | 39 | 129 |
|  |  | 960 | 41 | 130 |

Table 5: Cross-carrier BWP switch delay for inter-band NR CA between FR1 and FR2-2

|  | NR Slot length (ms) | SCSDCI (kHz) | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Type 1 | Type 2 |
| 5 | 31.25 | 60 | 22 | 67 |
|  |  | 30 | 24 | 69 |
|  |  | 15 | 27 | 72 |
| 6 | 15.625 | 60 | 44 | 134 |
|  |  | 30 | 47 | 137 |
|  |  | 15 | 54 | 144 |

Table 6: Cross-carrier BWP switch delay for inter-band NR DC between FR1 and FR2-2

|  | NR Slot length (ms) | SCSDCI (kHz) | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Type 1 | Type 2 |
| 5 | 31.25 | 60 | 22 | 67 |
|  |  | 30 | 24 | 69 |
|  |  | 15 | 28 | 72 |
| 6 | 15.625 | 60 | 44 | 134 |
|  |  | 30 | 48 | 137 |
|  |  | 15 | 55 | 144 |

Note: If the MRTD requirements different from the used in calculation will be agreed, tables 4-6 will be changed accordingly

# References

- R4-2115351, “WF on R17 NR_ext_to_71GHz_RRM_1”, RAN4 #100-e August 16 – 27, 2021

- R4-2118030, “Discussion on timing requirements for NR 52.6 – 71 GHz”, Intel Corporation, RAN4 #101-e, November 01-12, 2021