3GPP TR 37.717-11-21 V0.7.0 (2021-11)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Networks;

Dual Connectivity (DC) of x bands (x=1, 2, 3, 4) LTE inter-band CA (x DL/1 UL) and 2 bands NR Inter-band CA  (2 DL/1 UL) band combinations

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	37

1	Scope	39

2	References	39

3	Definitions, symbols and abbreviations	39

3.1	Definitions	39

3.2	Symbols	40

3.3	Abbreviations	40

4	Background	40

4.1	TR Maintenance	40

5	Dual Connectivity band combinations of LTE x bands DL/1UL(x=1,2,3,4) + NR 2 bands DL/1UL: General Part	41

5.1	UE RF architectures	41

5.2	General treatment of ∆TIB and ∆RIB values	41

5.3	Summary of self-interference analysis	41

6	Dual Connectivity band combinations of LTE 1 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	90

6.1	DC_3_n1-n257, DC_3-3_n1-n257	90

6.1.1	Operating bands for DC	90

6.1.2	Channel bandwidths per operating band for DC	90

6.1.3	Co-existence studies	91

6.1.4	∆TIB and ∆RIB values	91

6.1.5	MSD	91

6.2	DC_7_n1-n257, DC_7-7_n1-n257	92

6.2.1	Operating bands for DC	92

6.2.2	Channel bandwidths per operating band for DC	92

6.2.3	Co-existence studies	93

6.2.4	∆TIB and ∆RIB values	93

6.2.5	MSD	93

6.3	DC_11_n3-n28	93

6.3.1	Operating bands for DC	93

6.3.2	Channel bandwidths per operating band for DC	94

6.3.3	Co-existence studies	94

6.3.4	∆TIB and ∆RIB values	94

6.3.5	MSD	95

6.4	DC_1_n28-n41	95

6.4.1	Operating bands for DC	95

6.4.2	Channel bandwidths per operating band for DC	96

6.4.3	Co-existence studies	96

6.4.4	∆TIB and ∆RIB values	98

6.4.5	MSD	99

6.5	DC_3_n28-n41	100

6.5.1	Operating bands for DC	100

6.5.2	Channel bandwidths per operating band for DC	100

6.5.3	Co-existence studies	100

6.5.4	∆TIB and ∆RIB values	102

6.5.5	MSD	103

6.6	DC_2_n48-n66	104

6.6.1	Operating bands for DC	104

6.6.2	Configuration for DC	105

6.6.3	Co-existence studies	105

6.6.4	∆TIB and ∆RIB values	105

6.6.5	MSD	106

6.7	DC_48_n25-n48	106

6.7.1	Operating bands for DC	106

6.7.2	Configuration for DC	107

6.7.3	Co-existence studies	107

6.7.4	∆TIB and ∆RIB values	107

6.7.5	MSD	107

6.8	DC_48_n48-n66	108

6.8.1	Operating bands for DC	108

6.8.2	Configuration for DC	108

6.8.3	Co-existence studies	108

6.8.4	∆TIB and ∆RIB values	108

6.8.5	MSD	109

6.9	DC_66_n25-n48	109

6.9.1	Operating bands for DC	109

6.9.2	Configuration for DC	109

6.9.3	Co-existence studies	110

6.9.4	∆TIB and ∆RIB values	110

6.9.5	MSD	110

6.10	DC_7_n40-n78	111

6.10.1	Operating bands for DC	111

6.10.2	Configuration for DC	111

6.10.3	Co-existence studies	111

6.10.4	∆TIB and ∆RIB values	111

6.10.5	MSD	112

6.11	DC_8_n40-n78	113

6.11.1	Operating bands for DC	113

6.11.2	Configuration for DC	113

6.11.3	Co-existence studies	113

6.11.4	∆TIB and ∆RIB values	113

6.11.5	MSD	114

6.12	DC_28_n1-n40	115

6.12.1	Operating bands for DC	115

6.12.2	Configuration for DC	115

6.12.3	Co-existence studies	115

6.12.4	∆TIB and ∆RIB values	115

6.12.5	MSD	116

6.13	DC_1_n3-n77	116

6.13.1	Operating bands for DC	116

6.13.2	Channel bandwidths per operating band for DC	118

6.13.3	Co-existence studies	118

6.13.4	∆TIB and ∆RIB values	119

6.13.5	MSD	119

6.14	DC_8_n3-n77	120

6.14.1	Operating bands for DC	120

6.14.2	Channel bandwidths per operating band for DC	122

6.14.3	Co-existence studies	122

6.14.4	∆TIB and ∆RIB values	123

6.14.5	MSD	123

6.15	DC_1_n3-n41	124

6.15.1	Operating bands for DC	124

6.15.2	Channel bandwidths per operating band for DC	125

6.15.3	Co-existence studies	125

6.15.4	∆ TIB and ∆RIB values	125

6.15.5	MSD	126

6.16	DC_1_n41-n77	127

6.16.1	Operating bands for DC	127

6.16.2	Channel bandwidths per operating band for DC	127

6.16.3	Co-existence studies	127

6.16.4	∆TIB and ∆RIB values	128

6.16.5	MSD	128

6.17	DC_3_n3-n41	129

6.17.1	Operating bands for DC	129

6.17.2	Channel bandwidths per operating band for DC	129

6.17.3	Co-existence studies	129

6.17.4	∆TIB and ∆RIB values	130

6.17.5	MSD	130

6.18	DC_3_n41-n77	131

6.18.1	Operating bands for DC	131

6.18.2	Channel bandwidths per operating band for DC	131

6.18.3	Co-existence studies	131

6.18.4	∆TIB and ∆RIB values	132

6.18.5	MSD	132

6.19	DC_41_n3-n41	133

6.19.1	Operating bands for DC	133

6.19.2	Channel bandwidths per operating band for DC	133

6.19.3	Co-existence studies	133

6.19.4	∆TIB and ∆RIB values	134

6.19.5	MSD	134

6.20	DC_41_n41-n77	135

6.20.1	Operating bands for DC	135

6.20.2	Channel bandwidths per operating band for DC	135

6.20.3	Co-existence studies	135

6.20.4	∆TIB and ∆RIB values	136

6.20.5	MSD	136

6.21	DC_41_n41-n78	136

6.21.1	Operating bands for DC	136

6.21.2	Channel bandwidths per operating band for DC	137

6.21.3	Co-existence studies	137

6.21.4	∆TIB and ∆RIB values	137

6.21.5	MSD	138

6.22	DC_2_n7-n66	138

6.22.1	Operating bands for DC	138

6.22.2	Channel bandwidths per operating band for DC	139

6.22.3	Co-existence studies	139

6.22.4	∆TIB and ∆RIB values	139

6.22.5	MSD	140

6.23	DC_2_n38-n66	140

6.23.1	Operating bands for DC	140

6.23.2	Channel bandwidths per operating band for DC	141

6.23.3	Co-existence studies	141

6.23.4	∆TIB and ∆RIB values	141

6.23.5	MSD	142

6.24	DC_2_n66-n78	143

6.24.1	Operating bands for DC	143

6.24.2	Channel bandwidths per operating band for DC	143

6.24.3	Co-existence studies	143

6.24.4	∆TIB and ∆RIB values	144

6.24.5	MSD	144

6.25	DC_12_n7-n66	145

6.25.1	Operating bands for DC	145

6.25.2	Channel bandwidths per operating band for DC	145

6.25.3	Co-existence studies	145

6.25.4	∆TIB and ∆RIB values	146

6.25.5	MSD	146

6.26	DC_66_n38-n66	147

6.26.1	Operating bands for DC	147

6.26.2	Channel bandwidths per operating band for DC	147

6.26.3	Co-existence studies	147

6.26.4	∆TIB and ∆RIB values	148

6.26.5	MSD	148

6.27	DC_19_n1-n77	149

6.27.1	Operating bands for DC	149

6.27.2	Configuration for DC	149

6.27.3	Co-existence studies	149

6.27.4	∆TIB and ∆RIB values	149

6.27.5	MSD	150

6.28	DC_19_n1-n78	151

6.28.1	Operating bands for DC	151

6.28.2	Configuration for DC	151

6.28.3	Co-existence studies	151

6.28.4	∆TIB and ∆RIB values	151

6.28.5	MSD	152

6.29	DC_11_n3-n77	153

6.29.1	Operating bands for DC	153

6.29.2	Channel bandwidths per operating band for DC	154

6.29.3	Co-existence studies	154

6.29.4	∆TIB and ∆RIB values	155

6.29.5	MSD	155

6.30	DC_11_n28-n77	157

6.30.1	Operating bands for DC	157

6.30.2	Channel bandwidths per operating band for DC	158

6.30.3	Co-existence studies	158

6.30.4	∆TIB and ∆RIB values	159

6.30.5	MSD	159

6.31	DC_42_n3-n28	160

6.31.1	Operating bands for DC	160

6.31.2	Channel bandwidths per operating band for DC	161

6.31.3	Co-existence studies	162

6.31.4	∆TIB and ∆RIB values	162

6.31.5	MSD	162

6.32	DC_42_n3-n77	163

6.32.1	Operating bands for DC	163

6.32.2	Channel bandwidths per operating band for DC	164

6.32.3	Co-existence studies	166

6.32.4	∆TIB and ∆RIB values	166

6.32.5	MSD	166

6.33	DC_41_n28-n41	167

6.33.1	Operating bands for DC	167

6.33.2	Channel bandwidths per operating band for DC	167

6.33.3	Co-existence studies	168

6.33.4	∆TIB and ∆RIB values	171

6.33.5	MSD	171

6.34	DC_18_n28-n41	172

6.34.1	Operating bands for DC	172

6.34.2	Channel bandwidths per operating band for DC	172

6.34.3	Co-existence studies	173

6.34.4	∆TIB and ∆RIB values	173

6.34.5	MSD	173

6.35	DC_18_n28-n77	174

6.35.1	Operating bands for DC	174

6.35.2	Channel bandwidths per operating band for DC	175

6.35.3	Co-existence studies	175

6.35.4	∆TIB and ∆RIB values	175

6.35.5	MSD	176

6.36	DC_18_n28-n78	177

6.36.1	Operating bands for DC	177

6.36.2	Channel bandwidths per operating band for DC	177

6.36.3	Co-existence studies	177

6.36.4	∆TIB and ∆RIB values	178

6.36.5	MSD	178

6.37	DC_18_n3-n41	179

6.37.1	Operating bands for DC	179

6.37.2	Channel bandwidths per operating band for DC	179

6.37.3	Co-existence studies	179

6.37.4	∆TIB and ∆RIB values	180

6.37.5	MSD	180

6.38	DC_18_n41-n77	181

6.38.1	Operating bands for DC	181

6.38.2	Channel bandwidths per operating band for DC	182

6.38.3	Co-existence studies	182

6.38.4	∆TIB and ∆RIB values	182

6.38.5	MSD	183

6.39	DC_18_n41-n78	184

6.39.1	Operating bands for DC	184

6.39.2	Channel bandwidths per operating band for DC	184

6.39.3	Co-existence studies	185

6.39.4	∆TIB and ∆RIB values	185

6.39.5	MSD	185

6.40	DC_19_n1-n79	186

6.40.1	Operating bands for DC	186

6.40.2	Configuration for DC	186

6.40.3	Co-existence studies	187

6.40.4	∆TIB and ∆RIB values	187

6.40.5	MSD	187

6.41	DC_21_n1-n77	188

6.41.1	Operating bands for DC	188

6.41.2	Configuration for DC	188

6.41.3	Co-existence studies	188

6.41.4	∆TIB and ∆RIB values	189

6.41.5	MSD	189

6.42	DC_21_n1-n78	190

6.42.1	Operating bands for DC	190

6.42.2	Configuration for DC	190

6.42.3	Co-existence studies	190

6.42.4	∆TIB and ∆RIB values	191

6.42.5	MSD	191

6.43	DC_21_n1-n79	192

6.43.1	Operating bands for DC	192

6.43.2	Configuration for DC	192

6.43.3	Co-existence studies	192

6.43.4	∆TIB and ∆RIB values	192

6.43.5	MSD	193

6.44	DC_42_n1-n77	193

6.44.1	Operating bands for DC	193

6.44.2	Configuration for DC	194

6.44.3	Co-existence studies	194

6.44.4	∆TIB and ∆RIB values	194

6.44.5	MSD	194

6.45	DC_42_n1-n78	195

6.45.1	Operating bands for DC	195

6.45.2	Configuration for DC	195

6.45.3	Co-existence studies	195

6.45.4	∆TIB and ∆RIB values	195

6.45.5	MSD	196

6.46	DC_42_n1-n79	196

6.46.1	Operating bands for DC	196

6.46.2	Configuration for DC	197

6.46.3	Co-existence studies	197

6.46.4	∆TIB and ∆RIB values	197

6.46.5	MSD	198

6.47	DC_28_n1-n78	198

6.47.1	Operating bands for DC	198

6.47.2	Channel bandwidths per operating band for DC	199

6.47.3	Co-existence studies	199

6.47.4	∆TIB and ∆RIB values	199

6.47.5	MSD	200

6.48	DC_2_n5-n77	200

6.48.1	Operating bands for DC	200

6.48.2	Channel bandwidths per operating band for DC	201

6.48.3	Co-existence studies	201

6.48.4	∆TIB and ∆RIB values	201

6.48.5	MSD	202

6.49	DC_2_n66-n77	203

6.49.1	Operating bands for DC	203

6.49.2	Channel bandwidths per operating band for DC	204

6.49.3	Co-existence studies	204

6.49.4	∆TIB and ∆RIB values	204

6.49.5	MSD	205

6.50	DC_13_n2-n77	206

6.50.1	Operating bands for DC	206

6.50.2	Channel bandwidths per operating band for DC	207

6.50.3	Co-existence studies	207

6.50.4	∆TIB and ∆RIB values	207

6.50.5	MSD	208

6.51	DC_13_n5-n48	209

6.51.1	Operating bands for DC	209

6.51.2	Channel bandwidths per operating band for DC	210

6.51.3	Co-existence studies	210

6.51.4	∆TIB and ∆RIB values	210

6.51.5	MSD	211

6.52	DC_13_n48-n66	211

6.52.1	Operating bands for DC	211

6.52.2	Channel bandwidths per operating band for DC	212

6.52.3	Co-existence studies	212

6.52.4	∆TIB and ∆RIB values	212

6.52.5	MSD	213

6.53	DC_13_n66-n77	214

6.53.1	Operating bands for DC	214

6.53.2	Channel bandwidths per operating band for DC	214

6.53.3	Co-existence studies	214

6.53.4	∆TIB and ∆RIB values	215

6.53.5	MSD	215

6.54	DC_66_n2-n77	216

6.54.1	Operating bands for DC	216

6.54.2	Channel bandwidths per operating band for DC	217

6.54.3	Co-existence studies	217

6.54.4	∆TIB and ∆RIB values	217

6.54.5	MSD	218

6.55	DC_66_n5-n48	219

6.55.1	Operating bands for DC	219

6.55.2	Channel bandwidths per operating band for DC	220

6.55.3	Co-existence studies	220

6.55.4	∆TIB and ∆RIB values	220

6.55.5	MSD	221

6.56	DC_66_n5-n77	221

6.56.1	Operating bands for DC	221

6.56.2	Channel bandwidths per operating band for DC	222

6.56.3	Co-existence studies	222

6.56.4	∆TIB and ∆RIB values	222

6.56.5	MSD	223

6.57	DC_66_n66-n77	224

6.57.1	Operating bands for DC	224

6.57.2	Channel bandwidths per operating band for DC	224

6.57.3	Co-existence studies	224

6.57.4	∆TIB and ∆RIB values	225

6.57.5	MSD	225

6.58	DC_40_n41-n258	226

6.58.1	 Operating bands for DC	226

6.58.2	 Configuration for DC	226

6.58.3	 Co-existence studies	226

6.58.4	∆TIB and ∆RIB values	226

6.58.5	MSD	226

6.59	DC_40_n79-n258	227

6.59.1	 Operating bands for DC	227

6.59.2	 Configuration for DC	227

6.59.3	 Co-existence studies	227

6.59.4	∆TIB and ∆RIB values	227

6.59.5	MSD	227

6.60	 DC_41_n79-n258	228

6.60.1	 Operating bands for DC	228

6.60.2	Configuration for DC	228

6.60.3	Co-existence studies	228

6.60.4	∆TIB and ∆RIB values	228

6.60.5	MSD	228

6.61	DC_8_n78-n258	229

6.61.1	Operating bands for DC	229

6.61.2	Channel bandwidths per operating band for DC	229

6.61.3	Co-existence studies	229

6.61.4	∆TIB and ∆RIB values	230

6.61.5	MSD	230

6.62	DC_8_n40-n258	231

6.62.1	Operating bands for DC	231

6.62.2	Channel bandwidths per operating band for DC	231

6.62.3	Co-existence studies	231

6.62.4	∆TIB and ∆RIB values	231

6.62.5	MSD	232

6.63	DC_7_n78-n258	232

6.63.1	Operating bands for DC	232

6.63.2	Channel bandwidths per operating band for DC	233

6.63.3	Co-existence studies	233

6.63.4	∆TIB and ∆RIB values	233

6.63.5	MSD	234

6.64	DC_3_n78-n258	235

6.64.1	Operating bands for DC	235

6.64.2	Channel bandwidths per operating band for DC	235

6.64.3	Co-existence studies	235

6.64.4	∆TIB and ∆RIB values	235

6.64.5	MSD	236

6.65	DC_28_n78-n258	236

6.65.1	Operating bands for DC	236

6.65.2	Channel bandwidths per operating band for DC	237

6.65.3	Co-existence studies	237

6.65.4	∆TIB and ∆RIB values	237

6.65.5	MSD	238

6.66	DC_40_n1-n78	238

6.67	DC_12_n5-n77	243

6.67.5	MSD	243

6.68	DC_14_n5-n77	244

6.68.5	MSD	244

6.69	DC_2_n71-n78	245

6.69.1	Operating bands for DC	245

6.69.2	Channel bandwidths per operating band for DC	245

6.69.3	Co-existence studies	245

6.69.4	∆TIB and ∆RIB values	246

6.69.5	MSD	246

6.70	DC_7_n71-n78	247

6.70.1	Operating bands for DC	247

6.70.2	Channel bandwidths per operating band for DC	247

6.70.3	Co-existence studies	247

6.70.4	∆TIB and ∆RIB values	248

6.70.5	MSD	248

6.71	DC_71_n38-n78	249

6.71.1	Operating bands for DC	249

6.71.2	Channel bandwidths per operating band for DC	250

6.71.3	Co-existence studies	250

6.71.4	∆TIB and ∆RIB values	250

6.71.5	MSD	251

6.72	DC_5_n66-n78	252

6.72.1	Operating bands for DC	252

6.72.2	Channel bandwidths per operating band for DC	252

6.72.3	Co-existence studies	252

6.72.4	∆TIB and ∆RIB values	253

6.72.5	MSD	253

6.73	DC_71_n66-n78	254

6.73.1	Operating bands for DC	254

6.73.2	Channel bandwidths per operating band for DC	254

6.73.3	Co-existence studies	254

6.73.4	∆TIB and ∆RIB values	255

6.73.5	MSD	255

6.74	DC_5_n38-n66	256

6.74.1	Operating bands for DC	256

6.74.2	Channel bandwidths per operating band for DC	256

6.74.3	Co-existence studies	256

6.74.4	∆TIB and ∆RIB values	257

6.74.5	MSD	257

6.75	DC_12_n2-n38	258

6.75.1	Operating bands for DC	258

6.75.2	Channel bandwidths per operating band for DC	258

6.75.3	Co-existence studies	258

6.75.4	∆TIB and ∆RIB values	259

6.75.5	MSD	259

6.76	DC_12_n2-n41	260

6.76.1	Operating bands for DC	260

6.76.2	Channel bandwidths per operating band for DC	260

6.76.3	Co-existence studies	260

6.76.4	∆TIB and ∆RIB values	261

6.76.5	MSD	261

6.77	DC_71_n2-n78	262

6.77.1	Operating bands for DC	262

6.77.2	Channel bandwidths per operating band for DC	262

6.77.3	Co-existence studies	262

6.77.4	∆TIB and ∆RIB values	263

6.77.5	MSD	263

6.78	DC_1_n28-n79	264

6.78.1	Operating bands for DC	264

6.78.2	Configuration for DC	264

6.78.3	Co-existence studies	264

6.78.4	∆TIB and ∆RIB values	264

6.78.5	MSD	265

6.79	DC_3_n28-n79	266

6.79.1	Operating bands for DC	266

6.79.2	Configuration for DC	267

6.79.3	Co-existence studies	267

6.79.4	∆TIB and ∆RIB values	267

6.79.5	MSD	268

6.80	DC_21_n28-n77	268

6.80.1	Operating bands for DC	268

6.80.2	Configuration for DC	269

6.80.3	Co-existence studies	269

6.80.4	∆TIB and ∆RIB values	269

6.80.5	MSD	270

6.81	DC_21_n28-n78	271

6.81.1	Operating bands for DC	271

6.81.2	Configuration for DC	271

6.81.3	Co-existence studies	271

6.81.4	∆TIB and ∆RIB values	272

6.81.5	MSD	272

6.82	DC_66_n71-n78	273

6.82.1	Operating bands for DC	273

6.82.2	Channel bandwidths per operating band for DC	274

6.82.3	Co-existence studies	274

6.82.4	∆TIB and ∆RIB values	274

6.82.5	MSD	275

6.83	DC_2_n38-n71	275

6.83.1	Operating bands for DC	275

6.83.2	Channel bandwidths per operating band for DC	276

6.83.3	Co-existence studies	276

6.83.4	∆TIB and ∆RIB values	276

6.83.5	MSD	277

6.84	DC_66_n38-n71	277

6.84.1	Operating bands for DC	277

6.84.2	Channel bandwidths per operating band for DC	278

6.84.3	Co-existence studies	278

6.84.4	∆TIB and ∆RIB values	278

6.84.5	MSD	279

6.85	DC_71_n38-n66	279

6.85.1	Operating bands for DC	279

6.85.2	Channel bandwidths per operating band for DC	280

6.85.3	Co-existence studies	280

6.85.4	∆TIB and ∆RIB values	280

6.85.5	MSD	281

6.86	DC_2_n2-n38	281

6.86.1	Operating bands for DC	281

6.86.2	Channel bandwidths per operating band for DC	282

6.86.3	Co-existence studies	282

6.86.4	∆TIB and ∆RIB values	282

6.86.5	MSD	283

6.87	DC_66_n2-n38	283

6.87.1	Operating bands for DC	283

6.87.2	Channel bandwidths per operating band for DC	284

6.87.3	Co-existence studies	284

6.87.4	∆TIB and ∆RIB values	284

6.87.5	MSD	285

6.88	DC_2_n2-n41	285

6.88.1	Operating bands for DC	285

6.88.2	Channel bandwidths per operating band for DC	286

6.88.3	Co-existence studies	286

6.88.4	∆TIB and ∆RIB values	286

6.88.5	MSD	287

6.89	DC_71_n2-n41	287

6.89.1	Operating bands for DC	287

6.89.2	Channel bandwidths per operating band for DC	288

6.89.3	Co-existence studies	288

6.89.4	∆TIB and ∆RIB values	288

6.89.5	MSD	289

6.90	DC_7_n2-n78	290

6.90.1	Operating bands for DC	290

6.90.2	Channel bandwidths per operating band for DC	290

6.90.3	Co-existence studies	290

6.90.4	∆TIB and ∆RIB values	291

6.90.5	MSD	291

6.91	DC_2_n66-n71	292

6.91.1	Operating bands for DC	292

6.91.2	Channel bandwidths per operating band for DC	292

6.91.3	Co-existence studies	292

6.91.4	∆TIB and ∆RIB values	293

6.91.5	MSD	293

6.92	DC_7_n66-n71	293

6.92.1	Operating bands for DC	293

6.92.2	Channel bandwidths per operating band for DC	294

6.92.3	Co-existence studies	294

6.92.4	∆TIB and ∆RIB values	294

6.92.5	MSD	295

6.93	DC_66_n66-n71	295

6.93.1	Operating bands for DC	295

6.93.2	Channel bandwidths per operating band for DC	296

6.93.3	Co-existence studies	296

6.93.4	∆TIB and ∆RIB values	296

6.93.5	MSD	297

6.94	DC_2_n2-n71	297

6.94.1	Operating bands for DC	297

6.94.2	Channel bandwidths per operating band for DC	298

6.94.3	Co-existence studies	298

6.94.4	∆TIB and ∆RIB values	298

6.94.5	MSD	299

6.95	DC_7_n2-n71	299

6.95.1	Operating bands for DC	299

6.95.2	Channel bandwidths per operating band for DC	300

6.95.3	Co-existence studies	300

6.95.4	∆TIB and ∆RIB values	300

6.95.5	MSD	301

6.96	DC_66_n2-n71	301

6.96.1	Operating bands for DC	301

6.96.2	Channel bandwidths per operating band for DC	302

6.96.3	Co-existence studies	302

6.96.4	∆TIB and ∆RIB values	302

6.96.5	MSD	303

6.97	DC_2_n2-n66	303

6.97.1	Operating bands for DC	303

6.97.2	Channel bandwidths per operating band for DC	304

6.97.3	Co-existence studies	304

6.97.4	∆TIB and ∆RIB values	304

6.97.5	MSD	305

6.98	DC_7_n2-n66	305

6.98.1	Operating bands for DC	305

6.98.2	Channel bandwidths per operating band for DC	306

6.98.3	Co-existence studies	306

6.98.4	∆TIB and ∆RIB values	306

6.98.5	MSD	307

6.99	DC_71_n2-n66	307

6.99.1	Operating bands for DC	307

6.99.2	Channel bandwidths per operating band for DC	308

6.99.3	Co-existence studies	308

6.99.4	∆TIB and ∆RIB values	308

6.99.5	MSD	309

6.100	DC_21_n28-n79	309

6.100.1	Operating bands for DC	309

6.100.2	Configuration for DC	310

6.100.3	Co-existence studies	310

6.100.4	∆TIB and ∆RIB values	310

6.100.5	MSD	311

6.101	DC_8_n28-n78	311

6.101.1	 Operating bands for DC	311

6.101.2 	Configuration for DC	312

6.101.3	 Co-existence studies	312

6.101.4	 ∆TIB and ∆RIB values	316

6.101.5	MSD	316

6.102	DC_2_n2-n78	317

6.102.1	Operating bands for DC	317

6.102.2	Channel bandwidths per operating band for DC	318

6.102.3	Co-existence studies	318

6.102.4	∆TIB and ∆RIB values	318

6.102.5	MSD	319

6.103	DC_66_n2-n66	320

6.103.1	Operating bands for DC	320

6.103.2	Channel bandwidths per operating band for DC	320

6.103.3	Co-existence studies	320

6.103.4	∆TIB and ∆RIB values	321

6.103.5	MSD	321

6.104	DC_39_n40-n258	322

6.104.1	 Operating bands for DC	322

6.104.2	 Configuration for DC	322

6.104.3	 Co-existence studies	322

6.104.4	∆TIB and ∆RIB values	322

6.104.5	MSD	323

6.105	DC_39_n41-n258	323

6.105.1	 Operating bands for DC	323

6.105.2	 Configuration for DC	323

6.105.3	 Co-existence studies	323

6.105.4	∆TIB and ∆RIB values	324

6.105.5	MSD	324

6.106	DC_39_n79-n258	324

6.106.1	 Operating bands for DC	324

6.106.2	 Configuration for DC	324

6.106.3	 Co-existence studies	324

6.106.4	∆TIB and ∆RIB values	325

6.106.5	MSD	325

6.107	DC_13_n5-n77	325

6.107.1	Operating bands for DC	325

6.107.2	Configuratoin for DC	325

6.107.3	Co-existence studies	326

6.107.4	∆TIB and ∆RIB values	326

6.107.5	MSD	326

6.108 DC_28_n7-n258	328

6.108.1	Operating bands for DC	328

6.108.2	Channel bandwidths per operating band for DC	328

6.108.3	Co-existence studies	329

6.108.4	∆TIB and ∆RIB values	329

6.108.5	MSD	330

6.109	DC_7_n25-n66	330

6.109.1	Operating bands for DC	330

6.109.2	Channel bandwidths per operating band for DC	331

6.109.3	Co-existence studies	331

6.109.4	∆TIB and ∆RIB values	331

6.109.5	MSD	332

6.110	DC_66_n25-n66	332

6.110.1	Operating bands for DC	332

6.110.2	Channel bandwidths per operating band for DC	333

6.110.3	Co-existence studies	333

6.110.4	∆TIB and ∆RIB values	333

6.110.5	MSD	334

6.111	DC_30_n5-n77	334

6.111.5	MSD	334

6.112	DC_13_n25-n66	335

6.112.1	Operating bands for DC	335

6.112.2	Channel bandwidths per operating band for DC	336

6.112.3	Co-existence studies	336

6.112.4	∆TIB and ∆RIB values	336

6.112.5	MSD	337

6.113	 DC_8_n39-n40	337

6.113.1	Operating bands for DC	337

6.113.2	Configuration for DC	338

6.113.3	Co-existence studies	338

6.113.4	∆TIB and ∆RIB values	338

6.113.5	MSD	338

6.114	DC_3_n1-n8, DC_3-3_n1-n8	339

6.114.1	Operating bands for DC	339

6.114.2	Configuration for DC	339

6.114.3	Co-existence studies	339

6.114.4	∆TIB and ∆RIB values	339

6.114.5	MSD	340

6.115	DC_7_n1-n8, DC_7-7_n1-n8	341

6.115.1	Operating bands for DC	341

6.115.2	Configuration for DC	341

6.115.3	Co-existence studies	341

6.115.4	∆TIB and ∆RIB values	341

6.115.5	MSD	342

6.116	 DC_8_n39-n79	342

6.116.1	Operating bands for DC	342

6.116.2	Configuration for DC	342

6.116.3	Co-existence studies	342

6.116.4	∆TIB and ∆RIB values	343

6.116.5	MSD	343

6.117	DC_5_n66-n77	344

6.117.1	Operating bands for DC	344

6.117.2	Configuratoin for DC	344

6.117.3	Co-existence studies	344

6.117.4	∆TIB and ∆RIB values	344

6.117.5	MSD	345

6.118	DC_5_n5-n77	346

6.118.1	Operating bands for DC	346

6.118.2	Configuration for DC	346

6.118.3 Co-existence studies	347

6.118.4	∆TIB and ∆RIB values	347

6.118.5	MSD	347

6.119	DC_5_n2-n77	348

6.119.1	Operating bands for DC	348

6.119.2	Configuration for DC	348

6.119.3	Co-existence studies	349

6.119.4	∆TIB and ∆RIB values	349

6.119.5	MSD	349

6.120	DC_2_n2-n77	350

6.120.1	Operating bands for DC	350

6.120.2	Configuration for DC	350

6.120.3	     Co-existence studies	350

6.120.4	∆TIB and ∆RIB values	351

6.120.5	MSD	351

6.121	DC_3-3_n8-n78	353

6.121.1	Operating bands for DC	353

6.121.2	Configuration for DC	353

6.121.3	Co-existence studies	353

6.121.4	∆TIB and ∆RIB values	353

6.121.5	MSD	354

6.122	DC_7-7_n8-n78	354

6.122.1	Operating bands for DC	354

6.122.2	Configuration for DC	354

6.122.3	Co-existence studies	354

6.122.4	∆TIB and ∆RIB values	355

6.122.5	MSD	355

6.123.2	Configuration for DC	356

6.124	DC_20_n8-n78	357

6.125	DC_3_n1-n38	363

6.125.1	Operating bands for DC	363

6.125.2	Channel bandwidths per operating band for DC	364

6.125.3	Co-existence studies	364

6.125.4	∆TIB and ∆RIB values	364

6.125.5	MSD	364

6.126	DC_8_n1-n40	365

6.126.1	Operating bands for DC	365

6.126.2	Configuration for DC	365

6.126.3	Co-existence studies	365

6.126.4	∆TIB and ∆RIB values	365

6.126.5	MSD	366

6.127	DC_8_n3-n79	366

6.127.1	Operating bands for DC	366

6.127.2	Channel bandwidths per operating band for DC	367

6.127.3	Co-existence studies	367

6.127.4	∆TIB and ∆RIB values	367

6.127.5	MSD	368

6.128	DC_8_n77-n79	369

6.128.1	Operating bands for DC	369

6.128.2	Channel bandwidths per operating band for DC	369

6.128.3	Co-existence studies	370

6.128.4	∆TIB and ∆RIB values	370

6.128.5	MSD	370

6.129	DC_1_n3-n79	371

6.129.1	Operating bands for DC	371

6.129.2	Channel bandwidths per operating band for DC	371

6.129.3	Co-existence studies	372

6.129.4	∆TIB and ∆RIB values	372

6.129.5	MSD	372

6.130	DC_8_n28-n79	373

6.130.1	Operating bands for DC	373

6.130.2	Channel bandwidths per operating band for DC	374

6.130.3	Co-existence studies	374

6.130.4	∆TIB and ∆RIB values	374

6.130.5	MSD	375

6.131.2	Configuration for DC	376

6.132.2	Configuration for DC	378

6.133	DC_3_n1-n41	380

6.133.1	Operating bands for DC	380

6.133.2	Channel bandwidths per operating band for DC	380

6.133.3	Co-existence studies	380

6.133.4	∆TIB and ∆RIB values	380

6.133.5	MSD	381

6.134	DC_8_n1-n77	381

6.134.1	Operating bands for DC	381

6.134.2	Channel bandwidths per operating band for DC	383

6.134.3	Co-existence studies	383

6.134.4	∆TIB and ∆RIB values	384

6.134.5	MSD	384

6.135	DC_42_n1-n3	385

6.135.1	Operating bands for DC	385

6.135.2	Channel bandwidths per operating band for DC	386

6.135.3	Co-existence studies	387

6.135.4	∆TIB and ∆RIB values	387

6.135.5	MSD	387

6.136	DC_7_n12-n77	387

6.136.5	MSD	387

6.137	DC_7_n71-n77	388

6.137.5	MSD	388

6.138	DC_5_n2-n41	388

6.138.5	MSD	389

6.139	DC_3_n28-n75	389

6.139.5	MSD	389

6.140	DC_1_n38-n78	390

6.140.1	Operating bands for DC	390

6.140.2	Configuration for DC	390

6.140.3	Co-existence studies	390

6.140.4	∆TIB and ∆RIB values	390

6.140.5	MSD	391

6.141	DC_3_n38-n78	391

6.141.1	Operating bands for DC	391

6.141.2	Configuration for DC	392

6.141.3	Co-existence studies	392

6.141.4	∆TIB and ∆RIB values	392

6.141.5	MSD	392

6.142	DC_7_n38-n78	392

6.142.1	Operating bands for DC	392

6.142.2	Configuration for DC	393

6.142.3	Co-existence studies	393

6.142.4	∆TIB and ∆RIB values	393

6.142.5	MSD	394

6.143	DC_20_n38-n78	394

6.143.1	Operating bands for DC	394

6.143.2	Configuration for DC	394

6.143.3	Co-existence studies	394

6.143.4	∆TIB and ∆RIB values	394

6.143.5	MSD	395

6.144	DC_1_n3-n8	395

6.145	DC_8_n1-n28	401

6.145.1	 Operating bands for DC	401

6.145.2 	Configuration for DC	402

6.145.3	 Co-existence studies	402

6.145.4	 ∆TIB and ∆RIB values	402

6.145.5	MSD	403

6.146	DC_11_n77-n79	403

6.146.1	Operating bands for DC	403

6.146.2	Channel bandwidths per operating band for DC	404

6.146.3	Co-existence studies	404

6.146.4	∆TIB and ∆RIB values	405

6.146.5	MSD	405

6.147	DC_7_n78-n79	405

7	Dual Connectivity band combinations of LTE 2 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	412

7.1	DC_3-7_n1-n257, DC_3-3-7_n1-n257, DC_3-7-7_n1-n257, DC_3-3-7-7_n1-n257	412

7.1.1	Operating bands for DC	412

7.1.2	Channel bandwidths per operating band for DC	412

7.1.3	Co-existence studies	413

7.1.4	∆TIB and ∆RIB values	414

7.1.5	MSD	414

7.2	DC_1-42_n28-n77	414

7.2.1	Operating bands for DC	414

7.2.2	Channel bandwidths per operating band for DC	414

7.2.3	Co-existence studies	415

7.2.4	∆TIB and ∆RIB values	416

7.2.5	MSD	416

7.3	DC_3-42_n28-n77	416

7.3.1	Operating bands for DC	416

7.3.2	Channel bandwidths per operating band for DC	417

7.3.3	Co-existence studies	418

7.3.4	∆TIB and ∆RIB values	418

7.3.5	MSD	418

7.4	DC_8-42_n28-n77	419

7.4.1	Operating bands for DC	419

7.4.2	Channel bandwidths per operating band for DC	419

7.4.3	Co-existence studies	420

7.4.4	∆TIB and ∆RIB values	420

7.4.5	MSD	421

7.5	DC_1-3_n28-n41	421

7.5.1	Operating bands for DC	421

7.5.2	Channel bandwidths per operating band for DC	422

7.5.3	Co-existence studies	422

7.5.4	∆TIB and ∆RIB values	422

7.5.5	MSD	423

7.6	DC_2-48_n48-n66	423

7.6.1	Operating bands for DC	423

7.6.2	Configuration for DC	424

7.6.3	Co-existence studies	424

7.6.4	∆TIB and ∆RIB values	424

7.6.5	MSD	424

7.7	DC_48-66_n25-n48	425

7.7.1	Operating bands for DC	425

7.7.2	Configuration for DC	425

7.7.3	Co-existence studies	425

7.7.4	∆TIB and ∆RIB values	425

7.7.5	MSD	426

7.8	DC_1-8_n40-n78	426

7.8.1	Operating bands for DC	426

7.8.2	Configuration for DC	427

7.8.3	Co-existence studies	427

7.8.4	∆TIB and ∆RIB values	427

7.8.5	MSD	427

7.9	DC_3-7_n1-n40	428

7.9.1	Operating bands for DC	428

7.9.2	Configuration for DC	428

7.9.3	Co-existence studies	428

7.9.4	∆TIB and ∆RIB values	428

7.9.5	MSD	429

7.10	DC_3-8_n40-n78	429

7.10.1	Operating bands for DC	429

7.10.2	Configuration for DC	430

7.10.3	Co-existence studies	430

7.10.4	∆TIB and ∆RIB values	430

7.10.5	MSD	430

7.11	DC_3-28_n1-n40	431

7.11.1	Operating bands for DC	431

7.11.2	Configuration for DC	431

7.11.3	Co-existence studies	431

7.11.4	∆TIB and ∆RIB values	431

7.11.5	MSD	432

7.12	DC_7-8_n40-n78	432

7.12.1	Operating bands for DC	432

7.12.2	Configuration for DC	433

7.12.3	Co-existence studies	433

7.12.4	∆TIB and ∆RIB values	433

7.12.5	MSD	433

7.13	DC_7-28_n1-n40	434

7.13.1	Operating bands for DC	434

7.13.2	Configuration for DC	434

7.13.3	Co-existence studies	434

7.13.4	∆TIB and ∆RIB values	434

7.13.5	MSD	435

7.14	DC_1-3_n41-n77	435

7.14.1	Operating bands for DC	435

7.14.2	Channel bandwidths per operating band for DC	436

7.14.3	Co-existence studies	436

7.14.4	∆TIB and ∆RIB values	436

7.14.5	MSD	437

7.15	DC_1-3_n3-n41	437

7.15.1	Operating bands for DC	437

7.15.2	Channel bandwidths per operating band for DC	437

7.15.3	Co-existence studies	437

7.15.4	∆TIB and ∆RIB values	437

7.15.5	MSD	438

7.16	DC_1-3_n3-n77	438

7.16.1	Operating bands for DC	438

7.16.2	Channel bandwidths per operating band for DC	439

7.16.3	Co-existence studies	439

7.16.4	∆TIB and ∆RIB values	439

7.16.5	MSD	440

7.17	DC_1-3_n3-n78	440

7.17.1	Operating bands for DC	440

7.17.2	Channel bandwidths per operating band for DC	440

7.17.3	Co-existence studies	440

7.17.4	∆TIB and ∆RIB values	440

7.17.5	MSD	441

7.18	DC_1-18_n3-n77	441

7.18.1	Operating bands for DC	441

7.18.2	Channel bandwidths per operating band for DC	442

7.18.3	Co-existence studies	442

7.18.4	∆TIB and ∆RIB values	442

7.18.5	MSD	442

7.19	DC_1-41_n3-n41	442

7.19.1	Operating bands for DC	442

7.19.2	Channel bandwidths per operating band for DC	443

7.19.3	Co-existence studies	443

7.19.4	∆TIB and ∆RIB values	443

7.19.5	MSD	444

7.20	DC_1-41_n3-n77	444

7.20.1	Operating bands for DC	444

7.20.2	Channel bandwidths per operating band for DC	445

7.20.3	Co-existence studies	445

7.20.4	∆TIB and ∆RIB values	445

7.20.5	MSD	445

7.21	DC_1-41_n3-n78	445

7.21.1	Operating bands for DC	445

7.21.2	Channel bandwidths per operating band for DC	446

7.21.3	Co-existence studies	446

7.21.4	∆TIB and ∆RIB values	446

7.21.5	MSD	446

7.22	DC_1-41_n41-n77	446

7.22.1	Operating bands for DC	446

7.22.2	Channel bandwidths per operating band for DC	447

7.22.3	Co-existence studies	447

7.22.4	∆TIB and ∆RIB values	447

7.22.5	MSD	448

7.23	DC_1-41_n41-n78	448

7.23.1	Operating bands for DC	448

7.23.2	Channel bandwidths per operating band for DC	448

7.23.3	Co-existence studies	448

7.23.4	∆TIB and ∆RIB values	448

7.23.5	MSD	449

7.24	DC_3-18_n3-n77	449

7.24.1	Operating bands for DC	449

7.24.2	Channel bandwidths per operating band for DC	450

7.24.3	Co-existence studies	450

7.24.4	∆TIB and ∆RIB values	450

7.24.5	MSD	451

7.25	DC_3-18_n3-n78	451

7.25.1	Operating bands for DC	451

7.25.2	Channel bandwidths per operating band for DC	451

7.25.3	Co-existence studies	451

7.25.4	∆TIB and ∆RIB values	451

7.25.5	MSD	452

7.26	DC_3-41_n3-n41	452

7.26.1	Operating bands for DC	452

7.26.2	Channel bandwidths per operating band for DC	453

7.26.3	Co-existence studies	453

7.26.4	∆TIB and ∆RIB values	453

7.26.5	MSD	454

7.27	DC_3-41_n3-n77	454

7.27.1	Operating bands for DC	454

7.27.2	Channel bandwidths per operating band for DC	455

7.27.3	Co-existence studies	455

7.27.4	∆TIB and ∆RIB values	455

7.27.5	MSD	456

7.28	DC_3-41_n3-n78	456

7.28.1	Operating bands for DC	456

7.28.2	Channel bandwidths per operating band for DC	457

7.28.3	Co-existence studies	457

7.28.4	∆TIB and ∆RIB values	457

7.28.5	MSD	458

7.29	DC_3-41_n41-n77	458

7.29.1	Operating bands for DC	458

7.29.2	Channel bandwidths per operating band for DC	459

7.29.3	Co-existence studies	459

7.29.4	∆TIB and ∆RIB values	459

7.29.5	MSD	460

7.30	DC_3-41_n41-n78	460

7.30.1	Operating bands for DC	460

7.30.2	Channel bandwidths per operating band for DC	460

7.30.3	Co-existence studies	460

7.30.4	∆TIB and ∆RIB values	460

7.30.5	MSD	461

7.31	DC_2-7_n38-n66	461

7.31.1	Operating bands for DC	461

7.31.2	Channel bandwidths per operating band for DC	462

7.31.3	Co-existence studies	462

7.31.4	∆TIB and ∆RIB values	462

7.31.5	MSD	463

7.32	DC_7-66_n38-n78 and DC_7-7-66_n38-n78	463

7.32.1	Operating bands for DC	463

7.32.2	Channel bandwidths per operating band for DC	464

7.32.3	Co-existence studies	464

7.32.4	∆TIB and ∆RIB values	464

7.32.5	MSD	465

7.33	DC_1-8_n3-n77	465

7.33.1	Operating bands for DC	465

7.33.2	Channel bandwidths per operating band for DC	466

7.33.3	Co-existence studies	467

7.33.4	∆TIB and ∆RIB values	467

7.33.5	MSD	467

7.34	DC_1-11_n3-n28	468

7.34.1	Operating bands for DC	468

7.34.2	Channel bandwidths per operating band for DC	469

7.34.3	Co-existence studies	469

7.34.4	∆TIB and ∆RIB values	469

7.34.5	MSD	470

7.35	DC_8-11_n3-n28	470

7.35.1	Operating bands for DC	470

7.35.2	Channel bandwidths per operating band for DC	471

7.35.3	Co-existence studies	471

7.35.4	∆TIB and ∆RIB values	471

7.35.5	MSD	472

7.36	DC_1-18_n28-n41	472

7.36.1	Operating bands for DC	472

7.36.2	Channel bandwidths per operating band for DC	473

7.36.3	Co-existence studies	473

7.36.4	∆TIB and ∆RIB values	473

7.36.5	MSD	474

7.37	DC_1-18_n28-n77	474

7.37.1	Operating bands for DC	474

7.37.2	Channel bandwidths per operating band for DC	475

7.37.3	Co-existence studies	475

7.37.4	∆TIB and ∆RIB values	475

7.37.5	MSD	476

7.38	DC_1-18_n28-n78	476

7.38.1	Operating bands for DC	476

7.38.2	Channel bandwidths per operating band for DC	477

7.38.3	Co-existence studies	477

7.38.4	∆TIB and ∆RIB values	477

7.38.5	MSD	478

7.39	DC_1-18_n3-n41	478

7.39.1	Operating bands for DC	478

7.39.2	Channel bandwidths per operating band for DC	479

7.39.3	Co-existence studies	479

7.39.4	∆TIB and ∆RIB values	479

7.39.5	MSD	480

7.40	DC_1-18_n41-n77	480

7.40.1	Operating bands for DC	480

7.40.2	Channel bandwidths per operating band for DC	481

7.40.3	Co-existence studies	481

7.40.4	∆TIB and ∆RIB values	481

7.40.5	MSD	482

7.41	DC_1-18_n41-n78	482

7.41.1	Operating bands for DC	482

7.41.2	Channel bandwidths per operating band for DC	483

7.41.3	Co-existence studies	483

7.41.4	∆TIB and ∆RIB values	483

7.41.5	MSD	484

7.42	DC_1-41_n28-n41	484

7.42.1	Operating bands for DC	484

7.42.2	Channel bandwidths per operating band for DC	485

7.42.3	Co-existence studies	485

7.42.4	∆TIB and ∆RIB values	485

7.42.5	MSD	486

7.43	DC_3-18_n28-n41	486

7.43.1	Operating bands for DC	486

7.43.2	Channel bandwidths per operating band for DC	487

7.43.3	Co-existence studies	487

7.43.4	∆TIB and ∆RIB values	488

7.43.5	MSD	488

7.44	DC_3-18_n28-n77	489

7.44.1	Operating bands for DC	489

7.44.2	Channel bandwidths per operating band for DC	490

7.44.3	Co-existence studies	490

7.44.4	∆TIB and ∆RIB values	490

7.44.5	MSD	491

7.45	DC_3-18_n28-n78	491

7.45.1	Operating bands for DC	491

7.45.2	Channel bandwidths per operating band for DC	492

7.45.3	Co-existence studies	492

7.45.4	∆TIB and ∆RIB values	492

7.45.5	MSD	493

7.46	DC_3-18_n3-n41	493

7.46.1	Operating bands for DC	493

7.46.2	Channel bandwidths per operating band for DC	494

7.46.3	Co-existence studies	494

7.46.4	∆TIB and ∆RIB values	494

7.46.5	MSD	495

7.47	DC_3-18_n41-n77	495

7.47.1	Operating bands for DC	495

7.47.2	Channel bandwidths per operating band for DC	496

7.47.3	Co-existence studies	496

7.47.4	∆TIB and ∆RIB values	496

7.47.5	MSD	497

7.48	DC_3-18_n41-n78	497

7.48.1	Operating bands for DC	497

7.48.2	Channel bandwidths per operating band for DC	498

7.48.3	Co-existence studies	498

7.48.4	∆TIB and ∆RIB values	498

7.48.5	MSD	499

7.49	DC_3-41_n28-n41	499

7.49.1	Operating bands for DC	499

7.49.2	Channel bandwidths per operating band for DC	500

7.49.3	Co-existence studies	500

7.49.4	∆TIB and ∆RIB values	500

7.49.5	MSD	501

7.50	DC_3-19_n1-n77	501

7.50.1	Operating bands for DC	501

7.50.2	Configuration for DC	502

7.50.3	Co-existence studies	502

7.50.4	∆TIB and ∆RIB values	502

7.50.5	MSD	503

7.51	DC_3-19_n1-n78	503

7.51.1	Operating bands for DC	503

7.51.2	Configuration for DC	503

7.51.3	Co-existence studies	503

7.51.4	∆TIB and ∆RIB values	504

7.51.5	MSD	504

7.52	DC_3-19_n1-n79	504

7.52.1	Operating bands for DC	504

7.52.2	Configuration for DC	505

7.52.3	Co-existence studies	505

7.52.4	∆TIB and ∆RIB values	505

7.52.5	MSD	506

7.53	DC_3-21_n1-n77	506

7.53.1	Operating bands for DC	506

7.53.2	Configuration for DC	506

7.53.3	Co-existence studies	506

7.53.4	∆TIB and ∆RIB values	507

7.53.5	MSD	507

7.54	DC_3-21_n1-n78	507

7.54.1	Operating bands for DC	507

7.54.2	Configuration for DC	508

7.54.3	Co-existence studies	508

7.54.4	∆TIB and ∆RIB values	508

7.54.5	MSD	509

7.55	DC_3-21_n1-n79	509

7.55.1	Operating bands for DC	509

7.55.2	Configuration for DC	509

7.55.3	Co-existence studies	509

7.55.4	∆TIB and ∆RIB values	510

7.55.5	MSD	510

7.56	DC_3-42_n1-n77	510

7.56.1	Operating bands for DC	510

7.56.2	Configuration for DC	511

7.56.3	Co-existence studies	511

7.56.4	∆TIB and ∆RIB values	511

7.56.5	MSD	512

7.57	DC_3-42_n1-n78	512

7.57.1	Operating bands for DC	512

7.57.2	Configuration for DC	512

7.57.3	Co-existence studies	512

7.57.4	∆TIB and ∆RIB values	513

7.57.5	MSD	513

7.58	DC_3-42_n1-n79	513

7.58.1	Operating bands for DC	513

7.58.2	Configuration for DC	514

7.58.3	Co-existence studies	514

7.58.4	∆TIB and ∆RIB values	514

7.58.5	MSD	515

7.59	DC_19-21_n1-n77	515

7.59.1	Operating bands for DC	515

7.59.2	Configuration for DC	515

7.59.3	Co-existence studies	515

7.59.4	∆TIB and ∆RIB values	516

7.59.5	MSD	516

7.60	DC_19-21_n1-n78	516

7.60.1	Operating bands for DC	516

7.60.2	Configuration for DC	517

7.60.3	Co-existence studies	517

7.60.4	∆TIB and ∆RIB values	517

7.60.5	MSD	518

7.61	DC_19-21_n1-n79	518

7.61.1	Operating bands for DC	518

7.61.2	Configuration for DC	518

7.61.3	Co-existence studies	518

7.61.4	∆TIB and ∆RIB values	519

7.61.5	MSD	519

7.62	DC_19-42_n1-n77	519

7.62.1	Operating bands for DC	519

7.62.2	Configuration for DC	520

7.62.3	Co-existence studies	520

7.62.4	∆TIB and ∆RIB values	520

7.62.5	MSD	521

7.63	DC_19-42_n1-n78	521

7.63.1	Operating bands for DC	521

7.63.2	Configuration for DC	521

7.63.3	Co-existence studies	521

7.63.4	∆TIB and ∆RIB values	522

7.63.5	MSD	522

7.64	DC_19-42_n1-n79	522

7.64.1	Operating bands for DC	522

7.64.2	Configuration for DC	523

7.64.3	Co-existence studies	523

7.64.4	∆TIB and ∆RIB values	523

7.64.5	MSD	524

7.65	DC_21-42_n1-n77	524

7.65.1	Operating bands for DC	524

7.65.2	Configuration for DC	524

7.65.3	Co-existence studies	524

7.65.4	∆TIB and ∆RIB values	525

7.65.5	MSD	525

7.66	DC_21-42_n1-n78	525

7.66.1	Operating bands for DC	525

7.66.2	Configuration for DC	526

7.66.3	Co-existence studies	526

7.66.4	∆TIB and ∆RIB values	526

7.66.5	MSD	527

7.67	DC_21-42_n1-n79	527

7.67.1	Operating bands for DC	527

7.67.2	Configuration for DC	527

7.67.3	Co-existence studies	527

7.67.4	∆TIB and ∆RIB values	528

7.67.5	MSD	528

7.68	DC_3-20_n1-n78	528

7.68.1	Operating bands for DC	528

7.68.2	Channel bandwidths per operating band for DC	529

7.68.3	Co-existence studies	529

7.68.4	∆TIB and ∆RIB values	529

7.68.5	MSD	530

7.69	DC_7-20_n1-n78	530

7.69.1	Operating bands for DC	530

7.69.2	Channel bandwidths per operating band for DC	531

7.69.3	Co-existence studies	531

7.69.4	∆TIB and ∆RIB values	531

7.69.5	MSD	532

7.70	DC_3-7_n40-n78	532

7.70.1	Operating bands for DC	532

7.70.2	Channel bandwidths per operating band for DC	532

7.70.3	Co-existence studies	533

7.70.4	∆TIB and ∆RIB values	533

7.70.5	MSD	533

7.71	DC_1-7_n40-n78	534

7.71.1	Operating bands for DC	534

7.71.2	Channel bandwidths per operating band for DC	534

7.71.3	Co-existence studies	535

7.71.4	∆TIB and ∆RIB values	535

7.71.5	MSD	535

7.72	DC_7-28_n40-n78	536

7.72.1	Operating bands for DC	536

7.72.2	Channel bandwidths per operating band for DC	536

7.72.3	Co-existence studies	537

7.72.4	∆TIB and ∆RIB values	537

7.72.5	MSD	537

7.73	DC_2-13_n66-n77	538

7.73.1	Operating bands for DC	538

7.73.2	Channel bandwidths per operating band for DC	539

7.73.3	Co-existence studies	539

7.73.4	∆TIB and ∆RIB values	539

7.73.5	MSD	540

7.74	DC_2-66_n66-n77	540

7.74.1	Operating bands for DC	540

7.74.2	Channel bandwidths per operating band for DC	541

7.74.3	Co-existence studies	541

7.74.4	∆TIB and ∆RIB values	541

7.74.5	MSD	542

7.75	DC_2-66_n5-n77	542

7.75.1	Operating bands for DC	542

7.75.2	Channel bandwidths per operating band for DC	543

7.75.3	Co-existence studies	543

7.75.4	∆TIB and ∆RIB values	543

7.75.5	MSD	544

7.76	DC_13-66_n66-n77	544

7.76.1	Operating bands for DC	544

7.76.2	Channel bandwidths per operating band for DC	545

7.76.3	Co-existence studies	545

7.76.4	∆TIB and ∆RIB values	545

7.76.5	MSD	546

7.77	DC_13-66_n2-n77	546

7.77.1	Operating bands for DC	546

7.77.2	Channel bandwidths per operating band for DC	547

7.77.3	Co-existence studies	547

7.77.4	∆TIB and ∆RIB values	547

7.77.5	MSD	548

7.78	DC_13-66_n5-n48	548

7.78.1	Operating bands for DC	548

7.78.2	Channel bandwidths per operating band for DC	549

7.78.3	Co-existence studies	549

7.78.4	∆TIB and ∆RIB values	549

7.78.5	MSD	550

7.79	DC_1-8_n78-n258	550

7.79.1	Operating bands for DC	550

7.79.2	Channel bandwidths per operating band for DC	551

7.79.3	Co-existence studies	551

7.79.4	∆TIB and ∆RIB values	551

7.79.5	MSD	551

7.80	DC_3-8_n78-n258	552

7.80.1	Operating bands for DC	552

7.80.2	Channel bandwidths per operating band for DC	552

7.80.3	Co-existence studies	552

7.80.4	∆TIB and ∆RIB values	553

7.80.5	MSD	553

7.81	DC_7-8_n78-n258	553

7.81.1	Operating bands for DC	553

7.81.2	Channel bandwidths per operating band for DC	554

7.81.3	Co-existence studies	554

7.81.4	∆TIB and ∆RIB values	554

7.81.5	MSD	554

7.82	DC_1-8_n40-n258	555

7.82.1	Operating bands for DC	555

7.82.2	Channel bandwidths per operating band for DC	556

7.82.3	Co-existence studies	556

7.82.4	∆TIB and ∆RIB values	556

7.82.5	MSD	556

7.83	DC_3-8_n40-n258	557

7.83.1	Operating bands for DC	557

7.83.2	Channel bandwidths per operating band for DC	557

7.83.3	Co-existence studies	558

7.83.4	∆TIB and ∆RIB values	558

7.83.5	MSD	558

7.84	DC_7-8_n40-n258	558

7.84.1	Operating bands for DC	558

7.84.2	Channel bandwidths per operating band for DC	559

7.84.3	Co-existence studies	559

7.84.4	∆TIB and ∆RIB values	559

7.84.5	MSD	559

7.85	DC_3-11_n28-n77	560

7.85.1	Operating bands for DC	560

7.85.2	Channel bandwidths per operating band for DC	561

7.85.3	Co-existence studies	561

7.85.4	∆TIB and ∆RIB values	562

7.85.5	MSD	562

7.86	DC_8-11_n28-n77	563

7.86.1	Operating bands for DC	563

7.86.2	Channel bandwidths per operating band for DC	564

7.86.3	Co-existence studies	565

7.86.4	∆TIB and ∆RIB values	565

7.86.5	MSD	565

7.87	DC_1-11_n3-n77	566

7.87.1	Operating bands for DC	566

7.87.2	Channel bandwidths per operating band for DC	567

7.87.3	Co-existence studies	568

7.87.4	∆TIB and ∆RIB values	568

7.87.5	MSD	568

7.88	DC_8-11_n3-n77	569

7.88.1	Operating bands for DC	569

7.88.2	Channel bandwidths per operating band for DC	570

7.88.3	Co-existence studies	571

7.88.4	∆TIB and ∆RIB values	571

7.88.5	MSD	571

7.89	DC_1-11_n28-n77	572

7.89.1	Operating bands for DC	572

7.89.2	Channel bandwidths per operating band for DC	573

7.89.3	Co-existence studies	574

7.89.4	∆TIB and ∆RIB values	574

7.89.5	MSD	574

7.90	DC_1-42_n3-n28	575

7.90.1	Operating bands for DC	575

7.90.2	Channel bandwidths per operating band for DC	576

7.90.3	Co-existence studies	577

7.90.4	∆TIB and ∆RIB values	577

7.90.5	MSD	577

7.91	DC_1-42_n3-n77	578

7.91.1	Operating bands for DC	578

7.91.2	Channel bandwidths per operating band for DC	579

7.91.3	Co-existence studies	581

7.91.4	∆TIB and ∆RIB values	581

7.91.5	MSD	582

7.92	DC_8-42_n3-n28	582

7.92.1	Operating bands for DC	582

7.92.2	Channel bandwidths per operating band for DC	583

7.92.3	Co-existence studies	584

7.92.4	∆TIB and ∆RIB values	584

7.92.5	MSD	584

7.93	DC_8-42_n3-n77	585

7.93.1	Operating bands for DC	585

7.93.2	Channel bandwidths per operating band for DC	586

7.93.3	Co-existence studies	588

7.93.4	∆TIB and ∆RIB values	588

7.93.5	MSD	589

7.94	DC_1-21_n28-n77	589

7.94.1	Operating bands for DC	589

7.94.2	Configuration for DC	589

7.94.3	Co-existence studies	590

7.94.4	∆TIB and ∆RIB values	590

7.94.5	MSD	590

7.95	DC_1-21_n28-n78	590

7.95.1	Operating bands for DC	590

7.95.2	Configuration for DC	591

7.95.3	Co-existence studies	591

7.95.4	∆TIB and ∆RIB values	591

7.95.5	MSD	592

7.96	DC_1-21_n28-n79	592

7.96.1	Operating bands for DC	592

7.96.2	Configuration for DC	592

7.96.3	Co-existence studies	593

7.96.4	∆TIB and ∆RIB values	593

7.96.5	MSD	593

7.97	DC_1-3_n28-n79	593

7.97.1	Operating bands for DC	593

7.97.2	Configuration for DC	594

7.97.3	Co-existence studies	594

7.97.4	∆TIB and ∆RIB values	594

7.97.5	MSD	595

7.98	DC_3-21_n28-n77	595

7.98.1	Operating bands for DC	595

7.98.2	Configuration for DC	595

7.98.3	Co-existence studies	596

7.98.4	∆TIB and ∆RIB values	596

7.98.5	MSD	596

7.99	DC_3-21_n28-n78	596

7.99.1	Operating bands for DC	596

7.99.2	Configuration for DC	597

7.99.3	Co-existence studies	597

7.99.4	∆TIB and ∆RIB values	597

7.99.5	MSD	598

7.100	DC_3-21_n28-n79	598

7.100.1	Operating bands for DC	598

7.100.2	Configuration for DC	598

7.100.3	Co-existence studies	599

7.100.4	∆TIB and ∆RIB values	599

7.100.5	MSD	599

7.101	DC_1-8_n28-n78	600

7.101.1	 Operating bands for DC	600

7.101.2 	Configuration for DC	600

7.101.3	 Co-existence studies	600

7.101.4	 ∆TIB and ∆RIB values	601

7.101.5	MSD	601

7.102	DC_3-8_n28-n78	601

7.102.1	 Operating bands for DC	601

7.102.2 	Configuration for DC	602

7.102.3	 Co-existence studies	602

7.102.4	 ∆TIB and ∆RIB values	602

7.102.5	MSD	603

7.103	DC_7-8_n28-n78	603

7.103.1	 Operating bands for DC	603

7.103.2 	Configuration for DC	603

7.103.3	 Co-existence studies	604

7.103.4	 ∆TIB and ∆RIB values	604

7.103.5	MSD	604

7.104	DC_3-28_n1-n78	605

7.104.1	Operating bands for DC	605

7.104.2	Channel bandwidths per operating band for DC	605

7.104.3	Co-existence studies	606

7.104.4	∆TIB and ∆RIB values	606

7.104.5	MSD	606

7.105	DC_7-28_n1-n78	607

7.105.1	Operating bands for DC	607

7.105.2	Channel bandwidths per operating band for DC	607

7.105.3	Co-existence studies	608

7.105.4	∆TIB and ∆RIB values	608

7.105.5	MSD	608

7.106	DC_3-40_n1-n78	609

7.106.1	Operating bands for DC	609

7.106.2	Channel bandwidths per operating band for DC	609

7.106.3	Co-existence studies	610

7.106.4	∆TIB and ∆RIB values	610

7.106.5	MSD	610

7.107	DC_7-40_n1-n78	611

7.107.1	Operating bands for DC	611

7.107.2	Channel bandwidths per operating band for DC	611

7.107.3	Co-existence studies	612

7.107.4	∆TIB and ∆RIB values	612

7.107.5	MSD	612

7.108	DC_8-40_n1-n78	613

7.108.1	Operating bands for DC	613

7.108.2	Channel bandwidths per operating band for DC	613

7.108.3	Co-existence studies	614

7.108.4	∆TIB and ∆RIB values	614

7.108.5	MSD	614

7.109	DC_2-7_n25-n66	615

7.109.1	Operating bands for DC	615

7.109.2	Channel bandwidths per operating band for DC	616

7.109.3	Co-existence studies	617

7.109.4	∆TIB and ∆RIB values	617

7.109.5	MSD	617

7.110	DC_2-13_n25-n66	618

7.110.1	Operating bands for DC	618

7.110.2	Channel bandwidths per operating band for DC	618

7.110.3	Co-existence studies	619

7.110.4	∆TIB and ∆RIB values	619

7.110.5	MSD	619

7.111	DC_2-66_n25-n66	620

7.111.1	Operating bands for DC	620

7.111.2	Channel bandwidths per operating band for DC	620

7.111.3	Co-existence studies	621

7.111.4	∆TIB and ∆RIB values	621

7.111.5	MSD	621

7.112	DC_7-13_n25-n66	622

7.112.1	Operating bands for DC	622

7.112.2	Channel bandwidths per operating band for DC	623

7.112.3	Co-existence studies	623

7.112.4	∆TIB and ∆RIB values	624

7.112.5	MSD	624

7.113	DC_7-66_n25-n66	624

7.113.1	Operating bands for DC	624

7.113.2	Channel bandwidths per operating band for DC	626

7.113.3	Co-existence studies	626

7.113.4	∆TIB and ∆RIB values	627

7.113.5	MSD	627

7.114	DC_3-7_n3-n78	627

7.114.1	Operating bands for DC	627

7.114.2	Channel bandwidths per operating band for DC	628

7.114.3	Co-existence studies	628

7.114.4	∆TIB and ∆RIB values	628

7.114.5	MSD	629

7.115	DC_3-28_n3-n78	629

7.115.1	Operating bands for DC	629

7.115.2	Channel bandwidths per operating band for DC	630

7.115.3	Co-existence studies	630

7.115.4	∆TIB and ∆RIB values	630

7.115.5	MSD	631

7.116	DC_3-7_n1-n8, DC_3-3-7_n1-n8, DC_3-7-7_n1-n8, DC_3-3-7-7_n1-n8	632

7.116.1	Operating bands for DC	632

7.116.2	Configuration for DC	632

7.116.3	Co-existence studies	632

7.116.4	∆TIB and ∆RIB values	632

7.116.5	MSD	633

7.117	DC_3-7_n8-n78, DC_3-3-7_n8-n78, DC_3-7-7_n8-n78, DC_3-3-7-7_n8-n78	633

7.117.1	Operating bands for DC	633

7.117.2	Configuration for DC	633

7.117.3	Co-existence studies	634

7.117.4	∆TIB and ∆RIB values	634

7.117.5	MSD	634

7.118	DC_1-7_n8-n78	634

7.119	DC_1-20_n8-n78	636

7.120	DC_3-20_n8-n78	637

7.121	DC_7-20_n8-n78	639

7.122	DC_7-8_n1-n40	641

7.122.1	Operating bands for DC	641

7.122.2	Channel bandwidths per operating band for DC	642

7.122.3	Co-existence studies	642

7.122.4	∆TIB and ∆RIB values	642

7.122.5	MSD	643

7.123	DC_1-3_n8-n78	643

7.124	DC_3-8_n1-n40	645

7.124.1	Operating bands for DC	645

7.124.2	Channel bandwidths per operating band for DC	646

7.124.3	Co-existence studies	646

7.124.4	∆TIB and ∆RIB values	646

7.124.5	MSD	647

7.125	DC_1-8_n3-n79	647

7.125.1	Operating bands for DC	647

7.125.2	Channel bandwidths per operating band for DC	648

7.125.3	Co-existence studies	648

7.125.4	∆TIB and ∆RIB values	648

7.125.5	MSD	649

7.126	DC_1-8_n77-n79	649

7.126.1	Operating bands for DC	649

7.126.2	Channel bandwidths per operating band for DC	650

7.126.3	Co-existence studies	650

7.126.4	∆TIB and ∆RIB values	650

7.126.5	MSD	651

7.129	DC_2-5_n2-n41	654

7.129.5	MSD	654

7.130 	DC_2-5_n2-n77	655

7.130.1	Operating bands for DC	655

7.130.2	Configuration for DC	655

7.130.3	Co-existence studies	655

7.130.4	∆TIB and ∆RIB values	655

7.130.5	MSD	656

7.131 	DC_2-5_n5-n77	656

7.131.1	Operating bands for DC	656

7.131.2	Configuration for DC	657

7.131.3	Co-existence studies	657

7.131.4	∆TIB and ∆RIB values	657

7.131.5	MSD	658

7.132 	DC_5-66_n66-n77	658

7.132.1	Operating bands for DC	658

7.132.2	Configuration for DC	658

7.132.3	Co-existence studies	658

7.132.4	∆TIB and ∆RIB values	659

7.132.5	MSD	659

7.133 	DC_2-5_n66-n77	659

7.133.1	Operating bands for DC	659

7.133.2	Configuration for DC	660

7.133.3	Co-existence studies	660

7.133.4	∆TIB and ∆RIB values	660

7.133.5	MSD	661

7.134 	DC_13-66_n5-n77	661

7.134.1	Operating bands for DC	661

7.134.2	Configuration for DC	662

7.134.3	Co-existence studies	662

7.134.4	∆TIB and ∆RIB values	662

7.134.5	MSD	663

7.135 	DC_2-13_n2-n77	663

7.135.1	Operating bands for DC	663

7.135.2	Configuration for DC	663

7.135.3	Co-existence studies	664

7.135.4	∆TIB and ∆RIB values	664

7.135.5	MSD	664

7.136  DC_2-13_n5-n77	665

7.136.1	Operating bands for DC	665

7.136.2	Configuration for DC	665

7.136.3	Co-existence studies	665

7.136.4	∆TIB and ∆RIB values	666

7.136.5	MSD	666

7.137  DC_2-66_n2-n77	666

7.137.1	Operating bands for DC	666

7.137.2	Configuration for DC	667

7.137.3	Co-existence studies	667

7.137.4	∆TIB and ∆RIB values	667

7.137.5	MSD	668

7.138  DC_5-66_n2-n77	668

7.138.1	Operating bands for DC	668

7.138.2	Configuration for DC	669

7.138.3	Co-existence studies	669

7.138.4	∆TIB and ∆RIB values	669

7.138.5	MSD	670

7.139  DC_5-66_n5-n77	670

7.139.1	Operating bands for DC	670

7.139.2	Configuration for DC	671

7.139.3	Co-existence studies	671

7.139.4	∆TIB and ∆RIB values	671

7.139.5	MSD	672

7.140  DC_3-8_n1-n28	672

7.140.1	 Operating bands for DC	672

7.140.2 	Configuration for DC	672

7.140.3	 Co-existence studies	673

7.140.4	 ∆TIB and ∆RIB values	673

7.140.5	MSD	673

8	Dual Connectivity band combinations of LTE 3 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	674

8.1	DC_1-3-8_n28-n77	674

8.1.1	Operating bands for DC	674

8.1.2	Channel bandwidths per operating band for DC	675

8.1.3	Co-existence studies	675

8.1.4	∆TIB and ∆RIB values	675

8.1.5	MSD	676

8.2	DC_3-7-8_n40-n78	676

8.2.1	Operating bands for DC	676

8.2.2	Configuration for DC	677

8.2.3	Co-existence studies	677

8.2.4	∆TIB and ∆RIB values	677

8.2.5	MSD	678

8.3	DC_3-7-28_n1-n40	678

8.3.1	Operating bands for DC	678

8.3.2	Configuration for DC	678

8.3.3	Co-existence studies	679

8.3.4	∆TIB and ∆RIB values	679

8.3.5	MSD	679

8.4	DC_1-3-18_n3-n77	680

8.4.1	Operating bands for DC	680

8.4.2	Channel bandwidths per operating band for DC	680

8.4.3	Co-existence studies	680

8.4.4	∆TIB and ∆RIB values	680

8.4.5	MSD	681

8.5	DC_1-3-18_n3-n78	681

8.5.1	Operating bands for DC	681

8.5.2	Channel bandwidths per operating band for DC	682

8.5.3	Co-existence studies	682

8.5.4	∆TIB and ∆RIB values	682

8.5.5	MSD	683

8.6	DC_1-3-41_n3-n41	683

8.6.1	Operating bands for DC	683

8.6.2	Channel bandwidths per operating band for DC	684

8.6.3	Co-existence studies	684

8.6.4	∆TIB and ∆RIB values	684

8.6.5	MSD	685

8.7	DC_1-3-41_n3-n77	685

8.7.1	Operating bands for DC	685

8.7.2	Channel bandwidths per operating band for DC	686

8.7.3	Co-existence studies	686

8.7.4	∆TIB and ∆RIB values	686

8.7.5	MSD	687

8.8	DC_1-3-41_n3-n78	687

8.8.1	Operating bands for DC	687

8.8.2	Channel bandwidths per operating band for DC	688

8.8.3	Co-existence studies	688

8.8.4	∆TIB and ∆RIB values	688

8.8.5	MSD	689

8.9	DC_1-3-41_n41-n77	689

8.9.1	Operating bands for DC	689

8.9.2	Channel bandwidths per operating band for DC	690

8.9.3	Co-existence studies	690

8.9.4	∆TIB and ∆RIB values	690

8.9.5	MSD	691

8.10	DC_1-3-41_n41-n78	691

8.10.1	Operating bands for DC	691

8.10.2	Channel bandwidths per operating band for DC	691

8.10.3	Co-existence studies	691

8.10.4	∆TIB and ∆RIB values	691

8.10.5	MSD	692

8.11	DC_1-8-42_n28-n77	692

8.11.1	Operating bands for DC	692

8.11.2	Channel bandwidths per operating band for DC	694

8.11.3	Co-existence studies	696

8.11.4	∆TIB and ∆RIB values	696

8.11.5	MSD	697

8.12	DC_1-3-18_n28-n77	697

8.12.1	Operating bands for DC	697

8.12.2	Channel bandwidths per operating band for DC	699

8.12.3	Co-existence studies	700

8.12.4	∆TIB and ∆RIB values	700

8.12.5	MSD	700

8.13	DC_1-3-18_n28-n78	701

8.13.1	Operating bands for DC	701

8.13.2	Channel bandwidths per operating band for DC	702

8.13.3	Co-existence studies	703

8.13.4	∆TIB and ∆RIB values	703

8.13.5	MSD	703

8.14	DC_1-3-41_n28-n41	704

8.14.1	Operating bands for DC	704

8.14.2	Channel bandwidths per operating band for DC	705

8.14.3	Co-existence studies	705

8.14.4	∆TIB and ∆RIB values	705

8.14.5	MSD	706

8.15	DC_3-19-42_n1-n77	706

8.15.1	Operating bands for DC	706

8.15.2	Configuration for DC	707

8.15.3	Co-existence studies	707

8.15.4	∆TIB and ∆RIB values	707

8.15.5	MSD	708

8.16	DC_3-19-42_n1-n78	708

8.16.1	Operating bands for DC	708

8.16.2	Configuration for DC	709

8.16.3	Co-existence studies	709

8.16.4	∆TIB and ∆RIB values	709

8.16.5	MSD	710

8.17	DC_3-19-42_n1-n79	710

8.17.1	Operating bands for DC	710

8.17.2	Configuration for DC	711

8.17.3	Co-existence studies	711

8.17.4	∆TIB and ∆RIB values	711

8.17.5	MSD	712

8.18	DC_3-21-42_n1-n77	712

8.18.1	Operating bands for DC	712

8.18.2	Configuration for DC	713

8.18.3	Co-existence studies	713

8.18.4	∆TIB and ∆RIB values	713

8.18.5	MSD	714

8.19	DC_3-21-42_n1-n78	714

8.19.1	Operating bands for DC	714

8.19.2	Configuration for DC	715

8.19.3	Co-existence studies	715

8.19.4	∆TIB and ∆RIB values	715

8.19.5	MSD	716

8.20	DC_3-21-42_n1-n79	716

8.20.1	Operating bands for DC	716

8.20.2	Configuration for DC	717

8.20.3	Co-existence studies	717

8.20.4	∆TIB and ∆RIB values	717

8.20.5	MSD	718

8.21	DC_19-21-42_n1-n77	718

8.21.1	Operating bands for DC	718

8.21.2	Configuration for DC	719

8.21.3	Co-existence studies	719

8.21.4	∆TIB and ∆RIB values	719

8.21.5	MSD	720

8.22	DC_19-21-42_n1-n78	720

8.22.1	Operating bands for DC	720

8.22.2	Configuration for DC	721

8.22.3	Co-existence studies	721

8.22.4	∆TIB and ∆RIB values	721

8.22.5	MSD	722

8.23	DC_19-21-42_n1-n79	722

8.23.1	Operating bands for DC	722

8.23.2	Configuration for DC	723

8.23.3	Co-existence studies	723

8.23.4	∆TIB and ∆RIB values	723

8.23.5	MSD	724

8.24	DC_3-7-20_n1-n78	724

8.24.1	Operating bands for DC	724

8.24.2	Channel bandwidths per operating band for DC	725

8.24.3	Co-existence studies	725

8.24.4	∆TIB and ∆RIB values	725

8.24.5	MSD	726

8.25	DC_3-7-28_n40-n78	726

8.25.1	Operating bands for DC	726

8.25.2	Channel bandwidths per operating band for DC	727

8.25.3	Co-existence studies	728

8.25.4	∆TIB and ∆RIB values	728

8.25.5	MSD	729

8.26	DC_1-3-7_n40-n78	729

8.26.1	Operating bands for DC	729

8.26.2	Channel bandwidths per operating band for DC	730

8.26.3	Co-existence studies	731

8.26.4	∆TIB and ∆RIB values	731

8.26.5	MSD	732

8.27	DC_1-7-28_n40-n78	732

8.27.1	Operating bands for DC	732

8.27.2	Channel bandwidths per operating band for DC	733

8.27.3	Co-existence studies	734

8.27.4	∆TIB and ∆RIB values	734

8.27.5	MSD	735

8.28	DC_3-7-8_n78-n258	735

8.28.1	Operating bands for DC	735

8.28.2	Channel bandwidths per operating band for DC	736

8.28.3	Co-existence studies	736

8.28.4	∆TIB and ∆RIB values	736

8.28.5	MSD	736

8.29	DC_3-7-8_n40A-n258	737

8.29.1	Operating bands for DC	737

8.29.2	Channel bandwidths per operating band for DC	737

8.29.3	Co-existence studies	737

8.29.4	∆TIB and ∆RIB values	738

8.29.5	MSD	738

8.30	DC_1-8-11_n3-n28	738

8.30.1	Operating bands for DC	738

8.30.2	Channel bandwidths per operating band for DC	739

8.30.3	Co-existence studies	739

8.30.4	∆TIB and ∆RIB values	740

8.30.5	MSD	740

8.31	DC_1-8-11_n3-n77	741

8.31.1	Operating bands for DC	741

8.31.2	Channel bandwidths per operating band for DC	742

8.31.3	Co-existence studies	743

8.31.4	∆TIB and ∆RIB values	743

8.31.5	MSD	743

8.32	DC_1-3-11_n28-n77	744

8.32.1	Operating bands for DC	744

8.32.2	Channel bandwidths per operating band for DC	745

8.32.3	Co-existence studies	746

8.32.4	∆TIB and ∆RIB values	746

8.32.5	MSD	746

8.33	DC_1-8-11_n28-n77	747

8.33.1	Operating bands for DC	747

8.33.2	Channel bandwidths per operating band for DC	748

8.33.3	Co-existence studies	749

8.33.4	∆TIB and ∆RIB values	749

8.33.5	MSD	749

8.34	DC_1-8-42_n3-n28	750

8.34.1	Operating bands for DC	750

8.34.2	Channel bandwidths per operating band for DC	751

8.34.3	Co-existence studies	753

8.34.4	∆TIB and ∆RIB values	753

8.34.5	MSD	753

8.35	DC_1-8-42_n3-n77	754

8.35.1	Operating bands for DC	754

8.35.2	Channel bandwidths per operating band for DC	755

8.35.3	Co-existence studies	757

8.35.4	∆TIB and ∆RIB values	757

8.35.5	MSD	758

8.36	DC_3-8-11_n28-n77	758

8.36.1	Operating bands for DC	758

8.36.2	Channel bandwidths per operating band for DC	760

8.36.3	Co-existence studies	761

8.36.4	∆TIB and ∆RIB values	761

8.36.5	MSD	761

8.37	DC_1-3-42_n28-n77	762

8.37.1	Operating bands for DC	762

8.37.2	Channel bandwidths per operating band for DC	763

8.37.3	Co-existence studies	765

8.37.4	∆TIB and ∆RIB values	765

8.37.5	MSD	766

8.38	 DC_1-7-8_n28-n78	766

8.38.1	 Operating bands for DC	766

8.38.2 	Configuration for DC	767

8.38.3	 Co-existence studies	767

8.38.4	 ∆TIB and ∆RIB values	767

8.38.5	MSD	768

8.39	 DC_3-7-8_n28-n78	768

8.39.1	 Operating bands for DC	768

8.39.2 	Configuration for DC	768

8.39.3	 Co-existence studies	769

8.39.4	 ∆TIB and ∆RIB values	769

8.39.5	MSD	770

8.40	DC_3-7-28_n1-n78	770

8.40.1	Operating bands for DC	770

8.40.2	Channel bandwidths per operating band for DC	770

8.40.3	Co-existence studies	771

8.40.4	∆TIB and ∆RIB values	771

8.40.5	MSD	772

8.41	DC_1-7-28_n3-n78	772

8.41.1	Operating bands for DC	772

8.41.2	Channel bandwidths per operating band for DC	773

8.41.3	Co-existence studies	774

8.41.4	∆TIB and ∆RIB values	774

8.41.5	MSD	775

8.42	DC_3-7-40_n1-n78	775

8.42.1	Operating bands for DC	775

8.42.2	Channel bandwidths per operating band for DC	776

8.42.3	Co-existence studies	776

8.42.4	∆TIB and ∆RIB values	776

8.42.5	MSD	777

8.43	DC_3-8-40_n1-n78	778

8.43.1	Operating bands for DC	778

8.43.2	Channel bandwidths per operating band for DC	778

8.43.3	Co-existence studies	779

8.43.4	∆TIB and ∆RIB values	779

8.43.5	MSD	779

8.44	DC_7-8-40_n1-n78	780

8.44.1	Operating bands for DC	780

8.44.2	Channel bandwidths per operating band for DC	780

8.44.3	Co-existence studies	781

8.44.4	∆TIB and ∆RIB values	781

8.44.5	MSD	782

8.45	DC_1-3-18_n3-n41	782

8.45.1	Operating bands for DC	782

8.45.2	Channel bandwidths per operating band for DC	783

8.45.3	Co-existence studies	783

8.45.4	∆TIB and ∆RIB values	783

8.45.5	MSD	784

8.46	DC_1-3-18_n28-n41	785

8.46.1	Operating bands for DC	785

8.46.2	Channel bandwidths per operating band for DC	786

8.46.3	Co-existence studies	786

8.46.4	∆TIB and ∆RIB values	786

8.46.5	MSD	787

8.47	DC_1-3-18_n41-n77	788

8.47.1	Operating bands for DC	788

8.47.2	Channel bandwidths per operating band for DC	789

8.47.3	Co-existence studies	790

8.47.4	∆TIB and ∆RIB values	790

8.47.5	MSD	790

8.48	DC_1-3-18_n41-n78	791

8.48.1	Operating bands for DC	791

8.48.2	Channel bandwidths per operating band for DC	792

8.48.3	Co-existence studies	793

8.48.4	∆TIB and ∆RIB values	793

8.48.5	MSD	794

8.49	DC_1-3-8_n28-n78	794

8.49.1	 Operating bands for DC	794

8.49.2 	Configuration for DC	794

8.49.3	 Co-existence studies	795

8.49.4	 ∆TIB and ∆RIB values	795

8.49.5	MSD	795

8.50	DC_1-3-20_n7-n78	796

8.50.1	Operating bands for DC	796

8.50.2	Channel bandwidths per operating band for DC	797

8.50.3	Co-existence studies	797

8.50.4	∆TIB and ∆RIB values	797

8.50.5	MSD	798

8.51	DC_2-7-13_n25-n66	798

8.51.1	Operating bands for DC	798

8.51.2	Channel bandwidths per operating band for DC	799

8.51.3	Co-existence studies	800

8.51.4	∆TIB and ∆RIB values	800

8.51.5	MSD	800

8.52	DC_2-7-66_n25-n66	801

8.52.1	Operating bands for DC	801

8.52.2	Channel bandwidths per operating band for DC	802

8.52.3	Co-existence studies	803

8.52.4	∆TIB and ∆RIB values	803

8.52.5	MSD	803

8.53	DC_1-3-7_n3-n78	804

8.53.1	Operating bands for DC	804

8.53.2	Channel bandwidths per operating band for DC	805

8.53.3	Co-existence studies	805

8.53.4	∆TIB and ∆RIB values	805

8.53.5	MSD	806

8.54	DC_1-3-28_n3-n78	806

8.54.1	Operating bands for DC	806

8.54.2	Channel bandwidths per operating band for DC	807

8.54.3	Co-existence studies	807

8.54.4	∆TIB and ∆RIB values	807

8.54.5	MSD	808

8.55	DC_3-7-28_n3-n78	808

8.55.1	Operating bands for DC	808

8.55.2	Channel bandwidths per operating band for DC	809

8.55.3	Co-existence studies	809

8.55.4	∆TIB and ∆RIB values	809

8.55.5	MSD	810

8.56	DC_1-3-7_n8-n78	810

8.57	DC_1-3-20_n8-n78	812

8.58	DC_1-7-20_n8-n78	814

8.59	DC_3-7-20_n8-n78	815

8.60	DC_3-7-8_n1-n40	817

8.60.1	Operating bands for DC	817

8.60.2	Channel bandwidths per operating band for DC	818

8.60.3	Co-existence studies	819

8.60.4	∆TIB and ∆RIB values	819

8.60.5	MSD	819

8.61	DC_2-13-66_n5-n77	820

8.61.1	Operating bands for DC	820

8.61.2	Configuration for DC	820

8.61.3	Co-existence studies	820

8.61.4	∆TIB and ∆RIB values	821

8.61.5	MSD	821

8.62	DC_2-13-66_n66-n77	822

8.62.1	Operating bands for DC	822

8.62.2	Configuration for DC	822

8.62.3	Co-existence studies	822

8.62.4	∆TIB and ∆RIB values	823

8.62.5	MSD	823

8.63	DC_2-13-66_n2-n77	824

8.63.1	Operating bands for DC	824

8.63.2	Configuration for DC	824

8.63.3	Co-existence studies	824

8.63.4	∆TIB and ∆RIB values	825

8.63.5	MSD	825

8.64	DC_2-5-66_n5-n77	825

8.64.1	Operating bands for DC	825

8.64.2	Configuration for DC	826

8.64.3	Co-existence studies	826

8.64.4	∆TIB and ∆RIB values	826

8.64.5	MSD	827

8.65	DC_2-5-66_n2-n77	827

8.65.1	Operating bands for DC	827

8.65.2	Configuration for DC	828

8.65.3	Co-existence studies	828

8.65.4	∆TIB and ∆RIB values	828

8.65.5	MSD	829

9	Dual Connectivity band combinations of LTE 4 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	830

9.1	DC_1-3-7-28_n40-n78	830

9.1.1	Operating bands for DC	830

9.1.2	Channel bandwidths per operating band for DC	830

9.1.3	Co-existence studies	831

9.1.4	∆TIB and ∆RIB values	831

9.1.5	MSD	832

9.2	DC_1-3-8-11_n28-n77	832

9.2.1	Operating bands for DC	832

9.2.2	Channel bandwidths per operating band for DC	833

9.2.3	Co-existence studies	835

9.2.4	∆TIB and ∆RIB values	835

9.2.5	MSD	835

9.3	DC_1-3-7-8_n28-n78	836

9.3.1	 Operating bands for DC	836

9.3.2 	Configuration for DC	836

9.3.3	 Co-existence studies	837

9.3.4	 ∆TIB and ∆RIB values	837

9.3.5	MSD	837

9.4	DC_3-7-8-40_n1-n78	838

9.4.1	Operating bands for DC	838

9.4.2	Channel bandwidths per operating band for DC	838

9.4.3	Co-existence studies	839

9.4.4	∆TIB and ∆RIB values	839

9.4.5	MSD	840

9.5	DC_1-3-7-28_n78-n257	840

9.5.1	Operating bands for DC	840

9.5.2	Channel bandwidths per operating band for DC	841

9.5.3	Co-existence studies	841

9.5.4	∆TIB and ∆RIB values	841

9.5.5	MSD	841

9.6	DC_1-3-7-28_n3-n78	842

9.6.1	Operating bands for DC	842

9.6.2	Channel bandwidths per operating band for DC	843

9.6.3	Co-existence studies	843

9.6.4	∆TIB and ∆RIB values	843

9.6.5	MSD	844

9.7	 DC_1-3-7-20_n8-n78	844

9.8	 DC_1-3-7-20_n38-n78	846

Annex A: Change history	848

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document is a technical report for Dual connectivity (DC) band combinations of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL under Rel-17 time frame. The purpose is to gather the relevant background information and studies in order to address DC band combinations of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL in Rel-17 as shown in the WID for Rel-17.

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]		3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: “Guideline on WI/SI for new Operating Bands”

[3]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[4]	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”

[5]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

4	Background

The present document is a technical report for DC band combinations of LTE x bands (x=1,2,3,4) inter-band CA (x DL/1UL) + inter-band NR 2 bands (2DL/1UL) under Rel-17 time frame. It covers both the UE and BS side. The document is divided in two different parts:

- 	General part: this part covers BS and UE specific which is band combination independent.

- 	Specific band combination part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

The specific band combination parts are independent and therefore, the working speed also differs.

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

5	Dual Connectivity band combinations of LTE x bands DL/1UL(x=1,2,3,4) + NR 2 bands DL/1UL: General Part

5.1	UE RF architectures

RAN4 consider same example RF architecture which are described in section 5.1 in TR37.863-01-01.  One different point is that LTE and NR modemchip can be used one chip solutions from rel-16.

To support multiple component carriers in LTE and NR, RAN4 consider specific RF components such as Diplexer, Triplexer, Quadplexer and Hexaplexer to derive self-interference problems according to the LTE x bands DL/1UL(x=1,2,3,4) + NR 2 band DL/1UL DC band combinations.

5.2	General treatment of ∆TIB and ∆RIB values

For the LTE x bands DL/1UL(x=1,2,3,4) + NR 2 band DL/1UL DC band combinations, RAN4 should consider to reuse agreed additional insertion losses for all DC band combinations in rel-16 when new RF components are not introduced to support this basket WI. If the new RF components are introduced, then more detail decription will be captured in some specific DC band combinations.

5.3	Summary of self-interference analysis

Table 5.3-1 summarizes the EN- DC band combinations with self-interference problems for 3DL/2UL DC operation.

Table 5.3-1: Summary of Self-interference analysis for LTE 1 band & NR 2 bands DL and 2 bands UL DC operation

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_8_n28-n78 | DC_8A-n28 | 4th harmonic into n78 5th harmonic into n78 | 4th & 5th IMDs into n78 | - | - 4th harmonic problem will be solved in DC_8A-n78A - 5th harmonic problem will be solved in DC_28A-n78A - Same as DC_8_n28-n77A with 2UL_DC_8A_n28A |
|  | DC_8A_n78A | - | 4th IMD into n28 | - | - Same as DC_8_n28-n77A with 2UL_DC_8A_n77A |
| DC_(n)41AA-n78 DC_(n)41CA-n78 DC_(n)41DA-n78 | DC_41A_n78A | - | 4th IMD into n41 | - | B41 and n41 will be operated by synchronous operation. So there is no IMD problem |
| DC_40A_n1-n78 DC_40C_n1-n78 | DC_40A_n1A DC_40C_n1A |  | 4th IMD & 5th IMD into n78 | - | - Same as DC_1A_n40A-n78A with 2UL_DC_1A_n40A |
|  | DC_40A_n78A DC_40C_n78A |  | 4th IMD into n1 | - | -7.6dB by IMD4 |
| DC_3C_n1-n78 | DC_3A_n1A | 2nd harmonic from 3 into n78 | 2nd & 4th IMDs into n78 | Yes | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems same as DC_1A_n3A-n78A. - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_3A_n78A | - | 5th IMD into n1 | - | - Same as DC_3A_n1A-n78A with 2UL_DC_3A_n78A |
| DC_1_n28-n41 | DC_1A_n28A | - | 2nd & 4th IMDs into n41 |  | - Same as DC_1A-41A_n28A with 2UL_DC_1A_n28A |
|  | DC_1A_n41A |  | 2nd & 5th IMDs into n28 |  | - 29.3 dB by IMD2 - 4.5dB by IMD5 |
| DC_3_n28-n41 | DC_3A_n28A |  | 2nd & 3rd IMDs into n41 |  | - Same as DC_3A-41A_n28A with 2UL_DC_3A_n28A |
|  | DC_3A_n41A |  | 2nd & 3rd IMDs into n28 |  | - Same as DC_3A-28A_n41A with 2UL_DC_3A_n41A |
| DC_28_n1-n40 | DC_28A_n1A | - | 4th IMD into n40 | - | - Same as DC_1A_n28A-n40A with 2UL_DC_1A_n28A |
|  | DC_28A_n40A | 3rd harmonic from B28 into n1 | - | - | 3rd harmonic problem was covered in lower DC or CA band combinations |
| DC_7_n40-n78 | DC_7A_n40A | - | - | - | No issue |
|  | DC_7A_n78A |  | 4th IMD into n40 |  | - 9.2 dB by 4th IMD |
| DC_8_n40-n78 | DC_8A_n40A | 4th harmonic from B8 into n78 | 2nd & 3rd IMDs into n78 |  | - 28.7dB by IMD2 |
|  | DC_8A_n78A |  | 2nd IMD into n40 |  | - 27.3 dB by IMD2 |
| DC_1_n3-n77 DC_1_n3-n77(2A) DC_n3-n77_1 DC_n3-n77(2A)_1 | DC_1A_n3A or DC_n3A_1A | 2nd harmonic from B1 or n3 into n77 | 2nd & 4th IMDs into n77 | Yes | - Harmonic problems already covered in DC_n3A-n78A. - Same as IMD issues in DC_1A_n3A-n78A - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_1A_n77A or DC_n77A_1A | - | 2nd , 4th & 5th IMDs into n3 | - | - Same as IMD issues in DC_1A_n3A-n78A |
| DC_8_n3-n77A DC_8_n3-n77(2A) | DC_8A_n3A | 2nd harmonic from n3 into n77 4th harmonic from B8 into n77 | 3rd & 5th IMDs into n77 |  | - Harmonic problems already covered in CA_3A_n77A and DC_8A_n77A - Same as DC_3A_n8A-n78A with 2UL_DC_3A_n8A |
|  | DC_8A_n77A | 2nd harmonic from B8 into n3 | 3rd IMD into n3 |  | - Same as DC_3A-8A-n77A with 2UL_DC_8A_n77A |
| DC_11_n28-n77A DC_11_n28-n77(2A) | DC_11A_n28A | 5th harmonic from n28 into n77 | 3rd & 4th IMDs into n77 |  | - Harmonic problems already covered in CA_n28A_n77A - 17.5 dB by IMD3 - 11.2dB by IMD4 |
|  | DC_11A_n77A | - | 3rd & 4th IMDs into n28 |  | - 15.8 dB by IMD3 - 11.0 dB by IMD4 |
| DC_42A_n3-n28 DC_42C_n3-n28 | DC_42A_n3A DC_42C_n3A | - | - |  | - No issue |
|  | DC_42A_n28A DC_42C_n28A | - | - |  | - No issue |
| DC_42A_n3-n77A DC_42A_n3-n77(2A) DC_42C_n3-n77A DC_42C_n3-n77(2A) | DC_42A_n3A DC_42C_n3A | 2nd harmonic from n3 into n77 | 4th & 5th IMDs into n77 |  | - Harmonic problems already covered in CA_n3A_n77A - These IMD problem was not impact to n77 due to synchronous TDD operation |
| DC_2_n5-n77 or DC_2A-2A_n5A-n77A | DC_2A_n5A | 2nd harmonic from B2 into n77 5th harmonic from n5 into n77 | 3rd & 5th IMDs into n77 |  | - Harmonic problems already covered in DC_2A_n77A or CA_n5A-n77A. - 16.0 dB by IMD3 - 4.1 dB by IMD5 |
|  | DC_2A_n77A | - | 5th IMD into n5 |  | - 3.8 dB by IMD5 |
| DC_13_n2-n77 | DC_13A_n2A | 2nd harmonic from n2 into n77 5th harmonic from B13 into n77 | 3rd, 4th & 5th IMDs into n77 |  | - Harmonic problems already covered in CA_n2A-n77A or DC_13A_n77A. - 17.3 dB by IMD3 - 12.4 dB by IMD4 - 4.5 dB by IMD5 |
|  | DC_13A_n77A | - | 3rd IMD into n2 |  | - 16.0 dB by IMD3 |
| DC_66_n2-n77 | DC_66A_n2A | 2nd harmonic from B66 or n2 into n77 | 2nd & 4th IMDs into n77 | - | - Same as DC_2A_n66A-n78A with 2UL_DC_2A_n66A |
|  | DC_66A_n77A | - | 2nd , 4th & 5th IMDs into n2 | - | - Same as DC_2A-66A_n78A with 2UL_DC_66A_n78A |
| DC_66_n5-n77 DC_66-66_n5-n77 | DC_66A_n5A | 2nd harmonic from B66 into n77 4th & 5th harmonics from n5 into n77 | 3rd, 4th & 5th IMDs into n77 | - | - These harmonic issue already solved in DC_66A_n77A or DC_5A_n77A - 16.6 dB by IMD3 - 12.1 dB by IMD4 - 3.6 dB by IMD5 |
|  | DC_66A_n77A | - |  | - | - No issue |
| DC_28_n1-n78 | DC_28A_n1A | 5th harmonic from 28 into n78 | 3rd IMD into n78 | - | - Harmonic problems already solved in CA_28A-n78A - Same as DC_1A_n28A-n78A with 2UL_DC_1A_n28A |
|  | DC_28A_n78A | 3rd harmonic from B28 into n1 | 3rd IMD into n1 | - | - 3rd harmonic will be covered in CA_1A-28A - Same as DC_1A-28A_n78A with 2UL_DC_28A_n78A |
| DC_2_n48-n66 | DC_2A_n48A | - | 4th IMD into n66 | - | - Same as DC_2A-66A_n48A with 2UL_DC_2A_n48A |
|  | DC_2A_n66A | 2nd harmonic from n66 into n48 | 2nd IMD into n48 | - | - Same as DC_2A-48A_n66A with 2UL_DC_2A_n66A |
| DC_48_n25-n48 | DC_48A_n25A | - | 4th IMD into n48 |  | There was no IMD problem due to TDD n48 |
| DC_48_n48-n66 | DC_48A_n66A | 2nd harmonic from n66 into n48 | 4th IMD into n48 |  | There was no IMD problem due to TDD n48 |
| DC_66_n25-n48 | DC_66A_n25A | 2nd harmonic from n66 into n48 | 2nd IMD into n48 | - | - Same as DC_2A-48A_n66A with 2UL_DC_2A_n66A |
|  | DC_66A_n48A | - | 2nd IMD into n25 | - | - Same as DC_2A-48A_n66A with 2UL_DC_48A_n66A |
| DC_3_n40-n41C | DC_3A_n40A | 3rd harmonic from B3 into n41 | - | - | CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  | DC_3A_n41A | - | - | Yes | CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_3_n40-n79C | DC_3A_n40A | 2nd harmonic form n40 into n79 | 5th IMD into n79 | - | - Same as DC_3A_n40A-n79A with 2UL_DC_3A_n40A |
|  | DC_3A_n79A | - | 5th IMD into n40 | - | - Same as DC_3A_n40A-n79A with 2UL_DC_3A_n79A |
| DC_3_n41A-n79C DC_3_n41C-n79A DC_3_n41C-n79C | DC_3A_n41A | 2nd harmonic form n41 into n79 | 2nd & 5th IMDs into n79 | Yes | - The 2nd harmonic problem will be studied in CA_n41-n79. - Same as DC_3A_n41A-n79A with 2UL_DC_3A_n41A - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
|  | DC_3A_n79A | - | 2nd & 5th IMDs into n41 | - | - Same as DC_3A_n41A-n79A with 2UL_DC_3A_n79A |
| DC_8_n40-n41C | DC_8A_n40A | 3rd harmonic from B8 into n41 | - | - | - Harmonic problem will be studied in DC_8A_n41A. - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  | DC_8A_n41A | - | 5th IMD into n40 | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
| DC_8_n40-n79C | DC_8A_n40A | 2nd harmonic from n40 into n79 5th harmonic from B8 into n79 | 4th IMD into n79 | - | - These harmonic problem will be treated lower order DC or CA band combos. - Same as DC_8A_n40A-n79A with 2UL_DC_8A_n40A |
|  | DC_8A_n79A | - | 4th IMD into n40 | - | - Same as DC_8A_n40A-n79A with 2UL_DC_8A_n79A |
| DC_8_n41A-n79C DC_8_n41C-n79A DC_8_n41C-n79C | DC_8A_n41A | 2nd harmonic from n41 into n79 5th harmonic from B8 into n79 | 3rd IMD into n79 | - | - These harmonic problem will be treated lower order DC or CA band combos. - Same as DC_8A_n41A-n79A with 2UL_DC_8A_n41A |
|  | DC_8A_n79A | 3rd harmonic from B8 into n41 | 3rd IMD into n41 | - | - These harmonic problem will be treated lower order DC or CA band combos. - Same as DC_8A_n41A-n79A with 2UL_DC_8A_n79A |
| DC_39_n40-n41C | DC_39A_n40A | - | 3rd IMD into n41 | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  | DC_39A_n41A | - | - | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
| DC_39_n40-n79C | DC_39A_n40A | 2nd harmonic from n40 into n79 | 4th IMD into n79 | - | - The harmonic problem will be treated CA_n40-n79 band combos. - Same as DC_39A_n40A-n79A with 2UL_DC_39A_n40A |
|  | DC_39A_n79A | - | - | - | No issue |
| DC_39_n41A-n79C DC_39_n41C-n79A DC_39_n41C-n79C | DC_39A_n41A | 2nd harmonic from n41 into n79 | 2nd & 5th IMDs into n79 | - | - Over 4992MHz frequency in n79 was impacted by 2nd harmonic, but not used - Same as DC_39A_n41A-n79A with 2UL_DC_39A_n41A |
|  | DC_39A_n79A | - | 2nd & 5th IMDs into n41 | - | - Same as DC_39A_n41A-n79A with 2UL_DC_39A_n79A |
| DC_13_n5-n48 | DC_13A_n48A | - | - | - | No issue |
| DC_13_n48-n66 | DC_13A_n48A | - | 3rd IMD into n66 | - | Follow 17.1dB MSD for CA_13A-48A-66A in TS36.101 |
|  | DC_13A_n66A | 2nd harmonic from n66 into n48 | 5th IMD into n48 | - | - 2nd harmonic issue will be solved in CA_n48-n66 - In rel-16 TR, define as 2.8 dB |
| DC_66_n5-n48 | DC_66A_n5A | 2nd harmonic from n66 into n48 | 5th IMD into n48 | - | - 2nd harmonic issue will be solved in CA_n48-n66 - In rel-16 TR, define as 3.3 dB |
|  | DC_66A_n48A | - | - | - | No issue |
| DC_11_n3-n28 | DC_11A_n3A | - | 5th IMD into n28 |  | - In rel-16 TR, define as 2.5 dB |
|  | DC_11A_n28A | - | - | - | No issue |
| DC_3_n1-n257 DC_3-3_n1-n257 | DC_3A_n1A | - | - | Yes | No issue |
|  | DC_3A_n257A | - | - | - | No issue |
| DC_7_n1-n257 DC_7-7_n1-n257 | DC_7A_n1A | High order harmonics from B7 or n1 | - | - | No issue |
|  | DC_7A_n257A | - | - | - | No issue |
| DC_3_n78-n258 | DC_3A_n78A | 7th & 8th harmonics from n78 into n258 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_3A_n258A | 2nd harmonic from B3 into B78 | - | - | - Harmonic problem already covered in DC_3A_n78A |
| DC_7A_n78-n258 DC_7C_n78-n258 | DC_7A_n78A DC_7C_n78A | 7th & 8th harmonics from n78 into n258 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_7A_n258A DC_7C_n258A | - | - | - | - No issue |
| DC_28_n78-n258 | DC_28A_n78A | 7th & 8th harmonics from n78 into n258 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_28A_n258A | 5th harmonic from B28 into n78 | - | - | - Harmonic problem already covered in DC_28A_n78A |
| DC_8_n78-n258 | DC_8A_n78A | 7th & 8th harmonics from n78 into n258 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_8A_n258A | 4th harmonic from B8 into n78 | - | - | - Harmonic problem already covered in DC_8A_n78A |
| DC_8_n40-n258 | DC_8A_n40A | 11th harmonics from n40 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_8A_n258A | - | - | - | - No issue |
| DC_3_n34-n258 | DC_3A_n34A | 14th & 15th harmonics from B3 into n258 12th & 13th harmonics from n34 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_3A_n258A | - | - | - | - No issue |
| DC_3_n41-n258 | DC_3A_n41A | 10th harmonic from n41 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_3A_n258A | - | - | - | - No issue |
| DC_8_n34-n258 | DC_8A_n34A | 12th & 13th harmonics from n34 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_8A_n258A | - | - | - | - No issue |
| DC_8_n41-n258 | DC_8A_n41A | 10th harmonic from n41 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_8A_n258A | 3rd harmonic from B8 into n41 | - | - | - Harmonic problem already covered in DC_8A_n41A |
| DC_39_n40-n258 | DC_39A_n40A | 11th harmonics from n40 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_39A_n258A | - | - | - | - No issue |
| DC_39_n41A-n258 DC_39_n41C-n258 | DC_39A_n41A | 10th harmonic from n41 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_39A_n258A | - | - | - | - No issue |
| DC_8_n79-n258 | DC_8A_n79A | 5th & 6th harmonic from n79 into n258 | - |  | - These harmonic problem will be covered in CA_n79A_n258A |
|  | DC_8A_n258A | 5th harmonic from B8 into n79 | - | - | - Harmonic problem already covered in DC_8A_n79A |
| DC_39_n79A-n258 DC_39_n79C-n258 | DC_39A_n79A | 5th & 6th harmonic from n79 into n258 | - |  | - These harmonic problem will be covered in CA_n79A_n258A |
|  | DC_39A_n258A | - | - | - | - No issue |
| DC_40_n41A-n258 DC_40_n41C-n258 | DC_40A_n41A | 10th harmonic from n41 into n258 11th harmonics from B40 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_40A_n258A | - | - | - | - No issue |
| DC_40_n79A-n258 DC_40_n79C-n258 | DC_40A_n79A | 11th harmonics from B40 into n258 5th & 6th harmonic from n79 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 - These harmonic problem will be covered in CA_n79A_n258A |
|  | DC_40A_n258A | 2nd harmonic from B40 into n79 |  |  | - These harmonic problem will be covered in DC_40A_n79A |
| DC_41_n79A-n258 DC_41_n79C-n258 | DC_41A_n79A | 10th harmonic from n41 into n258 5th & 6th harmonic from n79 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 - These harmonic problem will be covered in CA_n79A_n258A |
|  | DC_41A_n258A | 2nd harmonic from n41 into n79 |  |  | - Over 4992MHz frequency in n79 was impacted by 2nd harmonic, but not used |
| DC_8_n77(2A)-n257 | DC_8A_n77A | 7th & 8th Harmonics from n77 into n257 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I | 4th harmonic from B8 into n77 | - | - | - Harmonic problem already covered in DC_8A_n77A |
| DC_11_n77(2A)-n257 | DC_11A_n77A | 7th & 8th harmonics from n77 into n257 | - | - | No harmonic problem to n257 |
|  | DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I | - | - | - | No issue |
| DC_28_n77(2A)-n257D | DC_28A_n77A | 7th & 8th harmonics from n77 into n257 | - | - | No harmonic problem to n257 |
|  | DC_28A_n257D | 5th harmonic from B28 into n77 | - | - | 5th harmonic problem was solved in DC_28A_n77A |
| DC_2_n7A-n66A DC_2_n7(2A)-n66A | DC_2A_n7A | - | - | - | No issue |
|  | DC_2A_n66A | - | - | - | No issue |
| DC_12_n7A-n66A DC_12_n7(2A)-n66A | DC_12A_n7A | 3rd harmonic into n66 from B12 | - | - | The 3rd harmonic issue already covered in DC_12A_n66A |
|  | DC_12A_n66A | - | - | - | No issue |
| DC_2_n66A-n78(2A) DC_2_n66(2A)-n78A DC_2_n66(2A)-n78(2A) | DC_2A_n66A | 2nd harmonics into n78 from B2 and n66 | 2nd & 4th IMDs into n78 | - | The 2nd harmonic issue already covered in DC_66A_n78A - Follow same as DC_2A_n66A-n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_2A_n78A | - | 4th IMD into n66 | - | - For 4th IMD problem, follow same as DC_2A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_12_n66A-n78A DC_12_n66A-n78(2A) DC_12_n66(2A)-n78A DC_12_n66(2A)-n78(2A) | DC_12A_n66A | 2nd harmonic into n78 from n66 5th harmonic into n78 from B12 | 5th IMD into n78 | - | These harmonic issues are already covered in DC_66A_n78A or DC_12A_n78A - 4.1dB by 5th IMD in TR37.716-21-21 |
|  | DC_12A_n78A | 3rd harmonic into n66 from B12 | 3rd IMD into n66 | - | 3rd harmonic issue already covered in DC_12A_n66A - 16.5dB by 3rd IMD in TR37.716-21-21 |
| DC_13_n7-n78 | DC_13A_n7A | - | 2nd & 4th IMDs into n78 | - | - 29.0 dB by 2nd IMD already analyzed in TR37.716-21-21 |
|  | DC_13A_n78A |  | 2nd IMD into n7 | - | - 27.9 dB by 2nd IMD already analyzed in TR37.716-21-21 |
| DC_2_n38-n66 | DC_2A_n38A | - | - | - | No issue |
|  | DC_2A_n66A | - | - | - | No issue |
| DC_66_n38-n66 | DC_66A_n38A | - | - | - | No issue |
|  | DC_66A_n66A | - | - | - | DC_66A_n66A only allowed single UL switched mode |
| DC_1-3_n3-n77, DC_1-3_n3-n77(2A) or DC_1-3_n3-n78 | DC_1A_n3A | 2nd harmonic from n3 into n77/n78 | 2nd & 4th IMDs into n77/n78 | Yes | - Harmonic problems was already covered in DC_3A_n77A. - 2nd &4th IMDs problems same as DC_1A_n3A-n78A. |
|  | DC_1A_n77A or DC_1A_n78A |  | 2nd, 4th & 5th IMDs into B3 | Yes | These MSD issues are same as DC_1A-3A_n77A. - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_3A_n3A DC_(n)3AA | 2nd harmonic from n3 into n77/n78 | 2nd & 4th IMD into n77/n78 | - | - Harmonic problems was already covered in DC_3A_n77A. - Due to apply MPR, no need to define MSD |
|  | DC_3A_n77A or DC_3A_n78A | - | 2nd & 5th IMD into B1 2nd, 4th &5th IMD into n3 | - | - 31.0 dB by 2nd IMD that follow DC_1A-3A_n77A in TR37.863-02-01 in rel-15. - Follow DC_3A_n1A-n78A for 5th IMD problem. - MSD problems in n3 were already solved in DC_3A_n77A |
| DC_18_n28-n77 or DC_18_n28-n78 | DC_18A_n28A | 4th & 5th harmonic from B18 and n28 fall into n77/n78 | 5th IMD into n77/n78 | - | - Harmonic problems was already covered in DC_18A_n77A or CA_n28-n77 - 4.0 dB by IMD5 |
|  | DC_18A_n77A or DC_18A_n78A | - | 5th IMD into n77 | - | - 5.7 dB by IMD5 |
| DC_1_n3-n41 | DC_1A_n3A | - | 5th IMD into n41 | - | - 5.0 dB by 5th IMD that follow DC_1A-41A_n3A in TS38.101-3 |
|  | DC_1A_n41A | - | - | Yes | Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
| DC_3_n3-n41 | DC_3A_n3A | - | - | - | No issue |
|  | DC_3A_n41A | - | 4th IMD into n3 | - | 4th IMD issue already covered by DC_3_n41 in Table 7.3B.2.3.5.1-1 in TS38.101-13 |
| DC_18_n3-n41 | DC_18A_n3A | - | 2nd & 3rd IMDs into B41 | - | 3rd IMD problem already covered in DC_18A-41A_n3A - 29.4 dB for 2nd IMD |
|  | DC_18A_n41A | - | 2nd IMD into n3 |  | - 28.2 dB by IMD2 |
| DC_1-3_n3-n41 | DC_1A_n3A | - | 5th IMD into n41 |  | 5th IMD problem already covered in CA_1A-3A-41A in TS36.101 |
|  | DC_1A_n41A | - | - | - | No issue |
|  | DC_3A_n3A | - | - | - | No issue |
|  | DC_3A_n41A | - | - | - | No issue |
| DC_18_n28-n41 | DC_18A_n28A |  | 5th IMD into n41 | - | - 5.1 dB by IMD5 |
|  | DC_18A_n41A |  | 5th IMD into n28 | - | - 4.7 dB by IMD5 |
| DC_1_n41-n77 | DC_1A_n41A | 2nd harmonic from B1 into n77 | 3rd , 4th & 5th IMDs into n77 | - | - The harmonic problem already covered in DC_1_n77 in TS38.101-3 - These IMDs problem follow DC_1_n41-n78 in TS38.101-3 |
|  | DC_1A_n77A | - | 4th & 5th IMDs into n41 | - | - The IMD problem follow DC_1-41_n77 in TS38.101-3 |
| DC_3_n41-n77 | DC_3A_n41A | 2nd harmonic from B3 into n77 | 3rd & 5th IMDs into n77 | - | - The harmonic problem already covered in DC_3_n77 in TS38.101-3 - These IMDs problem follow DC_3_n41-n78 in TS38.101-3 |
|  | DC_3A_n77A |  | 5th IMD into n41 |  | - The IMD problem follow DC_3-41_n77 in TS38.101-3 |
| DC_18_n41-n77 or DC_18_n41-n78 | DC_18A_n41A | 4th harmonic from B18 into n78 | 2nd & 4th IMDs into n77/n78 3rd IMD into n77 | - | - 30.1 dB by IMD2 -16.4 dB by IMD3 - 9.1 dB by IMD4 |
|  | DC_18A_n77A or DC_18A_n78A |  | 2nd IMD into n4177/n78 3rd IMD into n41 in DC_18A_n77A | - | - 28.5 dB by IMD2 |
| DC_41_n3-n41 | DC_41A_n3A | - | 4th IMD into n41 | Yes | For TDD band, the IMD problem is not impact to the 3rd Rx band |
|  | DC_41A_n41A | - | - | Yes | No issue |
| DC_41_n28-n41 | DC_41A_n28A | - | - | - | No issue |
|  | DC_41A_n41A | - | - | - | No issue |
| DC_41_n41-n77 or DC_41_n41-n78 | DC_41A_n41A | - | - | - | No issue |
|  | DC_41A_n77A or DC_41A_n78A | - | 4th IMD into n41 |  | For TDD band, the IMD problem is not impact to the 3rd Rx band |
| DC_(n)5AA-n77A | DC_(n)5AA | 4th & 5th harmonics from n5 into n77 | 4th & 5th IMD into n77 |  | 4th & 5th harmonic issues were covered in DC_5A_n77A The 4th & 5th IMD problem will not be impacted due to only single UL switching operation. |
|  | DC_5A_n77A |  | 4th & 5th IMDs |  | The 4th & 5th IMD problem will be covered in DC_5A_n77A |
| DC_12_n5-n77 | DC_12A_n5A | 4th & 5th harmonics from n5 into n77 | 5th IMD |  | The 4th & 5th IMD problem will be covered in DC_5A_n77A - 5.4 dB by IMD5 |
|  | DC_12A_n77A |  | 5th IMD |  | - 4.8 dB by IMD5 |
| DC_14_n5-n77 | DC_14A_n5A | 4th & 5th harmonics from n5 into n77 | 4th & 5th IMD |  | The 4th & 5th IMD problem will be covered in DC_5A_n77A - 10.6 dB by IMD4 -5.1 dB by IMD5 |
|  | DC_14A_n77A |  | 5th IMD |  | - 4.4 dB by IMD5 |
| DC_2_n71-n78 | DC_2A_n71A | 2nd harmonic from B2 into n78 5th harmonic from n71 into n78 | 3rd IMD into n78 | - | These harmonics issues were already solved in DC_2_n78 or CA_n71_n78 - 8.0 dB by IMD3 |
|  | DC_2A_n78A | - | - | - | No issue |
| DC_7_n71-n78 | DC_7A_n71A | 5th harmonic from n71 into n78 | 4th IMD into n78 | - | - The harmonic issue was already solved in CA_n71_n78 - 9.7 dB by IMD4 |
|  | DC_7A_n78A | - | 5th IMD into n71 | - | - 3.9 dB by IMD5 |
| DC_66_n71-n78 | DC_66A_n71A | -2nd harmonic from B66 into n78 5th harmonic from n71 into n78 | 4th & 5th IMDs into n78 | - | Harmonic issues were already solved in DC_66_n78 or CA_n71_n78 - MSD of 4th IMD can be reused in DC_48A-66A_n71A in TS38.101-3 |
|  | DC_66A_n78A | - | - | - | No issue |
| DC_2_n38-n71 | DC_2A_n38A | - | - | - | No issue |
|  | DC_2A_n71A | - | 2nd IMD into n38 | - | 29.2 dB of DC_2A_n41A-n71A can be reused in TS38.101-3 |
| DC_66_n38-n71 | DC_66A_n38A | - | - | - | No issue |
|  | DC_66A_n71A | - | - | - | No issue |
| DC_2_n38-n78 | DC_2A_n38A | 2nd harmonic from B2 | 3rd IMD into n78 |  | Harmonic issue were covered in DC_2A_n78A - 14.8 dB was captured in TR37.716-21-21 in Rel-16 |
|  | DC_2A_n78A | - | - | - | No issue |
| DC_66_n38-n78 | DC_66A_n38A | 2nd harmonic from B66 | 3rd IMD into n78 | - | Harmonic issue were covered in DC_66A_n78A - 15.0 dB was captured in TR37.716-21-21 in Rel-16 |
|  | DC_66A_n78A | - | - | - | No issue |
| DC_71_n38-n78 | DC_71A_n38A | 5th harmonic from B71 into n78 | 2nd & 4th IMDs into n78 |  | Harmonic issue were covered in DC_71A_n78A - 29.1 dB by IMD2 -10.3 dB by IMD4 |
|  | DC_71A_n78A |  | 2nd IMD into n38 |  | - 28.7 dB by IMD2 |
| DC_5_n66-n78 | DC_5A_n66A | -2nd harmonic from B66 into n78 4th harmonic from B5 into n78 | 3rd & 5th IMDs into n78 |  | Harmonic issue were covered in DC_66A_n78A or DC_5A_n78A - Same MSD levels can be reused in DC_66_n5-n77 in this TR |
|  | DC_5A_n78A | - | 3rd IMD into n66 |  | 13.2 dB by DC_5A-66A_n78A can be reused in TS38.101-3 |
| DC_71_n66-n78 | DC_71A_n66A | -2nd harmonic from n66 into n78 5th harmonic from B71 into n78 | 4th & 5th IMDs into n78 | - | Harmonic issues were already solved in DC_2_n78 or CA_n71_n78 - MSD of 4th IMD can be reused in DC_48A-66A_n71A in TS38.101-3 |
|  | DC_71A_n78A |  | 3rd IMD into n66 | - | - 15.5 dB by IMD3 |
| DC_5_n38-n66 | DC_5A_n38A | - | - | - | No issue |
|  | DC_5A_n66A |  | 2nd & 3rd IMD into n38 |  | - 28.9 dB by IMD2 - 16.4 dB by IMD3 |
| DC_71_n38-n66 | DC_71A_n38A | - | - | - | No issue |
|  | DC_71A_n66A | - | - | - | No issue |
| DC_2_n2-n38 or DC_2_n2-n41 | DC_2A_n2A | - | - | - | DC_2A_n2A only allowed single UL switched mode |
|  | DC_2A_n38A or DC_2A_n41A | - | - | - | No issue |
| DC_66_n2-n38 | DC_66A_n2A | - | - | - | No issue |
|  | DC_66A_n38A | - | - | - | No issue |
| DC_12_n2-n38 or DC_12_n2-n41 | DC_12A_n2A |  | 2nd IMD into n38/n41 | - | - 28.7 dB by IMD2 |
|  | DC_12A_n38A or DC_12A_n41A | - | - | - | No issue |
| DC_71_n2-n41 | DC_71A_n2A | 4th harmonic from B71 into n41 | 2nd IMD into n41 | - | Harmonic issue already covered in DC_71_n41 - 29.2dB MSD by DC_2A_n41A-n71A can be reused in TS38.101-3 |
|  | DC_71A_n41A | 3rd harmonic from B71 into n2 | 2nd IMD into n2 |  | Harmonic issue already covered in DC_71_n2 - 26.0dB MSD by DC_2A-71A_n38A can be reused in TS38.101-3 |
| DC_2_n2-n78 | DC_2A_n2A | 2nd harmonic from B2 into n78 | 2nd & 4th IMDs into n78 |  | DC_2A_n2A only allowed single UL switched mode. |
|  | DC_2A_n78A |  | 2nd,4th & 5th IMDs into n2 |  | The IMD problem already solved in DC_2A_n78A in TS38.101-3 |
| DC_7_n2-n78 | DC_7A_n2A | 2nd harmonic from n2 into n78 | 5th IMD into n78 |  | Harmonic issue already covered in DC_2_n78 4.2dB MSD of DC_2A_n7A-n78A can be reused in TS38.101-3 |
|  | DC_7A_n78A | - | 4th IMD into n2 |  | 8.6dB MSD of DC_2A-7A_n78A can be reused in TS38.101-3 |
| DC_71_n2-n78 | DC_71A_n2A | 2nd harmonic from n2 into n78 5th harmonic from B71 into n78 | 3rd IMD into n78 |  | Harmonic issue already covered in DC_2_n78 or DC_71_n78 - 3rd IMD can be reused in DC_2_n71-n78 in this TR |
|  | DC_71A_n78A | 3rd harmonic from B71 into n2 | 3rd IMD into n2 |  | Harmonic issue already covered in DC_71_n2 - 16.5dB MSD of DC_2A-71A_n78A can be reused in TS38.101-3 |
| DC_2_n66-n71 DC_2-2_n66-n71 | DC_2A_n66A | - | - | - | No issue |
|  | DC_2A_n71A | - | - | - | No issue |
| DC_7_n66-n71 | DC_7A_n66A | - | - | - | No issue |
|  | DC_7A_n71A | - | - | - | No issue |
| DC_66_n66-n71 | DC_66A_n66A | - | - | - | DC_66A_n66A only allowed single UL switched mode |
|  | DC_66A_n71A |  | 4th IMD into n66 |  | 5.0dB MSD for DC_66A_n71A can be reused in TS38.101-3 |
| DC_2_n2-n71 | DC_2A_n2A | - | - | - | DC_2A_n2A only allowed single UL switched mode |
|  | DC_2A_n71A | 3rd harmonic from n71 into B2 | - | - | Harmonic issue already covered in DC_2_n71 |
| DC_7_n2-n71 | DC_7A_n2A |  | 2nd & 5th IMDs into n71 |  | 28.7dB MSD for 2nd IMD of DC_2A_n41A-n71A can be reused in TS38.101-3 |
|  | DC_7A_n71A | 3rd harmonic from n71 into n2 | - | - | Harmonic issue already covered in DC_2_n71 |
| DC_66_n2-n71 | DC_66A_n2A | - | - | - | No issue |
|  | DC_66A_n71A | 3rd harmonic from n71 into n2 | - | - | Harmonic issue already covered in DC_2_n71 |
| DC_2_n2-n66 | DC_2A_n2A | - | - | - | DC_2A_n2A only allowed single UL switched mode |
|  | DC_2A_n66A |  | 3rd & 5th IMDs into n2 |  | 20.0dB MSD of for 3rd IMD DC_2A_n66A can be reused in TS38.101-3 |
| DC_7_n2-n66 | DC_7A_n2A | - | - | - | No issue |
|  | DC_7A_n66A | - | - | - | No issue |
| DC_66_n2-n66 | DC_66A_n2A |  | 5th IMD into n66 |  | 4.0dB MSD of for 5th IMD DC_2A_n66A can be reused in TS38.101-3 |
|  | DC_66A_n66A | - | - | - | DC_66A_n66A only allowed single UL switched mode |
| DC_71_n2-n66 | DC_71A_n2A | - | - | - | No issue |
|  | DC_71A_n66A | 3rd harmonic from B71 into n2 | - | - | Harmonic issue already covered in DC_2_n71 |
| DC_2_n66-n77 or DC_2-2_n66-n77 | DC_2A_n77A | - | 2nd , 4th & 5th IMDs into n66 | - | - 29.2 dB for 2nd IMD - 10.3 dB MSD by 4th IMD in DC_2A-66A_n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_2_n66-n77 DC_2-2_n66-n77 | DC_2A_n66A | 2nd harmonic from B2 or n66 | 2nd & 4th IMDs into n77 |  | - Harmonic issue already covered in DC_2_n77 or CA_n66-n77 - 29.4 dB and 8.9 dB MSD by 2nd & 4th IMDs in DC_2A_n66A-n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_13_n66-n77 | DC_13A_n77A |  | 3rd IMD into n66 |  | - 17.1dB MSD for DC_13-66A_n48A can be reused in TS38.101-3 |
|  | DC_13A_n66A | 2nd harmonic from n66 into n77 5th harmonic from B13 into n77 | 3rd, 4th & 5th IMDs into n77 | - | -Harmonic issue will be solved in CA_n66-n77 or DC_13_n77 - 16.4 dB by IMD3 - 11.7 dB by IMD4 - 4.4 dB by IMD5 |
| DC_66_n66-n77 | DC_66A_n77A | - | 2nd & 5th IMDs into B66 | - | - These IMDs problems already covered in DC_66A_n77A |
| DC_11_n3-n77 or DC_11_n3-n77(2A) | DC_11A_n3A | 2nd harmonic from n3 into n77 | 4th & 5th IMDs into n77 | - | Harmonic issues already solved in DC_3_n77 - 10.8 dB by IMD4 |
|  | DC_11A_n77A |  | 2nd & 5th IMDs into n3 |  | - 29.0 dB by IMD2 |
| DC_2_n25-n66 | DC_2A_n25A | - | 5th IMD into n66 |  | - Reuse 4.0 dB MSD in DC_2A-66A_n25A in TS38.101-3 |
|  | DC_2A_n66A |  | 3rd & 5th IMDs into n25 |  | Reuse 20.0 dB MSD in DC_2A_n66A for 3rd IMD in TS38.101-3 |
| DC_7_n25-n66 DC_7-7_n25-n66 | DC_7A_n25A | - | - | - | No issue |
|  | DC_7A_n66A | - | - | - | No issue |
| DC_13_n25-n66 | DC_13A_n25A | - | 4th IMD into n66 | - | - Reuse 7.2 dB MSD in DC_13A-66A_n2A in TS38.101-3 |
|  | DC_13A_n66A | - | 4th IMD into n25 | - | - Reuse 6.2 dB MSD in DC_2A-13A_n66A in TS38.101-3 |
| DC_66_n25-n66 | DC_66A_n25A | - | 3rd & 5th IMDs into n66 | - | - Reuse 23.0 dB & 4.0 dB MSD in DC_66A_n25A in TS38.101-3 |
| DC_7_n66-n77 DC_7-7_n66-n77 | DC_7A_n66A | 2nd harmonic from n66 into n77 | 3rd & 5th IMDs into n77 |  | - Harmonic issue already covered in CA_n66-n77 - Reuse 16.0dB MSD for 3rd IMD in DC_7A_n66A-n78A in TS38.101-3 |
|  | DC_7A_n77A | - | 4th IMD into n66 | - | - Reuse 8.7dB MSD for 4th IMD in DC_7A-66A_n78A in TS38.101-3 |
| DC_21_n28-n77 | DC_21A_n28A | 5th harmonic from n28 into n77 | 3rd & 4th IMDs into n77 | - | - 17.3 dB in n77 by IMD3 |
|  | DC_21A_n77A | - | 3rd & 4th IMDs into n28 | - | - Reuse 16.9 dB MSD for 3rd IMD in DC_21A-28A_n77A in TS38.101-3 |
| DC_1_n28-n79 | DC_1A_n28A | 6th & 7th harmonic from n28 into n79 | 3rd & 5th IMDs into n79 |  | - No harmonic issue by 6th & 7th harmonic order - 14.9dB in n79 by IMD3 |
|  | DC_1A_n79A | - | 3rd & 4th IMDs into n28 |  | - Reuse 15.2 dB/ 10.0 dB MSD for 3rd /4th IMD in DC_1A-28A_n79A in TS38.101-3 |
| DC_3_n28-n79 | DC_3A_n28A | 6th & 7th harmonic from n28 into n79 | 4th & 5th IMDs into n79 |  | - No harmonic issue by 6th & 7th harmonic order - 9.4dB in n79 by IMD4 |
|  | DC_3A_n79A | - | 4th IMD into n28 | - | - Reuse 10.3 dB MSD for 4th IMD in DC_3A-28A_n79A in TS38.101-3 |
| DC_21_n28-n79 | DC_21A_n28A | 6th & 7th harmonic from n28 into n79 | 4th & 5th IMDs into n79 |  | - No harmonic issue by 6th & 7th harmonic order - 6.3 dB in n79 by IMD4 |
|  | DC_21A_n79A | - | 5th IMD into n28 | - | - 2.8 dB in n28 by IMD5 |
| DC_21_n28-n78 | DC_21A_n28A | 5th harmonic from n28 into n78 | 3rd & 4th IMDs into n78 |  | - Harmonic issue already covered in CA_n28-n78 - same MSD value and test configuration of DC_21A_n28A-n77A in this TR |
|  | DC_21A_n78A | - | 3rd IMD into n28 | - | - Reuse 16.9 dB MSD for 3rd IMD in DC_21A-28A_n77A in TS38.101-3 |
| DC_2_n2-n77 | DC_2A_n77A | - | 2nd & 4th IMD IMDs into n2 | - | - Reuse 26 dB/ 8dB MSD for 2nd / 4th IMD in DC_2A_n78A in TS38.101-3 |
| DC_5_n2-n77 | DC_5A_n2A | 2nd harmonic from n2 and 4th & 5th harmonics from B5 into n77 | 3rd & 5th IMDs into n77 | - | - Harmonic issue already covered in CA_n2-n77 or DC_5_n77 - 16.0dB MSD by 3rd IMD of DC_2A_n5A-n77A in this TR can be used |
|  | DC_5A_n77A | - | - | - | No issue |
| DC_13_n5-n77 | DC_13A_n77A | - | 5th IMD into n5 | - | - 4.5 dB in n5 by IMD5 |
| DC_66-66_n2-n77 | DC_66A_n2A | 2nd harmonic from B66 or n2 into n77 | 2nd & 4th IMDs into n77 |  | - Harmonic issue already covered in CA_n2-n77 or DC_66_n77 - Reuse 29.4 dB/ 8.9dB MSD for 2nd / 4th IMD in DC_2A_n66A-n78A in TS38.101-3 |
|  | DC_66A_n77A | - | 2nd , 4th & 5th IMDs into n2 |  | - Reuse 32.1 dB/ 9.1dB/ 2.1dB MSD for 2nd / 4th / 5th IMD in DC_2A-66A_n78A in TS38.101-3 |
| DC_5_n5-n77 | DC_5A_n77A | - | 4th & 5th IMDs into n5 |  | - Reuse 8.3dB MSD for 4th IMD in DC_5A_n78A in TS38.101-3 |
| DC_2-66_n66-n77 | DC_2A_n66A | 2nd harmonic from B2 or n66 into n77 | 2nd & 4th IMDs into n77 |  | - Harmonic issue already covered in DC_2_n77 or CA_n66-n77 - 29.4 dB and 8.9 dB MSD by 2nd & 4th IMDs in DC_2A_n66A-n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_2A_n77A | - | 2nd , 4th & 5th IMDs into n66 | - | - 29.2 dB for 2nd IMD - 10.3 dB MSD by 4th IMD in DC_2A-66A_n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_66A_n77A | - | 2nd , 4th & 5th IMDs into n2 |  | - Reuse 32.1 dB/ 9.1dB/ 2.1dB MSD for 2nd / 4th / 5th IMD in DC_2A-66A_n78A in TS38.101-3 |
| DC_5-66_n66-n77 | DC_5A_n66A | -2nd harmonic from B66 into n77 4th & 5th harmonic from B5 into n77 | 3rd , 4th & 5th IMDs into n77 |  | Harmonic issue were covered in DC_66A_n77A or DC_5A_n77A - Same MSD levels can be reused in DC_66_n5-n77 in this TR |
|  | DC_5A_n77A |  | 3rd IMD into B66 or n66 |  | - Reuse 13.2 dB MSD for 3rd IMD in DC_5A-66A_n78A in TS38.101-3 |
|  | DC_66A_n77A | - | 2nd & 5th IMDs into n66 | - | - These IMD problem already solved in DC_66A_n77A |
| DC_5-66_n5-n77 | DC_5A_n77A |  | 3rd IMD into B66 or n66 4th & 5th IMDs into n5 |  | - Reuse 13.2 dB MSD for 3rd IMD in DC_5A-66A_n78A in TS38.101-3 - Reuse 8.3 dB MSD for 4th IMD in DC_5A_n78A in TS38.101-3 |
|  | DC_66A_n5A | -2nd harmonic from B66 into n77 4th & 5th harmonic from B5 into n77 | 3rd , 4th & 5th IMDs into n77 2nd & 5th IMD into B5 |  | Harmonic issue were covered in DC_66A_n77A or DC_5A_n77A - Same MSD levels can be reused in DC_66_n5-n77 in this TR - 30dB MSD for 2nd IMD in DC_5A_n66A in TS38.101-3 can be reused |
|  | DC_66A_n77A | - | - | - | No issue |
| DC_28_n7-n258 | DC_28A_n7A DC_28A_n7B | 10th harmonic from n7 into n258 | - | - | No issue |
|  | DC_28A_n258A/G/H/I | - | - | - | No issue |
| DC_30_n5-n77 | DC_30_n5 | 4th & 5th harmonic from n5 into n77 | 3rd IMD into n77 | - | 4th & 5th Harmonic issues already covered in DC_5A_n77A. -16.1 dBby 3rd IMD |
|  | DC_30_n77 | - | 3rd & 5th IMDs into n5 | - | - 15.3 dB by 3rd IMD |
| DC_3-3_n8-n78 | DC_3_n8 | 2nd harmonic from B3 into n78 4th harmonic from n8 into n78 | 3rd & 5th IMDs into n78 4th & 5th IMDs into B3 | - | The 3rd IMD problem, follow MSD (16.3dB) for DC_3A_n8A-n78A. The 4th &5th IMDs problem already covered in DC_3A_n8A. |
|  | DC_3_n78 | - | - | - | No issue |
| DC_7-7_n8-n78 | DC_7_n8 | 4th harmonic from n8 into n78 | 2nd & 4th IMDs into n78 |  | 4th Harmonic issues already covered in DC_8A_n78A. The 2nd IMD problem, follow MSD (28.5dB) for DC_7A_n8A-n78A. |
|  | DC_7_n78 | - | 2nd IMD into n8 |  | The 2nd IMDs problem, follow MSD (29.7dB) for DC_7A_n8A-n78A. |
| DC_3_n1-n8 DC_3-3_n1-n8 | DC_3_n1 | - | - | Yes | - Small freq. gap was covered in Table 7.3.1A-0bA in TS38.101-3 |
|  | DC_3_n8 | - | - | - | No issue |
| DC_7_n1-n8 DC_7-7_n1-n8 | DC_7_n1 | - | 5th IMD into n8 | - | - Reuse 4.5dB by 5th IMD from DC_1A-28A_n7A |
|  | DC_7_n8 | 3rd harmonic from n8 into B7 | - | - | The harmonic issue already covered in DC_7A_n8A. |
| DC_n1-n28_3 | DC_n1A_3A | - | - | - | No issue |
|  | DC_n28A_3A | 3rd harmonic from n28 into n1 | 4th IMD | - | - Harmonic problem already covered in DC_1A_n28A - 11.0dB by 4th IMD problem same as DC_1A-3A_28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_3_n28-n78 | DC_3C_n78A | - | - | - | No issue |
| DC_3_n7-n78 | DC_3C_n78A | - | - | - | No issues |
| DC_8_n1-n40 | DC_8A_n1A | - | - | - | No issues |
|  | DC_8A_n40A | - | 5th IMD into n1 | - | - FFS |
| DC_3_n1-n38 | DC_3A_n1A | - | - | - | No issues |
|  | DC_3A_n38A | - | - | - | No issues |
| DC_3_n1-n41 | DC_3A_n1A | - | 5th IMD into n41 | - | - 5.0 dB by 5th IMD that follow DC_1A-41A_n3A in TS38.101-3 |
|  | DC_3A_n41A | - | - | - | No issues |
| DC_1_n3-n79 | DC_1A_n3A |  | 5th IMD | Yes | - 4.7dB by 5th IMD that follow DC_3A_n1A-n79A in TR37.716-21-21 |
|  | DC_1A_n79A |  |  | Yes | Cross isolation issue already solved. |
| DC_8_n3-n79 | DC_8A_n3A |  | 3rd & 5th IMDs | - | - 15.7dB by 3rd IMD |
|  | DC_8A_n79A |  | 4th IMD | - | -8.8dB by 4th IMD that follow DC_3A-8A_n79A in TS38.101-3 |
| DC_8_n28-n79 | DC_8A_n28A | 5th harmonic from B8 into n79 | 5th IMD | - | - Harmonic problem already covered in DC_8A_n79A - FFS |
|  | DC_8A_n79A |  | 5th IMD | - | - FFS |
| DC_8_n77-n79 | DC_8A_n77A | 5th harmonic from B8 into n79 | 2nd , 4th & 5th IMDs |  | - Harmonic problem already covered in DC_8A_n79A - RAN4 agreed not to consider simultaneous Rx/Tx capability for the CA band combinations with n77 and n79. |
|  | DC_8A_n79A | - | 2nd IMD |  | - RAN4 agreed not to consider simultaneous Rx/Tx capability for the CA band combinations with n77 and n79. |
| DC_8_n77-n257D/G/H/I | DC_8A_n77A | - | - |  | No issues |
|  | DC_8A_n257D/G/H/I | - | - |  | No issues |
| DC_11_n77-n257D/G/H/I | DC_11A_n77A | - | - |  | No issues |
|  | DC_11A_n257D/G/H/I | - | - |  | No issues |
| DC_2_n12-n77 DC_2-2_n12-n77 | DC_2A_n12A | 2nd harmonic from B2 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_2A_n77A - 16.0 dB MSD by 3rd IMD from CA_n2-n12-n77 can be reused |
|  | DC_2A_n77A | - | - | - | No issues |
| DC_7_n12-n77 | DC_7A_n12A | - | 3rd & 4th IMDs | - | - FFS |
|  | DC_7A_n77A | - | 2nd IMD | - | - 30.8dB MSD by 2nd IMD from DC_7A-12A_n78A can be reused |
| DC_66_n12-n77 | DC_66A_n12A | 2nd harmonic from B66 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_66A_n77A - 16.0 dB MSD by 3rd IMD from CA_n12-n66-n77 can be reused |
|  | DC_66A_n77A |  | 3rd IMD | - | - 15.2dB MSD by 3rd IMD from DC_12A-66A_n77A can be reused |
| DC_12_n41-n66 | DC_12A_n41A | 3rd harmonic from B12 into n66 | - | - | - Harmonic problem already covered in DC_12A_n66A |
|  | DC_12A_n66A | - | 2nd IMD | - | Edge IMD2 regions will be impact in n41 - 28.7dB MSD by 2nd IMD from DC_12A_n2A-n41A can be reused |
| DC_12_n66-n77 | DC_12A_n66A | 2nd harmonic from n66 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_66A_n77A - 16.0 dB MSD by 3rd IMD from CA_n12-n66-n77 can be reused |
|  | DC_12A_n77A | 3rd harmonic from B12 into n66 | 3rd & 4th IMDs | - | - Harmonic problem already covered in DC_12A_n66A - 13.2dB MSD by 3rd IMD from DC_12A-66A_n77A can be reused |
| DC_71_n66-n77 | DC_71A_n66A | 2nd harmonic from n66 into n77 5th harmonic from B71 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_66A_n77A or DC_71A_n77A - 15.9dB MSD by 3rd IMD from CA_n66-n71-n77 can be reused |
|  | DC_71A_n77A | - | 3rd & 4th IMDs | - | - 15.5dB MSD by 3rd IMD from DC_71A_n66A-n78A can be reused |
| DC_2_n71-n77 DC_2-2_n71-n77 | DC_2A_n71A | 2nd harmonic from B2 into n77 5th harmonic from n71 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_2A_n77A or DC_71A_n77A - 8.0dB MSD by 3rd IMD from DC_2A_n71A-n78A can be reused |
|  | DC_2A_n77A | - | - | - | No issues |
| DC_7_n71-n77 | DC_7A_n71A | 5th harmonic from n71 into n77 | 3rd & 4th IMDs | - | - Harmonic problem already covered in DC_71A_n77A - FFS |
|  | DC_7A_n77A | - | - | - | No issues |
| DC_66_n71-n77 | DC_66A_n71A | 2nd harmonic from B66 into n77 5th harmonic from n71 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_66A_n77A or DC_71A_n77A - 15.9dB MSD by 3rd IMD from CA_n66-n71-n77 can be reused |
|  | DC_66A_n77A | - | 3rd IMD | - | - 15.2dB MSD by 3rd IMD from DC_12A-66A_n77A can be reused |
| DC_5_n2-n41 | DC_5A_n2A | 3rd harmonic from B5 into n41 | 2nd IMD | - | - Harmonic problem already covered in DC_5A_n41A - FFS |
|  | DC_5A_n41A | - | - | - | No issues |
| DC_5_n2-n66 | DC_5A_n2A | - | 4th IMD | - | - 7.2dB MSD by 4th IMD from CA_n2-n5-n66 can be reused |
|  | DC_5A_n66A | - | - | - | No issues |
| DC_12_n2-n66 | DC_12A_n2A | 3rd harmonic from B12 into n66 | - | - | - Harmonic problem already covered in DC_12A_n66A |
|  | DC_12A_n66A | - | 4th IMD | - | - 0dB MSD in DC_2A-12A_n66A in TS38.101-3 |
| DC_7_n2-n77 | DC_7A_n2A | 2nd harmonic from n2 into n77 | 5th IMD |  | - Harmonic problem already covered in DC_2A_n77A - 4.2dB MSD by 5th IMD from DC_2A_n7A-n78A can be reused |
|  | DC_7A_n77A | - | 4th IMD | - | - 8.6dB MSD by 4th IMD from DC_2A-7A_n78A can be reused |
| DC_12_n2-n77 | DC_12A_n2A | 2nd harmonic from n2 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_2A_n77A - 16.0dB MSD by 3rd IMD from CA_n2-n12-n77 can be reused |
|  | DC_12A_n77A | - | 3rd & 4th IMDs | - | - 16.5dB MSD by 3rd IMD from DC_2A-12A_n77A can be reused |
| DC_71_n2-n77 | DC_71A_n2A | 2nd harmonic from n2 into n77 5th harmonic from B71 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_2A_n77A or DC_71A_n77A - 8.0 dB MSD by 3rd IMD from DC_2A_n71A-n78A |
|  | DC_71A_n77A | - | 3rd & 4th IMDs | - | - 16.5dB MSD by 3rd IMD from DC_2A-71A_n78A can be reused |
| DC_5_n2-n78 | DC_5A_n2A | 2nd harmonic from n2 into n78 4th harmonic from B5 into n78 | 3rd IMD | - | - Harmonic problem already covered in DC_2A_n78A or DC_5A_n78A - 16.0dB MSD by 3rd IMD from DC_2A_n5A-n77A can be reused |
|  | DC_5A_n78A |  | 3rd IMD |  | - 16.5dB MSD by 3rd IMD from DC_2A-5A_n77A can be reused |
| DC_12_n2-n78 | DC_12A_n2A | 2nd harmonic from n2 into n78 | 3rd IMD | - | - Harmonic problem already covered in DC_2A_n77A - MSD will follow from DC_12_n2-n77 |
|  | DC_12A_n78A | - | 3rd IMD | - | - MSD will follow from DC_12_n2-n77 |
| DC_2-2_n41-n66 | DC_2A_n41A | - | - | - | No issues |
|  | DC_2A_n66A | - | - | - | No issues |
| DC_2-2_n41-n71 | DC_2A_n41A | - | 2nd & 5th IMDs | - | - 28.7dB MSD by 2nd IMD from DC_2A_n41A-n71A can be reused |
|  | DC_2A_n71A | 4th harmonic from n71 into n41 | - | - | - Harmonic problem already covered in DC_71A_n41A |
| DC_2-2_n66-n78 | DC_2A_n66A | 2nd harmonic from B2 or n66 into n78 | 2nd & 4th IMDs into n78 |  | - Harmonic issue already covered in DC_2_n78 or CA_n66-n78 - 29.4 dB and 8.9 dB MSD by 2nd & 4th IMDs in DC_2A_n66A-n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_2A_n78A | - | 4th IMD into n66 | - | - 10.3 dB MSD by 4th IMD in DC_2A-66A_n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_2-5_n2-n41 | DC_2A_n2A | - | - | - | No issues when RAN4 only consider the single switched uplink transmission |
|  | DC_2A_n41A | - | - | - | No issue |
|  | DC_5A_n2A | 3rd harmonics from n2 into n41 | 2nd IMD into n41 |  | - Harmonic issue already covered in DC_2_n41 - FFS |
|  | DC_5A_n41A | - | - | - | No issues |
| DC_2-12_n2-n41 | DC_2A_n2A | - | - | - | No issues when RAN4 only consider the single switched uplink transmission |
|  | DC_2A_n41A | - | 2nd & 5th IMDs into B12 | - | - 28.7dB MSD by 2nd IMD from DC_2A-12A_n41A can be reused |
|  | DC_12A_n2A | 3rd harmonics from n2 into n41 | 2nd IMD into n41 |  | - Harmonic issue already covered in DC_2_n41 - 28.7 dB by IMD2 from DC_12A_n2A-n41A can be reused |
|  | DC_12A_n41A | - | 2nd IMD into B2 and n2 | - | - 26.0dB MSD by 2nd IMD from DC_2A-12A_n41A can be reused |
| DC_2-66_n2-n41 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n41A | - | - | - | No issues |
|  | DC_66A_n2A | - | - | - | No issues |
|  | DC_66A_n41A |  | 4th IMD into B2 and n2 | - | - 11.0dB MSD by 2nd IMD from DC_2A-66A_n41A can be reused |
| DC_2-71_n2-n41 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n41A | - | 2nd & 5th IMDs into B71 | - | - 28.7dB MSD by 2nd IMD from DC_2A_n41A-n71A can be reused |
|  | DC_71A_n2A | 4th harmonic from B71 into n41 | - | - | - Harmonic problem already covered in DC_71A_n41A |
|  | DC_71A_n41A | 3rd harmonic from B71 into B2/n2 | 2nd IMD into B2 and n2 | - | - Harmonic issue already covered in DC_2_n71 - 26.0dB MSD by DC_71A_n2A-n41A can be reused in TS38.101-3 |
| DC_2-5_n2-n66 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n66A | - | - | - | No issues |
|  | DC_5A_n2A | - | 4th IMD into n66 | - | - 7.2dB MSD by 4th IMD from CA_n2-n5-n66 can be reused |
|  | DC_5A_n66A | - | - | - | No issues |
| DC_2-7_n2-n66 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n66A | - | - | - | No issues |
|  | DC_7A_n2A | - | - | - | No issues |
|  | DC_7A_n66A | - | - | - | No issues |
| DC_2-12_n2-n66 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n66A | - | - | - | No issues |
|  | DC_12A_n2A | 3rd harmonics from B12 into n66 | - | - | - Harmonic problem already covered in DC_12A_n66A |
|  | DC_12A_n66A | - | 4th IMD into B2 and n2 |  | - 0dB MSD in DC_2A-12A_n66A in TS38.101-3 |
| DC_2-71_n2-n66 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n66A | - | - | - | No issues |
|  | DC_71A_n2A | - | - | - | No issues |
|  | DC_71A_n66A | 3rd harmonics from B71 into B2 and n2 | - | - | - Harmonic problem already covered in DC_2A_n71A |
| DC_2-7_n2-n71 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n71A | - | - | - | No issues |
|  | DC_7A_n2A | - | 2nd & 5th IMDs into n71 | - | - 28.7dB MSD for 2nd IMD of DC_7A_n2A-n71A can be reused in TS38.101-3 |
|  | DC_7A_n71A | 3rd harmonics from n71 into B2 and n2 | - | - | - Harmonic problem already covered in DC_2A_n71A |
| DC_2-66_n2-n71 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n71A | - | - | - | No issues |
|  | DC_66A_n2A | - | - | - | No issues |
|  | DC_66A_n71A | 3rd harmonics from B71 into B2 and n2 | - | - | - Harmonic problem already covered in DC_2A_n71A |
| DC_2-7_n2-n77 | DC_2A_n2A | 2nd harmonic from B2 into n77 | 2nd & 4th IMDs into n77 | - | - Harmonic problem already covered in DC_2A_n77A - Only switched uplink transmission is allowed |
|  | DC_2A_n77A | - | 5th IMD into B7 |  | - 3.4dB MSD for 5th IMD of DC_2A-7A_n77A can be reused in TS38.101-3 |
|  | DC_7A_n2A | 2nd harmonic from n2 into n77 | 5th IMD into n77 |  | - Harmonic problem already covered in DC_2A_n77A - 4.2dB MSD for 5th IMD of DC_2A_n7A-n78A can be reused in TS38.101-3 |
|  | DC_7A_n77A | - | 4th IMD into B2 and n2 | - | - 8.6dB MSD for 4th IMD of DC_2A-7A_n77A can be reused in TS38.101-3 |
| DC_2-12_n2-n77 | DC_2A_n2A | 2nd harmonic from B2 into n77 | 2nd & 4th IMDs into n77 | - | - Harmonic problem already covered in DC_2A_n77A - Only switched uplink transmission is allowed |
|  | DC_2A_n77A | - |  |  | No issues |
|  | DC_12A_n2A | 2nd harmonic from n2 into n77 | 3rd , 4th & 5th IMDs into n77 |  | - Harmonic problem already covered in DC_2A_n77A - 16.0dB MSD by 3rd IMD from CA_n2-n12-n77 can be reused |
|  | DC_12A_n77A | - | 3rd & 4th IMDs into B2 and n2 | - | - 16.5dB MSD by 3rd IMD from DC_2A-12A_n77A can be reused |
| DC_2-71_n2-n77 | DC_2A_n2A | 2nd harmonic from B2 into n77 | 2nd & 4th IMDs into n77 | - | - Harmonic problem already covered in DC_2A_n77A - Only switched uplink transmission is allowed |
|  | DC_2A_n77A |  |  |  | No issues |
|  | DC_71A_n2A | 2nd harmonic from n2 into n77 5th harmonic from B71 into n77 | 3rd, 4th & 5th IMDs into n77 | - | - Harmonic problem already covered in DC_2A_n77A or DC_71A_n77A - 8.0dB MSD |
|  | DC_71A_n77A | - | 3rd & 4th IMDs into B2and n2 | - | - 16.5dB MSD by 3rd IMD from DC_2A-71A_n78A can be reused |
| DC_2-5_n2-n78 | DC_2A_n2A | 2nd harmonic from B2 into n78 | 2nd & 4th IMDs into n78 | - | - Harmonic problem already covered in DC_2A_n78A - Only switched uplink transmission is allowed |
|  | DC_2A_n78A |  | 5th IMD into B5 | - | - 3.8 dB by 5th IMD from DC_2A-5A_n77A can be reused |
|  | DC_5A_n2A | 2nd harmonic from n2 into n78 4th harmonic from B5 into n78 | 3rd IMD into n78 | - | - Harmonic problem already covered in DC_2A_n78A or DC_5A_n78A - 16.0dB MSD by 3rd IMD from DC_2A_n5A-n77A can be reused |
|  | DC_5A_n78A |  | 3rd IMD into B2 and n2 |  | - 16.5dB MSD by 3rd IMD from DC_2A-5A_n77A can be reused |
| DC_2-66_n2-n78 | DC_2A_n2A | 2nd harmonic from B2 into n78 | 2nd & 4th IMDs into n78 | - | - Harmonic problem already covered in DC_2A_n78A - Only switched uplink transmission is allowed |
|  | DC_2A_n78A | - | 4th IMD into n66 | - | - 10.3 dB MSD by 4th IMD in DC_2A-66A_n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_66A_n2A | 2nd harmonic from n2 into n78 | 2nd & 4th IMDs into n78 |  | - Harmonic issue already covered in DC_2_n78 or CA_n66-n78 - 29.4 dB and 8.9 dB MSD by 2nd & 4th IMDs in DC_2A_n66A-n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_66A_n78A |  | 2nd, 4th & 5th IMDs into B2 and n2 |  | - 32.1dB MSD by 2nd IMD from DC_2A-66A_n78A can be reused |
| DC_38_n3-n78 | DC_38A_n3A | 2nd harmonic from n3 into n78 | 3rd IMD into n78 |  | - Harmonic issue already covered in DC_3_n78 - 16.1dB MSD by 3rd IMD from DC_3A_n7A-n78A can be reused |
|  | DC_38A_n78A |  | 3rd & 4th IMDs into n3 |  | - 17.6dB MSD by 3rd IMD from DC_3A-7A_n78A can be reused |
| DC_1_n38-n78 | DC_1A_n78A | - | 4th IMD into n38 |  | - 9.1dB MSD by 4th IMD from DC_1A-7A_n78A can be reused |
| DC_3_n38-n78 | DC_3A_n78A | - | - |  | No issues |
| DC_7_n38-n78 | - | - | - |  | No issues |
| DC_20_n38-n78 | DC_20A_n78A | 3rd harmonic from B20 into n38 | 2nd IMD into n38 | - | - Harmonic issue already covered in DC_20_n38 - 30.8dB MSD by 2nd IMD from DC_7A-20A_n78A can be reused |
| DC_1_n28-n75 | DC_1A_n28A | 2nd harmonic from n28 into n75 | - | - | - Harmonic issue already covered in DC_28_n75 |
| DC_3_n28-n75 | DC_3A_n28A | 2nd harmonic from n28 into n75 | 5th IMD into n75 | - | - Harmonic issue already covered in DC_28_n75 - FFS |
| DC_20_n8-n78 | DC_20A_n8A | 4th harmonic from B20 or n8 into n78 | 4th IMD into n78 |  | - Harmonic issue already covered in DC_20_n78 or DC_8_n78 - 10.3 dB reuse by 4th IMD of DC_8A_n28A-n78A |
|  | DC_20A_n78A | - | 4th IMD into n8 |  | - 12.1 dB reuse by 4th IMD of DC_8A-20A_n78A |
| DC_1_n3-n8 | DC_1A_n3A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_1A_n8A | 2nd harmonic from n8 into n3 | - | Yes | - Harmonic issue already covered in DC_8_n3A - Cross band isolation issues already solved |
| DC_1_n8-n77 DC_1_n8-n77(2A) DC_n8-n77_1 DC_n8-n77(2A)_1 | DC_1A_n8A or DC_n8A_1A |  | 3rd IMD into n77 | - | - 14.9 dB MSD by 3rd IMD of DC_1A_n8A-n78A can be reused. |
|  | DC_1A_n77A or DC_n77A_1A | - | 5th IMD into n8 | - | - 3.3dB MSD by 5th IMD of DC_1A-8A_n77A can be reused. |
| DC_1-3_n3-n8 | DC_1A_n3A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_1A_n8A | 2nd harmonic from n8 into n3 | - | Yes | - Harmonic issue already covered in DC_8_n3A - Cross band isolation issues already solved |
|  | DC_(n)3AA | - | - | - | No issues |
|  | DC_3A_n8A | - | - | - | No issues |
| DC_3_n3-n77 DC_3_n3-n77(2A) DC_n3-n77_3 DC_n3-n77(2A)_3 | DC_(n)3AA | 2nd harmonic from n3 into n77 | 2nd & 4th IMD into n77 | - | - Harmonic problems was already covered in DC_3A_n77A. - Due to apply MPR, no need to define MSD |
|  | DC_3A_n77A DC_n77A_3A | - | 2nd, 4th &5th IMD into n3 | - | - MSD problems in n3 were already solved in DC_3A_n77A |
| DC_3_n3-n8 DC_n3-n8_3 | DC_(n)3AA | - | - | - | No issues |
|  | DC_3A_n8A or DC_n8A_3A | - | - | - | No issues |
| DC_3_n8-n77 DC_3_n8-n77(2A) DC_n8-n77_3 DC_n8-n77(2A)_3 | DC_3A_n8A or DC_n8A_3A | 2nd harmonic from B3 into n77 4th harmonic from n8 into n77 | 3rd & 5th IMDs into n77 |  | - Harmonic problems was already covered in DC_3A_n77A or DC_8A_n77A. - 16.3dB MSD by 3rd IMD in DC_3A_n8A-n78A can be reused |
|  | DC_3A_n77A or DC_n77A_3A | - | 4th IMD into n8 |  | - 9.7 dB MSD by 4th IMD in DC_3A-8A_n77A can be reused |
| DC_8-3_n3-n77 DC_8-3_n3-n77(2A) | DC_(n)3AA | 2nd harmonic from B3 into n77 | 2nd & 4th IMDs into n77 |  | - Harmonic problems was already covered in DC_3A_n77A - Due to apply MPR, no need to define MSD |
|  | DC_3A_n77A | - | 4th IMD into n8 |  | - 9.7 dB MSD by 4th IMD in DC_3A-8A_n77A can be reused |
|  | DC_8A_n3A | 2nd harmonic from B3 into n77 4th harmonic from n8 into n77 | 3rd & 5th IMDs into n77 |  | - Harmonic problems was already covered in DC_3A_n77A or DC_8A_n77A. - 16.3dB MSD by 3rd IMD in DC_3A_n8A-n78A can be reused |
|  | DC_8A_n77A | 2nd harmonic from B8 into B3 | 3rd IMD into B3 and n3 |  | - Harmonic problems was already covered in DC_3A_n8A - 16.5dB MSD by 3rd IMD in DC_3A-8A_n77A can be reused |
| DC_1-3_n3-n257A/G/H/I/J/K/L/M | DC_1A_n3A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_1A_n257A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_(n)3AA | - | - | - | No issues |
|  | DC_3A_n257A | - | - | - | No issues |
| DC_1_n77-n257J/K/L/M DC_1_n77(2A)-n257J/K/L/M DC_n77-n257_1 DC_n77(2A)-n257_1 | DC_1A_n77A or DC_n77A_1A | - | - | - | No issues |
|  | DC_1A_n257A or DC_n257A_1A | - | - | - | No issues |
| DC_1_n8-n257A/G/H/I/J/K/L/M DC_n8-n257_1 | DC_1A_n8A or DC_n8A_1A | - | - | - | No issues |
|  | DC_1A_n257A or DC_n257A_1A | - | - | - | No issues |
| DC_3_n3-n257A/G/H/I/J/K/L/M DC_n3-n257_3 | DC_(n)3AA | - | - | - | No issues |
|  | DC_3A_n257A or DC_n257A_3A | - | - | - | No issues |
| DC_3_n77-n257J/K/L/M, DC_3_n77(2A)-n257J/K/L/M DC_n77-n257_3 DC_n77(2A)-n257_3 | DC_3A_n77A or DC_n77A_3A | - | - | - | No issues |
|  | DC_3A_n257A or DC_n257A_3A | - | - | - | No issues |
| DC_3_n8-n257A/G/H/I/J/K/L/M DC_n8-n257_3 | DC_3A_n8A or DC_n8A_3A | - | - | - | No issues |
|  | DC_3A_n257A or DC_n257A_3A | - | - | - | No issues |
| DC_8_n3-n257A/G/H/I/J/K/L/M | DC_8A_n3A | - | - | - | No issues |
|  | DC_8A_n257A | 2nd harmonic from B8 into n3 | - | - | - Harmonic problems was already covered in DC_3A_n8A |
| DC_8-3_n3-n257A/G/H/I/J/K/L/M | DC_8A_n3A | - | - | - | No issues |
|  | DC_8A_n257A | 2nd harmonic from B8 into n3 | - | - | - Harmonic problems was already covered in DC_3A_n8A |
|  | DC_(n)3AA | - | - | - | No issues |
|  | DC_3A_n257A | - | - | - | No issues |
| DC_n3-n257A/G/H/I/J/K/L/M_1-3 | DC_n3A_1A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_(n)3AA | - | - | - | No issues |
|  | DC_n257A_1A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_n257A_3A | - | - | - | No issues |
| DC_n3-n77_1-3 DC_n3-n77(2A)_1-3 | DC_n3A_1A | 2nd harmonic from B1 or n3 into n77 | 2nd & 4th IMDs into n77 | Yes | - Harmonic problems already covered in DC_n3A-n78A. - Same as IMD issues in DC_1A_n3A-n78A - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_(n)3AA | 2nd harmonic from B3 into n77 | 2nd & 4th IMDs into n77 |  | - Harmonic problems was already covered in DC_3A_n77A - Due to apply MPR, no need to define MSD |
|  | DC_n77A_1A | - | 2nd , 4th & 5th IMDs into B3/n3 | - | - Same as IMD issues in DC_1A_n3A-n78A |
|  | DC_n77A_3A | - | 2nd & 5th IMD into B1 2nd, 4th &5th IMD into n3 | - | - 31.0 dB by 2nd IMD that follow DC_1A-3A_n77A in TR37.863-02-01 in rel-15. - Follow DC_3A_n1A-n78A for 5th IMD problem. - MSD problems in n3 were already solved in DC_3A_n77A |
| DC_n3-n8_1 | DC_n3A_1A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_n8A_1A | 2nd harmonic from n8 into n3 | - | Yes | - Harmonic issue already covered in DC_8_n3A - Cross band isolation issues already solved |
| DC_n3-n8_1-3 | DC_n3A_1A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_(n)3AA | - | - | - | No issues |
|  | DC_n8A_1A | 2nd harmonic from n8 into n3 | - | Yes | - Harmonic issue already covered in DC_8_n3A - Cross band isolation issues already solved |
|  | DC_n8A_3A | - | - | - | No issues |

For the high order xDL/2UL (x=4,5,6) DC combinations, the self-interference problems will be covered in 3DL/2UL or lower order DC operation. Hence, RAN4 do not make summary tables for high order xDL/2UL DC band combinations.

Based on these assumptions, we proposed the MSD test configuration and required MSD levels by self-interference as below.

Table 5.3-2: Proposed MSD levels and MSD test configuration for 3DL/2UL DC UE

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_40A_n1A-n78A DC_40C_n1A-n78A | B40 | IMD4 | ¦2*fB40 -2*fn78¦ | 2320 | 5 | 25 | 2320 | 5 | N/A |
|  | n78 |  |  | 3380 | 10 | 50 | 3380 | 10 |  |
|  | n1 |  |  | 1930 | 5 | 25 | 2120 | 5 | 7.6 |
| DC_7A_n40A-n78A | B7 | IMD4 | ¦2*fB7 -2*fn78¦ | 2520 | 10 | 50 | 2640 | 10 | N/A |
|  | n78 |  |  | 3700 | 10 | 50 | 3700 | 10 |  |
|  | n40 |  |  | 2360 | 5 | 25 | 2360 | 5 | 9.2 |
| DC_8A_n40A-n78A | 8 | IMD2 | ¦fB8 +fn40¦ | 910 | 5 | 25 | 955 | 5 | N/A |
|  | n40 |  |  | 2395 | 5 | 25 | 2395 | 5 |  |
|  | n78 |  |  | 3305 | 10 | 50 | 3305 | 10 | 28.7 |
|  | 8 | IMD3 | ¦fB8 -2*fn40¦ | 910 | 5 | 25 | 955 | 5 | N/A |
|  | n40 |  |  | 2340 | 5 | 25 | 2340 | 5 |  |
|  | n78 |  |  | 3370 | 10 | 50 | 3370 | 10 | 16.9 |
|  | 8 | IMD2 | ¦fB8 -fn78¦ | 910 | 5 | 25 | 955 | 5 | N/A |
|  | n78 |  |  | 3305 | 10 | 50 | 3305 | 10 |  |
|  | n40 |  |  | 2395 | 5 | 25 | 2395 | 5 | 27.3 |
| DC_11A_n28A-n77A DC_11A_n28A-n77(2A) | 11 | IMD3 | ¦2*fB11 +fn28¦ | 1443 | 5 | 25 | 1491 | 5 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 |  |
|  | n77 |  |  | 3629 | 10 | 50 | 3629 | 10 | 17.5 |
|  | 11 | IMD4 | ¦3*fB11 -fn28¦ | 1443 | 5 | 25 | 1491 | 5 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 |  |
|  | n77 |  |  | 3586 | 10 | 50 | 3586 | 10 | 11.2 |
|  | 11 | IMD3 | ¦2*fB11 - fn77¦ | 1443 | 5 | 25 | 1491 | 5 | N/A |
|  | n77 |  |  | 3684 | 10 | 50 | 3684 | 10 |  |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 | 15.8 |
|  | 11 | IMD4 | ¦3*fB11 -fn77¦ | 1443 | 5 | 25 | 1491 | 5 | N/A |
|  | n77 |  |  | 3564 | 10 | 50 | 3564 | 10 |  |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 | 11.0 |
| DC_2A_n5A-n77A DC_2A-2A_n5A-n77A | 2 | IMD3 | ¦fB2 +2*fn5¦ | 1880 | 5 | 25 | 1960 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 3540 | 10 | 50 | 3540 | 10 | 16.0 |
|  | 2 | IMD5 | ¦3*fB2 -2*fn5¦ | 1880 | 5 | 25 | 1960 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 3980 | 10 | 50 | 3980 | 10 | 4.1 |
|  | 2 | IMD5 | ¦3*fB2 -2*fn77¦ | 1907 | 5 | 25 | 1987 | 5 | N/A |
|  | n77 |  |  | 3305 | 10 | 50 | 3305 | 10 |  |
|  | n5 |  |  | 844 | 5 | 25 | 889 | 5 | 3.8 |
| DC_13A_n2A-n77A | 13 | IMD3 | ¦2*fB13 +fn2¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n2 |  |  | 1880 | 5 | 25 | 1960 | 5 |  |
|  | n77 |  |  | 3444 | 10 | 50 | 3444 | 10 | 17.3 |
|  | 13 | IMD4 | ¦3*fB13 +fn2¦ | 780 | 5 | 25 | 749 | 5 | N/A |
|  | n2 |  |  | 1855 | 5 | 25 | 1935 | 5 |  |
|  | n77 |  |  | 4195 | 10 | 50 | 4195 | 10 | 12.4 |
|  | 13 | IMD5 | ¦2*fB13 -3*fn2¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n2 |  |  | 1880 | 5 | 25 | 1960 | 5 |  |
|  | n77 |  |  | 4076 | 10 | 50 | 4076 | 10 | 4.5 |
|  | 13 | IMD3 | ¦2*fB13 -fn77¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n77 |  |  | 3524 | 10 | 50 | 3524 | 10 |  |
|  | n2 |  |  | 1880 | 5 | 25 | 1960 | 5 | 16.0 |
| DC_66A_n5A-n77A DC_66A-66A_n5A-n77A | 66 | IMD3 | ¦fB66 +2*fn5¦ | 1760 | 5 | 25 | 2160 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 3420 | 10 | 50 | 3420 | 10 | 16.6 |
|  | 66 | IMD4 | ¦fB66 +3*fn5¦ | 1714 | 5 | 25 | 2114 | 5 | N/A |
|  | n5 |  |  | 827 | 5 | 25 | 872 | 5 |  |
|  | n77 |  |  | 4195 | 10 | 50 | 4195 | 10 | 12.1 |
|  | 66 | IMD5 | ¦3*fB66 -2*fn5¦ | 1760 | 5 | 25 | 2160 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 3620 | 10 | 50 | 3620 | 10 | 3.6 |
| DC_18A_n28A-n77A DC_18A_n28A-n78A | 18 | IMD5 | ¦2*fB18 +3*fn28¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 |  |
|  | n77/n78 |  |  | 3770 | 10 | 50 | 3770 | 10 | 4.0 |
| DC_18A_n28A-n77A | 18 | IMD5 | ¦4*fB18 -fn77¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n77 |  |  | 4045 | 10 | 50 | 4045 | 10 |  |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 | 5.7 |
| DC_18A_n3A-n41A | 18 | IMD2 | ¦fB18 +fn3¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n3 |  |  | 1720 | 5 | 25 | 1815 | 5 |  |
|  | n41 |  |  | 2540 | 5 | 25 | 2540 | 5 | 29.4 |
|  | 18 | IMD2 | ¦fB18 -fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2655 | 5 | 25 | 2655 | 5 |  |
|  | n3 |  |  | 1740 | 5 | 25 | 1835 | 5 | 28.2 |
| DC_18A_n28A-n41A | 18 | IMD5 | ¦4*fB18 -fn28¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 |  |
|  | n41 |  |  | 2570 | 5 | 25 | 2570 | 5 | 5.1 |
|  | 18 | IMD5 | ¦4*fB18 -fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2515 | 5 | 25 | 2515 | 5 |  |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 | 4.7 |
| DC_18A_n41A-n77A DC_18A_n41A-n78A | 18 | IMD2 | ¦fB18 +fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2570 | 5 | 25 | 2570 | 5 |  |
|  | n77/n78 |  |  | 3390 | 10 | 50 | 3390 | 10 | 30.1 |
|  | 18 | IMD3 | ¦2*fB18 +fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2515 | 5 | 25 | 2515 | 5 |  |
|  | n77 |  |  | 4155 | 10 | 50 | 4155 | 10 | 16.4 |
|  | 18 | IMD4 | ¦2*fB18 -2*fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2515 | 5 | 25 | 2515 | 5 |  |
|  | n77/n78 |  |  | 3390 | 10 | 50 | 3390 | 10 | 9.1 |
|  | 18 | IMD2 | ¦fB18 -fn78¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n77/n78 |  |  | 3450 | 10 | 50 | 3450 | 10 |  |
|  | n41 |  |  | 2630 | 5 | 25 | 2630 | 5 | 28.5 |
| DC_12A_n5A-n77A | 12 | IMD5 | ¦4*fB12 +fn5¦ | 708 | 5 | 25 | 738 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 3662 | 10 | 50 | 3662 | 10 | 5.4 |
|  | 12 | IMD5 | ¦4*fB12 -fn77¦ | 708 | 5 | 25 | 738 | 5 | N/A |
|  | n77 |  |  | 3707 | 10 | 50 | 3707 | 10 |  |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 | 4.8 |
| DC_14A_n5A-n77A | 14 | IMD4 | ¦fB14 +3*fn5¦ | 793 | 5 | 25 | 763 | 5 | N/A |
|  | n5 |  |  | 845 | 5 | 25 | 890 | 5 |  |
|  | n77 |  |  | 3328 | 10 | 50 | 3328 | 10 | 10.6 |
|  | 14 | IMD5 | ¦fB14 +4*fn5¦ | 793 | 5 | 25 | 763 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 4113 | 10 | 50 | 4113 | 10 | 5.1 |
|  | 14 | IMD5 | ¦4*fB14 -fn77¦ | 793 | 5 | 25 | 763 | 5 | N/A |
|  | n77 |  |  | 4047 | 10 | 50 | 4047 | 10 |  |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 | 4.4 |
| DC_2A_n71A-n78A or DC_71A_n2A-n78A | 2/n2 | IMD3 | ¦fB2 +2*fn71¦ | 1907.5 | 5 | 25 | 1987.5 | 5 | N/A |
|  | 71/n71 |  |  | 695.5 | 5 | 25 | 649.5 | 5 |  |
|  | n78 |  |  | 3305 | 10 | 50 | 3305 | 10 | 8.0 |
| DC_7A_n71A-n78A | 7 | IMD4 | ¦2*fB7 -2*fn71¦ | 2550 | 5 | 25 | 2670 | 5 | N/A |
|  | n71 |  |  | 693 | 5 | 25 | 647 | 5 |  |
|  | n78 |  |  | 3714 | 10 | 50 | 3714 | 10 | 9.7 |
|  | 7 | IMD5 | ¦3*fB7 -2*fn78¦ | 2555 | 5 | 25 | 2675 | 5 | N/A |
|  | n78 |  |  | 3520 | 10 | 50 | 3520 | 10 |  |
|  | n71 |  |  | 671 | 5 | 25 | 625 | 5 | 3.9 |
| DC_71A_n38A-n78A | 71 | IMD2 | ¦fB71 +fn38¦ | 693 | 5 | 25 | 647 | 5 | N/A |
|  | n38 |  |  | 2615 | 5 | 25 | 2615 | 5 |  |
|  | n78 |  |  | 3308 | 10 | 50 | 3308 | 10 | 29.1 |
|  | 71 | IMD4 | ¦2*fB71 -2*fn38¦ | 693 | 5 | 25 | 647 | 5 | N/A |
|  | n38 |  |  | 2580 | 5 | 25 | 2580 | 5 |  |
|  | n78 |  |  | 3774 | 10 | 50 | 3774 | 10 | 10.3 |
|  | 71 | IMD2 | ¦fB71 -fn78¦ | 693 | 5 | 25 | 647 | 5 | N/A |
|  | n78 |  |  | 3308 | 10 | 50 | 3308 | 10 |  |
|  | n38 |  |  | 2615 | 5 | 25 | 2615 | 5 | 28.7 |
| DC_5A_n66A-n78A | 5 | IMD3 | ¦fn66 +2*fB5¦ | 830 | 5 | 25 | 875 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n78 |  |  | 3420 | 10 | 50 | 3420 | 10 | 16.6 |
|  | 5 | IMD5 | ¦3*fn66 -2*fB5¦ | 830 | 5 | 25 | 875 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n78 |  |  | 3620 | 10 | 50 | 3620 | 10 | 3.6 |
| DC_71A_n66A-n78A | 71 | IMD3 | ¦2*fB71 -fn78¦ | 693 | 5 | 25 | 647 | 5 | N/A |
|  | n78 |  |  | 3546 | 10 | 50 | 3546 | 10 |  |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 | 15.5 |
| DC_5A_n38A-n66A | 5 | IMD2 | ¦fB5 +fn66¦ | 830 | 5 | 25 | 875 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n38 |  |  | 2590 | 5 | 25 | 2590 | 5 | 28.9 |
|  | 5 | IMD3 | ¦fB5 -2*fn66¦ | 830 | 5 | 25 | 875 | 5 | N/A |
|  | n66 |  |  | 1720 | 5 | 25 | 2120 | 5 |  |
|  | n38 |  |  | 2610 | 5 | 25 | 2610 | 5 | 16.4 |
| DC_12A_n2A-n38A or DC_12A_n2A-n41A | 12 | IMD2 | ¦fn2 +fB12¦ | 708 | 5 | 25 | 738 | 5 | N/A |
|  | n2 |  |  | 1900 | 5 | 25 | 1980 | 5 |  |
|  | n38/n41 |  |  | 2608 | 5 | 25 | 2608 | 5 | 28.7 |
| DC_2A_n66A-n77A | 2 | IMD2 | ¦fB2 -fn77¦ | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n77 |  |  | 4060 | 10 | 50 | 4060 | 10 |  |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 | 29.2 |
|  | 2 | IMD4 | ¦3*fB2 -fn77¦ | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n77 |  |  | 3540 | 10 | 50 | 3540 | 10 |  |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 | 10.4 |
|  | 2 | IMD5 | ¦3*fB2 -2*fn77¦ | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n77 |  |  | 3930 | 10 | 50 | 3930 | 10 |  |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 | 4.0 |
| DC_13A_n66A-n77A | 13 | IMD3 | ¦2*fB13 +fn66¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n77 |  |  | 3324 | 10 | 50 | 3324 | 10 | 16.4 |
|  | 13 | IMD4 | ¦3*fB13 +fn66¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n77 |  |  | 4106 | 10 | 50 | 4106 | 10 | 11.7 |
|  | 13 | IMD5 | ¦2*fB13 -3*fn66¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n77 |  |  | 3716 | 10 | 50 | 3716 | 10 | 4.4 |
| DC_11A_n3A-n77A | 11 | IMD4 | ¦fB11 -3*fn3¦ | 1440 | 5 | 25 | 1488 | 5 | N/A |
|  | n3 |  |  | 1740 | 5 | 25 | 1835 | 5 |  |
|  | n77 |  |  | 3780 | 10 | 50 | 3780 | 10 | 10.8 |
|  | 11 | IMD5 | ¦4*fB11 -fn3¦ | 1440 | 5 | 25 | 1488 | 5 | N/A |
|  | n3 |  |  | 1740 | 5 | 25 | 1835 | 5 |  |
|  | n77 |  |  | 4020 | 10 | 50 | 4020 | 10 | 4.1 |
|  | 11 | IMD2 | ¦fB11 -fn77¦ | 1440 | 5 | 25 | 1488 | 5 | N/A |
|  | n77 |  |  | 3310 | 10 | 50 | 3310 | 10 |  |
|  | n3 |  |  | 1775 | 5 | 25 | 1870 | 5 | 29.0 |
|  | 11 | IMD5 | ¦4*fB11 -fn77¦ | 1440 | 5 | 25 | 1488 | 5 | N/A |
|  | n77 |  |  | 3925 | 10 | 50 | 3925 | 10 |  |
|  | n3 |  |  | 1740 | 5 | 25 | 1835 | 5 | 3.7 |
| DC_1A_n28A-n79A | 1 | IMD3 | ¦2*fB1 -fn79¦ | 1930 | 5 | 25 | 2120 | 5 | N/A |
|  | n28 |  |  | 733 | 5 | 25 | 788 | 5 | 15.2 |
|  | n79 |  |  | 4648 | 40 | 216 | 4648 | 40 | N/A |
|  | 1 | IMD3 | ¦2*fB1 +fn28¦ | 1950 | 5 | 25 | 2140 | 5 | N/A |
|  | n28 |  |  | 730 | 5 | 25 | 785 | 5 |  |
|  | n79 |  |  | 4630 | 40 | 216 | 4630 | 40 | 14.9 |
| DC_3A_n28A-n79A | 3 | IMD4 | ¦3*fB3 -fn79¦ | 1770 | 5 | 25 | 1865 | 5 | N/A |
|  | n28 |  |  | 725 | 5 | 25 | 780 | 5 | 10.3 |
|  | n79 |  |  | 4530 | 40 | 216 | 4530 | 40 | N/A |
|  | 3 | IMD4 | ¦3*fB3 -fn28¦ | 1770 | 5 | 25 | 1865 | 5 | N/A |
|  | n28 |  |  | 725 | 5 | 25 | 780 | 5 |  |
|  | n79 |  |  | 4585 | 40 | 216 | 4585 | 40 | 9.4 |
| DC_21A_n28A-n77A DC_21A_n28A-n78A | 21 | IMD3 | ¦2*fB21 -fn78¦ | 1452 | 5 | 25 | 1500 | 5 | N/A |
|  | n28 |  |  | 730.5 | 5 | 25 | 785.5 | 5 | 16.9 |
|  | n77/n78 |  |  | 3689.5 | 10 | 50 | 3689.5 | 10 | N/A |
|  | 21 | IMD3 | ¦2*fB21 +fn28¦ | 1452 | 5 | 25 | 1500 | 5 | N/A |
|  | n28 |  |  | 730.5 | 5 | 25 | 785.5 | 5 |  |
|  | n77/n78 |  |  | 3634.5 | 10 | 50 | 3634.5 | 10 | 17.3 |
| DC_21A_n28A-n79A | 21 | IMD5 | ¦4*fB21 -fn79¦ | 1450.4 | 5 | 25 | 1498.4 | 5 | N/A |
|  | n28 |  |  | 735.5 | 5 | 25 | 790.5 | 5 | 2.8 |
|  | n79 |  |  | 4980 | 40 | 216 | 4980 | 40 | N/A |
|  | 21 | IMD4 | ¦2*fB21 +2*fn28¦ | 1460.4 | 5 | 25 | 1508.4 | 5 | N/A |
|  | n28 |  |  | 745 | 5 | 25 | 800 | 5 |  |
|  | n79 |  |  | 4420 | 40 | 216 | 4420 | 40 | [6.3] |
| DC_13A_n5A-n77A | 13 | IMD5 | ¦4*fB13 -fn77¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n77 |  |  | 4013 | 10 | 50 | 4013 | 10 |  |
|  | n5 |  |  | 840 | 5 | 25 | 885 | 5 | 4.5 |

6	Dual Connectivity band combinations of LTE 1 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

6.1	DC_3_n1-n257, DC_3-3_n1-n257

6.1.1	Operating bands for DC

Table 6.1.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n1-n257 DC_3-3_n1-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.1.2	Channel bandwidths per operating band for DC

Table 6.1.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_3A_n1A-n257D DC_3A_n1A-n257E DC_3A_n1A-n257F DC_3A_n1A-n257G DC_3A_n1A-n257H DC_3A_n1A-n257I DC_3A_n1A-n257J DC_3A_n1A-n257K DC_3A_n1A-n257L DC_3A_n1A-n257M | DC_3A_n1A DC_3A_n257A | 3 | -- | 440 640 840 240 340 440 540 640 740 840 |
|  |  | n1 | See CA_n1A-n257D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |
| DC_3A-3A_n1A-n257D DC_3A-3A_n1A-n257E DC_3A-3A_n1A-n257F DC_3A-3A_n1A-n257G DC_3A-3A_n1A-n257H DC_3A-3A_n1A-n257I DC_3A-3A_n1A-n257J DC_3A-3A_n1A-n257K DC_3A-3A_n1A-n257L DC_3A-3A_n1A-n257M | DC_3A_n1A DC_3A_n257A | 3 | See CA_3A-3A in TS 36.101 | 460 660 860 260 360 460 560 660 760 860 |
|  |  | n1 | See CA_n1A-n257D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |

6.1.3	Co-existence studies

Based on co-existence studies of DC_3A_n1A and DC_3A_n257A specified in TR 37.716-11-11, TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.1.4	∆TIB and ∆RIB values

For DC_3_n1-n257, DC_3-3_n1-n257, TIB,c and RIB,c values are already mentioned in 37.716-21-21 under the same band combinations with n257A.

6.1.5	MSD

No additional MSD requirement is needed.

6.2	DC_7_n1-n257, DC_7-7_n1-n257

6.2.1	Operating bands for DC

Table 6.2.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n1-n257 DC_7-7_n1-n257 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.2.2	Channel bandwidths per operating band for DC

Table 6.2.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_7A_n1A-n257D DC_7A_n1A-n257E DC_7A_n1A-n257F DC_7A_n1A-n257G DC_7A_n1A-n257H DC_7A_n1A-n257I DC_7A_n1A-n257J DC_7A_n1A-n257K DC_7A_n1A-n257L DC_7A_n1A-n257M | DC_7A_n1A DC_7A_n257A | 7 | -- | 440 640 840 240 340 440 540 640 740 840 |
|  |  | n1 | See CA_n1A-n257D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |
| DC_7A-7A_n1A-n257D DC_7A-7A_n1A-n257E DC_7A-7A_n1A-n257F DC_7A-7A_n1A-n257G DC_7A-7A_n1A-n257H DC_7A-7A_n1A-n257I DC_7A-7A_n1A-n257J DC_7A-7A_n1A-n257K DC_7A-7A_n1A-n257L DC_7A-7A_n1A-n257M | DC_7A_n1A DC_7A_n257A | 3 | See CA_7A-7A in TS 36.101 | 460 660 860 260 360 460 560 660 760 860 |
|  |  | n1 | See CA_n1A-n257D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |

6.2.3	Co-existence studies

Based on co-existence studies of DC_7A_n1A and DC_7A_n257A specified in TR 37.716-11-11, TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.2.4	∆TIB and ∆RIB values

For DC_7_n1-n257, DC_7-7_n1-n257, TIB,c and RIB,c values are already mentioned in 37.716-21-21 under the same band combinations with n257A.

6.2.5	MSD

No additional MSD requirement is needed.

6.3	DC_11_n3-n28

6.3.1	Operating bands for DC

Table 6.3.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_11_n3-n28 | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

6.3.2	Channel bandwidths per operating band for DC

Table 6.3.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_11A_n3A-n28A | DC_11A_n3A DC_11A_n28A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 50 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.3.3	Co-existence studies

Based on the co-existence studies of DC_11A-n3A and DC_11A-n28A captured in TR 37.716-11-11, 5th order IMD generated by dual uplink of Band 11 + Band n3 may also fall into own Rx of band n28.

6.3.4	∆TIB and ∆RIB values

For DC_11_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.3.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11_n3-n28 | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n28 | 0.6 |

Table 6.3.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11_n3-n28 | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n28 | 0.2 |

6.3.5	MSD

As mentioned in 6.3.3, IMD5 of B11 and n3 to Band n28 Rx need to be addressed for REFSENS relaxation. Based on the relaxation value of CA_3-21-28, The following value is proposed:

Table 6.3.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_11A-n3A_n28A | 11 | 1435 | 5 | 25 | 1483 | N/A | FDD | N/A |
|  | n3 | 1753 | 5 | 25 | 1848 | N/A | TDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 3.0 | FDD | IMD5 |

6.4	DC_1_n28-n41

6.4.1	Operating bands for DC

Table 6.4.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A_n28A-n41A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.4.2	Channel bandwidths per operating band for DC

Table 6.4.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A_n28A-n41A | DC_1A_n28A DC_1A_n41A | 1 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n28 |  | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

6.4.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_1A_n28A-n41A with dual uplink on DC_1A_n28A and DC_1A_n41A were calculated and listed in Table 6.4.3-1 and 6.4.3-2 respectively.

Table 6.4.3-1: Harmonic and IMD analysis for DC_1_n28-n41 with UL on DC_1_n28

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2109 | 2244 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1172 | 1277 | 2623 | 2728 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3092 | 3257 | 424 | 574 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4543 | 4708 | 3326 | 3476 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5012 | 5237 | 129 | 324 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6463 | 6688 | 4029 | 4224 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2344 | 2554 | 5246 | 5456 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 832 | 1072 | 6932 | 7217 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4732 | 4972 | 8383 | 8668 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1596 | 1851 | 4264 | 4534 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5949 | 6204 | 7166 | 7436 |

Table 6.4.3-2: Harmonic and IMD analysis for DC_1_n28-n41 with UL on DC_1_n41

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7488 | 8070 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 516 | 770 | 4416 | 4670 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1150 | 1464 | 3012 | 3460 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6336 | 6650 | 6912 | 7360 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3070 | 3444 | 5508 | 6150 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8256 | 8630 | 9408 | 10050 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1032 | 1540 | 8832 | 9340 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8004 | 8840 | 4990 | 5424 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11904 | 12740 | 10176 | 10610 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3528 | 4230 | 380 | 948 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11328 | 12030 | 10752 | 11320 |

Based on Table 6.4.3-1 and Table 6.4.3-2:

- 2nd and 4th order IMD generated by dual uplink of Band 1 + Band n28 may also fall into own Rx of band n41, it could reuse the MSD value of DC_1A-41A_n28A

- 2nd and 5th order IMD generated by dual uplink of Band 1 + Band n41 may also fall into own Rx of band n28.

6.4.4	∆TIB and ∆RIB values

For DC_1A_n28A-n41A, the TIB,c and RIB,c values can reuse the values of DC_1A-41A_n28A as given in the tables below.

Table 6.4.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n28-n41 | 1 | 0.5 |
|  | n28 | 0.5 |
|  | n41 | 0.5 |

Table 6.4.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1_n28-n41 | 1 | 0 |
|  | n28 | 0.2 |
|  | n41 | 0 |

6.4.5	MSD

| Table 6.4.5-1 shows the required MSD:Table 6.4.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_1A_n28A-n41ADC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n28A-n41A | 1 | IMD2 | ¦fB1+fBn28¦ | 1935 | 5 | 25 | 2125 | N/A |
|  | n28 |  |  | 718 | 5 | 25 | 773 | N/A |
|  | n41 |  |  | 2653 | 10 | 50 | 2653 | 30.1 |
|  | 1 | IMD2 | ¦fBn41 -fB1¦ | 1923 | 5 | 25 | 2113 | N/A |
|  | n41 |  |  | 2685 | 10 | 50 | 2685 |  |
|  | n28 |  |  | 707 | 5 | 25 | 762 | 29.3 |
|  | 1 | IMD5 | ¦2*fBn41 -3*fB1¦ | 1935 | 5 | 25 | 2125 | N/A |
|  | n41 |  |  | 2510 | 10 | 50 | 2510 |  |
|  | n28 |  |  | 730 | 10 | 50 | 785 | 4.5 |
| NOTE 1: This band is subject to IMD4 also which MSD is not specified |  |  |  |  |  |  |  |  |

6.5	DC_3_n28-n41

6.5.1	Operating bands for DC

Table 6.5.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n28A-n41A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.5.2	Channel bandwidths per operating band for DC

Table 6.5.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A_n28A-n41A | DC_3A_n28A DC_3A_n41A | 3 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n28 |  | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

6.5.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_3A_n28A-n41A with dual uplink on DC_3A_n28A and DC_3A_n41A were calculated and listed in Table 6.5.3-1 and 6.5.3-2 respectively.

Table 6.5.3-1: Harmonic and IMD analysis for DC_3_n28-n41 with UL on DC_3_n28

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2109 | 2244 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 962 | 1082 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2672 | 2867 | 214 | 379 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4123 | 4318 | 3116 | 3281 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4382 | 4652 | 324 | 534 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5833 | 6103 | 3819 | 4029 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1924 | 2164 | 4826 | 5066 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1027 | 1282 | 6092 | 6437 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4522 | 4777 | 7543 | 7888 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1176 | 1461 | 3634 | 3949 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5529 | 5814 | 6536 | 6851 |

Table 6.5.3-2: Harmonic and IMD analysis for DC_3_n28-n41 with UL on DC_3_n41

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7488 | 8070 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 711 | 980 | 4206 | 4475 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 | 1074 | 3207 | 3670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6260 | 6702 | 7165 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2440 | 2859 | 5703 | 6360 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7626 | 8045 | 9198 | 9855 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1422 | 1960 | 8412 | 8950 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8199 | 9050 | 4150 | 4644 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 | 12545 | 9336 | 9830 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3918 | 4650 | 250 | 363 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 | 11640 | 10122 | 10735 |

Based on Table 6.5.3-1 and Table 6.5.3-2:

- 2nd and 3rd order IMD generated by dual uplink of Band 3 + Band n28 may also fall into own Rx of band n41, it could reuse the MSD value of DC_3A-41A_n28A

- 2nd and 3rd order IMD generated by dual uplink of Band 3 + Band n41 may also fall into own Rx of band n28.

6.5.4	∆TIB and ∆RIB values

For DC_3A_n28A-n41A, the TIB,c and RIB,c values can reuse the values of DC_3A-41A_n28A as given in the tables below.

Table 6.5.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n28-n41 | 3 | 0.5 |
|  | n28 | 0.3 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 6.5.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3_n28-n41 | 3 | 0 |
|  | n28 | 0 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

6.5.5	MSD

Table 6.5.5-1 shows the required MSD:

Table 6.5.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_3A_n28A-n41A

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A_n28A-n41A | 3 | 1720 | 5 | 25 | 1815 | N/A | TDD | N/A |
|  | n28 | 735 | 5 | 25 | 790 | 261 | FDD | IMD2 ¦fn41-fB3¦ |
|  | n41 | 2510 | 5 | 25 | 2510 | N/A | FDD | N/A |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n41 | 2518 | 5 | 25 | 2518 | 27.4 | TDD | IMD2 ¦fB3+fn28¦ |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2687 | 5 | 25 | 2687 | 15.9 | TDD | IMD3 ¦2*fB3-fn28¦ |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified |  |  |  |  |  |  |  |  |

6.6	DC_2_n48-n66

6.6.1	Operating bands for DC

Table 6.6.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n48-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.6.2	Configuration for DC

Table 6.6.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A_n48A-n66A | DC_2A_n48A DC_2A_n66A | 2A | CA_n48A-n66A |

6.6.3	Co-existence studies

Co-existence analysis for DC_2_n48 UL shows no impact to NR Band n66 DL.

Co-existence analysis for DC_2_n66 UL shows that 2nd HAM of band 66 may fall into the lowest 10MHz of NR Band n48 DL. This is already covered in Table 7.3B.2.3.1-1 and Table 7.3B.2.3.1-2 in 38.101-3. Further 2nd IMD might also fall in n48 DL.

6.6.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below. Values from DC_2-48_n66 already defined in 38.101-3 is reused.

Table 6.6.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n48-n66 | 2 | 0.6 |
|  | n48 | 0.8 |
|  | n66 | 0.6 |

Table 6.6.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n48-n66 | 2 | 0.3 |
|  | n48 | 0.4 |
|  | n66 | 0.3 |

6.6.5	MSD

Based on co-existence studies additional MSD is needed to be defined due to 2nd IMD. Similar values as for DC_2A-48A_n66A already defined in 38101-3 is proposed.

Table 6.6.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n48A-n66A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n48 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |

6.7	DC_48_n25-n48

6.7.1	Operating bands for DC

Table 6.7.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_48_n25-n48 | 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

6.7.2	Configuration for DC

Table 6.7.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_48A_n25A-n48A | DC_48A_n25A | 48A | CA_n25A-n48A |

6.7.3	Co-existence studies

Co-existence analysis for DC_48_n25 UL shows that 4th IMD may fall into NR Band n48 DL. However, since 48 and n48 will be operated as synchronous in same UL/DL configuration there is no IMD problem.

6.7.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.7.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48_n25-n48 | 48 | 0.8 |
|  | n25 | 0.6 |
|  | n48 | 0.8 |

Table 6.7.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48_n25-n48 | 48 | 0.4 |
|  | n25 | 0.3 |
|  | n48 | 0.4 |

6.7.5	MSD

Based on co-existence studies no additional MSD is needed.

6.8	DC_48_n48-n66

6.8.1	Operating bands for DC

Table 6.8.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_48_n48-n66 | 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.8.2	Configuration for DC

Table 6.8.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_48A_n48A-n66A | DC_48A_n66A | 48A | CA_n48A-n66A |

6.8.3	Co-existence studies

Co-existence analysis for DC_48_n66 UL shows that 2nd HAM of band 66 may fall into the lowest 10MHz of NR Band n48 DL. This is already covered in Table 7.3B.2.3.1-1 and Table 7.3B.2.3.1-2 in 38.101-3. Further 4th order IMD might fall in 48 DL However, since 48 and n48 will be operated as synchronous in same UL/DL configuration no IMD issue.

6.8.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.8.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48_n48-n66 | 48 | 0.8 |
|  | n48 |  |
|  | n66 | 0.6 |

Table 6.8.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48_n48-n66 | 48 | 0.4 |
|  | n48 |  |
|  | n66 | 0.3 |

6.8.5	MSD

Based on co-existence studies no additional MSD is needed.

6.9	DC_66_n25-n48

6.9.1	Operating bands for DC

Table 6.9.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n25-n48 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

6.9.2	Configuration for DC

Table 6.9.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_66A_n25A-n48A | DC_66A_n25A DC_66A_n48A | 66A | CA_n25A-n48A |

6.9.3	Co-existence studies

Co-existence analysis for DC_66_n48 UL shows that 2nd HAM of band 66 may fall into the lowest 10MHz of NR Band n48 DL. This is already covered in Table 7.3B.2.3.1-1 and Table 7.3B.2.3.1-2 in 38.101-3. Further 2nd IMD might also fall in n25 and 5th order IMD might fall in 66 DL. But the 5th IMD already covered in Table 7.3B.2.3.5.1-1 for DC_66A_n48A

Co-existence analysis for DC_66_n25 UL shows that 2nd HAM of band 66 may fall into the lowest 10MHz of NR Band n48 DL. This is already covered in Table 7.3B.2.3.1-1 and Table 7.3B.2.3.1-2 in 38.101-3. Further 2nd IMD might also fall in n48 DL.

6.9.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.9.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n25-n48 | 66 | 0.6 |
|  | n25 | 0.6 |
|  | n48 | 0.8 |

Table 6.9.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n25-n48 | 66 | 0.3 |
|  | n25 | 0.3 |
|  | n48 | 0.4 |

6.9.5	MSD

Based on co-existence studies additional MSD is needed to be defined. Similar values as for DC_2A-48A_n66A already defined in 38101-3 is proposed due to similarity in frequency range.

Table 6.9.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n25-n48 | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n48 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | 66 | 1735 | 5 | 25 | 2135 | N/A | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 28.3 | IMD2 |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |

6.10	DC_7_n40-n78

6.10.1	Operating bands for DC

Table 6.10.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n40-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.10.2	Configuration for DC

Table 6.10.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A_n40A-n78A | DC_7A_n40A DC_7A_n78A | 7A | CA_n40A-n78A |

6.10.3	Co-existence studies

Co-existence analysis for DC_7_n40 UL shows no impact to NR Band n78 DL.

Co-existence analysis for DC_7_n78 UL shows 4th IMD might fall into NR Band n40 DL.

6.10.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.10.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n40-n78 | 7 | 0.5 |
|  | n40 | 0.6 |
|  | n78 | 0.8 |

Table 6.10.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n40-n78 | 7 | 0 |
|  | n40 | 0.5 |
|  | n78 | 0.5 |

6.10.5	MSD

Based on co-existence studies additional MSD is needed to be defined due to 4nd IMD.

Table 6.10.5-1: Proposed MSD levels from interested companies for DC_7_n40-n78

| DC bands | UL DC | IMD |  | Nokia | LG | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n40A-n78A | 7 | IMD4 | ¦2*fB7 -2*fn78¦ | 9.3 | 9.1 | 9.2 |
|  | n78 |  |  |  |  |  |
|  | n40 |  |  |  |  |  |

Table 6.10.5-2: MSD test points for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n40A-n78A | 7 | 2520 | 10 | 50 | 2640 | N/A | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | 9.2 | IMD4 |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | N/A |

6.11	DC_8_n40-n78

6.11.1	Operating bands for DC

Table 6.11.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n40-n78 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.11.2	Configuration for DC

Table 6.11.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_8A_n40A-n78A | DC_8A_n40A DC_8A_n78A | 8A | CA_n40A-n78A |

6.11.3	Co-existence studies

Co-existence analysis for DC_8_n40 UL shows that 4th HAM of band 8 may fall into NR Band n78 DL. This is already covered in Table 7.3B.2.3.1-1 and Table 7.3B.2.3.1-2 in 38.101-3. Further, it shows that 2nd and 3rd IMD may fall in NR Band n78 DL.

Co-existence analysis for DC_8_n78 UL shows that 2nd IMD may fall into NR Band n40 DL.

6.11.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.11.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n40-n78 | 8 | 0.6 |
|  | n40 | 0.3 |
|  | n78 | 0.8 |

Table 6.11.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n40-n78 | 8 | 0.2 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

6.11.5	MSD

Based on co-existence studies additional MSD is needed.

Table 6.10.5-1: Proposed MSD levels from interested companies for DC_7_n40-n78

| DC bands | UL DC | IMD |  | Nokia | LG | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n40A-n78A | 8 | IMD2 | ¦fB8 + fn40¦ | 28.2 | 29.2 | 28.7 |
|  | n40 |  |  |  |  |  |
|  | n78 |  |  |  |  |  |
|  | 8 | IMD2 | ¦fB8 - fn78¦ | 26.0 | 28.6 | 27.3 |
|  | n78 |  |  |  |  |  |
|  | n40 |  |  |  |  |  |

Table 6.11.5-2: MSD test points for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n40A-n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | N/A |
|  | n78 | 3305 | 10 | 50 | 3307.5 | 28.7 | IMD21 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n40 | 2395 | 5 | 25 | 2395 | 27.3 | IMD2 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |

6.12	DC_28_n1-n40

6.12.1	Operating bands for DC

Table 6.12.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n1-n40 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

6.12.2	Configuration for DC

Table 6.12.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_28A_n1A-n40A | DC_28A_n1A DC_28A_n40A | 28A | CA_n1A-n40A |

6.12.3	Co-existence studies

Co-existence analysis for DC_28_n1 UL shows that 4th IMD may fall in NR Band n40 DL.

Co-existence analysis for DC_28_n40 UL shows that 3rd HAM of band 28 may fall into NR Band n1 DL. This is already covered in Table 7.3B.2.3.1-1 and Table 7.3B.2.3.1-2 in 38.101-3.

6.12.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.12.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n1-n40 | 28 | 0.6 |
|  | n1 | 0.3 |
|  | n40 | 0.5 |

Table 6.12.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n1-n40 | 28 | 0.2 |
|  | n1 | 0 |
|  | n40 | 0 |

6.12.5	MSD

Based on co-existence studies additional MSD is needed.

Table 6.12.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_28A_n1A-n40A | 28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n40 | 2374 | 5 | 25 | 2374 | 10.1 | IMD4 |

6.13	DC_1_n3-n77

6.13.1	Operating bands for DC

Table 6.13.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n3-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.13.2	Channel bandwidths per operating band for DC

Table 6.13.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A_n3A-n77A | DC_1A_n3A DC_1A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 250 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.13.3	Co-existence studies

Based on the co-existence studies of DC_1A-n3A and DC_1A-n77A captured in TR 37.716-11-11 and 37.863-01-01, 2nd and 4th order IMD generated by dual uplink of Band 1 + Band n3 may also fall into own Rx of band n77 and 2nd and 4th order IMD generated by dual uplink of Band 1 + Band n77 may also fall into own Rx of band n3.

6.13.4	∆TIB and ∆RIB values

For DC_1_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.13.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n3-n77 | 1 | 0.6 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 6.13.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n3-n77 | 1 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

6.13.5	MSD

As mentioned in 6.13.3, IMD2 and IMD4 of B1 and n3 to Band n77 Rx and those of B1 and n77 to Band n3 need to be addressed for REFSENS relaxation. Based on the exceptions of  DC_1-3_n77 and DC_1_n3-n78, the following values are proposed:

Table 6.13.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_1A_n3A-n77A DC_1A_n3A-n77(2A) | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | 28.4 | TDD | IMD2 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n77 | 3360 | 10 | 50 | 3360 | 11.2 | TDD | IMD4 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | TDD | N/A |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | 31.5 | FDD | IMD2 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | TDD | N/A |
|  | n3 | 1775 | 5 | 25 | 1870 | 8.5 | FDD | IMD4 |

6.14	DC_8_n3-n77

6.14.1	Operating bands for DC

Table 6.14.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n3-n77 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.14.2	Channel bandwidths per operating band for DC

Table 6.14.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A_n3A-n77A | DC_8A_n3A DC_8A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_8A_n3A-n77(2A) | DC_8A_n3A DC_8A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 240 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.14.3	Co-existence studies

Based on the co-existence studies of DC_8A-n3A and DC_8A-n77A captured in TR 37.716-11-11 and 37.863-01-01, 3rd and 5th order IMD generated by dual uplink of Band 8 + Band n3 may also fall into own Rx of band n77 and 3rd order IMD generated by dual uplink of Band 8 + Band n77 may also fall into own Rx of band n3.

6.14.4	∆TIB and ∆RIB values

For DC_8_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.14.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n3-n77 | 8 | 0.6 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 6.14.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n3-n77 | 8 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

6.14.5	MSD

As mentioned in 6.14.3, IMD3 of B8 and n3 to n77 Rx and IMD3 of B8 and n77 to n3 Rx need to be addressed for REFSENS relaxation. Based on REFSENS relaxation of DC_3-8_n78 and DC_3_n8-n78, the following values are proposed:

Table 6.14.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A-n3A_n77A DC_8A-n3A_n77(2A) | 8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | N/A | TDD | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | 16.3 | FDD | IMD3 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n77 | 3640 | 10 | 50 | 3640 | N/A | TDD | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | 16.5 | FDD | IMD3 |

6.15	DC_1_n3-n41

6.15.1	Operating bands for DC

Table 6.15.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1_n3-n41 | 1 | n3, n41 |

6.15.2	Channel bandwidths per operating band for DC

Table 6.15.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A_n3A-n41A | DC_1A_n3A DC_1A_n41A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n41A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.15.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.15.3-1 summarizes the EN-DC band combinations with self-interference problems for DC_1_n3-n41

Table 6.15.3-1: Summary of Self-interference analysis for DC_1_n3-n41 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_1_n3-n41 | DC_1A_n3A | No | IMD5 | Yes | Yes |
|  | DC_1A_n41A | No | No | Yes | Yes |

The MSD due to small frequency separation has been defined in the constituent fall-back modes DC_1_n3 and DC_1_n41.

6.15.4	∆ TIB and ∆RIB values

For DC_1_n3-n41, the TIB,c and RIB,c values are reuse the values of DC_1-3_n41 as given in the tables below.

Table 6.15.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n3-n41 | 1 | 0.5 |
|  | n3 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 6.15.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1_n3-n41 | 1 | 0 |
|  | n3 | 0 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

6.15.5	MSD

Table 6.15.5-1 shows the required MSD, its value reused the value of CA_1A-3A-41:

Table 6.15.5-1: MSD exception for Scell due to dual uplink operation for DC_1_n3-n41

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n3A-n41A | 1 | IMD5 | ¦3*fB1-2*fn3¦ | 1977.5 | 5 | 25 | 2167.5 | N/A |
|  | n3 |  |  | 1712.5 | 5 | 25 | 1807.5 | N/A |
|  | n41 |  |  | 2507.5 | 5 | 25 | 2507.5 | 5.0 |

6.16	DC_1_n41-n77

6.16.1	Operating bands for DC

Table 6.16.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1_n41-n77 | 1 | n41, n77 |

6.16.2	Channel bandwidths per operating band for DC

Table 6.16.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A_n41A-n77A | DC_1A_n41A DC_1A_n77A | 1 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.16.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.16.3-1 summarizes the EN-DC band combinations with self-interference problems for DC_1_n41-n77

Table 6.16.3-1: Summary of Self-interference analysis for DC_1_n41-n77 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_1_n41-n77 | DC_1A_n41A | H2 of B1 UL impacts to n77 DL | IMD3, IMD4, IMD5 | Yes | Yes |
|  | DC_1A_n77A | H2 of B1 UL impacts to n77 DL | IMD4, IMD5 | Yes | Yes |

The MSD due to harmonic issue has been defined in the constituent fall-back modes DC_1_n77.

The MSD due to small frequency separation has been defined in the constituent fall-back modes DC_1_n41.

6.16.4	∆TIB and ∆RIB values

For DC_1_n41-n77, the TIB,c and RIB,c values are reuse the values of DC_1-41_n77 as given in the tables below.

Table 6.16.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n41-n77 | 1 | 0.5 |
|  | n41 | 0.5 |
|  | n77 | 0.8 |

Table 6.16.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1_n41-n77 | 1 | 0 |
|  | n41 | 0 |
|  | n77 | 0.5 |

6.16.5	MSD

Table 6.16.5-1 shows the required MSD, its values can reuse the values for DC_1A_n41A-n78A:

Table 6.16.5-1: MSD exception for Scell due to dual uplink operation for DC_1_n41-n77

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n41A-n77A | 1 | IMD3 | ¦2*fn41- fB1¦ | 1970 | 5 | 25 | 2160 | N/A |
|  | n41 |  |  | 2650 | 10 | 25 | 2650 | N/A |
|  | n77 |  |  | 3330 | 10 | 50 | 3330 | 19.6 |
|  | 1 | IMD4 | ¦3*fB1 –fn41¦ | 1970 | 5 | 25 | 2160 | N/A |
|  | n41 |  |  | 2520 | 5 | 25 | 2640 | N/A |
|  | n77 |  |  | 3390 | 10 | 50 | 3390 | 10.11 |
|  | 1 | IMD4 | ¦2*fn77 –2*fB1¦ | 1970 | 5 | 25 | 2160 | N/A |
|  | n41 |  |  | 2680 | 5 | 25 | 2680 | 11.51 |
|  | n77 |  |  | 3310 | 10 | 50 | 3310 | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified |  |  |  |  |  |  |  |  |

6.17	DC_3_n3-n41

6.17.1	Operating bands for DC

Table 6.17.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_3_n3-n41 | 3 | n3, n41 |

6.17.2	Channel bandwidths per operating band for DC

Table 6.17.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_3A_n3A-n41A | DC_3A_n3A1 DC_3A_n41A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n41A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.17.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.17.3-1 summarizes the EN-DC band combinations with self-interference problems for DC_3_n3-n41

Table 6.17.3-1: Summary of Self-interference analysis for DC_3_n3-n41 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_3_n3-n41 | DC_3A_n3A | No | No | No | No |
|  | DC_3A_n41A | No | IMD4 | Yes | Yes |

The MSD due to small frequency separation has been defined in the constituent fall-back modes DC_3_n41.

6.17.4	∆TIB and ∆RIB values

For DC_3_n3-n41, the TIB,c and RIB,c values are reuse the values of DC_3_n41 as given in the tables below.

Table 6.17.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n3-n41 | 3 | 0.5 |
|  | n3 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 6.17.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3_n3-n41 | 3 | 0 |
|  | n3 | 0 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

6.17.5	MSD

Table 6.17.5-1 shows the required MSD, its value can reuse the value for DC_3A_n41A:

Table 6.17.5-1: MSD exception for Scell due to dual uplink operation for DC_3_n3-n41

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n3A-n41A | 3 | IMD4 | ¦2*fn41-2*fB3¦ | 1725 | 5 | 25 | 1820 | N/A |
|  | n3 |  |  | 1770 | 5 | 25 | 1865 | 8.2 |
|  | n41 |  |  | 2657.5 | 5 | 25 | 2657.5 | N/A |

6.18	DC_3_n41-n77

6.18.1	Operating bands for DC

Table 6.18.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_3_n41-n77 | 3 | n41, n77 |

6.18.2	Channel bandwidths per operating band for DC

Table 6.18.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_3A_n41A-n77A | DC_3A_n41A DC_3A_n77A | 3 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.18.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.18.3-1 summarizes the EN-DC band combinations with self-interference problems for DC_3_n41-n77

Table 6.18.3-1: Summary of Self-interference analysis for DC_3_n41-n77 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_3_n41-n77 | DC_3A_n41A | H2 of B3 UL impacts to n77 DL | IMD3, IMD5 | Yes | Yes |
|  | DC_3A_n77A | H2 of B3 UL impacts to n77 DL | IMD5 | No | Yes |

The MSD due to harmonic issue has been defined in the constituent fall-back modes DC_3_n77.

The MSD due to small frequency separation has been defined in the constituent fall-back modes DC_3_n41.

6.18.4	∆TIB and ∆RIB values

For DC_3_n41-n77, the TIB,c and RIB,c values are reuse the values of DC_3-41_n77 as given in the tables below.

Table 6.18.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n41-n77 | 3 | 0.6 |
|  | n41 | 0.31/0.82 |
|  | n77 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 6.18.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3_n41-n77 | 3 | 0.2 |
|  | n41 | 010.52 |
|  | n77 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

6.18.5	MSD

Table 6.18.5-1 shows the required MSD, its value can reuse the value for DC_3A_n41A-n78A:

Table 6.18.5-1: MSD exception for Scell due to dual uplink operation for DC_3_n41-n77

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n41A-n77A | 3 | IMD3 | ¦2*fn41- fB3¦ | 1730 | 5 | 25 | 1825 | N/A |
|  | n41 |  |  | 2560 | 10 | 50 | 2560 | N/A |
|  | n77 |  |  | 3390 | 10 | 50 | 3390 | 16.41 |
|  | 3 | IMD5 | ¦2*fn77 –3*fB3¦ | 1720 | 5 | 25 | 1815 | N/A |
|  | n41 |  |  | 2640 | 5 | 25 | 2640 | 5.3 |
|  | n77 |  |  | 3900 | 10 | 50 | 3900 | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified |  |  |  |  |  |  |  |  |

6.19	DC_41_n3-n41

6.19.1	Operating bands for DC

Table 6.19.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_41_n3-n41 | 41 | n3, n41 |

6.19.2	Channel bandwidths per operating band for DC

Table 6.19.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_41A_n3A-n41A | DC_41A_n3A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n41A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.19.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.19.3-1 summarizes the EN-DC band combinations with self-interference problems for DC_41_n3-n41

Table 6.19.3-1: Summary of Self-interference analysis for DC_41_n3-n41 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_41_n3-n41 | DC_41_n3 | No | No | Yes | Yes |
|  | DC_41_n41 | No | No | No | No |

The MSD due to small frequency separation has been defined in the constituent fall-back modes DC_41_n3.

6.19.4	∆TIB and ∆RIB values

For DC_41_n3-n41, the TIB,c and RIB,c values are reuse the values of DC_41_n3 as given in the tables below.

Table 6.19.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n3-n41 | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 6.19.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_41_n3-n41 | 41 | 01/0.52 |
|  | n3 | 0 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

6.19.5	MSD

There are no additional MSD requirements for this band combination.

6.20	DC_41_n41-n77

6.20.1	Operating bands for DC

Table 6.20.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_41_n41-n77 | 41 | n41, n77 |

6.20.2	Channel bandwidths per operating band for DC

Table 6.20.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_41A_n41A-n77A | DC_41A_n77A | 41 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.20.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.20.3-1 summarizes the EN-DC band combinations with self-interference problems for DC_41_n41-n77

Table 6.20.3-1: Summary of Self-interference analysis for DC_41_n41-n77 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_41_n41-n77 | DC_41A_n77A | No | IMD4 | Yes | No |

The MSD due to small frequency separation has been defined in the constituent fall-back modes DC_41_n77.

Since DC_41_n41 only apply for non-simultaneous Tx/Rx between all carriers, there is no IMD4 issue for DC_41_n41-n77 with UL DC_41_n77.

6.20.4	∆TIB and ∆RIB values

For DC_41_n41-n77, the TIB,c and RIB,c values are reuse the values of DC_41_n77 as given in the tables below.

Table 6.20.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n41-n77 | 41 | 0.3 |
|  | n41 | 0.3 |
|  | n77 | 0.8 |

Table 6.20.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_41_n41-n77 | 41 | 0 |
|  | n41 | 0 |
|  | n77 | 0.5 |

6.20.5	MSD

There is no additional MSD requirement for this band combination.

6.21	DC_41_n41-n78

6.21.1	Operating bands for DC

Table 6.21.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_41_n41-n78 | 41 | n41, n78 |

6.21.2	Channel bandwidths per operating band for DC

Table 6.21.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_41A_n41A-n78A | DC_41A_n78A | 41 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.21.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.21.3-1 summarizes the EN-DC band combinations with self-interference problems for DC_41_n41-n78

Table 6.21.3-1: Summary of Self-interference analysis for DC_41_n41-n78 with 2 bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_41_n41-n78 | DC_41_n78 | No | IMD4 | Yes | No |

The MSD due to small frequency separation has been defined in the constituent fall-back modes DC_41_n78.

Since DC_41_n41 only apply for non-simultaneous Tx/Rx between all carriers, there is no IMD4 issue for DC_41_n41-n78 with UL DC_41_n78.

6.21.4	∆TIB and ∆RIB values

For DC_41_n41-n78, the TIB,c and RIB,c values are reuse the values of DC_41_n78 as given in the tables below.

Table 6.21.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n41-n78 | 41 | 0.3 |
|  | n41 | 0.3 |
|  | n78 | 0.8 |

Table 6.21.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_41_n41-n78 | 41 | 0 |
|  | n41 | 0 |
|  | n78 | 0.5 |

6.21.5	MSD

There is no additional MSD requirement for this band combination.

6.22	DC_2_n7-n66

6.22.1	Operating bands for DC

Table 6.22.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_2_n7-n66 | 2 | n7, n66 |

6.22.2	Channel bandwidths per operating band for DC

Table 6.22.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 |  | 20 | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  | Maximum aggregated bandwidth [MHz] |  |
| DC_2A_n7A-n66A | DC_2A_n7A DC_2A_n66A | 2 |  | 15 | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 |  |
|  |  | n7 |  | See CA_n7A-n66A BCS configurations in Table 5.5A.3.1-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n7(2A)-n66A | DC_2A_n7A DC_2A_n66A | 2 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 |
|  |  | n7 |  | See CA_n7(2A)-n66A BCS configurations in Table 5.5A.3.1-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.22.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.22.3-1 summarizes the EN- DC band combinations with self-interference problems for DC_2_n7-n66

Table 6.22.3-1: Summary of Self-interference analysis for DC_2_n7-n66 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_2_n7-n66 | DC_2A_n7A | No | No | No | No |
|  | DC_2A_n66A | No | No | No | No |

6.22.4	∆TIB and ∆RIB values

For DC_2_n77-n66, the TIB,c and RIB,c values can reuse the values of DC_2A-7A_n66A as given in the tables below.

Table 6.22.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n7-n66 | 2 | 0.5 |
|  | n7 | 0.5 |
|  | n66 | 0.5 |

Table 6.22.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2_n7-n66 | 2 | 0.3 |
|  | n7 | 0.5 |
|  | n66 | 0.5 |

6.22.5	MSD

There are no additional MSD requirements for this band combination.

6.23	DC_2_n38-n66

6.23.1	Operating bands for DC

Table 6.23.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_2_n38-n66 | 2 | n38, n66 |

6.23.2	Channel bandwidths per operating band for DC

Table 6.23.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_2A_n38A-n66A | DC_2A_n38A DC_2A_n66A | 2 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 60 |
|  |  | n38 |  | See CA_n38A-n66A BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.23.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.23.3-1 summarizes the EN- DC band combinations with self-interference problems for DC_2_n38-n66

Table 6.23.3-1: Summary of Self-interference analysis for DC_2_n38-n66 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_2_n38-n66 | DC_2A_n38A | No | No | No | No |
|  | DC_2A_n66A | No | No | No | No |

6.23.4	∆TIB and ∆RIB values

For DC_2_n38-n66, the TIB,c and RIB,c values are given in the tables below.

Table 6.23.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n38-n66 | 2 | 0.5 |
|  | n38 | 0.9 |
|  | n66 | 0.5 |

Table 6.23.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2_n38-n66 | 2 | 0.3 |
|  | n38 | 0.5 |
|  | n66 | 0.5 |

6.23.5	MSD

There are no additional MSD requirements for this band combination.

6.24	DC_2_n66-n78

6.24.1	Operating bands for DC

Table 6.24.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_2_n66-n78 | 2 | n66, n78 |

6.24.2	Channel bandwidths per operating band for DC

Table 6.24.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_2A_n66A-n78(2A) DC_2A_n66(2A)-n78A DC_2A_n66(2A)-n78(2A) | DC_2A_n66A DC_2A_n78A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66A-n78(2A) BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 See CA_n66(2A)-n78A BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 See CA_n66(2A)-n78(2A) BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.24.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.24.3-1 summarizes the EN- DC band combinations with self-interference problems for DC_2_n66-n78

Table 6.24.3-1: Summary of Self-interference analysis for DC_2_n66-n78 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_2_n66-n78 | DC_2A_n66A | H2 of B2 UL impacts to n78 DL | IMD2 and IMD4 impacts to n78 | No | Yes |
|  | DC_2A_n78A | H2 of B2 UL impacts to n78 DL | IMD4 impacts to n66 | No | Yes |

The MSD due to harmonic issue has been defined in the constituent fall-back modes DC_2A_n78A in TS 38.101-3.

6.24.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values have been defined in the constituent fall-back modes DC_2A_n66A-n78A in TS 38.101-3.

6.24.5	MSD

The MSD due to intermodulation have been defined in the constituent fall-back modes DC_2A_n66A-n78A in TS 38.101-3. Table 6.24.5-1 shows the required MSD:

Table 6.24.5-1: MSD exception for Scell due to dual uplink operation for DC_2_n66-n78

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n66A-n78(2A) DC_2A_n66(2A)-n78A DC_2A_n66(2A)-n78(2A) | 2 | IMD4 | ¦3*fB2-fn78¦ | 1880 | 5 | 25 | 1960 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 10.3 |
|  | n78 |  |  | 3480 | 10 | 50 | 3480 | N/A |
|  | 2 | IMD2 | ¦fB2 +fn66¦ | 1880 | 5 | 25 | 1960 | N/A |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 | N/A |
|  | n78 |  |  | 3620 | 10 | 50 | 3620 | 29.4 |
|  | 2 | IMD4 | ¦3*fn66 -fB2¦ ¦3*fB2 –fn66¦ | 1880 | 5 | 25 | 1960 | N/A |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 | N/A |
|  | n78 |  |  | 3340 | 10 | 50 | 3340 | 8.9 |

6.25	DC_12_n7-n66

6.25.1	Operating bands for DC

Table 6.25.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_12_n7-n66 | 12 | n7, n66 |

6.25.2	Channel bandwidths per operating band for DC

Table 6.25.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 |  | 20 | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  | Maximum aggregated bandwidth [MHz] |  |
| DC_12A_n7A-n66A | DC_12A_n7A DC_12A_n66A | 12 |  | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 |  |
|  |  | n7 |  | See CA_n7A-n66A BCS configurations in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A_n7(2A)-n66A | DC_12A_n7A DC_12A_n66A | 12 |  | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 |
|  |  | n7 |  | See CA_n7(2A)-n66A BCS configurations in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.25.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.25.3-1 summarizes the EN-DC band combinations with self-interference problems for DC_12_n7-n66

Table 6.25.3-1: Summary of Self-interference analysis for DC_12_n7-n66 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_12_n7-n66 | DC_12A_n7A | H3 of B12 UL impacts to n66 DL | No | No | Yes |
|  | DC_12A_n66A | H3 of B12 UL impacts to n66 DL | No | No | Yes |

The MSD due to harmonic issue has been defined in the constituent fall-back modes DC_12_n66.

6.25.4	∆TIB and ∆RIB values

For DC_12_n7-n66, the TIB,c and RIB,c values can reuse the values of CA_7A-12A_66A as given in the tables below.

Table 6.25.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12_n7-n66 | 12 | 0.8 |
|  | n7 | 0.5 |
|  | n66 | 0.5 |

Table 6.25.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_12_n7-n66 | 12 | 0.5 |
|  | n7 | 0.5 |
|  | n66 | 0.5 |

6.25.5	MSD

There are no additional MSD requirements for this band combination.

6.26	DC_66_n38-n66

6.26.1	Operating bands for DC

Table 6.26.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_66_n38-n66 | 66 | n38, n66 |

6.26.2	Channel bandwidths per operating band for DC

Table 6.26.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_66A_n38A-n66A | DC_66A_n38A DC_66A_n66A1 | 66 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 60 |
|  |  | n38 |  | See CA_n38A-n66A BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.26.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.26.3-1 summarizes the EN- DC band combinations with self-interference problems for DC_66_n38-n66

Table 6.26.3-1: Summary of Self-interference analysis for DC_66_n38-n66 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_66_n38-n66 | DC_66A_n38A | No | No | No | No |
|  | DC_66A_n66A | No | No | No | No |

6.26.4	∆TIB and ∆RIB values

For DC_66_n38-n66, the TIB,c and RIB,c values are given in the tables below.

Table 6.26.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n38-n66 | 66 | 0.5 |
|  | n38 | 0.5 |
|  | n66 | 0.5 |

Table 6.26.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_66_n38-n66 | 66 | 0.5 |
|  | n38 | 0.5 |
|  | n66 | 0.5 |

6.26.5	MSD

There are no additional MSD requirements for this band combination.

6.27	DC_19_n1-n77

6.27.1	Operating bands for DC

Table 6.27.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19_n1-n77 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.27.2	Configuration for DC

Table 6.27.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A_n1A-n77A | DC_19A_n1A DC_19A_n77A | 19A | CA_n1A-n77A |

6.27.3	Co-existence studies

Based on the co-existence studies of DC_19_n1 and DC_19_n77, own Rx impact of the 3rd band is the followings.

-	3rd order IMD products generated by DC_19_n1 uplink may fall into own Rx of band n77.

-	5th order IMD products generated by DC_19_n1 uplink may fall into own Rx of band n77.

-	3rd order IMD products generated by DC_19_n77 uplink may fall into own Rx of band n1.

6.27.4	∆TIB and ∆RIB values

For DC_19_n1-n77, the TIB,c and RIB,c values are reused from DC_1-19_n77, and are given in the tables below.

Table 6.27.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19_n1-n77 | 19 | 0.3 |
|  | n1 | 0.3 |
|  | n77 | 0.8 |

Table 6.27.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19_n1-n77 | 19 | 0 |
|  | n1 | 0 |
|  | n77 | 0.5 |

6.27.5	MSD

The MSD values for own Rx impact of the 3rd band stated in 6.27.3 are shown in the following table [1].  (We will provide MSD result with [ ] because other companies wants to further study.)

Table 6.27.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_19A_n1A-n77A | 19 | 840 | 5 | 25 | 885 | N/A | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n77 | 3655 | 10 | 50 | 3655 | [21.4] | IMD34 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | 17.8 | IMD3 |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

6.28	DC_19_n1-n78

6.28.1	Operating bands for DC

Table 6.28.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19_n1-n78 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.28.2	Configuration for DC

Table 6.28.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A_n1A-n78A | DC_19A_n1A DC_19A_n78A | 19A | CA_n1A-n78A |

6.28.3	Co-existence studies

Based on the co-existence studies of DC_19_n1 and DC_19_n78, own Rx impact of the 3rd band is the followings.

-	3rd order IMD products generated by DC_19_n1 uplink may fall into own Rx of band n78.

-	3rd order IMD products generated by DC_19_n78 uplink may fall into own Rx of band n1.

6.28.4	∆TIB and ∆RIB values

For DC_19_n1-n78, the TIB,c and RIB,c values are reused from DC_1-19_n78, and are given in the tables below.

Table 6.28.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19_n1-n78 | 19 | 0.3 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |

Table 6.28.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19_n1-n78 | 19 | 0 |
|  | n1 | 0 |
|  | n78 | 0.5 |

6.28.5	MSD

The MSD values for own Rx impact of the 3rd band stated in 6.28.3 are shown in the following table [1]. (We will provide MSD result with [ ] because other companies wants to consider further.)

Table 6.28.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_19A_n1A-n78A | 19 | 840 | 5 | 25 | 885 | N/A | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n78 | 3655 | 10 | 50 | 3655 | [21.4] | IMD3 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | 17.8 | IMD3 |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |

6.29	DC_11_n3-n77

6.29.1	Operating bands for DC

Table 6.29.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_11_n3-n77 | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.29.2	Channel bandwidths per operating band for DC

Table 6.29.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_11A_n3A-n77A | DC_11A_n3A DC_11A_n77A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_11A_n3A-n77(2A) | DC_11A_n3A DC_11A_n77A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 240 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.29.3	Co-existence studies

Based on the co-existence studies of DC_11A-n3A and DC_11A-n77A captured in TR 37.716-11-11 and 37.863-01-01,

- 4th and 5th order IMD generated by dual uplink of Band 11 + Band n3 may also fall into own Rx of Band n77

- 2nd and 5th order IMD generated by dual uplink of Band 11 + Band n77 may also fall into own Rx of Band n3.

6.29.4	∆TIB and ∆RIB values

For DC_11_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.29.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11_n3-n77 | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n77 | 0.8 |

Table 6.29.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11_n3-n77 | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n77 | 0.5 |

6.29.5	MSD

As mentioned in 6.29.3, IMD4 of B11 and n3 to n77 Rx and IMD2 of B11 and n77 to n3 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 6.29.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_11A-n3A_n77A DC_11A-n3A_n77(2A) | 11 | 1440 | 5 | 25 | 1488 | N/A | FDD | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | N/A | FDD | N/A |
|  | n77 | 3780 | 10 | 50 | 3780 | 10.8 | TDD | IMD4 |
|  | 11 | 1440 | 5 | 25 | 1488 | N/A | FDD | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | N/A | FDD | N/A |
|  | n77 | 4020 | 10 | 50 | 4020 | 4.1 | TDD | IMD5 |
|  | 11 | 1440 | 5 | 25 | 1488 | N/A | FDD | N/A |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |
|  | n3 | 1775 | 5 | 25 | 1870 | 29.0 | FDD | IMD2 |
|  | 11 | 1440 | 5 | 25 | 1488 | N/A | FDD | N/A |
|  | n77 | 3925 | 10 | 50 | 3925 | N/A | TDD | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | 3.7 | FDD | IMD5 |

6.30	DC_11_n28-n77

6.30.1	Operating bands for DC

Table 6.30.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_11_n28-n77 | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.30.2	Channel bandwidths per operating band for DC

Table 6.30.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_11A_n28A-n77A | DC_11A_n28A DC_11A_n77A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_11A_n28A-n77(2A) | DC_11A_n28A DC_11A_n77A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 230 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.30.3	Co-existence studies

Based on the co-existence studies of DC_11A-n28A and DC_11A-n77A captured in TR 37.716-11-11 and 37.863-01-01,

- 3rd and 4th order IMD generated by dual uplink of Band 11 + Band n28 may also fall into own Rx of Band n77

- 3rd and 4th order IMD generated by dual uplink of Band 11 + Band n77 may also fall into own Rx of Band n28.

6.30.4	∆TIB and ∆RIB values

For DC_11_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.30.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11_n28-n77 | 11 | 0.4 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 6.30.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11_n28-n77 | 11 | 0 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

6.30.5	MSD

As mentioned in 6.30.3, IMD3 of B11 and n28 to n77 Rx and IMD3 of B11 and n77 to n28 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_11A_n28A-n77A DC_11A_n28A-n77(2A) | 11 | IMD3 | ¦2*fB11 +fn28¦ | 1443 | 5 | 25 | 1491 | 5 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 |  |
|  | n77 |  |  | 3629 | 10 | 50 | 3629 | 10 | 17.5 |
|  | 11 | IMD4 | ¦3*fB11 -fn28¦ | 1443 | 5 | 25 | 1491 | 5 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 |  |
|  | n77 |  |  | 3586 | 10 | 50 | 3586 | 10 | 11.2 |
|  | 11 | IMD3 | ¦2*fB11 - fn77¦ | 1443 | 5 | 25 | 1491 | 5 | N/A |
|  | n77 |  |  | 3684 | 10 | 50 | 3684 | 10 |  |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 | 15.8 |
|  | 11 | IMD4 | ¦3*fB11 -fn77¦ | 1443 | 5 | 25 | 1491 | 5 | N/A |
|  | n77 |  |  | 3564 | 10 | 50 | 3564 | 10 |  |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 | 11.0 |

6.31	DC_42_n3-n28

6.31.1	Operating bands for DC

Table 6.31.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42_n3-n28 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

6.31.2	Channel bandwidths per operating band for DC

Table 6.31.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_42A_n3A-n28A | DC_42A_n3A DC_42A_n28A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 70 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42C_n3A-n28A | DC_42A_n3A DC_42A_n28A DC_42C_n28A | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.31.3	Co-existence studies

Based on the co-existence studies of DC_42A-n3A and DC_42A-n28A, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.31.4	∆TIB and ∆RIB values

For DC_42_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.31.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42_n3-n28 | 42 | 0.8 |
|  | n3 | 0.6 |
|  | n28 | 0.8 |

Table 6.31.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42_n3-n28 | 42 | 0.5 |
|  | n3 | 0.2 |
|  | n28 | 0.5 |

6.31.5	MSD

As mentioned in 6.31.3, there is no need to specify additional MSD requirement for this UL DC configuration..

6.32	DC_42_n3-n77

6.32.1	Operating bands for DC

Table 6.32.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42_n3-n77 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.32.2	Channel bandwidths per operating band for DC

Table 6.32.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_42A_n3A-n77A | DC_42A_n3A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_42A_n3A-n77(2A) | DC_42A_n3A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 250 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42C_n3A-n77A | DC_42A_n3A DC_42C_n3A | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_42C_n3A-n77(2A) | DC_42A_n3A DC_42C_n3A | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 270 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.32.3	Co-existence studies

Based on the co-existence studies of DC_42A-n3A and DC_42A-n77A, 4th and 5th order IMD generated by dual uplink of Band 42 + Band n3 may also fall into own Rx of band n77. But due to the TDD synchronous operation of B42 and n77, it is not impacted.

6.32.4	∆TIB and ∆RIB values

For DC_42_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.32.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42_n3-n77 | 42 | 0.8 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 6.32.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42_n3-n77 | 42 | 0.5 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

6.32.5	MSD

As mentioned in 6.32.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.33	DC_41_n28-n41

6.33.1	Operating bands for DC

Table 6.33.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41A_n28A-n41A | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.33.2	Channel bandwidths per operating band for DC

Table 6.33.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_41A_n28A-n41A | DC_41A_n28A | 41 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n28 |  | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

6.33.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_41A_n28A-n41A with uplink on DC_41A_n28A was calculated and listed in Table 6.33.3-1.

Table 6.33.3-1: Harmonic and IMD analysis for DC_41_n28-n41 with UL on DC_41_n28

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2109 | 2244 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1082 | 962 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2672 | 2867 | 379 | 214 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4123 | 4318 | 3116 | 3281 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4382 | 4652 | 324 | 534 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1924 | 2164 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5833 | 6103 | 3819 | 4029 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 4826 | 5066 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1282 | 1027 | 6437 | 6092 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1176 | 1461 | 3949 | 3634 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4522 | 4777 | 7543 | 7888 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5529 | 5814 | 6536 | 6851 |

No IMD issue for this combination.

6.33.4	∆TIB and ∆RIB values

For DC_41A_n28A-n41A, the TIB,c and RIB,c values can reuse the values of DC_3A-41A_n28A as given in the tables below.

Table 6.33.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n28-n41 | 41 | 0.31/0.82 |
|  | n28 | 0.3 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

Table 6.33.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_41_n28-n41 | 41 | 01/0.52 |
|  | n28 | 0 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

6.33.5	MSD

There is no additional MSD requirement for this configuration.

6.34	DC_18_n28-n41

6.34.1	Operating bands for DC

Table 6.34.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A_n28A-n41A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.34.2	Channel bandwidths per operating band for DC

Table 6.34.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_18A_n28A-n41A | DC_18A_n28A DC_18A_n41A | 18 |  | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 145 |
|  |  | n28 |  | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

6.34.3	Co-existence studies

Based on the co-existence studies of DC_18A-n28A and DC_18A-n41, impact on own Rx of 3rd band is as follows.

- 5th order IMD generated by dual uplink of Band 18 + Band n28 may also fall into own Rx of band n41.

- 5th order IMD generated by dual uplink of Band 18 + Band n41 may also fall into own Rx of band n28. Since band 18 is only used in Japan, there will be no IMD issue for this dual uplink configuration considering Japanese spectrum allocation.

6.34.4	∆TIB and ∆RIB values

TIB,c and RIB,c values for DC_18A_n28A-n41A are shown in the following tables.

Table 6.34.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18_n28-n41 | 18 | 0.5 |
|  | n28 | 0.5 |
|  | n41 | 0.3 |

Table 6.34.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18_n28-n41 | 18 | 0 |
|  | n28 | 0 |
|  | n41 | 0 |

6.34.5	MSD

Table 6.34.5-1 shows the required MSD:

Table 6.34.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_18A_n28A-n41A

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_18A_n28A-n41A | 18 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n41 | 2510 | 5 | 25 | 2510 | 5.1 | TDD | IMD5 ¦4*fB18 -fn28¦ |
|  | 18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n41 | 2515 | 5 | 25 | 2515 | N/A | TDD | N/A |
|  | n28 | 710 | 5 | 25 | 765 | 4.7 | FDD | IMD5 ¦4*fB18 -fn41¦ |

6.35	DC_18_n28-n77

6.35.1	Operating bands for DC

Table 6.35.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A_n28A-n77A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.35.2	Channel bandwidths per operating band for DC

Table 6.35.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_18A_n28A-n77A | DC_18A_n28A DC_18A_n77A | 18 |  | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 145 |
|  |  | n28 |  | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.35.3	Co-existence studies

Based on the co-existence studies of DC_18A-n28A and DC_18A-n77, impact on own Rx of 3rd band is as follows.

- 5th order IMD generated by dual uplink of Band 18 + Band n28 may also fall into own Rx of band n77.

- 5th order IMD generated by dual uplink of Band 18 + Band n77 may also fall into own Rx of band n28.

6.35.4	∆TIB and ∆RIB values

TIB,c and RIB,c values for DC_18A_n28A-n77A are shown in the following tables.

Table 6.35.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18_n28-n77 | 18 | 0.5 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |

Table 6.35.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18_n28-n77 | 18 | 0 |
|  | n28 | 0 |
|  | n77 | 0.5 |

6.35.5	MSD

Table 6.35.5-1 shows the required MSD:

Table 6.35.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_18A_n28A-n77A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n28A-n77A | 18 | IMD5 | ¦2*fB18 +3*fn28¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 |  |
|  | n77 |  |  | 3770 | 10 | 50 | 3770 | 10 | 4.0 |
|  | 18 | IMD5 | ¦4*fB18 -fn77¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n77 |  |  | 4045 | 10 | 50 | 4045 | 10 |  |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 | 5.7 |

6.36	DC_18_n28-n78

6.36.1	Operating bands for DC

Table 6.36.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A_n28A-n78A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.36.2	Channel bandwidths per operating band for DC

Table 6.36.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_18A_n28A-n78A | DC_18A_n28A DC_18A_n78A | 18 |  | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 145 |
|  |  | n28 |  | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.36.3	Co-existence studies

Based on the co-existence studies of DC_18A-n28A and DC_18A-n78, impact on own Rx of 3rd band is as follows.

- 5th order IMD generated by dual uplink of Band 18 + Band n28 may also fall into own Rx of band n78.

6.36.4	∆TIB and ∆RIB values

TIB,c and RIB,c values for DC_18A_n28A-n78A are shown in the following tables.

Table 6.36.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18_n28-n78 | 18 | 0.5 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 6.36.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18_n28-n78 | 18 | 0 |
|  | n28 | 0 |
|  | n78 | 0.5 |

6.36.5	MSD

Table 6.36.5-1 shows the required MSD:

Table 6.36.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_18A_n28A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n28A-n78A | 18 | IMD5 | ¦2*fB18 +3*fn28¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 |  |
|  | n78 |  |  | 3770 | 10 | 50 | 3770 | 10 | 4.0 |

6.37	DC_18_n3-n41

6.37.1	Operating bands for DC

Table 6.37.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A_n3A-n41A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.37.2	Channel bandwidths per operating band for DC

Table 6.37.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_18A_n3A-n41A | DC_18A_n3A DC_18A_n41A | 18 |  | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 155 |
|  |  | n3 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n41 |  | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

6.37.3	Co-existence studies

Based on the co-existence studies of DC_18A-n3A and DC_18A-n41, impact on own Rx of 3rd band is as follows.

- 2nd and 3rd order IMD generated by dual uplink of Band 18 + Band n3 may also fall into own Rx of band n41.

- 2nd order IMD generated by dual uplink of Band 18 + Band n41 may also fall into own Rx of band n3.

6.37.4	∆TIB and ∆RIB values

TIB,c and RIB,c values for DC_18A_n3A-n41A are shown in the following tables.

Table 6.37.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18_n3-n41 | 18 | 0.3 |
|  | n3 | 0.5 |
|  | n41 | 0.3 |

Table 6.37.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18_n3-n41 | 18 | 0 |
|  | n3 | 0 |
|  | n41 | 0 |

6.37.5	MSD

Table 6.37.5-1 shows the required MSD:

Table 6.37.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_18A_n3A-n41A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n3A-n41A | 18 | IMD2 | ¦fB18 +fn3¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n3 |  |  | 1720 | 5 | 25 | 1815 | 5 |  |
|  | n41 |  |  | 2540 | 10 | 50 | 2540 | 10 | 29.4 |
|  | 18 | IMD2 | ¦fB18 -fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2670 | 10 | 50 | 2670 | 10 |  |
|  | n3 |  |  | 1755 | 5 | 25 | 1850 | 5 | 28.2 |
|  | 18 | IMD3 | ¦2fn3-fB18 ¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n3 |  |  | 1725 | 5 | 25 | 1820 | 5 |  |
|  | n41 |  |  | 2630 | 10 | 50 | 2630 | 10 | 16.1 |

6.38	DC_18_n41-n77

6.38.1	Operating bands for DC

Table 6.38.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A_n41A-n77A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.38.2	Channel bandwidths per operating band for DC

Table 6.38.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_18A_n41A-n77A | DC_18A_n41A DC_18A_n77A | 18 |  | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 215 |
|  |  | n41 |  | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n77 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.38.3	Co-existence studies

Based on the co-existence studies of DC_18A-n41A and DC_18A-n77, impact on own Rx of 3rd band is as follows.

- 2nd, 3rd and 4th order IMD generated by dual uplink of Band 18 + Band n41 may also fall into own Rx of band n77.

- 2nd and 3rd order IMD generated by dual uplink of Band 18 + Band n77 may also fall into own Rx of band n41.

6.38.4	∆TIB and ∆RIB values

TIB,c and RIB,c values for DC_18A_n41A-n77A are shown in the following tables.

Table 6.38.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18_n41-n77 | 18 | 0.3 |
|  | n41 | 0.3 |
|  | n77 | 0.8 |

Table 6.38.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18_n41-n77 | 18 | 0 |
|  | n41 | 0 |
|  | n77 | 0.5 |

6.38.5	MSD

Table 6.38.5-1 shows the required MSD:

Table 6.38.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_18A_n41A-n77A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n41A-n77A | 18 | IMD2 | ¦fB18 +fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2570 | 5 | 25 | 2570 | 5 |  |
|  | n77 |  |  | 3390 | 10 | 50 | 3390 | 10 | 30.1 |
|  | 18 | IMD3 | ¦2*fB18 +fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2515 | 5 | 25 | 2515 | 5 |  |
|  | n77 |  |  | 4155 | 10 | 50 | 4155 | 10 | 16.4 |
|  | 18 | IMD4 | ¦2*fB18 -2*fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2515 | 5 | 25 | 2515 | 5 |  |
|  | n77 |  |  | 3390 | 10 | 50 | 3390 | 10 | 9.1 |
|  | 18 | IMD2 | ¦fB18 -fn77¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n77 |  |  | 3450 | 10 | 50 | 3450 | 10 |  |
|  | n41 |  |  | 2630 | 5 | 25 | 2630 | 5 | 28.5 |

6.39	DC_18_n41-n78

6.39.1	Operating bands for DC

Table 6.39.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A_n41A-n78A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.39.2	Channel bandwidths per operating band for DC

Table 6.39.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_18A_n41A-n78A | DC_18A_n41A DC_18A_n78A | 18 |  | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 215 |
|  |  | n41 |  | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.39.3	Co-existence studies

Based on the co-existence studies of DC_18A-n41A and DC_18A-n78, impact on own Rx of 3rd band is as follows.

- 2nd and 4th order IMD generated by dual uplink of Band 18 + Band n41 may also fall into own Rx of band n78.

- 2nd order IMD generated by dual uplink of Band 18 + Band n78 may also fall into own Rx of band n41.

6.39.4	∆TIB and ∆RIB values

TIB,c and RIB,c values for DC_18A_n41A-n78A are shown in the following tables.

Table 6.39.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18_n41-n78 | 18 | 0.3 |
|  | n41 | 0.3 |
|  | n78 | 0.8 |

Table 6.39.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18_n41-n78 | 18 | 0 |
|  | n41 | 0 |
|  | n78 | 0.5 |

6.39.5	MSD

Table 6.39.5-1 shows the required MSD:

Table 6.39.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_18A_n41A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n41A-n78A | 18 | IMD2 | ¦fB18 +fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2570 | 5 | 25 | 2570 | 5 |  |
|  | n78 |  |  | 3390 | 10 | 50 | 3390 | 10 | 30.1 |
|  | 18 | IMD4 | ¦2*fB18 -2*fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2515 | 5 | 25 | 2515 | 5 |  |
|  | n78 |  |  | 3390 | 10 | 50 | 3390 | 10 | 9.1 |
|  | 18 | IMD2 | ¦fB18 -fn78¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n78 |  |  | 3450 | 10 | 50 | 3450 | 10 |  |
|  | n41 |  |  | 2630 | 5 | 25 | 2630 | 5 | 28.5 |

6.40	DC_19_n1-n79

6.40.1	Operating bands for DC

Table 6.40.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19_n1-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.40.2	Configuration for DC

Table 6.40.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A_n1A-n79 AX1,X2 | DC_19A_n1A DC_19A_n79A | 19A | CA_n1A-n79A |
| NOTE X1: The frequency range in band n1 is restricted for this band combination to 1940 - 1960 MHz for the UL and 2130-2150 MHz for the DL. NOTE X2: The frequency range in band n79 is restricted for this band combination to 4500 - 4600 MHz. |  |  |  |

6.40.3	Co-existence studies

Based on the co-existence studies of DC_19_n1 and DC_19_n79, own Rx impact of the 3rd band is the followings.

-	3rd order IMD products generated by DC_19_n1 uplink may fall into own Rx of band n79.

-	4th order IMD products generated by DC_19_n1 uplink may fall into own Rx of band n79.

-	4th order IMD products generated by DC_19_n79 uplink may fall into own Rx of band n1.

6.40.4	∆TIB and ∆RIB values

For DC_19_n1-n79, the TIB,c and RIB,c values are reused from DC_1-19_n79, and are given in the tables below.

Table 6.40.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19_n1-n79 | 19 | 0.3 |
|  | n1 | 0.3 |
|  | n79 | 0 |

Table 6.40.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19_n1-n79 | 19 | 0.3 |
|  | n1 | 0.3 |
|  | n79 | 0 |

6.40.5	MSD

When IMD was calculated based on the frequency range that the operator actually owned, IMD3 and IMD4 don’t fall into own Rx of band n79 and IMD4 doesn’t fall into own Rx of band n1. Therefore, there is no need to have MSD added. NOTE X1 and NOTE X2 in Table 6.40.2-1 indicate these frequency ranges.

6.41	DC_21_n1-n77

6.41.1	Operating bands for DC

Table 6.41.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21_n1-n77 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.41.2	Configuration for DC

Table 6.41.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_21A_n1A-n77AX1,X2 | DC_21A_n1A DC_21A_n77A | 21A | CA_n1A-n77A |
| NOTE X1: The frequency range in band n1 is restricted for this band combination to 1940 - 1960 MHz for the UL and 2130-2150 MHz for the DL. NOTE X2: The frequency range in band n77 is restricted for this band combination to 3440 – 3520 MHz and 3600 - 3700 MHz. |  |  |  |

6.41.3	Co-existence studies

Based on the co-existence studies of DC_21_n1 and DC_21_n77, own Rx impact of the 3rd band is the followings.

-	2nd order IMD products generated by DC_21_n1 uplink may fall into own Rx of band n77.

-	5th order IMD products generated by DC_21_n1 uplink may fall into own Rx of band n77.

-	2nd order IMD products generated by DC_21_n77 uplink may fall into own Rx of band n1.

-	5th order IMD products generated by DC_21_n77 uplink may fall into own Rx of band n1.

6.41.4	∆TIB and ∆RIB values

For DC_21_n1-n77, the TIB,c and RIB,c values are reused from DC_1-21_n77, and are given in the tables below.

Table 6.41.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21_n1-n77 | 21 | 0.3 |
|  | n1 | 0.3 |
|  | n77 | 0.8 |

Table 6.41.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21_n1-n77 | 21 | 0 |
|  | n1 | 0 |
|  | n77 | 0.5 |

6.41.5	MSD

IMD2 of band 21 and band n77 to band n1 Rx need to be addressed for REFSENS relaxation. For DC_21_n1-n77, REFSENS exceptions are reused from DC_1-21_n77, already defined in TS 38.101-3. On the other hand, IMD2 and IMD5 don’t fall into own Rx of band n77 when IMD was calculated based on the frequency range that the operator actually owned. Therefore, there is no need to have MSD added about these. NOTE X1 and NOTE X2 in Table 6.41.2-1 indicate these frequency ranges.

Table 6.41.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_21A_n1A-n77A | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n1 | 1964.6 | 5 | 25 | 2154.6 | 30.6 | IMD24 |
|  | n77 | 3605 | 10 | 50 | 3605 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

6.42	DC_21_n1-n78

6.42.1	Operating bands for DC

Table 6.42.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21_n1-n78 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.42.2	Configuration for DC

Table 6.42.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_21A_n1A-n78AX1,X2 | DC_21A_n1A DC_21A_n78A | 21A | CA_n1A-n78A |
| NOTE X1: The frequency range in band n1 is restricted for this band combination to 1940 - 1960 MHz for the UL and 2130-2150 MHz for the DL. NOTE X2: The frequency range in band n78 is restricted for this band combination to 3440 – 3520 MHz and 3600 - 3700 MHz. |  |  |  |

6.42.3	Co-existence studies

Based on the co-existence studies of DC_21_n1 and DC_21_n78, own Rx impact of the 3rd band is the followings.

-	2nd order IMD products generated by DC_21_n1 uplink may fall into own Rx of band n78.

-	2nd order IMD products generated by DC_21_n78 uplink may fall into own Rx of band n1.

-	5th order IMD products generated by DC_21_n78 uplink may fall into own Rx of band n1.

6.42.4	∆TIB and ∆RIB values

For DC_21_n1-n78, the TIB,c and RIB,c values are reused from DC_1-21_n78, and are given in the tables below.

Table 6.42.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21_n1-n78 | 21 | 0.4 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 6.42.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21_n1-n78 | 21 | 0 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

6.42.5	MSD

IMD2 of band 21 and band n78 to band n1 Rx need to be addressed for REFSENS relaxation. For DC_21_n1-n78, REFSENS exceptions are reused from DC_1-21_n78, already defined in TS 38.101-3. On the other hand, IMD2 doesn’t fall into own Rx of band n78 when IMD was calculated based on the frequency range that the operator actually owned. Therefore, there is no need to have MSD added about this. NOTE X1 and NOTE X2 in Table 6.42.2-1 indicate these frequency ranges.

Table 6.42.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_21A_n1A-n78A | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n1 | 1964.6 | 5 | 25 | 2154.6 | 30.6 | IMD24 |
|  | n78 | 3605 | 10 | 50 | 3605 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

6.43	DC_21_n1-n79

6.43.1	Operating bands for DC

Table 6.43.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21_n1-n79 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.43.2	Configuration for DC

Table 6.43.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_21A_n1A-n79A | DC_21A_n1A DC_21A_n79A | 21A | CA_n1A-n79A |
| NOTE X1: The frequency range in band n1 is restricted for this band combination to 1940 - 1960 MHz for the UL and 2130-2150 MHz for the DL. NOTE X2: The frequency range in band n79 is restricted for this band combination to 4500 - 4600 MHz. |  |  |  |

6.43.3	Co-existence studies

Based on the co-existence studies of DC_21_n1 and DC_21_n79, own Rx impact of the 3rd band is the followings.

-	3rd order IMD products generated by DC_21_n1 uplink may fall into own Rx of band n79.

-	4th order IMD products generated by DC_21_n1 uplink may fall into own Rx of band n79.

6.43.4	∆TIB and ∆RIB values

For DC_21_n1-n79, the TIB,c and RIB,c values are reused from DC_1-21_n79, and are given in the tables below.

Table 6.43.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21_n1-n79 | 21 | 0.3 |
|  | n1 | 0.3 |
|  | n79 | 0 |

Table 6.43.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21_n1-n79 | 21 | 0 |
|  | n1 | 0 |
|  | n79 | 0 |

6.43.5	MSD

When IMD was calculated based on the frequency range that the operator actually owned, IMD3 and IMD4 don’t fall into own Rx of band n79. Therefore, there is no need to have MSD added. NOTE X1 and NOTE X2 in Table 6.43.2-1 indicate these frequency ranges.

6.44	DC_42_n1-n77

6.44.1	Operating bands for DC

Table 6.44.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42_n1-n77 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.44.2	Configuration for DC

Table 6.44.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_42A_n1A-n77A | DC_42A_n1A | 42A | CA_n1A-n77A |
| DC_42C_n1A-n77A | DC_42A_n1A DC_42C_n1A | CA_42C | CA_n1A-n77A |

6.44.3	Co-existence studies

Based on the co-existence studies of DC_42A-n1A, 4th and 5th order IMD generated by dual uplink of Band 42 + Band n1 may also fall into own Rx of band n77. But due to the TDD synchronous operation of B42 and n77, it is not impacted.

6.44.4	∆TIB and ∆RIB values

For DC_42_n1-n77, the TIB,c and RIB,c values are reused from DC_1-42_n77, and are given in the tables below.

Table 6.44.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42_n1-n77 | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 6.44.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42_n1-n77 | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

6.44.5	MSD

MSD are not needed considering the co-existence studies in 6.44.3.

.

6.45	DC_42_n1-n78

6.45.1	Operating bands for DC

Table 6.45.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42_n1-n78 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.45.2	Configuration for DC

Table 6.45.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_42A_n1A-n78AX1 | N/A | 42A | CA_n1A-n78A |
| DC_42C_n1A-n78AX1 | N/A | CA_42C | CA_n1A-n78A |
| NOTE X1: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used. |  |  |  |

6.45.3	Co-existence studies

Co-existence studies are not needed because Uplink EN-DC configuration is N/A for DC_42_n1-n78.

6.45.4	∆TIB and ∆RIB values

For DC_42_n1-n78, the TIB,c and RIB,c values are reused from DC_1-42_n78, and are given in the tables below.

Table 6.45.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42_n1-n78 | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |

Table 6.45.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42_n1-n78 | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

6.45.5	MSD

MSD are not needed because Uplink EN-DC configuration is N/A for DC_42_n1-n78.

6.46	DC_42_n1-n79

6.46.1	Operating bands for DC

Table 6.46.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42_n1-n79 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.46.2	Configuration for DC

Table 6.46.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_42A_n1A-n79AX1 | N/A | 42A | CA_n1A-n79A |
| DC_42C_n1A-n79 AX1 | N/A | CA_42C | CA_n1A-n79A |
| NOTE X1: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used. |  |  |  |

6.46.3	Co-existence studies

Co-existence studies are not needed because Uplink EN-DC configuration is N/A for DC_42_n1-n79.

6.46.4	∆TIB and ∆RIB values

For DC_42_n1-n79, the TIB,c and RIB,c values are reused from DC_1-42_n79, and are given in the tables below.

Table 6.46.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42_n1-n79 | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n79 | 0 |

Table 6.46.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42_n1-n79 | 42 | 0.5 |
|  | n1 | 0 |
|  | n79 | 0 |

6.46.5	MSD

MSD are not needed because Uplink EN-DC configuration is N/A for DC_42_n1-n79.

6.47	DC_28_n1-n78

6.47.1	Operating bands for DC

Table 6.47.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n1-n78 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.47.2	Channel bandwidths per operating band for DC

Table 6.47.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_28A_n1A-n78A | DC_28A_ n1A DC_28A_ n78A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.47.3	Co-existence studies

Based on co-existence studies, 3rd order IMD generated by dual uplink of 28_n1 may fall into own Rx of band n78.

Based on co-existence studies, 3rd order IMD generated by dual uplink of 28_n78 may fall into own Rx of band n1.

6.47.4	∆TIB and ∆RIB values

For DC_28_n1-n78, the TIB,c and RIB,c values are derived from DC_1-28_n78 and are given in the tables below.

Table 6.47.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n1-n78 | 28 | 0.6 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |

Table 6.47.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n1-n78 | 28 | 0.2 |
|  | n1 | 0 |
|  | n78 | 0.5 |

6.47.5	MSD

As mentioned in 6.47.3, 3rd order IMD generated by dual uplink of 28_n1 may fall into own Rx of band n78. Relaxation is derived from DC_1A_n28A-n78A and proposed in table below.

As mentioned in 6.47.3, 3rd order IMD generated by dual uplink of 28_n78 may fall into own Rx of band n1. Relaxation is derived from DC_1A-28A_n78A and proposed in table below.

Table 6.47.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A_n1A-n78A | 28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3416 | 10 | 50 | 3416 | 15.7 | IMD3 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n1 | 1960 | 5 | 25 | 2150 | 15.7 | IMD3 |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |

6.48	DC_2_n5-n77

6.48.1	Operating bands for DC

Table 6.48.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n5-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.48.2	Channel bandwidths per operating band for DC

Table 6.48.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n5A-n77A | DC_2A_n5A DC_2A_ n77A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.48.3	Co-existence studies

Based on co-existence studies, 3rd and 5th order IMD generated by dual uplink of 2_n5 may fall into own Rx of band n77.

Based on co-existence studies, 5th order IMD generated by dual uplink of 2_n77 may fall into own Rx of band n5.

6.48.4	∆TIB and ∆RIB values

For DC_2_n5-n77, the TIB,c and RIB,c values are derived from CA_2-13-48 and are given in the tables below.

Table 6.48.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n5-n77 | 2 | 0.6 |
|  | n5 | 0.3 |
|  | n77 | 0.8 |

Table 6.48.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n5-n77 | 2 | 0.2 |
|  | n5 | 0 |
|  | n77 | 0.5 |

6.48.5	MSD

As mentioned in 6.48.3, 3rd and 5th order IMD generated by dual uplink of 2_n5 may fall into own Rx of band n77.

As mentioned in 6.48.3, 5th order IMD generated by dual uplink of 2_n77 may fall into own Rx of band n5.

Table 6.48.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A_n5A-n77A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | 16.0 | IMD3 |
|  | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n77 | 3980 | 10 | 50 | 3980 | 4.1 | IMD5 |
|  | 2 | 1907 | 5 | 25 | 1987 | N/A | N/A |
|  | n5 | 844 | 5 | 25 | 889 | 3.8 | IMD5 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |

6.49	DC_2_n66-n77

6.49.1	Operating bands for DC

Table 6.49.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n66-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.49.2	Channel bandwidths per operating band for DC

Table 6.49.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n66A-n77A DC_2A_n66A-n77C | DC_2A_ n77A DC_2A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 260 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| DC_2A-2A_n66A-n77A DC_2A-2A_n66A-n77C | DC_2A_ n77A DC_2A_n66A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 280 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.49.3	Co-existence studies

Based on co-existence studies, 2nd, 4th and 5th order IMD generated by dual uplink of 2_n77 may fall into own Rx of band n66.

Based on co-existence studies, 2nd and 4th order IMD generated by dual uplink of 2_n66 may fall into own Rx of band n77

6.49.4	∆TIB and ∆RIB values

For DC_2_n66-n77, the TIB,c and RIB,c values are derived from CA_n25-n66-n78 and are given in the tables below.

Table 6.49.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n66-n77 | 2 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 6.49.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n66-n77 | 2 | 0.3 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

6.49.5	MSD

As mentioned in 6.49.3, 2nd 4th and 5th order IMD generated by dual uplink of 2_n77 may fall into own Rx of band n66.

Table 6.49.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A_n66A-n77A11 DC_2A-2A_n66A-n77A11 | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 29.2 | IMD2 |
|  | n77 | 4060 | 10 | 50 | 4060 | N/A | N/A |
| DC_2A_n66A-n77A DC_2A-2A_n66A-n77A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 10.3 | IMD4 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| DC_2A_n66A-n77A DC_2A-2A_n66A-n77A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 4.0 | IMD5 |
|  | n77 | 3930 | 10 | 50 | 3930 | N/A | N/A |
| DC_2A_n66A-n77A11 DC_2A-2A_n66A-n77A11 |  | 2 |  | 1853 |  | 5 |  | 25 |  | 1933 |  | N/A |  | N/A |  |
|  |  | n66 |  | 1713 |  | 5 |  | 25 |  | 2113 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3566 |  | 10 |  | 50 |  | 3566 |  | 29.4 |  | IMD2 |  |
| DC_2A_n66A-n77A DC_2A-2A_n66A-n77A |  | 2 |  | 1853 |  | 5 |  | 25 |  | 1933 |  | N/A |  | N/A |  |
|  |  | n66 |  | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3784 |  | 10 |  | 50 |  | 3784 |  | 8.9 |  | IMD4 |  |
| NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

6.50	DC_13_n2-n77

6.50.1	Operating bands for DC

Table 6.50.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13_n2-n77 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.50.2	Channel bandwidths per operating band for DC

Table 6.50.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_13A_n2A-n77A | DC_13A_n2A DC_13A_ n77A | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.50.3	Co-existence studies

Based on co-existence studies, 3rd order IMD generated by dual uplink of 13_n77 may fall into own Rx of band n2.

Based on co-existence studies, 3rd 4th and 5th order IMD generated by dual uplink of 13_n2 may fall into own Rx of band n77.

6.50.4	∆TIB and ∆RIB values

For DC_13_n2-n77, the TIB,c and RIB,c values are derived from DC_13-48_n2 and are given in the tables below.

Table 6.50.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13_n2-n77 | 13 | 0.3 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 6.50.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13_n2-n77 | 13 | 0 |
|  | n2 | 0.2 |
|  | n77 | 0.5 |

6.50.5	MSD

As mentioned in 6.50.3, 3rd order IMD generated by dual uplink of 13_n77 may fall into own Rx of band n2.

As mentioned in 6.50.3, 3rd 4th and 5th order IMD generated by dual uplink of 13_n2 may fall into own Rx of band n77.

Table 6.50.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n2A-n77A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n77 | 3444 | 10 | 50 | 3444 | 17.3 | IMD3 |
|  | 13 | 780 | 5 | 25 | 749 | N/A | N/A |
|  | n2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | n77 | 4195 | 10 | 50 | 4195 | 12.4 | IMD4 |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n77 | 4076 | 10 | 50 | 4076 | 4.5 | IMD5 |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 16.0 | IMD3 |
|  | n77 | 3524 | 10 | 50 | 3524 | N/A | N/A |

6.51	DC_13_n5-n48

6.51.1	Operating bands for DC

Table 6.51.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13_n5-n48 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n48 | 3550 MHz | – | 3800 MHz | 3550 MHz | – | 3800 MHz | TDD |

6.51.2	Channel bandwidths per operating band for DC

Table 6.51.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_13A_n5A-n48A | DC_13A_ n48A | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n48 | 15 | Yes5 | Yes | Yes | Yes |  |  | Yes | Yes6 |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.51.3	Co-existence studies

Based on co-existence studies, there are no IMD generated by dual uplink of 13_n48 that fall into own Rx of band n5.

6.51.4	∆TIB and ∆RIB values

For DC_13_n5-n48, the TIB,c and RIB,c values are derived from CA_5-12-48 and are given in the tables below.

Table 6.51.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13_n5-n48 | 13 | 0.4 |
|  | n5 | 0.8 |
|  | n48 | 0.3 |

Table 6.51.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13_n5-n48 | 13 | 0.3 |
|  | n5 | 0.5 |
|  | n48 | 0 |

6.51.5	MSD

There is no need for MSD to be defined.

6.52	DC_13_n48-n66

6.52.1	Operating bands for DC

Table 6.52.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13_n48-n66 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n48 | 3550 MHz | – | 3800 MHz | 3550 MHz | – | 3800 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.52.2	Channel bandwidths per operating band for DC

Table 6.52.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_13A_n48A-n66A | DC_13A_n48A DC_13A_ n66A | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n48 | 15 | Yes5 | Yes | Yes | Yes |  |  | Yes | Yes6 |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.52.3	Co-existence studies

Based on co-existence studies, 5th order IMD generated by dual uplink of 13_n66 may fall into own Rx of band n48.

Based on co-existence studies, 3rd order IMD generated by dual uplink of 13_n48 may fall into own Rx of band n66.

6.52.4	∆TIB and ∆RIB values

For DC_13_n48-n66, the TIB,c and RIB,c values are derived from DC_13-48_n66 and are given in the tables below.

Table 6.52.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13_n48-n66 | 13 | 0.3 |
|  | n48 | 0.8 |
|  | n66 | 0.6 |

Table 6.52.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13_n48-n66 | 13 | 0 |
|  | n48 | 0.5 |
|  | n66 | 0.2 |

6.52.5	MSD

As mentioned in 6.52.3, 5th order IMD generated by dual uplink of 13_n66 may fall into own Rx of band n48. Relaxation is given in the TR [2] and proposed in table below.

As mentioned in 6.52.3, 3rd order IMD generated by dual uplink of 13_n48 may fall into own Rx of band n66. Relaxation is derived from CA_13A-48A-66A and DC_13A-66A_n48A and proposed in table below.

Table 6.52.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n48A-n66A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n48 | 3584 | 5 | 25 | 3584 | 2.8 | IMD5 |
|  | n66 | 1716 | 5 | 25 | 2116 | N/A | N/A |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
|  | n66 | 1731 | 5 | 25 | 2131 | 17.1 | IMD3 |

6.53	DC_13_n66-n77

6.53.1	Operating bands for DC

Table 6.53.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13_n66-n77 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.53.2	Channel bandwidths per operating band for DC

Table 6.53.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_13A_n66A-n77A | DC_13A_n66A DC_13A_ n77A | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.53.3	Co-existence studies

Based on co-existence studies, 3rd order IMD generated by dual uplink of 13_n77 may fall into own Rx of band n66.

Based on co-existence studies, 3rd 4th and 5th order IMD generated by dual uplink of 13_n66 may fall into own Rx of band n77.

6.53.4	∆TIB and ∆RIB values

For DC_13_n66-n77, the TIB,c and RIB,c values are derived from DC_13-48_n66 and are given in the tables below.

Table 6.53.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13_n66-n77 | 13 | 0.3 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 6.53.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13_n66-n77 | 13 | 0 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |

6.53.5	MSD

As mentioned in 6.53.3, 3rd order IMD generated by dual uplink of 13_n77 may fall into own Rx of band n66.

As mentioned in 6.53.3, 3rd 4th and 5th order IMD generated by dual uplink of 13_n66 may fall into own Rx of band n77.

Table 6.53.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n66A-n77A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3324 | 10 | 50 | 3324 | 16.4 | IMD3 |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 4106 | 10 | 50 | 4106 | 11.7 | IMD4 |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3716 | 10 | 50 | 3716 | 4.4 | IMD5 |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n66 | 1731 | 5 | 25 | 2131 | 17.1 | IMD3 |
|  | n77 | 3695 | 10 | 50 | 3695 | N/A | N/A |

6.54	DC_66_n2-n77

6.54.1	Operating bands for DC

Table 6.54.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n2-n77 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.54.2	Channel bandwidths per operating band for DC

Table 6.54.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n2A-n77A | DC_66A_n2A DC_66A_ n77A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.54.3	Co-existence studies

Based on co-existence studies, 2nd, 4th and 5th order IMD generated by dual uplink of 66_n77 may fall into own Rx of band n2.

Based on co-existence studies, 2nd and 4th order IMD generated by dual uplink of 66_n2 may fall into own Rx of band n77.

6.54.4	∆TIB and ∆RIB values

For DC_66_n2-n77, the TIB,c and RIB,c values are derived from CA_n25-n66-n78 and are given in the tables below.

Table 6.54.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n2-n77 | 66 | 0.6 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 6.54.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n2-n77 | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n77 | 0.5 |

6.54.5	MSD

As mentioned in 6.54.3, 2nd 4th and 5th order IMD generated by dual uplink of 66_n77 may fall into own Rx of band n2. Relaxation are derived from DC_2A-66A_n78A and proposed in table below.

As mentioned in 6.54.3, 2nd and 4th order IMD generated by dual uplink of 66_n2 may fall into own Rx of band n77. Relaxation are derived from DC_2A_n66A-n78A and proposed in table below.

Table 6.54.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n2A-n77A | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | 8.9 | IMD4 |
|  | 2 | 1880 | 5 | 25 | 1960 | 32.1 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | 9.1 | IMD4 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | 2.1 | IMD5 |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | N/A |

6.55	DC_66_n5-n48

6.55.1	Operating bands for DC

Table 6.55.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n5-n48 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n48 | 3350 MHz | – | 3700 MHz | 3350 MHz | – | 3700 MHz | TDD |

6.55.2	Channel bandwidths per operating band for DC

Table 6.55.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n5A-n48A | DC_66A_n5A DC_66A_ n48A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n48 | 15 | Yes5 | Yes | Yes | Yes |  |  | Yes | Yes6 |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.55.3	Co-existence studies

Based on co-existence studies, 5th order IMD generated by dual uplink of 66_n5 may fall into own Rx of band n48.

6.55.4	∆TIB and ∆RIB values

For DC_66_n5-n48, the TIB,c and RIB,c values are derived from DC_48-66_n5 and are given in the tables below.

Table 6.55.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n5-n48 | 66 | 0.6 |
|  | n5 | 0.3 |
|  | n48 | 0.8 |

Table 6.55.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n5-n48 | 66 | 0.2 |
|  | n5 | 0 |
|  | n48 | 0.5 |

6.55.5	MSD

As mentioned in 6.55.3, 5th order IMD generated by dual uplink of 66_n5 may fall into own Rx of band n48. Relaxation are defined in Rel-16 TR [2] and proposed in table below.

Table 6.55.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n5A-n48A | 66 | 1750 | 5 | 25 | 2150 | N/A | N/A |
|  | n5 | 834 | 5 | 25 | 879 | N/A | N/A |
|  | n48 | 3582 | 5 | 25 | 3582 | 3.3 | IMD5 |

6.56	DC_66_n5-n77

6.56.1	Operating bands for DC

Table 6.56.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n5-n77 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.56.2	Channel bandwidths per operating band for DC

Table 6.56.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n5A-n77A | DC_66A_n5A DC_66A_ n77A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| DC_66A-66A_n5A-n77A | DC_66A_n5A DC_66A_ n77A | 66 | See CA_66A-66A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.56.3	Co-existence studies

Based on co-existence studies, 3rd 4th and 5th order IMD generated by dual uplink of 66_n5 may fall into own Rx of band n77.

6.56.4	∆TIB and ∆RIB values

For DC_66_n5-n77, the TIB,c and RIB,c values are derived from DC_48-66_n5 and are given in the tables below.

Table 6.56.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n5-n77 | 66 | 0.6 |
|  | n5 | 0.3 |
|  | n77 | 0.8 |

Table 6.56.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n5-n77 | 66 | 0.2 |
|  | n5 | 0 |
|  | n77 | 0.5 |

6.56.5	MSD

As mentioned in 6.56.3, 3rd 4th and 5th order IMD generated by dual uplink of 66_n5 may fall into own Rx of band n77.

Table 6.56.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n5A-n77A DC_66A-66A_n5A-n77A | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n77 | 3420 | 10 | 50 | 3420 | 16.6 | IMD3 |
|  | 66 | 1714 | 5 | 25 | 2114 | N/A | N/A |
|  | n5 | 827 | 5 | 25 | 872 | N/A | N/A |
|  | n77 | 4195 | 10 | 50 | 4195 | 12.1 | IMD4 |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | 3.6 | IMD5 |

6.57	DC_66_n66-n77

6.57.1	Operating bands for DC

Table 6.57.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n66-n77 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.57.2	Channel bandwidths per operating band for DC

Table 6.57.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n66A-n77A | DC_66A_ n77A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.57.3	Co-existence studies

Based on co-existence studies, 2nd and 5th order IMD generated by dual uplink of 66_n77 may fall into own Rx of band n66.

6.57.4	∆TIB and ∆RIB values

For DC_66_n66-n77, the TIB,c and RIB,c values are derived from CA_n66-n77 and are given in the tables below.

Table 6.57.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n66-n77 | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 6.57.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n66-n77 | 66 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |

6.57.5	MSD

As mentioned in 6.57.3, 2nd and 5th order IMD generated by dual uplink of 66_n77 may fall into own Rx of band n66. Relaxation are derived from CA_n66A-n77A and proposed in table below.

Table 6.57.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n66A-n77A | 66 | 1730 | 5 | 25 | 2140 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2170 | 31 | IMD2 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
|  | 66 | 1730 | 5 | 25 | 2140 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2170 | 5.0 | IMD5 |
|  | n77 | 3680 | 10 | 50 | 3680 | N/A | N/A |

6.58	DC_40_n41-n258

6.58.1		Operating bands for DC

Table 6.58.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_40_n41-n258 | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n257 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

    - 6.58.2		Configuration for DC

Table 6.58.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_40A_n41A-n258A DC_40A_n41C-n258A | DC_40A_n41A DC_40A_n258A |

6.58.3		Co-existence studies

- For combination of DC_40_n41, the co-existence studies have been covered in TR37.716-11-11, where band 40 and band n41 are synchronized operaiton.

- For combination of DC_40_n258, it is no need to consider the harmonic and intermodulation co-existence studies due to the large frequency seperation.

6.58.4	∆TIB and ∆RIB values

For the DC conbination of DC_40_n41-n258, the TIB,c and RIB,c for band n258 equals to 0, and the TIB,c and RIB,c for band 40 and n79 are the same with the DC_40_n41.

6.58.5	MSD

No additional MSD requirement is needed.

6.59	DC_40_n79-n258

6.59.1		Operating bands for DC

Table 6.59.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_40_n79-n258 | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

    - 6.59.2		Configuration for DC

Table 6.59.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_40A_n79A-n258A DC_40A_n79C-n258A | DC_40A_n79A DC_40A_n258A |

6.59.3		Co-existence studies

- For combination of DC_40_n79, the co-existence studies have been covered in TR37.716-11-11, where 2nd harmonic from B40 into n79

- For combination of DC_40_n258, it is no need to consider the harmonic and intermodulation co-existence studies due to the large frequency seperation.

6.59.4	∆TIB and ∆RIB values

For the DC conbination of DC_40_n79-n258, the TIB,c and RIB,c for band n258 equals to 0, and the TIB,c and RIB,c for band 40 and n79 are the same with the DC_40_n79.

6.59.5	MSD

No additional MSD requirement is needed.

6.60		DC_41_n79-n258

6.60.1		Operating bands for DC

Table 6.60.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41_n79-n258 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

    - 6.60.2	Configuration for DC

Table 6.60.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_41A_n79A-n258A DC_41A_n79C-n258A | DC_41A_n79A DC_41A_n258A |

6.60.3	Co-existence studies

- For combination of DC_41_n79, the co-existence studies have been covered in TR37.716-11-11, where over 4992MHz frequency in n79 was impacted by 2nd harmonic, but not used.

- For combination of DC_41_n258, it is no need to consider the harmonic and intermodulation co-existence studies due to the large frequency seperation.

6.60.4	∆TIB and ∆RIB values

For the DC conbination of DC_41_n79-n258, the TIB,c and RIB,c for band n258 equals to 0, and the TIB,c and RIB,c for band 41 and n79 are the same with the DC_41_n79.

6.60.5	MSD

No additional MSD requirement is needed.

6.61	DC_8_n78-n258

6.61.1	Operating bands for DC

Table 6.61.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n78-n258 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz |  |

6.61.2	Channel bandwidths per operating band for DC

Table 6.61.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_8A_n78A-n258A DC_8A_n78A-n258D DC_8A_n78A-n258E DC_8A_n78A-n258F DC_8A_n78A-n258G DC_8A_n78A-n258H DC_8A_n78A-n258I DC_8A_n78A-n258J DC_8A_n78A-n258K DC_8A_n78A-n258L DC_8A_n78A-n258M | DC_8A_n78A DC_8A_n258A | 8 | - | 310 410 510 610 710 810 910 |
|  |  | n78 | CA_n78A-n258A/D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n258 |  |  |

6.61.3	Co-existence studies

The co-existence summary in section 5.3 of 37.717-11-21 already captures this combination. There is no additional harmonic and/or intermodulation impact for this combination.

6.61.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

6.61.5	MSD

No additional MSD requirement is needed.

6.62	DC_8_n40-n258

6.62.1	Operating bands for DC

Table 6.62.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n40-n258 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz |  |

6.62.2	Channel bandwidths per operating band for DC

Table 6.62.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_8A_n40A-n258A DC_8A_n40A-n258D DC_8A_n40A-n258E DC_8A_n40A-n258F DC_8A_n40A-n258G DC_8A_n40A-n258H DC_8A_n40A-n258I DC_8A_n40A-n258J DC_8A_n40A-n258K DC_8A_n40A-n258L DC_8A_n40A-n258M | DC_8A_n40A DC_8A_n258A | 8 | - | 290 390 490 590 690 790 890 |
|  |  | n40 | CA_n40A-n258A/D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n258 |  |  |

6.62.3	Co-existence studies

The co-existence summary in section 5.3 of 37.717-11-21 already captures this combination. There is no additional harmonic and/or intermodulation impact for this combination.

6.62.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

6.62.5	MSD

No additional MSD requirement is needed.

6.63	DC_7_n78-n258

6.63.1	Operating bands for DC

Table 6.63.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n78-n258 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.63.2	Channel bandwidths per operating band for DC

Table 6.63.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_7A_n78A-n258A DC_7A_n78A-n258G DC_7A_n78A-n258H DC_7A_n78A-n258I DC_7A_n78A-n258J DC_7A_n78A-n258K DC_7A_n78A-n258L DC_7A_n78A-n258M DC_7C_n78A-n258A DC_7C_n78A-n258G DC_7C_n78A-n258H DC_7C_n78A-n258I DC_7C_n78A-n258J DC_7C_n78A-n258K DC_7C_n78A-n258L DC_7C_n78A-n258M | DC_7A_n78A DC_7A_n258A DC_7C_n78A DC_7C_n258A | 7 | - | 920 |
|  |  | n78 | - |  |
|  |  | n258 | - |  |

6.63.3	Co-existence studies

Based on co-existence studies of DC_7_n78 and DC_7_n258 specified in TR 37.717-11-21, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.63.4	∆TIB and ∆RIB values

For DC_7_n78-n258, the TIB,c and RIB values are given in the tables below.

Table 6.63.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n78-n258 | 7 | 0.5 |
|  | n78 | 0.8 |
|  | n258 | 0 |

Table 6.63.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n78-n258 | 7 | 0 |
|  | n78 | 0.5 |
|  | n258 | 0 |

6.63.5	MSD

There is no additional MSD issue for DC_7_n78-n258.

6.64	DC_3_n78-n258

6.64.1	Operating bands for DC

Table 6.64.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n78-n258 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.64.2	Channel bandwidths per operating band for DC

Table 6.64.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_3A_n78A-n258A DC_3A_n78A-n258G DC_3A_n78A-n258H DC_3A_n78A-n258I DC_3A_n78A-n258J DC_3A_n78A-n258K DC_3A_n78A-n258L DC_3A_n78A-n258M | DC_3A_n78A DC_3A_n258A | 3 | - | 920 |
|  |  | n78 | - |  |
|  |  | n258 | - |  |

6.64.3	Co-existence studies

Based on co-existence studies of DC_3_n78 and DC_3_n258 specified in TR 37.717-11-21, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.64.4	∆TIB and ∆RIB values

For DC_3_n78-n258, the TIB,c and RIB values are given in the tables below.

Table 6.64.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n78-n258 | 3 | 0.6 |
|  | n78 | 0.8 |
|  | n258 | 0 |

Table 6.64.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n78-n258 | 3 | 0.2 |
|  | n78 | 0.5 |
|  | n258 | 0 |

6.64.5	MSD

There is no additional MSD issue for DC_3_n78-n258.

6.65	DC_28_n78-n258

6.65.1	Operating bands for DC

Table 6.65.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n78-n258 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.65.2	Channel bandwidths per operating band for DC

Table 6.65.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_28A_n78A-n258A DC_28A_n78A-n258G DC_28A_n78A-n258H DC_28A_n78A-n258I DC_28A_n78A-n258J DC_28A_n78A-n258K DC_28A_n78A-n258L DC_28A_n78A-n258M | DC_28A_n78A DC_28A_n258A | 28 | - | 920 |
|  |  | n78 | - |  |
|  |  | n258 | - |  |

6.65.3	Co-existence studies

Based on co-existence studies of DC_28_n78 and DC_28_n258 specified in TR 37.717-11-21, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.65.4	∆TIB and ∆RIB values

For DC_28_n78-n258, the TIB,c and RIB values are given in the tables below.

Table 6.65.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n78-n258 | 28 | 0.5 |
|  | n78 | 0.8 |
|  | n258 | 0 |

Table 6.65.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n78-n258 | 28 | 0.2 |
|  | n78 | 0.5 |
|  | n258 | 0 |

6.65.5	MSD

There is no additional MSD issue for DC_28_n78-n258.

6.66	DC_40_n1-n78

### 6.66.1	 Operating bands for DC

Table 6.66.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_40A_n1A-n78A DC_40C_n1A-n78A | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n1 | 1920 MHz |  | 1980 MHz | 1920 MHz |  | 1980 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.66.2 	Configuration for DC

Table 6.66.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_40A_n1A-n78A DC_40C_n1A-n78A |  | 40 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.66.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_40A_n1A and DC_40A_n78A are shown below.

Table 6.66.3-1: The harmonic and IMD products caused by UL DC_40A_n1A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2300 | 2400 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4600 | 4800 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 6900 | 7200 | 5760 | 5940 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 9200 | 9600 | 7680 | 7920 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 11500 | 12000 | 9600 | 9900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 480 | 320 | 4220 | 4380 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2620 | 2880 | 1440 | 1660 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6520 | 6780 | 6140 | 6360 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4920 | 5280 | 3360 | 3640 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8820 | 9180 | 8060 | 8340 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 640 | 960 | 8440 | 8760 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5620 | 5280 | 7680 | 7220 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1340 | 960 | 3360 | 2940 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9980 | 10320 | 11120 | 11580 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10360 | 10740 | 10740 | 11160 |

Table 6.66.3-2: The harmonic and IMD products caused by UL DC_40A_n78A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2300 | 2400 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4600 | 4800 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 6900 | 7200 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 9200 | 9600 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 11500 | 12000 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 900 | 1500 | 5600 | 6200 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 800 | 1500 | 4200 | 5300 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7900 | 8600 | 8900 | 10000 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3100 | 3900 | 7500 | 9100 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10200 | 11000 | 12200 | 13800 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3000 | 1800 | 11200 | 12400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12900 | 10800 | 6300 | 5400 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6800 | 5100 | 600 | 700 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15500 | 17600 | 12500 | 13400 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14500 | 16200 | 13500 | 14800 |

IMD4 and IMD5 caused by UL DC_40_n1 may fall into Rx band n78.

IMD4 caused by UL DC_40_n78 may fall into Rx band n1.

### 6.66.4	 ∆TIB and ∆RIB values

Table 6.66.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_40_n1-n78 | 40 | 0.5 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |

Table 6.66.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_40_n1-n78 | 40 | 0.4 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

### 6.66.5	MSD

All MSD requirements for DC_40A_n1A-n78A can reuse the requriements of DC_1A-40A_n78A and DC_1A_n40A-n78A. The MSD exceptions are shown below.

Table 6.66.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_40A_n1A-n78A DC_40C_n1A-n78A | 40 | 2340 | 5 | 25 | 2340 | N/A | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n78 | 3450 | 10 | 50 | 3450 | 9.8 | IMD4 |
|  | 40 | 2360 | 5 | 25 | 2360 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 9.1 | IMD4 |

6.67	DC_12_n5-n77

6.67.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD5 produced by Band 12 and NR band n5 that impact the reference sensitivity of NR band n77. Also, there is IMD5 produced by Band 12 and NR band n77 that impact the reference sensitivity of NR band n5.

The required MSD levels and test configuration are shown in the following Table.

Table 6.67.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_12A_n5A-n77A | 12 | IMD5 | ¦4*fB12 +fn5¦ | 708 | 5 | 25 | 738 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 3662 | 10 | 50 | 3662 | 10 | 5.4 |
|  | 12 | IMD5 | ¦4*fB12 -fn77¦ | 708 | 5 | 25 | 738 | 5 | N/A |
|  | n77 |  |  | 3707 | 10 | 50 | 3707 | 10 |  |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 | 4.8 |

6.68	DC_14_n5-n77

6.68.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD4 and IMD5 produced by Band 14 and NR band n5 that impact the reference sensitivity of NR band n77. Also, there is IMD5 produced by Band 14 and NR band n77 that impact the reference sensitivity of NR band n5.

The required MSD levels and test configuration are shown in the following Table.

Table 6.68.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_14A_n5A-n77A | 14 | IMD4 | ¦fB14 +3*fn5¦ | 793 | 5 | 25 | 763 | 5 | N/A |
|  | n5 |  |  | 845 | 5 | 25 | 890 | 5 |  |
|  | n77 |  |  | 3328 | 10 | 50 | 3328 | 10 | 10.6 |
|  | 14 | IMD5 | ¦fB14 +4*fn5¦ | 793 | 5 | 25 | 763 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 4113 | 10 | 50 | 4113 | 10 | 5.1 |
|  | 14 | IMD5 | ¦4*fB14 -fn77¦ | 793 | 5 | 25 | 763 | 5 | N/A |
|  | n77 |  |  | 4047 | 10 | 50 | 4047 | 10 |  |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 | 4.4 |

6.69	DC_2_n71-n78

6.69.1	Operating bands for DC

Table 6.69.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n71-n78 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.69.2	Channel bandwidths per operating band for DC

Table 6.69.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n71A-n78A | DC_2A_n71A DC_2A_ n78A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.69.3	Co-existence studies

Based on co-existence studies, IMD3 generated by dual uplink of 2_n71 may fall into own Rx of band n78.

Based on co-existence studies, there are no IMD generated by dual uplink of 2_n78 into own Rx of band n71.

6.69.4	∆TIB and ∆RIB values

For DC_2_n71-n78, the TIB,c and RIB,c values are derived from DC_2-71_n78 and are given in the tables below.

Table 6.69.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n71-n78 | 2 | 0.6 |
|  | n71 | 0.6 |
|  | n78 | 0.8 |

Table 6.69.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n71-n78 | 2 | 0.2 |
|  | n71 | 0.2 |
|  | n78 | 0.5 |

6.69.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD3 produced by Band 2 and NR band n71 that impact the reference sensitivity of NR band n78.

The required MSD levels and test configuration are shown in the following Table.

Table 6.69.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n71A-n78A | 2 | IMD3 | ¦fB2 +2*fn71¦ | 1907.5 | 5 | 25 | 1987.5 | 5 | N/A |
|  | n71 |  |  | 695.5 | 5 | 25 | 649.5 | 5 |  |
|  | n78 |  |  | 3305 | 10 | 50 | 3305 | 10 | 8.0 |

6.70	DC_7_n71-n78

6.70.1	Operating bands for DC

Table 6.70.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n71-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.70.2	Channel bandwidths per operating band for DC

Table 6.70.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_7A_n71A-n78A | DC_7A_n71A DC_7A_ n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.70.3	Co-existence studies

Based on co-existence studies, IMD4 generated by dual uplink of 7_n71 may fall into own Rx of band n78.

Based on co-existence studies, IMD5 generated by dual uplink of 7_n78 may fall into own Rx of band n71.

6.70.4	∆TIB and ∆RIB values

For DC_7_n71-n78, the TIB,c and RIB,c values are derived from DC_41_n28-n78 and are given in the tables below.

Table 6.70.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n71-n78 | 7 | 0.3 |
|  | n71 | 0.5 |
|  | n78 | 0.8 |

Table 6.70.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n71-n78 | 7 | 0 |
|  | n71 | 0.2 |
|  | n78 | 0.5 |

6.70.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD4 produced by Band 7 and NR band n71 that impact the reference sensitivity of NR band n78. Also there is IMD5 produced by Band 7 and NR band n78 that impact the reference sensitivity of NR band n71.

The required MSD levels and test configuration are shown in the following Table.

Table 6.70.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n71A-n78A | 7 | IMD4 | ¦2*fB7 -2*fn71¦ | 2550 | 5 | 25 | 2670 | 5 | N/A |
|  | n71 |  |  | 693 | 5 | 25 | 647 | 5 |  |
|  | n78 |  |  | 3714 | 10 | 50 | 3714 | 10 | 9.7 |
|  | 7 | IMD5 | ¦3*fB7 -2*fn78¦ | 2555 | 5 | 25 | 2675 | 5 | N/A |
|  | n78 |  |  | 3520 | 10 | 50 | 3520 | 10 |  |
|  | n71 |  |  | 671 | 5 | 25 | 625 | 5 | 3.9 |

6.71	DC_71_n38-n78

6.71.1	Operating bands for DC

Table 6.71.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n38-n78 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.71.2	Channel bandwidths per operating band for DC

Table 6.71.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_71A_n38A-n78A | DC_71A_n38A DC_71A_ n78A | 71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.71.3	Co-existence studies

Based on co-existence studies, IMD2 and IMD4 generated by dual uplink of 71_n38 may fall into own Rx of band n78.

Based on co-existence studies, IMD2 generated by dual uplink of 71_n78 may fall into own Rx of band n38.

6.71.4	∆TIB and ∆RIB values

For DC_71_n38-n78, the TIB,c and RIB,c values are derived from DC_41_n28-n78 and are given in the tables below.

Table 6.71.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_71_n38-n78 | 71 | 0.5 |
|  | n38 | 0.3 |
|  | n78 | 0.8 |

Table 6.71.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_71_n38-n78 | 71 | 0.2 |
|  | n38 | 0 |
|  | n78 | 0.5 |

6.71.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD2 and IMD4 produced by Band 71 and NR band n38 that impact the reference sensitivity of NR band n78. Also there is IMD2 produced by Band 71 and NR band n78 that impact the reference sensitivity of NR band n38.

The required MSD levels and test configuration are shown in the following Table.

Table 6.71.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_71A_n38A-n78A | 71 | IMD2 | ¦fB71 +fn38¦ | 693 | 5 | 25 | 647 | 5 | N/A |
|  | n38 |  |  | 2615 | 5 | 25 | 2615 | 5 |  |
|  | n78 |  |  | 3308 | 10 | 50 | 3308 | 10 | 29.1 |
|  | 71 | IMD4 | ¦2*fB71 -2*fn38¦ | 693 | 5 | 25 | 647 | 5 | N/A |
|  | n38 |  |  | 2580 | 5 | 25 | 2580 | 5 |  |
|  | n78 |  |  | 3774 | 10 | 50 | 3774 | 10 | 10.3 |
|  | 71 | IMD2 | ¦fB71 -fn78¦ | 693 | 5 | 25 | 647 | 5 | N/A |
|  | n78 |  |  | 3308 | 10 | 50 | 3308 | 10 |  |
|  | n38 |  |  | 2615 | 5 | 25 | 2615 | 5 | 28.7 |

6.72	DC_5_n66-n78

6.72.1	Operating bands for DC

Table 6.72.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n66-n78 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.72.2	Channel bandwidths per operating band for DC

Table 6.72.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_5A_n66A-n78A | DC_5A_n66A DC_5A_ n78A | 5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.72.3	Co-existence studies

Based on co-existence studies, IMD3 and IMD5 generated by dual uplink of 5_n66 may fall into own Rx of band n78.

Based on co-existence studies, IMD3 generated by dual uplink of 5_n78 may fall into own Rx of band n66.

6.72.4	∆TIB and ∆RIB values

For DC_5_n66-n78, the TIB,c and RIB,c values are derived from DC_5-66_n78 and are given in the tables below.

Table 6.72.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n66-n78 | 71 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |

Table 6.72.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5_n66-n78 | 71 | 0.2 |
|  | n66 | 0.2 |
|  | n78 | 0.5 |

6.72.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD3 and IMD5 produced by Band 5 and NR band n66 that impact the reference sensitivity of NR band n78.

The required MSD levels and test configuration are shown in the following Table.

Table 6.72.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A_n66A-n78A | 5 | IMD3 | ¦fn66 +2*fB5¦ | 830 | 5 | 25 | 875 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n78 |  |  | 3420 | 10 | 50 | 3420 | 10 | 16.6 |
|  | 5 | IMD5 | ¦3*fn66 -2*fB5¦ | 830 | 5 | 25 | 875 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n78 |  |  | 3620 | 10 | 50 | 3620 | 10 | 3.6 |

6.73	DC_71_n66-n78

6.73.1	Operating bands for DC

Table 6.73.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n66-n78 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.73.2	Channel bandwidths per operating band for DC

Table 6.73.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_71A_n66A-n78A | DC_71A_n66A DC_71A_ n78A | 71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.73.3	Co-existence studies

Based on co-existence studies, IMD4 generated by dual uplink of 71_n66 may fall into own Rx of band n78.

Based on co-existence studies, IMD3 generated by dual uplink of 71_n78 may fall into own Rx of band n66.

6.73.4	∆TIB and ∆RIB values

For DC_71_n66-n78, the TIB,c and RIB,c values are derived from DC_66-71_n78 and are given in the tables below.

Table 6.73.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_71_n66-n78 | 71 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |

Table 6.73.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_71_n66-n78 | 71 | 0.2 |
|  | n66 | 0.2 |
|  | n78 | 0.5 |

6.73.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD3 produced by Band 71 and NR band n78 that impact the reference sensitivity of NR band n66.

The required MSD levels and test configuration are shown in the following Table.

Table 6.73.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_71A_n66A-n78A | 71 | IMD3 | ¦2*fB71 -fn78¦ | 693 | 5 | 25 | 647 | 5 | N/A |
|  | n78 |  |  | 3546 | 10 | 50 | 3546 | 10 |  |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 | 15.5 |

6.74	DC_5_n38-n66

6.74.1	Operating bands for DC

Table 6.74.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n38-n66 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.74.2	Channel bandwidths per operating band for DC

Table 6.74.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_5A_n38A-n66A | DC_5A_n38A DC_5A_ n66A | 5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 100 |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

6.74.3	Co-existence studies

Based on co-existence studies, there are no issues with dual uplink of 5_n38 falling into own Rx of band n66.

Based on co-existence studies, IMD2 and IMD3 generated by dual uplink of 5_n66 may fall into own Rx of band n38.

6.74.4	∆TIB and ∆RIB values

For DC_5_n38-n66, the TIB,c and RIB,c values are derived from DC_5-7_n71 and are given in the tables below.

Table 6.74.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n38-n66 | 5 | 0.5 |
|  | n38 | 0.8 |
|  | n66 | 0.5 |

Table 6.74.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5_n38-n66 | 5 | 0.2 |
|  | n38 | 0 |
|  | n66 | 0 |

6.74.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD2 and IMD3 produced by Band 5 and NR band n66 that impact the reference sensitivity of NR band n38.

The required MSD levels and test configuration are shown in the following Table.

Table 6.74.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A_n38A-n66A | 5 | IMD2 | ¦fB5 +fn66¦ | 830 | 5 | 25 | 875 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n38 |  |  | 2590 | 5 | 25 | 2590 | 5 | 28.9 |
|  | 5 | IMD3 | ¦fB5 -2*fn66¦ | 830 | 5 | 25 | 875 | 5 | N/A |
|  | n66 |  |  | 1720 | 5 | 25 | 2120 | 5 |  |
|  | n38 |  |  | 2610 | 5 | 25 | 2610 | 5 | 16.4 |

6.75	DC_12_n2-n38

6.75.1	Operating bands for DC

Table 6.75.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12_n2-n38 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

6.75.2	Channel bandwidths per operating band for DC

Table 6.75.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_12A_n2A-n38A | DC_12A_n2A DC_12A_ n38A | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 70 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

6.75.3	Co-existence studies

Based on co-existence studies, there are no IMD generated by dual uplink of 12_n38 into own Rx of band n2.

Based on co-existence studies, IMD2 generated by dual uplink of 12_n2 may fall into own Rx of band n38.

6.75.4	∆TIB and ∆RIB values

For DC_12_n2-n38, the TIB,c and RIB,c values are derived from DC_2_n41-n71 and are given in the tables below.

Table 6.75.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12_n2-n38 | 12 | 0.3 |
|  | n2 | 0.5 |
|  | n38 | 0.5 |

Table 6.75.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12_n2-n38 | 12 | 0 |
|  | n2 | 0 |
|  | n38 | 0 |

6.75.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD2 produced by Band 12 and NR band n2 that impact the reference sensitivity of NR band n38.

The required MSD levels and test configuration are shown in the following Table.

Table 6.75.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_12A_n2A-n38A | 12 | IMD2 | ¦fn2 +fB12¦ | 708 | 5 | 25 | 738 | 5 | N/A |
|  | n2 |  |  | 1900 | 5 | 25 | 1980 | 5 |  |
|  | n38 |  |  | 2608 | 5 | 25 | 2608 | 5 | 28.7 |

6.76	DC_12_n2-n41

6.76.1	Operating bands for DC

Table 6.76.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12_n2-n41 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.76.2	Channel bandwidths per operating band for DC

Table 6.76.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_12A_n2A-n41A | DC_12A_n2A DC_12A_ n41A | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

6.76.3	Co-existence studies

Based on co-existence studies, there are no IMD generated by dual uplink of 12_n41 into own Rx of band n2.

Based on co-existence studies, IMD2 generated by dual uplink of 12_n2 may fall into own Rx of band n41.

6.76.4	∆TIB and ∆RIB values

For DC_12_n2-n41, the TIB,c and RIB,c values are derived from DC_2_n41-n71 and are given in the tables below.

Table 6.76.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12_n2-n41 | 12 | 0.3 |
|  | n2 | 0.5 |
|  | n41 | 0.5 |

Table 6.76.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12_n2-n41 | 12 | 0 |
|  | n2 | 0 |
|  | n41 | 0 |

6.76.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD2 produced by Band 12 and NR band n2 that impact the reference sensitivity of NR band n41.

The required MSD levels and test configuration are shown in the following Table.

Table 6.76.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_12A_n2A-n41A | 12 | IMD2 | ¦fn2 +fB12¦ | 708 | 5 | 25 | 738 | 5 | N/A |
|  | n2 |  |  | 1900 | 5 | 25 | 1980 | 5 |  |
|  | n41 |  |  | 2608 | 5 | 25 | 2608 | 5 | 28.7 |

6.77	DC_71_n2-n78

6.77.1	Operating bands for DC

Table 6.77.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n2-n78 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.77.2	Channel bandwidths per operating band for DC

Table 6.77.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_71A_n2A-n78A | DC_71A_n2A DC_71A_ n78A | 71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.77.3	Co-existence studies

Based on co-existence studies, IMD3 generated by dual uplink of 71_n2 may fall into own Rx of band n78.

Based on co-existence studies, IMD3 generated by dual uplink of 71_n78 may fall into own Rx of band n2.

6.77.4	∆TIB and ∆RIB values

For DC_71_n2-n78, the TIB,c and RIB,c values are derived from DC_2-71_n78 and are given in the tables below.

Table 6.77.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_71_n2-n78 | 71 | 0.6 |
|  | n2 | 0.6 |
|  | n78 | 0.8 |

Table 6.77.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_71_n2-n78 | 71 | 0.2 |
|  | n2 | 0.2 |
|  | n78 | 0.5 |

6.77.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD3 produced by Band 71 and NR band n2 that impact the reference sensitivity of NR band n78.

The required MSD levels and test configuration are shown in the following Table.

Table 6.77.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_71A_n2A-n78A | n2 | IMD3 | ¦fn2 +2*fB71¦ | 1907.5 | 5 | 25 | 1987.5 | 5 | N/A |
|  | B71 |  |  | 695.5 | 5 | 25 | 649.5 | 5 |  |
|  | n78 |  |  | 3305 | 10 | 50 | 3305 | 10 | 8.0 |

6.78	DC_1_n28-n79

6.78.1	Operating bands for DC

Table 6.78.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n28-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.78.2	Configuration for DC

Table 6.78.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A_n28A-n79A | DC_1A_n28A DC_1A_n79A | 1A | CA_n28A-n79A |

6.78.3	Co-existence studies

Based on the co-existence studies of DC_1_n28 and DC_1_n79, own Rx impact of the 3rd band is the followings.

-	3rd order IMD products generated by DC_1_n28 uplink may fall into own Rx of band n79.

-	5th order IMD products generated by DC_1_n28 uplink may fall into own Rx of band n79.

-	3rd order IMD products generated by DC_1_n79 uplink may fall into own Rx of band n28.

-	4th order IMD products generated by DC_1_n79 uplink may fall into own Rx of band n28.

6.78.4	∆TIB and ∆RIB values

For DC_1_n28-n79, the TIB,c and RIB,c values are reused from DC_1-28_n79, and are given in the tables below.

Table 6.78.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n28-n79 | 1 | 0.3 |
|  | n28 | 0.6 |
|  | n79 | 0 |

Table 6.78.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1_n28-n79 | 1 | 0 |
|  | n28 | 0.2 |
|  | n79 | 0 |

6.78.5	MSD

-	3rd and 5th order IMD products generated by DC_1_n28 uplink may fall into own Rx of band n79.

⇒ The MSD values are shown in the following table. These values are the average of the analysis results of the two companies. [1][2]

-	3rd and 4th order IMD products generated by DC_1_n79 uplink may fall into own Rx of band n28.

⇒ For DC_1_n28-n79, REFSENS exceptions are reused from DC_1-28_n79, already defined in TS 38.101-3. The MSD values are shown in the following table.

Table 6.78.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n28A-n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n28 | 733 | 5 | 25 | 788 | 15.2 | IMD3X1 |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n79 | 4630 | 40 | 216 | 4630 | 14.9 | IMD34 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE X1: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

6.79	DC_3_n28-n79

6.79.1	Operating bands for DC

Table 6.79.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n28-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.79.2	Configuration for DC

Table 6.79.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A_n28A-n79A | DC_3A_n28A DC_3A_n79A | 3A | CA_n28A-n79A |

6.79.3	Co-existence studies

Based on the co-existence studies of DC_3_n28 and DC_3_n79, own Rx impact of the 3rd band is the followings.

-	4th order IMD products generated by DC_3_n28 uplink may fall into own Rx of band n79.

-	5th order IMD products generated by DC_3_n28 uplink may fall into own Rx of band n79.

-	4th order IMD products generated by DC_3_n79 uplink may fall into own Rx of band n28.

6.79.4	∆TIB and ∆RIB values

For DC_3_n28-n79, the TIB,c and RIB,c values are reused from DC_3-28_n79, and are given in the tables below.

Table 6.79.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n28-n79 | 3 | 0.3 |
|  | n28 | 0.3 |
|  | n79 | 0 |

Table 6.79.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3_n28-n79 | 3 | 0 |
|  | n28 | 0 |
|  | n79 | 0 |

6.79.5	MSD

-	4th and 5th order IMD products generated by DC_3_n28 uplink may fall into own Rx of band n79.

⇒ The MSD values are shown in the following table. These values are the average of the analysis results of the two companies. [1][2]

-	4th order IMD products generated by DC_3_n79 uplink may fall into own Rx of band n28.

⇒ For DC_3_n28-n79, REFSENS exceptions are reused from DC_3-28_n79, already defined in TS 38.101-3. The MSD values are shown in the following table.

Table 6.79.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n28A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 10.3 | IMD4 |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | N/A |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | N/A |
|  | n79 | 4585 | 40 | 216 | 4585 | 9.4 | IMD44 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

6.80	DC_21_n28-n77

6.80.1	Operating bands for DC

The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783-793 MHz for the DL because only a certain operator uses band 21. This restriction is mentioned in TP for DC_21_n28 (R4-2100352).

Table 6.80.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21_n28-n77 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.80.2	Configuration for DC

Table 6.80.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_21A_n28A-n77A | DC_21A_n28A DC_21A_n77A | 21A | CA_n28A-n77A |

6.80.3	Co-existence studies

Based on the co-existence studies of DC_21_n28 and DC_21_n77, own Rx impact of the 3rd band is the followings.

-	3rd order IMD products generated by DC_21_n28 uplink may fall into own Rx of band n77.

-	4th order IMD products generated by DC_21_n28 uplink may fall into own Rx of band n77.

-	3rd order IMD products generated by DC_21_n77 uplink may fall into own Rx of band n28.

-	4th order IMD products generated by DC_21_n77 uplink may fall into own Rx of band n28.

6.80.4	∆TIB and ∆RIB values

For DC_21_n28-n77, the TIB,c and RIB,c values are reused from DC_21-28_n77, and are given in the tables below.

Table 6.80.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21_n28-n77 | 21 | 0.4 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |

Table 6.80.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_21_n28-n77 | 21 | 0.5 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

6.80.5	MSD

-	3rd and 4th order IMD products generated by DC_21_n28 uplink may fall into own Rx of band n77.

⇒ The MSD values are shown in the following table. These values are the average of the analysis results of the two companies. [1][2]

-	3rd and 4th order IMD products generated by DC_21_n77 uplink may fall into own Rx of band n28.

⇒ For DC_21_n28-n77, REFSENS exceptions are reused from DC_21-28_n77, already defined in TS 38.101-3. The MSD values are shown in the following table.

Table 6.80.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_21A_n28A-n77A | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |
|  | n28 | 730.5 | 5 | 25 | 785.5 | 16.9 | IMD3X1 |
|  | n77 | 3689.5 | 10 | 50 | 3689.5 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |
|  | n28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |
|  | n77 | 3634.5 | 10 | 50 | 3634.5 | 17.3 | IMD3X1 |
| NOTE X1: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

6.81	DC_21_n28-n78

6.81.1	Operating bands for DC

The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783-793 MHz for the DL because only a certain operator uses band 21. This restriction is mentioned in TP for DC_21_n28 (R4-2100352).

Table 6.81.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21_n28-n78 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.81.2	Configuration for DC

Table 6.81.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_21A_n28A-n78A | DC_21A_n28A DC_21A_n78A | 21A | CA_n28A-n78A |

6.81.3	Co-existence studies

Based on the co-existence studies of DC_21_n28 and DC_21_n78, own Rx impact of the 3rd band is the followings.

-	3rd order IMD products generated by DC_21_n28 uplink may fall into own Rx of band n78.

-	4th order IMD products generated by DC_21_n28 uplink may fall into own Rx of band n78.

-	3rd order IMD products generated by DC_21_n78 uplink may fall into own Rx of band n28.

-	4th order IMD products generated by DC_21_n78 uplink may fall into own Rx of band n28.

6.81.4	∆TIB and ∆RIB values

For DC_21_n28-n78, the TIB,c and RIB,c values are reused from DC_21-28_n78, and are given in the tables below.

Table 6.81.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21_n28-n78 | 21 | 0.4 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 6.81.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_21_n28-n78 | 21 | 0.5 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

6.81.5	MSD

-	3rd and 4th order IMD products generated by DC_21_n28 uplink may fall into own Rx of band n78.

⇒ The MSD values are shown in the following table. These values are the average of the analysis results of the two companies. [1][2]

-	3rd and 4th order IMD products generated by DC_21_n78 uplink may fall into own Rx of band n28.

⇒ For DC_21_n28-n78, REFSENS exceptions are reused from DC_21-28_n78, already defined in TS 38.101-3. The MSD values are shown in the following table.

Table 6.81.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_21A_n28A-n78A | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |
|  | n28 | 730.5 | 5 | 25 | 785.5 | 16.9 | IMD3X1 |
|  | n78 | 3689.5 | 10 | 50 | 3689.5 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |
|  | n28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |
|  | n78 | 3634.5 | 10 | 50 | 3634.5 | 17.3 | IMD3X1 |
| NOTE X1: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

6.82	DC_66_n71-n78

6.82.1	Operating bands for DC

Table 6.82.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n71-n78 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.82.2	Channel bandwidths per operating band for DC

Table 6.82.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n71A-n78A | DC_66A_n71A DC_66A_ n78A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.82.3	Co-existence studies

Based on co-existence studies, IMD4 and IMD5 generated by dual uplink of 66_n71 may fall into own Rx of band n78.

6.82.4	∆TIB and ∆RIB values

For DC_66_n71-n78, the TIB,c and RIB,c values are derived from DC_66-71_n78 and are given in the tables below.

Table 6.82.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n71-n78 | 66 | 0.6 |
|  | n71 | 0.6 |
|  | n78 | 0.8 |

Table 6.82.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n71-n78 | 66 | 0.2 |
|  | n71 | 0.2 |
|  | n78 | 0.5 |

6.82.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD4 and IMD5 produced by Band 66 and NR band n71 that impact the reference sensitivity of NR band n78.

The required MSD levels and test configuration are shown in the following Table. IMD4 as in DC_48-66_n71 and IMD5 as in DC_3_n28-n78.

Table 6.82.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n71A-n78A | n71 | IMD4 | ¦3*fB71 + fn66¦ | 665.5 | 5 | 25 | 619.5 | 5 | N/A |
|  | n78 |  |  | 3709 | 5 | 25 | 3709 | 10 | 13.0 |
|  | 66 |  |  | 1712.5 | 5 | 25 | 2112.5 | 5 | N/A |
|  | n71 | IMD5 | ¦2*fn71 -3*fB66¦ | 695 | 5 | 25 | 649 | 5 | N/A |
|  | n78 |  |  | 3755 | 10 | 50 | 3755 | 10 | 4.5 |
|  | 66 |  |  | 1715 | 5 | 25 | 2115 | 5 | N/A |

6.83	DC_2_n38-n71

6.83.1	Operating bands for DC

Table 6.83.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n38-n71 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

6.83.2	Channel bandwidths per operating band for DC

Table 6.83.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n38A-n71A | DC_2A_n38A DC_2A_ n71A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.83.3	Co-existence studies

Based on co-existence studies, there are no IMD generated by dual uplink of 2_n38 into own Rx of band n71.

Based on co-existence studies, IMD2 generated by dual uplink of 2_n71 may fall into own Rx of band n38.

6.83.4	∆TIB and ∆RIB values

For DC_2_n38-n71, the TIB,c and RIB,c values are derived from DC_2_n41-n71 and are given in the tables below.

Table 6.83.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n38-n71 | 2 | 0.5 |
|  | n38 | 0.5 |
|  | n71 | 0.3 |

Table 6.83.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n38-n71 | 2 | 0 |
|  | n38 | 0 |
|  | n71 | 0 |

6.83.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD2 produced by Band 2 and NR band n71 that impact the reference sensitivity of NR band n38.

The required MSD levels and test configuration are shown in the following Table. IMD2 as in DC_2_n41-n71.

Table 6.83.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n38A-n71A | 2 | IMD2 | ¦fB2 +fn71¦ | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n38 |  |  | 2586 | 5 | 25 | 2586 | 5 | 29.2 |
|  | n71 |  |  | 686 | 5 | 25 | 640 | 5 | N/A |

6.84	DC_66_n38-n71

6.84.1	Operating bands for DC

Table 6.84.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n38-n71 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

6.84.2	Channel bandwidths per operating band for DC

Table 6.84.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n38A-n71A | DC_66A_n38A DC_66A_ n71A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.84.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

6.84.4	∆TIB and ∆RIB values

For DC_66_n38-n71, the TIB,c and RIB,c values are derived from DC_66-71_n38 and are given in the tables below.

Table 6.84.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n38-n71 | 66 | 0.5 |
|  | n38 | 0.8 |
|  | n71 | 0.5 |

Table 6.84.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n38-n71 | 66 | 0.5 |
|  | n38 | 0.5 |
|  | n71 | 0.5 |

6.84.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

6.85	DC_71_n38-n66

6.85.1	Operating bands for DC

Table 6.85.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n38-n66 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.85.2	Channel bandwidths per operating band for DC

Table 6.85.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_71A_n38A-n66A | DC_71A_n38A DC_71A_ n66A | 71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 100 |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

6.85.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

6.85.4	∆TIB and ∆RIB values

For DC_71_n38-n66, the TIB,c and RIB,c values are derived from DC_66-71_n38 and are given in the tables below.

Table 6.85.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_71_n38-n66 | 71 | 0.5 |
|  | n38 | 0.8 |
|  | n66 | 0.5 |

Table 6.85.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_71_n38-n66 | 71 | 0.5 |
|  | n38 | 0.5 |
|  | n66 | 0.5 |

6.85.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

6.86	DC_2_n2-n38

6.86.1	Operating bands for DC

Table 6.86.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n2-n38 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

6.86.2	Channel bandwidths per operating band for DC

Table 6.86.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n2A-n38A | DC_2A_ n38A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

6.86.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

6.86.4	∆TIB and ∆RIB values

For DC_2_n2-n38, the TIB,c and RIB,c values are derived from DC_2_n38 and are given in the tables below.

Table 6.86.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n2-n38 | 2 | 0.5 |
|  | n2 | 0.5 |
|  | n38 | 0.9 |

Table 6.86.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n2-n38 | 2 | 0 |
|  | n2 | 0 |
|  | n38 | 0 |

6.86.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

6.87	DC_66_n2-n38

6.87.1	Operating bands for DC

Table 6.87.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n2-n38 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

6.87.2	Channel bandwidths per operating band for DC

Table 6.87.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n2A-n38A | DC_66A_n2A DC_66A_ n38A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

6.87.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

6.87.4	∆TIB and ∆RIB values

For DC_66_n2-n38, the TIB,c and RIB,c values are derived from DC_2-66_n38 and are given in the tables below.

Table 6.87.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n2-n38 | 66 | 0.5 |
|  | n2 | 0.5 |
|  | n38 | 0.9 |

Table 6.87.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n2-n38 | 66 | 0.5 |
|  | n2 | 0.3 |
|  | n38 | 0.5 |

6.87.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

6.88	DC_2_n2-n41

6.88.1	Operating bands for DC

Table 6.88.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n2-n41 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.88.2	Channel bandwidths per operating band for DC

Table 6.88.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n2A-n41A | DC_2A_ n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

6.88.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

6.88.4	∆TIB and ∆RIB values

For DC_2_n2-n41, the TIB,c and RIB,c values are derived from DC_2_n38 and are given in the tables below.

Table 6.88.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n2-n41 | 2 | 0.5 |
|  | n2 | 0.5 |
|  | n41 | 0.5 |

Table 6.88.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n2-n41 | 2 | 0 |
|  | n2 | 0 |
|  | n41 | 0 |

6.88.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

6.89	DC_71_n2-n41

6.89.1	Operating bands for DC

Table 6.89.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n2-n41 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.89.2	Channel bandwidths per operating band for DC

Table 6.89.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_71A_n2A-n41A | DC_71A_n2A DC_71A_ n41A | 71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

6.89.3	Co-existence studies

Based on co-existence studies, IMD2 generated by dual uplink of 71_n2 may fall into own Rx of band n41.

Based on co-existence studies, IMD2 generated by dual uplink of 71_n41 may fall into own Rx of band n2.

6.89.4	∆TIB and ∆RIB values

For DC_71_n2-n41, the TIB,c and RIB,c values are derived from DC_2_n41-n71 and are given in the tables below.

Table 6.89.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_71_n2-n41 | 71 | 0.3 |
|  | n2 | 0.5 |
|  | n41 | 0.5 |

Table 6.89.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_71_n2-n41 | 71 | 0 |
|  | n2 | 0 |
|  | n41 | 0 |

6.89.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD2 produced by Band 71 and NR band n2 that impact the reference sensitivity of NR band n41. The required MSD levels and test configuration are shown in the following Table and with values as in DC_2_n41-n71.

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD2 produced by Band 71 and NR band n41 that impact the reference sensitivity of NR band n2. The required MSD levels and test configuration are shown in the following Table and with values as in DC_2-71_n38.

Table 6.89.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_71A_n2A-n41A | n2 | IMD2 | ¦fB71 +fn2¦ | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n41 |  |  | 2586 | 5 | 25 | 2586 | 10 | 29.2 |
|  | 71 |  |  | 686 | 5 | 50 | 640 | 5 | N/A |
|  | n2 | IMD2 | ¦ fn41 - fB71¦ | 1862 | 5 | 25 | 1942 | 5 | 26 |
|  | n41 |  |  | 2610 | 5 | 25 | 2610 | 10 | N/A |
|  | 71 |  |  | 668 | 5 | 25 | 622 | 5 | N/A |

6.90	DC_7_n2-n78

6.90.1	Operating bands for DC

Table 6.90.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n2-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.90.2	Channel bandwidths per operating band for DC

Table 6.90.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_7A_n2A-n78A | DC_7A_n2A DC_7A_ n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.90.3	Co-existence studies

Based on co-existence studies, IMD5 generated by dual uplink of 7_n2 may fall into own Rx of band n78.

Based on co-existence studies, IMD4 generated by dual uplink of 7_n78 may fall into own Rx of band n2.

6.90.4	∆TIB and ∆RIB values

For DC_7_n2-n78, the TIB,c and RIB,c values are derived from DC_2_n7-n78 and are given in the tables below.

Table 6.90.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n2-n78 | 7 | 0.5 |
|  | n2 | 0.6 |
|  | n78 | 0.8 |

Table 6.90.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n2-n78 | 7 | 0.5 |
|  | n2 | 0.2 |
|  | n78 | 0.5 |

6.90.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD4 generated by dual uplink of 7_n78 may fall into own Rx of band n2. MSD of DC_2A-7A_n78A is reused

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD5 generated by dual uplink of 7_n2 may fall into own Rx of band n78. MSD of DC_2A_n7A-n78A is reused.

The required MSD levels and test configuration are shown in the following Table.

Table 6.90.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n2A-n78A | B7 | IMD4 | ¦2*fB7 -2*fn78¦ | 2550 | 5 | 25 | 2685 | 5 | N/A |
|  | n2 |  |  | 1870 | 5 | 25 | 1950 | 5 | 8.6 |
|  | n78 |  |  | 3525 | 10 | 50 | 3525 | 10 | N/A |
|  | B7 | IMD5 | ¦2*fn2 -3*fB7¦ | 2525 | 5 | 25 | 2645 | 5 | N/A |
|  | n2 |  |  | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n78 |  |  | 3775 | 10 | 50 | 3775 | 10 | 4.2 |

6.91	DC_2_n66-n71

6.91.1	Operating bands for DC

Table 6.91.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n66-n71 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

6.91.2	Channel bandwidths per operating band for DC

Table 6.91.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n66A-n71A | DC_2A_n66A DC_2A_ n71A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.91.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

6.91.4	∆TIB and ∆RIB values

For DC_2_n66-n71, the TIB,c and RIB,c values are derived from DC_2-66_n71 and are given in the tables below.

Table 6.91.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n66-n71 | 2 | 0.5 |
|  | n66 | 0.5 |
|  | n71 | 0.3 |

Table 6.91.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n66-n71 | 2 | 0.3 |
|  | n66 | 0.3 |
|  | n71 | 0 |

6.91.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

6.92	DC_7_n66-n71

6.92.1	Operating bands for DC

Table 6.92.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n66-n71 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

6.92.2	Channel bandwidths per operating band for DC

Table 6.92.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_7A_n66A-n71A | DC_7A_n66A DC_7A_ n71A | 71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.92.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

6.92.4	∆TIB and ∆RIB values

For DC_7_n66-n71, the TIB,c and RIB,c values are derived from DC_7-66_n71 and are given in the tables below.

Table 6.92.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n66-n71 | 7 | 0.5 |
|  | n66 | 0.5 |
|  | n71 | 0.5 |

Table 6.92.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n66-n71 | 7 | 0.5 |
|  | n66 | 0.5 |
|  | n71 | 0.1 |

6.92.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

6.93	DC_66_n66-n71

6.93.1	Operating bands for DC

Table 6.93.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n66-n71 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

6.93.2	Channel bandwidths per operating band for DC

Table 6.93.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n66A-n71A | DC_66A_n66A DC_66A_ n71A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.93.3	Co-existence studies

Based on co-existence studies, IMD4 generated by dual uplink of 66_n71 may fall into own Rx of band n66.

6.93.4	∆TIB and ∆RIB values

For DC_66_n66-n71, the TIB,c and RIB,c values are derived from DC_66_n71 and are given in the tables below.

Table 6.93.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n66-n71 | 66 | 0.3 |
|  | n66 | 0.3 |
|  | n71 | 0.3 |

Table 6.93.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n66-n71 | 66 | 0 |
|  | n66 | 0 |
|  | n71 | 0 |

6.93.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD4 generated by dual uplink of 66_n71 may fall into own Rx of band n66. MSD of DC_66A_n71A is reused.

The required MSD levels and test configuration are shown in the following Table.

Table 6.93.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n66A-n71A | B66 | IMD4 | ¦2*fn71 -2*fB66¦ | 1752 | 5 | 25 | 2152 | 5 | N/A |
|  | n66 |  |  | 1718 | 5 | 25 | 2118 | 5 | 5.0 |
|  | n71 |  |  | 693 | 5 | 25 | 647 | 5 | N/A |

6.94	DC_2_n2-n71

6.94.1	Operating bands for DC

Table 6.94.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n2-n71 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

6.94.2	Channel bandwidths per operating band for DC

Table 6.94.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n2A-n71A | DC_2A_ n71A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 60 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.94.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

6.94.4	∆TIB and ∆RIB values

For DC_2_n2-n71, the TIB,c and RIB,c values are derived from DC_2_n71 and are given in the tables below.

Table 6.94.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n2-n71 | 2 | 0.3 |
|  | n2 | 0.3 |
|  | n71 | 0.3 |

Table 6.94.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n2-n71 | 2 | 0 |
|  | n2 | 0 |
|  | n71 | 0 |

6.94.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

6.95	DC_7_n2-n71

6.95.1	Operating bands for DC

Table 6.95.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n2-n71 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

6.95.2	Channel bandwidths per operating band for DC

Table 6.95.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_7A_n2A-n71A | DC_7A_n2A DC_7A_ n71A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 60 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.95.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are IMD2 and IMD5 generated by dual uplink of 7_n2 that may fall into own Rx of band n71.

6.95.4	∆TIB and ∆RIB values

For DC_7_n2-n71, the TIB,c and RIB,c values are derived from DC_2-66_n71 and are given in the tables below.

Table 6.95.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n2-n71 | 7 | 0.5 |
|  | n2 | 0.5 |
|  | n71 | 0.3 |

Table 6.95.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n2-n71 | 7 | 0.3 |
|  | n2 | 0.3 |
|  | n71 | 0 |

6.95.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are IMD2 and IMD5 generated by dual uplink of 7_n2 that may fall into own Rx of band n71. MSD IMD2 of DC_2A_n41A-n71A and MSD IMD5 of CA_n1A-n7A-n28A are reused.

The required MSD levels and test configuration are shown in the following Table.

Table 6.95.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n2A-n71A | B7 | IMD2 | ¦fB7 -fn2¦ | 2530 | 5 | 25 | 2530 | 5 | N/A |
|  | n2 |  |  | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n71 |  |  | 676 | 5 | 25 | 630 | 5 | 28.7 |
|  | B7 | IMD5 | ¦2*f B7 -3*fn2¦ | 2530 | 5 | 25 | 2530 | 5 | N/A |
|  | n2 |  |  | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n71 |  |  | 686 | 5 | 25 | 640 | 5 | 4.5 |

6.96	DC_66_n2-n71

6.96.1	Operating bands for DC

Table 6.96.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n2-n71 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

6.96.2	Channel bandwidths per operating band for DC

Table 6.96.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n2A-n71A | DC_66A_n2A DC_66A_ n71A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 60 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.96.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

6.96.4	∆TIB and ∆RIB values

For DC_66_n2-n71, the TIB,c and RIB,c values are derived from DC_2-66_n71 and are given in the tables below.

Table 6.96.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n2-n71 | 66 | 0.5 |
|  | n2 | 0.5 |
|  | n71 | 0.3 |

Table 6.96.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n2-n71 | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n71 | 0 |

6.96.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

6.97	DC_2_n2-n66

6.97.1	Operating bands for DC

Table 6.97.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n2-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.97.2	Channel bandwidths per operating band for DC

Table 6.97.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n2A-n66A | DC_2A_ n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

6.97.3	Co-existence studies

Based on co-existence studies, IMD3 and IMD5 generated by dual uplink of 2_n66 may fall into own Rx of band n2.

6.97.4	∆TIB and ∆RIB values

For DC_2_n2-n66, the TIB,c and RIB,c values are derived from DC_2_n66 and are given in the tables below.

Table 6.97.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n2-n66 | 2 | 0.5 |
|  | n2 | 0.5 |
|  | n66 | 0.5 |

Table 6.97.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n2-n66 | 2 | 0.3 |
|  | n2 | 0.3 |
|  | n66 | 0.3 |

6.97.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD3 and IMD5 generated by dual uplink of 2_n66 may fall into own Rx of band n2.

For IMD3 MSD of DC_2A_n66A is reused.

IMD5 from dual uplink of 2_n66 can only fall into own Rx of band n2 using the band edge frequencies 1780 (n66 UL) and 1850 (B2 UL). Therefore, test points for MSD for IMD5 cannot be defined.

The required MSD levels and test configuration are shown in the following Table.

Table 6.97.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n2A-n66A | B2 | IMD3 | ¦2*fB2 -fn66¦ | 1875 | 5 | 25 | 1955 | 5 | N/A |
|  | n2 |  |  | 1895 | 5 | 25 | 1975 | 5 | 20 |
|  | n66 |  |  | 1775 | 5 | 25 | 2175 | 5 | N/A |

6.98	DC_7_n2-n66

6.98.1	Operating bands for DC

Table 6.98.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n2-n66 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.98.2	Channel bandwidths per operating band for DC

Table 6.98.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_7A_n2A-n66A | DC_7A_n2A DC_7A_ n66A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

6.98.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

6.98.4	∆TIB and ∆RIB values

For DC_7_n2-n66, the TIB,c and RIB,c values are derived from DC_2-7_n66 and are given in the tables below.

Table 6.98.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n2-n66 | 7 | 0.5 |
|  | n2 | 0.5 |
|  | n66 | 0.5 |

Table 6.98.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n2-n66 | 7 | 0.5 |
|  | n2 | 0.3 |
|  | n66 | 0.5 |

6.98.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

6.99	DC_71_n2-n66

6.99.1	Operating bands for DC

Table 6.99.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n2-n66 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.99.2	Channel bandwidths per operating band for DC

Table 6.99.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_71A_n2A-n66A | DC_71A_n2A DC_71A_ n66A | 71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

6.99.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

6.99.4	∆TIB and ∆RIB values

For DC_71_n2-n66, the TIB,c and RIB,c values are derived from DC_2-66_n71 and are given in the tables below.

Table 6.99.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_71_n2-n66 | 71 | 0.3 |
|  | n2 | 0.5 |
|  | n66 | 0.5 |

Table 6.99.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_71_n2-n66 | 71 | 0 |
|  | n2 | 0.3 |
|  | n66 | 0.3 |

6.99.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

6.100	DC_21_n28-n79

6.100.1	Operating bands for DC

The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783-793 MHz for the DL because only a certain operator uses band 21. This restriction is mentioned in TP for DC_21_n28 (R4-2100352).

Table 6.100.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21_n28-n79 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.100.2	Configuration for DC

Table 6.100.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_21A_n28A-n79A | DC_21A_n28A DC_21A_n79A | 21A | CA_n28A-n79A |

6.100.3	Co-existence studies

Based on the co-existence studies of DC_21_n28 and DC_21_n79, own Rx impact of the 3rd band is the followings.

-	4th order IMD products generated by DC_21_n28 uplink may fall into own Rx of band n79.

-	5th order IMD products generated by DC_21_n28 uplink may fall into own Rx of band n79.

-	5th order IMD products generated by DC_21_n79 uplink may fall into own Rx of band n28.

6.100.4	∆TIB and ∆RIB values

For DC_21_n28-n79, the TIB,c and RIB,c values are reused from DC_21-28_n79, and are given in the tables below.

Table 6.100.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21_n28-n79 | 21 | 0.4 |
|  | n28 | 0.3 |
|  | n79 | 0 |

Table 6.100.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_21_n28-n79 | 21 | 0 |
|  | n28 | 0 |
|  | n79 | 0 |

6.100.5	MSD

-	4th and 5th order IMD products generated by DC_21_n28 uplink may fall into own Rx of band n79.

⇒ The MSD values are shown in the following table. These values are the average of the analysis results of the two companies. [1][2]

-	5th order IMD products generated by DC_21_n79 uplink may fall into own Rx of band n28.

⇒ The MSD values are shown in the following table. These values are the average of the analysis results of the two companies. [1][2]

Table 6.100.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_21A_n28A-n79A | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n28 | 735.5 | 5 | 25 | 790.5 | 2.8 | IMD5 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
|  | 21 | 1460.4 | 5 | 25 | 1508.4 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | [6.3] | IMD44 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

6.101	DC_8_n28-n78

6.101.1	 Operating bands for DC

Table 6.101.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8A_n28A-n78A | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz |  | 748 MHz | 758 MHz |  | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.101.2 	Configuration for DC

Table 6.101.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_8A_n28A-n78A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.101.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_8A_n28A and DC_8A_n78A are shown below.

Table 6.101.3-1: The harmonic and IMD products caused by UL DC_8A_n28A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 2109 | 2244 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3520 | 3660 | 2812 | 2992 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4400 | 4575 | 3515 | 3740 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 212 | 132 | 1583 | 1663 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1012 | 1127 | 491 | 616 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2463 | 2578 | 2286 | 2411 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1892 | 2042 | 1194 | 1364 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3343 | 3493 | 2989 | 3159 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 264 | 424 | 3166 | 3326 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2112 | 1897 | 2957 | 2772 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 484 | 279 | 1339 | 1144 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3692 | 3907 | 4223 | 4408 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3869 | 4074 | 4046 | 4241 |

Table 6.101.3-2: The harmonic and IMD products caused by UL DC_8A_n78A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3520 | 3660 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4400 | 4575 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2385 | 2920 | 4180 | 4715 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2040 | 1470 | 5685 | 6720 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5060 | 5630 | 7480 | 8515 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1160 | 555 | 8985 | 10520 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5940 | 6545 | 10780 | 12315 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5840 | 4770 | 8360 | 9430 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14320 | 12285 | 360 | 280 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9640 | 8070 | 3855 | 4960 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14080 | 16115 | 6820 | 7460 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11660 | 13230 | 9240 | 10345 |

IMD4 and IMD5 caused by UL DC_8_n28 may fall into Rx band n78.

IMD4 caused by UL DC_8_n78 may fall into Rx band n28.

6.101.4	 ∆TIB and ∆RIB values

Table 6.101.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8A_n28A-n78A | 8 | 0.6 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 6.101.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8A_n28A-n78A | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

6.101.5	MSD

All MSD requirements for DC_8A_n28A-n78A can reuse the requriements of DC_8A_n28A-n77A. The MSD exceptions are shown below.

Table 6.101.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A_n28A-n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | N/A |
|  | n78 | 3455 | 10 | 50 | 3455 | 10.3 | IMD4 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n28 | 710 | 5 | 25 | 765 | 11.6 | IMD4 |
|  | n78 | 3495 | 10 | 50 | 3495 | N/A | N/A |

6.102	DC_2_n2-n78

6.102.1	Operating bands for DC

Table 6.102.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n2-n78 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.102.2	Channel bandwidths per operating band for DC

Table 6.102.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n2A-n78A | DC_2A_ n78A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.102.3	Co-existence studies

Based on co-existence studies, IMD2, IMD4 and IMD5 generated by dual uplink of 2_n78 may fall into own Rx of band n2.

6.102.4	∆TIB and ∆RIB values

For DC_2_n2-n78, the TIB,c and RIB,c values are derived from DC_2_n78 and are given in the tables below.

Table 6.102.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n2-n78 | 2 | 0.6 |
|  | n2 | 0.6 |
|  | n78 | 0.8 |

Table 6.102.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n2-n78 | 2 | 0.2 |
|  | n2 | 0.2 |
|  | n78 | 0.5 |

6.102.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD2, IMD3 and IMD5 generated by dual uplink of 2_n78 may fall into own Rx of band n2. MSD of DC_2A_n78A is reused.

The required MSD levels and test configuration are shown in the following Table.

Table 6.102.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n2A-n78A | B2 | IMD23 | ¦2*fn78 -fB2¦ | 1852.5 | 5 | 25 | 1932.5 | 5 | N/A |
|  | n2 |  |  | 1862.5 | 5 | 25 | 1942.5 | 5 | 26 |
|  | n78 |  |  | 3795 | 10 | 50 | 3795 | 10 | N/A |
| DC_2A_n2A-n78A | B2 | IMD43 | ¦3*fB2 -fn78¦ | 1885 | 5 | 25 | 1965 | 5 | N/A |
|  | n2 |  |  | 1865 | 5 | 25 | 1945 | 5 | 8.0 |
|  | n78 |  |  | 3710 | 10 | 50 | 3710 | 10 | N/A |
| NOTE 3: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |

6.103	DC_66_n2-n66

6.103.1	Operating bands for DC

Table 6.103.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n2-n66 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.103.2	Channel bandwidths per operating band for DC

Table 6.103.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n2A-n66A | DC_66A_ n2A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

6.103.3	Co-existence studies

Based on co-existence studies

- IMD5 generated by dual uplink of 66_n2 may fall into own Rx of band n66

- IMD3 generated bv dual uplink of 66_n2 may fall into own Rx of band n2.

6.103.4	∆TIB and ∆RIB values

For DC_66_n2-n66, the TIB,c and RIB,c values are derived from DC_66_n66 and are given in the tables below.

Table 6.103.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n2-n66 | 66 | 0.5 |
|  | n2 | 0.5 |
|  | n66 | 0.5 |

Table 6.103.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n2-n66 | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n66 | 0.3 |

6.103.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD5 generated by dual uplink of 66_n2 that may fall into own Rx of band n66 and IMD3 generated bv dual uplink of 66_n2 that may fall into own Rx of band n2. MSD of DC_2A_n66A is reused.

The required MSD levels and test configuration are shown in the following Table.

Table 6.103.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n2A-n66A | B66 | IMD3 | ¦2*fn2 -fB66¦ | 1775 | 5 | 25 | 2175 | 5 | N/A |
|  | n2 |  |  | 1855 | 5 | 25 | 1935 | 5 | 20 |
|  | n66 |  |  | 1720 | 5 | 25 | 2120 | 5 | N/A |
|  | B66 | IMD5 | ¦2*fB66 -3*fn2¦ | 1720 | 5 | 25 | 2120 | 5 | N/A |
|  | n2 |  |  | 1870 | 5 | 25 | 1950 | 5 | N/A |
|  | n66 |  |  | 1770 | 5 | 25 | 2170 | 5 | 4.0 |

6.104	DC_39_n40-n258

6.104.1		Operating bands for DC

Table 6.104.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_39_n40-n258 | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n257 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.104.2		Configuration for DC

Table 6.104.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_39A_n40A-n258A | DC_39A_n40A DC_39A_n258A |

6.104.3		Co-existence studies

- For combination of DC_39_n40, the co-existence studies have been covered in TR37.716-11-11, where band 39 and band n40 are synchronized operaiton.

- For combination of DC_39_n258, it is no need to consider the harmonic and intermodulation co-existence studies due to the large frequency seperation.

6.104.4	∆TIB and ∆RIB values

For the DC conbination of DC_39_n40-n258, the TIB,c and RIB,c for band n258 equals to 0, and the TIB,c and RIB,c for band 39 and n40 are the same with the values of the constiture DC_39_n40 defined in TS38.101-3.

6.104.5	MSD

No additional MSD requirement is needed.

6.105	DC_39_n41-n258

6.105.1		Operating bands for DC

Table 6.105.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_39_n41-n258 | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.105.2		Configuration for DC

Table 6.105.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_39A_n41A-n258A DC_39A_n41C-n258A | DC_39A_n41A DC_39A_n258A |

6.105.3		Co-existence studies

- For combination of DC_39_n41, the co-existence studies have been covered in TR37.716-11-11, where band 39 and band n41 are synchronized operaiton.

- For combination of DC_39_n258, it is no need to consider the harmonic and intermodulation co-existence studies due to the large frequency seperation.

6.105.4	∆TIB and ∆RIB values

For the DC conbination of DC_39_n41-n258, the TIB,c and RIB,c for band n258 equals to 0, and the TIB,c and RIB,c for band 39 and n41 are the same with the values of the constiture DC_39_n41 defined in TS38.101-3.

6.105.5	MSD

No additional MSD requirement is needed.

6.106	DC_39_n79-n258

6.106.1		Operating bands for DC

Table 6.106.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_39_n79-n258 | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.106.2		Configuration for DC

Table 6.106.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_39A_n79A-n258A DC_39A_n79C-n258A | DC_39A_n79A DC_39A_n258A |

6.106.3		Co-existence studies

- For combination of DC_39_n79, the co-existence studies have been covered in TR37.716-11-11, where no harmonic issues for this combination.

- For combination of DC_39_n258, it is no need to consider the harmonic and intermodulation co-existence studies due to the large frequency seperation.

6.106.4	∆TIB and ∆RIB values

For the DC conbination of DC_39_n79-n258, the TIB,c and RIB,c for band n258 equals to 0, and the TIB,c and RIB,c for band 39 and n79 are the same with the values of the constiture DC_39_n79 defined in TS38.101-3.

6.106.5	MSD

No additional MSD requirement is needed.

6.107	DC_13_n5-n77

6.107.1	Operating bands for DC

Table 6.107.1-1: DC band combination of one LTE + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13_n5-n77 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.107.2	Configuratoin for DC

Table 6.107.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_13A_n5A-n77A | DC_13A_ n77A | 13A | CA_n5A-n77A |

6.107.3	Co-existence studies

According to the studies in 37.717-11-11, the Rx impacts are identified as below,

- For UL DC_13A_n77A, IMD5 of the 2UL may fall into band n5 Rx

Thus additional MSD should be considered to mitigate the impact of the interference for DC_13A_n5A-n77A combination.

6.107.4	∆TIB and ∆RIB values

For DC_13_n5-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.107.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13_n5-n77 | 13 | 0.5 |
|  | n5 | 0.6 |
|  | n77 | 0.8 |

Table 6.107.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13_n5-n77 | 13 | 0.2 |
|  | n5 | 0.2 |
|  | n77 | 0.5 |

6.107.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD5 produced by Band 13 and NR band n77 that impact the reference sensitivity of NR band n5.

The required MSD level and test configuration are are specified in Table 6.107.5-1 for this dual connectivity configuration.

Table 6.107.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_13A_n5A-n77A11 | 13 | IMD5 | ¦4*fB13 -fn77¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n77 |  |  | 4013 | 10 | 50 | 4013 | 10 |  |
|  | n5 |  |  | 840 | 5 | 25 | 885 | 5 | 4.5 |
| NOTE 11:The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |  |

6.108 DC_28_n7-n258

6.108.1	Operating bands for DC

Table 6.108.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n7-n258 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.108.2	Channel bandwidths per operating band for DC

Table 6.108.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_28A_n7A-n258A DC_28A_n7A-n258B DC_28A_n7A-n258C DC_28A_n7A-n258D DC_28A_n7A-n258E DC_28A_n7A-n258F DC_28A_n7A-n258G DC_28A_n7A-n258H DC_28A_n7A-n258I DC_28A_n7A-n258J DC_28A_n7A-n258K DC_28A_n7A-n258L DC_28A_n7A-n258M DC_28A_n7B-n258A DC_28A_n7B-n258B DC_28A_n7B-n258C DC_28A_n7B-n258D DC_28A_n7B-n258E DC_28A_n7B-n258F DC_28A_n7B-n258G DC_28A_n7B-n258H DC_28A_n7B-n258I DC_28A_n7B-n258J DC_28A_n7B-n258K DC_28A_n7B-n258L DC_28A_n7B-n258M | DC_28A_n7A DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I | 28 | - | 880 |
|  |  | n7 | - |  |
|  |  | n258 | - |  |

6.108.3	Co-existence studies

Based on co-existence studies of DC_28_n7 and DC_28_n258 specified in TR 37.717-11-21, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.108.4	∆TIB and ∆RIB values

For DC_28_n7-n258, the TIB,c and RIB values are derived from CA_n7-n28 and are given in the tables below.

Table 6.108.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n7-n258 | 28 | 0.3 |
|  | n7 | 0.3 |
|  | n258 | 0 |

Table 6.108.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n7-n258 | 28 | 0 |
|  | n7 | 0 |
|  | n258 | 0 |

6.108.5	MSD

There is no additional MSD issue for DC_28_n7-n258.

6.109	DC_7_n25-n66

6.109.1	Operating bands for DC

Table 6.109.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n25-n66 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.109.2	Channel bandwidths per operating band for DC

Table 6.109.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_7A_n25A-n66A | DC_7A_n25A DC_7A_n66A | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 100 |
|  |  | n25 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| DC_7A-7A_n25A-n66A | DC_7A_n25A DC_7A_n66A | 7 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 |
|  |  | n25 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| DC_7C_n25A-n66A | DC_7A_n25A DC_7A_n66A | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 |
|  |  | n25 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |

6.109.3	Co-existence studies

Based on co-existence studies there are no IMD issues into own Rx of bands.

6.109.4	∆TIB and ∆RIB values

For DC_7_n25-n66, the TIB,c and RIB,c values are derived from DC_2-7_n66 and are given in the tables below.

Table 6.109.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n25-n66 | 7 | 0.5 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 6.109.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n25-n66 | 7 | 0.5 |
|  | n25 | 0.3 |
|  | n66 | 0.5 |

6.109.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is no IMD issues.

6.110	DC_66_n25-n66

6.110.1	Operating bands for DC

Table 6.110.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n25-n66 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.110.2	Channel bandwidths per operating band for DC

Table 6.110.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n25A-n66A | DC_66A_n25A DC_66A_n66A2 | 66 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 100 |
|  |  | n25 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| NOTE 2: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.110.3	Co-existence studies

Based on co-existence studies, IMD3 and IMD5 generated by dual uplink of 66_n25 may fall into own Rx of band n66.

6.110.4	∆TIB and ∆RIB values

For DC_66_n25-n66, the TIB,c and RIB,c values are derived from DC_2-66_n66 and are given in the tables below.

Table 6.110.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n25-n66 | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 6.110.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n25-n66 | 66 | 0.3 |
|  | n25 | 0.3 |
|  | n66 | 0.3 |

6.110.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD3 and IMD5 produced by UL 66_n25 that impact DL n66. MSD value as in DC_66_n25.

Table 6.110.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n25A-n66A | 66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
|  | n66 | 1717.5 | 5 | 25 | 2117.5 | 23 | IMD3 |
|  | 66 | 1750 | 5 | 25 | 2150 | N/A | N/A |
|  | n25 | 1873 | 5 | 25 | 1953 | N/A | N/A |
|  | n66 | 1719 | 5 | 25 | 2119 | 4 | IMD5 |

6.111	DC_30_n5-n77

6.111.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD3 produced by Band 30 and NR band n5 that impact the reference sensitivity of NR band n77. Also there are IMD3 & IMD5 produced by Band 30 and NR band n77 that impact the reference sensitivity of NR band n5.

The required MSD levels and test configuration are shown in the following Table.

Table 6.111.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_30A_n5A-n77A | B30 | IMD3 | ¦2*fB30 –fn5¦ | 2310 | 5 | 25 | 2355 | 5 | N/A |
|  | n5 |  |  | 834 | 5 | 25 | 879 | 5 |  |
|  | n77 |  |  | 3786 | 10 | 50 | 3786 | 10 | 16.1 |
|  | B30 | IMD3 | ¦2*fB30 -fn77¦ | 2310 | 5 | 25 | 2355 | 5 | N/A |
|  | n77 |  |  | 3741 | 10 | 50 | 3741 | 10 |  |
|  | n5 |  |  | 834 | 5 | 25 | 879 | 5 | 15.3 |
|  | B30 | IMD5 | ¦3*fB30 -2*fn77¦ | 2310 | 5 | 25 | 2355 | 5 | N/A |
|  | n77 |  |  | 3905 | 10 | 50 | 3905 | 10 |  |
|  | n5 |  |  | 835 | 5 | 25 | 880 | 5 | 4.4 |

6.112	DC_13_n25-n66

6.112.1	Operating bands for DC

Table 6.112.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13_n25-n66 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.112.2	Channel bandwidths per operating band for DC

Table 6.112.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_13A_n25A-n66A | DC_13A_n25A DC_13A_n66A | 13 | 15 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 90 |
|  |  | n25 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |

6.112.3	Co-existence studies

Based on co-existence studies, IMD4 generated by dual uplink of 13_n25 may fall into own Rx of band n66.

Based on co-existence studies, IMD4 generated by dual uplink of 13_n66 may fall into own Rx of band n25.

6.112.4	∆TIB and ∆RIB values

For DC_13_n25-n66, the TIB,c and RIB,c values are derived from DC_2-13_n66 and are given in the tables below.

Table 6.112.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13_n25-n66 | 13 | 0.3 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 6.112.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13_n25-n66 | 13 | 0 |
|  | n25 | 0.3 |
|  | n66 | 0.3 |

6.112.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD4 produced by UL 13_n25 that impact DL n66. MSD value as in DC_13-66_n2.

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD4 produced by UL 13_n66 that impact DL n25. MSD value as in DC_2-13_n66.

Table 6.112.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n25A-n66A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n25 | 1860 | 5 | 25 | 1940 | N/A | N/A |
|  | n66 | 1736 | 5 | 25 | 2156 | 7.2 | IMD4 |
| DC_13A_n25A-n66A | 13 | 780 | 5 | 25 | 749 | N/A | N/A |
|  | n25 | 1860 | 5 | 25 | 1940 | 6.2 | IMD4 |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | N/A |

  - 6.113		DC_8_n39-n40

    - 6.113.1	Operating bands for DC

Table 6.113.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n39-n40 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

    - 6.113.2	Configuration for DC

Table 6.113.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_8A_n39A-n40A | DC_8A_n39A DC_8A_n40A | 8A | CA_n39A-n40A |

    - 6.113.3	Co-existence studies

Co-existence analysis for DC_8_n39 UL in TR 37.716-11-11 shows no IMD produce impact to NR Band n40 DL.

Co-existence analysis for DC_8_n40 UL in TR TR 37.863-01-01 shows IMD5 produce impact to NR Band n39 DL.

    - 6.113.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.113.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n39-n40 | 8 | 0.3 |
|  | n39 | 0.3 |
|  | n40 | 0.3 |

Table 6.113.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n39-n40 | 8 | 0 |
|  | n39 | 0.3 |
|  | n40 | 0.3 |

    - 6.113.5	MSD

Based on co-existence studies, there is IMD5 products cause by DC_8_n40 will falls into band n39. However, simultaneous Rx/Tx is not supported for CA_n39-n40, which means synchronization between band n39 and n40. Therefore no additional MSD is needed to be defined

6.114	DC_3_n1-n8, DC_3-3_n1-n8

6.114.1	Operating bands for DC

Table 6.114.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n1-n8 DC_3-3_n1-n8 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz |  |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |

6.114.2	Configuration for DC

Table 6.114.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A_n1A-n8A DC_3A-3A_n1A-n8A | DC_3A_n1A DC_3A_n8A | 3A CA_3A-3A | CA_n1A-n8A |

6.114.3	Co-existence studies

Based on co-existence studies of DC_3A_n1A and DC_3A_n8A specified in TR 37.716-11-11, there is no additional  intermodulation impact for the additional band receiver.

6.114.4	∆TIB and ∆RIB values

For DC_3_n1-n8 and DC_3-3_n1-n8, the TIB,c and RIB,c values are given in the tables below.

Table 6.114.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n1-n8, DC_3-3_n1-n8 | 3 | 0.3 |
|  | n1 | 0.3 |
|  | n8 | 0.3 |

Table 6.114.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n1-n8, DC_3-3_n1-n8 | 3 | 0 |
|  | n1 | 0 |
|  | n8 | 0 |

6.114.5	MSD

No additional MSD requirement is needed.

6.115	DC_7_n1-n8, DC_7-7_n1-n8

6.115.1	Operating bands for DC

Table 6.115.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n1-n8 DC_7-7_n1-n8 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz |  |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |

6.115.2	Configuration for DC

Table 6.115.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A_n1A-n8A DC_7A-7A_n1A-n8A | DC_7A_n1A DC_7A_n8A | 7A CA_7A-7A | CA_n1A-n8A |

6.115.3	Co-existence studies

Based on co-existence studies of DC_7A_n1A and DC_7A_n8A specified in TR 37.716-11-11:

- 5th order of IMD from band 7 and band n1 will fall into the own Rx frequency of Band n8. But in the studies for LTE DL CA_1A-7A-8A with UL CA_1A-7A in TS 36.714-00-02, it is mentioned that the IMD5 is fairly small and the overlapped region is quite small portion, which concluded that no MSD is needed.

6.115.4	∆TIB and ∆RIB values

For DC_7_n1-n8 and DC_7-7_n1-n8, the TIB,c and RIB,c values are given in the tables below.

Table 6.115.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n1-n8, DC_7-7_n1-n8 | 7 | 0.6 |
|  | n1 | 0.5 |
|  | n8 | 0.5 |

Table 6.115.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n1-n8, DC_7-7_n1-n8 | 7 | 0 |
|  | n1 | 0 |
|  | n8 | 0.2 |

6.115.5	MSD

No additional MSD requirement is needed.

  - 6.116		DC_8_n39-n79

    - 6.116.1	Operating bands for DC

Table 6.116.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n39-n79 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

    - 6.116.2	Configuration for DC

Table 6.116.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_8A_n39A-n79A | DC_8A_n39A DC_8A_n79A | 8A | CA_n39A-n79A |

    - 6.116.3	Co-existence studies

Co-existence analysis for DC_8_n39 UL in TR 37.716-11-11 shows 3rd IMD and 4th IMD produce impact to NR Band n79 DL.

Co-existence analysis for DC_8_n79 UL in TR TR 37.863-01-01 shows 4th IMD produce impact to NR Band n39 DL.

    - 6.116.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.116.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n39-n79 | 8 | 0.3 |
|  | n39 | 0.3 |
|  | n79 | 0 |

Table 6.116.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n39-n79 | 8 | 0 |
|  | n39 | 0 |
|  | n79 | 0 |

    - 6.116.5	MSD

Based on co-existence studies additional MSD is needed to be defined, shown in table 6.116.5-1.

Table 6.116.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n39A-n79A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n39 | 1890 | 10 | 50 | 1890 | N/A | N/A |
|  | n79 | 4680 | 40 | 216 | 4680 | 15.9 | IMD3 |
| DC_8A_n39A-n79A | 8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | n39 | 1890 | 10 | 50 | 1890 | N/A | N/A |
|  | n79 | 4560 | 40 | 216 | 4560 | 12.1 | IMD4 |
| DC_8A_n39A-n79A | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | n39 | 1907.5 | 10 | 50 | 1907.5 | 13.8 | IMD4 |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | N/A |

6.117	DC_5_n66-n77

6.117.1	Operating bands for DC

Table 6.117.1-1: DC band combination of one LTE + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n66-n77 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.117.2	Configuratoin for DC

Table 6.117.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A_n66A-n77A DC_5A_n66A-n77C | DC_5A_n66A DC_5A_ n77A | 5A | CA_n66A-n77A |

6.117.3	Co-existence studies

According to the studies in in 37.717-21-11, the Rx impacts are identified as below,

- For UL DC_5A_n66A, IMD3, IMD4 and IMD5 may fall into own Rx of band n77

- For UL DC_5A_n77A, IMD3 of the 2UL may fall into own Rx of band n66.

Thus additional MSD should be considered to mitigate the impact of the interference for DC_5A_n66A-n77A combination.

6.117.4	∆TIB and ∆RIB values

For DC_5_n66-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.117.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n66-n77 | 5 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 6.117.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5_n66-n77 | 5 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |

6.117.5	MSD

Based on co-existence studies, additional MSD is specified in Table 6.117.5-1 for this dual connectivity configuration.

Table 6.117.5-1: MSD test points for SCell due to uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n66A-n77A | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | n66 | 1742 | 5 | 25 | 2142 | 13.2 | IMD3 |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  |  | 5 |  | 845 |  | 5 |  | 25 |  | 890 |  | N/A |  | N/A |  |
|  |  | n66 |  | 1785 |  | 5 |  | 25 |  | 2185 |  | N/A |  | N/A |  |
|  | n77 | 3475 | 10 | 50 | 3475 | 16.1 | IMD3 |
|  |  | 5 |  | 827 |  | 5 |  | 25 |  | 872 |  | N/A |  | N/A |  |
|  |  | n66 |  | 1713 |  | 5 |  | 25 |  | 2113 |  | N/A |  | N/A |  |
|  | n77 | 4194 | 10 | 50 | 4194 | 8.2 | IMD411 |
|  |  | 5 |  | 827 |  | 5 |  | 25 |  | 872 |  | N/A |  | N/A |  |
|  |  | n66 |  | 1713 |  | 5 |  | 25 |  | 2113 |  | N/A |  | N/A |  |
|  |  | n77 |  | 3485 |  | 10 |  | 50 |  | 3485 |  | 3.3 |  | IMD5 |  |
| NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction |  |  |  |  |  |  |  |

6.118	DC_5_n5-n77

6.118.1	Operating bands for DC

Table 6.118.1-1: DC band combination of one LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n5-n77 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.118.2	Configuration for DC

Table 6.118.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A_n5A-n77A | DC_5A_ n77A | 5A | CA_n5A-n77A |

6.118.3 Co-existence studies

According to lower order coexistence study, the Rx impacts are identified as below,

- For UL DC_5A_n77A, IMD4 and IMD5 of 2UL may fall into band n5 Rx frequency range

Thus additional MSD should be considered to mitigate the impact of the interference for DC_5A_n5A-n77A combination.

6.118.4	∆TIB and ∆RIB values

For DC_5_n5-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.118.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n5-n77 | 5 | 0.6 |
|  | n5 | 0.6 |
|  | n77 | 0.8 |

Table 6.118.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5_n5-n77 | 5 | 0.2 |
|  | n5 | 0.2 |
|  | n77 | 0.5 |

6.118.5	MSD

Based on co-existence studies, additional MSD is specified in Table 6.118.5-1 for this dual connectivity configuration.

Table 6.118.5-1: MSD test points for SCell due to uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC CA Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) |  | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n5A-n77A | 5 | 844 |  | 5 | 25 | 889 | N/A | N/A |
|  | n5 | 844 |  | 5 | 25 | 889 | 8.3 | IMD4 |
|  | n77 | 3421 |  | 10 | 50 | 3421 | N/A | N/A |
|  | 5 | 826.5 | 5 |  | 25 | 871.5 | N/A | N/A |
|  | n5 | 827 | 5 |  | 25 | 872 | 5.5 | IMD511 |
|  | n77 | 4178 | 10 |  | 50 | 4178 | N/A | N/A |
| NOTE 11:The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |

6.119	DC_5_n2-n77

6.119.1	Operating bands for DC

Table 6.119.1-1: DC band combination of one LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n2-n77 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.119.2	Configuration for DC

Table 6.119.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A_n2A-n77A | DC_5A_ n77A | 5A | CA_n2A-n77A |

6.119.3	Co-existence studies

According to the studies in 37.717-21-11, the Rx impacts are identified as below

- For UL DC_5A_n77A, IMD3 of 2UL may fall into own Rx of band n2.

Thus additional MSD should be considered to mitigate the impact of the interference for DC_5A_n2A-n77A combination.

6.119.4	∆TIB and ∆RIB values

For DC_5_n2-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.119.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n2-n77 | 5 | 0.6 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 6.119.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5_n2-n77 | 5 | 0.2 |
|  | n2 | 0.2 |
|  | n77 | 0.5 |

6.119.5	MSD

Based on co-existence studies, additional MSD is specified in Table 6.119.5-1 for this dual connectivity configuration.

Table 6.119.5-1: MSD test points for SCell due to uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n2A-n77A | n2 | 1907 | 5 | 25 | 1987 | 16.5 | IMD3 |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
|  | n77 | 3680 | 5 | 25 | 3680 | N/A | N/A |

6.120	DC_2_n2-n77

6.120.1	Operating bands for DC

Table 6.120.1-1: DC band combination of one LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n2-n77 | 2 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n2 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.120.2	Configuration for DC

Table 6.120.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A_n2A-n77A | DC_2A_ n77A | 2A | CA_n2A-n77A |

6.120.3	     Co-existence studies

According to lower order coexistence study, the Rx impacts are identified as below,

- For UL DC_2A_n77A, IMD2, IMD4 and IMD5 of 2UL may fall into band n2 Rx frequency range

Thus additional MSD should be considered to mitigate the impact of the interference for DC_2A_n2A-n77A combination.

6.120.4	∆TIB and ∆RIB values

For DC_2_n2-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.120.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n2-n77 | 2 | 0.6 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 6.120.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n2-n77 | 2 | 0.2 |
|  | n2 | 0.2 |
|  | n77 | 0.5 |

6.120.5	MSD

Based on co-existence studies, additional MSD is specified in Table 6.120.5-1 for this dual connectivity configuration.

Table 6.120.5-1: MSD test points for SCell due to uplink operation for EN-DC in NR FR1 (three bands)

|  | Operating band/ Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A_n2A-n77A | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | n2 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
|  |  |  |  |  |  | 28.75 |  |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | N/A | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | 8.0 | IMD44 |
|  |  |  |  |  |  | 10.75 |  |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |

6.121	DC_3-3_n8-n78

6.121.1	Operating bands for DC

Table 6.121.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-3_n8-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.121.2	Configuration for DC

Table 6.121.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-3A_n8A-n78A5 | DC_3A_n8A DC_3A_n78A | 3A CA_3A-3A | CA_n8A-n78A |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |

6.121.3	Co-existence studies

Based on co-existence studies of DC_3A_n8A and DC_3A_n78A specified in TR 37.716-11-11, the impact on the third bands includes:

- 3rd and 5th order IMD from band 3 and band n8 will fall into the own Rx frequency of Band n78.

6.121.4	∆TIB and ∆RIB values

For DC_3-3_n8-n78, the TIB,c and RIB,c values are given in the tables below.

Table 6.121.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-3_n8-n78 | 3 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 6.121.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-3_n8-n78 | 3 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

6.121.5	MSD

Since the MSD can be covered by DC_3_n8-n78, no additional requirement is needed.

6.122	DC_7-7_n8-n78

6.122.1	Operating bands for DC

Table 6.122.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-7_n8-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.122.2	Configuration for DC

Table 6.122.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-7A_n8A-n78A5 | DC_7A_n8A DC_7A_n78A | 7A CA_7A-7A | CA_n8A-n78A |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |

6.122.3	Co-existence studies

Based on co-existence studies of DC_7A_n8A and DC_7A_n78A specified in TR 37.716-11-11, the impact on the third bands includes:

- 2nd and 4th order IMD from band 7 and band n8 will fall into the own Rx frequency of Band n78.

- 2nd order IMD from band 7 and band n78 will fall into the own Rx frequency of Band n8.

6.122.4	∆TIB and ∆RIB values

For DC_7-7_n8-n78, the TIB,c and RIB,c values are given in the tables below.

Table 6.122.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-7_n8-n78 | 7 | 0.5 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 6.122.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-7_n8-n78 | 7 | 0 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

6.122.5	MSD

Since the MSD can be covered by DC_7_n8-n78, no additional requirement is needed.

## 6.123	DC_7_n66-n77

### 6.123.1	Operating bands for DC

Table 6.123.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n66-n77 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

    - 6.123.2	Configuration for DC

Table 6.123.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_7A_n66A-n77A DC_7A-7A_n66A-n77A DC_7C_n66A-n77A | DC_7A_n66A DC_7A_n77A |

### 6.123.3	Co-existence studies

Based on co-existence studies,

- 3rd and 5th order IMD generated by dual uplink of 7_n66 may fall into own Rx of band n77

- 4th order IMD generated by dual uplink of 7_n77 may fall into own Rx of band n66

### 6.123.4	∆TIB and ∆RIB values

For DC_7_n66-n77, the TIB,c and RIB,c values are derived from DC_7_n66-n78 and are given in the tables below.

Table 6.123.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n66-n77 | 7 | 0.5 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 6.123.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n66-n77 | 7 | 0.5 |
|  | n66 | 0.5 |
|  | n77 | 0.5 |

### 6.123.5	MSD

As mentioned in 6.123.3, 3rd and 5th order IMD generated by dual uplink of 7_n66 may fall into own Rx of band n77, and 4th order IMD generated by dual uplink of 7_n77 may fall into own Rx of band n66. The MSD due to 3rd and 4rd order IMD can be derived from DC_7_n66-n78. The MSD due to 5th order IMD generated by DC_7_n66 can be derived from DC_2A_n7A-n78A.

Table 6.123.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n66A-n77A DC_7A-7A_n66A-n77A DC_7C_n66A-n77A | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 8.7 | IMD4 |
|  | n77 | 3625 | 10 | 50 | 3625 | N/A | N/A |
| DC_7A_n66A-n77A DC_7A-7A_n66A-n77A DC_7C_n66A-n77A | 7 | 2542 | 5 | 25 | 2662 | N/A | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3344 | 10 | 50 | 3344 | 16.0 | IMD3 |
| DC_7A_n66A-n77A DC_7A-7A_n66A-n77A DC_7C_n66A-n77A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 4040 | 10 | 50 | 4040 | 4.2 | IMD5 |

6.124	DC_20_n8-n78

### 6.124.1	 Operating bands for DC

Table 6.124.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20A_n8A-n78A | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.124.2 	Configuration for DC

Table 6.124.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_20A_n8A-n78A |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.124.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_20A_n78A and DC_20A_n8A are shown below.

Table 6.124.3-1: The harmonic and IMD products caused by UL DC_20A_n8A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3328 | 3448 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4160 | 4310 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 18 | 83 | 1712 | 1777 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 749 | 844 | 898 | 998 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2544 | 2639 | 2592 | 2692 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1581 | 1706 | 1778 | 1913 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3376 | 3501 | 3472 | 3607 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 166 | 36 | 3424 | 3554 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2828 | 2658 | 2568 | 2413 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1081 | 916 | 826 | 666 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4352 | 4522 | 4208 | 4363 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4304 | 4469 | 4256 | 4416 |

Table 6.124.3-2: The harmonic and IMD products caused by UL DC_20A_n78A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3328 | 3448 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4160 | 4310 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 | 2968 | 4132 | 4662 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2136 | 1576 | 5738 | 6768 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 | 5524 | 7432 | 8462 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1304 | 714 | 9038 | 10568 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 | 6386 | 10732 | 12262 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5936 | 4876 | 8264 | 9324 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14368 | 12338 | 148 | 472 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9736 | 8176 | 4014 | 5104 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 | 16062 | 6628 | 7248 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 | 13124 | 9096 | 10186 |

IMD4 caused by UL DC_20_n8 may fall into Rx band n78.

IMD4 caused by UL DC_20_n78 may fall into Rx band n8.

### 6.124.4	 ∆TIB and ∆RIB values

Table 6.124.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20A_n8A-n78A | 20 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 6.124.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20A_n8A-n78A | 20 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 6.124.5	MSD

All MSD requirements for DC_20A_n8A-n78A can reuse the requriements of DC_8A-20A_n78A and DC_8A_n28A-n78A. The MSD exceptions are shown below.

Table 6.124.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A_n8A-n78A | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 837 | 5 | 25 | 796 | N/A | N/A |
|  | n78 | 3567 | 10 | 50 | 3567 | 10.3 | IMD4 |
|  | n8 | 895 | 5 | 25 | 940 | 12.1 | IMD4 |
|  | n78 | 3481 | 10 | 50 | 3481 | N/A | N/A |
|  | 20 | 847 | 5 | 25 | 806 | N/A | N/A |

6.125	DC_3_n1-n38

6.125.1	Operating bands for DC

Table 6.125.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n1-n38 | 3 | 1710MHz | – | 1785MHz | 1805MHz | – | 1880MHz | FDD |
|  | n1 | 1920MHz | – | 1980MHz | 2110MHz | – | 2170MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

6.125.2	Channel bandwidths per operating band for DC

Table 6.125.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n1A-n38A | DC_3A_n1A DC_3A_n38A | 3 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 110 |
|  |  | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n38 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |

6.125.3	Co-existence studies

Based on co-existence studies there are no IMD issues into own Rx of bands.

6.125.4	∆TIB and ∆RIB values

For DC_3A_n1A-n38, the TIB,c and RIB,c values are derived from DC_1-3_n38 and are given in the tables below.

Table 6.125.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n1-n38 | 3 | 0.5 |
|  | n1 | 0.5 |
|  | n38 | 0.5 |

Table 6.125.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n1-n38 | 3 | 0 |
|  | n1 | 0 |
|  | n38 | 0 |

6.125.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is no IMD issues.

6.126	DC_8_n1-n40

6.126.1	Operating bands for DC

Table 6.126.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n1-n40 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

6.126.2	Configuration for DC

Table 6.126.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_8A_n1A-n40A | DC_8A_n1A DC_8A_n40A | 8A | CA_n1A-n40A |

6.126.3	Co-existence studies

Co-existence analysis for DC_8_n1 UL shows no impact to NR Band n40 DL.

Co-existence analysis for DC_8_n40 UL shows 5th IMD might fall in NR Band n1 DL.

6.126.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.126.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n1-n40 | 8 | 0.3 |
|  | n1 | 0.3 |
|  | n40 | 0.5 |

Table 6.126.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n1-n40 | 8 | 0.2 |
|  | n1 | 0 |
|  | n40 | 0.5 |

6.126.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD5 produced by Band 8 and NR band n40 that impact the reference sensitivity of NR band n1.

The required MSD levels and test configuration are shown in the following Table.

Table 6.126.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n1A-n40A | B8 | IMD5 | ¦3*fB8 -2*fn40¦ | 885 | 5 | 25 | 930 | 5 | N/A |
|  | n40 |  |  | 2390 | 5 | 25 | 2390 | 5 |  |
|  | n1 |  |  | 1935 | 5 | 25 | 2125 | 5 | 3.3 |

6.127	DC_8_n3-n79

6.127.1	Operating bands for DC

Table 6.127.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n3-n79 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.127.2	Channel bandwidths per operating band for DC

Table 6.127.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A_n3A-n79A | DC_8A_n3A DC_8A_n79A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

6.127.3	Co-existence studies

Based on the co-existence studies of DC_8A-n3A and DC_8A-n79A, 3rd and 4th order IMD generated by dual uplink of Band 8 + Band n3 may also fall into own Rx of band n79 and 4th order IMD generated by dual uplink of Band 8 + Band n79 may also fall into own Rx of band n3.

6.127.4	∆TIB and ∆RIB values

For DC_8_n3-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.127.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n3-n79 | 8 | 0.3 |
|  | n3 | 0.3 |
|  | n79 | 0.8 |

Table 6.127.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n3-n79 | 8 | 0 |
|  | n3 | 0 |
|  | n79 | 0.5 |

6.127.5	MSD

As mentioned in 6.127.3, IMD3 and IMD4 of B8 and n3 to Band n79 Rx and IMD4 of B8 and n79 to Band n3 need to be addressed for REFSENS relaxation. The following values are proposed:

Table 6.127.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A_n3A-n79A | 8 | 885 | 5 | 25 | 930 | N/A | FDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n79 | 4425 | 40 | 216 | 4425 | 15.7 | TDD | IMD31 |
| DC_8A_n3A-n79A | 8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n79 | 4580 | 40 | 216 | 4580 | N/A | TDD | N/A |
|  | n3 | 1755 | 5 | 25 | 1850 | 8.8 | FDD | IMD4 |
| NOTE 1: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

6.128	DC_8_n77-n79

6.128.1	Operating bands for DC

Table 6.128.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n77-n79 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.128.2	Channel bandwidths per operating band for DC

Table 6.128.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A_n77A-n79A | DC_8A_n77A DC_8A_n79A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

6.128.3	Co-existence studies

Based on the co-existence studies of DC_8A-n77A and DC_8A-n79A, 2nd, 4th and 5th order IMD generated by dual uplink of Band 8 + Band n77 may also fall into own Rx of band n79 and 2nd order IMD generated by dual uplink of Band 8 + Band n79 may also fall into own Rx of band n77. But those IMDs are not occurred since the non-simultaneous Rx/Tx operation is considered for CA_n77-n79.

6.128.4	∆TIB and ∆RIB values

For DC_8_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.128.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n77-n79 | 8 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 6.128.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n77-n79 | 8 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0 |

6.128.5	MSD

As mentioned in 6.128.3, there is no need to specify additional MSD requirement for this UL DC configuration

6.129	DC_1_n3-n79

6.129.1	Operating bands for DC

Table 6.129.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n3-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.129.2	Channel bandwidths per operating band for DC

Table 6.129.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A_n3A-n79A | DC_1A_n3A DC_1A_n79A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

6.129.3	Co-existence studies

Based on the co-existence studies of DC_1A-n3A and DC_1A-n79A, 5th order IMD generated by dual uplink of Band 1 + Band n3 may also fall into own Rx of band n79.

6.129.4	∆TIB and ∆RIB values

For DC_1_n3-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.129.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n3-n79 | 1 | 0.3 |
|  | n3 | 0.3 |
|  | n79 | 0.8 |

Table 6.129.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n3-n79 | 1 | 0 |
|  | n3 | 0 |
|  | n79 | 0.5 |

6.129.5	MSD

As mentioned in 6.129.3, IMD5 of B1 and n3 to Band n79 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_1A_n3A-n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n79 | 4950 | 40 | 216 | 4950 | 4.7 | TDD | IMD5 |

6.130	DC_8_n28-n79

6.130.1	Operating bands for DC

Table 6.130.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n28-n79 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.130.2	Channel bandwidths per operating band for DC

Table 6.130.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A_n28A-n79A | DC_8A_n28A DC_8A_n79A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

6.130.3	Co-existence studies

Based on the co-existence studies of DC_8A-n28A and DC_8A-n79A, 5th order IMD generated by dual uplink of Band 8 + Band n28 may also fall into own Rx of band n79 and 5th order IMD generated by dual uplink of Band 8 + Band n79 may also fall into own Rx of band n28.

6.130.4	∆TIB and ∆RIB values

For DC_8_n28-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.130.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n28-n79 | 8 | 0.6 |
|  | n28 | 0.5 |
|  | n79 | 0.8 |

Table 6.130.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n28-n79 | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n79 | 0.5 |

6.130.5	MSD

As mentioned in 6.130.3, IMD5 of B8 and n28 to Band n79 Rx and IMD5 of B8 and n79 to Band n28 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A_n28A-n79A |  |  | 8 |  | 912.5 |  | 5 |  | 25 |  | 957.5 |  | N/A | FDD |  | N/A |  |
|  |  |  | n28 |  | 745.5 |  | 5 |  | 25 |  | 800.5 |  | N/A | FDD |  | N/A |  |
|  |  |  | n79 |  | 4420 |  | 40 |  | 216 |  | 4420 |  | 0.0 | FDD |  | IMD5 |  |
| DC_8A_n28A-n79A |  |  | 8 |  | 905 |  | 5 |  | 25 |  | 950 |  | N/A | FDD |  | N/A |  |
|  |  |  | n79 |  | 4420 |  | 40 |  | 216 |  | 4420 |  | N/A | TDD |  | N/A |  |
|  |  |  | n28 |  | 745 |  | 5 |  | 25 |  | 800 |  | 3.9 | FDD |  | IMD5 |  |

## 6.131	DC_12_n66-n78

### 6.131.1	Operating bands for DC

Table 6.131.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12_n66-n78 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.131.2	Configuration for DC

Table 6.131.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_12A_n66A-n78A DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) | DC_12A_n66A DC_12A_n78A |

### 6.131.3	Co-existence studies

Based on co-existence studies,

- 5th order IMD generated by dual uplink of 12_n66 may fall into own Rx of band n78

- 3rd order IMD generated by dual uplink of 12_n78 may fall into own Rx of band n66

### 6.131.4	∆TIB and ∆RIB values

For DC_12_n66-n78, the TIB,c and RIB,c values are derived from DC_12-66_n78 and are given in the tables below.

Table 6.131.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12_n66-n78 | 12 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |

Table 6.131.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12_n66-n78 | 12 | 0.2 |
|  | n66 | 0.2 |
|  | n78 | 0.5 |

### 6.131.5	MSD

As mentioned in 6.131.3, 3rd order IMD generated by dual uplink of 12_n78 may fall into own Rx of band n66, and 5th order IMD generated by dual uplink of 12_n66 may fall into own Rx of band n78. As agreed in TR 37.716-21-21, the MSD is defined in Table 6.131.5-1.

Table 6.131.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A_n66A-n78A DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) | 12 | 703 | 5 | 25 | 733 | N/A | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | 16.5 | IMD3 |
|  | n78 | 3546 | 10 | 50 | 3546 | N/A | N/A |
| DC_12A_n66A-n78A DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) | 12 | 703 | 5 | 25 | 733 | N/A | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n78 | 3754 | 10 | 50 | 3754 | 4.1 | IMD5 |

## 6.132	DC_13_n7-n78

### 6.132.1	Operating bands for DC

Table 6.132.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13_n7-n78 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.132.2	Configuration for DC

Table 6.132.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_13A_n7A-n78A | DC_13A_n7A DC_13A_n78A |

### 6.132.3	Co-existence studies

Based on co-existence studies,

- 2nd and 4th order IMD generated by dual uplink of 13_n7 may fall into own Rx of band n78

- 2nd order IMD generated by dual uplink of 13_n78 may fall into own Rx of band n7

### 6.132.4	∆TIB and ∆RIB values

For DC_13_n7-n78, the TIB,c and RIB,c values are derived from DC_12_n7-n78 and are given in the tables below.

Table 6.132.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13_n7-n78 | 13 | 0.5 |
|  | n7 | 0.5 |
|  | n78 | 0.8 |

Table 6.132.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13_n7-n78 | 13 | 0.2 |
|  | n7 | 0.5 |
|  | n78 | 0.5 |

### 6.132.5	MSD

As mentioned in 6.132.3, -	2nd and 4th order IMD generated by dual uplink of 13_n7 may fall into own Rx of band n78, and 2nd order IMD generated by dual uplink of 13_n78 may fall into own Rx of band n7. As agreed in TR 37.716-21-21, the MSD is defined in Table 6.132.5-1.

Table 6.132.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n7A-n78A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n78 | 3432 | 10 | 50 | 3432 | N/A | N/A |
|  | n7 | 2530 | 5 | 25 | 2650 | 27.9 | IMD2 |
| DC_13A_n7A-n78A | 13 | 749 | 5 | 25 | 780 | N/A | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n78 | 3622 | 10 | 50 | 3622 | 9 | IMD4 |
| DC_13A_n7A-n78A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n78 | 3312 | 10 | 50 | 3312 | 29.0 | IMD2 |

6.133	DC_3_n1-n41

6.133.1	Operating bands for DC

Table 6.133.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n1-n41 | 3 | 1710MHz | – | 1785MHz | 1805MHz | – | 1880MHz | FDD |
|  | n1 | 1920MHz | – | 1980MHz | 2110MHz | – | 2170MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.133.2	Channel bandwidths per operating band for DC

Table 6.133.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n1A-n41A | DC_3A_n1A DC_3A_n41A | 3 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.133.3	Co-existence studies

Based on co-existence studies there are IMD5 impact from UL 3_n1 into DL n41.

6.133.4	∆TIB and ∆RIB values

For DC_3A_n1A-n41, the TIB,c and RIB,c values are derived from DC_1-3_n41 and are given in the tables below.

Table 6.133.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n1-n41 | 3 | 0.5 |
|  | n1 | 0.5 |
|  | n41 | 0.5 |

Table 6.133.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n1-n41 | 3 | 0 |
|  | n1 | 0 |
|  | n41 | 0 |

6.133.5	MSD

Based on co-existence studies MSD need to be defined. MSD value in table below is same as for DC_1A_n3A-n41A.

Table 6.133.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n1A-n41A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n41 | 2507.5 | 5 | 25 | 2507.5 | 5.0 | IMD5 |

6.134	DC_8_n1-n77

6.134.1	Operating bands for DC

Table 6.134.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n1-n77 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.134.2	Channel bandwidths per operating band for DC

Table 6.134.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A_n1A-n77A5 | DC_8A_n1A DC_8A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  |  |  | n1 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | 15 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
| DC_8A_n1A-n77(2A)5 |  | DC_8A_n1A DC_8A_n77A |  | 8 |  | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 230 |  |
|  |  |  |  | n1 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.134.3	Co-existence studies

Based on the co-existence studies of DC_8A-n1A and DC_8A-n77A, 3rd and 5th order IMD generated by dual uplink of Band 8 + Band n1 may also fall into own Rx of band n77 and 3rd order IMD generated by dual uplink of Band 8 + Band n77 may also fall into own Rx of band n1.

6.134.4	∆TIB and ∆RIB values

For DC_8_n1-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.134.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n1-n77 | 8 | 0.6 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 6.134.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n1-n77 | 8 | 0.2 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

6.134.5	MSD

As mentioned in 6.134.3, IMD3 and IMD5 of B 8 and n1 to Band n77 Rx and IMD3 of B 8 and n77 to Band n1 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A_n1A-n77A | 8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n1 | 1945 | 5 | 25 | 2135 | N/A | FDD | N/A |
|  | n77 | 3745 | 10 | 50 | 3745 | 14.9 | TDD | IMD31 |
| DC_8A_n1A-n77A | 8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n77 | 3960 | 10 | 50 | 3960 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 14.4 | FDD | IMD3 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

6.135	DC_42_n1-n3

6.135.1	Operating bands for DC

Table 6.135.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42_n1-n3 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |

6.135.2	Channel bandwidths per operating band for DC

Table 6.135.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_42A_n1A-n3A | DC_42A_n1A DC_42A_n3A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 60 |
|  |  |  |  | n1 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n3 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42C_n1A-n3A | DC_42A_n1A DC_42C_n1A DC_42A_n3A DC_42C_n3A | 42 | CA_42C |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 |
|  |  |  |  | n1 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n3 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.135.3	Co-existence studies

Based on the co-existence studies of DC_42A-n1A and DC_42A-n3A, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.135.4	∆TIB and ∆RIB values

For DC_42_n1-n3, the TIB,c and RIB,c values are given in the tables below.

Table 6.135.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42_n1-n3 | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n3 | 0.6 |

Table 6.135.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42_n1-n3 | 42 | 0.5 |
|  | n1 | 0 |
|  | n3 | 0.2 |

6.135.5	MSD

As mentioned in 6.135.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.136	DC_7_n12-n77

6.136.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD3 produced by Band 7 and NR band n12 that impact the reference sensitivity of NR band n77. Also IMD4 product impacted to the n77. But the MSD by IMD4 is not specified.

The required MSD level and test configuration are shown in the following Table.

Table 6.136.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR CA inter-bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n12A-n77A | B7 | IMD3 | ¦2*fn12 +fB7¦ | 2520 | 10 | 50 | 2640 | 10 | N/A |
|  | n12 |  |  | 708 | 5 | 25 | 738 | 5 |  |
|  | n77 |  |  | 3936 | 10 | 50 | 3936 | 10 | 15.2 |

6.137	DC_7_n71-n77

6.137.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD3 produced by Band 7 and NR band n71 that impact the reference sensitivity of NR band n77. Also IMD4 product impacted to the n77. But the MSD by IMD4 is not specified.

The required MSD level and test configuration are shown in the following Table.

Table 6.137.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR CA inter-bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n71A-n77A | B7 | IMD3 | ¦2*fn71 +fB7¦ | 2520 | 10 | 50 | 2640 | 10 | N/A |
|  | n71 |  |  | 693 | 5 | 25 | 647 | 5 |  |
|  | n77 |  |  | 3906 | 10 | 50 | 3906 | 10 | 15.2 |

6.138	DC_5_n2-n41

6.138.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD2 produced by Band 5 and NR band n2 that impact the reference sensitivity of NR band n41.

The required MSD levels and test configuration are shown in the following Table.

Table 6.138.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR CA inter-bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A_n2A-n41A | B5 | IMD2 | ¦fB5 +fn2¦ | 829 | 5 | 25 | 874 | 5 | N/A |
|  | n2 |  |  | 1855 | 5 | 25 | 1935 | 5 |  |
|  | n41 |  |  | 2684 | 5 | 25 | 2684 | 5 | 29.7 |

6.139	DC_3_n28-n75

6.139.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD5 produced by Band 3 and NR band n28 that impact the reference sensitivity of NR band n75.

The required MSD levels and test configuration are shown in the following Table.

Table 6.139.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR CA inter-bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n28A-n75A | B3 | IMD5 | ¦3*fn28 -2*fB3¦ | 1780 | 5 | 25 | 1875 | 5 | N/A |
|  | n28 |  |  | 708 | 5 | 25 | 763 | 5 |  |
|  | n75 |  |  | - | - | - | 1436 | 5 | 3.3 |

  - 6.140	DC_1_n38-n78

    - 6.140.1	Operating bands for DC

Table 6.140.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n38-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

    - 6.140.2	Configuration for DC

Table 6.140.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A_n38A-n78A | DC_1A_n78A | 1A | CA_n38A-n78A |

    - 6.140.3	Co-existence studies

Co-existence analysis for DC_1_n78 UL in TR 37.863-01-01 shows IMD4 produce may fall into NR Band n38 DL.

    - 6.140.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.140.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n38-n78 | 1 | 0.5 |
|  | n38 | 0.5 |
|  | n78 | 0.8 |

Table 6.140.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n38-n78 | 1 | 0 |
|  | n38 | 0 |
|  | n78 | 0.5 |

    - 6.140.5	MSD

Based on co-existence studies additional MSD is needed to be defined. The MSD value is given in table 6.140.5-1.

Table 6.140.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n38A-n78A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n38 | 2590 | 5 | 25 | 2590 | 12.7 | IMD4 |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |

  - 6.141	DC_3_n38-n78

    - 6.141.1	Operating bands for DC

Table 6.141.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n38-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

    - 6.141.2	Configuration for DC

Table 6.141.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A_n38A-n78A | DC_3A_n78A | 3A | CA_n38A-n78A |

    - 6.141.3	Co-existence studies

Co-existence analysis for DC_3_n78 UL have already been captured in TR 37.863-01-01, where there are no IMD produces may fall into NR Band n38 DL.

    - 6.141.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.141.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n38-n78 | 3 | 0.6 |
|  | n38 | 0.5 |
|  | n78 | 0.8 |

Table 6.141.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n38-n78 | 3 | 0.5 |
|  | n38 | 0 |
|  | n78 | 0.5 |

    - 6.141.5	MSD

According to the co-existence studies, there are no additional MSD will be defined.

  - 6.142	DC_7_n38-n78

    - 6.142.1	Operating bands for DC

Table 6.142.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n38-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

    - 6.142.2	Configuration for DC

Table 6.142.2-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_7A_n38A-n78A17,18 | N/A | NA | n78A |
| NOTE 17: The combination is not used alone as fall back mode of other band combinations. NOTE 18: Power imbalance between downlink carriers on Band 7 and Band 38 or band n38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |  |

To avoid duplicated NOTEs in the TS38.101-3 specification, it is proposed to add ‘or band n38’ on top of the existing NOTE 18 in table 5.5B.4.2 in the TS38.101-3, shown as above.

    - 6.142.3	Co-existence studies

No need to analysis the co-existence studies for this configuration.

    - 6.142.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.142.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n38-n78 | n78 | 0.8 |

Table 6.142.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n38-n78 | 7 | 0 |
|  | n38 | 0 |
|  | n78 | 0.5 |

    - 6.142.5	MSD

There are no additional MSD is needed to be defined.

  - 6.143	DC_20_n38-n78

    - 6.143.1	Operating bands for DC

Table 6.143.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20_n38-n78 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

    - 6.143.2	Configuration for DC

Table 6.143.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_20A_n38A-n78A | DC_20A_n78A | 20A | CA_n38A-n78A |

    - 6.143.3	Co-existence studies

Co-existence analysis for DC_20_n78 UL in TR 37.863-01-01 shows IMD2 produce may fall into NR Band n38 DL.

    - 6.143.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.143.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n38-n78 | 20 | 0.6 |
|  | n38 | 0.3 |
|  | n78 | 0.8 |

Table 6.143.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20_n38-n78 | 20 | 0.2 |
|  | n38 | 0 |
|  | n78 | 0.5 |

    - 6.143.5	MSD

Based on co-existence studies additional MSD is needed to be defined. The MSD value is given in table 6.143.5-1.

Table 6.143.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n38A-n78A | 20 | 850 | 5 | 25 | 809 | N/A | N/A |
|  | n38 | 2600 | 5 | 25 | 2600 | 30.9 | IMD2 |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |

6.144	DC_1_n3-n8

### 6.144.1	 Operating bands for DC

Table 6.144.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A_n3A-n8A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |

### 6.144.2 	Configuration for DC

Table 6.144.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A_n3A-n8A | DC_1A_n3A DC_1A_n8A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 50 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.144.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_1A_n3A and DC_1A_n8A are shown below.

Table 6.144.3-1: The harmonic and IMD products caused by UL DC_1A_n3A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 8550 | 8925 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 135 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |

Table 6.144.3-2: The harmonic and IMD products caused by UL DC_1A_n8A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 | 1005 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 | 3080 | 220 | 90 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 | 4875 | 3680 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 | 5060 | 660 | 825 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 | 6855 | 4560 | 4725 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2010 | 2200 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 | 1540 | 7040 | 6765 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 | 1320 | 4180 | 3930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 | 5640 | 8560 | 8835 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 | 6705 | 7520 | 7770 |

### 6.144.4	 ∆TIB and ∆RIB values

Table 6.144.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A_n3A-n8A | 1 | 0.3 |
|  | n3 | 0.3 |
|  | n8 | 0.3 |

Table 6.144.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A_n3A-n8A | 1 | 0 |
|  | n3 | 0 |
|  | n8 | 0 |

### 6.144.5	MSD

The second harmonic of n8 may occur on the Rx of n3. The corresponding MSD values are already mentionned in 38.101-3 Table 7.3B.2.3.1-1

6.145	DC_8_n1-n28

6.145.1	 Operating bands for DC

Table 6.145.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n1-n28 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

6.145.2 	Configuration for DC

Table 6.145.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_8A_n1A-n28A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 100 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n28 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |

6.145.3	 Co-existence studies

When Uplink EN-DC configuration is DC_8_n28, 5th IMD will fall into Rx band of Band 1. However, when consider the 5MHz UL channel BW for DC_8_n28, the IMD5 does not fall into the B1 own Rx band. This was decision made for DC_1-8_n28 in 37.716-21-11.

IMD4 caused by UL DC_8_n1 may fall into Rx band n28.

6.145.4	 ∆TIB and ∆RIB values

Relaxation values of DC_1A-8A_n28A are re-used

Table 6.145.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n1-n28 | 8 | 0.6 |
|  | n1 | 0.3 |
|  | n28 | 0.6 |

Table 6.145.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n1-n28 | 8 | 0.2 |
|  | n28 | 0.2 |

6.145.5	MSD

IMD4 MSD is copied from DC_8A_n28A-n78A and IMD5 treatment from DC_2A-5A_n12A is reused

Table 6.145.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A_n1A-n28A1 | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n1 | 1965 | 5 | 25 | 2155 | N/A | N/A |
|  | n28 | 710 | 5 | 25 | 765 | 11.6 | IMD4 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

6.146	DC_11_n77-n79

6.146.1	Operating bands for DC

Table 6.146.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_11_n77-n79 | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.146.2	Channel bandwidths per operating band for DC

Table 6.146.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_11A_n77A-n79A | DC_11A_n77A DC_11A_n79A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  |  |  | n77 |  | 15 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
| DC_11A_n77(2A)-n79A | DC_11A_n77A DC_11A_n79A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 310 |
|  |  | n77 | CA_n77(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

6.146.3	Co-existence studies

Based on the co-existence studies of DC_11A-n77A and DC_11A-n79A, 2nd and 4th order IMD generated by dual uplink of Band 11 + Band n77 may also fall into own Rx of band n79 and 2nd order IMD generated by dual uplink of Band 11 + Band n79 may also fall into own Rx of band n77. But due to the TDD synchronous operation of n77 and n79, they are not impacted.

6.146.4	∆TIB and ∆RIB values

For DC_11_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.146.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11_n77-n79 | 11 | 0.4 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 6.146.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11_n77-n79 | 11 | 0 |
|  | n77 | 0.5 |
|  | n79 | 0 |

6.146.5	MSD

As mentioned in 6.146.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.147	DC_7_n78-n79

### 6.147.1	 Operating bands for DC

Table 6.147.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n78-n79 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.147.2 	Configuration for DC

Table 6.147.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n78A-n79A |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_7A_n78A-n79C |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 320 |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.147.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_7_n78 and DC_7_n79 are shown below.

Table 6.147.3-1: The harmonic and IMD products caused by UL DC_7_n79

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 4400 | 5000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 8800 | 10000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 13200 | 15000 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 17600 | 20000 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 22000 | 25000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1830 | 2500 | 6900 | 7570 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 0 | 740 | 6230 | 7500 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9400 | 10140 | 11300 | 12570 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2500 | 3310 | 10630 | 12500 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11900 | 12710 | 15700 | 17570 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5000 | 3660 | 13800 | 15140 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 17500 | 15030 | 5880 | 5000 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10000 | 8060 | 1090 | 2500 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 20100 | 22570 | 14400 | 15280 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 18200 | 20140 | 16300 | 17710 |

Table 6.147.3-2: The harmonic and IMD products caused by UL DC_7_n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730 | 1300 | 5800 | 6370 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1200 | 1840 | 4030 | 5100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300 | 8940 | 9100 | 10170 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700 | 4410 | 7330 | 8900 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800 | 11510 | 12400 | 13970 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2600 | 1460 | 11600 | 12740 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12700 | 10630 | 6980 | 6200 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6400 | 4760 | 1110 | 100 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 | 17770 | 13300 | 14080 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 | 16540 | 14100 | 15310 |

IMD4 caused by UL DC_7_n79 may fall into Rx band n78.

IMD3, IMD4 and IMD5 caused by UL DC_7_n78 may fall into Rx band n79.

### 6.147.4	 ∆TIB and ∆RIB values

Table 6.147.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n78-n79 | 7 | 0.5 |
|  | n78 | 0.8 |
|  | n79 | 0.8 |

Table 6.147.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n78-n79 | 7 | 0.5 |
|  | n78 | 0.5 |
|  | n79 | 0.5 |

### 6.147.5	MSD

Table 6.147.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n78A-n79A DC_7A_n78A-n79C | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3600 | 10 | 50 | 3600 | N/A | N/A |
|  | n79 | 4680 | 10 | 50 | 4680 | [24.5] | IMD31, X |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3770 | 10 | 50 | 3770 | [2.4] | IMD4 |
|  | n79 | 4450 | 10 | 50 | 4450 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE X: This band is subject to IMD4 also which MSD is not specified. NOTE Y: For the DC band combination, simultaneous Rx/Tx capability is allowed between n78 and n79. |  |  |  |  |  |  |  |

7	Dual Connectivity band combinations of LTE 2 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

7.1	DC_3-7_n1-n257, DC_3-3-7_n1-n257, DC_3-7-7_n1-n257, DC_3-3-7-7_n1-n257

7.1.1	Operating bands for DC

Table 7.1.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n1-n257 DC_3-3-7_n1-n257 DC_3-7-7_n1-n257 DC_3-3-7-7_n1-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.1.2	Channel bandwidths per operating band for DC

Table 7.1.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_3A-7A_n1A-n257D DC_3A-7A_n1A-n257E DC_3A-7A_n1A-n257F DC_3A-7A_n1A-n257G DC_3A-7A_n1A-n257H DC_3A-7A_n1A-n257I DC_3A-7A_n1A-n257J DC_3A-7A_n1A-n257K DC_3A-7A_n1A-n257L DC_3A-7A_n1A-n257M | DC_3A_n1A DC_3A_n257A DC_7A_n1A DC_7A_n257A | 3 | See CA_3A-7A in TS 36.101 | 460 660 860 260 360 460 560 660 760 860 |
|  |  | 7 |  |  |
|  |  | n1 | See CA_n1A-n257D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |
| DC_3A-3A-7A_n1A-n257D DC_3A-3A-7A_n1A-n257E DC_3A-3A-7A_n1A-n257F DC_3A-3A-7A_n1A-n257G DC_3A-3A-7A_n1A-n257H DC_3A-3A-7A_n1A-n257I DC_3A-3A-7A_n1A-n257J DC_3A-3A-7A_n1A-n257K DC_3A-3A-7A_n1A-n257L DC_3A-3A-7A_n1A-n257M | DC_3A_n1A DC_3A_n257A DC_7A_n1A DC_7A_n257A | 3 | See CA_3A-3A-7A in TS 36.101 | 480 680 880 280 380 480 580 680 780 880 |
|  |  | 7 |  |  |
|  |  | n1 | See CA_n1A-n257D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |
| DC_3A-7A-7A_n1A-n257D DC_3A-7A-7A_n1A-n257E DC_3A-7A-7A_n1A-n257F DC_3A-7A-7A_n1A-n257G DC_3A-7A-7A_n1A-n257H DC_3A-7A-7A_n1A-n257I DC_3A-7A-7A_n1A-n257J DC_3A-7A-7A_n1A-n257K DC_3A-7A-7A_n1A-n257L DC_3A-7A-7A_n1A-n257M | DC_3A_n1A DC_3A_n257A DC_7A_n1A DC_7A_n257A | 3 | See CA_3A-7A-7A in TS 36.101 | 480 680 880 280 380 480 580 680 780 880 |
|  |  | 7 |  |  |
|  |  | n1 | See CA_n1A-n257D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |
| DC_3A-3A-7A-7A_n1A-n257D DC_3A-3A-7A-7A_n1A-n257E DC_3A-3A-7A-7A_n1A-n257F DC_3A-3A-7A-7A_n1A-n257G DC_3A-3A-7A-7A_n1A-n257H DC_3A-3A-7A-7A_n1A-n257I DC_3A-3A-7A-7A_n1A-n257J DC_3A-3A-7A-7A_n1A-n257K DC_3A-3A-7A-7A_n1A-n257L DC_3A-3A-7A-7A_n1A-n257M | DC_3A_n1A DC_3A_n257A DC_7A_n1A DC_7A_n257A | 3 | See CA_3A-3A-7A-7A in TS 36.101 | 500 700 900 300 400 500 600 700 800 900 |
|  |  | 7 |  |  |
|  |  | n1 | See CA_n1A-n257D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |

7.1.3	Co-existence studies

Based on co-existence studies of DC_3A_n1A and DC_3A_n257A specified in TR 37.716-11-11, TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.1.4	∆TIB and ∆RIB values

For DC_3-7_n1-n257, DC_3-3-7_n1-n257, DC_3-7-7_n1-n257, DC_3-3-7-7_n1-n257, TIB,c and RIB,c values are already mentioned in 37.716-21-21 under the same band combinations with n257A.

7.1.5	MSD

No additional MSD requirement is needed.

7.2	DC_1-42_n28-n77

7.2.1	Operating bands for DC

Table 7.2.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-42_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.2.2	Channel bandwidths per operating band for DC

Table 7.2.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-42A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_42A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-42A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_42A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_42A_n28A DC_42C_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-42C_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_42A_n28A DC_42C_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 280 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.2.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-42_n28, DC_1-42_n77, DC_1_n28-n77 and DC_42_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.2.4	∆TIB and ∆RIB values

For DC_1-42_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-42_n28-n77 | 1 | 0.6 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |
|  | n77 | 0.8 |

Table 7.2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-42_n28-n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |
|  | n77 | 0.5 |

7.2.5	MSD

As mentioned in 7.2.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.3	DC_3-42_n28-n77

7.3.1	Operating bands for DC

Table 7.3.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-42_n28-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.3.2	Channel bandwidths per operating band for DC

Table 7.3.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_3A-42A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_42A_n28A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_3A-42A_n28A-n77(2A) | DC_3A_n28A DC_3A_n77A DC_42A_n28A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42C_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_3A-42C_n28A-n77(2A) | DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 280 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.3.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_3-42_n28, DC_3-42_n77, DC_3_n28-n77 and DC_42_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.3.4	∆TIB and ∆RIB values

For DC_3-42_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.3.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-42_n28-n77 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |
|  | n77 | 0.8 |

Table 7.3.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-42_n28-n77 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |
|  | n77 | 0.5 |

7.3.5	MSD

As mentioned in 7.3.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.4	DC_8-42_n28-n77

7.4.1	Operating bands for DC

Table 7.4.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-42_n28-n77 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.4.2	Channel bandwidths per operating band for DC

Table 7.4.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-42A_n28A-n77A | DC_8A_n28A DC_8A_n77A DC_42A_n28A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_8A-42A_n28A-n77(2A) | DC_8A_n28A DC_8A_n77A DC_42A_n28A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 250 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A-42C_n28A-n77A | DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_8A-42C_n28A-n77(2A) | DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 270 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.4.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-42_n28, DC_8-42_n77, DC_8_n28-n77 and DC_42_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.4.4	∆TIB and ∆RIB values

For DC_8-42_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.4.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-42_n28-n77 | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |
|  | n77 | 0.8 |

Table 7.4.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-42_n28-n77 | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |
|  | n77 | 0.5 |

7.4.5	MSD

As mentioned in 7.4.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.5	DC_1-3_n28-n41

7.5.1	Operating bands for DC

Table 7.5.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A_n28A-n41A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

7.5.2	Channel bandwidths per operating band for DC

Table 7.5.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_3A_n28A DC_3A_n41A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

7.5.3	Co-existence studies

Co-existence studies of DC_1A-3A_n28A-n41A and protected bands are already covered in the constituent fall-back modes.

7.5.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_1-3_n28-n41 are given in the tables below reusing the values for DC_ DC_1-3-41_n28.

Table 7.5.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n28-n41 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n28 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.5.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n28-n41 | 1 | 0 |
|  | 3 | 0 |
|  | n28 | 0.2 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

7.5.5	MSD

All MSD requirements forDC_1-3_n28-n41 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.6	DC_2-48_n48-n66

7.6.1	Operating bands for DC

Table 7.6.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-48_n48-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

7.6.2	Configuration for DC

Table 7.6.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-48A_n48A-n66A | DC_2A_n48A DC_2A_n66A DC_48A_n66A | CA_2A-48A | CA_n48A-n66A |

7.6.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

7.6.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.6.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-48_n48-n66 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | n48 | 0.8 |
|  | n66 | 0.6 |

Table 7.6.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-48_n48-n66 | 2 | 0.3 |
|  | 48 | 0.4 |
|  | n48 | 0.4 |
|  | n66 | 0.3 |

7.6.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.7	DC_48-66_n25-n48

7.7.1	Operating bands for DC

Table 7.7.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_48-66_n25-n48 | 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

7.7.2	Configuration for DC

Table 7.7.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_48A-66A_n25A-n48A | DC_48A_n25A DC_66A_n25A DC_66A_n48A | CA_48A-66A | CA_n25A-n48A |

7.7.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

7.7.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.7.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48-66_n25-n48 | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n25 | 0.6 |
|  | n48 | 0.8 |

Table 7.7.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48-66_n25-n48 | 48 | 0.4 |
|  | 66 | 0.3 |
|  | n25 | 0.3 |
|  | n48 | 0.4 |

7.7.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.8	DC_1-8_n40-n78

7.8.1	Operating bands for DC

Table 7.8.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n40-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.8.2	Configuration for DC

Table 7.8.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-8A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_8A_n40A DC_8A_n78A | CA_1A-8A | CA_n40A-n78A |

7.8.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

7.8.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.8.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n40-n78 | 1 | 0.5 |
|  | 8 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 7.8.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n40-n78 | 1 | 0 |
|  | 8 | 0.2 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

7.8.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.9	DC_3-7_n1-n40

7.9.1	Operating bands for DC

Table 7.9.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n1-n40 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

7.9.2	Configuration for DC

Table 7.9.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_7A_n1A DC_7A_n40A | CA_3A-7A | CA_n1A-n40A |

7.9.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

7.9.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.9.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n1-n40 | 3 | 0.6 |
|  | 7 | 0.8 |
|  | n1 | 0.6 |
|  | n40 | 0.9 |

Table 7.9.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n1-n40 | 3 | 0 |
|  | 7 | 0.3 |
|  | n1 | 0 |
|  | n40 | 0.8 |

7.9.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.10	DC_3-8_n40-n78

7.10.1	Operating bands for DC

Table 7.10.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8_n40-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.10.2	Configuration for DC

Table 7.10.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-8A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_8A_n40A DC_8A_n78A | CA_3A-8A | CA_n40A-n78A |

7.10.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

7.10.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.10.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8_n40-n78 | 3 | 0.6 |
|  | 8 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 7.10.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-8_n40-n78 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

7.10.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.11	DC_3-28_n1-n40

7.11.1	Operating bands for DC

Table 7.11.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-28_n1-n40 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

7.11.2	Configuration for DC

Table 7.11.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-28A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_28A_n1A DC_28A_n40A | CA_3A-28A | CA_n1A-n40A |

7.11.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

7.11.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.11.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n1-n40 | 3 | 0.5 |
|  | 28 | 0.6 |
|  | n1 | 0.5 |
|  | n40 | 0.5 |

Table 7.11.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n1-n40 | 3 | 0 |
|  | 28 | 0.2 |
|  | n1 | 0 |
|  | n40 | 0 |

7.11.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.12	DC_7-8_n40-n78

7.12.1	Operating bands for DC

Table 7.12.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-8_n40-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.12.2	Configuration for DC

Table 7.12.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-8A_n40A-n78A | DC_7A_n40A DC_7A_n78A DC_8A_n40A DC_8A_n78A | CA_7A-8A | CA_n40A-n78A |

7.12.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

7.12.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.12.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8_n40-n78 | 7 | 0.5 |
|  | 8 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 7.12.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-8_n40-n78 | 7 | 0 |
|  | 8 | 0.2 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

7.12.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.13	DC_7-28_n1-n40

7.13.1	Operating bands for DC

Table 7.13.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-28_n1-n40 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

7.13.2	Configuration for DC

Table 7.13.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-28A_n1A-n40A | DC_7A_n1A DC_7A_n40A DC_28A_n1A DC_28A_n40A | CA_7A-28A | CA_n1A-n40A |

7.13.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

7.13.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.13.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-28_n1-n40 | 7 | 0.8 |
|  | 28 | 0.6 |
|  | n1 | 0.6 |
|  | n40 | 0.9 |

Table 7.13.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-28_n1-n40 | 7 | 0.3 |
|  | 28 | 0.2 |
|  | n1 | 0 |
|  | n40 | 0.8 |

7.13.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.14	DC_1-3_n41-n77

7.14.1	Operating bands for DC

Table 7.14.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3_n41-n77 | 1, 3 | n41, n77 |

7.14.2	Channel bandwidths per operating band for DC

Table 7.14.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A | 1 | See CA_1A-3A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.14.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.14.4	∆TIB and ∆RIB values

For DC_1-3_n41-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.14.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n41-n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n41 | 0.5 |
|  | n77 | 0.8 |

Table 7.14.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n41-n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n41 | 0 |
|  | n77 | 0.5 |

7.14.5	MSD

There are no additional MSD requirements for this band combination.

7.15	DC_1-3_n3-n41

7.15.1	Operating bands for DC

Table 7.15.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3_n3-n41 | 1, 3 | n3, n41 |

7.15.2	Channel bandwidths per operating band for DC

Table 7.15.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A1 DC_3A_n41A | 1 | See CA_1A-3A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n41A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.15.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.15.4	∆TIB and ∆RIB values

For DC_1-3_n3-n41, the TIB,c and RIB,c values are given in the tables below.

Table 7.15.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n3-n41 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | n3 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.15.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n3-n41 | 1 | 0 |
|  | 3 | 0 |
|  | n3 | 0 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

7.15.5	MSD

There are no additional MSD requirements for this band combination.

7.16	DC_1-3_n3-n77

7.16.1	Operating bands for DC

Table 7.16.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3_n3-n77 | 1, 3 | n3, n77 |

7.16.2	Channel bandwidths per operating band for DC

Table 7.16.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A1 DC_3A_n77A | 1 | See CA_1A-3A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.16.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.16.4	∆TIB and ∆RIB values

For DC_1-3_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.16.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n3-n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 7.16.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n3-n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

7.16.5	MSD

There are no additional MSD requirements for this band combination.

7.17	DC_1-3_n3-n78

7.17.1	Operating bands for DC

Table 7.17.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3_n3-n78 | 1, 3 | n3, n78 |

7.17.2	Channel bandwidths per operating band for DC

Table 7.17.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A1 DC_3A_n78A | 1 | See CA_1A-3A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.17.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.17.4	∆TIB and ∆RIB values

For DC_1-3_n3-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.17.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n3-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 7.17.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n3-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

7.17.5	MSD

There are no additional MSD requirements for this band combination.

7.18	DC_1-18_n3-n77

7.18.1	Operating bands for DC

Table 7.18.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-18_n3-n77 | 1, 18 | n3, n77 |

7.18.2	Channel bandwidths per operating band for DC

Table 7.18.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-18A_n3A-n77A | DC_1A_n3A DC_1A_n77A | 1 | See CA_1A-18A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.18.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.18.4	∆TIB and ∆RIB values

For DC_1-18_n3-n77, the TIB,c and RIB,c values have existed in TS 38.101-3.

7.18.5	MSD

There are no additional MSD requirements for this band combination.

7.19	DC_1-41_n3-n41

7.19.1	Operating bands for DC

Table 7.19.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-41_n3-n41 | 1, 41 | n3, n41 |

7.19.2	Channel bandwidths per operating band for DC

Table 7.19.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-41A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_41A_n3A | 1 | See CA_1A-41A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n41A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.19.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.19.4	∆TIB and ∆RIB values

For DC_1-41_n3-n41, the TIB,c and RIB,c values are given in the tables below.

Table 7.19.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n3-n41 | 1 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.19.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n3-n41 | 1 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

7.19.5	MSD

There are no additional MSD requirements for this band combination.

7.20	DC_1-41_n3-n77

7.20.1	Operating bands for DC

Table 7.20.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-41_n3-n77 | 1, 41 | n3, n77 |

7.20.2	Channel bandwidths per operating band for DC

Table 7.20.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-41A_n3A-n77A | DC_1A_n3A DC_1A_n77A | 1 | See CA_1A-41A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.20.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.20.4	∆TIB and ∆RIB values

For DC_1-41_n3-n77, the TIB,c and RIB,c values are already defined in TS 38.101-3.

7.20.5	MSD

There are no additional MSD requirements for this band combination.

7.21	DC_1-41_n3-n78

7.21.1	Operating bands for DC

Table 7.21.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-41_n3-n78 | 1, 41 | n3, n78 |

7.21.2	Channel bandwidths per operating band for DC

Table 7.21.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-41A_n3A-n78A | DC_1A_n3A DC_1A_n78A | 1 | See CA_1A-41A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.21.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.21.4	∆TIB and ∆RIB values

For DC_1-41_n3-n78, the TIB,c and RIB,c values are already defined in TS 38.101-3.

7.21.5	MSD

There are no additional MSD requirements for this band combination.

7.22	DC_1-41_n41-n77

7.22.1	Operating bands for DC

Table 7.22.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-41_n41-n77 | 1, 41 | n41, n77 |

7.22.2	Channel bandwidths per operating band for DC

Table 7.22.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-41A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_41A_n77A | 1 | See CA_1A-41A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.22.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.22.4	∆TIB and ∆RIB values

For DC_1-41_n41-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.22.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n41-n77 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n41 | 0.5 |
|  | n77 | 0.8 |

Table 7.22.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n41-n77 | 1 | 0 |
|  | 41 | 0 |
|  | n41 | 0 |
|  | n77 | 0.5 |

7.22.5	MSD

There are no additional MSD requirements for this band combination.

7.23	DC_1-41_n41-n78

7.23.1	Operating bands for DC

Table 7.23.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-41_n41-n78 | 1, 41 | n41, n78 |

7.23.2	Channel bandwidths per operating band for DC

Table 7.23.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-41A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_41A_n78A | 1 | See CA_1A-41A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.23.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.23.4	∆TIB and ∆RIB values

For DC_1-41_n41-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.23.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n41-n78 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n41 | 0.5 |
|  | n78 | 0.8 |

Table 7.23.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n41-n78 | 1 | 0 |
|  | 41 | 0 |
|  | n41 | 0 |
|  | n78 | 0.5 |

7.23.5	MSD

There are no additional MSD requirements for this band combination.

7.24	DC_3-18_n3-n77

7.24.1	Operating bands for DC

Table 7.24.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_3-18_n3-n77 | 3, 18 | n3, n77 |

7.24.2	Channel bandwidths per operating band for DC

Table 7.24.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_3A-18A_n3A-n77A | DC_3A_n3A1 DC_3A_n77A DC_18A_n3A DC_18A_n77A | 3 | See CA_3A-18A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.24.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.24.4	∆TIB and ∆RIB values

For DC_3-18_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.24.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n3-n77 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 7.24.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n3-n77 | 3 | 0.2 |
|  | 18 | 0 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

7.24.5	MSD

There are no additional MSD requirements for this band combination.

7.25	DC_3-18_n3-n78

7.25.1	Operating bands for DC

Table 7.25.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_3-18_n3-n78 | 3, 18 | n3, n78 |

7.25.2	Channel bandwidths per operating band for DC

Table 7.25.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_3A-18A_n3A-n78A | DC_3A_n3A DC_3A_n78A DC_18A_n3A DC_18A_n78A | 3 | See CA_3A-18A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.25.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.25.4	∆TIB and ∆RIB values

For DC_3-18_n3-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.25.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n3-n78 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 7.25.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n3-n78 | 3 | 0.2 |
|  | 18 | 0 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

7.25.5	MSD

There are no additional MSD requirements for this band combination.

7.26	DC_3-41_n3-n41

7.26.1	Operating bands for DC

Table 7.26.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_3-41_n3-n41 | 3, 41 | n3, n41 |

7.26.2	Channel bandwidths per operating band for DC

Table 7.26.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_3A-41A_n3A-n41A | DC_3A_n3A1 DC_3A_n41A DC_41A_n3A | 3 | See CA_3A-41A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n41A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.26.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.26.4	∆TIB and ∆RIB values

For DC_3-41_n3-n41, the TIB,c and RIB,c values are given in the tables below.

Table 7.26.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n3-n41 | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.26.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n3-n41 | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

7.26.5	MSD

There are no additional MSD requirements for this band combination.

7.27	DC_3-41_n3-n77

7.27.1	Operating bands for DC

Table 7.27.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_3-41_n3-n77 | 3, 41 | n3, n77 |

7.27.2	Channel bandwidths per operating band for DC

Table 7.27.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_3A-41A_n3A-n77A | DC_3A_n3A1 DC_3A_n77A DC_41A_n3A DC_41A_n77A | 3 | See CA_3A-41A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41C_n3A-n77A | DC_3A_n3A1 DC_3A_n77A DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A | 3 | See CA_3A-41C BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.27.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.27.4	∆TIB and ∆RIB values

For DC_3-41_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.27.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n3-n77 | 3 | 0.6 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.27.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n3-n77 | 3 | 0.2 |
|  | 41 | 01/0.52 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

7.27.5	MSD

There are no additional MSD requirements for this band combination.

7.28	DC_3-41_n3-n78

7.28.1	Operating bands for DC

Table 7.28.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_3-41_n3-n78 | 3, 41 | n3, n78 |

7.28.2	Channel bandwidths per operating band for DC

Table 7.28.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_3A-41A_n3A-n78A | DC_3A_n3A1 DC_3A_n78A DC_41A_n3A DC_41A_n78A | 3 | See CA_3A-41A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41C_n3A-n78A | DC_3A_n3A1 DC_3A_n78A DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A | 3 | See CA_3A-41C BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.28.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.28.4	∆TIB and ∆RIB values

For DC_3-41_n3-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.28.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n3-n78 | 3 | 0.6 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.28.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n3-n78 | 3 | 0.2 |
|  | 41 | 01/0.52 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

7.28.5	MSD

There are no additional MSD requirements for this band combination.

7.29	DC_3-41_n41-n77

7.29.1	Operating bands for DC

Table 7.29.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_3-41_n41-n77 | 3, 41 | n41, n77 |

7.29.2	Channel bandwidths per operating band for DC

Table 7.29.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_3A-41A_n41A-n77A | DC_3A_n41A DC_3A_n77A DC_41A_n77A | 3 | See CA_3A-41A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.29.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.29.4	∆TIB and ∆RIB values

For DC_3-41_n41-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.29.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n41-n77 | 3 | 0.6 |
|  | 41 | 0.31/0.82 |
|  | n41 | 0.31/0.82 |
|  | n77 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.29.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n41-n77 | 3 | 0.2 |
|  | 41 | 01/0.52 |
|  | n41 | 01/0.52 |
|  | n77 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

7.29.5	MSD

There are no additional MSD requirements for this band combination.

7.30	DC_3-41_n41-n78

7.30.1	Operating bands for DC

Table 7.30.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_3-41_n41-n78 | 3, 41 | n41, n78 |

7.30.2	Channel bandwidths per operating band for DC

Table 7.30.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_3A-41A_n41A-n78A | DC_3A_n41A DC_3A_n78A DC_41A_n78A | 3 | See CA_3A-41A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.30.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.30.4	∆TIB and ∆RIB values

For DC_3-41_n41-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.30.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n41-n78 | 3 | 0.6 |
|  | 41 | 0.31/0.82 |
|  | n41 | 0.31/0.82 |
|  | n78 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.30.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n41-n78 | 3 | 0.2 |
|  | 41 | 01/0.52 |
|  | n41 | 01/0.52 |
|  | n78 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

7.30.5	MSD

There are no additional MSD requirements for this band combination.

7.31	DC_2-7_n38-n66

7.31.1	Operating bands for DC

Table 7.31.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_2-7_n38-n66 DC_2-7-7_n38-n66 | 2, 7 | n38, n66 |

7.31.2	Channel bandwidths per operating band for DC

Table 7.31.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_2A-7A_n38A-n66A | DC_2A_n38A DC_2A_n66A DC_7A_n66A | 2 | See CA_2A-7A BCS configurations in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | See CA_n38A-n66A BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-7C_n38A-n66A | DC_2A_n38A DC_2A_n66A DC_7A_n66A | 2 | See CA_2A-7C BCS configurations in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | See CA_n38A-n66A BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-7A-7A_n38A-n66A | DC_2A_n38A DC_2A_n66A DC_7A_n66A | 2 | See CA_2A-7A-7A BCS configurations in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | See CA_n38A-n66A BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.31.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.31.4	∆TIB and ∆RIB values

For DC_2-7_n38-n66, the TIB,c and RIB,c values are given in the tables below.

Table 7.31.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n38-n66 DC_2-7-7_n38-n66 | 2 | 0.5 |
|  | n66 | 0.5 |

Table 7.31.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n38-n66 DC_2-7-7_n38-n66 | 2 | 0.3 |
|  | n66 | 0.5 |

7.31.5	MSD

There are no additional MSD requirements for this band combination.

7.32	DC_7-66_n38-n78 and DC_7-7-66_n38-n78

7.32.1	Operating bands for DC

Table 7.32.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_7-66_n38-n78 DC_7-7-66_n38-n78 | 7, 66 | n38, n78 |

7.32.2	Channel bandwidths per operating band for DC

Table 7.32.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_7A-66A_n38A-n78A | DC_66A_n38A DC_66A_n78A | 7 | See CA_7A-66A BCS configurations in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | See CA_n38A-n78A BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A-7A-66A_n38A-n78A | DC_66A_n38A DC_66A_n78A | 7 | See CA_7A-7A-66A BCS configurations in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | See CA_n38A-n78A BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7C-66A_n38A-n78A | DC_66A_n38A DC_66A_n78A | 7 | See CA_7C-66A BCS configurations in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | See CA_n38A-n78A BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.32.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.32.4	∆TIB and ∆RIB values

For DC_7-66_n38-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.32.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n38-n78 DC_7-7-66_n38-n78 | 66 | 0.6 |
|  | n78 | 0.8 |

Table 7.32.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n38-n78 DC_7-7-66_n38-n78 | 66 | 0.2 |
|  | n78 | 0.5 |

7.32.5	MSD

There are no additional MSD requirements for this band combination.

7.33	DC_1-8_n3-n77

7.33.1	Operating bands for DC

Table 7.33.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n3-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.33.2	Channel bandwidths per operating band for DC

Table 7.33.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-8A_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.33.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8_n3, DC_1-8_n77, DC_1_n3-n77 and DC_8_n3-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.33.4	∆TIB and ∆RIB values

For DC_1-8_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.33.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n3-n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | n3 | 0.8 |
|  | n77 | 0.8 |

Table 7.33.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n3-n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

7.33.5	MSD

As mentioned in 7.33.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.34	DC_1-11_n3-n28

7.34.1	Operating bands for DC

Table 7.34.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-11_n3-n28 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

7.34.2	Channel bandwidths per operating band for DC

Table 7.34.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-11A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_11A_n3A DC_11A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.34.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-11_n3, DC_1-11_n28, DC_1_n3-n28 and DC_11_n3-n28. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.34.4	∆TIB and ∆RIB values

For DC_1-11_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 7.34.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11_n3-n28 | 1 | 0.3 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n28 | 0.6 |

Table 7.34.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-11_n3-n28 | 1 | 0 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n28 | 0.2 |

7.34.5	MSD

As mentioned in 7.34.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.35	DC_8-11_n3-n28

7.35.1	Operating bands for DC

Table 7.35.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-11_n3-n28 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

7.35.2	Channel bandwidths per operating band for DC

Table 7.35.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-11A_n3A-n28A | DC_8A_n3A DC_8A_n28A DC_11A_n3A DC_11A_n28A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 70 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.35.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-11_n3, DC_8-11_n28, DC_8_n3-n28 and DC_11_n3-n28. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.35.4	∆TIB and ∆RIB values

For DC_8-11_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 7.35.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-11_n3-n28 | 8 | 0.6 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n28 | 0.6 |

Table 7.35.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-11_n3-n28 | 8 | 0.2 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n28 | 0.2 |

7.35.5	MSD

As mentioned in 7.35.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.36	DC_1-18_n28-n41

7.36.1	Operating bands for DC

Table 7.36.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-18A_n28A-n41A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

7.36.2	Channel bandwidths per operating band for DC

Table 7.36.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-18A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_18A_n28A DC_18A_n41A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

7.36.3	Co-existence studies

Co-existence studies of DC_1A-18A_n28A-n41A and protected bands are already covered in the constituent fall-back modes.

7.36.4	∆TIB and ∆RIB values

For DC_1-18_n28-n41, the TIB,c and RIB,c values are given in the tables below.

Table 7.36.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n28-n41 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n28 | 0.5 |
|  | n41 | 0.31 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

Table 7.36.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n28-n41 | 1 | 0 |
|  | 18 | 0 |
|  | n28 | 0.2 |
|  | n41 | 01 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

7.36.5	MSD

All MSD requirements forDC_1-18_n28-n41 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.37	DC_1-18_n28-n77

7.37.1	Operating bands for DC

Table 7.37.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-18A_n28A-n77A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.37.2	Channel bandwidths per operating band for DC

Table 7.37.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-18A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_18A_n28A DC_18A_n77A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

7.37.3	Co-existence studies

Co-existence studies of DC_1A-18A_n28A-n77A and protected bands are already covered in the constituent fall-back modes.

7.37.4	∆TIB and ∆RIB values

For DC_1-18_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.37.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n28-n77 | 1 | 0.3 |
|  | 18 | 0.5 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |

Table 7.37.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n28-n77 | 1 | 0 |
|  | 18 | 0 |
|  | n28 | 0 |
|  | n77 | 0.5 |

7.37.5	MSD

All MSD requirements forDC_1-18_n28-n77 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.38	DC_1-18_n28-n78

7.38.1	Operating bands for DC

Table 7.38.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-18A_n28A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.38.2	Channel bandwidths per operating band for DC

Table 7.38.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-18A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_18A_n28A DC_18A_n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

7.38.3	Co-existence studies

Co-existence studies of DC_1A-18A_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

7.38.4	∆TIB and ∆RIB values

For DC_1-18_n28-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.38.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n28-n78 | 1 | 0.3 |
|  | 18 | 0.5 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 7.38.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n28-n78 | 1 | 0 |
|  | 18 | 0 |
|  | n28 | 0 |
|  | n78 | 0.5 |

7.38.5	MSD

All MSD requirements forDC_1-18_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.39	DC_1-18_n3-n41

7.39.1	Operating bands for DC

Table 7.39.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-18A_n3A-n41A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

7.39.2	Channel bandwidths per operating band for DC

Table 7.39.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-18A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_18A_n3A DC_18A_n41A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 175 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

7.39.3	Co-existence studies

Co-existence studies of DC_1A-18A_n3A-n41A and protected bands are already covered in the constituent fall-back modes.

7.39.4	∆TIB and ∆RIB values

For DC_1-18_n3-n41, the TIB,c and RIB,c values are given in the tables below.

Table 7.39.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n3-n41 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n3 | 0.3 |
|  | n41 | 0.31 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

Table 7.39.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n3-n41 | 1 | 0 |
|  | 18 | 0 |
|  | n3 | 0 |
|  | n41 | 01 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

7.39.5	MSD

All MSD requirements forDC_1-18_n3-n41 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.40	DC_1-18_n41-n77

7.40.1	Operating bands for DC

Table 7.40.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-18A_n41A-n77A | 1 | 1920MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.40.2	Channel bandwidths per operating band for DC

Table 7.40.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-18A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_18A_n41A DC_18A_n77A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 235 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

7.40.3	Co-existence studies

Co-existence studies of DC_1A-18A_n41A-n77A and protected bands are already covered in the constituent fall-back modes.

7.40.4	∆TIB and ∆RIB values

For DC_1-18_n41-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.40.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n41-n77 | 1 | 0.6 |
|  | 18 | 0.3 |
|  | n41 | 0.5 |
|  | n77 | 0.8 |

Table 7.40.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n41-n77 | 1 | 0.2 |
|  | 18 | 0 |
|  | n41 | 0 |
|  | n77 | 0.5 |

7.40.5	MSD

All MSD requirements forDC_1-18_n41-n77 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.41	DC_1-18_n41-n78

7.41.1	Operating bands for DC

Table 7.41.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-18A_n41A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.41.2	Channel bandwidths per operating band for DC

Table 7.41.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-18A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_18A_n41A DC_18A_n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 235 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

7.41.3	Co-existence studies

Co-existence studies of DC_1A-18A_n41A-n78A and protected bands are already covered in the constituent fall-back modes.

7.41.4	∆TIB and ∆RIB values

For DC_1-18_n41-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.41.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n41-n78 | 1 | 0.5 |
|  | 18 | 0.3 |
|  | n41 | 0.5 |
|  | n78 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

Table 7.41.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n41-n78 | 1 | 0 |
|  | 18 | 0 |
|  | n41 | 0 |
|  | n78 | 0.5 |

7.41.5	MSD

All MSD requirements forDC_1-18_n41-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.42	DC_1-41_n28-n41

7.42.1	Operating bands for DC

Table 7.42.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-41A_n28A-n41A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

7.42.2	Channel bandwidths per operating band for DC

Table 7.42.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-41A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_41A_n28A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

7.42.3	Co-existence studies

Co-existence studies of DC_1A-41A_n28A-n41A and protected bands are already covered in the constituent fall-back modes.

7.42.4	∆TIB and ∆RIB values

For DC_1-41_n28-n41, the TIB,c and RIB,c values are given in the tables below.

Table 7.42.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n28-n41 | 1 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n28 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

Table 7.42.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n28-n41 | 1 | 0 |
|  | 41 | 01/0.52 |
|  | n28 | 0.2 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

7.42.5	MSD

All MSD requirements forDC_1-41_n28-n41 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.43	DC_3-18_n28-n41

7.43.1	Operating bands for DC

Table 7.43.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-18A_n28A-n41A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

7.43.2	Channel bandwidths per operating band for DC

Table 7.43.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-18A_n28A-n41A | DC_3A_n28A DC_3A_n41A DC_18A_n28A DC_18A_n41A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

7.43.3	Co-existence studies

Co-existence studies of DC_3A-18A_n28A-n41A and protected bands are already covered in the constituent fall-back modes.

7.43.4	∆TIB and ∆RIB values

For DC_3-18_n28-n41, the TIB,c and RIB,c values are given in the tables below.

Table 7.43.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n28-n41 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n28 | 0.5 |
|  | n41 | 0.31 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

Table 7.43.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n28-n41 | 3 | 0.2 |
|  | 18 | 0 |
|  | n28 | 0.2 |
|  | n41 | 01 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

7.43.5	MSD

All MSD requirements forDC_3-18_n28-n41 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.44	DC_3-18_n28-n77

7.44.1	Operating bands for DC

Table 7.44.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-18A_n28A-n77A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.44.2	Channel bandwidths per operating band for DC

Table 7.44.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-18A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_18A_n28A DC_18A_n77A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

7.44.3	Co-existence studies

Co-existence studies of DC_3A-18A_n28A-n77A and protected bands are already covered in the constituent fall-back modes.

7.44.4	∆TIB and ∆RIB values

For DC_3-18_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.44.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n28-n77 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |

Table 7.44.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n28-n77 | 3 | 0.2 |
|  | 18 | 0 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

7.44.5	MSD

All MSD requirements forDC_3-18_n28-n77 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.45	DC_3-18_n28-n78

7.45.1	Operating bands for DC

Table 7.45.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-18A_n28A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.45.2	Channel bandwidths per operating band for DC

Table 7.45.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-18A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_18A_n28A DC_18A_n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

7.45.3	Co-existence studies

Co-existence studies of DC_3A-18A_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

7.45.4	∆TIB and ∆RIB values

For DC_3-18_n28-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.45.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n28-n78 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 7.45.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n28-n78 | 3 | 0.2 |
|  | 18 | 0 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

7.45.5	MSD

All MSD requirements forDC_3-18_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.46	DC_3-18_n3-n41

7.46.1	Operating bands for DC

Table 7.46.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-18A_n3A-n41A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

7.46.2	Channel bandwidths per operating band for DC

Table 7.46.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-18A_n3A-n41A | DC_3A_n3A1 DC_3A_n41A DC_18A_n3A DC_18A_n41A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 175 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.46.3	Co-existence studies

Co-existence studies of DC_3A-18A_n3A-n41A and protected bands are already covered in the constituent fall-back modes.

7.46.4	∆TIB and ∆RIB values

For DC_3-18_n3-n41, the TIB,c and RIB,c values are given in the tables below.

Table 7.46.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n3-n41 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n3 | 0.6 |
|  | n41 | 0.31 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

Table 7.46.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n3-n41 | 3 | 0.2 |
|  | 18 | 0 |
|  | n3 | 0.2 |
|  | n41 | 01 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

7.46.5	MSD

All MSD requirements forDC_3-18_n3-n41 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.47	DC_3-18_n41-n77

7.47.1	Operating bands for DC

Table 7.47.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-18A_n41A-n77A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.47.2	Channel bandwidths per operating band for DC

Table 7.47.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-18A_n41A-n77A | DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 235 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

7.47.3	Co-existence studies

Co-existence studies of DC_3A-18A_n41A-n77A and protected bands are already covered in the constituent fall-back modes.

7.47.4	∆TIB and ∆RIB values

For DC_3-18_n41-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.47.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n41-n77 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n41 | 0.5 |
|  | n77 | 0.8 |

Table 7.47.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n41-n77 | 3 | 0.2 |
|  | 18 | 0 |
|  | n41 | 0 |
|  | n77 | 0.5 |

7.47.5	MSD

All MSD requirements forDC_3-18_n41-n77 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.48	DC_3-18_n41-n78

7.48.1	Operating bands for DC

Table 7.48.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-18A_n41A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.48.2	Channel bandwidths per operating band for DC

Table 7.48.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-18A_n41A-n78A | DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 235 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

7.48.3	Co-existence studies

Co-existence studies of DC_3A-18A_n41A-n78A and protected bands are already covered in the constituent fall-back modes.

7.48.4	∆TIB and ∆RIB values

For DC_3-18_n41-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.48.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n41-n78 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n41 | 0.6 |
|  | n78 | 0.8 |

Table 7.48.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n41-n78 | 3 | 0.2 |
|  | 18 | 0 |
|  | n41 | 01 |
|  | n78 | 0.5 |

7.48.5	MSD

All MSD requirements forDC_3-18_n41-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.49	DC_3-41_n28-n41

7.49.1	Operating bands for DC

Table 7.49.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-41A_n28A-n41A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

7.49.2	Channel bandwidths per operating band for DC

Table 7.49.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-41A_n28A-n41A | DC_3A_n28A DC_3A_n41A DC_41A_n28A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

7.49.3	Co-existence studies

Co-existence studies of DC_3A-41A_n28A-n41A and protected bands are already covered in the constituent fall-back modes.

7.49.4	∆TIB and ∆RIB values

For DC_3-41_n28-n41, the TIB,c and RIB,c values are given in the tables below.

Table 7.49.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n28-n41 | 3 | 0.6 |
|  | 41 | 0.31/0.82 |
|  | n28 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

Table 7.49.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n28-n41 | 3 | 0.2 |
|  | 41 | 01/0.52 |
|  | n28 | 0.2 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

7.49.5	MSD

All MSD requirements forDC_3-41_n28-n41 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.50	DC_3-19_n1-n77

7.50.1	Operating bands for DC

Table 7.50.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-19_n1-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.50.2	Configuration for DC

Table 7.50.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-19A_n1A-n77A | DC_3A_n1A DC_3A_n77A DC_19A_n1A DC_19A_n77A | CA_3A-19A | CA_n1A-n77A |

7.50.3	Co-existence studies

Co-existence studies of DC_3-19_n1-n77 are already covered by the fallback configurations, DC_3-19_n1, DC_3-19_n77, DC_3_n1-n77 and DC_19_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.50.4	∆TIB and ∆RIB values

For DC_3-19_n1-n77, the TIB,c and RIB,c values are reused from DC_1-3-19_n77, and are given in the tables below.

Table 7.50.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19_n1-n77 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 7.50.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-19_n1-n77 | 3 | 0.2 |
|  | 19 | 0 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

7.50.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.51	DC_3-19_n1-n78

7.51.1	Operating bands for DC

Table 7.51.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-19_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.51.2	Configuration for DC

Table 7.51.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-19A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_19A_n1A DC_19A_n78A | CA_3A-19A | CA_n1A-n78A |

7.51.3	Co-existence studies

Co-existence studies of DC_3-19_n1-n78 are already covered by the fallback configurations, DC_3-19_n1, DC_3-19_n78, DC_3_n1-n78 and DC_19_n1-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.51.4	∆TIB and ∆RIB values

For DC_3-19_n1-n78, the TIB,c and RIB,c values are reused from DC_1-3-19_n78, and are given in the tables below.

Table 7.51.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19_n1-n78 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 7.51.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-19_n1-n78 | 3 | 0.2 |
|  | 19 | 0 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

7.51.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.52	DC_3-19_n1-n79

7.52.1	Operating bands for DC

Table 7.52.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-19_n1-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.52.2	Configuration for DC

Table 7.52.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-19A_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_19A_n1A DC_19A_n79A | CA_3A-19A | CA_n1A-n79A |

7.52.3	Co-existence studies

Co-existence studies of DC_3-19_n1-n79 are already covered by the fallback configurations, DC_3-19_n1, DC_3-19_n79, DC_3_n1-n79 and DC_19_n1-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.52.4	∆TIB and ∆RIB values

For DC_3-19_n1-n79, the TIB,c and RIB,c values are reused from DC_1-3-19_n79, and are given in the tables below.

Table 7.52.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19_n1-n79 | 3 | 0.3 |
|  | 19 | 0.3 |
|  | n1 | 0.3 |
|  | n79 | 0 |

Table 7.52.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-19_n1-n79 | 3 | 0 |
|  | 19 | 0 |
|  | n1 | 0 |
|  | n79 | 0 |

7.52.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.53	DC_3-21_n1-n77

7.53.1	Operating bands for DC

Table 7.53.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-21_n1-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.53.2	Configuration for DC

Table 7.53.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-21A_n1A-n77A | DC_3A_n1A DC_3A_n77A DC_21A_n1A DC_21A_n77A | CA_3A-21A | CA_n1A-n77A |

7.53.3	Co-existence studies

Co-existence studies of DC_3-21_n1-n77 are already covered by the fallback configurations, DC_3-21_n1, DC_3-21_n77, DC_3_n1-n77 and DC_21_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.53.4	∆TIB and ∆RIB values

For DC_3-21_n1-n77, the TIB,c and RIB,c values are reused from DC_1-3-21_n77, and are given in the tables below.

Table 7.53.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n1-n77 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 7.53.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21_n1-n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

7.53.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.54	DC_3-21_n1-n78

7.54.1	Operating bands for DC

Table 7.54.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-21_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.54.2	Configuration for DC

Table 7.54.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-21A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_21A_n1A DC_21A_n78A | CA_3A-21A | CA_n1A-n78A |

7.54.3	Co-existence studies

Co-existence studies of DC_3-21_n1-n78 are already covered by the fallback configurations, DC_3-21_n1, DC_3-21_n78, DC_3_n1-n78 and DC_21_n1-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.54.4	∆TIB and ∆RIB values

For DC_3-21_n1-n78, the TIB,c and RIB,c values are reused from DC_1-3-21_n78, and are given in the tables below.

Table 7.54.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n1-n78 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 7.54.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21_n1-n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

7.54.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.55	DC_3-21_n1-n79

7.55.1	Operating bands for DC

Table 7.55.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-21_n1-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.55.2	Configuration for DC

Table 7.55.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-21A_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_21A_n1A DC_21A_n79A | CA_3A-21A | CA_n1A-n79A |

7.55.3	Co-existence studies

Co-existence studies of DC_3-21_n1-n79 are already covered by the fallback configurations, DC_3-21_n1, DC_3-21_n79, DC_3_n1-n79 and DC_21_n1-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.55.4	∆TIB and ∆RIB values

For DC_3-21_n1-n79, the TIB,c and RIB,c values are reused from DC_1-3-21_n79, and are given in the tables below.

Table 7.55.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n1-n79 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n1 | 0.3 |
|  | n79 | 0 |

Table 7.55.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21_n1-n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n1 | 0 |
|  | n79 | 0 |

7.55.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.56	DC_3-42_n1-n77

7.56.1	Operating bands for DC

Table 7.56.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-42_n1-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.56.2	Configuration for DC

Table 7.56.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-42A_n1A-n77A | DC_3A_n1A DC_3A_n77A | CA_3A-42A | CA_n1A-n77A |
| DC_3A-42C_n1A-n77A | DC_3A_n1A DC_3A_n77A | CA_3A-42C | CA_n1A-n77A |

7.56.3	Co-existence studies

Co-existence studies of DC_3-42_n1-n77 are already covered by the fallback configurations, DC_3-42_n1, DC_3-42_n77 and DC_3_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.56.4	∆TIB and ∆RIB values

For DC_3-42_n1-n77, the TIB,c and RIB,c values are reused from DC_1-3-42_n77, and are given in the tables below.

Table 7.56.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-42_n1-n77 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 7.56.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-42_n1-n77 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

7.56.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.57	DC_3-42_n1-n78

7.57.1	Operating bands for DC

Table 7.57.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-42_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.57.2	Configuration for DC

Table 7.57.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-42A_n1A-n78A | DC_3A_n1A DC_3A_n78A | CA_3A-42A | CA_n1A-n78A |
| DC_3A-42C_n1A-n78A | DC_3A_n1A DC_3A_n78A | CA_3A-42C | CA_n1A-n78A |

7.57.3	Co-existence studies

Co-existence studies of DC_3-42_n1-n78 are already covered by the fallback configurations, DC_3-42_n1, DC_3-42_n78 and DC_3_n1-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.57.4	∆TIB and ∆RIB values

For DC_3-42_n1-n78, the TIB,c and RIB,c values are reused from DC_1-3-42_n78, and are given in the tables below.

Table 7.57.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-42_n1-n78 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 7.57.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-42_n1-n78 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

7.57.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.58	DC_3-42_n1-n79

7.58.1	Operating bands for DC

Table 7.58.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-42_n1-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.58.2	Configuration for DC

Table 7.58.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-42A_n1A-n79A | DC_3A_n1A DC_3A_n79A | CA_3A-42A | CA_n1A-n79A |
| DC_3A-42C_n1A-n79A | DC_3A_n1A DC_3A_n79A | CA_3A-42C | CA_n1A-n79A |

7.58.3	Co-existence studies

Co-existence studies of DC_3-42_n1-n79 are already covered by the fallback configurations, DC_3-42_n1, DC_3-42_n79 and DC_3_n1-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.58.4	∆TIB and ∆RIB values

For DC_3-42_n1-n79, the TIB,c and RIB,c values are reused from DC_1-3-42_n79, and are given in the tables below.

Table 7.58.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-42_n1-n79 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n79 | 0 |

Table 7.58.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-42_n1-n79 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n79 | 0 |

7.58.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.59	DC_19-21_n1-n77

7.59.1	Operating bands for DC

Table 7.59.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21_n1-n77 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.59.2	Configuration for DC

Table 7.59.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A-21A_n1A-n77A | DC_19A_n1A DC_19A_n77A DC_21A_n1A DC_21A_n77A | CA_19A-21A | CA_n1A-n77A |

7.59.3	Co-existence studies

Co-existence studies of DC_19-21_n1-n77 are already covered by the fallback configurations, DC_19-21_n1, DC_19-21_n77, DC_19_n1-n77 and DC_21_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.59.4	∆TIB and ∆RIB values

For DC_19-21_n1-n77, the TIB,c and RIB,c values are reused from DC_1-19-21_n77, and are given in the tables below.

Table 7.59.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21_n1-n77 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n1 | 0.3 |
|  | n77 | 0.8 |

Table 7.59.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21_n1-n77 | 19 | 0 |
|  | 21 | 0 |
|  | n1 | 0 |
|  | n77 | 0.5 |

7.59.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.60	DC_19-21_n1-n78

7.60.1	Operating bands for DC

Table 7.60.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21_n1-n78 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.60.2	Configuration for DC

Table 7.60.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A-21A_n1A-n78A | DC_19A_n1A DC_19A_n78A DC_21A_n1A DC_21A_n78A | CA_19A-21A | CA_n1A-n78A |

7.60.3	Co-existence studies

Co-existence studies of DC_19-21_n1-n78 are already covered by the fallback configurations, DC_19-21_n1, DC_19-21_n78, DC_19_n1-n78 and DC_21_n1-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.60.4	∆TIB and ∆RIB values

For DC_19-21_n1-n78, the TIB,c and RIB,c values are reused from DC_1-19-21_n78, and are given in the tables below.

Table 7.60.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21_n1-n78 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 7.60.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21_n1-n78 | 19 | 0 |
|  | 21 | 0 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

7.60.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.61	DC_19-21_n1-n79

7.61.1	Operating bands for DC

Table 7.61.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21_n1-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.61.2	Configuration for DC

Table 7.61.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A-21A_n1A-n79A | DC_19A_n1A DC_19A_n79A DC_21A_n1A DC_21A_n79A | CA_19A-21A | CA_n1A-n79A |

7.61.3	Co-existence studies

Co-existence studies of DC_19-21_n1-n79 are already covered by the fallback configurations, DC_19-21_n1, DC_19-21_n79, DC_19_n1-n79 and DC_21_n1-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.61.4	∆TIB and ∆RIB values

For DC_19-21_n1-n79, the TIB,c and RIB,c values are reused from DC_1-19-21_n79, and are given in the tables below.

Table 7.61.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21_n1-n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n1 | 0.3 |
|  | n79 | 0 |

Table 7.61.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21_n1-n79 | 19 | 0 |
|  | 21 | 0 |
|  | n1 | 0 |
|  | n79 | 0 |

7.61.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.62	DC_19-42_n1-n77

7.62.1	Operating bands for DC

Table 7.62.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-42_n1-n77 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.62.2	Configuration for DC

Table 7.62.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A-42A_n1A-n77A | DC_19A_n1A DC_19A_n77A | CA_19A-42A | CA_n1A-n77A |
| DC_19A-42C_n1A-n77A | DC_19A_n1A DC_19A_n77A | CA_19A-42C | CA_n1A-n77A |

7.62.3	Co-existence studies

Co-existence studies of DC_19-42_n1-n77 are already covered by the fallback configurations, DC_19-42_n1, DC_19-42_n77 and DC_19_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.62.4	∆TIB and ∆RIB values

For DC_19-42_n1-n77, the TIB,c and RIB,c values are reused from DC_1-19-42_n77, and are given in the tables below.

Table 7.62.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-42_n1-n77 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 7.62.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-42_n1-n77 | 19 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

7.62.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.63	DC_19-42_n1-n78

7.63.1	Operating bands for DC

Table 7.63.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-42_n1-n78 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.63.2	Configuration for DC

Table 7.63.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A-42A_n1A-n78A | DC_19A_n1A DC_19A_n78A | CA_19A-42A | CA_n1A-n78A |
| DC_19A-42C_n1A-n78A | DC_19A_n1A DC_19A_n78A | CA_19A-42C | CA_n1A-n78A |

7.63.3	Co-existence studies

Co-existence studies of DC_19-42_n1-n78 are already covered by the fallback configurations, DC_19-42_n1, DC_19-42_n78 and DC_19_n1-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.63.4	∆TIB and ∆RIB values

For DC_19-42_n1-n78, the TIB,c and RIB,c values are reused from DC_1-19-42_n78, and are given in the tables below.

Table 7.63.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-42_n1-n78 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |

Table 7.63.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-42_n1-n78 | 19 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0 |
|  | n78 | 0.5 |

7.63.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.64	DC_19-42_n1-n79

7.64.1	Operating bands for DC

Table 7.64.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-42_n1-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.64.2	Configuration for DC

Table 7.64.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A-42A_n1A-n79A | DC_19A_n1A DC_19A_n79A | CA_19A-42A | CA_n1A-n79A |
| DC_19A-42C_n1A-n79A | DC_19A_n1A DC_19A_n79A | CA_19A-42C | CA_n1A-n79A |

7.64.3	Co-existence studies

Co-existence studies of DC_19-42_n1-n79 are already covered by the fallback configurations, DC_19-42_n1, DC_19-42_n79 and DC_19_n1-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.64.4	∆TIB and ∆RIB values

For DC_19-42_n1-n79, the TIB,c and RIB,c values are reused from DC_1-19-42_n79, and are given in the tables below.

Table 7.64.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-42_n1-n79 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n79 | 0 |

Table 7.64.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-42_n1-n79 | 19 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0 |
|  | n79 | 0 |

7.64.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.65	DC_21-42_n1-n77

7.65.1	Operating bands for DC

Table 7.65.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21-42_n1-n77 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.65.2	Configuration for DC

Table 7.65.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_21A-42A_n1A-n77A | DC_21A_n1A DC_21A_n77A | CA_21A-42A | CA_n1A-n77A |
| DC_21A-42C_n1A-n77A | DC_21A_n1A DC_21A_n77A | CA_21A-42C | CA_n1A-n77A |

7.65.3	Co-existence studies

Co-existence studies of DC_21-42_n1-n77 are already covered by the fallback configurations, DC_21-42_n1, DC_21-42_n77 and DC_21_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.65.4	∆TIB and ∆RIB values

For DC_21-42_n1-n77, the TIB,c and RIB,c values are reused from DC_1-21-42_n77, and are given in the tables below.

Table 7.65.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21-42_n1-n77 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 7.65.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21-42_n1-n77 | 21 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

7.65.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.66	DC_21-42_n1-n78

7.66.1	Operating bands for DC

Table 7.66.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21-42_n1-n78 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.66.2	Configuration for DC

Table 7.66.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_21A-42A_n1A-n78A | DC_21A_n1A DC_21A_n78A | CA_21A-42A | CA_n1A-n78A |
| DC_21A-42C_n1A-n78A | DC_21A_n1A DC_21A_n78A | CA_21A-42C | CA_n1A-n78A |

7.66.3	Co-existence studies

Co-existence studies of DC_21-42_n1-n78 are already covered by the fallback configurations, DC_21-42_n1, DC_21-42_n78 and DC_21_n1-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.66.4	∆TIB and ∆RIB values

For DC_21-42_n1-n78, the TIB,c and RIB,c values are reused from DC_1-21-42_n78, and are given in the tables below.

Table 7.66.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21-42_n1-n78 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |

Table 7.66.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21-42_n1-n78 | 21 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0 |
|  | n78 | 0.5 |

7.66.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.67	DC_21-42_n1-n79

7.67.1	Operating bands for DC

Table 7.67.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21-42_n1-n79 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.67.2	Configuration for DC

Table 7.67.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_21A-42A_n1A-n79A | DC_21A_n1A DC_21A_n79A | CA_21A-42A | CA_n1A-n79A |
| DC_21A-42C_n1A-n79A | DC_21A_n1A DC_21A_n79A | CA_21A-42C | CA_n1A-n79A |

7.67.3	Co-existence studies

Co-existence studies of DC_21-42_n1-n79 are already covered by the fallback configurations, DC_21-42_n1, DC_21-42_n79 and DC_21_n1-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.67.4	∆TIB and ∆RIB values

For DC_21-42_n1-n79, the TIB,c and RIB,c values are reused from DC_1-21-42_n79, and are given in the tables below.

Table 7.67.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21-42_n1-n79 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n79 | 0 |

Table 7.67.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21-42_n1-n79 | 21 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0 |
|  | n79 | 0 |

7.67.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.68	DC_3-20_n1-n78

7.68.1	Operating bands for DC

Table 7.68.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-20A_n1A-n78A DC_3C-20A_n1A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.68.2	Channel bandwidths per operating band for DC

Table 7.68.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3C-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A DC_3C_n1A DC_3C_n78A | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.68.3	Co-existence studies

Co-existence studies of DC_3-20_n1-n78 and protected bands are already covered in the constituent fall-back modes.

7.68.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_3-20_n1-n78 are given in the tables below reusing the values for DC_1-3-20_n78.

Table 7.68.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20_n1-n78 | n1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |

Table 7.68.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-20_n1-n78 | n1 | 0.2 |
|  | 3 | 0.2 |
|  | 20 | 0 |
|  | n78 | 0.5 |

7.68.5	MSD

All MSD requirements for DC_3-20_n1-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.69	DC_7-20_n1-n78

7.69.1	Operating bands for DC

Table 7.69.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-20A_n1A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.69.2	Channel bandwidths per operating band for DC

Table 7.69.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_7A-20A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_20A_n1A DC_20A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.69.3	Co-existence studies

Co-existence studies of DC_7-20_n1-n78 and protected bands are already covered in the constituent fall-back modes.

7.69.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_7-20_n1-n78 are given in the tables below reusing the values for DC_1-7-20_n78.

Table 7.69.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20_n1-n78 | n1 | 0.6 |
|  | 7 | 0.7 |
|  | 20 | 0.4 |
|  | n78 | 0.8 |

Table 7.69.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7-20_n1-n78 | n1 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n78 | 0.5 |

7.69.5	MSD

All MSD requirements for DC_7-20_n1-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.70	DC_3-7_n40-n78

7.70.1	Operating bands for DC

Table 7.70.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n40-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.70.2	Channel bandwidths per operating band for DC

Table 7.70.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_3A-7A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes9 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.70.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

7.70.4	∆TIB and ∆RIB values

For DC_3-7_n40-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.70.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n40-n78 | 3 | 0.6 |
|  | 7 | 0.5 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 7.70.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n40-n78 | 3 | 0.2 |
|  | 7 | 0 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

7.70.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.71	DC_1-7_n40-n78

7.71.1	Operating bands for DC

Table 7.71.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7_n40-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.71.2	Channel bandwidths per operating band for DC

Table 7.71.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_1A-7A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_7A_n40A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes9 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.71.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

7.71.4	∆TIB and ∆RIB values

For DC_1-7_n40-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.71.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7_n40-n78 | 1 | 0.6 |
|  | 7 | 0.5 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 7.71.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7_n40-n78 | 1 | 0.2 |
|  | 7 | 0 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

7.71.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.72	DC_7-28_n40-n78

7.72.1	Operating bands for DC

Table 7.72.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-28_n40-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.72.2	Channel bandwidths per operating band for DC

Table 7.72.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_7A-28A_n40A-n78A | DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes9 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.72.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

7.72.4	∆TIB and ∆RIB values

For DC_7-28_n40-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.72.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-28_n40-n78 | 7 | 0.5 |
|  | 28 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 7.72.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-28_n40-n78 | 7 | 0 |
|  | 28 | 0.2 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

7.72.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.73	DC_2-13_n66-n77

7.73.1	Operating bands for DC

Table 7.73.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-13_n66-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.73.2	Channel bandwidths per operating band for DC

Table 7.73.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A-13A_n66A-n77A DC_2A-13A_n66A-n77C | DC_2A_n66A DC_2A_n77A DC_13A_n66A DC_13A_n77A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 270 |
|  |  | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| DC_2A-2A-13A_n66A-n77A |  | DC_2A_n66A DC_2A_n77A DC_13A_n66A DC_13A_n77A |  | 2 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 190 |  |
|  |  |  |  | 13 |  | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n66 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | 15 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes4 |  | Yes |  | Yes4 |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 1: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.73.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

7.73.4	∆TIB and ∆RIB values

For DC_2-13_n66-n77, the TIB,c and RIB,c values are derived from DC_2-13-66_n48 and are given in the tables below.

Table 7.73.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n66-n77 | 2 | 0.6 |
|  | 13 | 0.3 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 7.73.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-13_n66-n77 | 2 | 0.3 |
|  | 13 | 0 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

7.73.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.74	DC_2-66_n66-n77

7.74.1	Operating bands for DC

Table 7.74.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-66_n66-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.74.2	Channel bandwidths per operating band for DC

Table 7.74.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A-66A_n66A-n77A DC_2A-66A_n66A-n77C | DC_2A_n66A DC_2A_n77A DC_66A_n77A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 280 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| DC_2A-2A-66A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_66A_n77A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.74.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

7.74.4	∆TIB and ∆RIB values

For DC_2-66_n66-n77, the TIB,c and RIB,c values are derived from DC_2_n66-n78 and are given in the tables below.

Table 7.74.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n66-n77 DC_2-2-66_n66-n77 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 7.74.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n66-n77 DC_2-2-66_n66-n77 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

7.74.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.75	DC_2-66_n5-n77

7.75.1	Operating bands for DC

Table 7.75.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-66_n5-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.75.2	Channel bandwidths per operating band for DC

Table 7.75.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A-66A_n5A-n77A | DC_66A_n5A DC_66A_n77A DC_2A_n5A DC_2A_n77A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.75.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

7.75.4	∆TIB and ∆RIB values

For DC_2-66_n5-n77, the TIB,c and RIB,c values are derived from DC_2-13-66_n48 and are given in the tables below.

Table 7.75.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n5-n77 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n5 | 0.3 |
|  | n77 | 0.8 |

Table 7.75.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n5-n77 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n5 | 0 |
|  | n77 | 0.5 |

7.75.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.76	DC_13-66_n66-n77

7.76.1	Operating bands for DC

Table 7.76.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13-66_n66-n77 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.76.2	Channel bandwidths per operating band for DC

Table 7.76.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_13A-66A_n66A-n77A | DC_13A_n66A DC_13A_n77A DC_66A_n77A | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.76.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

7.76.4	∆TIB and ∆RIB values

For DC_13-66_n66-n77, the TIB,c and RIB,c values are derived from DC_13-66_n48 and are given in the tables below.

Table 7.76.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n66-n77 | 13 | 0.3 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 7.76.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13-66_n66-n77 | 13 | 0 |
|  | 66 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |

7.76.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.77	DC_13-66_n2-n77

7.77.1	Operating bands for DC

Table 7.77.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13-66_n2-n77 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.77.2	Channel bandwidths per operating band for DC

Table 7.77.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_13A-66A_n2A-n77A | DC_13A_n2A DC_13A_n77A DC_66A_n2A DC_66A_n77A | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.77.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

7.77.4	∆TIB and ∆RIB values

For DC_13-66_n2-n77, the TIB,c and RIB,c values are derived from DC_2-13-66_n48 and are given in the tables below.

Table 7.77.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n2-n77 | 13 | 0.3 |
|  | 66 | 0.6 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 7.77.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13-66_n2-n77 | 13 | 0 |
|  | 66 | 0.2 |
|  | n2 | 0.2 |
|  | n77 | 0.5 |

7.77.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.78	DC_13-66_n5-n48

7.78.1	Operating bands for DC

Table 7.78.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13-66_n5-n48 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n48 | 3550 MHz | – | 3800 MHz | 3550 MHz | – | 3800 MHz | TDD |

7.78.2	Channel bandwidths per operating band for DC

Table 7.78.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_13A-66A_n5A-n48A | DC_13A_n48A DC_66A_n5A DC_66A_n48A | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n48 | 15 | Yes5 | Yes | Yes | Yes |  |  | Yes | Yes6 |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.78.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

7.78.4	∆TIB and ∆RIB values

For DC_13-66_n5-n48, the TIB,c and RIB,c values are derived from DC_5-48-66_n12 and are given in the tables below.

Table 7.78.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n5-n48 | 13 | 0.4 |
|  | 66 | 0.6 |
|  | n5 | 0.8 |
|  | n48 | 0.8 |

Table 7.78.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13-66_n5-n48 | 13 | 0.3 |
|  | 66 | 0.2 |
|  | n5 | 0.5 |
|  | n48 | 0.5 |

7.78.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.79	DC_1-8_n78-n258

7.79.1	Operating bands for DC

Table 7.79.1-1: DC band combination LTE 2bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n78-n258 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz |  |

7.79.2	Channel bandwidths per operating band for DC

Table 7.79.2-1: Supported bandwidths per DC LTE 2bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_1A-8A_n78A-n258A DC_1A-8A_n78A-n258D DC_1A-8A_n78A-n258E DC_1A-8A_n78A-n258F DC_1A-8A_n78A-n258G DC_1A-8A_n78A-n258H DC_1A-8A_n78A-n258I DC_1A-8A_n78A-n258J DC_1A-8A_n78A-n258K DC_1A-8A_n78A-n258L DC_1A-8A_n78A-n258M | DC_1A_n78A DC_1A_n258A DC_8A_n78A DC_8A_n258A | 1 | See CA_1A-8A in TS 36.101 | 330 430 530 630 730 830 930 |
|  |  | 8 |  |  |
|  |  | n78 | CA_n78A-n258A/D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n258 |  |  |

7.79.3	Co-existence studies

The co-existence summary in section 5.3 of 37.717-11-21 already captures the needed studies. There is no additional harmonic and/or intermodulation impact for this combination.

7.79.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

7.79.5	MSD

No additional MSD requirement is needed.

7.80	DC_3-8_n78-n258

7.80.1	Operating bands for DC

Table 7.80.1-1: DC band combination of LTE 2bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8_n78-n258 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz |  |

7.80.2	Channel bandwidths per operating band for DC

Table 7.80.2-1: Supported bandwidths per DC LTE 2bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_3A-8A_n78A-n258A DC_3A-8A_n78A-n258D DC_3A-8A_n78A-n258E DC_3A-8A_n78A-n258F DC_3A-8A_n78A-n258G DC_3A-8A_n78A-n258H DC_3A-8A_n78A-n258I DC_3A-8A_n78A-n258J DC_3A-8A_n78A-n258K DC_3A-8A_n78A-n258L DC_3A-8A_n78A-n258M | DC_3A_n78A DC_3A_n258A DC_8A_n78A DC_8A_n258A | 3 | See CA_3A-8A in TS 36.101 | 330 430 530 630 730 830 930 |
|  |  | 8 |  |  |
|  |  | n78 | CA_n78A-n258A/D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n258 |  |  |

7.80.3	Co-existence studies

The co-existence summary in section 5.3 of 37.717-11-21 already captures the needed studies. There is no additional harmonic and/or intermodulation impact for this combination.

7.80.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

7.80.5	MSD

No additional MSD requirement is needed.

7.81	DC_7-8_n78-n258

7.81.1	Operating bands for DC

Table 7.81.1-1: DC band combination of LTE 2bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-8_n78-n258 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz |  |

7.81.2	Channel bandwidths per operating band for DC

Table 7.81.2-1: Supported bandwidths per DC LTE 2bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_7A-8A_n78A-n258A DC_7A-8A_n78A-n258D DC_7A-8A_n78A-n258E DC_7A-8A_n78A-n258F DC_7A-8A_n78A-n258G DC_7A-8A_n78A-n258H DC_7A-8A_n78A-n258I DC_7A-8A_n78A-n258J DC_7A-8A_n78A-n258K DC_7A-8A_n78A-n258L DC_7A-8A_n78A-n258M | DC_7A_n78A DC_7A_n258A DC_8A_n78A DC_8A_n258A | 7 | See CA_7A-8A in TS 36.101 | 330 430 530 630 730 830 930 |
|  |  | 8 |  |  |
|  |  | n78 | CA_n78A-n258A/D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n258 |  |  |

7.81.3	Co-existence studies

The co-existence summary in section 5.3 of 37.717-11-21 already captures the needed studies. There is no additional harmonic and/or intermodulation impact for this combination.

7.81.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

7.81.5	MSD

No additional MSD requirement is needed.

7.82	DC_1-8_n40-n258

7.82.1	Operating bands for DC

Table 7.82.1-1: DC band combination of LTE 2bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n40-n258 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz |  |

7.82.2	Channel bandwidths per operating band for DC

Table 7.82.2-1: Supported bandwidths per DC LTE 2bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_1A-8A_n40A-n258A DC_1A-8A_n40A-n258D DC_1A-8A_n40A-n258E DC_1A-8A_n40A-n258F DC_1A-8A_n40A-n258G DC_1A-8A_n40A-n258H DC_1A-8A_n40A-n258I DC_1A-8A_n40A-n258J DC_1A-8A_n40A-n258K DC_1A-8A_n40A-n258L DC_1A-8A_n40A-n258M | DC_1A_n40A DC_1A_n258A DC_8A_n40A DC_8A_n258A | 1 | See CA_1A-8A in TS 36.101 | 310 410 510 610 710 810 910 |
|  |  | 8 |  |  |
|  |  | n40 | CA_n40A-n258A/D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n258 |  |  |

7.82.3	Co-existence studies

The co-existence summary in section 5.3 of 37.717-11-21 already captures the needed studies. There is no additional harmonic and/or intermodulation impact for this combination.

7.82.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

7.82.5	MSD

No additional MSD requirement is needed.

7.83	DC_3-8_n40-n258

7.83.1	Operating bands for DC

Table 7.83.1-1: DC band combination of LTE 2 bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8_n40-n258 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz |  |

7.83.2	Channel bandwidths per operating band for DC

Table 7.83.2-1: Supported bandwidths per DC LTE 2 bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_3A-8A_n40A-n258A DC_3A-8A_n40A-n258D DC_3A-8A_n40A-n258E DC_3A-8A_n40A-n258F DC_3A-8A_n40A-n258G DC_3A-8A_n40A-n258H DC_3A-8A_n40A-n258I DC_3A-8A_n40A-n258J DC_3A-8A_n40A-n258K DC_3A-8A_n40A-n258L DC_3A-8A_n40A-n258M | DC_3A_n40A DC_3A_n258A DC_8A_n40A DC_8A_n258A | 3 | See CA_3A-8A in TS 36.101 | 310 410 510 610 710 810 910 |
|  |  | 8 |  |  |
|  |  | n40 | CA_n40A-n258A/D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n258 |  |  |

7.83.3	Co-existence studies

The co-existence summary in section 5.3 of 37.717-11-21 already captures the needed studies. There is no additional harmonic and/or intermodulation impact for this combination.

7.83.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

7.83.5	MSD

No additional MSD requirement is needed.

7.84	DC_7-8_n40-n258

7.84.1	Operating bands for DC

Table 7.84.1-1: DC band combination of LTE 2 bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-8_n40-n258 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz |  |

7.84.2	Channel bandwidths per operating band for DC

Table 7.84.2-1: Supported bandwidths per DC LTE 2 bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_7A-8A_n40A-n258A DC_7A-8A_n40A-n258D DC_7A-8A_n40A-n258E DC_7A-8A_n40A-n258F DC_7A-8A_n40A-n258G DC_7A-8A_n40A-n258H DC_7A-8A_n40A-n258I DC_7A-8A_n40A-n258J DC_7A-8A_n40A-n258K DC_7A-8A_n40A-n258L DC_7A-8A_n40A-n258M | DC_7A_n40A DC_7A_n258A DC_8A_n40A DC_8A_n258A | 7 | See CA_7A-8A in TS 36.101 | 310 410 510 610 710 810 910 |
|  |  | 8 |  |  |
|  |  | n40 | CA_n40A-n258A/D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n258 |  |  |

7.84.3	Co-existence studies

The co-existence summary in section 5.3 of 37.717-11-21 already captures the needed studies. There is no additional harmonic and/or intermodulation impact for this combination.

7.84.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

7.84.5	MSD

No additional MSD requirement is needed.

7.85	DC_3-11_n28-n77

7.85.1	Operating bands for DC

Table 7.85.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-11_n28-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.85.2	Channel bandwidths per operating band for DC

Table 7.85.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_3A-11A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_3A-11A_n28A-n77(2A) | DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 250 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.85.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_3-11_n28, DC_3-11_n77, DC_3_n28-n77 and DC_11_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.85.4	∆TIB and ∆RIB values

For DC_3-11_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.85.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-11_n28-n77 | 3 | 0.8 |
|  | 11 | 0.9 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 7.85.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-11_n28-n77 | 3 | 0.3 |
|  | 11 | 0.5 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

7.85.5	MSD

As mentioned in 7.85.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.86	DC_8-11_n28-n77

7.86.1	Operating bands for DC

Table 7.86.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-11_n28-n77 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.86.2	Channel bandwidths per operating band for DC

Table 7.86.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-11A_n28A-n77A | DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
| DC_8A-11A_n28A-n77(2A) | DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 240 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.86.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-11_n28, DC_8-11_n77, DC_8_n28-n77 and DC_11_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.86.4	∆TIB and ∆RIB values

For DC_8-11_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.86.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-11_n28-n77 | 8 | 0.6 |
|  | 11 | 0.4 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 7.86.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-11_n28-n77 | 8 | 0.2 |
|  | 11 | 0 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

7.86.5	MSD

As mentioned in 7.86.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.87	DC_1-11_n3-n77

7.87.1	Operating bands for DC

Table 7.87.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-11_n3-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.87.2	Channel bandwidths per operating band for DC

Table 7.87.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-11A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_11A_n3A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-11A_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_11A_n3A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.87.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-11_n3, DC_1-11_n77, DC_1_n3-n77 and DC_11_n3-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.87.4	∆TIB and ∆RIB values

For DC_1-11_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.87.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11_n3-n77 | 1 | 0.6 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n77 | 0.8 |

Table 7.87.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-11_n3-n77 | 1 | 0.2 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n77 | 0.5 |

7.87.5	MSD

As mentioned in 7.87.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.88	DC_8-11_n3-n77

7.88.1	Operating bands for DC

Table 7.88.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-11_n3-n77 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.88.2	Channel bandwidths per operating band for DC

Table 7.88.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-11A_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_8A-11A_n3A-n77(2A) | DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 250 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.88.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-11_n3, DC_8-11_n77, DC_8_n3-n77 and DC_11_n3-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.88.4	∆TIB and ∆RIB values

For DC_8-11_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.88.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-11_n3-n77 | 8 | 0.6 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n77 | 0.8 |

Table 7.88.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-11_n3-n77 | 8 | 0.2 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n77 | 0.5 |

7.88.5	MSD

As mentioned in 7.88.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.89	DC_1-11_n28-n77

7.89.1	Operating bands for DC

Table 7.89.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-11_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.89.2	Channel bandwidths per operating band for DC

Table 7.89.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-11A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_11A_n28A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-11A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_11A_n28A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 250 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.89.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-11_n28, DC_1-11_n77, DC_1_n28-n77 and DC_11_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.89.4	∆TIB and ∆RIB values

For DC_1-11_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.89.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11_n28-n77 | 1 | 0.6 |
|  | 11 | 0.4 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 7.89.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-11_n28-n77 | 1 | 0.2 |
|  | 11 | 0 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

7.89.5	MSD

As mentioned in 7.89.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.90	DC_1-42_n3-n28

7.90.1	Operating bands for DC

Table 7.90.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-42_n3-n28 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

7.90.2	Channel bandwidths per operating band for DC

Table 7.90.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-42A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_42A_n3A DC_42A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 90 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42C_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_42A_n3A DC_42A_n28A DC_42C_n3A DC_42C_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 110 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.90.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-42_n3, DC_1-42_n28, DC_1_n3-n28 and DC_42_n3-n28. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.90.4	∆TIB and ∆RIB values

For DC_1-42_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 7.90.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-42_n3-n28 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n3 | 0.6 |
|  | n28 | 0.8 |

Table 7.90.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-42_n3-n28 | 1 | 0 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |
|  | n28 | 0.5 |

7.90.5	MSD

As mentioned in 7.90.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.91	DC_1-42_n3-n77

7.91.1	Operating bands for DC

Table 7.91.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-42_n3-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.91.2	Channel bandwidths per operating band for DC

Table 7.91.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-42A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_42A_n3A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-42A_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_42A_n3A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 270 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42C_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_42A_n3A DC_42C_n3A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-42C_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_42A_n3A DC_42C_n3A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 290 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.91.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-42_n3, DC_1-42_n77, DC_1_n3-n77 and DC_42_n3-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.91.4	∆TIB and ∆RIB values

For DC_1-42_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.91.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-42_n3-n77 | 1 | 0.6 |
|  | 42 | 0.8 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 7.91.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-42_n3-n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

7.91.5	MSD

As mentioned in 7.91.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.92	DC_8-42_n3-n28

7.92.1	Operating bands for DC

Table 7.92.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-42_n3-n28 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

7.92.2	Channel bandwidths per operating band for DC

Table 7.92.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-42A_n3A-n28A | DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42A_n28A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 80 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A-42C_n3A-n28A | DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 100 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.92.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-42_n3, DC_8-42_n28, DC_8_n3-n28 and DC_42_n3-n28. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.92.4	∆TIB and ∆RIB values

For DC_8-42_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 7.92.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-42_n3-n28 | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n3 | 0.6 |
|  | n28 | 0.8 |

Table 7.92.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-42_n3-n28 | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |
|  | n28 | 0.5 |

7.92.5	MSD

As mentioned in 7.92.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.93	DC_8-42_n3-n77

7.93.1	Operating bands for DC

Table 7.93.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-42_n3-n77 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.93.2	Channel bandwidths per operating band for DC

Table 7.93.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-42A_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_8A-42A_n3A-n77(2A) | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A-42C_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n77A DC_42C_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_8A-42C_n3A-n77(2A) | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n77A DC_42C_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 280 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.93.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-42_n3, DC_8-42_n77, DC_8_n3-n77 and DC_42_n3-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.93.4	∆TIB and ∆RIB values

For DC_8-42_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.93.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-42_n3-n77 | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 7.93.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-42_n3-n77 | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

7.93.5	MSD

As mentioned in 7.93.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.94	DC_1-21_n28-n77

7.94.1	Operating bands for DC

Table 7.94.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |  |
| DC_1-21_n28-n77 | 1 | 1920 MHz | – | 1980 MHz |  | 2110 MHz | – | 2170 MHz |  | FDD |  |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz |  | 1495.9 MHz | – | 1510.9 MHz |  | FDD |  |
|  | n28 | 703 MHz | – | 748 MHz |  | 758 MHz | – | 803 MHz |  | FDD |  |
|  | n77 | 3300 MHz | – | 4200 MHz |  | 3300 MHz | – | 4200 MHz |  | TDD |  |

7.94.2	Configuration for DC

Table 7.94.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-21A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_21A_n28A DC_21A_n77A | CA_1A-21A | CA_n28A-n77A |

7.94.3	Co-existence studies

Co-existence studies are omitted since those are covered by the co-existence studies for the lower order of fallback band combinations such as DC of LTE 1 band + NR 1band, LTE 2 bands + NR 1band, and LTE 1band + NR 2bands.

7.94.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.94.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21_n28-n77 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 7.94.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-21_n28-n77 | 1 | 0.2 |
|  | 21 | 0 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

7.94.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.95	DC_1-21_n28-n78

7.95.1	Operating bands for DC

Table 7.95.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |  |
| DC_1-21_n28-n78 | 1 | 1920 MHz | – | 1980 MHz |  | 2110 MHz | – | 2170 MHz |  | FDD |  |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz |  | 1495.9 MHz | – | 1510.9 MHz |  | FDD |  |
|  | n28 | 703 MHz | – | 748 MHz |  | 758 MHz | – | 803 MHz |  | FDD |  |
|  | n78 | 3300 MHz | – | 3800 MHz |  | 3300 MHz | – | 3800 MHz |  | TDD |  |

7.95.2	Configuration for DC

Table 7.95.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-21A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_21A_n28A DC_21A_n78A | CA_1A-21A | CA_n28A-n78A |

7.95.3	Co-existence studies

Co-existence studies are omitted since those are covered by the co-existence studies for the lower order of fallback band combinations such as DC of LTE 1 band + NR 1band, LTE 2 bands + NR 1band, and LTE 1band + NR 2bands.

7.95.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.95.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21_n28-n78 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 7.95.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-21_n28-n78 | 1 | 0.2 |
|  | 21 | 0 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

7.95.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.96	DC_1-21_n28-n79

7.96.1	Operating bands for DC

Table 7.96.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |  |
| DC_1-21_n28-n79 | 1 | 1920 MHz | – | 1980 MHz |  | 2110 MHz | – | 2170 MHz |  | FDD |  |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz |  | 1495.9 MHz | – | 1510.9 MHz |  | FDD |  |
|  | n28 | 703 MHz | – | 748 MHz |  | 758 MHz | – | 803 MHz |  | FDD |  |
|  | n79 | 4400 MHz | – | 5000 MHz |  | 4400 MHz | – | 5000 MHz |  | TDD |  |

7.96.2	Configuration for DC

Table 7.96.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-21A_n28A-n79A | DC_1A_n28A DC_1A_n79A DC_21A_n28A DC_21A_n79A | CA_1A-21A | CA_n28A-n79A |

7.96.3	Co-existence studies

Co-existence studies are omitted since those are covered by the co-existence studies for the lower order of fallback band combinations such as DC of LTE 1 band + NR 1band, LTE 2 bands + NR 1band, and LTE 1band + NR 2bands.

7.96.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.96.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21_n28-n79 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | n28 | 0.6 |
|  | n79 | 0 |

Table 7.96.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-21_n28-n79 | 1 | 0.3 |
|  | 21 | 0 |
|  | n28 | 0.3 |
|  | n79 | 0 |

7.96.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.97	DC_1-3_n28-n79

7.97.1	Operating bands for DC

Table 7.97.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |  |
| DC_1-3_n28-n79 | 1 | 1920 MHz | – | 1980 MHz |  | 2110 MHz | – | 2170 MHz |  | FDD |  |
|  | 3 | 1710 MHz | – | 1785 MHz |  | 1805 MHz | – | 1880 MHz |  | FDD |  |
|  | n28 | 703 MHz | – | 748 MHz |  | 758 MHz | – | 803 MHz |  | FDD |  |
|  | n79 | 4400 MHz | – | 5000 MHz |  | 4400 MHz | – | 5000 MHz |  | TDD |  |

7.97.2	Configuration for DC

Table 7.97.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A_n28A-n79A | DC_1A_n28A DC_1A_n79A DC_3A_n28A DC_3A_n79A | CA_3A-21A | CA_n28A-n79A |

7.97.3	Co-existence studies

Co-existence studies are omitted since those are covered by the co-existence studies for the lower order of fallback band combinations such as DC of LTE 1 band + NR 1band, LTE 2 bands + NR 1band, and LTE 1band + NR 2bands.

7.97.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.97.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n28-n79 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n28 | 0.6 |
|  | n79 | 0 |

Table 7.97.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n28-n79 | 1 | 0 |
|  | 3 | 0 |
|  | n28 | 0.2 |
|  | n79 | 0 |

7.97.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.98	DC_3-21_n28-n77

7.98.1	Operating bands for DC

Table 7.98.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |  |
| DC_3-21_n28-n77 | 3 | 1710 MHz | – | 1785 MHz |  | 1805 MHz | – | 1880 MHz |  | FDD |  |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz |  | 1495.9 MHz | – | 1510.9 MHz |  | FDD |  |
|  | n28 | 703 MHz | – | 748 MHz |  | 758 MHz | – | 803 MHz |  | FDD |  |
|  | n77 | 3300 MHz | – | 4200 MHz |  | 3300 MHz | – | 4200 MHz |  | TDD |  |

7.98.2	Configuration for DC

Table 7.98.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-21A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_21A_n28A DC_21A_n77A | CA_3A-21A | CA_n28A-n77A |

7.98.3	Co-existence studies

Co-existence studies are omitted since those are covered by the co-existence studies for the lower order of fallback band combinations such as DC of LTE 1 band + NR 1band, LTE 2 bands + NR 1band, and LTE 1band + NR 2bands.

7.98.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.98.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n28-n77 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |

Table 7.98.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21_n28-n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

7.98.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.99	DC_3-21_n28-n78

7.99.1	Operating bands for DC

Table 7.99.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |  |
| DC_3-21_n28-n78 | 3 | 1710 MHz | – | 1785 MHz |  | 1805 MHz | – | 1880 MHz |  | FDD |  |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz |  | 1495.9 MHz | – | 1510.9 MHz |  | FDD |  |
|  | n28 | 703 MHz | – | 748 MHz |  | 758 MHz | – | 803 MHz |  | FDD |  |
|  | n78 | 3300 MHz | – | 3800 MHz |  | 3300 MHz | – | 3800 MHz |  | TDD |  |

7.99.2	Configuration for DC

Table 7.99.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-21A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_21A_n28A DC_21A_n78A | CA_3A-21A | CA_n28A-n78A |

7.99.3	Co-existence studies

Co-existence studies are omitted since those are covered by the co-existence studies for the lower order of fallback band combinations such as DC of LTE 1 band + NR 1band, LTE 2 bands + NR 1band, and LTE 1band + NR 2bands.

7.99.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.99.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n28-n78 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 7.99.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21_n28-n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

7.99.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.100	DC_3-21_n28-n79

7.100.1	Operating bands for DC

Table 7.100.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |  |
| DC_3-21_n28-n79 | 3 | 1710 MHz | – | 1785 MHz |  | 1805 MHz | – | 1880 MHz |  | FDD |  |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz |  | 1495.9 MHz | – | 1510.9 MHz |  | FDD |  |
|  | n28 | 703 MHz | – | 748 MHz |  | 758 MHz | – | 803 MHz |  | FDD |  |
|  | n79 | 4400 MHz | – | 5000 MHz |  | 4400 MHz | – | 5000 MHz |  | TDD |  |

7.100.2	Configuration for DC

Table 7.100.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-21A_n28A-n79A | DC_3A_n28A DC_3A_n79A DC_21A_n28A DC_21A_n79A | CA_3A-21A | CA_n28A-n79A |

7.100.3	Co-existence studies

Co-existence studies are omitted since those are covered by the co-existence studies for the lower order of fallback band combinations such as DC of LTE 1 band + NR 1band, LTE 2 bands + NR 1band, and LTE 1band + NR 2bands.

7.100.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.100.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n28-n79 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n28 | 0.3 |
|  | n79 | 0 |

Table 7.100.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21_n28-n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n28 | 0.3 |
|  | n79 | 0 |

7.100.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.101	DC_1-8_n28-n78

7.101.1	 Operating bands for DC

Table 7.101.1-1: LTE 2 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-8A_n28A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.101.2 	Configuration for DC

Table 7.101.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-8A_n28A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.101.3	 Co-existence studies

Co-existence studies of DC_1A-8A_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

7.101.4	 ∆TIB and ∆RIB values

Table 7.101.4-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n28-n78 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 7.101.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n28-n78 | 1 | 0 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

7.101.5	MSD

All MSD requirements for DC_1-8_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.102	DC_3-8_n28-n78

7.102.1	 Operating bands for DC

Table 7.102.1-1: LTE 2 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-8A_n28A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.102.2 	Configuration for DC

Table 7.102.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-8A_n28A-n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.102.3	 Co-existence studies

Co-existence studies of DC_3A-8A_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

7.102.4	 ∆TIB and ∆RIB values

Table 7.102.4-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8_n28-n78 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 7.102.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-8_n28-n78 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

7.102.5	MSD

All MSD requirements for DC_3-8_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.103	DC_7-8_n28-n78

7.103.1	 Operating bands for DC

Table 7.103.1-1: LTE 2 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-8A_n28A-n78A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.103.2 	Configuration for DC

Table 7.103.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A-8A_n28A-n78A |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.103.3	 Co-existence studies

Co-existence studies of DC_7A-8A_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

7.103.4	 ∆TIB and ∆RIB values

Table 7.103.4-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8_n28-n78 | 7 | 0.5 |
|  | 8 | 0.6 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 7.103.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-8_n28-n78 | 7 | 0.2 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

7.103.5	MSD

All MSD requirements for DC_7-8_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.104	DC_3-28_n1-n78

7.104.1	Operating bands for DC

Table 7.104.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-28_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.104.2	Channel bandwidths per operating band for DC

Table 7.104.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_3A-28A_n1A-n78A | DC_3A_n1A DC_28A_n1A DC_3A_n78A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.104.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

7.104.4	∆TIB and ∆RIB values

For DC_3-28_n1-n78, the TIB,c and RIB,c values are derived from DC_1-28_n3-n78 and given in the tables below.

Table 7.104.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n1-n78 | 3 | 0.6 |
|  | 28 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 7.104.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n1-n78 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

7.104.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.105	DC_7-28_n1-n78

7.105.1	Operating bands for DC

Table 7.105.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-28_n1-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.105.2	Channel bandwidths per operating band for DC

Table 7.105.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_7A-28A_n1A-n78A | DC_7A_n1A DC_28A_n1A DC_7A_n78A DC_28A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.105.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

7.105.4	∆TIB and ∆RIB values

For DC_7-28_n1-n78, the TIB,c and RIB,c values are derived from DC_1-7-28_n78 and given in the tables below.

Table 7.105.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-28_n1-n78 | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 7.105.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-28_n1-n78 | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

7.105.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.106	DC_3-40_n1-n78

7.106.1	Operating bands for DC

Table 7.106.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-40A_n1A-n78A DC_3A-40C_n1A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.106.2	Channel bandwidths per operating band for DC

Table 7.106.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_40A_n1A DC_40A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 40 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_40A_n1A DC_40A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.106.3	Co-existence studies

Co-existence studies of DC_3-40_n1-n78 and protected bands are already covered in the constituent fall-back modes.

7.106.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_3-40_n1-n78 are given in the tables below reusing the values for DC_1-3_n40-n78.

Table 7.106.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-40_n1-n78 | n1 | 0.5 |
|  | 3 | 0.6 |
|  | 40 | 0.31 |
|  | n78 | 0.81 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 7.106.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-40_n1-n78 | n1 | 0 |
|  | 3 | 0.2 |
|  | 40 | 0.41 |
|  | n78 | 0.51 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

7.106.5	MSD

All MSD requirements for DC_3-40_n1-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.107	DC_7-40_n1-n78

7.107.1	Operating bands for DC

Table 7.107.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-40A_n1A-n78A DC_7A-40C_n1A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.107.2	Channel bandwidths per operating band for DC

Table 7.107.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_7A-40A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 40 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_7A-40C_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.107.3	Co-existence studies

Co-existence studies of DC_7-40_n1-n78 and protected bands are already covered in the constituent fall-back modes.

7.107.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_7-40_n1-n78 are given in the tables below reusing the values for DC_1-7-40_n78.

Table 7.107.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-40_n1-n78 | n1 | 0.6 |
|  | 7 | 0.5 |
|  | 40 | 0.51 |
|  | n78 | 0.81 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 7.107.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7-40_n1-n78 | n1 | 0.2 |
|  | 7 | 0 |
|  | 40 | 0.41 |
|  | n78 | 0.51 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

7.107.5	MSD

All MSD requirements for DC_7-40_n1-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.108	DC_8-40_n1-n78

7.108.1	Operating bands for DC

Table 7.108.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8A-40A_n1A-n78A DC_8A-40C_n1A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.108.2	Channel bandwidths per operating band for DC

Table 7.108.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_8A-40A_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 40 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_8A-40C_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.108.3	Co-existence studies

Co-existence studies of DC_8-40_n1-n78 and protected bands are already covered in the constituent fall-back modes.

7.108.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_8-40_n1-n78 are given in the tables below reusing the values for DC_1-8_n40-n78.

Table 7.108.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-40_n1-n78 | n1 | 0.5 |
|  | 8 | 0.3 |
|  | 40 | 0.51 |
|  | n78 | 0.81 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 7.108.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_8-40_n1-n78 | n1 | 0 |
|  | 8 | 0.2 |
|  | 40 | 0.41 |
|  | n78 | 0.51 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

7.108.5	MSD

All MSD requirements for DC_8-40_n1-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.109	DC_2-7_n25-n66

7.109.1	Operating bands for DC

Table 7.109.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-7_n25-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

7.109.2	Channel bandwidths per operating band for DC

Table 7.109.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A-7A_n25A-n66A6,7 | DC_2A_n66A DC_7A_n25A DC_7A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 120 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| DC_2A-7A-7A_n25A-n66A6,7 | DC_2A_n66A DC_7A_n25A DC_7A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | 7 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| DC_2A-7C_n25A-n66A6,7 | DC_2A_n66A DC_7A_n25A DC_7A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band contiguous or non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply for the Band 42 and Band n77/n78 combination when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.109.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

7.109.4	∆TIB and ∆RIB values

For DC_2-7_n25-n66, the TIB,c and RIB,c values are derived from DC_2-7_n66 and given in the tables below.

Table 7.109.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n25-n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 7.109.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7_n25-n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | n25 | 0.3 |
|  | n66 | 0.5 |

7.109.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.110	DC_2-13_n25-n66

7.110.1	Operating bands for DC

Table 7.110.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-13_n25-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

7.110.2	Channel bandwidths per operating band for DC

Table 7.110.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A-13A_n25A-n66A6,7 | DC_2A_n66A DC_13A_n25A DC_13A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 110 |
|  |  | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band contiguous or non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply for the Band 42 and Band n77/n78 combination when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.110.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

7.110.4	∆TIB and ∆RIB values

For DC_2-13_n25-n66, the TIB,c and RIB,c values are derived from DC_2-13_n66 and given in the tables below.

Table 7.110.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n25-n66 | 2 | 0.5 |
|  | 13 | 0.3 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 7.110.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-13_n25-n66 | 2 | 0.3 |
|  | 13 | 0 |
|  | n25 | 0.3 |
|  | n66 | 0.3 |

7.110.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.111	DC_2-66_n25-n66

7.111.1	Operating bands for DC

Table 7.111.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-66_n25-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

7.111.2	Channel bandwidths per operating band for DC

Table 7.111.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A-66A_n25A-n66A6,7 | DC_2A_n66A DC_66A_n25A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 120 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band contiguous or non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply for the Band 42 and Band n77/n78 combination when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.111.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

7.111.4	∆TIB and ∆RIB values

For DC_2-66_n25-n66, the TIB,c and RIB,c values are derived from DC_2_n66 and given in the tables below.

Table 7.111.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n25-n66 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 7.111.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n25-n66 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n25 | 0.3 |
|  | n66 | 0.3 |

7.111.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.112	DC_7-13_n25-n66

7.112.1	Operating bands for DC

Table 7.112.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-13_n25-n66 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

7.112.2	Channel bandwidths per operating band for DC

Table 7.112.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_7A-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 110 |
|  |  | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| DC_7A-7A-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A | 7 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| DC_7C-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

7.112.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

7.112.4	∆TIB and ∆RIB values

For DC_7-13_n25-n66, the TIB,c and RIB,c values are derived from DC_2-7-13_n66 and given in the tables below.

Table 7.112.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-13_n25-n66 | 7 | 0.5 |
|  | 13 | 0.3 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 7.112.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-13_n25-n66 | 7 | 0.5 |
|  | 13 | 0 |
|  | n25 | 0.3 |
|  | n66 | 0.5 |

7.112.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.113	DC_7-66_n25-n66

7.113.1	Operating bands for DC

Table 7.113.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-66_n25-n66 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

7.113.2	Channel bandwidths per operating band for DC

Table 7.113.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_7A-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 120 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| DC_7A-7A-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A | 7 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| DC_7C-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

7.113.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

7.113.4	∆TIB and ∆RIB values

For DC_7-66_n25-n66, the TIB,c and RIB,c values are derived from DC_2-7_n66 and given in the tables below.

Table 7.113.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n25-n66 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 7.113.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-66_n25-n66 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n25 | 0.3 |
|  | n66 | 0.5 |

7.113.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.114	DC_3-7_n3-n78

7.114.1	Operating bands for DC

Table 7.114.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n3-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.114.2	Channel bandwidths per operating band for DC

Table 7.114.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_3A-7A_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_3A_n78A DC_7A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-7C_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_3A_n78A DC_7C_n78A DC_7A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.114.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

7.114.4	∆TIB and ∆RIB values

For DC_3-7_n3-n78, the TIB,c and RIB,c values are derived from CA_n3-n7-n78 and given in the tables below.

Table 7.114.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n3-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 7.114.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n3-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

7.114.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.115	DC_3-28_n3-n78

7.115.1	Operating bands for DC

Table 7.115.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-28_n3-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.115.2	Channel bandwidths per operating band for DC

Table 7.115.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_3A-28A_n3A-n78A | DC_3A_n3A4 DC_28A_n3A DC_3A_n78A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.115.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

7.115.4	∆TIB and ∆RIB values

For DC_3-28_n3-n78, the TIB,c and RIB,c values are derived from CA_n3-n28-n78 and given in the tables below.

Table 7.115.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n3-n78 | 3 | 0.5 |
|  | 28 | 0.3 |
|  | n3 | 0.5 |
|  | n78 | 0.8 |

Table 7.115.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n3-n78 | 3 | 0 |
|  | 28 | 0.2 |
|  | n3 | 0 |
|  | n78 | 0.5 |

7.115.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.116	DC_3-7_n1-n8, DC_3-3-7_n1-n8, DC_3-7-7_n1-n8, DC_3-3-7-7_n1-n8

7.116.1	Operating bands for DC

Table 7.116.1-1: DC band combination of two LTE bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n1-n8, DC_3-3-7_n1-n8, DC_3-7-7_n1-n8, DC_3-3-7-7_n1-n8 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz |  |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |

7.116.2	Configuration for DC

Table 7.116.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A_n1A-n8A DC_3A-3A-7A_n1A-n8A DC_3A-7A-7A_n1A-n8A DC_3A-3A-7A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A | CA_3A-7A CA_3A-3A-7A CA_3A-7A-7A CA_3A-3A-7A-7A | CA_n1A-n8A |

7.116.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.116.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below.

Table 7.116.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n1-n8, DC_3-3-7_n1-n8, DC_3-7-7_n1-n8, DC_3-3-7-7_n1-n8 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n1 | 0.6 |
|  | n8 | 0.6 |

Table 7.116.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n1-n8, DC_3-3-7_n1-n8, DC_3-7-7_n1-n8, DC_3-3-7-7_n1-n8 | 3 | 0 |
|  | 7 | 0 |
|  | n1 | 0 |
|  | n8 | 0.2 |

7.116.5	MSD

No additional MSD requirement is needed.

7.117	DC_3-7_n8-n78, DC_3-3-7_n8-n78, DC_3-7-7_n8-n78, DC_3-3-7-7_n8-n78

7.117.1	Operating bands for DC

Table 7.117.1-1: DC band combination of two LTE bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n8-n78, DC_3-3-7_n8-n78, DC_3-7-7_n8-n78, DC_3-3-7-7_n8-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.117.2	Configuration for DC

Table 7.117.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A_n8A-n78A2 DC_3A-3A-7A_n8A-n78A2 DC_3A-7A-7A_n8A-n78A2 DC_3A-3A-7A-7A_n8A-n78A2 | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A | CA_3A-7A CA_3A-3A-7A CA_3A-7A-7A CA_3A-3A-7A-7A | CA_n8A-n78A |
| NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |

7.117.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

7.117.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below, which is the same as the values for DC_3-7-8_n78.

Table 7.117.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n8-n78, DC_3-3-7_n8-n78, DC_3-7-7_n8-n78, DC_3-3-7-7_n8-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 7.117.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n8-n78, DC_3-3-7_n8-n78, DC_3-7-7_n8-n78, DC_3-3-7-7_n8-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

7.117.5	MSD

No additional MSD requirement is needed.

7.118	DC_1-7_n8-n78

### 7.118.1	 Operating bands for DC

Table 7.118.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-7A_n8A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.118.2 	Configuration for DC

Table 7.118.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-7A_n8A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.118.3	 Co-existence studies

Co-existence studies of DC_1A-7A_n8A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.118.4	 ∆TIB and ∆RIB values

Table 7.118.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A_n8A-n78A | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 7.118.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-7A_n8A-n78A | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 7.118.5	MSD

All MSD requirements for DC_1A-7A_n8A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.119	DC_1-20_n8-n78

### 7.119.1	 Operating bands for DC

Table 7.119.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-20A_n8A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.119.2 	Configuration for DC

Table 7.119.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-20A_n8A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.119.3	 Co-existence studies

Co-existence studies of DC_1A-20A_n8A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.119.4	 ∆TIB and ∆RIB values

Table 7.119.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-20A_n8A-n78A | 1 | 0.3 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 7.119.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-20A_n8A-n78A | 1 | 0.2 |
|  | 20 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 7.119.5	MSD

All MSD requirements for DC_1A-20A_n8A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.120	DC_3-20_n8-n78

### 7.120.1	 Operating bands for DC

Table 7.120.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-20A_n8A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.120.2 	Configuration for DC

Table 7.120.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-20A_n8A-n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.120.3	 Co-existence studies

Co-existence studies of DC_3A-20A_n8A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.120.4	 ∆TIB and ∆RIB values

Table 7.120.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-20A_n8A-n78A | 3 | 0.6 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 7.120.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-20A_n8A-n78A | 3 | 0.2 |
|  | 20 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 7.120.5	MSD

All MSD requirements for DC_3A-20A_n8A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.121	DC_7-20_n8-n78

### 7.121.1	 Operating bands for DC

Table 7.121.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-20A_n8A-n78A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.121.2 	Configuration for DC

Table 7.121.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A-20A_n8A-n78A |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.121.3	 Co-existence studies

Co-existence studies of DC_7A-20A_n8A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.121.4	 ∆TIB and ∆RIB values

Table 7.121.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-20A_n8A-n78A | 7 | 0.5 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 7.121.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-20A_n8A-n78A | 7 | 0 |
|  | 20 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 7.121.5	MSD

All MSD requirements for DC_7A-20A_n8A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.122	DC_7-8_n1-n40

7.122.1	Operating bands for DC

Table 7.122.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n1-n40 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

Table 7.122.1-2: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-8A_n1A-n40A | DC_7A_n1A DC_8A_n1A DC_7A_n40A DC_8A_n40A | CA_7A-8A | CA_n1A-n40A |

7.122.2	Channel bandwidths per operating band for DC

Table 7.122.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_7A-8A_n1A-n40A | DC_7A_n1A DC_8A_n1A DC_7A_n40A DC_8A_n40A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |

7.122.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back modes.

7.122.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.122.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8_n1-n40 | 7 | 0.8 |
|  | 8 | 0.6 |
|  | n1 | 0.6 |
|  | n40 | 0.9 |

Table 7.122.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-8_n1-n40 | 7 | 0.3 |
|  | 8 | 0.2 |
|  | n1 | 0 |
|  | n40 | 0.8 |

7.122.5	MSD

Based on co-existence studies no additional MSD is needed.

7.123	DC_1-3_n8-n78

### 7.123.1	 Operating bands for DC

Table 7.123.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A_n8A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.123.2 	Configuration for DC

Table 7.123.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A_n8A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.123.3	 Co-existence studies

Co-existence studies of DC_1A-3A_n8A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.123.4	 ∆TIB and ∆RIB values

Table 7.123.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A_n8A-n78A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 7.123.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A_n8A-n78A | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 7.123.5	MSD

All MSD requirements for DC_1A-3A_n8A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.124	DC_3-8_n1-n40

7.124.1	Operating bands for DC

Table 7.124.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8_n1-n40 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

Table 7.124.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-8A_n1A-n40A | DC_3A_n1A DC_8A_n1A DC_3A_n40A DC_8A_n40A | CA_3A-8A | CA_n1A-n40A |

7.124.2	Channel bandwidths per operating band for DC

Table 7.124.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_3A-8A_n1A-n40A | DC_3A_n1A DC_8A_n1A DC_3A_n40A DC_8A_n40A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |

7.124.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back modes.

7.124.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.124.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8_n1-n40 | 3 | 0.5 |
|  | 8 | 0.5 |
|  | n1 | 0.5 |
|  | n40 | 0.6 |

Table 7.124.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-8_n1-n40 | 3 | 0 |
|  | 8 | 0 |
|  | n1 | 0.1 |
|  | n40 | 0.2 |

7.124.5	MSD

Based on co-existence studies no additional MSD is needed.

7.125	DC_1-8_n3-n79

7.125.1	Operating bands for DC

Table 7.125.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n3-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.125.2	Channel bandwidths per operating band for DC

Table 7.125.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A_n3A-n79A | DC_1A_n3A DC_1A_n79A DC_8A_n3A DC_8A_n79A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n3 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | n79 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |

7.125.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8_n3, DC_1-8_n79, DC_1_n3-n79 and DC_8_n3-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.125.4	∆TIB and ∆RIB values

For DC_1-8_n3-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.125.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n3-n79 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | n3 | 0.3 |
|  | n79 | 0.8 |

Table 7.125.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n3-n79 | 1 | 0 |
|  | 8 | 0 |
|  | n3 | 0 |
|  | n79 | 0.5 |

7.125.5	MSD

As mentioned in 7.125.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.126	DC_1-8_n77-n79

7.126.1	Operating bands for DC

Table 7.126.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.126.2	Channel bandwidths per operating band for DC

Table 7.126.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_8A_n77A DC_8A_n79A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 230 |
|  |  |  |  | 8 |  | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | 15 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | n79 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |

7.126.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8_n77, DC_1-8_n79, DC_1_n77-n79 and DC_8_n77-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.126.4	∆TIB and ∆RIB values

For DC_1-8_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.126.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n77-n79 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 7.126.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n77-n79 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0 |

7.126.5	MSD

As mentioned in 7.126.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.127	DC_2-7_n66-n77

### 7.127.1	Operating bands for DC

Table 7.127.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-7_n66-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.127.2	Channel bandwidths per operating band for DC

Table 7.127.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-7A_n66A-n77A DC_2A-7C_n66A-n77A DC_2A-7A-7A_n66A-n77A | DC_2A_n66A DC_7A_n66A DC_2A_n77A DC_7A_n77A |

### 7.127.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.127.4	∆TIB and ∆RIB values

For DC_2-7_n66-n77, the TIB,c and RIB,c values are derived from DC_2-7_n66-n78 and are given in the tables below.

Table 7.127.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n66-n77 | 2 | 0.6 |
|  | 7 | 0.5 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 7.127.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7_n66-n77 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |
|  | n77 | 0.5 |

### 7.127.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.128	DC_7-66_n66-n77

### 7.128.1	Operating bands for DC

Table 7.128.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-66_n66-n77 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.128.2	Channel bandwidths per operating band for DC

Table 7.128.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_7A-66A_n66A-n77A DC_7C-66A_n66A-n77A DC_7A-7A-66A_n66A-n77A | DC_7A_n66A DC_7A_n77A DC_66A_n77A |

### 7.128.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.128.4	∆TIB and ∆RIB values

For DC_7-66_n66-n77, the TIB,c and RIB,c values are derived from DC_7-66_n66-n78 and are given in the tables below.

Table 7.128.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n66-n77 | 7 | 0.5 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 7.128.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-66_n66-n77 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
|  | n77 | 0.5 |

### 7.128.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.129	DC_2-5_n2-n41

7.129.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD2 produced by Band 5 and NR band n2 that impact the reference sensitivity of NR band n41.

The required MSD levels and test configuration are shown in the following Table.

Table 7.129.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR CA inter-bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-5A_n2A-n41A | B5 | IMD2 | ¦fB5 +fn2¦ | 829 | 5 | 25 | 874 | 5 | N/A |
|  | n2 |  |  | 1855 | 5 | 25 | 1935 | 5 |  |
|  | n41 |  |  | 2684 | 5 | 25 | 2684 | 5 | 29.7 |

7.130 	DC_2-5_n2-n77

7.130.1	Operating bands for DC

Table 7.130.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-5_n2-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.130.2	Configuration for DC

Table 7.130.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A_n2A-n77A | DC_2A_n77A DC_5A_n2A DC_5A_n77A | CA_2A-5A | CA_n2A-n77A |

7.130.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

7.130.4	∆TIB and ∆RIB values

For DC_2-5_n2-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.130.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n2-n77 | 2 | 0.6 |
|  | 5 | 0.6 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 7.130.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5_n2-n77 | 2 | 0.2 |
|  | 5 | 0.2 |
|  | n2 | 0.2 |
|  | n77 | 0.5 |

7.130.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.131 	DC_2-5_n5-n77

7.131.1	Operating bands for DC

Table 7.131.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-5_n5-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.131.2	Configuration for DC

Table 7.131.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_5A_n77A | CA_2A-5A | CA_n5A-n77A |

7.131.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

7.131.4	∆TIB and ∆RIB values

For DC_2-5_n5-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.131.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n5-n77 | 2 | 0.6 |
|  | 5 | 0.6 |
|  | n5 | 0.6 |
|  | n77 | 0.8 |

Table 7.131.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5_n5-n77 | 2 | 0.2 |
|  | 5 | 0.2 |
|  | n5 | 0.2 |
|  | n77 | 0.5 |

7.131.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.132 	DC_5-66_n66-n77

7.132.1	Operating bands for DC

Table 7.132.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5-66_n66-n77 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.132.2	Configuration for DC

Table 7.132.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-66A_n66A-n77A | DC_5A_n66A DC_5A_n77A DC_66A_n77A | CA_5A-66A | CA_n66A-n77A |

7.132.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

7.132.4	∆TIB and ∆RIB values

For DC_5-66_n66-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.132.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n66-n77 | 5 | 0.6 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 7.132.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-66_n66-n77 | 5 | 0.2 |
|  | 66 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |

7.132.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.133 	DC_2-5_n66-n77

7.133.1	Operating bands for DC

Table 7.133.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-5_n66-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.133.2	Configuration for DC

Table 7.133.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_5A_n66A DC_5A_n77A | CA_2A-5A | CA_n66A-n77A |

7.133.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

7.133.4	∆TIB and ∆RIB values

For DC_2-5_n66-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.133.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n66-n77 | 2 | 0.6 |
|  | 5 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 7.133.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5_n66-n77 | 2 | 0.3 |
|  | 5 | 0.2 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

7.133.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.134 	DC_13-66_n5-n77

7.134.1	Operating bands for DC

Table 7.134.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13-66_n5-n77 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.134.2	Configuration for DC

Table 7.134.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_13A-66A_n5A-n77A | DC_13A_n77A DC_66A_n5A DC_66A_n77A | CA_13A-66A | CA_ n5A-n77 |
| DC_13A-66A-66A_n5A-n77A | DC_13A_n77A DC_66A_n5A DC_66A_n77A | CA_13A-66A-66A | CA_ n5A-n77 |

7.134.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

7.134.4	∆TIB and ∆RIB values

For DC_13-66_n5-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.134.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n5-n77 DC_13-66-66_n5-n77 | 13 | 0.5 |
|  | 66 | 0.6 |
|  | n5 | 0.6 |
|  | n77 | 0.8 |

Table 7.134.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13-66_n5-n77 DC_13-66-66_n5-n77 | 13 | 0.2 |
|  | 66 | 0.2 |
|  | n5 | 0.2 |
|  | n77 | 0.5 |

7.134.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.135 	DC_2-13_n2-n77

7.135.1	Operating bands for DC

Table 7.135.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-13_n2-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.135.2	Configuration for DC

Table 7.135.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-13A_n2A-n77A | DC_2A_n77A DC_13A_n2A DC_13A_n77A | CA_2A-13A | CA_n2A-n77A |

7.135.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

7.135.4	∆TIB and ∆RIB values

For DC_2-13_n2-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.135.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n2-n77 | 2 | 0.6 |
|  | 13 | 0.3 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 7.135.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-13_n2-n77 | 2 | 0.2 |
|  | 13 | 0 |
|  | n2 | 0.2 |
|  | n77 | 0.5 |

7.135.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.136  DC_2-13_n5-n77

7.136.1	Operating bands for DC

Table 7.136.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-13_n5-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.136.2	Configuration for DC

Table 7.136.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-13A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A | CA_2A-13A | CA_n5A-n77A |
| DC_2A-2A-13A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A | CA_2A-2A-13A | CA_n5A-n77A |

7.136.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

7.136.4	∆TIB and ∆RIB values

For DC_2-13_n5-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.136.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n5-n77 DC_2-2-13_n5-n77 | 2 | 0.6 |
|  | 13 | 0.5 |
|  | n5 | 0.6 |
|  | n77 | 0.8 |

Table 7.136.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-13_n5-n77 DC_2-2-13_n5-n77 | 2 | 0.2 |
|  | 13 | 0.2 |
|  | n5 | 0.2 |
|  | n77 | 0.5 |

7.136.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.137  DC_2-66_n2-n77

7.137.1	Operating bands for DC

Table 7.137.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-66_n2-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.137.2	Configuration for DC

Table 7.137.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-66A_n2A-n77A | DC_2A_n77A DC_66A_n2A DC_66A_n77A | CA_2A-66A | CA_n2A-n77A |
| DC_2A-66A-66A_n2A-n77A | DC_2A_n77A DC_66A_n2A DC_66A_n77A | CA_2A-66A-66A | CA_n2A-n77A |

7.137.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

7.137.4	∆TIB and ∆RIB values

For DC_2-66_n2-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.137.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n2-n77 DC_2-66-66_n2-n77 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 7.137.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n2-n77 DC_2-66-66_n2-n77 | 2 | 0.2 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n77 | 0.5 |

7.137.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.138  DC_5-66_n2-n77

7.138.1	Operating bands for DC

Table 7.138.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5-66_n2-n77 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 66 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.138.2	Configuration for DC

Table 7.138.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-66A_n2A-n77A | DC_5A_n2A DC_5A_n77A DC_66A_n2A DC_66A_n77A | CA_5A-66A | CA_n2A-n77A |
| DC_5A-66A-66A_n2A-n77A | DC_5A_n2A DC_5A_n77A DC_66A_n2A DC_66A_n77A | CA_5A-66A-66A | CA_n2A-n77A |

7.138.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

7.138.4	∆TIB and ∆RIB values

For DC_5-66_n2-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.138.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n2-n77 DC_5-66-66_n2-n77 | 5 | 0.6 |
|  | 66 | 0.6 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 7.138.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-66_n2-n77 DC_5-66-66_n2-n77 | 5 | 0.2 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n77 | 0.5 |

7.138.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.139  DC_5-66_n5-n77

7.139.1	Operating bands for DC

Table 7.139.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5-66_n5-n77 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.139.2	Configuration for DC

Table 7.139.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-66A_n5A-n77A | DC_5A_n77A DC_66A_n5A DC_66A_n77A | CA_5A-66A | CA_n5A-n77A |
| DC_5A-66A-66A_n5A-n77A | DC_5A_n77A DC_66A_n5A DC_66A_n77A | CA_5A-66A-66A | CA_n5A-n77A |

7.139.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

7.139.4	∆TIB and ∆RIB values

For DC_5-66_n5-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.139.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n5-n77 DC_5-66-66_n5-n77 | 5 | 0.6 |
|  | 66 | 0.6 |
|  | n5 | 0.6 |
|  | n77 | 0.8 |

Table 7.139.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-66_n5-n77 DC_5-66-66_n5-n77 | 5 | 0.2 |
|  | 66 | 0.2 |
|  | n5 | 0.2 |
|  | n77 | 0.5 |

7.139.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.140  DC_3-8_n1-n28

7.140.1	 Operating bands for DC

Table 7.140.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8_n1-n28 | 3 | 1710 MHz | – | 1755 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

7.140.2 	Configuration for DC

Table 7.140.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-8A_n1A-n28A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n28 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |

7.140.3	 Co-existence studies

Co-existence studies are captured for lower order combinations.

7.140.4	 ∆TIB and ∆RIB values

Relaxation values of DC_1-3-8_n28 are re-used

Table 7.140.4-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8_n1-n28 | 3 | 0.3 |
|  | 8 | 0.6 |
|  | n1 | 0.3 |
|  | n28 | 0.6 |

Table 7.140.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-8_n1-n28 | 8 | 0.2 |
|  | n28 | 0.2 |

7.140.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8	Dual Connectivity band combinations of LTE 3 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

8.1	DC_1-3-8_n28-n77

8.1.1	Operating bands for DC

Table 8.1.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-8_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

8.1.2	Channel bandwidths per operating band for DC

Table 8.1.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |  | 30 MHz |  | 40 MHz |  | 50 MHz |  | 60 MHz |  | 70 MHz |  | 80 MHz |  | 90 MHz |  | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |  |
| DC_1A-3A-8A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
| DC_1A-3A-8A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 270 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.1.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-3-8_n28, DC_1-3-8_n77, DC_1-3_n28-n77, DC_1-8_n28-n77 and DC_3-8_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.1.4	∆TIB and ∆RIB values

For DC_1-3-8_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.1.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-8_n28-n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 8.1.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-8_n28-n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

8.1.5	MSD

As mentioned in 8.1.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.2	DC_3-7-8_n40-n78

8.2.1	Operating bands for DC

Table 8.2.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-8_n40-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.2.2	Configuration for DC

Table 8.2.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A-8A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A DC_8A_n40A DC_8A_n78A | CA_3A-7A-8A | CA_n40A-n78A |

8.2.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

8.2.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 8.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8_n40-n78 | 3 | 0.6 |
|  | 7 | 0.5 |
|  | 8 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 8.2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-8_n40-n78 | 3 | 0.2 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

8.2.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.3	DC_3-7-28_n1-n40

8.3.1	Operating bands for DC

Table 8.3.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-28_n1-n40 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

8.3.2	Configuration for DC

Table 8.3.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A-28A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_7A_n1A DC_7A_n40A DC_28A_n1A DC_28A_n40A | CA_3A-7A-28A | CA_n1A-n40A |

8.3.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

8.3.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 8.3.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-28_n1-n40 | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 28 | 0.6 |
|  | n1 | 0.6 |
|  | n40 | 0.9 |

Table 8.3.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-28_n1-n40 | 3 | 0 |
|  | 7 | 0.3 |
|  | 28 | 0.2 |
|  | n1 | 0 |
|  | n40 | 0.8 |

8.3.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.4	DC_1-3-18_n3-n77

8.4.1	Operating bands for DC

Table 8.4.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3-18_n3-n77 | 1, 3, 18 | n3, n77 |

8.4.2	Channel bandwidths per operating band for DC

Table 8.4.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A-18A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A1 DC_3A_n77A DC_18A_n3A DC_18A_n77A | 1 | See CA_1A-3A-18A BCS0 in Table 5.6A.1-2a of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.4.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered in the constituent fall-back mode.

8.4.4	∆TIB and ∆RIB values

For DC_1-3-18_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.4.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n3-n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 8.4.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n3-n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 18 | 0 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

8.4.5	MSD

There are no additional MSD requirements for this band combination.

8.5	DC_1-3-18_n3-n78

8.5.1	Operating bands for DC

Table 8.5.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3-18_n3-n78 | 1, 3, 18 | n3, n78 |

8.5.2	Channel bandwidths per operating band for DC

Table 8.5.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A-18A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A DC_3A_n78A DC_18A_n3A DC_18A_n78A | 1 | See CA_1A-3A-18A BCS0 in Table 5.6A.1-2a of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.5.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered in the constituent fall-back mode.

8.5.4	∆TIB and ∆RIB values

For DC_1-3-18_n3-n78, the TIB,c and RIB,c values are given in the tables below.

Table 8.5.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n3-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 8.5.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n3-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 18 | 0 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

8.5.5	MSD

There are no additional MSD requirements for this band combination.

8.6	DC_1-3-41_n3-n41

8.6.1	Operating bands for DC

Table 8.6.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3-41_n3-n41 | 1, 3, 41 | n3, n41 |

8.6.2	Channel bandwidths per operating band for DC

Table 8.6.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A-41A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A1 DC_3A_n41A DC_41A_n3A | 1 | See CA_1A-3A-41A BCS0 in Table 5.6A.1-2a of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n41A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.6.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered in the constituent fall-back mode.

8.6.4	∆TIB and ∆RIB values

For DC_1-3-41_n3-n41, the TIB,c and RIB,c values are given in the tables below.

Table 8.6.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n3-n41 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 8.6.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n3-n41 | 1 | 0 |
|  | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

8.6.5	MSD

There are no additional MSD requirements for this band combination.

8.7	DC_1-3-41_n3-n77

8.7.1	Operating bands for DC

Table 8.7.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3-41_n3-n77 | 1, 3, 41 | n3, n77 |

8.7.2	Channel bandwidths per operating band for DC

Table 8.7.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A-41A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A1 DC_3A_n77A DC_41A_n3A DC_41A_n77A | 1 | See CA_1A-3A-41A BCS0 in Table 5.6A.1-2a of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41C_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A1 DC_3A_n77A DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A | 1 | See CA_1A-3A-41C BCS0 in Table 5.6A.1-2a of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.7.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered in the constituent fall-back mode.

8.7.4	∆TIB and ∆RIB values

For DC_1-3-41_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.7.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n3-n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 8.7.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n3-n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 0 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

8.7.5	MSD

There are no additional MSD requirements for this band combination.

8.8	DC_1-3-41_n3-n78

8.8.1	Operating bands for DC

Table 8.8.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3-41_n3-n78 | 1, 3, 41 | n3, n78 |

8.8.2	Channel bandwidths per operating band for DC

Table 8.8.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A-41A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A1 DC_3A_n78A DC_41A_n3A DC_41A_n78A | 1 | See CA_1A-3A-41A BCS0 in Table 5.6A.1-2a of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41C_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A1 DC_3A_n78A DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A | 1 | See CA_1A-3A-41C BCS0 in Table 5.6A.1-2a of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.8.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered in the constituent fall-back mode.

8.8.4	∆TIB and ∆RIB values

For DC_1-3-41_n3-n78, the TIB,c and RIB,c values are given in the tables below.

Table 8.8.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n3-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 8.8.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n3-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 0 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

8.8.5	MSD

There are no additional MSD requirements for this band combination.

8.9	DC_1-3-41_n41-n77

8.9.1	Operating bands for DC

Table 8.9.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3-41_n41-n77 | 1, 3, 41 | n41, n77 |

8.9.2	Channel bandwidths per operating band for DC

Table 8.9.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A-41A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A DC_41A_n77A | 1 | See CA_1A-3A-41A BCS0 in Table 5.6A.1-2a of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.9.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered in the constituent fall-back mode.

8.9.4	∆TIB and ∆RIB values

For DC_1-3-41_n41-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.9.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n41-n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | n41 | 0.5 |
|  | n77 | 0.8 |

Table 8.9.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n41-n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 0 |
|  | n41 | 0 |
|  | n77 | 0.5 |

8.9.5	MSD

There are no additional MSD requirements for this band combination.

8.10	DC_1-3-41_n41-n78

8.10.1	Operating bands for DC

Table 8.10.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3-41_n41-n78 | 1, 3, 41 | n41, n78 |

8.10.2	Channel bandwidths per operating band for DC

Table 8.10.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A-41A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_41A_n78A | 1 | See CA_1A-3A-41A BCS0 in Table 5.6A.1-2a of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.10.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered in the constituent fall-back mode.

8.10.4	∆TIB and ∆RIB values

For DC_1-3-41_n41-n78, the TIB,c and RIB,c values are given in the tables below.

Table 8.10.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n41-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | n41 | 0.5 |
|  | n78 | 0.8 |

Table 8.10.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n41-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 0 |
|  | n41 | 0 |
|  | n78 | 0.5 |

8.10.5	MSD

There are no additional MSD requirements for this band combination.

8.11	DC_1-8-42_n28-n77

8.11.1	Operating bands for DC

Table 8.11.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8-42_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

8.11.2	Channel bandwidths per operating band for DC

Table 8.11.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A-42A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-8A-42A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 270 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A-42C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-8A-42C_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 290 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.11.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8-42_n28, DC_1-8-42_n77, DC_1-8_n28-n77, DC_1-42_n28-n77 and DC_8-42_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.11.4	∆TIB and ∆RIB values

For DC_1-8-42_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.11.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-42_n28-n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |
|  | n77 | 0.8 |

Table 8.11.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8-42_n28-n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |
|  | n77 | 0.5 |

8.11.5	MSD

As mentioned in 8.11.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.12	DC_1-3-18_n28-n77

8.12.1	Operating bands for DC

Table 8.12.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-18_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

8.12.2	Channel bandwidths per operating band for DC

Table 8.12.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-18A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_18A_n28A DC_18A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 185 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
| DC_1A-3A-18A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_18A_n28A DC_18A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 285 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.12.3	Co-existence studies

Co-existence studies of DC_1A-3A-18A_n28A-n77A and protected bands are already covered in the constituent fall-back modes.

8.12.4	∆TIB and ∆RIB values

For DC_1-3-18_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.12.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n28-n77 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 18 | 0.3 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 8.12.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18_n28-n77 | 1 | 0 |
|  | 3 | 0 |
|  | 18 | 0 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

8.12.5	MSD

As mentioned in 8.12.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.13	DC_1-3-18_n28-n78

8.13.1	Operating bands for DC

Table 8.13.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-18_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.13.2	Channel bandwidths per operating band for DC

Table 8.13.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-18A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n77A DC_18A_n28A DC_18A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 185 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
| DC_1A-3A-18A_n28A-n78(2A) | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_18A_n28A DC_18A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 285 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.13.3	Co-existence studies

Co-existence studies of DC_1A-3A-18A_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

8.13.4	∆TIB and ∆RIB values

For DC_1-3-18_n28-n78, the TIB,c and RIB,c values are given in the tables below.

Table 8.13.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n28-n78 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 18 | 0.3 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 8.13.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18_n28-n78 | 1 | 0 |
|  | 3 | 0 |
|  | 18 | 0 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

8.13.5	MSD

As mentioned in 8.13.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.14	DC_1-3-41_n28-n41

8.14.1	Operating bands for DC

Table 8.14.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-41_n28-n41 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

8.14.2	Channel bandwidths per operating band for DC

Table 8.14.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-41A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_3A_n28A DC_3A_n41A DC_41A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |

8.14.3	Co-existence studies

Co-existence studies of DC_1A-3A-41A_n28A-n41A and protected bands are already covered in the constituent fall-back modes.

8.14.4	∆TIB and ∆RIB values

For DC_1-3-41_n28-n41, the TIB,c and RIB,c values are given in the tables below.

Table 8.14.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n28-n41 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 41 | 0.31/0.82 |
|  | n28 | 0.6 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

Table 8.14.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-41_n28-n41 | 1 | 0 |
|  | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n28 | 0.2 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

8.14.5	MSD

As mentioned in 8.14.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.15	DC_3-19-42_n1-n77

8.15.1	Operating bands for DC

Table 8.15.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-19-42_n1-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

8.15.2	Configuration for DC

Table 8.15.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-19A-42A_n1A-n77A | DC_3A_n1A DC_3A_n77A DC_19A_n1A DC_19A_n77A | CA_3A-19A-42A | CA_n1A-n77A |
| DC_3A-19A-42C_n1A-n77A | DC_3A_n1A DC_3A_n77A DC_19A_n1A DC_19A_n77A | CA_3A-19A-42C | CA_n1A-n77A |

8.15.3	Co-existence studies

Co-existence studies of DC_3-19-42_n1-n77 are already covered by the fallback configurations, DC_3-19-42_n1, DC_3-19-42_n77, DC_3-19_n1-n77, DC_3-42_n1-n77 and DC_19-42_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.15.4	∆TIB and ∆RIB values

For DC_3-19-42_n1-n77, the TIB,c and RIB,c values are reused from DC_1-3-19-42_n77, and are given in the tables below.

Table 8.15.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19-42_n1-n77 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 8.15.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-19-42_n1-n77 | 3 | 0.2 |
|  | 19 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

8.15.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.16	DC_3-19-42_n1-n78

8.16.1	Operating bands for DC

Table 8.16.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-19-42_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.16.2	Configuration for DC

Table 8.16.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-19A-42A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_19A_n1A DC_19A_n78A | CA_3A-19A-42A | CA_n1A-n78A |
| DC_3A-19A-42C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_19A_n1A DC_19A_n78A | CA_3A-19A-42C | CA_n1A-n78A |

8.16.3	Co-existence studies

Co-existence studies of DC_3-19-42_n1-n78 are already covered by the fallback configurations, DC_3-19-42_n1, DC_3-19-42_n78, DC_3-19_n1-n78, DC_3-42_n1-n78 and DC_19-42_n1-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.16.4	∆TIB and ∆RIB values

For DC_3-19-42_n1-n78, the TIB,c and RIB,c values are reused from DC_1-3-19-42_n78, and are given in the tables below.

Table 8.16.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19-42_n1-n78 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 8.16.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-19-42_n1-n78 | 3 | 0.2 |
|  | 19 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

8.16.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.17	DC_3-19-42_n1-n79

8.17.1	Operating bands for DC

Table 8.17.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-19-42_n1-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

8.17.2	Configuration for DC

Table 8.17.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-19A-42A_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_19A_n1A DC_19A_n79A | CA_3A-19A-42A | CA_n1A-n79A |
| DC_3A-19A-42C_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_19A_n1A DC_19A_n79A | CA_3A-19A-42C | CA_n1A-n79A |

8.17.3	Co-existence studies

Co-existence studies of DC_3-19-42_n1-n79 are already covered by the fallback configurations, DC_3-19-42_n1, DC_3-19-42_n79, DC_3-19_n1-n79, DC_3-42_n1-n79 and DC_19-42_n1-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.17.4	∆TIB and ∆RIB values

For DC_3-19-42_n1-n79, the TIB,c and RIB,c values are reused from DC_1-3-19-42_n79, and are given in the tables below.

Table 8.17.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19-42_n1-n79 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n79 | 0 |

Table 8.17.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-19-42_n1-n79 | 3 | 0.2 |
|  | 19 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n79 | 0 |

8.17.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.18	DC_3-21-42_n1-n77

8.18.1	Operating bands for DC

Table 8.18.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-21-42_n1-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

8.18.2	Configuration for DC

Table 8.18.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-21A-42A_n1A-n77A | DC_3A_n1A DC_3A_n77A DC_21A_n1A DC_21A_n77A | CA_3A-21A-42A | CA_n1A-n77A |
| DC_3A-21A-42C_n1A-n77A | DC_3A_n1A DC_3A_n77A DC_21A_n1A DC_21A_n77A | CA_3A-21A-42C | CA_n1A-n77A |

8.18.3	Co-existence studies

Co-existence studies of DC_3-21-42_n1-n77 are already covered by the fallback configurations, DC_3-21-42_n1, DC_3-21-42_n77, DC_3-21_n1-n77, DC_3-42_n1-n77 and DC_21-42_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.18.4	∆TIB and ∆RIB values

For DC_3-21-42_n1-n77, the TIB,c and RIB,c values are reused from DC_1-3-21-42_n77, and are given in the tables below.

Table 8.18.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21-42_n1-n77 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n77 | 0.6 |

Table 8.18.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21-42_n1-n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n77 | 0.2 |

8.18.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.19	DC_3-21-42_n1-n78

8.19.1	Operating bands for DC

Table 8.19.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-21-42_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.19.2	Configuration for DC

Table 8.19.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-21A-42A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_21A_n1A DC_21A_n78A | CA_3A-21A-42A | CA_n1A-n78A |
| DC_3A-21A-42C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_21A_n1A DC_21A_n78A | CA_3A-21A-42C | CA_n1A-n78A |

8.19.3	Co-existence studies

Co-existence studies of DC_3-21-42_n1-n78 are already covered by the fallback configurations, DC_3-21-42_n1, DC_3-21-42_n78, DC_3-21_n1-n78, DC_3-42_n1-n78 and DC_21-42_n1-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.19.4	∆TIB and ∆RIB values

For DC_3-21-42_n1-n78, the TIB,c and RIB,c values are reused from DC_1-3-21-42_n78, and are given in the tables below.

Table 8.19.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21-42_n1-n78 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n78 | 0.6 |

Table 8.19.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21-42_n1-n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n78 | 0.2 |

8.19.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.20	DC_3-21-42_n1-n79

8.20.1	Operating bands for DC

Table 8.20.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-21-42_n1-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

8.20.2	Configuration for DC

Table 8.20.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-21A-42A_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_21A_n1A DC_21A_n79A | CA_3A-21A-42A | CA_n1A-n79A |
| DC_3A-21A-42C_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_21A_n1A DC_21A_n79A | CA_3A-21A-42C | CA_n1A-n79A |

8.20.3	Co-existence studies

Co-existence studies of DC_3-21-42_n1-n79 are already covered by the fallback configurations, DC_3-21-42_n1, DC_3-21-42_n79, DC_3-21_n1-n79, DC_3-42_n1-n79 and DC_21-42_n1-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.20.4	∆TIB and ∆RIB values

For DC_3-21-42_n1-n79, the TIB,c and RIB,c values are reused from DC_1-3-21-42_n79, and are given in the tables below.

Table 8.20.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21-42_n1-n79 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n79 | 0 |

Table 8.20.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21-42_n1-n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n79 | 0 |

8.20.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.21	DC_19-21-42_n1-n77

8.21.1	Operating bands for DC

Table 8.21.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21-42_n1-n77 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

8.21.2	Configuration for DC

Table 8.21.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A-21A-42A_n1A-n77A | DC_19A_n1A DC_19A_n77A DC_21A_n1A DC_21A_n77A | CA_19A-21A-42A | CA_n1A-n77A |
| DC_19A-21A-42C_n1A-n77A | DC_19A_n1A DC_19A_n77A DC_21A_n1A DC_21A_n77A | CA_19A-21A-42C | CA_n1A-n77A |

8.21.3	Co-existence studies

Co-existence studies of DC_19-21-42_n1-n77 are already covered by the fallback configurations, DC_19-21-42_n1, DC_19-21-42_n77, DC_19-21_n1-n77, DC_19-42_n1-n77 and DC_21-42_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.21.4	∆TIB and ∆RIB values

For DC_19-21-42_n1-n77, the TIB,c and RIB,c values are reused from DC_1-19-21-42_n77, and are given in the tables below.

Table 8.21.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21-42_n1-n77 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n77 | 0.8 |

Table 8.21.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21-42_n1-n77 | 19 | 0 |
|  | 21 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

8.21.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.22	DC_19-21-42_n1-n78

8.22.1	Operating bands for DC

Table 8.22.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21-42_n1-n78 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.22.2	Configuration for DC

Table 8.22.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A-21A-42A_n1A-n78A | DC_19A_n1A DC_19A_n78A DC_21A_n1A DC_21A_n78A | CA_19A-21A-42A | CA_n1A-n78A |
| DC_19A-21A-42C_n1A-n78A | DC_19A_n1A DC_19A_n78A DC_21A_n1A DC_21A_n78A | CA_19A-21A-42C | CA_n1A-n78A |

8.22.3	Co-existence studies

Co-existence studies of DC_19-21-42_n1-n78 are already covered by the fallback configurations, DC_19-21-42_n1, DC_19-21-42_n78, DC_19-21_n1-n78, DC_19-42_n1-n78 and DC_21-42_n1-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.22.4	∆TIB and ∆RIB values

For DC_19-21-42_n1-n78, the TIB,c and RIB,c values are reused from DC_1-19-21-42_n78, and are given in the tables below.

Table 8.22.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21-42_n1-n78 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |

Table 8.22.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21-42_n1-n78 | 19 | 0 |
|  | 21 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0 |
|  | n78 | 0.5 |

8.22.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.23	DC_19-21-42_n1-n79

8.23.1	Operating bands for DC

Table 8.23.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21-42_n1-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

8.23.2	Configuration for DC

Table 8.23.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A-21A-42A_n1A-n79A | DC_19A_n1A DC_19A_n79A DC_21A_n1A DC_21A_n79A | CA_19A-21A-42A | CA_n1A-n79A |
| DC_19A-21A-42C_n1A-n79A | DC_19A_n1A DC_19A_n79A DC_21A_n1A DC_21A_n79A | CA_19A-21A-42C | CA_n1A-n79A |

8.23.3	Co-existence studies

Co-existence studies of DC_19-21-42_n1-n79 are already covered by the fallback configurations, DC_19-21-42_n1, DC_19-21-42_n79, DC_19-21_n1-n79, DC_19-42_n1-n79 and DC_21-42_n1-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.23.4	∆TIB and ∆RIB values

For DC_19-21-42_n1-n79, the TIB,c and RIB,c values are reused from DC_1-19-21-42_n79, and are given in the tables below.

Table 8.23.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21-42_n1-n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n79 | 0 |

Table 8.23.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21-42_n1-n79 | 19 | 0 |
|  | 21 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0 |
|  | n79 | 0 |

8.23.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.24	DC_3-7-20_n1-n78

8.24.1	Operating bands for DC

Table 8.24.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-7A-20A_n1A-n78A DC_3C-7A-20A_n1A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.24.2	Channel bandwidths per operating band for DC

Table 8.24.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_20A_n1A DC_20A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3C-7A-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_3C_n1A DC_3C_n78A DC_7A_n1A DC_7A_n78A DC_20A_n1A DC_20A_n78A | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.24.3	Co-existence studies

Co-existence studies of DC_3-7-20_n1-n78 and protected bands are already covered in the constituent fall-back modes.

8.24.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_3-7-20_n1-n78 are given in the tables below reusing the values for DC_1-3-7-20_n78.

Table 8.24.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-20_n1-n78 | n1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.7 |
|  | 20 | 0.6 |
|  | n78 | 0.8 |

Table 8.24.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-7-20_n1-n78 | n1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n78 | 0.5 |

8.24.5	MSD

All MSD requirements for DC_3-7-20_n1-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.25	DC_3-7-28_n40-n78

8.25.1	Operating bands for DC

Table 8.25.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-28_n40-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.25.2	Channel bandwidths per operating band for DC

Table 8.25.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_3A-7A-28A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 240 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes9 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.25.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

8.25.4	∆TIB and ∆RIB values

For DC_3-7-28_n40-n78, the TIB,c and RIB,c values are given in the tables below.

Table 8.25.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-28_n40-n78 | 3 | 0.6 |
|  | 7 | 0.5 |
|  | 28 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 8.25.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-28_n40-n78 | 3 | 0.2 |
|  | 7 | 0 |
|  | 28 | 0.2 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

8.25.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.26	DC_1-3-7_n40-n78

8.26.1	Operating bands for DC

Table 8.26.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7_n40-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.26.2	Channel bandwidths per operating band for DC

Table 8.26.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_1A-3A-7_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 240 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes9 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.26.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

8.26.4	∆TIB and ∆RIB values

For DC_1-3-7_n40-n78, the TIB,c and RIB,c values are given in the tables below.

Table 8.26.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7_n40-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.8 |
|  | n40 | 0.9 |
|  | n78 | 0.8 |

Table 8.26.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7_n40-n78 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0.3 |
|  | n40 | 0.8 |
|  | n78 | 0.5 |

8.26.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.27	DC_1-7-28_n40-n78

8.27.1	Operating bands for DC

Table 8.27.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7-28_n40-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.27.2	Channel bandwidths per operating band for DC

Table 8.27.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_1A-7A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 240 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes9 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.27.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

8.27.4	∆TIB and ∆RIB values

For DC_1-7-28_n40-n78, the TIB,c and RIB,c values are given in the tables below.

Table 8.27.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-28_n40-n78 | 1 | 0.6 |
|  | 7 | 0.5 |
|  | 28 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 8.27.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-28_n40-n78 | 1 | 0.2 |
|  | 7 | 0 |
|  | 28 | 0.2 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

8.27.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.28	DC_3-7-8_n78-n258

8.28.1	Operating bands for DC

Table 8.28.1-1: DC band combination of LTE 3 bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-8_n78-n258 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz |  |

8.28.2	Channel bandwidths per operating band for DC

Table 8.28.2-1: Supported bandwidths per DC  LTE 3 bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_3A-7A-8A_n78A-n258A DC_3A-7A-8A_n78A-n258D DC_3A-7A-8A_n78A-n258E DC_3A-7A-8A_n78A-n258F DC_3A-7A-8A_n78A-n258G DC_3A-7A-8A_n78A-n258H DC_3A-7A-8A_n78A-n258I DC_3A-7A-8A_n78A-n258J DC_3A-7A-8A_n78A-n258K DC_3A-7A-8A_n78A-n258L DC_3A-7A-8A_n78A-n258M | DC_3A_n78A DC_3A_n258A DC_7A_n78A DC_7A_n258A DC_8A_n78A DC_8A_n258A | 3 | See CA_3A-7A-8A in TS 36.101 | 350 450 550 650 750 850 950 |
|  |  | 7 |  |  |
|  |  | 8 |  |  |
|  |  | n78 | CA_n78A-n258A/D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n258 |  |  |

8.28.3	Co-existence studies

The co-existence summary in section 5.3 of 37.717-11-21 already captures the needed studies. There is no additional harmonic and/or intermodulation impact for this combination.

8.28.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

8.28.5	MSD

No additional MSD requirement is needed.

8.29	DC_3-7-8_n40A-n258

8.29.1	Operating bands for DC

Table 8.29.1-1: DC band combination of LTE 3 bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-3-7-8_n40-n258 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz |  |

8.29.2	Channel bandwidths per operating band for DC

Table 8.29.2-1: Supported bandwidths per DC LTE 3 bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_3A-7A-8A_n40A-n258A DC_3A-7A-8A_n40A-n258D DC_3A-7A-8A_n40A-n258E DC_3A-7A-8A_n40A-n258F DC_3A-7A-8A_n40A-n258G DC_3A-7A-8A_n40A-n258H DC_3A-7A-8A_n40A-n258I DC_3A-7A-8A_n40A-n258J DC_3A-7A-8A_n40A-n258K DC_3A-7A-8A_n40A-n258L DC_3A-7A-8A_n40A-n258M | DC_3A_n40A DC_3A_n258A DC_7A_n40A DC_7A_n258A DC_8A_n40A DC_8A_n258A | 3 | See CA_3A-7A-8A in TS 36.101 | 330 430 530 630 730 830 930 |
|  |  | 7 |  |  |
|  |  | 8 |  |  |
|  |  | n40 | CA_n40A-n258A/D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n258 |  |  |

8.29.3	Co-existence studies

The co-existence summary in section 5.3 of 37.717-11-21 already captures the needed studies. There is no additional harmonic and/or intermodulation impact for this combination.

8.29.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

8.29.5	MSD

No additional MSD requirement is needed.

8.30	DC_1-8-11_n3-n28

8.30.1	Operating bands for DC

Table 8.30.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8-11_n3-n28 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

8.30.2	Channel bandwidths per operating band for DC

Table 8.30.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A-11A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A DC_11A_n3A DC_11A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 90 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.30.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8-11_n3, DC_1-8-11_n28, DC_1-8_n3-n28, DC_1-11_n3-n28 and DC_8-11_n3-n28. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.30.4	∆TIB and ∆RIB values

For DC_1-8-11_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 8.30.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n3-n28 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n28 | 0.6 |

Table 8.30.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8-11_n3-n28 | 1 | 0 |
|  | 8 | 0.2 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n28 | 0.2 |

8.30.5	MSD

As mentioned in 8.30.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.31	DC_1-8-11_n3-n77

8.31.1	Operating bands for DC

Table 8.31.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8-11_n3-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

8.31.2	Channel bandwidths per operating band for DC

Table 8.31.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A-11A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-8A-11A_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 270 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.31.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8-11_n3, DC_1-8-11_n77, DC_1-8_n3-n77, DC_1-11_n3-n77 and DC_8-11_n3-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.31.4	∆TIB and ∆RIB values

For DC_1-8-11_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.31.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n3-n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n77 | 0.8 |

Table 8.31.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8-11_n3-n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n77 | 0.5 |

8.31.5	MSD

As mentioned in 8.31.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.32	DC_1-3-11_n28-n77

8.32.1	Operating bands for DC

Table 8.32.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-11_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

8.32.2	Channel bandwidths per operating band for DC

Table 8.32.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-11A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-3A-11A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 270 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.32.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-3-11_n28, DC_1-3-11_n77, DC_1-3_n28-n77, DC_1-11_n28-n77 and DC_3-11_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.32.4	∆TIB and ∆RIB values

For DC_1-3-11_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.32.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-11_n28-n77 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 11 | 0.9 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 8.32.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-11_n28-n77 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 11 | 0.5 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

8.32.5	MSD

As mentioned in 8.32.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.33	DC_1-8-11_n28-n77

8.33.1	Operating bands for DC

Table 8.33.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8-11_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

8.33.2	Channel bandwidths per operating band for DC

Table 8.33.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A-11A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-8A-11A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.33.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8-11_n28, DC_1-8-11_n77, DC_1-8_n28-n77, DC_1-11_n28-n77 and DC_8-11_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.33.4	∆TIB and ∆RIB values

For DC_1-8-11_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.33.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n28-n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 11 | 0.4 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 8.33.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8-11_n28-n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | 11 | 0 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

8.33.5	MSD

As mentioned in 8.33.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.34	DC_1-8-42_n3-n28

8.34.1	Operating bands for DC

Table 8.34.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8-42_n3-n28 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

8.34.2	Channel bandwidths per operating band for DC

Table 8.34.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A-42A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 100 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A-42C_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 120 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.34.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8-42_n3, DC_1-8-42_n28, DC_1-8_n3-n28, DC_1-42_n3-n28 and DC_8-42_n3-n28. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.34.4	∆TIB and ∆RIB values

For DC_1-8-42_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 8.34.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-42_n3-n28 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n3 | 0.6 |
|  | n28 | 0.8 |

Table 8.34.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8-42_n3-n28 | 1 | 0 |
|  | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |
|  | n28 | 0.5 |

8.34.5	MSD

As mentioned in 8.34.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.35	DC_1-8-42_n3-n77

8.35.1	Operating bands for DC

Table 8.35.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8-42_n3-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

8.35.2	Channel bandwidths per operating band for DC

Table 8.35.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A-42A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-8A-42A_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 280 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A-42C_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-8A-42C_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 300 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.35.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8-42_n3, DC_1-8-42_n77, DC_1-8_n3-n77, DC_1-42_n3-n77 and DC_8-42_n3-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.35.4	∆TIB and ∆RIB values

For DC_1-8-42_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.35.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-42_n3-n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n3 | 0.8 |
|  | n77 | 0.8 |

Table 8.35.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8-42_n3-n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

8.35.5	MSD

As mentioned in 8.35.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.36	DC_3-8-11_n28-n77

8.36.1	Operating bands for DC

Table 8.36.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8-11_n28-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

8.36.2	Channel bandwidths per operating band for DC

Table 8.36.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_3A-8A-11A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_3A-8A-11A_n28A-n77(2A) | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.36.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_3-8-11_n28, DC_3-8-11_n77, DC_3-8_n28-n77, DC_3-11_n28-n77 and DC_8-11_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.36.4	∆TIB and ∆RIB values

For DC_3-8-11_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.36.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8-11_n28-n77 | 3 | 0.8 |
|  | 8 | 0.6 |
|  | 11 | 0.9 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 8.36.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-8-11_n28-n77 | 3 | 0.3 |
|  | 8 | 0.2 |
|  | 11 | 0.5 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

8.36.5	MSD

As mentioned in 8.36.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.37	DC_1-3-42_n28-n77

8.37.1	Operating bands for DC

Table 8.37.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-42_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

8.37.2	Channel bandwidths per operating band for DC

Table 8.37.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-42A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-3A-42A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 280 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-42C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-3A-42C_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 300 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.37.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-3-42_n28, DC_1-3-42_n77, DC_1-3_n28-n77, DC_1-42_n28-n77 and DC_3-42_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.37.4	∆TIB and ∆RIB values

For DC_1-3-42_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.37.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-42_n28-n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |
|  | n77 | 0.8 |

Table 8.37.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-42_n28-n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |
|  | n77 | 0.5 |

8.37.5	MSD

As mentioned in 8.37.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.38		DC_1-7-8_n28-n78

8.38.1	 Operating bands for DC

Table 8.38.1-1: LTE 3 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-7A-8A_n28A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.38.2 	Configuration for DC

Table 8.38.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-7A-8A_n28A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.38.3	 Co-existence studies

Co-existence studies of DC_1A-7A-8A_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

8.38.4	 ∆TIB and ∆RIB values

Table 8.38.4-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-8_n28-n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 8.38.4-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-8_n28-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

8.38.5	MSD

All MSD requirements for DC_1-7-8_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.39		DC_3-7-8_n28-n78

8.39.1	 Operating bands for DC

Table 8.39.1-1: LTE 3 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-7A-8A_n28A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.39.2 	Configuration for DC

Table 8.39.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A-8A_n28A-n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.39.3	 Co-existence studies

Co-existence studies of DC_3A-7A-8A_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

8.39.4	 ∆TIB and ∆RIB values

Table 8.39.4-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-8_n28-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 8.39.4-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-8_n28-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

8.39.5	MSD

All MSD requirements for DC_3-7-8_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.40	DC_3-7-28_n1-n78

8.40.1	Operating bands for DC

Table 8.40.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-28_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.40.2	Channel bandwidths per operating band for DC

Table 8.40.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_3A-7A-28A_n1A-n78A | DC_3A_n1A DC_7A_n1A DC_28A_n1A DC_3A_n78A DC_7A_n78A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.40.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

8.40.4	∆TIB and ∆RIB values

For DC_3-7-28_n1-n78, the TIB,c and RIB,c values are derived from DC_1-3-7-28_n78 and are given in the tables below.

Table 8.40.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-28_n1-n78 | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 28 | 0.6 |
|  | n1 | 0.7 |
|  | n78 | 0.6 |

Table 8.40.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-28_n1-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

8.40.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.41	DC_1-7-28_n3-n78

8.41.1	Operating bands for DC

Table 8.41.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7-28_n3-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.41.2	Channel bandwidths per operating band for DC

Table 8.41.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_1A-7A-28A_n3A-n78A | DC_1A_n3A DC_7A_n3A DC_28A_n3A DC_1A_n78A DC_7A_n78A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| DC_1A-7C-28A_n3A-n78A | DC_1A_n3A DC_7A_n3A DC_7C_n3A DC_28A_n3A DC_1A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
|  |  | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.41.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

8.41.4	∆TIB and ∆RIB values

For DC_1-7-28_n3-n78, the TIB,c and RIB,c values are derived from DC_1-3-7-28_n78 and are given in the tables below.

Table 8.41.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-28_n3-n78 | 1 | 0.7 |
|  | 7 | 0.7 |
|  | 28 | 0.6 |
|  | n3 | 0.7 |
|  | n78 | 0.8 |

Table 8.41.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-28_n3-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

8.41.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.42	DC_3-7-40_n1-n78

8.42.1	Operating bands for DC

Table 8.42.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-7A-40A_n1A-n78A DC_3A-7A-40C_n1A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.42.2	Channel bandwidths per operating band for DC

Table 8.42.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-7A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.42.3	Co-existence studies

Co-existence studies of DC_3-7-40_n1-n78 and protected bands are already covered in the constituent fall-back modes.

8.42.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_3-7-40_n1-n78 are given in the tables below reusing the values for DC_1-3-7-40_n78.

Table 8.42.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-40_n1-n78 | n1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.5 |
|  | 40 | 0.31 |
|  | n78 | 0.81 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 8.42.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-7-40_n1-n78 | n1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0 |
|  | 40 | 0.41 |
|  | n78 | 0.51 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

8.42.5	MSD

All MSD requirements for DC_3-7-40_n1-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.43	DC_3-8-40_n1-n78

8.43.1	Operating bands for DC

Table 8.43.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-8A-40A_n1A-n78A DC_3A-8A-40C_n1A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.43.2	Channel bandwidths per operating band for DC

Table 8.43.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-8A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-8A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.43.3	Co-existence studies

Co-existence studies of DC_3-8-40_n1-n78 and protected bands are already covered in the constituent fall-back modes.

8.43.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_3-8-40_n1-n78 are given in the tables below reusing the values for DC_1-3-8-40_n78.

Table 8.43.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8-40_n1-n78 | n1 | 0.6 |
|  | 3 | 0.6 |
|  | 8 | 0.6 |
|  | 40 | 0.31 |
|  | n78 | 0.81 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 8.43.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-8-40_n1-n78 | n1 | 0.2 |
|  | 3 | 0.2 |
|  | 8 | 0.2 |
|  | 40 | 0.41 |
|  | n78 | 0.51 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

8.43.5	MSD

All MSD requirements for DC_3-8-40_n1-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.44	DC_7-8-40_n1-n78

8.44.1	Operating bands for DC

Table 8.44.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-8A-40A_n1A-n78A DC_7A-8A-40C_n1A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.44.2	Channel bandwidths per operating band for DC

Table 8.44.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_7A-8A-40A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_7A-8A-40C_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.44.3	Co-existence studies

Co-existence studies of DC_7-8-40_n1-n78 and protected bands are already covered in the constituent fall-back modes.

8.44.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_7-8-40_n1-n78 are given in the tables below reusing the values for DC_1-7-8-40_n78.

Table 8.44.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8-40_n1-n78 | n1 | 0.6 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
|  | 40 | 0.31 |
|  | n78 | 0.81 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 8.44.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7-8-40_n1-n78 | n1 | 0.2 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 40 | 0.41 |
|  | n78 | 0.51 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

8.44.5	MSD

All MSD requirements for DC_7-8-40_n1-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.45	DC_1-3-18_n3-n41

8.45.1	Operating bands for DC

Table 8.45.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-18_n3-n41 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

8.45.2	Channel bandwidths per operating band for DC

Table 8.45.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-18A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A DC_3A_n41A DC_18A_n3A DC_18A_n41A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 175 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |

8.45.3	Co-existence studies

Co-existence studies of DC_1A-3A-18A_n3A-n41A and protected bands are already covered in the constituent fall-back modes.

8.45.4	∆TIB and ∆RIB values

For DC_1-3-18_n3-n41, the TIB,c and RIB,c values are given in the tables below.

Table 8.45.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n3-n41 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 18 | 0.3 |
|  | n3 | 0.5 |
|  | n41 | 0.31 |
|  |  | 0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2 Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 8.45.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18_n3-n41 | 1 | 0.0 |
|  | 3 | 0.5 |
|  | 18 | 0 |
|  | n3 | 0.5 |
|  | n41 | 01 |
|  |  | 0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

8.45.5	MSD

As mentioned in 8.45.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.46	DC_1-3-18_n28-n41

8.46.1	Operating bands for DC

Table 8.46.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-18_n28-n41 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

8.46.2	Channel bandwidths per operating band for DC

Table 8.46.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-18A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_3A_n28A DC_3A_n41A DC_18A_n28A DC_18A_n41A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 175 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |

8.46.3	Co-existence studies

Co-existence studies of DC_1A-3A-18A_n28A-n41A and protected bands are already covered in the constituent fall-back modes.

8.46.4	∆TIB and ∆RIB values

For DC_1-3-18_n28-n41, the TIB,c and RIB,c values are given in the tables below.

Table 8.46.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n28-n41 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 18 | 0.3 |
|  | n28 | 0.6 |
|  | n41 | 0.31 |
|  |  | 0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2 Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 8.46.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18_n28-n41 | 1 | 0.0 |
|  | 3 | 0.5 |
|  | 18 | 0 |
|  | n28 | 0.2 |
|  | n41 | 01 |
|  |  | 0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2 Applicable for the frequency range of 2496-2515 MHz. |  |  |

8.46.5	MSD

As mentioned in 8.46.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.47	DC_1-3-18_n41-n77

8.47.1	Operating bands for DC

Table 8.47.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-18_n41-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

8.47.2	Channel bandwidths per operating band for DC

Table 8.47.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-18A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 255 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
| DC_1A-3A-18A_n41A-n77(2A) | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 355 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.47.3	Co-existence studies

Co-existence studies of DC_1A-3A-18A_n41A-n77A and protected bands are already covered in the constituent fall-back modes.

8.47.4	∆TIB and ∆RIB values

For DC_1-3-18_n41-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.47.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n41-n77 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 18 | 0.3 |
|  | n41 | 0.31 |
|  |  | 0.82 |
|  | n77 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz NOTE 2 Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 8.47.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18_n41-n77 | 1 | 0.0 |
|  | 3 | 0.5 |
|  | 18 | 0 |
|  | n41 | 01 |
|  |  | 0.52 |
|  | n77 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz NOTE 2 Applicable for the frequency range of 2496-2515 MHz. |  |  |

8.47.5	MSD

As mentioned in 8.47.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.48	DC_1-3-18_n41-n78

8.48.1	Operating bands for DC

Table 8.48.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-18_n41-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.48.2	Channel bandwidths per operating band for DC

Table 8.48.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-18A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 255 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
| DC_1A-3A-18A_n41A-n78(2A) | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 355 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.48.3	Co-existence studies

Co-existence studies of DC_1A-3A-18A_n41A-n78A and protected bands are already covered in the constituent fall-back modes.

8.48.4	∆TIB and ∆RIB values

For DC_1-3-18_n41-n78, the TIB,c and RIB,c values are given in the tables below.

Table 8.48.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n41-n77 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 18 | 0.3 |
|  | n41 | 0.31 |
|  |  | 0.82 |
|  | n78 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz NOTE 2 Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 8.48.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18_n41-n77 | 1 | 0.0 |
|  | 3 | 0.5 |
|  | 18 | 0 |
|  | n41 | 01 |
|  |  | 0.52 |
|  | n78 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz NOTE 2 Applicable for the frequency range of 2496-2515 MHz. |  |  |

8.48.5	MSD

As mentioned in 8.48.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.49	DC_1-3-8_n28-n78

8.49.1	 Operating bands for DC

Table 8.49.1-1: LTE 3 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-8A_n28A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.49.2 	Configuration for DC

Table 8.49.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-8A_n28A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.49.3	 Co-existence studies

Co-existence studies of DC_1A-3A-8A_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

8.49.4	 ∆TIB and ∆RIB values

Table 8.49.4-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-8_n28-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 8.49.4-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-8_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

8.49.5	MSD

All MSD requirements for DC_1-3-8_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

  - 8.50	DC_1-3-20_n7-n78

    - 8.50.1	Operating bands for DC

Table 8.50.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-20_n7-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

    - 8.50.2	Channel bandwidths per operating band for DC

Table 8.50.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |  | 30 MHz |  | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-20A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_20A_n7A DC_1A_n78A DC_3A_n78A DC_20A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

    - 8.50.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-3-20_n7, DC_1-3-20_n78, DC_1-3_n7-n78, DC_3-20_n7-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

    - 8.50.4	∆TIB and ∆RIB values

For DC_1-3-8_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.50.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-20_n7-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.3 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |

Table 8.50.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-20_n7-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 20 | 0 |
|  | n7 | 0 |
|  | n78 | 0.5 |

    - 8.50.5	MSD

As mentioned in 8.50.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.51	DC_2-7-13_n25-n66

8.51.1	Operating bands for DC

Table 8.51.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-7-13_n25-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

8.51.2	Channel bandwidths per operating band for DC

Table 8.51.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A-7A-13A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 130 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| DC_2A-7A-7A-13A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 7 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| DC_2A-7C-13A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| NOTE 5: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band contiguous or non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply for the Band 42 and Band n77/n78 combination when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.51.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

8.51.4	∆TIB and ∆RIB values

For DC_2-7-13_n25-n66, the TIB,c and RIB,c values are derived from DC_2-7-13_n66 and are given in the tables below.

Table 8.51.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-13_n25-n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 13 | 0.3 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 8.51.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-13_n25-n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 13 | 0 |
|  | n25 | 0.3 |
|  | n66 | 0.5 |

8.51.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.52	DC_2-7-66_n25-n66

8.52.1	Operating bands for DC

Table 8.52.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-7-66_n25-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

8.52.2	Channel bandwidths per operating band for DC

Table 8.52.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A-7A-66A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_66A_n25A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| DC_2A-7A-7A-66A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_66A_n25A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 7 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| DC_2A-7C-66A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_66A_n25A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| NOTE 5: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band contiguous or non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply for the Band 42 and Band n77/n78 combination when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.52.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

8.52.4	∆TIB and ∆RIB values

For DC_2-7-66_n25-n66, the TIB,c and RIB,c values are derived from DC_2-7_n66 and are given in the tables below.

Table 8.52.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-66_n25-n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 8.52.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-66_n25-n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n25 | 0.3 |
|  | n66 | 0.5 |

8.52.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.53	DC_1-3-7_n3-n78

8.53.1	Operating bands for DC

Table 8.53.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7_n3-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.53.2	Channel bandwidths per operating band for DC

Table 8.53.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_1A-3A-7A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3A-7C_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_7C_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.53.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

8.53.4	∆TIB and ∆RIB values

For DC_1-3-7_n3-n78, the TIB,c and RIB,c values are derived from DC_1-3-7_n78 and are given in the tables below.

Table 8.53.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7_n3-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n3 | 0.7 |
|  | n78 | 0.8 |

Table 8.53.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7_n3-n78 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 7 | 0.3 |
|  | n3 | 0.3 |
|  | n78 | 0.5 |

8.53.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.54	DC_1-3-28_n3-n78

8.54.1	Operating bands for DC

Table 8.54.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-28_n3-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.54.2	Channel bandwidths per operating band for DC

Table 8.54.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_1A-3A-28A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_28A_n3A DC_1A_n78A DC_3A_n78A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.54.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

8.54.4	∆TIB and ∆RIB values

For DC_1-3-28_n3-n78, the TIB,c and RIB,c values are derived from DC_1-3-28_n78 and are given in the tables below.

Table 8.54.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-28_n3-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 8.54.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-28_n3-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

8.54.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.55	DC_3-7-28_n3-n78

8.55.1	Operating bands for DC

Table 8.55.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-28_n3-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.55.2	Channel bandwidths per operating band for DC

Table 8.55.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_3A-7A-28A_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_28A_n3A DC_3A_n78A DC_7A_n78A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-7C-28A_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_28A_n3A DC_3A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
|  |  | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.55.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

8.55.4	∆TIB and ∆RIB values

For DC_3-7-28_n3-n78, the TIB,c and RIB,c values are derived from DC_3-7-28_n78 and are given in the tables below.

Table 8.55.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-28_n3-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 8.55.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-28_n3-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

8.55.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.56	DC_1-3-7_n8-n78

### 8.56.1	 Operating bands for DC

Table 8.56.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-7A_n8A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.56.2 	Configuration for DC

Table 8.56.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-7A_n8A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 8.56.3	 Co-existence studies

Co-existence studies of DC_1A-3A-7A_n8A-n78A and protected bands are already covered in the constituent fall-back modes.

### 8.56.4	 ∆TIB and ∆RIB values

Table 8.56.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-7A_n8A-n78A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 8.56.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-7A_n8A-n78A | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 8.56.5	MSD

All MSD requirements for DC_1A-3A-7A_n8A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.57	DC_1-3-20_n8-n78

### 8.57.1	 Operating bands for DC

Table 8.57.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-20A_n8A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.57.2 	Configuration for DC

Table 8.57.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-20A_n8A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 8.57.3	 Co-existence studies

Co-existence studies of DC_1A-3A-20A_n8A-n78A and protected bands are already covered in the constituent fall-back modes.

### 8.57.4	 ∆TIB and ∆RIB values

Table 8.57.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-20A_n8A-n78A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 8.57.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-20A_n8A-n78A | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 20 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 8.57.5	MSD

All MSD requirements for DC_1A-3A-20A_n8A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.58	DC_1-7-20_n8-n78

### 8.58.1	 Operating bands for DC

Table 8.58.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-7A-20A_n8A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.58.2 	Configuration for DC

Table 8.58.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-7A-20A_n8A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 8.58.3	 Co-existence studies

Co-existence studies of DC_1A-7A-20A_n8A-n78A and protected bands are already covered in the constituent fall-back modes.

### 8.58.4	 ∆TIB and ∆RIB values

Table 8.58.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A-20A_n8A-n78A | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 8.58.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-7A-20A_n8A-n78A | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 8.58.5	MSD

All MSD requirements for DC_1A-7A-20A_n8A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.59	DC_3-7-20_n8-n78

### 8.59.1	 Operating bands for DC

Table 8.59.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-7A-20A_n8A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.59.2 	Configuration for DC

Table 8.59.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A-20A_n8A-n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 8.59.3	 Co-existence studies

Co-existence studies of DC_3A-7A-20A_n8A-n78A and protected bands are already covered in the constituent fall-back modes.

### 8.59.4	 ∆TIB and ∆RIB values

Table 8.59.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-7A-20A_n8A-n78A | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 8.59.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-7A-20A_n8A-n78A | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 8.59.5	MSD

All MSD requirements for DC_3A-7A-20A_n8A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.60	DC_3-7-8_n1-n40

8.60.1	Operating bands for DC

Table 8.60.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-8_n1-n40 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

Table 8.60.1-2: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A-8A_n1A-n40A | DC_3A_n1A DC_7A_n1A DC_8A_n1A DC_3A_n40A DC_7A_n40A DC_8A_n40A | CA_3A-7A-8A | CA_n1A-n40A |

8.60.2	Channel bandwidths per operating band for DC

Table 8.60.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_3A-7A-8A_n1A-n40A | DC_3A_n1A DC_7A_n1A DC_8A_n1A DC_3A_n40A DC_7A_n40A DC_8A_n40A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |

8.60.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered in the constituent fall-back modes.

8.60.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 8.60.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-8_n1-n40 | 3 | 0.5 |
|  | 7 | 0.8 |
|  | 8 | 0.6 |
|  | n1 | 0.6 |
|  | n40 | 0.9 |

Table 8.60.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-8_n1-n40 | 3 | 0 |
|  | 7 | 0.3 |
|  | 8 | 0.2 |
|  | n1 | 0.1 |
|  | n40 | 0.8 |

8.60.5	MSD

Based on co-existence studies no additional MSD is needed.

8.61	DC_2-13-66_n5-n77

8.61.1	Operating bands for DC

Table 8.61.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-13-66_n5-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

8.61.2	Configuration for DC

Table 8.61.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-13A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A DC_66A_n5A DC_66A_n77A | CA_2A-13A-66A | CA_n5A-n77A |
| DC_2A-2A-13A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A DC_66A_n5A DC_66A_n77A | CA_2A-2A-13A-66A | CA_n5A-n77A |
| DC_2A-13A-66A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A DC_66A_n5A DC_66A_n77A | CA_2A-13A-66-66A | CA_n5A-n77A |

8.61.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

8.61.4	∆TIB and ∆RIB values

For DC_2-13-66_n5-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.61.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13-66_n5-n77 DC_2-2-13-66_n5-n77 DC_2-13-66-66_n5-n77 | 2 | 0.6 |
|  | 13 | 0.5 |
|  | 66 | 0.6 |
|  | n5 | 0.6 |
|  | n77 | 0.8 |

Table 8.61.4-2: ΔRIB

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-13-66_n5-n77 DC_2-2-13-66_n5-n77 DC_2-13-66-66_n5-n77 | 2 | 0.3 |
|  | 13 | 0.2 |
|  | 66 | 0.2 |
|  | n5 | 0.2 |
|  | n77 | 0.5 |

8.61.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.62	DC_2-13-66_n66-n77

8.62.1	Operating bands for DC

Table 8.62.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-13-66_n66-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

8.62.2	Configuration for DC

Table 8.62.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-13A-66A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_13A_n66A DC_13A_n77A DC_66A_n77A | CA_2A-13A-66A | CA_n66A-n77A |
| DC_2A-2A-13A-66A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_13A_n66A DC_13A_n77A DC_66A_n77A | CA_2A-2A-13A-66A | CA_n66A-n77A |

8.62.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

8.62.4	∆TIB and ∆RIB values

For DC_2-13-66_n66-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.62.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13-66_n66-n77 DC_2-2-13-66_n66-n77 | 2 | 0.6 |
|  | 13 | 0.3 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 8.62.4-2: ΔRIB

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-13-66_n66-n77 DC_2-2-13-66_n66-n77 | 2 | 0.3 |
|  | 13 | 0 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

8.62.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.63	DC_2-13-66_n2-n77

8.63.1	Operating bands for DC

Table 8.63.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-13-66_n2-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

8.63.2	Configuration for DC

Table 8.63.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-13A-66A_n2A-n77A | DC_2A_n77A DC_13A_n2A DC_13A_n77A DC_66A_n2A DC_66A_n77A | CA_2A-13A-66A | CA_n2A-n77A |
| DC_2A-13A-66A-66A_n2A-n77A | DC_2A_n77A DC_13A_n2A DC_13A_n77A DC_66A_n2A DC_66A_n77A | CA_2A-13A-66A-66A | CA_n2A-n77A |

8.63.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

8.63.4	∆TIB and ∆RIB values

For DC_2-13-66_n2-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.63.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13-66_n2-n77, DC_2-13-66-66_n2-n77 | 2 | 0.6 |
|  | 13 | 0.3 |
|  | 66 | 0.6 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 8.63.4-2: ΔRIB

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-13-66_n2-n77 DC_2-13-66-66_n2-n77 | 2 | 0.2 |
|  | 13 | 0 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n77 | 0.5 |

8.63.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.64	DC_2-5-66_n5-n77

8.64.1	Operating bands for DC

Table 8.64.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-5-66_n5-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

8.64.2	Configuration for DC

Table 8.64.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_5A_n77A DC_66A_n5A DC_66A_n77A | CA_2A-5A-66A | CA_n5A-n77A |
| DC_2A-5A-66A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_5A_n77A DC_66A_n5A DC_66A_n77A | CA_2A-5A-66A-66A | CA_n5A-n77A |

8.64.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

8.64.4	∆TIB and ∆RIB values

For DC_2-5-66_n5-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.64.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-66_n5-n77 DC_2-5-66-66_n5-n77 | 2 | 0.6 |
|  | 5 | 0.6 |
|  | 66 | 0.6 |
|  | n5 | 0.6 |
|  | n77 | 0.8 |

Table 8.64.4-2: ΔRIB

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-66_n5-n77 DC_2-5-66-66_n5-n77 | 2 | 0.3 |
|  | 5 | 0.2 |
|  | 66 | 0.2 |
|  | n5 | 0.2 |
|  | n77 | 0.5 |

8.64.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.65	DC_2-5-66_n2-n77

8.65.1	Operating bands for DC

Table 8.65.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-5-66A_n2-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

8.65.2	Configuration for DC

Table 8.65.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A-66A_n2A-n77A | DC_2A_n77A DC_5A_n2A DC_5A_n77A DC_66A_n2A DC_66A_n77A | CA_2A-5A-66A | CA_n2A-n77A |
| DC_2A-5A-66A-66A_n2A-n77A | DC_2A_n77A DC_5A_n2A DC_5A_n77A DC_66A_n2A DC_66A_n77A | CA_2A-5A-66A-66A | CA_n2A-n77A |

8.65.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

8.65.4	∆TIB and ∆RIB values

For DC_2-5-66_n2-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.65.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-66_n2-n77 DC_2-5-66-66_n2-n77 | 2 | 0.6 |
|  | 5 | 0.6 |
|  | 66 | 0.6 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 8.65.4-2: ΔRIB

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-66_n2-n77 DC_2-5-66-66_n2-n77 | 2 | 0.2 |
|  | 5 | 0.2 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n77 | 0.5 |

8.65.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

9	Dual Connectivity band combinations of LTE 4 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

9.1	DC_1-3-7-28_n40-n78

9.1.1	Operating bands for DC

Table 9.1.1-1: DC band combination of LTE 4DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7-28_n40-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

9.1.2	Channel bandwidths per operating band for DC

Table 9.1.2-1: Supported bandwidths per DC LTE 4DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_1A-3A-7A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 260 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes9 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

9.1.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

9.1.4	∆TIB and ∆RIB values

For DC_1-3-7-28_n40-n78, the TIB,c and RIB,c values are given in the tables below.

Table 9.1.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7-28_n40-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 28 | 0.3 |
|  | n40 | 0.9 |
|  | n78 | 0.8 |

Table 9.1.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7-28_n40-n78 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0.3 |
|  | 28 | 0.2 |
|  | n40 | 0.8 |
|  | n78 | 0.5 |

9.1.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

9.2	DC_1-3-8-11_n28-n77

9.2.1	Operating bands for DC

Table 9.2.1-1: DC band combination of LTE 4DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-8-11_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

9.2.2	Channel bandwidths per operating band for DC

Table 9.2.2-1: Supported bandwidths per DC LTE 4DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-8A-11A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-3A-8A-11A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 280 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

9.2.3	Co-existence studies

Co-existence studies of this DC LTE inter-band DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-3-8-11_n28, DC_1-3-8-11_n77, DC_1-3-8_n28-n77, DC_1-3-11_n28-n77, DC_1-8-11_n28-n77 and DC_3-8-11_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

9.2.4	∆TIB and ∆RIB values

For DC_1-3-8-11_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 9.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-8-11_n28-n77 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 8 | 0.6 |
|  | 11 | 0.9 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 9.2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-8-11_n28-n77 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 8 | 0.2 |
|  | 11 | 0.5 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

9.2.5	MSD

As mentioned in 9.2.3, there is no need to specify additional MSD requirement for this UL DC configuration.

9.3	DC_1-3-7-8_n28-n78

9.3.1	 Operating bands for DC

Table 9.3.1-1: LTE 4 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-7A-8A_n28A-n78A | 1 | 1920 MHz | – | 1980 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

9.3.2 	Configuration for DC

Table 9.3.2-1: Supported bandwidths per DC band combination of LTE 4DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-7A-8A_n28A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

9.3.3	 Co-existence studies

Co-existence studies of DC_1A-3A-7A-8A_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

9.3.4	 ∆TIB and ∆RIB values

Table 9.3.4-1: ΔTIB,c due to EN-DC(six bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7-8_n28-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 9.3.4-2: ΔRIB,c due to EN-DC (six bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7-8_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

9.3.5	MSD

All MSD requirements for DC_1-3-7-8_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

9.4	DC_3-7-8-40_n1-n78

9.4.1	Operating bands for DC

Table 9.4.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-7A-8A-40A_n1A-n78A DC_3A-7A-8A-40C_n1A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

9.4.2	Channel bandwidths per operating band for DC

Table 9.4.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A-8A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-7A-8A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 240 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

9.4.3	Co-existence studies

Co-existence studies of DC_3-7-8-40_n1-n78 and protected bands are already covered in the constituent fall-back modes.

9.4.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_3-7-8-40_n1-n78 are given in the tables below reusing the values for DC_1-3-7-8-40_n78.

Table 9.4.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-8-40_n1-n78 | n1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
|  | 40 | 0.31 |
|  | n78 | 0.81 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 9.4.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-7-8-40_n1-n78 | n1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 40 | 0.41 |
|  | n78 | 0.51 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

9.4.5	MSD

All MSD requirements for DC_3-7-8-40_n1-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

9.5	DC_1-3-7-28_n78-n257

9.5.1	Operating bands for DC

Table 9.5.1-1: DC band combination of LTE 4DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7-28_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

9.5.2	Channel bandwidths per operating band for DC

Table 9.5.2-1: Supported bandwidths per DC LTE 4 bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_1A-3A-7A-28A_n78A-n257A DC_1A-3A-7A-28A_n78A-n257G DC_1A-3A-7A-28A_n78A-n257H DC_1A-3A-7A-28A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I | 1 | See CA_1A-3A-7A-28A in TS 36.101 | 580 380 480 580 |
|  |  | 3 |  |  |
|  |  | 7 |  |  |
|  |  | 28 |  |  |
|  |  | n78 | CA_n78A-n257A/G/H/I in TS 38.101-3 |  |
|  |  | n257 |  |  |

9.5.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

9.5.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

9.5.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

9.6	DC_1-3-7-28_n3-n78

9.6.1	Operating bands for DC

Table 9.6.1-1: DC band combination of LTE 4DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7-28_n3-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

9.6.2	Channel bandwidths per operating band for DC

Table 9.6.2-1: Supported bandwidths per DC LTE 4DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_1A-3A-7A-28A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_28A_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3A-7C-28A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3ADC_7C_n3A DC_28A_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 240 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

9.6.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

9.6.4	∆TIB and ∆RIB values

For DC_1-3-7-28_n3-n78, the TIB,c and RIB,c values are derived from DC_1-3-7-28_n78 and are given in the tables below.

Table 9.6.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7-28_n3-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 28 | 0.6 |
|  | n3 | 0.7 |
|  | n78 | 0.8 |

Table 9.6.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7-28_n3-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

9.6.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

9.7		DC_1-3-7-20_n8-n78

### 9.7.1	 Operating bands for DC

Table 9.7.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-7A-20A_n8A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 9.7.2 	Configuration for DC

Table 9.7.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-7A-20A_n8A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 9.7.3	 Co-existence studies

Co-existence studies of DC_1A-3A-7A-20A_n8A-n78A and protected bands are already covered in the constituent fall-back modes.

### 9.7.4	 ∆TIB and ∆RIB values

Table 9.7.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-7A-20A_n8A-n78A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 9.7.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-7A-20A_n8A-n78A | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 9.7.5	MSD

All MSD requirements for DC_1A-3A-7A-20A_n8A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

  - 9.8		DC_1-3-7-20_n38-n78

### 9.8.1 	Operating bands for DC

Table 9.8.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7-20_n38-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2500 MHz | – | 2570 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 9.8.2 	Configuration for DC

Table 9.8.2-1: Inter-band EN-DC configurations (six bands)

| EN-DC configuration | Uplink EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-7A-20A_n38A-n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A | CA_1A-3A-20A | n78A |

### 9.8.3	 Co-existence studies

Co-existence studies of  DC_1A-3A-7A-20A_n38A-n78A with the UL ENDC configurations of DC_1A_n78A

, DC_3A_n78A and DC_20A_n78A are already covered in the constituent fall-back modes.

### 9.8.4	 ∆TIB and ∆RIB values

Table 9.8.4-1: ΔTIB,c due to EN-DC(six bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7-20_n38-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.6 |
|  | n78 | 0.8 |

Table 9.8.4-2: ΔRIB,c due to EN-DC (six bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7-20_n38-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n38 | 0.2 |
|  | n78 | 0.5 |

### 9.8.5	MSD

All MSD requirements for DC_1A-3A-7A-20A_n38A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Subject/Comment | Old | New |
| 2020-08 | RAN4 #96-e | R4-2010130 |  |  | TR37.717-11-21 Skeleton for DC band combinations of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL | N/A | 0.0.1 |
| 2020-09 | RAN4 #96-e | R4-2011885 |  |  | TR 37.717-11-21 v0.1.0 for LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17 | 0.0.1 | 0.1.0 |
| 2020-11 | RAN4 #97-e | R4-2014304 |  |  | TR 37.717-11-21 v0.2.0 for LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17 - Included all approved TPs in section 10.7 in RAN4 97e meeting report | 0.1.0 | 0.2.0 |
| 2021-02 | RAN4 #98-e | R4-2100272 |  |  | TR 37.717-11-21 v0.3.0 for LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17 - Included all approved TPs in section 9.7 in RAN4 98e meeting report | 0.2.0 | 0.3.0 |
| 2021-04 | RAN4 #98-e-bis | R4-2104965 |  |  | TR 37.717-11-21 v0.4.0 for LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17 - Included all approved TPs in section 7.7 in RAN4 98bis-e meeting report | 0.3.0 | 0.4.0 |
| 2021-05 | RAN4 #99-e | R4-2109841 |  |  | TR 37.717-11-21 v0.5.0 for LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17 - Included all approved TPs in section 8.21 in RAN4 99-e meeting report | 0.4.0 | 0.5.0 |
| 2021-08 | RAN4 #100-e | R4-2112735 |  |  | TR 37.717-11-21 v0.6.0 for LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17 - Included all approved TPs in section 8.19 in RAN4 100-e meeting report | 0.5.0 | 0.6.0 |
| 2021-11 | RAN4 #101-e | R4-2118065 |  |  | TR 37.717-11-21 v0.7.0 for LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17 - Included all approved TPs in section 7.18 in RAN4 101-e meeting report | 0.6.0 | 0.7.0 |