3GPP TSG-RAN WG4 #101-e	R4-2118337

Electronic meeting, 1st – 12th Nov, 2021

Source: 	Ericsson

Title:  	L1-SINR measurements requirements enhancement for NR HST in FR1

Agenda Item:	8.8.2.3

Document for:	Discussion

# Introduction

According to work plan [1] on enhancement for NR high speed train scenario in FR1, work continued during the RAN4#100e meeting, with outcome in terms of agreements and open issues captured in WF [2].

This contribution focus on L1-SINR and provide simulation results related.

# Discussion

Issue 3-1: whether L1-SINR measurement is applicable to HST - Option 1 (CMCC, vivo): Yes. The configuration of CMR/IMR is up to network configuration, there is no restriction on L1-SINR for HST scenario - Option 2 (MTK, OPPO, Ericsson, HW, Apple, QC, Nokia, Intel): RAN4 needs to confirm whether L1-SINR measurement is applicable for HST or not

It’s expected that a credible example can establish L1-SINR necessity in the HST scenario, if there is, as we recommended at the prior meeting.

Proposal 1: We're not leaning towards that the L1-SINR configuration should be avoided in HST scenario from a specification standpoint, but an invalid case doesn't need to be enhanced in particular, even simulation results show necessity of upper boundary.

Issue 3-2: If L1-SINR measurement is applicable to HST, whether it is necessary to specify upper bound of side condition for L1-SINR measurement - Companies are encouraged to evaluate whether an applicable upper bound for L1-SINR is needed and what is the concrete value of the upper bound, if needed.

Simulation assumptions are listed in the following.

Table 1 Simulation assumption (CMR-only L1-SINR)

Based on above assumptions, results are categorized with 2 options on parameter in Frequency domain filtering:

- Number of samples: 1; Frequency domain filtering over 8 PRBs

|  | FR1_15kHz_AWGN 2rx | FR1_15kHz_HST_single_972 2rx | FR1_15kHz_HST_single_1944 2rx | FR1_30kHz_AWGN 2rx | FR1_30kHz_HST_single_1667 2rx | FR1_30kHz_HST_single_3334 2rx |
| --- | --- | --- | --- | --- | --- | --- |
| 5% | -0.38 | -0.47 | -0.65 | -0.41 | -0.42 | -0.57 |
| 95% | 1.39 | 1.39 | 1.25 | 1.4 | 1.32 | 1.29 |
| MAX(5%,95%) | 1.39 | 1.39 | 1.25 | 1.4 | 1.32 | 1.29 |

- Number of samples: 1; Frequency domain filtering over 4 PRBs

|  | FR1_15kHz_AWGN 2rx | FR1_15kHz_HST_single_972 2rx | FR1_15kHz_HST_single_1944 2rx | FR1_30kHz_AWGN 2rx | FR1_30kHz_HST_single_1667 2rx | FR1_30kHz_HST_single_3334 2rx |
| --- | --- | --- | --- | --- | --- | --- |
| 5% | -0.81 | 0.17 | -0.04 | -0.81 | 0.15 | 0.06 |
| 95% | 1.06 | 1.9 | 1.78 | 1.05 | 1.91 | 1.83 |
| MAX(5%,95%) | 1.06 | 1.9 | 1.78 | 1.05 | 1.91 | 1.83 |

Proposal 2: Reuse Rel-16 L1-SINR measurement requirements, including the measurement accuracy and sample number for HST; 1.5x relaxation factor is removed when TCSI-RS <= 40ms, and 1.5x relaxation factor is kept when TCSI-RS > 40ms, if it’s agreed to add note for HST.

Proposal 3: The upper bound of the side condition Ês/Iot for CSI-RS CMR can be 5 dB, if it’s agreed to add note for HST.

# Conclusion

Proposal 1: We're not leaning towards that the L1-SINR configuration should be avoided in HST scenario from a specification standpoint, but an invalid case doesn't need to be enhanced in particular, even simulation results show necessity of upper boundary.

Proposal 2: Reuse Rel-16 L1-SINR measurement requirements, including the measurement accuracy and sample number for HST; 1.5x relaxation factor is removed when TCSI-RS <= 40ms, and 1.5x relaxation factor is kept when TCSI-RS > 40ms, if it’s agreed to add note for HST.

Proposal 3: The upper bound of the side condition Ês/Iot for CSI-RS CMR can be 5 dB, if it’s agreed to add note for HST.

# References

[1] R4-2017267, Work plan for enhancement for NR high speed train scenario in FR1, CMCC

[2] R4-2115333, WF on RRM for FR1 HST, CMCC