3GPP TSG-RAN WG4 Meeting #101-e	R4- 2118545

Electronic Meeting, 1 – 12 November 2021

Source: 	Huawei, HiSilicon

Title:	PC6 MPR for NB-IoT 16QAM

Agenda Item:	10.9.2.2

Document for:	Approval

# 1	Introduction

As per the WF [1] agreed in RAN4#100-E, one of the few remaining issues is the PC6 MPR for 16QAM. In this paper we provide our simulation results.

# 2	Discussion

Following the agreements in [1], the EVM limit for 16QAM is set to 12.5%, and the IBE mask is updated accordingly. The other simulation assumptions are in line with the agreements in [2]. Table 1 shows the MPR results for 16QAM PC6.

Table 1: Simulated MPR for 16QAM (power class 6)

| Tone spacing | Number of tones | Allocated tones | Gating factor | Simulated MPR [dB] |
| --- | --- | --- | --- | --- |
| 15 kHz | 3 | 0-2 | IBE | 2.1 |
|  |  | 3-5 | IBE | 2.1 |
|  |  | 6-8 | IBE | 2.1 |
|  |  | 9-11 | IBE | 2.1 |
|  | 6 | 0-5 | IBE | 2.1 |
|  |  | 6-11 | IBE | 2.1 |
|  | 12 | 0-11 | OBW | 2.9 |

Proposal 1: Take the above simulation results into account when deciding the PC6 MPR for 16QAM.

# 3	Conclusion

The simulation results for 16QAM PC6 MPR are provided.

Proposal 1: Take the above simulation results into account when deciding the PC6 MPR for 16QAM.

# References

[1] R4-2114943 WF on BS and UE RF requirements in support of NB-IoT 16QAM, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson, RAN4#100-e, August 2021

[2] R4-2105433 WF on UE RF requirements for R17 NB-IoT 16QAM, Nokia, RAN4#98bis-e, April 2021