3GPP TSG-RAN WG4 Meeting # 101-e                              R4-2118714

Electronic Meeting, 1– 12 November, 2021

Source: 	Huawei, HiSilicon

Title: 	Discussion on HAPS simulation assumptions

Agenda Item:	8.13.2.2

Document for:	Approval

Introduction

RAN4 has discussed HAPS simulations assumptions in several meetings. However, there are some open issues that need to be confirmed. In this paper, we’d like to further discuss the simulation assumptions for HAPS.

Discussion

Based on the simulation assumption [1], there are some issues to be clarified and confirmed. The cell layout is shown as below for HAPS. However, some areas that are not covered by the HAPS are highlighted considering 100km radius circular. Some serious co-channel interferences can be observed if contiguous coverage is considered.

Figure 1 cell layout

Based on the Coverage area, 100km radius circular area was assumed. It’s also unclear what the definition of 100km radius circular area is. In my understanding, it’s more reasonable to assume the cell layout for HAPS as figure 2. And 100km radius circular can have a clear definition.

Figure 2 New proposal for HAPS cell layout

Since the HPBW for horizontal/vertical was assumed as 65⁰. The cell radius for the 1st layer can be calculated by 20km timing tan(32.5⁰). It’s about 12.7km.

Proposal 1: To use figure 2 for HAPS cell layout and definition of coverage area as assumption.

The HAPS network parameters are shown below. Since the antenna array configuration are less than or equal to 4 elements for row or column, the antenna model should be non-AAS antenna pattern for HAPS 1st and 2nd layers.

Table 1 HAPS network parameters

| Number of cells | 7 |
| --- | --- |
| Antenna array configuration (row x column) | 2 x 2 for 1st layer cell 4 x 2 for 2nd layer cell |
| Antenna polarization | Linear ±45° |
| Element gain | 7.8 dBi |
| Element HPBW horizontal/vertical | 65⁰ for both H/V |
| Element front-to-back ratio horizontal/vertical | 30 dB for both H/V |
| Element spacing horizontal/vertical | 0.7 wavelength for both H/V |
| Antenna panel tilt (from the horizon) | 90⁰ for 1st layer cell 23⁰for 2nd layer cell |
| EIPR/cell | 56.8 dBm (1st layer cell), 59.8 dBm (2nd layer cell) |
| EIRP spectral density/cell | 43.8 dBm/MHz (1st layer cell), 46.8 dBm/MHz (2nd layer cell) |
| Tx power per antenna panel | 43 dBm |
| Conducted power (before ohmic loss) per antenna element (dBm) | 34 dBm for 2 x 2 (x 2 polarizations) [31] dBm for 4 x 2 (x 2 polarizations) |
| Noise figure | 5 dB |
| Indoor UE percentage | 0% |
| Coverage area (7 cells combined) | A 100 Km radius circular area centered by the serving HAPS |
| UE distribution | Uniformly distributed in the coverage area |

Based on the approved BS non-AAS antenna pattern, it’s proposed to assume the following HAPS non-AAS antenna pattern for 2GHz.

Table 2 HAPS Non-AAS antenna pattern for 2GHz

| Parameter for BS | Values |  |
| --- | --- | --- |
| Antenna vertical radiation pattern (dB) | AE,Vθ″=-min12θ″-90°θ3dB2,SLAV,θ3dB=65°,SLAV=20 or 30dB ? |  |
| Antenna horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=65°,Am=20 or 30dB ? |  |
| Combining method for 3D antenna pattern (dB) |  |  |
| Maximum directional gain of an antenna GE,max | 13.8 dBi for 1st layer 16.8 dBi for 2nd layer |  |
| Conducted power | 43 dBm |  |
| Mechanical downtilt in degrees | 1st layer | 90 |
|  | 2nd layer | 23 |

Proposal 2: To use the table 2 for HAPS antenna pattern in 2GHz.

For indoor UE percentage issue, we can follow the same assumption 0% just like satellite communication considering the higher adjacent channel interference.

Proposal 3: To assume 0% indoor UE percentage for terrestrial network.

- Summary

Based on the discussion, all the observations and proposals are listed below:

Proposal 1: To use figure 2 for HAPS cell layout and definition of coverage area as assumption.

Figure 2 New proposal for HAPS cell layout

Proposal 2: To use the table 2 for HAPS antenna pattern in 2GHz.

Table 2 HAPS Non-AAS antenna pattern for 2GHz

| Parameter for BS | Values |  |
| --- | --- | --- |
| Antenna vertical radiation pattern (dB) | AE,Vθ″=-min12θ″-90°θ3dB2,SLAV,θ3dB=65°,SLAV=20 or 30dB ? |  |
| Antenna horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=65°,Am=20 or 30dB ? |  |
| Combining method for 3D antenna pattern (dB) |  |  |
| Maximum directional gain of an antenna GE,max | 13.8 dBi for 1st layer 16.8 dBi for 2nd layer |  |
| Conducted power | 43 dBm |  |
| Mechanical downtilt in degrees | 1st layer | 90 |
|  | 2nd layer | 23 |

Proposal 3: To assume 0% indoor UE percentage for terrestrial network.

- References

- R4-2115751, Simulation assumptions for HAPS co-existence, Nokia, Nokia Shanghai Bell