3GPP TSG-RAN WG4 Meeting #101-e															R4-2118737

Electronic meeting, November 1st – 12th 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	System parameters for a NR band in the range 52.6GHz – 71GHz

Agenda item:	8.16.2

Document for:	Approval

# Introduction

Agreement:

- For channel raster and sync raster, use Option 1C and Option 1D as starting point to seek the compromised solution.

  - Option 1C: No IEEE 802.11ad/ay alignment and floating channelization (Nokia, Ericsson, ZTE, Xiaomi, OPPO, CMCC, Huawei)

  - Option 1D: Hybrid between IEEE and no IEEE alignment with fixed channelization depending on max spectrum utilization and better coexistence (Intel, Charter, CATT, Sony, MTK, QCOM)

Agreement: For intermediate CBWs between min and max CBWs,

- Integer multiples of the min CBW for each SCS

  - 120 kHz: 100 MHz (min), 400 MHz (max)

  - 480 kHz: 400 MHz (min), 800 MHz, 1600 MHz (max)

  - 960 kHz: 400 MHz (min), 800 MHz, 1600 MHz, 2000 MHz (max)

- FFS whether 1200Mhz CBW is needed for 480KHz SCS and 960Khz SCS

- FFS whether 200MHz CBW is needed for 120KHz SCS

Agreement

- 2 GHz as the maximum CBW with 960 kHz SCS for both licensed and unlicensed bands.

In RAN4#100-e some agreements for system parameters were reached and those were captured in WF [1], and are summarized below.

Building on top of the earlier agreements, this contribution discusses open issues related to systems parameters for a NR band in the range 52.6GHz – 71GHz.

# Discussion

#### Synchronization raster and channel raster

The current WID for extending current NR operation to 71 GHz in [2] states the following on synchronization raster:

  - In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints:

    - Limited sync raster entry numbers

      - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n257 is 599). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz.

The global synchronization rasters is defined in 38.101-2, and furthermore applicable SS raster entries are limited in some operating bands.

Table 5.4.3.1-1: GSCN parameters for the global frequency raster

| Frequency range | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 24250 – 100000 MHz | 24250.08 MHz + N * 17.28 MHz, N = 0:4383 | 22256 + N | 22256 – 26639 |

Table 5.4.3.3-1: Applicable SS raster entries per operating band

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n257 | 120 kHz | Case D | 22388 - <1> - 22558 |
|  | 240 kHz | Case E | 22390 - <2> - 22556 |
| n258 | 120 kHz | Case D | 22257 - <1> - 22443 |
|  | 240 kHz | Case E | 22258 - <2> - 22442 |
| n259 | 120 kHz | Case D | 23140 – <1> – 23369 |
|  | 240 kHz | Case E | 23142 – <2> – 23368 |
| n260 | 120 kHz | Case D | 22995 - <1> - 23166 |
|  | 240 kHz | Case E | 22996 - <2> - 23164 |
| n261 | 120 kHz | Case D | 22446 - <1> - 22492 |
|  | 240 kHz | Case E | 22446 - <2> - 22490 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [10]. |  |  |  |

The unlicensed operating band being proposed for FR2-2 covers from 57-71, a total of 14 GHz of spectrum. Given the limitation of raster points to maximum of 665 and with the aim to reduce the total number of raster points to save resources needed for UE raster search, it is proposed to apply the synchronization raster entries shown in Table 1, where raster points both for the full range as well as the 57-71 GHz unlicensed operating band are shown.

Proposal 1: Adopt the synchronization raster entries for FR2-2 as shown in Table 1:

Table 1. Proposal for synchronization raster entries for FR2-2

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| nXXX 52.6-71GHz | 120kHz | TBD | 23899 < 2 > 24959 |
|  | 480kHz | TBD | 23901 < 8 > 24957 |
| nXXX 57-71GHz | 120kHz | TBD | 24153 < 2 > 24959 |
|  | 480kHz | TBD | 24157 < 8 > 24957 |
| NOTE 1: SS Block pattern is defined in subclause 4.1 in TS 38.213 [10] |  |  |  |

The proposed raster points result in following frequency points and number of raster points. It can be seen that 480 kHz raster points are overlapping with 120 kHz, reducing the total number of points which is set by 120 kHz SCS.

| NR Operating Band | SS Block SCS | Frequency of first raster point | Frequency of last raster point | Raster point spacing | Number of raster points |
| --- | --- | --- | --- | --- | --- |
| nXXX 52.6-71GHz | 120kHz | 52.64112 GHz | 70.95792 GHz | 34.56 MHz | 531 |
|  | 480kHz | 52.67568 GHz | 70.92336 GHz | 138.24 MHz | 133 |
| nXXX 57-71GHz | 120kHz | 57.03024 GHz | 70.95792 GHz | 34.56 MHz | 404 |
|  | 480kHz | 57.09936 GHz | 70.92336 GHz | 138.24 MHz | 101 |

The co-existence between Wigig and other technologies have been extensively studied in ETSI BRAN, where three standards are within scope of the work: 302 567, 303 753 and 303 722 each targeting different scenarios in 57 – 71 GHz band from indoor operation to high power outdoor operation with fixed point-to-point links. None of these standards intended for regulative purposes necessitates usage of a specific channel raster or channel bandwidth. The same applies for FCC 47 CFR § 15.255 governing operation within the band 57-71 GHz.

From the ETSI standards work on 303 753 and 303 722 include or will include beamforming as a co-existence mechanism, requiring no listen-before-talk (LBT) operation to be able to access the channel. Therefore. LBT and channel alignment are not mandatory features from regulatory perspective.

Observation 1: Current ETSI (303 753, 303 722) and FCC (FCC 47 CFR § 15.255) rules do not mandate usage of specific channel bandwidths or channel rasters, therefore there is no issue in using a floating channel raster which is not tied to IEEE channel positions.

Proposal 2: Channel raster for unlicensed operation is defined as a floating raster not limited to IEEE channel positions, however attention needs to be put on number of raster points.

#### Channel bandwidths

In previous meeting agreement on supported channel bandwidths was made, leaving 200 MHz and 1200 MHz channel bandwidth support as FFS. In our view it is more beneficial to simplify the number of channel bandwidths and rather support these cases via carrier aggregation framework. It has been also discussed in earlier meetings that the implementation especially for 1200 MHz bandwidth will not be efficient.

When it comes to whether channel bandwidths are optional or mandatory, we need to take into account the vast amount of spectrum available. For unlicensed operations this will be up to 14 GHz, from 57 GHz to 71 GHz, and therefore mandatory support for wide channel bandwidths is meaningful. Therefore, we propose to make 400 MHz channel bandwidth mandatory for 120 kHz SCS.

In addition, we need to look at the differences between subcarrier spacings. 2000 MHz is the only channel bandwidth that is wider than the channel bandwidths supported for 480 kHz. Therefore, 2000 MHz is proposed to be mandatory for 960 kHz SCS.

Proposal 3: Do not specify 200 and 1200 MHz intermediate channel bandwidths

Proposal 4: 400 MHz shall be mandatory ChBW for 120 kHz SCS.

Proposal 5: 2000 MHz shall be mandatory for 960 kHz SCS.

#### Carrier aggregation

Carrier aggregation provides opportunities for wideband operation with 120 kHz SCS (at the expense of implementation complexity & overhead). Nevertheless, it allows maximal reuse of FR2 hardware in the scenarios where the phase noise is manageable. Furthermore, it provides efficient coexistence among UEs with different bandwidth capabilities. In order to support CA within a 2 GHz band, n x 400 MHz BW capabilities need to be supported (n= [ 2, 3, 4, 5]).

There are multiple 2 GHz chunks available at 60 GHz band. Based on that it makes sense to consider wideband operation involving multiple 2 GHz channels (m*2 GHz).

In RAN4#98-bis-e it was agreed that minimum channel bandwidth for 120 kHz SCS is 100 MHz. To enable smooth implementation transfer from current FR2 to above 52.6 GHz, it is reasonable to enable CA support also for m x 100 MHz CA configurations. It should however be considered that the overhead of CA configuration increases as number of carriers increase, and from performance point of view using wider channel bandwidths is more favorable.

Proposal 6:  Support CA both between 2 GHz channels and for narrower frequency allocations which combine up to 2 GHz.

Proposal 7:  Consider n x 400 MHz, n= [2, 3, 4, 5] and m x 100 MHz, m=[ 2..8] as the supported channel BW options for​ CA operation in unlicensed band for total bandwidths up to 2000 MHz.

Observation 2: From performance point of view wider channel bandwidths are more favorable compared to CA configurations of many CCs.

#### Spectrum utilization

To enable smooth implementation transfer from current FR2 to above 52.6 GHz, it is reasonable to minimize changes to 120 kHz design and apply the same spectrum utilization for 120 kHz SCS in FR2-2 as already agreed and implemented for FR2-1. In simulations submitted to RAN4 in study item phase [4] it was found that EVM and in-band emissions are typically the gating factor for output power and reducing spectrum utilization will not result in higher output power.

Proposal 8: Apply same spectrum utilization for 120 kHz SCS in FR2-2 as in FR2-1

Same behavior was observed also for higher SCS, but given these will be new implementations, it is reasonable to consider other options for SU. FFT utilization is one issue that needs to be considered when defining the spectrum utilization for 480 kHz and 960 kHz SCS. The reason is that it is one factor defining the implementation complexity.

Based on that we think that for 800 MHz and 1600 MHz cases, the starting point would be to aim at similar spectrum utilization as 120 kHz SCS in FR2-1.

However, when considering 2 GHz CBW, the specification should facilitate smooth implementation based on 2k FFT. For example, 156 PRBs, which provides 90.0% spectrum utilization, means that the FFT utilization would be as high as 91.4%.  This would be too much e.g. from the filtering complexity point of view.

Hence, in order to facilitate smooth implementation also for 2k FFT, we propose to support reduced spectrum utilization for 2GHz CBW when using 960 kHz SCS.

Proposal 9: Consider similar spectrum utilization for scenarios with 800MHz and 1600MHz as 120 kHz SCS in FR2-1

Proposal 10: Support reduced spectrum utilization for 960 kHz SCS & 2 GHz CBW

# Conclusion

In this contribution numerologies, channel bandwidths and channel arrangement for NR operation above 52.6 GHz was discussed. The following observations and proposals were made

Proposal 1: Adopt the synchronization raster entries for FR2-2 as shown in Table 1:

Table 1. Proposal for synchronization raster entries for FR2-2

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| nXXX 52.6-71GHz | 120kHz | TBD | 23899 < 2 > 24959 |
|  | 480kHz | TBD | 23901 < 8 > 24957 |
| nXXX 57-71GHz | 120kHz | TBD | 24153 < 2 > 24959 |
|  | 480kHz | TBD | 24157 < 8 > 24957 |
| NOTE 1: SS Block pattern is defined in subclause 4.1 in TS 38.213 [10] |  |  |  |

Observation 1: Current ETSI (303 753, 303 722) and FCC (FCC 47 CFR § 15.255) rules do not mandate usage of specific channel bandwidths or channel rasters, therefore there is no issue in using a floating channel raster which is not tied to IEEE channel positions.

Proposal 2: Channel raster for unlicensed operation is defined as a floating raster not limited to IEEE channel positions, however attention needs to be put on number of raster points.

Proposal 3: Do not specify 200 and 1200 MHz intermediate channel bandwidths

Proposal 4: 400 MHz shall be mandatory ChBW for 120 kHz SCS.

Proposal 5: 2000 MHz shall be mandatory for 960 kHz SCS.

Proposal 6:  Support CA both between 2 GHz channels and for narrower frequency allocations which combine up to 2 GHz.

Proposal 7:  Consider n x 400 MHz, n= [2, 3, 4, 5] and m x 100 MHz, m=[ 2..8] as the supported channel BW options for​ CA operation in unlicensed band for total bandwidths up to 2000 MHz.

Observation 2: From performance point of view wider channel bandwidths are more favorable compared to CA configurations of many CCs.

Proposal 8: Apply same spectrum utilization for 120 kHz SCS in FR2-2 as in FR2-1

Proposal 9: Consider similar spectrum utilization for scenarios with 800MHz and 1600MHz as 120 kHz SCS in FR2-1

Proposal 10: Support reduced spectrum utilization for 960 kHz SCS & 2 GHz CBW

# References

[1] R4-2114988, “WF on NR extension to 71 GHz: Part 1 (System Parameters)”, Intel

[2] RP-212637, “Revised WID: Extending current NR operation to 71 GHz”, Qualcomm, Intel

[3] R4-2102006,	“Numerology and channel bandwidth discussion for NR beyond 52.6 GHz”, Nokia, Nokia Shanghai Bell

[4] R4-2015444, “UE RF for NR beyond 52.6 GHz”, Nokia, Nokia Shanghai Bell