# 3GPP TSG-RAN WG4 Meeting # 101-e	R4-2118843

# Electronic Meeting, November 1-12, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on introducing RRM requirements for SRS antenna switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Core |  |  |  |  |  | Date: |  |  | 2021-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on previous agreements on RRM requirements for SRS antenna switching, the interruption requirements need to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the interruption requirements for SRS antenna switching for NR SA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for SRS antenna switching are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <End of Change 1>

##### 8.2.2.2.x	 Interruptions at NR SRS antenna port switching

When SRS antenna port switching is conducted on SRS resource in SRS resource set with higher layer parameter usage set as 'antennaSwitching', UE is allowed to cause DL interruption of up to X slots on carriers in the band affected by antenna port switching indicated by txSwitchImpactToRx, and UE is allowed to cause UL interruption of up to X slots on carriers in the band which switches together indicated by txSwitchWithAnotherBand, provided that:

- the SRS switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26].

- the SRS antenna port switching is not colliding with any SSB/CSI-RS based L3/L1 measurements and the measurements for RLM/BFD.

Where X is specified in Table 8.2.2.2.x-1

Table 8.2.2.2.x-1: Interruption length X (slot)

|  | NR Slot length of victim cell (ms) | Interruption length X (slots) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Sub carrier spacing for agressor cell (kHz) |  |  |
|  |  | 15 | 30 | 60 |
| 0 | 1 | 2 | 2 | 2 |
| 1 | 0.5 | 2 | 2 | 2 |
| 2 | 0.25 | 3 | 2 | 2 |
| 3 | 0.125 | 5 | 3 | 3 |
| Note: Performance degradation can be expected on one OFDM symbol before each SRS resource and one OFDM symbol after each SRS resource configured for antenna switching which is not overlapped with the guard period defined in TS 38.214 [26] on the carrier where SRS antenna switching occurs. |  |  |  |  |

### <End of Change 1>