3GPP TSG-RAN WG4 Meeting # 101-e									R4-2118849

Electronic Meeting, November 01-12, 2021

Agenda item:	8.16.7.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on interruption requirements for extending NR operation to 71GHz

Document for:	Discussion

# Introduction

The WI on extending NR operation to 71GHz was discussed in the last RAN4 meeting. For interruption requirements, the principles to update the current RRM requirements were discussed, and some interruption requirements were discussed for new introduced SCS [1]. In this paper, we further provide views on remaining issues on interruption requirements in terms of scenarios and requirements updating for remaining requirements.

# Discussion

The general principle to update the interruption requirements of FR2-2 based on the existing requirements of FR2-1 was agreed as follows:

Define the interruption requirements for FR2-2 based on the same assumptions (RF retunings, AGC etc) as in legacy FR2 (FR2-1). - The baseline interruption time (Ti) in FR2-2 to be same as FR2-1 - Nslot,interrupted = ⌈ Ti/Tslot ⌉ for synchronous scenario - Nslot,interrupted = ⌈ Ti/Tslot ⌉ + 1, for asynchronous scenario

The interruption length is based on the same assumptions in FR2-1 and the absolute time is rounded up to the number of slots of corresponding SCS. Based on the this principle, the interruption requirements of SCell activation/deactivation, SCell addition/release, measurement on deactivated SCC, active BWP switching, UE-specific CBW change and NR SRS carrier based switching were updated for 480 KHz and 960 KHz SCS. It is straightforward that other interruption requirements shall be updated based on the same principle.

In the current spec, the interruption requirements are defined for EN-DC, NR SA, NE-DC and NR-DC separately. During the 1st round discussion, the prioritized scenarios of interruption requirements were discussed and companies agreed to follow the agreement in prioritized scenarios. The deployment scenarios were discussed with the following agreements:

GTW agreements: - Define the requirements for the following deployment scenarios with equal priority: - Standalone single carrier and CA in FR2-2 - FR2-2 CA and DC with anchor on FR1 - Note: the scenario may be further adjusted pending further discussion in the RF session

Based on the agreements above, NR SA and NR-DC are feasible scenarios to be considered.

Observation 1: Interruptions requirements of NR SA and NR-DC shall be updated for new introduced SCS based on the agreed deployment scenarios.

Therefore, apart from what has been discussed in the last meeting, e.g. SCell activation/deactivation, the remaining interruption requirement for NR SA and NR-DC shall be updated accordingly.  Based on the same principle, the corresponding requirements are suggested to be updated as follows:

Interruptions at UL carrier RRC reconfiguration

Table 8.2.2.2.4-1: Interruption duration for UL carrier RRC reconfiguration

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 4 |
| 3 | 0.125 | 8 |
| 5 | 0.03125 | 32 |
| 6 | 0.015625 | 64 |

Interruptions at inter-frequency SFTD measurement

Table 8.2.2.2.6-1: Requirements on maximum percentage of interrupted slots in serving cell in inter-frequency SFTD

| SFTD configuration | Serving cell µ | Neighbour cell SMTC periodicity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5ms | 10ms | 20ms | 40ms | 80ms | 160ms |
| With RSRP | 0 | 8.4% | 6.3% | 8.4% | 6.3% | 5.3% | 4.7% |
| report | 1 |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |
|  | 3 |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |
|  | 6 |  |  |  |  |  |  |
| Without RSRP | 0 | 11.4% | 8.6% | 7.9% | 6.8% | 6.3% | 6.0% |
| report | 1 |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |
|  | 3 |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |
|  | 6 |  |  |  |  |  |  |

Table 8.2.2.2.6-2: Interruption duration for FR1 serving cell in inter-frequency SFTD with neighbour cell in FR1

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 4 |
| 3 | 0.125 | 8 |
| 5 | 0.03125 | 32 |
| 6 | 0.015625 | 64 |

Interruptions when identifying CGI of an NR cell with autonomous gaps

Table 8.2.2.2.14-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 6 | 7 | 6 |
| 1 | 0.5 | 12 | 13 | 10 |
| 2 | 0.25 | 24 | 25 | 19 |
| 3 | 0.125 | 48 | 49 | 37 |
| 5 | 0.03125 | 192 | 193 | 145 |
| 6 | 0.015625 | 384 | 385 | 289 |

In additional, the impact of MRTD was also discussed in the last meeting with the following agreement:

The interruption requirements for FR2-2 shall consider the MRTD between CCs where additional slot may be allowed (asynchronous scenarios) when the MRTD is non-negligible considering the slot length of 480 kHz and 960 kHz.

It is suggested to investigate whether the updated interruption requirements need to be revisited when the MRTD requirements are concluded.

Proposal 1: Update the interruption requirements for new introduced SCS 480 KHz and 960 KHz based on the above tables. The value could be revisited when the MRTD requirements are concluded.

# Conclusions

Observation 1: Interruptions requirements of NR SA and NR-DC shall be updated for new introduced SCS based on the agreed deployment scenarios.

Proposal 1: Update the interruption requirements for new introduced SCS 480 KHz and 960 KHz based on the above tables. The value could be revisited when the MRTD requirements are concluded.

# References

[1] R4-2115351 WF on R17 NR_ext_to_71GHz_RRM_1, Qualcomm