3GPP TSG-RAN WG4 Meeting # 101-e 	R4- 2118870

# Electronic Meeting, 01 Nov - 12 Nov, 2021

Source:	LG Electronics

Title:	60GHz channel and synchronization raster

Agenda item:	8.16.2	Operation bands and system parameters (channelization, raster, CBW, etc)

Document for:	Discussion and Approval

# Introduction

Work Item Description for Extending current NR operation to 71 GHz was revised in the 3GPP TSG RAN Meeting #92-e in June 2021 [1]. As part of this revision support for 480kHz SSB for initial access was included in addition to 120k.

- In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints:

  - Limited sync raster entry numbers

    - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n257 is 599). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz.

  - only 480kHz CORESET#0/Type0-PDCCH SCS supported for 480 kHz SSB SCS.

  - Prioritize support SSB-CORESET#0 multiplexing pattern 1. Other patterns discussed on a best effort basis.

  - 960 kHz numerology for the SSB is not supported by the UE for initial access in Rel-17.

In RAN4#100-e meeting the discussion on channel and synchronization started and following way forward was agreed.

- For channel raster and sync raster, use Option 1C and Option 1D as starting point to seek the compromised solution.

  - Option 1A: Align with IEEE 802.11ad/ay with fixed channelization

  - Option 1B: No IEEE 802.11ad/ay alignment with fixed channelization

  - Option 1C: No IEEE 802.11ad/ay alignment and floating channelization.

  - Option 1D: Hybrid between IEEE and no IEEE alignment with fixed channelization depending on max spectrum utilization and better coexistence.

  - Option 1E: Fixed channelization with proper channel raster granularity to consider the co-existence with IEEE 802.11ad/ay alignment if needed.

In this document we provide our views and proposal for channel and synchronization raster for 57-71GHz frequency range intended for un-licensed operation.

# Channel raster for 57-71GHz frequency range

When defining the ARFCN raster for 57-71GHz band following points need to be considered.

- There is 14GHz of contiguous spectrum and hence it is natural to target operation with wide bandwidth channels, 2GHz, while also enabling the narrower bandwidth transmission, like 100MHz, to boost the coverage

- In order to support the wide bandwidths and wide-band operation as in NRU the channel raster and SCS raster with highest SCS, which is 960k in this case, should be aligned.

- In order to support co-existence with other wide band systems operating within the same frequency range, like 802.11ad and 802.11ay, the ARFCN should enable the positioning of the NR channels in a way that NR-channel does not overlap with two 802.11ad or 802.11ay channels.

- Channel raster design shall support maximal spectrum utilization and minimize the overall system complexity also including the test and verification aspects.

Proposal#1:

In order to address the targets above following proposal is made.

- As minimum supported channel bandwidth is 100MHz, the ARFCN is defined in a way that there is a raster location with about 50MHz steps between ~57 050MHz and ~70 950MHz. This enables supporting all channel bandwidths in a way that results in maximal band utilization, which is also illustrated in Figure 1.

- Channel raster starting point is selected in a way that starting frequency is as close to 57 050MHz as possible.

  - NREF= 2 563 333 corresponds to frequency of 57 050.40MHz, which is 40kHz above the target of 57 050.00MHz.

- As 50MHz is not divisible with 960kHz the raster is based on combination of 49.92 and 50.88MHz steps. These can also be expressed as multiples of 60kHz, which is the basis for the global raster in existing specifications. 49.92MHz= 52*960kHz= 832*60kHz and 50.88MHz= 53*960kHz= 848*60kHz.

- In order to minimize the delta between 50MHz mathematical raster and 49.92 and 50.88MHz steps there is a need to take 11x 49.92 and 1x 50.88MHz, which adds up to 600MHz and is divisible with 50MHz and then repeat this through the band. With proposed step size vector of < 832 832 832 832 832 832 848 832 832 832 832 832 > the delta between the 50MHz mathematical raster and the actual channel raster varies between -440kHz and 440kHz. The “100MHz” steps with this kind of raster are either 49.92+49.92= 99.84MHz or 49.92+50.88= 100.8MHz. Although 99.84MHz is slightly smaller than the minimum channel bandwidth of 100MHz this is not expected to cause problems with performance requirements, emissions, guards bands etc. as delta is small (only about 1 SC with 120kHz SCS) and for FR2-2 SU smaller than what is currently used (~95%) has also been discussed.

- Channel raster end-point becomes 70 949.88MHz, which is 120kHz lower than the target of 70 950.00MHz and number of channel raster entries is 279.

Figure 1. ~50MHz channel raster to support all bandwidths with maximal band utilization

This proposal can also be presented using the similar approach as in 38.101-2 table 5.3.2.3-1 as shown below.

Table 5.2-1: NR operating bands in FR2

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |
| n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |
| n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |
| n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |
| nXXX | 57000 MHz | – | 71000 MHz | 57000 MHz | – | 71000 MHz | TDD |

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| Operating Band | ΔFRaster (kHz) | Uplink and Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| n257 | 60 | 2054166 – <1> – 2104165 |
|  | 120 | 2054167 – <2> – 2104165 |
| n258 | 60 | 2016667 – <1> – 2070832 |
|  | 120 | 2016667 – <2> – 2070831 |
| n260 | 60 | 2229166 – <1> – 2279165 |
|  | 120 | 2229167 – <2> – 2279165 |
| n261 | 60 | 2070833 – <1> – 2084999 |
|  | 120 | 2070833 – <2> – 2084999 |
| nXXX | 120, 480, 960 | 2563333 – <NOTE1> – 2794997 |
| NOTE1: Step size vector with length of 12: < 832 832 832 832 832 832 848 832 832 832 832 832 > |  |  |

# Channel raster compatibility with other wide-band systems operated in 57-71GHz frequency range

The applicability of the proposal 1 for supporting the co-existence with 802.11ad and 802.11ay channels with channel center frequencies in Table 1 is analyzed below.

Table 1. 802.11ad and 802.11ay channels

|  | Channel | Flow | Fcenter | Fhigh | Channel BW | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| 802.11ad | #1 | 57 240 | 58 320 | 59 400 | 2 160 | MHz |
|  | #2 | 59 400 | 60 480 | 61 560 | 2 160 | MHz |
|  | #3 | 61 560 | 62 640 | 63 720 | 2 160 | MHz |
|  | #4 | 63 720 | 64 800 | 65 880 | 2 160 | MHz |
|  | #5 | 65 880 | 66 960 | 68 040 | 2 160 | MHz |
|  | #6 | 68 040 | 69 120 | 70 200 | 2 160 | MHz |
| 802.11ay | #9 | 57 240 | 59 400 | 61 560 | 4 320 | MHz |
|  | #10 | 59 400 | 61 560 | 63 720 | 4 320 | MHz |
|  | #11 | 61 560 | 63 720 | 65 880 | 4 320 | MHz |
|  | #12 | 63 720 | 65 880 | 68 040 | 4 320 | MHz |
|  | #13 | 65 880 | 68 040 | 70 200 | 4 320 | MHz |

The frequency delta between the channel center frequencies of 802.11ad and 802.11ay and closest ARFCN following the proposal#1 is shown in Table 2.

Table 2. Channel center frequency delta between the 802.11ad/ay system and proposed raster

|  | Channel | Fcenter 802 | Fcenter NR | NR ARFCN | Delta | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| 802.11ad | #1 | 58 320 | 58 299,96 | 2 584 165 | -20,04 | MHz |
|  | #2 | 60 480 | 60 500,28 | 2 620 837 | 20,28 | MHz |
|  | #3 | 62 640 | 62 649,72 | 2 656 661 | 9,72 | MHz |
|  | #4 | 64 800 | 64 800,12 | 2 692 501 | 0,12 | MHz |
|  | #5 | 66 960 | 66 949,56 | 2 728 325 | -10,44 | MHz |
|  | #6 | 69 120 | 69 099,96 | 2 764 165 | -20,04 | MHz |
| 802.11ay | #9 | 59 400 | 59 400,12 | 2 602 501 | 0,12 | MHz |
|  | #10 | 61 560 | 61 549,56 | 2 638 325 | -10,44 | MHz |
|  | #11 | 63 720 | 63 699,96 | 2 674 165 | -20,04 | MHz |
|  | #12 | 65 880 | 65 900,28 | 2 710 837 | 20,28 | MHz |
|  | #13 | 68 040 | 68 049,72 | 2 746 661 | 9,72 | MHz |

As can be seen the delta between these two center frequencies varies between -20.04 and +20.28MHz, which means that it is possible to place up to 2100MHz wide NR carrier inside the 2160MHz wide 802.11ad channel. Similarly, in the case of 802.11ay, it is possible to place up to 4200MHz wide NR carrier inside the 4320MHz wide channel.

Examples for these co-existence configurations in the case of 802.11ad channels #1 and #2, which represent the positive and negative worst cases, are illustrated in the Figure 2 and Figure 3.

Observation#1:

The proposed channel raster scheme supports also co-existence with 802.11ad and 802.11ay and therefore there is no need to define additional ARFCN to avoid cases where NR channel would overlap with two 802.11ad channels.

Figure 2. Alignment of NR and 802.11ad Channel #1

Figure 3. Alignment of NR and 802.11ad Channel #2

# SSB raster for 57 – 71GHz frequency range

A global synchronization raster defined in 38.101-2 [2] can be used as starting point also for FR2-2. The frequency range specified already covers the 57 to 71GHz frequency range and 17.28MHz step size is suitable for 100 and 400MHz minimum channel bandwidths agreed for 120kHz and 480kHz SCS respectively.

GSCN parameters for the global frequency raster and the applicable SS raster entries for existing FR2-1 bands are shown below.

With 120kHz SCS the SSB block will occupy 28.8MHz of spectrum and with 480kHz SCS SSB width will be 115.2MHz. These bandwidths for SSB and global raster of 17.28MHz make it possible to define more than one SSB location per 100 or 400MHz wide channel bandwidths, which may be beneficial for the network operation, while still “down-selects” the global raster in a way that number of the entries does not become too high. Such proposal is explained in more detail below.

Figure 4. High level description on selection of GSCN locations for 100MHz channels and further down-selection for 400MHz.

The down-selection of the global raster for 100MHz channels can be done with step size vector with length of 19 and values of < 6 6 6 6 6 6 6 6 4 6 6 6 6 6 6 6 6 4 6 >. In order to find two GSCN locations for each 100MHz channel two different starting points 24153 and 24154 can be used. For 480k SCS with further down-selected entries (from 100MHz locations) a step size vectors with length of 19 need to be used as described in Table 3.

Table 3. Example 1 for defining the SS raster entries

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| nYYY 57-71GHz | 120kHz |  | 24153 < NOTE2 > 24959 24154 < NOTE2 > 24960 |
|  | 480kHz |  | 24159 < NOTE3 > 24947 24165 < NOTE4 > 24953 |
| NOTE1: SS Block pattern is defined in sub clause 4.1 in TS 38.213 [10] NOTE2: Step size vector with length of 19: < 6 6 6 6 6 6 6 6 4 6 6 6 6 6 6 6 6 4 6 > NOTE3: Step size vector with length of 19: < 24 22 24 24 22 24 22 24 22 24 24 22 24 22 24 24 22 24 22 > NOTE4: Step size vector with length of 19: < 24 22 24 22 24 24 22 24 22 24 24 22 24 22 24 22 24 24 22 > |  |  |  |

With this approach the total number of SS raster entries would be 250 as shown in Table 4. Below. This number is below the targeted 655 that is mentioned in [1].

Table 4. Number of SS raster entries

|  | Channels | Locations per channel | Total |
| --- | --- | --- | --- |
| 120k SCS | 140 | 2 | 280 |
| 480k SCS | 35 | 2 | 70 |
| TOTAL |  |  | 350 |

Figures 5 and 6 below show how the SSBs falls inside the 100 and 400MHz channels when defined as in Table 3 and Table 5 shows the width of the upper and lower guard-band (gap between SSB and channel edge), which are always larger than the minimum guard-bands currently defined in 38.101-2, which are 2.42MHz for 100MHz bandwidth and 9.86MHz for 400MHz wide bandwidth.

Table 5. Minimum guard bands

|  | Low | High |
| --- | --- | --- |
| 120k SCS | 12,96 | 3,84 |
| 480k SCS | 70,56 | 77,92 |

Figure 5. 120k SCS SSB locations

Figure 6. 480k SCS SSB locations

# Conclusion

In this document channel raster and SSB raster for 57-71GHz frequency range is discussed and proposals for both are made.

Proposal#1: Channel raster for 57-71GHz as explained in Section 2, Channel raster for 57-71GHz frequency range.

Observation#1: The proposed channel raster scheme supports also co-existence with 802.11ad and 802.11ay and therefore there is no need to define additional ARFCN to avoid cases where NR channel would overlap with two 802.11ad channels.

Proposal#2: Sync raster for 57-71GHz as explained in Section 4, SSB raster for 57-71GHz frequency range.

# References

[1] RP-211584, Revised WID:  Extending current NR operation to 71 GHz, RAN#92-e, June 2021, Qualcomm, Intel

[2] 3GPP TS 38.101-2, User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone

[3] R4-2114988, WF on NR extension to 71 GHz: Part 1 (System Parameters), RAN4#100-e, August 2021, Intel