3GPP TSG-RAN WG4 Meeting # 101-e	R4-2118937

Electronic Meeting, November 01-12, 2021

Source:	ZTE Corporation

Title:	Positioning measurements in RRC_INACTIVE state

Agenda item:	8.21.2.3

Document for:	Discussion

# 1	Introduction

In RAN4 100-e, there were discussions on measurement during RRC_INACTIVE period for positioning enhancement. Some of the WF [1] are copied below.

Issue 2-2-1 The factors considered for the measurement requirements in RRC_INACTIVE state Agreements: MG is not to be considered in the measurement period requirements in RRC_INACTIVE state. FFS: Further study the following factors and impacts for the measurement requirements in RRC_INACTIVE state: - Whether to use the reduced number of samples if it is agreed in Rel-17. - Whether to use the summation-based approach for total frequency layers. - The impact of paging periods. - Analysis on PRS resource configuration, positioning measurement period and DRX behaviors in the UE RRC_INACTIVE state. - How to define the measurement interval Teffect. - How to define the parameter Kcarrier. - Others

- This paper discusses the issues listed above and provide our view on the other related issues.

2	Discussions

To support positioning in INACTIVE mode, one major difference is that in INACTIVE mode, the UE only wakes up to receive paging information. This is similar with the UE behavior under measurement gap and DRX in CONNECTED mode. Thus, we think that when defining core requirements for UE positioning under INACTIVE mode, we can replace the measurement gap related parameters with paging periods.

- Observation 1: The UE behavior in INACTIVE mode with respect to positioning measurement is similar with the UE behavior under measurement gap in CONNECTED mode.

When defining core requirements for UE positioning under INACTIVE mode, the principle can be to replace the measurement gap related parameters with paging periods and re-use the R16 requirements for CONNECTED mode.

3	Conclusion

- Observation 1: The UE behavior in INACTIVE mode with respect to positioning measurement is similar with the UE behavior under measurement gap in CONNECTED mode.

Proposal 1: When defining core requirements for UE positioning under INACTIVE mode, the principle can be to replace the measurement gap related parameters with paging periods and re-use the R16 requirements for CONNECTED mode.

References

- R4-2115367