3GPP TSG-RAN WG4 Meeting # 101-e	R4-2118944

Electronic Meeting, November 01-12, 2021

Source:	ZTE Corporation

Title:	On Te requirements for NR systems extended to 71 GHz

Agenda item:	8.16.7.2

Document for:	Discussion

# 1	Introduction

- In RAN4 100-e, some agreements regarding Te requirements were captured in the WF [1] and some of the content is copied below.

Issue 2-1-1: General principles in defining the Te requirements GTW agreements: - Further study percentage of UL CP length Te can occupy without impacting UL system performance? - Option 1: Keep the Te within the same percentage of the CP length as existing SCS - Option 2: 30% - Option 3: 50% - Further study achievable Te from UE perspective - Study different combinations of SSB SCS and UL signal SCS for FR2-2

- This paper discusses the options lusted above.

2	Discussions

In general, we think that Te requirement is critical to maintain UL transmissions and it ensures the overall performance of not only one UE, but potentially the network and also other UEs. Current Te requirements are derived based on SCS and CP length, and we see it reasonable to re-use the current percentage and formulation.

Keep the Te within the same percentage of the CP length as existing SCS.

However, as pointed out by some companies, there might be technical challenges such that a part of Te is not proportional to SCS and thus cannot be decreased automatically as SCS increases. Since current Te requirements indicate a percentage of UL CP of around 20%, it is also reasonable to relax to 30% depending on UE implementations.

Te can occupy 30% of UL CP length if UE implementations are proven to be impossible to keep current Te requirements.

3	Conclusion

- Proposal 1: Keep the Te within the same percentage of the CP length as existing SCS.

- Proposal 2: Te can occupy 30% of UL CP length if UE implementations are proven to be impossible to keep current Te requirements.

References

- R4-2115351