3GPP TSG-RAN4 Meeting #101-e	R4-2119038

# Electronic Meeting, Nov.1-12,2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR |  | rev | - | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR: cleanup the square brackets for LTE PUSCH performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI |  |  |  |  |  | Date: |  |  | 2021-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are still brackets exsiting in some requirements in Table 8.2.1.1-4, Table 8.2.1.1-5 and Table 8.2.1.1-6 |  |  |  |  |  |  |  |  |
| And |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the corrosponding brackets |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The brackets will still be existing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.141 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

Table 8.2.1.1-4 Minimum requirements for PUSCH, 10 MHz Channel Bandwidth, 1Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | EPA 5Hz Low | A3-5 | 30% | -4.2 |
|  |  |  |  |  | 70% | -0.4 |
|  |  |  |  | A4-6 | 70% | 10.8 |
|  |  |  |  | A5-5 | 70% | 18.3 |
|  |  |  |  | A17-4 | 70% | 22.6 |
|  |  |  |  | A18-4 | 70% | 8.5 |
|  |  |  |  | A19-4 | 70% | 19.5 |
|  |  |  | EVA 5Hz Low | A3-1 | 30% | -2.7 |
|  |  |  |  |  | 70% | 1.9 |
|  |  |  |  | A4-1 | 30% | 4.3 |
|  |  |  |  |  | 70% | 11.4 |
|  |  |  |  | A5-1 | 70% | 18.8 |
|  |  |  | EVA 70Hz Low | A3-5 | 30% | -4.1 |
|  |  |  |  |  | 70% | 0.1 |
|  |  |  |  | A4-6 | 30% | 4.5 |
|  |  |  |  |  | 70% | 12.6 |
|  |  |  | ETU 70Hz* Low | A3-1 | 30% | -2.5 |
|  |  |  |  |  | 70% | 2.4 |
|  |  |  | ETU 300Hz* Low | A3-1 | 30% | -2.2 |
|  |  |  |  |  | 70% | 2.9 |
|  |  | Extended | ETU 70Hz* Low | A4-2 | 30% | 4.8 |
|  |  |  |  |  | 70% | 13.6 |
|  | 4 | Normal | EPA 5Hz Low | A3-5 | 30% | -6.8 |
|  |  |  |  |  | 70% | -3.5 |
|  |  |  |  | A4-6 | 70% | 7.5 |
|  |  |  |  | A5-5 | 70% | 14.7 |
|  |  |  |  | A17-4 | 70% | 19.2 |
|  |  |  |  | A18-4 | 70% | 5.3 |
|  |  |  |  | A19-4 | 70% | 15.8 |
|  |  |  | EVA 5Hz Low | A3-1 | 30% | -5.0 |
|  |  |  |  |  | 70% | -1.2 |
|  |  |  |  | A4-1 | 30% | 1.2 |
|  |  |  |  |  | 70% | 7.9 |
|  |  |  |  | A5-1 | 70% | 15.5 |
|  |  |  | EVA 70Hz Low | A3-5 | 30% | -6.7 |
|  |  |  |  |  | 70% | -2.9 |
|  |  |  |  | A4-6 | 30% | 0.7 |
|  |  |  |  |  | 70% | 8.0 |
|  |  |  | ETU 70Hz* Low | A3-1 | 30% | -4.8 |
|  |  |  |  |  | 70% | -0.9 |
|  |  |  | ETU 300Hz* Low | A3-1 | 30% | -4.6 |
|  |  |  |  |  | 70% | -0.6 |
|  |  |  | ETU 600Hz** Low | A13-4 | 30% | -1.0 |
|  |  |  |  |  | 70% | 6.2 |
|  |  | Extended | ETU 70Hz* Low | A4-2 | 30% | 1.7 |
|  |  |  |  |  | 70% | 10.3 |
|  | 8 | Normal | EPA 5Hz Low | A3-5 | 30% | -9.8 |
|  |  |  |  |  | 70% | -6.7 |
|  |  |  |  | A4-6 | 70% | 4.2 |
|  |  |  |  | A5-5 | 70% | 11.5 |
|  |  |  |  | A17-4 | 70% | 15.9 |
|  |  |  |  | A18-4 | 70% | 2.1 |
|  |  |  |  | A19-4 | 70% | 12.5 |
|  |  |  | EVA 5Hz Low | A3-1 | 30% | -6.9 |
|  |  |  |  |  | 70% | -3.8 |
|  |  |  |  | A4-1 | 30% | -1.7 |
|  |  |  |  |  | 70% | 4.5 |
|  |  |  |  | A5-1 | 70% | 11.9 |
|  |  |  | EVA 70Hz Low | A3-5 | 30% | -9.7 |
|  |  |  |  |  | 70% | -6.2 |
|  |  |  |  | A4-6 | 30% | -2.6 |
|  |  |  |  |  | 70% | 4.7 |
|  |  |  | ETU 70Hz* Low | A3-1 | 30% | -6.8 |
|  |  |  |  |  | 70% | -3.6 |
|  |  |  | ETU 300Hz* Low | A3-1 | 30% | -6.8 |
|  |  |  |  |  | 70% | -3.3 |
|  |  | Extended | ETU 70Hz* Low | A4-2 | 30% | -1.1 |
|  |  |  |  |  | 70% | 6.5 |
| Note*: Not applicable for Local Area BS and Home BS. Note**: Not applicable for Local Area BS and Home BS, and only applicable for BS supporting ETU600. |  |  |  |  |  |  |

Table 8.2.1.1-5 Minimum requirements for PUSCH, 15 MHz Channel Bandwidth, 1Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | EPA 5Hz Low | A3-6 | 30% | -4.5 |
|  |  |  |  |  | 70% | -0.8 |
|  |  |  |  | A4-7 | 70% | 11.3 |
|  |  |  |  | A5-6 | 70% | 18.8 |
|  |  |  |  | A17-5 | 70% | 22.8 |
|  |  |  |  | A18-5 | 70% | 9.4 |
|  |  |  |  | A19-5 | 70% | 21.4 |
|  |  |  | EVA 5Hz Low | A3-1 | 30% | -2.8 |
|  |  |  |  |  | 70% | 1.8 |
|  |  |  |  | A4-1 | 30% | 4.2 |
|  |  |  |  |  | 70% | 11.4 |
|  |  |  |  | A5-1 | 70% | 18.7 |
|  |  |  | EVA 70Hz Low | A3-6 | 30% | -4.5 |
|  |  |  |  |  | 70% | -0.3 |
|  |  |  |  | A4-7 | 30% | 4.2 |
|  |  |  |  |  | 70% | 12.9 |
|  |  |  | ETU 70Hz* Low | A3-1 | 30% | -2.5 |
|  |  |  |  |  | 70% | 2.4 |
|  |  |  | ETU 300Hz* Low | A3-1 | 30% | -2.2 |
|  |  |  |  |  | 70% | 2.9 |
|  |  | Extended | ETU 70Hz* Low | A4-2 | 30% | 4.9 |
|  |  |  |  |  | 70% | 13.6 |
|  | 4 | Normal | EPA 5Hz Low | A3-6 | 30% | -7.2 |
|  |  |  |  |  | 70% | -3.8 |
|  |  |  |  | A4-7 | 70% | 7.6 |
|  |  |  |  | A5-6 | 70% | 15.0 |
|  |  |  |  | A17-5 | 70% | 18.9 |
|  |  |  |  | A18-5 | 70% | 5.9 |
|  |  |  |  | A19-5 | 70% | 17.1 |
|  |  |  | EVA 5Hz Low | A3-1 | 30% | -5.0 |
|  |  |  |  |  | 70% | -1.2 |
|  |  |  |  | A4-1 | 30% | 1.2 |
|  |  |  |  |  | 70% | 7.9 |
|  |  |  |  | A5-1 | 70% | 15.7 |
|  |  |  | EVA 70Hz Low | A3-6 | 30% | -7.0 |
|  |  |  |  |  | 70% | -3.3 |
|  |  |  |  | A4-7 | 30% | 0.7 |
|  |  |  |  |  | 70% | 8.5 |
|  |  |  | ETU 70Hz* Low | A3-1 | 30% | -4.8 |
|  |  |  |  |  | 70% | -1.0 |
|  |  |  | ETU 300Hz* Low | A3-1 | 30% | -4.6 |
|  |  |  |  |  | 70% | -0.6 |
|  |  |  | ETU 600Hz** Low | A13-5 | 30% | -0.9 |
|  |  |  |  |  | 70% | 6.4 |
|  |  | Extended | ETU 70Hz* Low | A4-2 | 30% | 1.6 |
|  |  |  |  |  | 70% | 10.1 |
|  | 8 | Normal | EPA 5Hz Low | A3-6 | 30% | -10.4 |
|  |  |  |  |  | 70% | -7.3 |
|  |  |  |  | A4-7 | 70% | 4.4 |
|  |  |  |  | A5-6 | 70% | 11.8 |
|  |  |  |  | A17-5 | 70% | 15.5 |
|  |  |  |  | A18-5 | 70% | 2.8 |
|  |  |  |  | A19-5 | 70% | 13.8 |
|  |  |  | EVA 5Hz Low | A3-1 | 30% | -7.1 |
|  |  |  |  |  | 70% | -4.0 |
|  |  |  |  | A4-1 | 30% | -1.7 |
|  |  |  |  |  | 70% | 4.4 |
|  |  |  |  | A5-1 | 70% | 11.7 |
|  |  |  | EVA 70Hz Low | A3-6 | 30% | -10.1 |
|  |  |  |  |  | 70% | -6.8 |
|  |  |  |  | A4-7 | 30% | -2.5 |
|  |  |  |  |  | 70% | 5.0 |
|  |  |  | ETU 70Hz* Low | A3-1 | 30% | -7.0 |
|  |  |  |  |  | 70% | -3.6 |
|  |  |  | ETU 300Hz* Low | A3-1 | 30% | -6.9 |
|  |  |  |  |  | 70% | -3.3 |
|  |  | Extended | ETU 70Hz* Low | A4-2 | 30% | -1.1 |
|  |  |  |  |  | 70% | 6.7 |
| Note*: Not applicable for Local Area BS and Home BS. Note**: Not applicable for Local Area BS and Home BS, and only applicable for BS supporting ETU600. |  |  |  |  |  |  |

Table 8.2.1.1-6 Minimum requirements for PUSCH, 20 MHz Channel Bandwidth, 1Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | EPA 5Hz Low | A3-7 | 30% | -4.2 |
|  |  |  |  |  | 70% | -0.4 |
|  |  |  |  | A4-8 | 70% | 11.5 |
|  |  |  |  | A5-7 | 70% | 19.7 |
|  |  |  |  | A17-6 | 70% | 23.7 |
|  |  |  |  | A18-6 | 70% | 9.3 |
|  |  |  |  | A19-6 | 70% | 21.0 |
|  |  |  | EVA 5Hz Low | A3-1 | 30% | -2.7 |
|  |  |  |  |  | 70% | 1.8 |
|  |  |  |  | A4-1 | 30% | 4.3 |
|  |  |  |  |  | 70% | 11.5 |
|  |  |  |  | A5-1 | 70% | 18.7 |
|  |  |  | EVA 70Hz Low | A3-7 | 30% | -4.1 |
|  |  |  |  |  | 70% | 0.2 |
|  |  |  |  | A4-8 | 30% | 4.2 |
|  |  |  |  |  | 70% | 13.0 |
|  |  |  | ETU 70Hz* Low | A3-1 | 30% | -2.4 |
|  |  |  |  |  | 70% | 2.4 |
|  |  |  | ETU 300Hz* Low | A3-1 | 30% | -2.1 |
|  |  |  |  |  | 70% | 2.9 |
|  |  | Extended | ETU 70Hz* Low | A4-2 | 30% | 4.7 |
|  |  |  |  |  | 70% | 13.6 |
|  | 4 | Normal | EPA 5Hz Low | A3-7 | 30% | -6.8 |
|  |  |  |  |  | 70% | -3.5 |
|  |  |  |  | A4-8 | 70% | 7.5 |
|  |  |  |  | A5-7 | 70% | 15.9 |
|  |  |  |  | A17-6 | 70% | 19.8 |
|  |  |  |  | A18-6 | 70% | 5.7 |
|  |  |  |  | A19-6 | 70% | 16.6 |
|  |  |  | EVA 5Hz Low | A3-1 | 30% | -5.1 |
|  |  |  |  |  | 70% | -1.3 |
|  |  |  |  | A4-1 | 30% | 1.2 |
|  |  |  |  |  | 70% | 7.9 |
|  |  |  |  | A5-1 | 70% | 15.6 |
|  |  |  | EVA 70Hz Low | A3-7 | 30% | -6.7 |
|  |  |  |  |  | 70% | -2.9 |
|  |  |  |  | A4-8 | 30% | 0.7 |
|  |  |  |  |  | 70% | 8.6 |
|  |  |  | ETU 70Hz* Low | A3-1 | 30% | -4.4 |
|  |  |  |  |  | 70% | -0.9 |
|  |  |  | ETU 300Hz* Low | A3-1 | 30% | -4.6 |
|  |  |  |  |  | 70% | -0.7 |
|  |  |  | ETU 600Hz** Low | A13-6 | 30% | -0.9 |
|  |  |  |  |  | 70% | 6.4 |
|  |  | Extended | ETU 70Hz* Low | A4-2 | 30% | 1.6 |
|  |  |  |  |  | 70% | 10.0 |
|  | 8 | Normal | EPA 5Hz Low | A3-7 | 30% | -9.7 |
|  |  |  |  |  | 70% | -6.7 |
|  |  |  |  | A4-8 | 70% | 4.3 |
|  |  |  |  | A5-7 | 70% | 12.5 |
|  |  |  |  | A17-6 | 70% | 16.3 |
|  |  |  |  | A18-6 | 70% | 2.6 |
|  |  |  |  | A19-6 | 70% | 13.2 |
|  |  |  | EVA 5Hz Low | A3-1 | 30% | -7.0 |
|  |  |  |  |  | 70% | -3.9 |
|  |  |  |  | A4-1 | 30% | -1.7 |
|  |  |  |  |  | 70% | 4.6 |
|  |  |  |  | A5-1 | 70% | 12.0 |
|  |  |  | EVA 70Hz Low | A3-7 | 30% | -9.7 |
|  |  |  |  |  | 70% | -6.1 |
|  |  |  |  | A4-8 | 30% | -2.2 |
|  |  |  |  |  | 70% | 4.9 |
|  |  |  | ETU 70Hz* Low | A3-1 | 30% | -6.9 |
|  |  |  |  |  | 70% | -3.5 |
|  |  |  | ETU 300Hz* Low | A3-1 | 30% | -6.8 |
|  |  |  |  |  | 70% | -3.3 |
|  |  | Extended | ETU 70Hz* Low | A4-2 | 30% | -1.2 |
|  |  |  |  |  | 70% | 6.5 |
| Note*: Not applicable for Local Area BS and Home BS. Note**: Not applicable for Local Area BS and Home BS, and only applicable for BS supporting ETU600. |  |  |  |  |  |  |

<End of Change 1>