3GPP TSG-RAN WG4 Meeting # 101-e 			R4-2119231

# Electronic Meeting, November 1-12, 2021

Source: 		Nokia, Nokia Shanghai Bell

Title: 			On 900MHz RMR RAN4 BS RF requirements impact due to ECC Decision (20)02

Agenda Item: 	7.3.3

Document for:	Approval

# Introduction

During RAN4#100e meeting, a WF on BS RF requirements for RMR was agreed [1]. This document addresses the following part of the WF, in order to progress the work on BS RF requirements from ECC Decision (20)02:

Way Forward 1 (Sub topic 1-1: EIRP requirements conversion to conducted requirements):

For the next meeting, provided pros and cons analyses for the following options for the EIRP-based ECC requirements conversion to conducted requirements:

- Manufacturer declaration based approach,

- Fixed antenna gain approach.

Other options are not precluded for the analysis next meeting

# Discussion

As requested in the WF, below is the summary of pros and cons for manufacturer declaration vs. fixed antenna gain:

| Approach | Pros | Cons |
| --- | --- | --- |
| Manufacturer declaration (option A) | Allows for deployment flexibility, optimized for all considered scenarios while keeping conducted requirements not too restrictive for some (lower antenna gain) deployments. | Not preferred option for ETSI harmonized standard. |
| Fixed antenna gain (option B) | Preferred option for ETSI harmonized standard. | Reduced deployment flexibility, defined requirements may be too restrictive since fixed antenna gain needs to be defined as maximum antenna gain used for all scenarios. |

Furthermore, below is the summary of how relevant ECC Decision 20(02) requirements would need to be addressed in 3GPP specifications for both option A and B.

Table 1: Specific in-block requirements for 5 MHz channels
mandatory for uncoordinated deployment

| RMR channel BW | Maximum e.i.r.p. |
| --- | --- |
| 5 MHz | = 64.5 dBm/5MHz + (fDL – 922.1)×40/3 dB |
| fDL is the centre frequency in MHz. NB-IoT in-band operation mode without power boost is allowed. NB-IoT guard-band operation mode and in-band operation mode with power boost are not allowed. |  |

Option A: new declaration parameters would need to be introduced in 3GPP specifications

Option B: new conducted requirement would need to be added, taking into account 17dBi antenna gain

Table 2: Out-of-band requirements

| MHz from block edge (919.4-925 MHz) | e.i.r.p. limit |
| --- | --- |
| 0 f < 0.2 | 32.5 dBm/200kHz |
| 0.2 f < 1 | 14 dBm/800kHz |
| 1 f < 10 | 5 dBm/MHz |
| On a case-by-case basis, at a national level, higher OOB limits may be applied. |  |

Option A: new declaration parameters would need to be introduced in 3GPP specifications

Option B: new conducted requirement would need to be added, taking into account 17dBi antenna gain

Table 3: Baseline requirement

| Frequency range | e.i.r.p. limit |
| --- | --- |
| 880-915 MHz | -49 dBm/5MHz |
| This requirement prevails over out-of-band requirements. |  |

Option A: new declaration parameters would need to be introduced in 3GPP specifications

Option B: new conducted requirement would need to be added, taking into account 17dBi antenna gain

Table 4: Requirements on wideband RMR BS receiver characteristics

| Parameter | Value |
| --- | --- |
| Level of the wanted signal | RefSens + 3 dB |
| Maximum interfering signal in 870-874.4 MHz (Note 1) | -34 dBm |
| The antenna connector of the radio module is the reference point. The reference sensitivity (RefSens) is the minimum mean power received at the antenna connector at which a specified minimum performance shall be met. These requirements cover both blocking and third-order intermodulation. Note 1: It is up to ETSI to define a relevant interfering signal against which the conformity test will be performed. In this Decision, CEPT considered a bandwidth of 200 kHz for the interfering signal. |  |

Option A: new blocking requirement would need to be added

Option B: new blocking requirement would need to be added

# Conclusion

This document provides the summary of pros and cons for manufacturer declaration vs. fixed antenna gain as well as how relevant ECC Decision 20(02) requirements would need to be addressed in 3GPP specifications for both options. These shall be taken into account for final 3GPP decision on which option is preferred for these requirements.

# References

[1] R4-2115637	WF on BS RF requirements for RMR900 and RMR1900; Huawei; RAN4#100-e