3GPP TSG-RAN WG4 Meeting # 101-e																  R4-2119710

Electronic Meeting, November. 01-12, 2021

Agenda item:			7.24

Source:	Moderator (CATT)

Title:	Email discussion summary for [101-e][110] NR_LTE_V2X_PC5_combos

Document for:	Information

Introduction

In this email discussion summary, the MSD for V2X_n79A-n47A and V2X_n79A_47A will be discussed.

The candidate targets of this email discussion for 1st round and 2nd round:

- 1st round

  - Companies to provide comments on open issues involved.

  - Companies to provide comments on TPs and CRs involved.

- 2nd round

  - Further check the revised TPs and CRs if any.

  - Recommend the final status of TPs and CRs.

Topic #1: MSD for V2X_n79A-n47A and V2X_n79A_47A

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117645 | Qualcomm Incorporated | Title: Calculation of MSD for n79 for V2X_n79A-n47A and V2X_n79A-47A and accompanying TP Observation2: MSD values ranging from 1 dB to 3.3 dB were suggested for n79 in RAN4#100-e Observation2: It is noted that UEs need not operate with the maximum specified MSD. They can use the lower MSD values that enables the specifications to be met. Observation3: Specifying the maximum MSD from the range discussed in RAN4#100-e in no way harms any UEs that can use lower MSDs. On the contrary it permits a broader range of UEs to meet the required specifications Proposal 1: The MSD value of 3.3 dB be adopted for band n79 in V2X_n79A-47A Proposal 2: The MSD value of 3.3 dB be adopted for band n79 V2X_n79A-n47A ## --Start of changes1 #### 6.2.4.4 MSD The REFSENS exception due to cross band isolation can be observed for V2X_n79A-n47A. One MSD test configuration is specified as below. The MSD values given in tables 6.2.4.1-1and 6.2.4.1-2 shall apply for all n79 and n47 BWs and SCSs combinations given in table 6.2.4.2-1 Table 6.2.4.1-1: Reference sensitivity exceptions (MSD) due to cross band isolation for V2X UL band; SL band; SCS of UL/DL band (kHz); LCRB of UL band; UL band BW(MHz); LCRB of SL band; SL band BW (MHz); MSD value of SL band (dB). n79; n47; 15; 216; 40; 50; 10; 3.3. Table 6.2.4.1-2: Reference sensitivity exceptions (MSD) due to cross band isolation for Uu UL band; SL band; SCS of UL/DL band (kHz); LCRB of UL band; UL band BW(MHz); LCRB of SL band; SL band BW (MHz); MSD value of Uu band (dB). n79; n47; 15; 216; 40; 50; 10; 3.3. ## --End of changes1 ## --Start of changes2 #### 6.3.4.4 MSD The REFSENS exception due to cross band isolation can be observed for V2X_n79A_47A. One MSD test configuration is specified as below. The MSD values given in tables 6.3.4.4-1 and 6.3.4.4-2 shall apply for all n79 and 47 BWs and SCSs combinations given in table 6.3.3.2-1 Table 6.3.4.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR V2X UL band; DL band; SCS of UL/DL band (kHz); LCRB of UL band; UL band BW(MHz); LCRB of DL band; DL band BW (MHz); MSD value of DL band (dB). n79; 47; 15; 216; 40; 50; 10; 3.3. Table 6.3.4.4-2: Reference sensitivity exceptions (MSD) due to cross band isolation for Uu UL band; SL band; SCS of UL/DL band (kHz); LCRB of UL band; UL band BW(MHz); LCRB of SL band; SL band BW (MHz); MSD value of Uu band (dB). n79; n47; 15; 216; 40; 50; 10; 3.3. --End of changes2 |
| R4-2118706 | Huawei, HiSilicon | Title: TP for TR 37.875 on MSD for V2X_n79A-n47A and V2X_n79A_47A - ---Start of changes1--- - 6.2.4.4 MSD The REFSENS exception due to cross band isolation can be observed for V2X_n79A-n47A. One MSD test configuration is specified as below for each band combination. The MSD values given in tables 6.2.4.4-1and 6.2.4.4-2 shall apply for all n79 and n47 BWs and SCSs combinations given in table 6.2.4.2-1. Table 6.2.4.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for V2X band UL band; SL reception band; SCS of UL/DL band (kHz); LCRB of UL band; UL band BW(MHz); LCRB of SL band; SL band BW (MHz); MSD value of SL band (dB). n79; n47; 15; 216; 40; 50; 10; 3.3. Table 6.2.4.4-2: Reference sensitivity exceptions (MSD) due to cross band isolation for Uu band SL transmission band; DL band; SCS of UL/DL band (kHz); LCRB of SL transmission band; SL transmission BW(MHz); LCRB of DL band; DL band BW (MHz); MSD value of DL band (dB). n47; n79; 15; 50; 10; 216; 40; 2.1. - ---End of changes1--- - ---Start of changes2--- - 6.3.4.4 MSD The REFSENS exception due to cross band isolation can be observed for V2X_n79A_47A. One MSD test configuration is specified as below for each band combination. The MSD values given in tables 6.3.4.4-1and 6.3.4.4-2 shall apply for all n79 and n47 BWs and SCSs combinations given in table 6.3.4.2-1. Table 6.3.4.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for V2X band UL band; SL reception band; SCS of UL/DL band (kHz); LCRB of UL band; UL band BW(MHz); LCRB of DL band; DL band BW (MHz); MSD value of DL band (dB). n79; 47; 15; 216; 40; 50; 10; 3.3. Table 6.3.4.4-2: Reference sensitivity exceptions (MSD) due to cross band isolation for Uu band SL transmission band; DL band; SCS of UL/DL band (kHz); LCRB of SL transmission band; SL transmission BW(MHz); LCRB of DL band; DL band BW (MHz); MSD value of DL band (dB). 47; n79; 15; 50; 10; 216; 40; 2.1. - ---End of changes2--- |

Open issues summary

Based on above contributions, the following sub-topics and issues are summarized.

Issue 1-1-1: MSD for V2X_n79A-n47A and V2X_n79A_47A

- Proposals

  - Option 1: Adopt the MSD (3.3dB) for Uu bands proposed in paper R4-2117645.

  - Option 2: Adopt the MSD (2.1dB) for Uu bands proposed in paper R4-2118706.

- Recommended WF

  - To be concluded in 1st round.

Companies views’ collection for 1st round

Open issues

Issue 1-1-1: MSD for V2X_n79A-n47A and V2X_n79A_47A

| Company | Comments |
| --- | --- |
| Qualcomm | Option 1: Adopt the MSD (3.3 dB) for Uu bands proposed in paper R4-2117645. As mentioned in our paper specifying the Max MSD from the considered range of values in no way harms the performance of any UE that can use lower MSDs. |
| LGE | Maybe 3.3 dB can be specified for Uu bands. But the Table format and configuration is follow the R4-2118706. It is more consistent and aligned in specification. |
| Huawei | In order to make progress, we are OK with 3.3dB MSD. We can revise R4-2118706 to capture this MSD value. |
| CATT | OK with 3.3dB MSD for Uu band. |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117645 (Calculation of MSD for n79 for V2X_n79A-n47A and V2X_n79A-47A and accompanying TP) | Qualcomm: agree with MSD of 3.3 dB for n79 |
|  | LGE: The table format will be updated based on the R4-2118706 |
|  | Huawei: We are OK with 3.3dB MSD. But the table format is not correct. |
|  | CATT: The table format in 8706 is more appropriate. |
| R4-2118706 (Discussion and TP for TR 37.875 on MSD for V2X_n79A-n47A and V2X_n79A_47A) | Qualcomm: cannot agree with R4-2118706 proposing a MSD=2.1 dB as our analysis indicates that a larger value is required for n79. |
|  | LGE: compromise to define the 3.3 dB for NR Uu band. |
|  | Huawei: In order to make progress, we are OK with 3.3dB MSD. We can revise R4-2118706 to capture this MSD value. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1-1: MSD for V2X_n79A-n47A and V2X_n79A_47A | Tentative agreements: Adopt the MSD (3.3dB) for Uu bands. Candidate options: None Recommendations for 2nd round: No further discussion needed in 2nd round. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2117645 | To be noted. The MSD value 3.3dB will be captured in the revision of R4-2118706. |
| R4-2118706 | To be revised. To capture 3.3dB MSD for band 79/n79. |

Discussion on 2nd round (if applicable)

Open issues summary

Companies views’ collection for 2st round

Open issues

| Company | Comments |
| --- | --- |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
| Revised WID: Band combinations for concurrent operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X PC5 band | CATT | Some new band combinations are requested so that the WID needs to be updated. This revised WID is for post-meeting email approval. |
| TR 37.875, Band combinations for con-current operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X PC5 band | CATT | TP approved in this meeting should be implemented in TR 37.875. |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2117645 | Calculation of MSD for n79 for V2X_n79A-n47A and V2X_n79A-47A and accompanying TP | Qualcomm Incorporated | To be noted. | The MSD value 3.3 will be captured in the revision of R4-2118706. |
| R4-2118706 | TP for TR 37.875 on MSD for V2X_n79A-n47A and V2X_n79A_47A | Huawei, HiSilicon | To be revised. | To capture 3.3dB MSD for band 79/n79. |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)