3GPP TSG-RAN WG4 Meeting #101-e 													R4-2119714

Electronic Meeting, November 01-12, 2021

Agenda item:			7.29

Source:	Moderator (China Telecom)

Title:	Email discussion summary for [101-e][114] NR_PC2_SUL_CA_lowMSD

Document for:	Information

Introduction

This discussion summary will cover three agendas:

7.29 High power UE (power class 2) for NR inter-band Carrier Aggregation with 2 bands downlink and 2 bands uplink

7.31 Power Class 2 UE for NR inter-band CA and SUL configurations with x (x>2) bands DL and y (y=1, 2) bands UL

7.5.3 Feasibility study of defining “low MSD” for CA and DC

According to the contributions submitted, this discussion summary will focus on the following topics:

- [7.29] Topic#1: [NR_PC2_CA_R17_2BDL_2BUL]

  - Sub-topic 1-1: Improvement for PC2 combos indication in the spec

  - Sub-topic 1-2: TP/draft CR to introduce UE requirements for combos

  - Sub-topic 1-3: Corrections

- [7.31] Topic#2: [NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL]

  - Sub-topic 2-1: MSD requirements for CAs

  - Sub-topic 2-2: TPs/draft CRs to introduce UE requirements for combos

- [7.5.3] Topic #3: Feasibility study of defining “low MSD”

  - Sub-topic 3-1: Objectives for feasibility study

  - Sub-topic 3-2: Work manner on Low MSD in Rel-18

Note that the tables for collecting comments for sub-topic issues are arranged just below each issue...

[7.29] Topic #1: [NR_PC2_CA_R17_2BDL_2BUL]

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations/Abstracts |
| --- | --- | --- |
| R4-2117747 | Xiaomi | Draft CR to 38.101-1: exceptions for out-of-band blocking for high power UE inter-band CA |
| R4-2118174 | ZTE Corporation | Observation 1: It is not clear which power allocations of each band in the existing MSD tables of receiver harmonic and cross band isolation issues for PC2 NR inter-band CA combination. Also, no distinguish for PC3 or PC2 NR inter-band CA combination for harmonic MSD requirements. Observation 2: Same UL configurations table is applied for the same band combinations configured for PC2 and PC3 Observation 3: For PC2 FDD+TDD NR inter-band CA combination, same PC3 harmonic MSD requirements are applied for the power allocations of 23dBm+23dBm and 23dBm+26dBm Observation 4: For PC2 NR inter-band CA combination, in case of PC3 within both two NR bands (i.e. aggressor UL band is PC3 band), same PC3 harmonic mixing MSD requirements are applied, and in case of PC2 within at least one of the NR TDD band (i.e. at least one of the aggressor UL band is PC2 band), the new harmonic mixing MSD requirement should be defined. Observation 5: For PC2 FDD+TDD NR inter-band CA combination, new 2UL intermodulation MSD should be defined for Case a (23dBm+23dBm) and Case b (23dBm+26dBm), and same requirements are applied for these two cases. Observation 6: For PC2 NR inter-band CA combination, in case of PC3 within both two NR bands (i.e. aggressor UL band is PC3 band), same PC3 cross-band isolation MSD requirements are applied, and in case of PC2 within at least one of the NR TDD band (i.e. at least one of the aggressor UL band is PC2 band), the new cross-band isolation MSD requirement should be defined. |
| R4-2118175 | ZTE Corporation | CR to TS38.101-1: Correction on MSD table to apply PC2 NR inter-band CA combination |
| R4-2119371 | T-Mobile USA | Draft CR for 38.101-1: PC2 Corrections and improvements |
| R4-2119372 | T-Mobile USA | Draft CR for 38.101-1: PC2 and PC1.5 combinations |
| R4-2119433 | Qualcomm Incorporated | Correction to uplink Tx power for PC2 2UL CA MSD |
| R4-2119475 | AT&T | DraftCR 38.101-1 Corrections for NR CA 2DL PC2 MOP Band-edge Relaxations |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1: Improvement for PC2 combos indication in the spec

Issue 1-1-1: Improvement for PC2 combos indication in the spec

Draft CR in R4-2119371 proposed to

- Proposal1: Capture the PC2/PC1.5 information in the existing CA configuration tables in type of footnote in order to avoid creating new tables or greatly expanding the size of existing tables

- Proposal2: Remove the Table 6.2A.1.3-2 as it no longer needed if proposal 1 is agreed.

Recommended WF: Collect views on the draft CR R4-2119371.

| Company | Comments on Issue 1-1-1: Improvement for PC2 combos indication in the spec |
| --- | --- |
| ZTE | We agree the approach to further improve the PC2 combs table. As we know, 2DL CA including both 1UL and 2UL for both PC2 and PC3. But we don’t know which combinations support PC2 from the current table before, so we think it should specific the PC2 combs with both 1UL and 2UL in one table, as 9371 did. Also, we have a question, how to define the BCS for PC2 combs comparing to the corresponding PC3? |
| MTK | R4-2119371, part of transmitter output power requirement is merged into sub clause of bandwidth configuration. Uplink bandwidth class need to be shown in the table also. Another concern is, not sure if this is good way for spec table simplification. And how to indicate MOP tolerance of PC1.5 if this is merged to BW configuration table? |
| T-Mobile USA | To ZTE: We think that the same BCSs should apply to each power class. We don’t see any need to have different BCSs for different power classes. To MTK: Uplink BW class is included for UL CA. For single band UL, the UL is usually implied or -. Listing the single band for PC other than PC3 seems to work. The MOP tolerance for PC1.5 is the defined in the single band MOP table. The PC1.5 MPL does not change based o the DL combination. |
| MTK2 | Some texts of Table 6.2A.1.3-2 were not cleaned up. Note y need to say MOP and its tolerance refer to Table 6.2.1-1. With above, we are fine with the change |
| Nokia | We agree with that these changes are necessary. |
| China Telecom | We also identified the issues when captured more high order band combinations. So from simplifying the spec point, we support the changes in the CR. |
| Huawei | We support the two proposals. It seems that EN-DC has been using this footnote method, for example, “NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations.” In Table 5.5B.4.2-1. We also share the view with TMUS that BCS should not depend on power classes. The remaining question is: does the WID include PC1.5 for CA? |
| AT&T | We support the changes as it definitely simplifies the specification. However, the PC2 note is missing from some of the PC2 combinations that have already been completed. Please revise the document and add the same corresponding note and single UL config to the following combinations. FYI, the PC2 note should not be added to the n77(2A) uplink case. It should only be added to n77A and CA_nZZa-n77A UL cases. CA_n2A-n77(2A) CA_n5A-n77(2A) CA_n66A-n77(2A) CA_n30A-n77(2A) CA_n12A-n77(2A) CA_n14A-n77(2A) |
| Skyworks | We believe it is useful to capture the HPUE cases but it should be done by notes on the UL configuration so that which UL configuration it applies to is clear and not to the generic CA configuration |

Sub-topic 1-2: TP/draft CR to introduce UE requirements for combos

Issue 1-2-1: TPs/draft CRs to introduce UE requirements for combos

Proposed draft CRs:

- R4-2119372, R4-2117747

Recommended WF:

- Collect the comments for proposed TPs/CRs. If no comments for certain of TP or CR, the TP or CR will be recommended as approved.

| CR/TP number | Comments collection for Issue 1-2-1: TPs/draft CRs to introduce UE requirements for combos |
| --- | --- |
| R4-2119372 | MTK: same comment as in Issue 1-1-1 |
|  | T-Mobile USA: Same reply as in issue 1-1-1. 😊 |
|  | Huawei: Same question about PC1.5. |
| R4-2117747 | ZTE: Not sure why we need separate tables for PC2 and PC3 since PC2 are based on PC3. We are wondering if it is feasible to remove table 7.6A.3.3-1a for simplification. |
|  | Xiaomi: To ZTE, the table 7.6A.3.3-1a is not introduced by this contribution. We are agree 7.6A.3.3-1a can be merged to Table 7.6A.3.3-1 as it can be expected the band combination with exceptions allowed for PC3 also applied to PC2. We can revise our paper if no other comments is received. |
|  | MTK: We support the idea of the CR. The exception requirements can be applied to PC2 as well as PC1.5 combos also. We also agree with ZTE’s idea to merge table 7.6A.3.3-1a to Table 7.6A.3.3-1. Maybe can add a note to describe: The exception also applies to the combinations supporting PC2 or PC1.5. |
|  | T-Mobile USA: We agree with ZTE. We think that 7.6A.3.3-1a can be removed and Table 7.6A.3.3-1 relabeled: Table 7.6A.3.3-1: CA band combination with exceptions allowed |

Sub-topic 1-3: Corrections

Issue 1-3-1: Correction on the MSD

Proposed CRs:

- CR R4-2119433 Correction to uplink Tx power for PC2 2UL CA MSD

- CR R4-2118175 CR to TS38.101-1: Correction on MSD table to apply PC2 NR inter-band CA combination

Recommended WF:

- Collect the comments for proposed TPs/CRs.

| CR/TP number | Comments collection for Issue 1-3-1: Correction on the MSD |
| --- | --- |
| R4-2119433 |  |
| R4-2118175 | Nokia: we understand the motivation. But the added texts need modification since if we introduce PC1.5 or whatever, we need to modify all the relevant table captions. Apart from dual UL IMD, only the aggressors’ PC is relevant the total power class does not matter. The caption just should mention aggressors’ PC then, no revisit is required even if we introduce MSD for PC1.5 etc… |
|  | Huawei: we agree that the aggressor’s PC is the key info to be included in the captions. |
|  | Skyworks, the clarification is required for the UL configuration (if FDD+FDD or FDD+TDD) there is no ambiguity but for TDD-TDD it may be that one or both are HPUE |

Issue 1-3-2: Correction on the PC2 MOP Band-edge Relaxations

Proposed CRs

- R4-2119475 DraftCR 38.101-1 Corrections for NR CA 2DL PC2 MOP Band-edge Relaxations

Recommended WF

- Collect the comments for proposed TPs/CRs.

| CR/TP number | Comments collection for Issue 1-3-2: Correction on the PC2 MOP Band-edge Relaxations |
| --- | --- |
| R4-2119475 | ZTE: Overlapp with R4-2119371. |
|  | T-Mobile USA: To ZTE: Since they are draft CRs we could either merge could happen in the big CR. |
|  | Nokia: No issue in the content of the CR, but the content is completely covered by a CR of R4-2117094. We shared this info with the moderator before the meeting. It would be great if the moderator of this thread could share the outcome of coordination with a moderator to handle R4-2117094. |
|  | China Telecom: As moderator, I received the information but didn’t see views from source company AT&T. If the source company from AT&T is ok, I will suggest to focus on draft CR R4-2117094 as commented by Nokia which has covered this CR R4-2119475. But both of the two CRs need to be revised based on the outcome of the CR R4-2119371. Regarding to merge the outcome of the draft CR into big CR, I think it could be handled in thread 103, since no inputs from this thread to the tables that the CR R4-2117094 has touched. |
|  | AT&T: We can take the Nokia CR in R4-2119371 since it contains all of the changes in the AT&T draftCR in R4-2119475. R4-2119475 can be noted or withdrawn. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1 Improvement for PC2 combos indication in the spec | Recommended WF: Issue 1-1-1: Improvement for PC2 combos indication in the spec - The general approach to capture the PC2/PC1.5 information in the existing CA configuration tables by footnote is agreed. Corresponding draft CR R4-2119371: Draft CR for 38.101-1: PC2 Corrections and improvements is recommended to be revised to capture the comments. |
| Sub-topic 1-2: TP/draft CR to introduce UE requirements for combos | Recommended WF: Issue 1-2-1: TPs/draft CRs to introduce UE requirements for combos - For draft CR R4-2119372: Draft CR for 38.101-1: PC2 and PC1.5 combinations, it shares same story as mentioned in issue 1-1-1, thus the draft CR will be revised. - For draft CR R4-2117747: Draft CR to 38.101-1: exceptions for out-of-band blocking for high power UE inter-band CA, it will be revised according to the comments. |
| Sub-topic 1-3: Corrections | Recommended WF: Issue 1-3-1: Correction on the MSD - For CR R4-2119433 Correction to uplink Tx power for PC2 2UL CA MSD, since no comments, the CR is recommended as endorsed. Then it will be merged into the big CR. - For CR R4-2118175 CR to TS38.101-1: Correction on MSD table to apply PC2 NR inter-band CA combination, it will be revised according to the comments. Issue 1-3-2: Correction on the PC2 MOP Band-edge Relaxations - The R4-2119475 DraftCR 38.101-1 Corrections for NR CA 2DL PC2 MOP Band-edge Relaxations is merged by R4-2119371 in thread 103, thus it is recommended as noted. |

Discussion on 2nd round (if applicable)

Summary for 2nd round

[7.31] Topic #2: [NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL]

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations/Abstracts |
| --- | --- | --- |
| R4-2117106 | Qualcomm Incorporated | Use MSD values as shown in Sections 2.1 to 2.12 |
| R4-2117251 | AT&T | TP for TR 38.842 Addition of CA_n2-n5-n77 |
| R4-2117252 | AT&T | TP for TR 38.842 Addition of CA_n2-n12-n77 |
| R4-2117253 | AT&T | TP for TR 38.842 Addition of CA_n2-n14-n77 |
| R4-2117254 | AT&T | TP for TR 38.842 Addition of CA_n2-n30-n77 |
| R4-2117255 | AT&T | TP for TR 38.842 Addition of CA_n5-n12-n77 |
| R4-2117256 | AT&T | TP for TR 38.842 Addition of CA_n5-n14-n77 |
| R4-2117257 | AT&T | TP for TR 38.842 Addition of CA_n5-n30-n77 |
| R4-2117258 | AT&T | TP for TR 38.842 Addition of CA_n12-n30-n77 |
| R4-2117259 | AT&T | TP for TR 38.842 Addition of CA_n12-n66-n77 |
| R4-2117260 | AT&T | TP for TR 38.842 Addition of CA_n14-n30-n77 |
| R4-2117261 | AT&T | TP for TR 38.842 Addition of CA_n14-n66-n77 |
| R4-2117262 | AT&T | TP for TR 38.842 Addition of CA_n30-n66-n77 |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1: MSD requirements

Issue 2-1-1: MSD requirements for CAs

Proposal:

- Use MSD values as shown in Sections 2.1 to 2.12 in R4-2117106.

Recommended WF:

- As the MSD requirements have been captured in TPs, it is suggested to discuss based on the corresponding TPs in Issue 2-2-2.

Sub-topic 2-2: TPs/draft CRs to introduce UE requirements for combos

Issue 2-2-1: TPs/draft CRs to introduce UE requirements for combos

Proposed TPs:

- R4-2117251, R4-2117252, R4-2117253, R4-2117254, R4-2117255, R4-2117256, R4-2117257, R4-2117258, R4-2117259, R4-2117260, R4-2117261, R4-2117262

Recommended WF:

- Collect the comments for proposed TPs/CRs. If no comments for certain of TP or CR, the TP or CR will be recommended as approved.

| CR/TP number | Comments collection for Issue 2-2-1: TPs/draft CRs to introduce UE requirements for combos |
| --- | --- |
| R4-2117251 |  |
| R4-2117252 |  |
| R4-2117253 |  |
| R4-2117254 |  |
| R4-2117255 | Huawei: The MSD is much lower than the one for a similar band combo DC_13A_n5A-n77A. Please clarify. |
|  | AT&T: The PC2 MSD is based on the QC contribution in R4-2117106. I cannot comment on the DC_13A_n5A-n77A case as to why the MSD is higher. Maybe, the MSD for DC_13A_n5A-n77A needs to be revisited. Perhaps, QC may be able to comment. |
| R4-2117256 | Huawei: The MSD is much lower than the one for a similar band combo DC_13A_n5A-n77A. Please clarify. |
|  | AT&T: The PC2 MSD is based on the QC contribution in R4-2117106. I cannot comment on the DC_13A_n5A-n77A case as to why the MSD is higher. Maybe, the MSD for DC_13A_n5A-n77A needs to be revisited. Perhaps, QC may be able to comment. |
| R4-2117257 |  |
| R4-2117258 |  |
| R4-2117259 |  |
| R4-2117260 |  |
| R4-2117261 |  |
| R4-2117262 |  |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 2-2: TPs/draft CRs to introduce UE requirements for combos | Recommended WF: Issue 2-2-1: TPs/draft CRs to introduce UE requirements for combos - There are 12TPs: R4-2117251, R4-2117252, R4-2117253, R4-2117254, R4-2117255, R4-2117256, R4-2117257, R4-2117258, R4-2117259, R4-2117260, R4-2117261, R4-2117262, in which two TPs R4-2117255 and R4-2117256 received comments, thus they are recommended as revised. The others are recommended as approved. |

Discussion on 2nd round (if applicable)

Summary for 2nd round

[8.35] Topic #3: UE RF requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations/Abstracts |
| --- | --- | --- |
| R4-2117035 | Nokia, Nokia Shanghai Bell | Observation 1: At least more than 20 dB MSD improvement is feasible for some devices in some band combinations. Observation 2: Still some more discussion is needed to clarify relationship between following aspects. - The amount of the expected improvement of MSD - Source of MSD such as harmonics, IMD2/3/4/5, cross band isolation and harmonic mixing - Per band combination, per source of MSD, or per UE Observation 3: Following band combinations are example ones. - CA_n3-n77 for harmonics, IMDs and harmonic mixing - CA_n41-n77 for cross band isolation Observation 4: A signalling fixed delta value per band combination from the specified MSD would cause at least following issues and delta may not have to refer to the specified MSD but rather refer to reference sensitivity. - A suitable delta value for a certain specified MSD may not be suitable for the other specified MSDs for the same band combination. Observation 5: Need to discuss if the information on the amount of the improved MSD should be reported per source of MSD, per band combination and/or per UE. Observation 6: It would be realistic to handle Low MSD topic in a Rel-18 WI dedicated to this topic. |
| R4-2117750 | Xiaomi | Observation 1: many affects should be studied when trying to get a reasonable value on MSD improvement. Observation 2: for the low SNR condition (e.g. in the cell edge), only few dB sensitivity degradation would cause large impact on throughput. Observation 3: the sensitivity degradation for UE in the cell center is much lower than when it in the cell edge due to the Tx power difference even for UE with very large MSD. Observation 4: if UE is under a high SNR condition, the impact on throughput can be small even with a high sensitivity degradation. Observation 5: Even for the UE having high MSD issue, the impact on throughput can be small when it is in the cell center. Observation 6: it is meaningful and worth to study on how to treat UEs with high MSD dynamically by considering actual Tx power range. Proposal: the dynamical sensitivity degradation (the actual desense) caused by Tx link for UE in a cell should be considered when dealing with the high MSD band combination issue. |
| R4-2118287 | vivo | Proposal 1: RAN4 focus on study and reach a set of objectives as main target. Proposal 2: UE implemetation should be taken into account into the ”Low MSD” work. Proposal 3: More understanding of introducing ”Low MSD” for the network is needed. Proposal 4: A SI is proposed, and WI can be followed and release independency is possible. Proposal 5: SI objectives can use previous task to RAN4 as starting point, and at least add the analysis of network behaviour and possible benefits. |
| R4-2118548 | Huawei,HiSilicon | Observation 1: The existing MSD specifications are obtained based on largely agreed RF assumptions. And a modern smartphone needs to support a large number of band combinations within a small form factor. Proposal 1: The objectives of the SI should include how to improve the MSD and how much can be improved, potentially for different types of IMDs. Observation 2: The UE self-interference from IMD/harmonics varies with Tx power levels and carrier frequency configurations. Proposal 2: The objectives of the SI should include how the network can benefit from MSD improvement, taking into account the dynamics of UE self-interference. Proposal 3: The objectives of the SI should include the impact to conformance tests. Any performance gain should be verifiable. Proposal 4: RAN4 should be mindful about the potential outcome of the SI and try to avoid overloading the group with a large number of band combinations that have MSD re-evaluation requested. |
| R4-2118890 | OPPO | Proposal: It is proposed to close the MSD improvement discussion in Rel-17, and further consider in Rel-18 SI/WI package with a more focused and clear objectives. |
| R4-2119375 | T-Mobile USA | Observation 1: MSD in real world devices is often non-existent, even for NR CA combinations with high MSD in the specs. Observation 2: Even in commercial UEs with discrete RFFE we are seeing little to no MSD even for combinations with high MSD in the specs. Observation 3: Because real world UEs perform much better than the MSD specs would indicate, operator RAN4 delegates are like The Boy Who Cried Wolf, warning about potential issues that don’t show up in the real world. Observation 4: Operators seem to have the following options for dealing with the issue of high MSD: - Continue to ignore MSD - Require that UEs have zero MSD for all combinations, regardless of the specs - Avoid the use of combinations with high MSD - Require RAN vendors to implement intelligent schedulers to schedule around potential MSD issues - Push for optional low MSD signalling in the specs - Pursue dynamic real time feedback on self-interference via the proposed IDC WID - Propose a new capability signalling mechanism to allow a UE to declare actual MSD for each problematic band combination. Proposal 1: Discuss the options and decide how to proceed Proposal 2: Vendors are requested to describe how MSD tests are tested today. Proposal 3: RAN4 should discussion how MSD should be tested. Proposal 4: RAN4 could consider adding new UE capability signalling or actual MSD measured in a device. Proposal 5: RAN4 should recommend that the MSD issue be formally part of the study phase of a Work Item or a separate Study Item so that objectives can be identified, and work focused. |
| R4-2119438 | Qualcomm Incorporated | Proposal 1: Low MSD would be an additional optional specification fo the UE per band combination. Not all UE’s are required or expected to meet it as it is proposed to be met based on a signaled capability. Proposal 2: Define thresholds for the UE to report; i.e., [10] dB better than minimum requirement, [20] dB better than minimum requirement, etc. Proposal 3: The signaling of low MSD is applicable to both PC2 and PC3. Whether the same value or separate values are used per power class is TBD. Proposal 4: Release independence should be applied to the earliest release available subject to signaling constraints in RAN2. |
| R4-2117986 | Apple | Proposal: 3GPP to Introduce UE signal to self-interference ratio measurement mechanism for assisting network scheduling on band combinations subject to MSD impact. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3-1: Objectives for feasibility study

Issue 3-1-1: Feasible method to improve MSD

Proposal:

- Option1: Define low MSD by considering one or more cases such as per band combination, per source of MSD, per UE

  - Dynamical sensitivity degradation/UE self-interference, MSD improvement threshold/granularity would be considered when studying to derive the specific improvement values.

- Option2: Define low MSD by a generic optional approach which applies to all combos intended to improve MSD.

Recommended WF:

- Collect views from companies.

| Company | Comments on Issue 3-1-1: Feasible method to improve MSD |
| --- | --- |
| OPPO | The improved MSD discussion was for the band combinations with interference issues, not for all combinations. Therefore if low MSD is to be further studied, it should be per band combination, maybe per souse of MSD also, i.e. Option 1. However, our observation on this topic is that it probably hard to move forward without being included in a clear objective in a WID/SID. Suggest to consider it in Rel-18 package. |
| LGE | In RAN #94 pre-discussion, RAN4 made some stable agreement to add the SI/WI on lower MSD as follow - Investigate and introduce lower MSD for inter-band CA/EN-DC combinations (RAN4) ◾ [FFS example band combinations ◾ Study how the MSD performance can be improved for example band combiantions ◾ Study whether the generic approach can be applied based on UE indication of capability ◾ Study and if possible specify the requirements and/or capability signaling ◾ Power class 2 (PC2) is considered] NOTE: for the lower MSD, the above objecives are subject to modifications based on RAN4 progress Based on this, RAN4 can further study the feasibility in Rel-18 and can close this issue in Rel-17. |
| ZTE | We believe there exits lots of approaches to reduce the MSD, which are implementation dependent. But we are interested of a generic option approach which appies to all combs. Also, we may need to discuss and define more realistic parameters for the MSD calculation, otherwise, more and more band combination would be defined with high MSD in the spec. Also, we agree it can be further discussed in Rel-18 and close it in Rel-17. |
| Xiaomi | From UE implementation of view, it is hard to define a generic optional lower MSD for all problematic band combination since the factors that affect MSD value are not the same. However, if case by case manner is considered, the workload would be very high. It is better only very limited band combination is considered. From performance point of view, it is more meaningful to study on how to treat UEs with high MSD dynamically by considering actual Tx power range as shown in our paper 7750 |
| MTK | We share Xiaomi’s view on “it is hard to define a generic optional lower MSD for all problematic band combination since the factors that affect MSD value are not the same” Case by case study may be better with above reason. The calculation assumption shall be same for all MSD mechanisms for the same combo as we pointed out before. Work load is not an issue. All MSD values in the spec were come out case by case analysis. If there are too many combos, RAN4 may need discuss how to prioritize them. |
| Qualcomm | A generic approach that can be applicable for all combos for which MSD needs to be improved is preferred. A case-by-case approach is too time consuming to be practical due to the large number of combinations and expected number of requests. |
| vivo | Option 1. Our proposal is to consider different band combination and also different types of MSD, since they could be quite different. |
| Sony | We think a dynamically MSD/interference mechanism might be a good solution eventually as it provides better indication of UE performance in real life. However, we share the same view as other companies and think it might be more feasible to discuss the signaling in Rel-18. |
| T-Mobile USA | We agree with Qualcomm that a general approach would be best because a case by case approach will be too time consuming. We also agree with Sony that a dynamically MSD/interference mechanism might be a good solution eventually as it provides better indication of UE performance in real life. Our concern with waiting for Rel-18 is that we don’t want to have to wait for Rel-18 ASN.1 to add new signalling. Hopefully the signalling can be made in Rel-17 and usable in earlier releases. |
| Apple | CA/DC MSD defined in current specifications represents the worst-case REFSENS degradation scenario. It would not be surprised that in real world the devices are performing better than specified. We felt the sympathy that those numbers on paper do not look pretty. But we would like to understand whether those high MSD numbers are really impacting the network operation for those combinations. We agree with Xiaomi that MSD can be affected by many different factors and it would not be technically sound to apply a constant delta dB to all band combinations to represent a “low MSD” UE capability. In our view, the main concern on the network side is that there was no clear indication whether UL interference would be impacting the DL signal SNR at different operating conditions. If a UE signal-to-interference ratio (SIR) detection mechanism as proposed in R4-2117986 can be developed, network can base on the UE reported SIR to schedule the MSD affected combinations more effectively and efficiently, including those combinations with high MSD. |
| Nokia | None of the options (Two options handed exclusively where one of them suddenly include different aspects together which can be applicable to the other option so that it is not possible to select one of them…) Regarding dynamic MSD reporting, this would be handled by RAN2 led WI or at least we need to discuss if the RAN4 is the main place to discuss this approach since the amount of MSD that UE can achieve under the specified test condition is not to main point of the discussion for dynamic approach. This approach is not related to the original motivation of Low MSD since it is on-site handling based on the real time MSD. The original motivation was even if UEs indicate the same capability supporting the same band configuration, their performance in terms of MSD is totally different. Hence, if a network aggressively configures UEs with CA, in some cases, the resources may be in vain due to MSD, but in some other cases, if a network conservatively configures UEs with CA, some UEs lose opportunity to enjoy CA. To avoid this situation, if a network can differentiate the inherent difference in advance, the network would optimize its scheduling not to cause MSD as much as possible while to make maximum use of CA as much as possible based on the information on the difference. As commented in T-Mobile USA contribution, dynamic MSD reporting or SIR approach has following aspects to be considered, - It may require response time as quick as possible - Signaling overhead - UEs and NWs may need to exchange messages quite often specifically if the information on what to do next is not provided. Concerning the option 2, we are not sure if we understand the option 2 or not. If the signaling is per band combination or not needs more discussion. At least the capability signaling should be optional. |
| Huawei | It’s too early to decide which option to use. It should be part of the feasibility study of the SI. |
| AT&T | We prefer the more generic approach as proposed by Qualcomm. We think that per source of MSD may be a good way to consider but we can also look at per combo type (e.g. Low-mid-high, etc.). We use the same approach to justify MSD values for similar combo types when specifying the minimum performance specs. We also agree with T-Mobile that any signalling solution should apply to earlier releases. |
| Skyworks | Given that agreeing improved MSD value will be a long process we suggest it is only done based on request and for cases with high MSDs, not all possible BC/MSD. There is no issue in a network for MSD <10dB and good UEs will perform well. Independently to how improved MSD is capture (an offset, an absolute value….) it should only apply to cases with high values (>15dB?) and improved value should not be 0dB. such applicability threshold and floor value (3dB) may help cover cases with multiple MSD issues |
| CHTTL | We agree with Qualcomm that a general approach would be best because a case by case approach will be too time consuming. We prefer the low MSD approach in 7035/9438. We are not sure what is the dynamically MSD/interference mechanism mentioned here. We think Xiami’s proposal is on how to deal with the UE with large MSD performance, which is quite different from the original intention of low MSD. In our understanding, the network can deal with this issue by scheduling, but the network don’t know which UE is actually with low MSD and can schedule more aggressively. |

Issue 3-1-2: Capability signaling necessity study

Proposal:

- Option1: Capability signaling is necessary for network to distinguish UE with different MSD

  - Release independence manner will be applied

- Option2: No need to report capability as system benefit for network is limited or not clear

Recommended WF:

- Collect views from companies.

| Company | Comments on Issue 3-1-2: Capability signaling necessity |
| --- | --- |
| OPPO | Whether capability signaling will benefit the system performance is pending on further study, and it is not clear how the NW can do with the capability of different MSD in the NW. Suggest to consider it in Rel-18 package for thorough study. |
| LGE | Currently, RAN4 not guaranteed the benefit and usage in gNB perspectives when RAN4 specify the lower MSD capability signaling from UE. So RAN4 firstly clearly understand the usage and benefits of the capability signaling. Based on this RAN4 can request to define the capability signaling to RAN2 in Rel-18. |
| ZTE | We tend to agree with the capability signalling, but we are open to discuss the purpose and the benefit which pending on the approaches discussed in issue 3-1-1. |
| Xiaomi | If only static lower MSD is indicated by signaling, it is not clear how the NW can be benefit from this signaling. However, we are open do discuss this. |
| MTK | We prefer option 1 if MSD is re-evaluated or different/improved MSD comes in later release. |
| Qualcomm | Some kind of capability signaling is needed since not all UE’s may be able to achieve the improvement across all band combinations. |
| Vivo | The benefit of new signaling is unclear, further study is needed. |
| Sony | We think its bit early to discuss the capability. |
| T-Mobile USA | If we go with a real-time dynamic signalling approach, then we may need other new signalling, and also maybe UE capability signalling to indicate if the UE supports the new dynamic signalling. |
| Apple | Option 2: No need to report capability as system benefit for network is limited or not clear. As we have commented several times, MSD represents the REFSENS degradation under the worst-case scenario, the number should not be used to determine how the MSD affected combinations should be scheduled. |
| Nokia | Option 1, but we can discuss release independent manner later. |
| Huawei | Again it’s too early to decide. |
| AT&T | Option 1. |
| Skyworks | In our understanding the main issue for the network is whether high MSD cases need to have specific scheduling. In that sense improved MSD signaling does not help anyhow only the understanding of the condition under which high MSD may/may not happen helps. |
| CHTTL | Support option 1. |

Issue 3-1-3: Example band combinations proposed

It has been mentioned in several contributions to include example combos to specify MSD if feasible. Thus this issue3-1-3 will collect the combos proposed which suffer from large MSD and is intended to improve MSD.

Comments on Issue 3-1-3: Example band combinations proposal

| Company | Band combinations | MSD value | Source to induce large MSD(H2, IMD2,3,4, etc) | Any comments |
| --- | --- | --- | --- | --- |

Sub-topic 3-2: Work manner on Low MSD

Issue 3-2-1: Work manner on Low MSD in Rel-18

Proposal:

- Option1: SI

- Option2: WI with study phase

- Option3: WI

Recommended WF:

- Collect views from companies.

| Company | Comments on Issue 3-2-1: Work manner on Low MSD in Rel-18 |
| --- | --- |
| OPPO | Pending on RAN discussion including the scope and contents. Premature to decide on SI or WI. |
| LGE | Prefer option 1 in Rel-18. Need to feasibility study how to achieve the lower MSD values and what is the benefit of the capability signaling? |
| ZTE | Prefer Option 1 in Rel-18. We may need to study the approaches on reduction of MSD value first. |
| Xiaomi | Share the same view with OPPO, it is pending on the scope and objective. |
| MTK | If there’s consensus on sub-topic 3-1, it is ok for option 3. If there’s no consensus on sub-topic 3-1, option 1 is better that RAN4 continue study/discuss improved MSD in R-18 |
| Qualcomm | This should be a RAN plenary discussion, but one thing for RAN4 to consider is release independence. Since we envision this to be capability-based, we suggest to make this release independent as early as signaling allows. |
| vivo | Prefer option 1. |
| Sony | Same view as OPPO and Xiaomi, it is up to RAN discussion. |
| T-Mobile USA | We don’t have a strong view on WI vs. SI vs. WI with a study phase. Obviously it is up to RAN plenary, but we will be happy to have clear objectives agreed to. |
| Nokia | If something dynamic approach is included, we need to discuss if the WI or SI is RAN2 led or RAN4 led since IDC WI is led by RAN2 and/or the amount of MSD that UE can achieve under the specified test condition is not to main point of the discussion for dynamic approach. |
| Huawei | Option 1. |
| AT&T | We prefer either Option 2 or Option 3. |
| Skyworks | This is a RAN discussion based on what objectives are agreed |
| CHTTL | We also think it depends on the scope and objective. But in general we prefer option 2/3. |

Issue 3-2-2: UE SIR measurement for network scheduling assistance

There is one contribution R4-2117986 proposed

- UE SIR measurement to assistant network scheduling subject to MSD impact

Recommended WF:

- It is not sure if SIR measurement could work in parallel with Low MSD or Low MSD is no longer needed. So further clarification from Apple and views from companies are encouraged.

| Company | Comments on Issue 3-2-2: UE SIR measurement for network scheduling assistance |
| --- | --- |
| OPPO | In our understanding, the SIR measurement is similar as the dynamic MSD reporting, both to indicate the real time interference conditions. It is different from the static MSD reporting, i.e. NW only consider to configure band combination to UE with low MSD. Both method can be considered in Rel-18 time frame. |
| LGE | It is quite different issues between lower MSD and SIR measurement for NW scheduling assistance. The conformance test aspect the MSD can be proved in link level. But the SIR measurement issues will complex to setup the system levels environments with the changed the SIR in time domain. So we do not prefer to add the scope in the lower MSD SI/WI. |
| Xiaomi | We support to discuss the similar dynamic MSD reporting in the SI phase. |
| MTK | Similar view with OPPO. SIR is dynamic reporting while MSD in the spec is static requirement. There’s no need to study SIR since this is highly UE implementation dependent. UE SIR measurement to assistant network scheduling is not relevant to MSD improvement that is under RAN4 discussion. |
| Qualcomm | As others have commented, the two ideas are separate. |
| vivo | Since it is the first time for such a scheme to be shared, propose to have further discussion in the future. |
| Sony | Similar understanding as OPPO, which the UE SIR measurement reporting is similar to dynamic MSD reporting. As we commented in issue 3-1-1, we are happy to see a solution can provide a dynamic indication of UE real time performance, but it may need to be discussed in Rel-18. |
| T-Mobile USA | We agree that this SIR approach is similar to dynamic MSD reporting, and is different than static low MSD reporting. We think that both approaches can be studied. |
| Nokia | The same view on dynamic MSD report mentioned in issue 3-1-1 applies to UE SIR measurement approach. |
| Huawei | Very interesting idea. Just wonder if the proposed DL-SIR measurement procedure can be more accurate or bring more gain than the UE simply estimate the SIR based on real-time Tx power as well as IMD type etc, i.e. the exercise we do for MSD calculation. |
| AT&T | We are OK to study but this would greatly increase the level of effort and there may not be available time units to include both aspects. We would want to prioritize the static low MSD signalling approach if there is only room for one. |
| Skyworks | Since the only issue that needs solving in the network (low MSD is operating well in network whatever) is whether the scheduler needs to treat a UE in a different way scheduling assistance is the right solution as especially it can distinguish the case where SNR is bad only due to interference limited scenario and not self-interference. |
| CHTTL | We think the proposed SIR approach might be a RAN2 led work… And we have concern on deactivating the UL tranmission will have impact on the performance. Also the scheme is time consuming, and the signalling overhead might be an issue. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 3-1: Objectives for feasibility study | Issue 3-1-1: Feasible method to improve MSD Companies standpoints on this issue: - Option 1(non-general): OPPO, Xiaomi, MTK, vivo, Skyworks - Option 2(general): Qualcomm, T-Mobile USA, ZTE, AT&T, CHTTL - Other options such as dynamic MSD or SIR: Sony, Apple Recommended WF: - Continue discuss in 2nd Round. Issue 3-1-2: Capability signaling necessity study Companies standpoints on this issue: - Option 1(signaling): ZTE, MTK, Qualcomm, T-Mobile USA, Nokia, AT&T, CHTTL - Option 2(no signaling): Apple, Skyworks - Other views such as further study or consider it later (OPPO, LGE , Xiaomi, MTK, vivo, Sony, Huawei) Recommended WF: - Continue discuss in 2nd Round. Issue 3-1-3: Example band combinations proposal Recommended WF: - Closed as no inputs. |
| Sub-topic 3-2: Work manner on Low MSD | Issue 3-2-1: Work manner on Low MSD in Rel-18 Companies standpoints on this issue: - Option1(SI in Rel-18): LGE, ZTE, MTK, vivo, Huawei - Option2(WI with study phase): AT&T, CHTTL - Option3(WI): MTK, T-Mobile USA, AT&T, CHTTL - Other views such as pending on RAN discussion: OPPO, Xiaomi, Sony, Qualcomm, T-Mobile USA, Skyworks Recommended WF: - As majority companies think this issue depends on RAN discussion, it is recommended to stop the discussion in 2nd round. Issue 3-2-2: UE SIR measurement for network scheduling assistance Companies standpoints on this issue: - SIR measurement is separate from low MSD. (Majority views) - SIR measurement can be further discussed. Recommended WF: - Continue discuss in 2nd Round. |

Discussion on 2nd round (if applicable)

Summary for 2nd round

|  | Status summary |
| --- | --- |

Recommendations for Tdocs

1st round

[7.29] Topic #1: [NR_PC2_CA_R17_2BDL_2BUL]

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2117747 | Draft CR to 38.101-1 R17 exceptions for out-of-band blocking for high power UE inter-band CA | Xiaomi | revised |  |
| R4-2118174 | On the MSD requirements for PC2 and PC3 NR inter-band CA | ZTE Corporation | noted |  |
| R4-2118175 | CR to TS38.101-1: Correction on MSD table to apply PC2 NR inter-band CA combination | ZTE Corporation | revised |  |
| R4-2119371 | Draft CR for 38.101-1: PC2 Corrections and improvements | T-Mobile USA | revised |  |
| R4-2119372 | Draft CR for 38.101-1: PC2 and PC1.5 combinations | T-Mobile USA | revised |  |
| R4-2119433 | Correction to uplink Tx power for PC2 2UL CA MSD | Qualcomm Incorporated | Endorsed | Will be merged into the big CR. |
| R4-2119475 | DraftCR 38.101-1 Corrections for NR CA 2DL PC2 MOP Band-edge Relaxations | AT&T | noted |  |

[7.31] Topic #2: [NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL]

New tdocs

|  | Title | Source | Comments |
| --- | --- | --- | --- |
| #1 | Revised WID on Rel-17 Power Class 2 UE for NR inter-band CA/DC with and without SUL configurations with x (6>=x>2) bands DL and y (y=1, 2) bands UL | Huawei, HiSilicon, China Unicom | Based on rapporteur’s request , the revised WID is for email approval. |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2117106 | PC2 MSD NRCA 3DL 2UL for TR 38.842 | Qualcomm Incorporated | noted |  |
| R4-2117251 | TP for TR 38.842 Addition of CA_n2-n5-n77 | AT&T | Endorsed |  |
| R4-2117252 | TP for TR 38.842 Addition of CA_n2-n12-n77 | AT&T | Endorsed |  |
| R4-2117253 | TP for TR 38.842 Addition of CA_n2-n14-n77 | AT&T | Endorsed |  |
| R4-2117254 | TP for TR 38.842 Addition of CA_n2-n30-n77 | AT&T | Endorsed |  |
| R4-2117255 | TP for TR 38.842 Addition of CA_n5-n12-n77 | AT&T | revised |  |
| R4-2117256 | TP for TR 38.842 Addition of CA_n5-n14-n77 | AT&T | revised |  |
| R4-2117257 | TP for TR 38.842 Addition of CA_n5-n30-n77 | AT&T | Endorsed |  |
| R4-2117258 | TP for TR 38.842 Addition of CA_n12-n30-n77 | AT&T | Endorsed |  |
| R4-2117259 | TP for TR 38.842 Addition of CA_n12-n66-n77 | AT&T | Endorsed |  |
| R4-2117260 | TP for TR 38.842 Addition of CA_n14-n30-n77 | AT&T | Endorsed |  |
| R4-2117261 | TP for TR 38.842 Addition of CA_n14-n66-n77 | AT&T | Endorsed |  |
| R4-2117262 | TP for TR 38.842 Addition of CA_n30-n66-n77 | AT&T | Endorsed |  |

[7.5.3] Topic #3: Feasibility study of defining “low MSD”

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2117035 | Issues to be addressed in Low MSD WI | Nokia, Nokia Shanghai Bell | noted |  |
| R4-2117750 | Discussion on MSD improvement for inter-band CA or DC | Xiaomi | noted |  |
| R4-2118287 | Discussion on "Low MSD" for CA and DC | vivo | noted |  |
| R4-2118548 | Further discussion on low MSD feasibility | Huawei,HiSilicon | noted |  |
| R4-2118890 | R17 MSD improvement | OPPO | noted |  |
| R4-2119375 | MSD Issue: One operator's perspective | T-Mobile USA | noted |  |
| R4-2119438 | Proposals on capability-based MSD for CA and DC combinations | Qualcomm Incorporated | noted |  |
| R4-2117986 | UE SIR measurement for network scheduling assistance | Apple | noted |  |

2nd round