3GPP TSG-RAN WG4 Meeting # 101-e 	R4-2119773

# Electronic Meeting, 1st – 12th November 2021

Source:		Verizon, Nokia, Samsung

Title:			TP for TR 37.717-11-21 to include DC_2-5_n66-n77

Agenda item:		7.18.2

Document for:		Approval

# Introduction

This contribution is a text proposal for TR 37.717-11-21 to include DC_2-5_n66-n77 configuration according to the request in [1] and the approved test proposals [2].

- Reference

[1] RP-211750:	Revised WID on Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)

[2] TPs for TR 37.717-11-21

- R4-2016315:	TP for TR 37.717-11-21 to include DC_2A_n66A-n77A, DC_2A-2A_n66A-n77A

- R4-2112657:	TP for 37.717-11-21 for DC_5_n66-n77

# Text Proposal

<Start of Text Proposal>

## 7.x 	DC_2-5_n66-n77

### 7.x.1	Operating bands for DC

Table 7.x.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-5_n66-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.x.2	Configuration for DC

Table 7.x.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_5A_n66A DC_5A_n77A | CA_2A-5A | CA_n66A-n77A |

### 7.x.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

### 7.x.4	∆TIB and ∆RIB values

For DC_2-5_n66-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.x.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n66-n77 | 2 | 0.6 |
|  | 5 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 7.x.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5_n66-n77 | 2 | 0.3 |
|  | 5 | 0.2 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

### 7.x.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

<End of Text Proposal>