3GPP TSG-RAN WG4 Meeting #101-e	R4-2119962

Electronic Meeting, Nov. 1 – 12, 2021

Agenda Item:	8.4.3 and 8.4.6.3

Source:	Apple

Title:	WF on FR2 enhancement part 2: UL gaps

Document for:	Approval

# 1	Tx power management: RF aspect

Summary of 1st round email discussion is captured in [1].

Agreement:

- The minimum Delta EIRP gain is at least max((Ppeak_EIRP-[21]dBm-margin) + 10*log10(Z/20), 3dB), where the margin is 2dB, where

  - Ppeak_EIRP is peak EIPR with zero MPR

  - Margin is the implementation margin including false alarm and etc

  - Z is duty cycle in number of percentage for reference measurement channel, e.g. when UL duty cycle is 10%, Z=10.

Agreement:

- UE will report P-MPRgapon when UL gap is activated

  - At most UE should report 0~3dB P-MPR in the PHR

- UE supports UL gap shall also support R16 MPE reporting at least when UL gap is activated.

- It is FFS about P-MPRgapoff reporting when UL gap is not activated and the related delta P-MPR, i.e. (P-MPRgapoff -P-MPRgapon).

# 2	UL Tx power management: RRM aspect

Summary of 1st round email discussion is captured in [1].

Agreements:

- In R17, the following 4 gap configurations are introduced

  - The same requirements should be specified for all gap configurations

  - UGL indicates the number of consecutive static UL slots configured as UL gap per UGRP

|  | UGL [ms] | UGRP [ms] | UGL/UGRP |
| --- | --- | --- | --- |
| ULGP #0 | 1.0 | 20 | 5% |
| ULGP #1 | 1.0 | 40 | 2.5% |
| ULGP #2 | 0.5 | 160 | ~0.31% |
| ULGP #3 | 0.125 | 5 | 2.5% |

Agreements:

- UL gaps are configured by the network using RRC configuration.

- UL gaps are deconfigured by the network using RRC configuration.

- Related to activation and deactivation of UL gaps:

- The UL gaps can be activated when configured (using RRC signalling).

- FFS: The UL gaps can additionally and optionally be activated and deactivated using MAC command after UL gap is configured by RRC Signaling

- The UL gaps are deactivated when deconfigured (using RRC signalling).

Agreements:

With the agreement that UE can explicitly indicate to NW on “need for UL gap” and “no need for UL gap”, RAN4 will leave the detailed signaling design to RAN2

Agreements:

- Once the UL gap is configured, and activated, the UE will perform BPS sensing only in the static UL slot, i.e. no DL slot or special slot will be used as UL gap. For example, in case of ULGP#1, DDDSU configuration with 120KHz SCS, the following figure depicts the available slots for UL gap.

- UE is not expected to be scheduled with UL transmission during the gap. Details FFS, e.g., RACH

Agreements:

- No interruption across FR. FR2 UL gap does not cause FR1 interruption.

Agreements:

UL gap slot is static UL slot. UL gap slot applies same TA as other UL slots.

Agreements:

No conflict b/w TDD config and UL gap. UL gap is always configured in static UL slot

Agreements:

FFS the impact on measurement gap configuration and requirement due to UL gap.

# 3	UL coherent MIMO

Summary of 1st round email discussion is captured in [1].

Agreements:

The gain and feasibility of approach 2 can be confirmed.

- Approach 2

Gap occupying multiple symbols should be adjacent to PUSCH transmission, as shown in the following figure, and UE can adjust the phase difference between last transmitted SRS and PUSCH transmission. Requirements to be tested can be the same as the gap for BPS. The gap depends on side conditions including DRX on, BWP switching, SRS switching, DL measurement gap, etc. Once the side condition occurs, the gap should be triggered to detect the phase error before PUSCH transmission. If no side conditions, periodically triggered gap is not necessary. It is also worth noting that, with this case, UE need to know the PUSCH transmission phase status in advance, the compensation accuracy would rely on more calibration work.

Such gap pattern are just examples for coherence calibration, it still needs RAN4 to further study in phase II.

Agreements:

- Details of the relative power error and relative phase errors and their measurements can be added as appropriate.

# References

[1] R4-211xxxx, “Email discussion summary for [101-e][121] NR_RF_FR2_req_enh2_Part_2”, Apple

[2] GTW_(Draft) WF_[121]_FR2_UL_gap_after-GTW-Session

[3] [121]_(Draft) R4-2119962 - WF_121_FR2_RF_UL_gap_QC_after-GTW-session.docx