3GPP TSG-RAN WG4 Meeting # 101-e 												R4-2119964

Electronic Meeting, Nov. 1st – 12th, 2021

Agenda item:			8.4.4

Source:	vivo

Title:	WF on DC-Location

Document for:	Approval

- 1st round GTW Agreements

Issue 1-1-1: Further clarification of candidate default DC locations

Agreement: Introducing the concept “frequency component” depends on UE declaration, which is a per band configuration capability.

Issue 1-1-2: Further clarification of “in the middle” for Contiguous CA

Agreement: Mathematical center of the UE bandwidth rounded to the subcarrier grid defined for the component carrier on which the DC is located.

Issue 1-1-3: Further clarification of “in the middle” for NC CA

Agreement: If the center of the UE bandwidth lands on frequencies where there is no sub carrier grid defined, the nearest lower frequency component carrier subcarrier grid shall be extended to cover the frequency of the mathematical DC location.

Issue 1-2-1 & 1-2-2 “static” & “dynamic” reporting

Agreement:

- for DC location, there is no need to further discuss the definitions of “static” and “dynamic”

- Further discussion on the DC location reporting with default DC and offset

Issue 1-2-3: What is the offset granularity?

Agreement: for offset granularity, the smallest SCS in the configured or activated CCs/BWPs

Issue 1-2-5: Confirm DC location reporting framework accommodates signalling a location outside configured frequency spectrum.

Agreement: Confirm DC location reporting framework accommodates signalling a location outside configured frequency spectrum.

- 2nd round Agreements (For Discussion)

(New) Issue 1. Trigger conditions of DC offset reporting

Proposal 1: NW aware the default DC location is wrong;

Proposal 2: When affecting factor changes;

Proposal 3: Multiple possibilities including proposal 1 & 2;

Proposal 4: Others.

Proposal 5: No need to discuss in RAN4.

Conclusion:

Depending on the discussion of the outgoing LS.

(New) Issue 2. Work Split between RAN2 and RAN4 on the trigger conditions of DC offset reporting

Option 1: RAN4 provides detailed guidance to RAN2;

Option 2: RAN4 provides general/tentative information to RAN2;

Option 3: RAN4 doesn’t need to provide this information.

Conclusion:

Depending on the discussion of the outgoing LS.

Issue 1-2-4: What is the offset range?

- Proposals

  - Option 1: restrict UE DC offset ranges for example +/-20MHz from the default DC location

  - Option 2: Largest offset is 1.5 GHz i.e. +/- 25000 sub carriers in a 60 kHz grid.

  - Option 3: Others.

- Recommended WF

  - TBA

Conclusion:

FFS.

(New) Issue 3 (Based on Issue 1-3-3). Whether multiple DC locations for multiple PA/panel case would impact Signalling and how to inform RAN2 if needed?

Conclusion:

Depending on the discussion of the outgoing LS.