3GPP TSG-WG RAN4 Meeting #101-e	R4-2120022

Electronic meeting, November 1-12, 2021

Agenda Item: 9.4.1

WF on Optimisations of Pi/2 BPSK UL Power in NR

Huawei, HiSilicon, Qualcomm, Intel

# Background

References:

[1] R4-2119736 Summary_136_1st_Round, Moderator (Qualcomm), RAN4#101-e

[2] R4-2117471 Receiver performance for pi/2 BPSK with spectral shaping, Nokia, Nokia Shanghai Bell

[3] R4-2117472 Shaping filter characteristics including transmitter and link performance, Nokia, Nokia Shanghai Bell

[4] R4-2117473 Transmitter performance for pi/2 BPSK with spectral shaping, Nokia, Nokia Shanghai Bell

[5] R4-2117474 Identify potential changes for RAN4 requirements, Nokia, Nokia Shanghai Bell

[6] R4-2117610 Link level evaluations of pi/2 BPSK with spectrum shaping, Indian Institute of Tech (H)

[7] R4-2117968 Considerations and simulation results for pi/2 BPSK, Apple

[8] R4-2118043 Views on Pi/2 BPSK pulse shaping, Intel Corporation

[9] R4-2118044 Pi/2 BPSK pulse shaping link-level performance, Intel Corporation

[10] R4-2118078 Power measurements of pi/2 BPSK waveform with spectrum shaping, Indian Institute of Tech (H)

[11] R4-2118549 Discussion on BPSK Tx Power for PC2 UEs, Huawei, HiSilicon

[12] R4-2119486 PC2 Pi_2 BPSK measurements, Skyworks Solutions Inc., Nokia

# Output power capability of Pi/2 BPSK DFT-s-OFDM signals

Agreements

- Power boost based on 1Tx PC2 PAs

  - Option 1: power boost ≤ 1 dB

  - Option 2: power boost ≤ 2 dB

  - Option 3: power boost ≤ 3 dB

- RAN4 is to determine the absolute max output power in future meetings

  - The impact of higher PA output power levels on the RF-FE components technology should be considered, including thermal dissipation, peak power impact on filters, couplers, power detectors, antenna switches, peak current impact on DC-DC converters, etc.

  - Consider Tx signal placement restrictions within a given BW for maximising power boost

- UE handhelds with PC2 as baseline, other power classes are not precluded.

# MPR Study

Agreements

- Update 1Tx PC2 MPR table to support power enhancement for Pi/2 BPSK signals

  - The exact modifications are left for the WI stage

  - Companies are encouraged to bring proposals in future meetings

- The RB regions may be reclassified to optimise the power enhancement for Pi/2 BPSK signals

  - Companies are encouraged to bring proposals in future meetings

# Transparent spectral shaping

Agreements

- Both data and DMRS are filtered

  - Option 1: The choice of filters is up to UE implementations and transparent to the network

  - Option 2: The filters are explicitly specified

  - Option 3: The filtering is signalled

- For optimal performances, UE may apply different spectral shaping filters for different RB allocations

# Spectral flatness requirements

Agreements

- Revisit the requirements based on other decisions

  - Option 1: Maintain current Rel-16 specification

  - Option 2: Tighten the flatness requirement for #PRB<16

- A significant net gain should be observed if the spectral flatness requirements are to be changed

# Use of Net Gain for Evaluating Candidate Filters

Agreements

- Net gain that combines both the transmitter and receiver performances should be the deciding criterion for filter evaluation