3GPP TSG-RAN WG4 Meeting # 101-e	 												R4-2120033

Electronic Meeting, 1-12 November, 2021

Agenda item:			7.25.2.2

Source:	Qualcomm

Title:	WF on adding 100 MHz channel BW in NR-U bands n46 and n96

Document for:	Approval

# Topic #3: NR-U bands n46 and n96 – 100 MHz channel BW

### Sub-topic 3-1: channelization

Issue 3-1-1: Channelization for 100MHz for n46

- Proposals: Following table captures the different proposals.

| Nominal channel freq (MHz) | Charter, CableLabs, HPE, Comcast | Qualcomm | Huawei | Skyworks |
| --- | --- | --- | --- | --- |
| 5200 | Y | Y | Y | (DL only) |
| 5220 |  | Y |  | Y |
| 5240 |  | Y |  |  |
| 5260 |  | Y |  |  |
| 5280 |  | Y |  |  |
| 5300 | Y | Y | Y | (DL only) |
| 5520 | Y | Y | Y | (DL only) |
| 5540 |  | Y | Y | Y |
| 5560 |  | Y | Y |  |
| 5580 |  | Y | Y |  |
| 5600 |  | Y |  |  |
| 5620 |  | Y | Y |  |
| 5640 |  | Y | Y |  |
| 5660 |  | Y | Y |  |
| 5680 |  | Y | Y |  |
| 5785 |  | Y | Y | Y |
| 5805 |  | Y |  |  |
| 5825 |  | Y |  |  |
| 5845 |  | Y |  |  |
| 5865 | Y | Y | Y | (DL only) |

Issue 3-1-2: Channelization for 100MHz for n96

Agreement

- All companies commented they could accept Charter’s proposal. The following list is agreeable: 5995, 6055, 6155, 6215, 6315, 6375, 6475, 6535, 6635, 6695, 6795, 6855, 6955, 7015, 7035, 7055, 7075.

### Sub-topic 3-2: SEM

Issue 3-2-1: SEM for triple puncture

- Proposals: For interior triple puncture (depending on agreement on issue XX) the SEM should be

  - Option1: floor at -25 dBr (Huawei, ZTE)

  - Option2: -28dBr at 10MHz from the edges (CableLabs, Charter)

  - Other

### Sub-topic 3-3: Others

Issue 3-3-1: MPR

Agreement

- Existing PC5 MPR values can be applied to 100 MHz channel BW as well.

Issue 3-3-2: A-MPR

Agreement

- A-MPR values for NS_54

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | See Table 6.2F.2-1 | ≤ [2.5] | ≤ [5.0] |
|  | 16 QAM |  | ≤ [3.0] | ≤ [5.0] |
|  | 64 QAM |  | ≤ [3.5] | ≤ [5.0] |
|  | 256 QAM |  | ≤ [5.0] | ≤ [6.0] |
| CP-OFDM | QPSK |  | ≤ [4.5] | ≤ [6.0] |
|  | 16 QAM |  | ≤ [4.5] | ≤ [6.0] |
|  | 64 QAM |  | ≤ [5.5] | ≤ [6.0] |
|  | 256 QAM |  | ≤ [7.0] | ≤ [7.0] |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], 80 MHz channels centered at the nearest NR-ARFCN corresponding to [5985 MHz], and 100 MHz channels centered at the nearest NR-ARFCN corresponding to [5995 MHz and 6055 MHz].. |  |  |  |  |

- A-MPR values for NS_53

| Pre-coding | Modulation |
| --- | --- |
|  |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ [2.5] | ≤ [4.5] |
|  | 16 QAM | ≤ [2.5] | ≤ [4.5] |
|  | 64 QAM | ≤ [3.5] | ≤ [4.5] |
|  | 256 QAM | ≤ [5.0] | ≤ [5.5] |
| CP-OFDM | QPSK | ≤ [4.0] | ≤ [4.5] |
|  | 16 QAM | ≤ [4.0] | ≤ [4.5] |
|  | 64 QAM | ≤ [5.5] | ≤ [5.5] |
|  | 256 QAM | ≤ [7.0] | ≤ [7.0] |

Issue 3-3-3: Intra-carrier guard band

- Proposals: Intra-carrier GB configuration

  - Option 1 (ZTE)

| CBW | SCS [KHz] | GB@80MHz | GB@20MHz | Intra-band carrier PRB configuration |
| --- | --- | --- | --- | --- |
| 100MHz | 30 | 845KHz | 805KHz | 6PRB(51~56) 5PRB(107~111) 6PRB(162~167) 6PRB(218~223) |
|  | 60 | 1370KHz | 970KHz | 4PRB(25~28) 3PRB(53~55) 4PRB(80~83) 4PRB(108~111) |
| Note: PRB index is starting from 1. |  |  |  |  |

  - Option 2 (Apple)

    - For the 30kHz SCS, consider a smaller center bin which will result in same intra-carrier guard bands between all the bins: 50-6-50-6-49-6-50-6-50

    - For the 60kHz SCS, the guard bands may look as presented below. We already have 5RB intra-carrier guard band for the 80MHz channel, so the following pattern is aligned with patterns for smaller channels: 23-5-23-5-23-5-23-5-23