3GPP TSG-RAN WG4 Meeting #101-e	R4-211xxxx

e-Meeting, Nov 2021

Title:	WF on ETC exemptions in existing version of Rel-15 of 38.101-2

Source:	Qualcomm Incorporated

# Background

The relevant recorded agreements, as captured in UERF session meeting notes from RAN4#89, are provided below:

# WF – ETC exemptions Case 1 of core requirements in Rel-15 of TS 38.101-2

- Sub-clause 6.3.4.1: The requirements on power control accuracy apply under normal conditions

- Sub-clause 6.4.2.1: EVM requirements are specified over Normal operating conditions

  - Exemption applies to dependent requirements (6.4.2.4 EVM equalizer spectrum flatness)

| Company | Comments |
| --- | --- |
| Huawei | We think it has no ambiguity that the above requirements with exemption are specified over NTC |

# WF – ETC exemptions Case 2 of core requirements in rel-15 of TS 38.101-2

- Sub-clause 6.2.1.x (table note): The requirements for UE EIRP spherical coverage are verified only under normal temperature conditions as defined in Annex E.2.1

  - Exemption also applies to dependent requirements (6.6 Beam Correspondence)

- Sub-clause 7.3.4.x (table note): The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in Annex E.2.1

- This item is to be discussed further. This statement conflicts with the principle of the core requirements. Any changes in the temperature conditions when verifying the requirements in the list of Case 1 and Case 2 exemptions incur impact on the corresponding core requirements

- Options to establish common understanding for the future:

  - (Moved to option a: it is relevant to the completion of the spec per option a, rather than a replacement to the section that aims to understand what the core requirement currently says)

  - Spherical coverage core requirements were based on NTC-only proposals from some companies, therefore:

    - Wording in existing tables shall be changed to ‘requirements in this table apply only over NTC’ and current wording about verification shall be removed

    - RAN4 should study and quantify the deltas in the performance between NTC and ETC; once these gaps are identified, then RAN4 should discuss how to complete the core requirements

    - For compliance with Annex E.2.1, specification shall be completed to include requirements over ETC, based on updated proposals

  - Spherical coverage core requirements apply over ETC

    - Wording in existing table shall be changed to ‘The requirements for … spherical coverage are applicable over ETC but shall be verified only under normal temperature conditions as defined in Annex E.2.1’

  - (this option just maintains the ambiguity that this WF is trying to resolve)

| Company | Comments |
| --- | --- |
| Huawei | Based on the GTW discussion, companies have different understanding for the core requirements listed above are specified or verified under NTC condition. To avoid further confusion, make changes of the WF to case 1 and case 2. Thanks Qualcomm for giving the history discussion background. Based on the discussion, it can be seen that companies have concern of the beam directions under ETC and NTC, consequently, the spherical coverage requirements are specified based on inputs under NTC and noted that the side conditions for Beam Correspondence are also defined under NTC. So we disagree that these requirements are applicable for ETC but verified over NTC. For option a, we don't understand why ii) sub-bullet is needed. Option a) is just the same as requirements in case 1. We think this sub-bullet if needed, should be added to option b) instead. Also add an additional option, i.e. keep the requirements unchanged in the specification. |
| OPPO | Agree with adding Option C |
| Apple | To help with the historical perspective, we added the discussion captured in the RAN4 #89 meeting from the Chair's notes which directly preceeded the agreement of the CR on spherical coverage EIS. This discussion provides context to the compromise taken on EIS spherical coverage and clarifies that the "verified under NTC" note was motivated by all companies' willingness to conclude the topic while acknoledging that form factor de-sense (due to extreme temperature conditions) is a factor which wass not considered in the agreed requirement. We think RAN4 should study and quantify the deltas in the above core requirements between NTC and ETC; once these gaps are identified, then RAN4 should disucss how to handle the differences in the core requirements. One option is to define additional requirements under ETC (as an example, EVM equalizer flatness contains separate tables for NTC and ETC). Another option is to recommend to RAN5 to incorporate the delta into the test tolerance. In our understanding, only after RAN4 concludes the "homework" related to quantifying these deltas and agrees how they can be handled in core requirements, can RAN5 proceed with an expansion of ETC applicability to more test cases beyond the set they already verify under ETC. In the WF body we've included the key points of this commentary to help progress toward an agreement. |
| Qualcomm | To Huawei, how is your option b doing anything to clarify the understanding? To Apple; it would be better to first understand what the standard is saying. In our view there is no agreement to substantiate your view that core requirements do not apply over ETC. Your example of form-factor desense is an interesting example. Would you provide a reference to any measured data or detailed explanation of this mechanism? |
| Keysight | Our understanding from core specification was as per original option 3b/ new option 4b (Spherical coverage requirement apply over ETC). However, we are open to trade-off as per original option 3a (i+ii) or similar option with new wording under new option 4a (all sub-options). It should be pointed out that spherical coverage results could be leveraged from the beam peak search procedures and hence potentially not adding additional testing time. Regarding Huawei’s proposal on Keeping core requirements as they are, we think that core specification will remain incomplete on Case 2 extreme conditions requirements. Regarding leaving the measurement issue to RAN5, as outlined before, there is no measurement issue as FR2 ETC spherical coverage have been concluded as feasible and can be obtained as part of the beam peak search under ETC. |
| Sony | We share similar understanding as Qualcomm that the core requirement applied under ETC here but with verification exemption, based on our interpretation of the specification. However, consider the view from companies are still diverge and we are closing to the end of this meeting, we suggest to simply put everything under WF3 to be FFS, e.g., “FFS on ETC exemptions Case 2 of core requirements in rel-15 of TS 38.101-2….” In overall, we support this WF and think it is necessary to resolve the ambiguity. Actually, the fact that different companies have different views here demonstrates the importance of this discussion. |