3GPP TSG-RAN WG4 Meeting #101-e	R4-2120241

Electronic Meeting, 16 – 27 August, 2021

Agenda item:			4.1.6

Source:	Huawei, HiSilicon

Title:	WF on Rel-15 NR RRM core requirements maintenance

Document for:	Approval

Introduction

This document captures the agreements as well as open issues and corresponding options for Rel-15 NR RRM core requirements maintenance.

Way Forward

- Issue 1: Clarification for SSB and SMTC periodicity

Common understanding:

It is an error case if the SSB transmission periodicity is greater than the configured SMTC for the same carrier.

Candidate options:

- Option 1

  - Add the following clarification or similar to the spec (e.g. clause 3.6)

    - Requirements in this specification assume that SSB transmission periodicity shall not be greater than the configured SMTC for the same carrier.

- Option 2

  - Clarification in option 1 not needed based on common understanding

- Issue 2: Applicable DRX cycle for inter-frequency and inter-RAT measurement in NE-DC and NR-DC

Candidate options:

- Option 1

  - In NR-DC mode and NE-DC mode, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the longer DRX cycle between master cell group and secondary cell group.

- Option 2

  - Other principles than using longer DRX cycle between CGs (as in option 1) should be used.

- Option 3

  - Other options clarifying the applicable DRX cycle

- Issue 3: Time period before cell selection in Idle mode

Note: this issue is about the time period before UE entering cell selection in Idle mode, which is currently defined in clause 4.2.2.2 of 38.133

If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information for 10 s, the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1].

Candidate options:

- Option 1

  - Introduce the max function between the fixed value 10s and [K1]*N1* DRX cycles, where N1 is defined in Table 4.2.2.2-1, and K1 is FFS

    - Option 1-1: from Rel-15

    - Option 1-2: from Rel-17

- Option 2

  - No change is needed