3GPP TSG-RAN WG4 Meeting #101-e	R4-2120266

Electronic Meeting, 16 – 27 August, 2021

Agenda item:			5.1.4.1

Source:	Huawei, HiSilicon

Title:	WF on NR Positioning UE PRS measurement requirements maintenance

Document for:	Approval

Introduction

This document captures the agreements as well as open issues and corresponding options for UE PRS measurement requirements, based on email discussion summary R4-2120351.

- Topic #1: Maintenance issues for positioning measurement requirements

- Sub-topic 1-1: Common issues

- Issue 1-1-1: Applicability condition for option-1 muting

- Update the definition of Nmuting

  - If bitmap b1  for higher-layer parameter DL-PRS-MutingPattern is provided, then Nmuting=TmutingPRS*L where L is the length of the bitmap; otherwise Nmuting=1.

- No need to define applicability condition for option-1 muting, based on the updated definition

- Issue 1-1-2: Consideration of RSTD search window

- Take into account expected RSTD and expected RSTD uncertainty in defining overlap between PRS resource and MG.

- Issue 1-1-3: Requirements with HO during measurement period

Add a general statement that if CSSF changes during the measurement period, the measurement period could be longer

Replace current HO requirements with a general statement that measurement period can be longer when HO occurs, and the measurement accuracy requirements shall be met

- Sub-topic 1-2: UE Rx-Tx specific issues

- Issue 1-2-1: UE behavior and requirements when proximity condition is not met

Candidate options:

- Proposals for UE behaviour

  - Option 1 (ZTE, vivo, HW, Intel, CATT, OPPO)

    - UE should measure and report the UE Rx-Tx time difference

  - Option 2 (QC, Ericsson)

    - Up to UE implementation whether to transmit the UE Rx-Tx measurement results to LMF

- Proposals for requirements

  - Option 1a (CATT, OPPO, HW, Intel, OPPO, vivo)

    - Measurement period requirements shall apply

    - Measurement accuracy requirements are not applied

  - Option 1b (ZTE, vivo)

    - Both measurement period and measurement accuracy requirements shall apply

  - Option 2 (QC, Ericsson)

    - UE Rx-Tx time difference measurement requirements, including measurement period requirements and measurement accuracy requirements, do not apply.

- Issue 1-2-2: Measurement period requirements with cell change impacting SRS

When PSCell or SCell addition or release causes SRS reconfiguration during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete.

FFS: When SRS is reconfigured without cell change during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete.

- Issue 1-2-3: Measurement period requirements with cell change not impacting SRS

When PSCell or SCell addition or release does not causes SRS reconfiguration during the measurement period, UE continues the UE Rx-Tx time difference measurement, and the measurement period requirements apply.