3GPP TSG-RAN WG4 Meeting #101-e		R4-2120316

E-meeting, November 01-12, 2021

Agenda item:	8.16.7

Source:	Qualcomm

Title:	WF on NR extension to 71 GHz RRM requirements (Part 1)

Document for:	Approval

# Introduction

In this contribution we capture the agreements achieved during the email discussion on the topic Rel-17 NR ext. to 71GHz RRM core requirements (AI 8.16.7) in RAN4#101-e under email thread [101-e][228] NR_ext_to_71GHz_RRM_1. This email thread covers following sub-agenda items:

- AI 8.16.7.1	General and RRM requirements inpacts

- AI 8.16.7.2	Timing requirements

The details on the discussion can be found at the [228] thread discussion summary [1].

# Way forward

Deployment scenarios

- To clarify the scope of RRM, RRM requirements for Ext_to_71GHz should be defined for the scenarios below in Rel-17.

  - FR2-2 single carrier and CA in SA

  - FR1+FR2-2 CA (FR1 is PCell)

  - FR1+FR2-2 DC (FR1 is PCell)

  - Other scenarios are precluded in Rel-17.

- Applicability of inter-band CA requirements for FR2-2

  - RAN4 agreed not to define requirements for inter-band CA within FR2-2 as currently there is only one agreed band in FR2-2.

    - The agreement can be revised if more bands agreed in RF

Rx beam sweeping scaling factor

- Wait for more agreements from RF session before deciding the Rx beam scaling factor for FR2-2.

CGI identification and SFTD measurements

- De-prioritizing CGI identification and SFTD measurement requirements for Rel-17 FR2-2

CR Split

- The following list of Draft CRs to be prepared by RAN4 is proposed:

  - Draft CR for timing requirements for FR2-2 – UE transmit timing and Timing Advance – Nokia

  - Draft CR for timing requirements for FR2-2 – MRTD, MTTD - Qualcomm

  - Draft CR for interruption requirements for FR2-2 - vivo

  - Draft CR for active BWP switching delay requirements for FR2-2 - Huawei

  - Draft CR for general measurement requirements for FR2-2 - Ericsson

  - Additional Draft CR(s) shall be considered based on the outcome of open issues

    - Scheduling restriction – Mediatek

- Whenever the FR2 is referred, both FR2-1 and FR2-2 frequency sub-ranges shall be considered in this release, unless otherwise stated.

- The designations FR2-1 and FR2-2 should only be used when needed to differentiate the requirements between the two FRs

- FFS: How to adapt the existing terminology (FR2) when corresponding requirements are not defined for FR2-2 (e.g. inter band CA scenarios in FR2)

Scheduling restrictions

- Scheduling restrictions due to beam switching time to continue waiting for the conclusion about beam switching time from RF session

- Assumptions on deriveSSB-IndexFromCell

  - Do not change cell phase synchronization accuracy

  - For 120kHz SCS

    - deriveSSB-IndexFromCell is always enabled

  - For 480kHz and 960kHz SCS

    - Option 1: deriveSSB-IndexFromCell is always enabled

      - Option 1A: revisit UE assumptions on the maximum timing difference between the reference cell and target cell from 2 SSB symbols to a larger value

      - Option 1B: introduce [1-3] symbols scheduling restriction before and/or after SSB transmission

    - Option 2: deriveSSB-IndexFromCell may be either enabled or not enabled and up to network configuration

    - FFS whether to have different approaches for licensed/unlicensed bands

UL Timing accuracy requirements

- Basic principles

  - FFS: Choose Te such that the condition TCP - TCH  - 2 ( Te + TAC,Q /2 ) > 0 holds

  - FFS: When defining the margin for the Te calculation, discuss the values for maximum RMS channel delay spread for 480 kHz SCS, and 960 kHz SCS

  - FFS: Cases for which the UE cannot meet the Te requirements derived in the abovementioned manner.

- Percentage of UL CP length Te can occupy

  - Option 1: 40% - 50%

  - Option 2: 30% - 40%

  - Option 3: <30%

- SSB and UL SCS combinations

  - RAN4 to specify UL timing accuracy requirements for the following (SSB SCS, UL SCS) combinations

    - 120, 120

    - 480, 480

    - 960, 960

  - FFS whether to define requirements for

    - 120, 960

    - 120, 480

    - 480, 960

    - Other options with SSB SCS > UL SCS

- NTA Offset

  - Current FR2 value for NTA offset 13792*TC is applicable for NR operation in 52.6 – 71 GHz range

- FFS: whether TRS can be used for RRC connected Te requirements

- Availability of SSB

  - For UL SCS of 480/960 kHz, a UE is required to meet the UL timing accuracy requirements if an SSB is available in the last X ms.

    - FFS: [X = 20ms], [X = 40ms], [X=TBD]

- Initial transmit timing accuracy test

  - RAN4 to discuss if the initial transmit timing accuracy test for 480/960kHz SCS should be defined as of statistical nature during the RRM performance part of the work

MRTD/MTTD

- Basic principles – FFS

- Propagation delay

  - FFS: Assumptions on propagation delay for FR2-2

- MRTD for intra-band non-contiguous CA

  - Wait for conclusions on TAE before defining the MRTD requirement for intra-band non-contiguous NR CA within FR2-2

- MRTD for FR2-2 inter-band CA

  - No requirements needed unless more bands are introduced in RF room

- MRTD for FR1 and FR2-2 inter-band CA – FFS

- MRTD for FR1 and FR2-2 NR DC

  - FFS – Inter-band synchronous NR DC between FR1 and FR2-2

    - Option 1: Use the existing requirements for FR1 and FR2-1 MRTD requirements

    - Option 2: Specify shorter MRTD requirements

    - Option 3: Wait for conclusions on TAE requirements

  - FFS – Inter-band asynchronous NR DC between FR1 and FR2-2

Measurement procedures

- Measurement capability

  - As a starting point, reuse FR2-1 measurement capability of number of cells and beams for FR2-2.

- FFS - Cell detection, PBCH detection for SSB index acquisition, SSB measurements

  - Channel model to be considered for FR2-2

    - Option 1: Channel model from RAN1

    - Option 2: FR2-1 channel models

      - Further discuss whether additional samples are needed

# References

- R4-2120370, Email discussion summary for [101-e][228] NR_ext_to_71GHz_RRM_2, Qualcomm, RAN4 #101-e, November 01-12, 2021