3GPP TSG-RAN WG4 Meeting # 101-e                                                              R4-2120343

Electronic Meeting, November 1-12, 2021

Title:	Reply LS on RRM relaxation in power saving

Response to:	R2-2108877/R4-2117015

Release:	Release 16

Work Item:	NR_UE_pow_sav-Core

Source:	RAN4

To:	RAN2

Cc:	-

Contact person:	Name: Yanze Fu

E-mail Address: fuyanze@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1	Overall description

RAN4 would like to thank RAN2 for the reply LS (R2-2108877) on RRM relaxation in power saving. RAN4 has discussed the question in 2) of the reply LS

1) RAN2 agreed to follow the request from RAN4 for the change to 38.304 on RRM relaxation on inter-frequency layers/E-UTRA inter-RAT frequency layers of higher priority for the case when the lowMobilityEvalutation criterion is fulfilled and when highPriorityMeasRelax is configured, referring to clauses 4.2.2.10.2 and 4.2.2.11.2 in 38.133. 2) RAN2 also noted that in RAN4 specifications, for the case when both lowMobilityEvalutation and cellEdgeEvaluation criteria are fulfilled, referring to clauses 4.2.2.10.4 and 4.2.2.11.4 in 38.133, the UE shall search for inter-frequency layers/E-UTRA inter-RAT frequency layers at least every hour (for example, below extract for inter-frequency layers): ##### 4.2.2.10.4 Measurements for UE fulfilling low mobility and not-at-cell edge criterion This clause contains requirements for measurements on inter-frequency NR cells provided that: - T331 timer is not running for EMR measurements on inter-frequency NR carrier, and - UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criterion, and - Has also fulfilled both criteria, and - less than 1 hour have passed since measurements for cell reselection were last performed In this case the UE is not required to meet Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter as defined in Table 4.2.2.4-1.. Given the above includes the case when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ this would result, for frequency layers of higher priority, in a less relaxed measurement requirement (1 hour measurement interval) compared with the case when only the lowMobilityEvalutation criterion is fulfilled (1 hour * Nlayers measurement interval), RAN2 kindly asks RAN4 whether this change (1 hour -> 1 hour * Nlayers) should also apply to the case when both lowMobilityEvalutation and cellEdgeEvaluation criteria are fulfilled, for consistency, or if RAN2 misinterpreted RAN4 specifications.

The view from RAN4 is described as below:

For the case when UE is configured with both lowMobilityEvaluation and cellEdgeEvaluation criteria and has also fulfilled both criteria, the change (1 hour -> 1 hour * Nlayers) should also be applied only when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ for higher priority search specified in clauses 4.2.2.10.4 and 4.2.2.11.4 in 38.133.

2	Actions

To RAN WG2

ACTION: 	RAN4 respectfully asks RAN2 to consider the above in their future work.

3	Dates of next TSG-RAN WG4 meetings

TSG-RAN4 #101-bis-e	Jan 17 - Jan 25, 2022	Electronic Meeting

TSG-RAN4 #102-e	Feb 21 - Mar 03, 2022	Electronic Meeting