3GPP TSG-RAN WG4 Meeting # 101-e 												R4-2120599

# Electronic Meeting, November 01-12, 2021

Agenda item:			4.1.8.1, 4.1.8.2, 4.2.4, 5.1.3.2, 5.1.6.4.1, 6.1.4, 7.36.3

Source:	Moderator (Apple)

Title:	Email discussion summary for [101-e][317] Demod_Maintenance_UE

Document for:	Information

# Introduction

In this email thread for UE Demod Maintenance, the following topics will be covered:

- Rel-15 NR and LTE UE Demodulation and CSI requirements maintenance (4.1.8.1, 4.1.8.2, 4.2.4.1)

- Rel-16 NR UE Demodulation and CSI requirements maintenance (5.1.3.2, 5.1.6.4.1)

- Rel-17 NR UE Demodulation and CSI requirements maintenance (6.1.4, 7.36.3)

# Topic #1: Rel-15 NR and LTE UE Demodulation and CSI requirements maintenance

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2118059 | Rohde & Schwarz | DraftCR: Alignment of common test parameters Change wording for “SSB postion in burst from “1” to “First SSB in Slot #0” |
| R4-2118060 | Rohde & Schwarz | Cat A CR: Alignment of common test parameters |
| R4-2118061 | Rohde & Schwarz | Cat A CR: Alignment of common test parameters |
| R4-2117995 | Intel Corporation | Draft CR on CSI reference measurement channels Type of CSI-RS for PDSCH scheduling assumptions in CSI RMC is clarified. |
| R4-2117996 | Intel Corporation | Cat A: Draft CR on CSI reference measurement channels |
| R4-2117997 | Intel Corporation | Cat A: Draft CR on CSI reference measurement channels |
| R4-2119442 | Intel Corporation | Discussion on PDSCH scheduling impact on CSI requirements Observation: we can observe that for CQI values we have very small increasing of effective code rate in slots with and without TRS Observation: we can observe that BLER performance is very close for both considered scenarios – RI test with PDSCH scheduled only on slots without TRS, PDSCH scheduled on slots with and without TRS Proposal 1: Endorse Draft CR R4-2117995. |
| R4-2119513 | Qualcomm Incorporated | Views on PDSCH grant on TRS slots for CSI Reporting Tests Observation 1: It was agreed in Rel-15 not to schedule PDSCH on slots containing CSI-RS for CSI reporting tests as captured in [1]. Observation 2: TRS is also a type of CSI-RS. Observation 3: Scheduling grant on slots containing TRS will increase the code rate on those slots and will impact CQI reporting computation. Observation 4: Whether we PDSCH is scheduled on slots containing TRS or not, will not impact PMI reporting. Proposal 1: Do not schedule PDSCH grant on slots containing TRS for CSI reporting tests. |
| R4-2119030 | Huawei, HiSilicon | Draft CR Updates to LTE V2X PSBCH requirements in 36.101 (Rel-14) - Change the sentence ” The Sidelink UE 1 is synchronized to SLSS as synchronization reference.” to “The Sidelink UE 1 transmit PSBCH to tested UE and tested UE is synchronized to SLSS of Sidelink UE 1 - Change the synchronization source of Sidelink UE 1 from SLSS to GNSS |
| R4-2119031 | Huawei, HiSilicon | Cat A: Draft CR Updates to LTE V2X PSBCH requirements in 36.101 (Rel-15) |
| R4-2119032 | Huawei, HiSilicon | Cat A: Draft CR Updates to LTE V2X PSBCH requirements in 36.101 (Rel-16) |
| R4-2119033 | Huawei, HiSilicon | Cat A: Draft CR Updates to LTE V2X PSBCH requirements in 36.101 (Rel-17) |

## Open issues summary

### Sub-topic 1-1 PDSCH Scheduling on TRS slots

It was agreed in Rel-15 not to schedule PDSCH on slots containing CSI-RS for CSI reporting tests. But it wasn’t clarified if it refers to CSI-RS for CSI acquisition, beam management and tracking.

Issue 1-1-1: For CSI reporting requirements should PDSCH be scheduled on slots with TRS

- Proposals

  - Option 1: Yes (Intel)

  - Option 2: No (Qualcomm)

  - Option 3: Align with the real TE implementation

- Recommended WF

  - Discuss further

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue 1-1-1: For CSI reporting requirements should PDSCH be scheduled on slots with TRS |
| Intel | Support Option 1. Based on our analysis for this meeting (R4-2119442), we didn’t observe any issue with scheduling of PDSCH on one slot with TRS per 20 ms. We would like to check if any issue with scheduling of PDSCH on TRS slot is observed for Rel-15 CQI or RI test cases by companies. We understand the Observation #1 and #2 from Qualcomm. Same time, we would like to note that in the Rel-15 discussion we usually used terminology CSI-RS for CSI-RS for CSI acquisition and terminology TRS for CSI-RS for tracking. And the way how the PMI FRC is defined proves such approach. |
| Qualcomm | Support Option 2. Although, our preference is not to make any changes to the existing spec. We don’t see the need to change anything in the spec. 38.101-4 Spec clearly states that no PDSCH should be scheduled on slots containing CSI-RS slots. Based on that assumption, RAN5 has already defined these tests by not scheduling grant on TRS slots, TE vendors have already implemented them and many UEs in the field have already passed those tests. We don’t see any reason to change this config and unnecessarily increase burden for revalidating those tests again. At the point when this was discussed in RAN4, companies’ understanding was that we follow reference resource settings defined by RAN1 in 38.214 because that is what UE will base its CQI reporting calculations on. RAN1 defined the reference resources in that way so that UE can assume same code rate on all slots. We should honor that agreement in RAN4 based on this technical understanding since RAN4 is defining minimum requirements. We don’t agree with Intel that CSI-RS usually just means CSI-RS for CSI acquisition. In the spec, there is no such understanding. As far as FRCs are concerned, RAN4 is still correcting some of them and this could be one of those corrections. If the intent is to align PDSCH configuration in all CSI reporting tests, we are ok to not schedule PDSCH on TRS slots in PMI reporting tests since that will not impact PMI reporting performance and doesn’t violate reference resource setting defined by RAN1. |
| Huawei | Based on the simulation results from Intel, very minor BLER performance difference between PDSCH scheduled only on slots without TRS and on slots with and without, so we can expect that it should not impact the real testing, i.e. UE pass or not the corresponding tests. The main concern should focus on the TE implementation for real testing as Qualcomm pointed out, because many Rel-15 UEs in the field have verified those tests, additional reevaluation and TE implementation updates should be avoided as much as possible. We checked the real testing setup, PDSCH is scheduled on slots with TRS for TDD 30kHz SCS, but not scheduled on slots with TRS for FDD 15kHz SCS. Maybe other companies can further check the real test setup. We added Option 3 for consideration. |
| Apple | Although there is no impact to performance, our preference is not to change the spec at this point. TRS is also a kind of CSI-RS and we should honor the agreement made in Rel-15. Since the idea is to maintain the same code-rate in all slots for CSI reporting requirements, slots with CSI-RS for acquisition, tracking and BM should not have PDSCH scheduled. |
| Anritsu | For information, our current implementation in the test equipment is following the existing note. Thus PDSCH is not scheduled at the slots which include CSI-RS (for TRS/ CSI acquisition/ beam refinement). |
| Intel2 | Thank you for comments. First, it looks like that we still have the different understanding of the Rel-15 agreement on PDSCH scheduling. @Qualcomm and Apple: If the Rel-15 CSI requirements agreement on not to schedule PDSCH in CSI-RS slots also assumes TRS slots then why PDSCH scheduling for PMI requirements does not follow this agreement? Second, we think that update of RAN5 specification is not the big issue, because most of parameters are just copy of RAN4 specification. Same time, TE implementation and understanding is rather good point. Based on feedback from HW and Anritsu, it looks like that we already have different implementations due to ambiguity of PDSCH scheduling configuration. Therefore, it looks like that if implementation of some TE vendors is not aligned with final agreement on this issue then update will be required. Probably we need to collect more comments from TE vendors to understand how we can move forward. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2118059 (R&S) | Moderator: The reason for change and consequences if not approved in the coversheet are not for the proposed change. |
|  | Qualcomm: Same comment as moderator. It is not clear why this change is needed. |
|  | Ericsson: Same comment as moderator/Qualcomm. Correction is ok. But the reason for change and correspondence if not approved in the cover sheet looks wrong. |
|  | &S: Sorry for the confusion, by mistake I uploaded the wrong version of the CR with a not updated coversheet. Intention is to align the wording for the for the SSB position in the burst across the different common test parameters in the spec to the same wording, like in e.g 5.2-1. “First SSB in slot #0” is used in all other cases in the spec, so we align to this wording. Also putting “1” there is confusing since the TCI state refers to SSB #0, so this was somewhat confusing how to interpret the “1”. A revised draft CR with the updated coversheet has been uploaded to the revised CR folder. |
|  | Huawei: We also observed the confusing “Reason for change” and “Consequences if not approved” descriptions in the coversheet. With the revised draft CR from R&S, we are ok with the udpates. |
|  | Apple: Okay with change. |
|  | Anritsu: We support the change. |
| R4-2117995 (Intel) | Qualcomm: Depends on outcome of Issue 1-1-1. |
|  | Company B: |
| R4-2119030 (Huawei) | Company A: |
|  | Company B: |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Sub-topic 1-1 PDSCH Scheduling on TRS slots Issue 1-1-1: For CSI reporting requirements should PDSCH be scheduled on slots with TRS Companies have different views on what Note 2 means in RMC tables for CSI requirements. Whether CSI-RS includes CSI-RS for CSI acquisition, BM and tracking or only CSI-RS for CSI acquisition and BM. Note 2: PDSCH is not scheduled on slots containing CSI-RS or slots which are not full DL In Rel-15 we made an agreement to not schedule PDSCH in slots CSI-RS for CSI reporting tests. The FRCs for PMI reporting requirements indicate that PDSCH is scheduled in TRS slots. The issue is whether the Note 2 in Annex A.4 should be modified to say CSI-RS for CSI acquisition and beam refinement. One company provided simulation results and analysis that code rate change is small with and without PDSCH scheduled in TRS slots and observe no significant performance delta for SNR used in requirements. Some companies have concern on updating the spec now and how it would affect the assumption used in RAN5 conformance testing for UEs already in the field. If we don’t update Note 2, should we change FRCs for PMI reporting requirements to not schedule PDSCH in TRS slots. Tentative agreements: None Candidate options: Option 1: Don’t update note 2 in CSI RMC tables and don’t update FRCs for PMI reporting requirements Option 2: Don’t update note 2 in CSI RMC tables and update FRCs for PMI reporting requirements Option 3: Update note 2 in CSI RMC tables and don’t update FRCs for PMI reporting requirements Recommendations for 2nd round: TE vendors input would be helpful to understand the configuration used for CQI and RI reporting requirements for R15. And based on their inputs we decide whether to update the note and/or FRCs. |

### CRs/TPs

Captured in Section 4.1.

## Discussion in 2nd round

Sub-topic 1-1 PDSCH Scheduling on TRS slots

Issue 1-1-1: For CSI reporting requirements should PDSCH be scheduled on slots with TRS

Request TE vendors to share the assumption used for PDSCH scheduling in TRS slots for CQI and RI reporting.

Discuss following options

Option 1: Don’t update note 2 in CSI RMC tables and don’t update FRCs for PMI reporting requirements

Option 2: Don’t update note 2 in CSI RMC tables and update FRCs for PMI reporting requirements

Option 2: Update note 2 in CSI RMC tables and don’t update FRCs for PMI reporting requirements

### Comments on Open Issues

| Company | Comments |
| --- | --- |
| XXX | Issue 1-1-1: For CSI reporting requirements should PDSCH be scheduled on slots with TRS |

### Comments on CRs/TPs

| CR/TP number | Comments collection |
| --- | --- |
| Revision of R4-2118059 (R&S) | Moderator: Company shared the revised CR with updated cover sheet and is acceptable to companies |

# Topic #2: Rel-16 NR UE Demodulation and CSI requirements maintenance

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| eMIMO |  |  |
| R4-2117428 | Apple | Draft CR to 38.101-4 on Applicability for multi-TRxP test cases-R16 Added applicability for single DCI SDM and FDM multi-TRxP requirements based on UE capability of maxNumberActiveTCI-PerBWP > n1. |
| R4-2117429 | Apple | Cat A: Draft CR to 38.101-4 on Applicability for multi-TRxP test cases-R17 |
| NR V2X |  |  |
| R4-2119041 | Huawei, HiSilicon | Draft CR Updates to NR V2X requirements in 38.101-4 (Rel-16) - Add definiation of abbreviation ”OCC” and “AGC”. - Add all the V2X tests numbers to the Test list of UE feature “Support of synchronization sources for NR sidelink” - Change the note of timeoffset to “Time offset of transmitted Sidelink UE transmit signal with respect to GNSS reference timing”. Change the note of frequency offset to “Frequency offset of transmitted Sidelink UE transmit signal with respect to GNSS reference frequency”. - For PSBCH performance test, change the sentence “The Sidelink UE 1 is synchronized to SLSS as synchronization reference.” to “The Sidelink UE 1 transmit PSBCH to tested UE and tested UE is synchronized to SLSS of Sidelink UE 1.” Change the synchronization source of Sidelink UE 1 from SLSS to GNSS - Add the unit of parameter “Allocated resource blocks” and in Table A.6.3.2 and Table A.6.4.2 and add the unit of parameter “OFDM Symbols per slot ” |
| R4-2119042 | Huawei, HiSilicon | Cat A: Draft CR Updates to NR V2X requirements in 38.101-4 (Rel-17) |

## Open issues summary

None

## Companies’ views collection for 1st round

### Open issues

None

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117428 (Apple) | Intel: At current stage we still have the same view as in the previous meeting. Taking into account that RAN1 have not defined any dedicated fields for number of supported TCIs for these schemes (in comparison to other MIMO schemes) and, also, there is no any connection between Rel-15 feature maxNumberActiveTCI-PerBWP and Rel-16 features singleDCI-SDM-scheme-Parameters-r16 and supportFDM-SchemeA-r16 in RAN1 specifications, we assume that reporting of support of certain Single DCI scheme should be sufficient for testing. If UE reports support of one of these schemes, then it means that such UE supports the two active TCI states for these multi-TRP scenarios. Same time, maxNumberActiveTCI-PerBWP feature was introduced in Rel-15 for other scenarios/purposes, and we should not mandate the signalling of capability which was not designed for considered multi-TRP scenarios. |
|  | Qualcomm: Ok with the CR. In our opinion, indicating support for single DCI schemes does not necessarily mean support of multiple TCI states. |
|  | Huawei: Based on the comments from Intel and Qualcomm, as if companies still have different understanding that whether it is mandatory for UE to support at least two active TCI states for singleDCI-SDM-scheme-Parameters-r16 and supportFDM-SchemeA-r16 operation. If still different understanding, maybe formal clarification from RAN1 is needed by LS. Without any other clarifications, the current CR updates imply that supporting two active TCI states are optional for singleDCI-SDM-scheme-Parameters-r16 and supportFDM-SchemeA-r16 operation. By checking with our RAN1 colleagues, we think that it is mandatory to support two active TCI states for s-DCI SDM and FDM operation. If companies reach common understanding that supporting two active TCI states is mandatory, then RAN4 can discuss how to capture this in the specification, based on discussions in last and this meeting, we observed the following options for discussion: For Single DCI based SDM and FDM requirements: - Option 1: Add applicability that maxNumberActiveTCI-PerBWP should be larger than n1. - Option 2: No additional applicability based on UE capability is needed. - Option 3: Add clarification in TS 38.306 that singleDCI-SDM-scheme-Parameters-r16 and supportFDM-SchemeA-r16 are applicable for UE supporting at least two active TCI states. |
|  | Apple: The testcase is configured with 2 TCI states and we need to add this capability for the tests to be applicable. To Intel, we could have other UE capabilities from Rel-15 that are needed to support the testcase. For example- maxSimultaneousResourceSetsPerCC is a UE capability from Rel-15 and is used to define applicability for mTRP requirements. Based on UE feature list we don’t see any connection between support of mTRP transmission schemes and support for maximum number of TRS resource sets per CC which the UE can track simultaneously. But we have to define this based on how the test is defined. We don’t think UE supporting 1 active TCI state can meet requirements for SDM or FDM transmission schemes, with only single DCI transmissions scheme capability. To Huawei’s suggestion, if no consensus can be reached, we are fine with seeking further clarification from RAN1 and also involving RAN2 if needed to clarify and/or update UE capability. |
|  | Intel2: Thank you for comments. We think that it would be good to check the RAN1 assumptions for this topic. Probably, we can try to focus of LS preparation next week and send LS as soon as possible. Probably, RAN1 will be able to provide response during 3GPP RAN1#107-bis-e and we will have reply for the next RAN4 meeting. |
| R4-2119041 (Huawei) | Intel: We suggest to modify the following sentence “Time offset of transmitted Sidelink UE transmit signal” to “Time offset of transmitted Sidelink UE signal” or “Time offset of Sidelink UE transmit signal”. Same comment is applied for frequency offset sentence. |
|  | Qualcomm: In our opinion, 11.1.8 should be added to both capabilities in Table 11.1.1.1.2-1 |
|  | LGE: same view with Qualcomm. And we need one clarification for Note 1 and Note 2 in Table 11.1.4.1.1-1 of PSBCH test. SLSS is used to verify PSBCH demod performance, so we think that time offset and frequency offset is based on SLSS timing (sidelinke UE 1 Tx timing). Is it correct understanding? If yes, “with respect to GNSS reference timing” of Note 1 and Note 2 should be revised to “with respect to SLSS reference timing”. |
|  | Huawei: Ok with the comments about the wording from Intel; For case 11.1.8, we think that it needs to be included only to UE capability of sync-Sidelink-r16 instead of psfch-FormatZeroSidelink-r16 considering that no PSFCH feedback is used in the test setup. @LGE the Note 1 and Note 2 in Table 11.1.4.1.1-1 of PSBCH test is for Sidelink UE 1 that is used to transmit signal to UE under test, so it should be synchronized with GNSS, but the UE under test should be synchronized with SLSS of the Sidelink UE 1. The revised draft CR is uploaded for further review. |
|  | QC: This is from 11.1.8: The probability of PSCCH miss detection is calculated as follows: ProbPSCCH miss detection = #(missing ACK/NACK)#(Tx high priority PSCCH/PSSCH) Where: - # (Tx high priority PSCCH/PSSCH) denotes the total number of transmitted PSCCH/PSSCH with high priority level. - # (missing ACK/NACK) denotes the total number of missing ACK/NACK with high priority. ACK/NACK feedback is used in this test, hence we consider the support of psfch-FormatZeroSidelink-r16 is necessary. |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic 2-1 | Issue 2-1-1: UE capability for applicability of Single DCI SDM and FDM requirements Companies have different view whether applicability based on UE capability of maxNumberActiveTCI-PerBWP larger than n1 should be added explicitly to single DCI FDM and SDM requirements for mTRP demod requirements. Some companies think that UEs supporting single DCI SDM and FDM transmission schemes for mTRP should by default support maxNumberActiveTCI-PerBWP larger than n1. While some companies think that since test case is defined for 2 active TCI states, we need to add explicit applicability since there is no prerequisite in UE capability for singleDCI-SDM-scheme-Parameters-r16 and supportFDM-SchemeA-r16 that UE should support maxNumberActiveTCI-PerBWP >1. Since we cannot reach consensus in RAN4 for 2 meetings on this issue, we plan to send LS to RAN1 to seek clarification. Tentative agreements: Send LS to RAN1 to seek clarification. Candidate options: Based on RAN1 feedback Option 1: Add applicability based on UE capability of maxNumberActiveTCI-PerBWP >1 explicitly Option 2: Request RAN1/2 to update TS38.306 to add pre-requisite to UE capability for singleDCI-SDM-scheme-Parameters-r16 and supportFDM-SchemeA-r16 that UE should support maxNumberActiveTCI-PerBWP >1. Recommendations for 2nd round: Discuss LS to RAN1 to clarify UE capability for maxNumberActiveTCI-PerBWP for supporting single DCI SDM and FDM transmission schemes for mTRP. |

### CRs/TPs

Captured in Section 4.1.

## Discussion in 2nd round

Draft and discuss LS to RAN1 to clarify UE capability for maxNumberActiveTCI-PerBWP for supporting single DCI SDM and FDM transmission schemes for mTRP.

### Comments on CRs/TPs

| CR/TP number | Comments collection |
| --- | --- |
| Revision of R4-2119041 (Huawei) |  |

# Topic #3: Rel-17 NR UE Demodulation and CSI requirements maintenance

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| 47 GHz Band |  |  |
| R4-2118012 | Intel Corporation | Applicability of 256QAM PDSCH requirements for band n262 Observation #1: Example 2 UE model induces higher degradation to the useful signal under baseline Rx processing assumptions. Observation #2: There is not a negligible performance difference between scenarios with 39GHz and 48.22GHz carrier frequencies: - 1.6 dB difference is observed with Example 2 UE model - 0.8 dB difference is observed with phase noise model from R4-2010176 Proposal #1: Consider 1 dB additional margin for TS 38.101-4 Table 7.2.2.2.1-3 Test 1-4 to extend applicable carrier frequency range to band n262. |
| R4-2118685 | Ericsson | Applicability of FR2 UE demodulation requirements for NR 47GHz band Proposal: Set the additional margin of 2.0 dB for the existing FR2 UE performance requirements in TS38.101-4 Table 7.2.2.2.1-3 Test 1-4. |
| R4-2118686 | Ericsson | CR: Applicability of FR2 demodulation requirements to n262 Set the additional margin for FR2 256QAM rank 1 for the operating carrier frequency higher than 40GHz. |
| R4-2119023 | Huawei, HiSilicon | Discussion on NR UE demodulation for 47GHz band Extra 2dB margin needs to be added on top of the existing 256QAM cases for performance requirements of 47GHz band. |
| R4-2119322 | Qualcomm Incorporated | Extension of 256QAM PDSCH Demodulation Requirements to 47 GHz band Observation 1: PN Model UE Example 2 in TR 38.803 does not provide a useful performance reference when evaluating the degradation 256QAM performance test for 48GHz. Observation 2: The observed SNR degradation in demodulation performance between 39 GHz and 48GHz (band n262) for 256QAM is 1.2dB. Proposal 1: Use the Phase Noise model proposed in R4-2010176 as reference to evaluate the extension of the applicability of the PDSCH requirements to band n262. Proposal 2: According to the presented simulation results and accounting for additional 0.3dB margin, apply the FR2 256QAM Rank 1 PDSCH Test to band n262 (up to 48.2GHz) with +1.5dB margin on top of the current requirement; |
| R4-2119410 | Intel Corporation | draft CR to TR 38.847 UE performance requirements Update analysis for UE performance requirements extension to 47GHz using the recent sumbitted results from different companies |
| FWA |  |  |
| R4-2118687 | Ericsson | CR: Introduction of Noc power level for n259 PC5 TS38.101-2 V17.2.3 Clauses 7.3.2.5-1 specifie REFSENS of n259 with CBW=50MHz as -89.7 dBm for PC5. Noc values are derived according TS38.101-4 4.5.3.3: Noc(PC_P, Band_X) = REFSENSPC_P, Band_X, 50MHz – 10log10(12 x 120kHz x PRBREFSENS) – SNRREFSENS + Δthermal = REFSENSPC_P, Band_X, 50MHz - 69.8 * Noc(PC5, Band_n259) = -89.7 - 69.8 = -159.5 Introducing the applicability rule of FR2 UE demodulation requirements for UE supporting band n259. |

## Open issues summary

### Sub-topic 3-1 256QAM PDSCH demod requirements for 47GHz band

Issue 3-1-1: Additional margin for 256QAM demod requirements for 47GHz band

Additional margin

- Proposals

  - Option 1: 1 dB (Intel)

  - Option 2: 2 dB (Ericsson, Huawei)

  - Option 3: 1.5 dB (Qualcomm)

- Recommended WF

  - Would 2dB be acceptable to all companies?

  - Discuss further.

## Companies’ views collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue 3-1-1: Additional margin for 256QAM demod requirements for 47GHz band |
| Intel | Issue 3-1-1: Additional margin for 256QAM demod requirements for 47GHz band In first round, we suggest to double check if 1.5 dB can be acceptable for everyone as the median value between three options. Based on our analysis, 2 dB margin can relax the requirements quite a lot. |
| Qualcomm | Issue 3-1-1: We share Intel’s view and are ok to agree on 1.5dB relaxation |
| Ericsson | Issue 3-1-1: Additional margin for 256QAM demod requirements for 47GHz band We can accept to add 1.5dB as an extra margin. |
| Huawei | Issue 3-1-1: We are fine with 1.5dB additional margin. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2118686 (Ericsson) | Moderator: Pending outcome of Issue 3-1-1 |
|  | Company A: |
|  | Company B: |
| R4-2119410 (Intel) | Ericsson: Thank you very much for your capturing the companies contributions. Once we agree on the extra margin for 256QAM, we propose to update [TBA]dB as same as CR R4-2118686. |
|  | Intel2: Thank you for comment. We will update this part. During the revision, we think that probably this Draft CR can be converted to CR, because for this meeting we have only these changes for TR 38.847 and this WI is closed. |
| R4-2118687 (Ericsson) | Company A: |
|  | Company B: |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic 3-1 256QAM PDSCH demod requirements for 47GHz band | Issue 3-1-1: Additional margin for 256QAM demod requirements for 47GHz band Tentative agreements: Additional margin of 1.5 dB is added for 256QAM demod requirements for 47GHz band Recommendations for 2nd round: Update the corresponding CRs with the agreed margin. |

### CRs/TPs

Captured in Section 4.1.

## Discussion on 2nd round (if applicable)

### Comments on CRs/TPs

| CR/TP number | Comments collection |
| --- | --- |
| Revision of R4-2118686 (Ericsson) |  |
| Revision of R4-2119410 (Intel) |  |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| LS on UE capability for supporting single DCI transmission schemes for multi-TRP | Apple | To: RAN1; Cc: RAN2 |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2118059 | Alignment of common test parameters | Rohde & Schwarz | Revised |  |
| R4-2117995 | Draft CR on CSI reference measurement channels | Intel Corporation | Return to |  |
| R4-2119442 | Discussion on PDSCH scheduling impact on CSI requirements | Intel Corporation | Noted |  |
| R4-2119513 | Views on PDSCH grant on TRS slots for CSI Reporting Tests | Qualcomm Incorporated | Noted |  |
| R4-2119030 | Draft CR Updates to LTE V2X PSBCH requirements in 36.101 (Rel-14) | Huawei, HiSilicon | Agreeable |  |
| R4-2117428 | Draft CR to 38.101-4 on Applicability for multi-TRxP test cases-R16 | Apple | Return to |  |
| R4-2119041 | Draft CR Updates to NR V2X requirements in 38.101-4 (Rel-16) | Huawei, HiSilicon | Revised |  |
| R4-2118012 | Applicability of 256QAM PDSCH requirements for band n262 | Intel Corporation | Noted |  |
| R4-2118685 | Applicability of FR2 UE demodulation requirements for NR 47GHz band | Ericsson | Noted |  |
| R4-2118686 | CR: Applicability of FR2 demodulation requirements to n262 | Ericsson | Revised |  |
| R4-2119023 | Discussion on NR UE demodulation for 47GHz band | Huawei, HiSilicon | Noted |  |
| R4-2119322 | Extension of 256QAM PDSCH Demodulation Requirements to 47 GHz band | Qualcomm Incorporated | Noted |  |
| R4-2119410 | draft CR to TR 38.847 UE performance requirements | Intel Corporation | Revised | TDoc reserved as Draft CR, should be CR. |
| R4-2118687 | CR: Introduction of Noc power level for n259 PC5 | Ericsson | Agreeable |  |

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Qualcomm | Gaurav Nigam | gnigam@qti.qualcomm.com |
| Qualcomm | Pierpaolo Vallese | pvallese@qti.qualcomm.com |
| Apple | Manasa Raghavan | Manasa.raghavan@apple.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)