3GPP TSG-RAN WG4 Meeting # 101-e 												R4-2120631

Electronic Meeting, 1st – 12th Nov, 2021

Agenda item:			8.9.5

Source:	Moderator (Samsung)

Title:	Email discussion summary for [101-e][321] NR_HST_FR2_Demod

Document for:	Information

Introduction

In RAN Plenary #89-e, the RAN4-led work item of NR support for high speed train (HST) scenario in FR2 has been approved [RP-202118] (which has been further revised to [RP-210800] with editorial revisions and updates on time schedule).

Based on the agreement captured in WF [R4-2108637], the test scope of UE/BS demodulation was under discussion. For this meeting, companies are encouraged to further discuss the test scope for UE/BS demodulation based on the FR2 HST deployment scenarios, and the related test setup for each identified requirements

In this email thread, the following agenda items will be discussed:

- 8.9.5.1 General

- 8.9.5.2 UE demodulation requirements

- 8.9.5.3 BS demodulation requirements

- 8.9.5.3.1 PUSCH requirements

- 8.9.5.3.2 PUSCH with UL timing adjustment requirements

- 8.9.5.3.3 PRACH requirements

It is suggested to have the following target of 1st and 2nd round email discussion

- 1st round: Further discussion the test scope of UE/BS demodulation based on FR2 HST deployment scenarios and the related test setup for each requirements

- 2nd round: Based on the output of 1st round, try to agree the simulation assumption for each demodulation requirements as much as possible for alignment in future meeting.

Topic #1: General

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117592 | Samsung | Proposal 3: Assume single probe and static UE for HST UE demodulation |
| R4-2118234 | Ericsson | Proposal 1: Bi-directional testing for both BS and UE is based on a single panel |
| R4-2119409 | Intel | Proposal 1: Assume that using of single panel is sufficient to verify demodulation processing in HTS FR2 bi-directional deployment scenario. |
| R4-2119018 | Huawei | Observation 1: For the current channel model for the UL timing case, the transient timing jump caused by beam switching from one RRH to another RRH under HST FR2 scenario still cannot be simulated. Proposal 1: Discuss timing jump issue later when conclusions are made in RRM session. If large timing jump cannot be verified in RRM session, the channel model for the UL timing case should be changed to involve the timing jump, e.g.: |
| R4-2119388 | Nokia | - Observation 1: The behaviour of Doppler shift trajectories in uni-directional deployment of HST FR2 scenarios is similar: In Scenario-A, the drops in Doppler offset are sharper; In Scenario-B, they are smoother but deeper. Therefore, no meaningful difference in performance can be expected between Scenario-A and Scenario-B. Observation 2: Uni-directional Scenario-A where the train is moving towards the serving beam demonstrates worse system performance and may have mobility problems. Proposal 1: If uni-directional deployment in Scenario-A where the train moves towards serving beam is found to be impractical, change the uni-directional channel mode to match the scenario where the train is moving from the serving beam. Observation 3: More frequent beam/RRH change/HOs are typical for bi-directional deployments. cosnθt=Ds-vtDmin2+Ds-vt2, Ds*nv*N<t≤ Ds*n+12*v*N -vtDmin2+vt2, Ds*n+12*v*N<t≤ Ds*n+1v*N, n=0,…,N-1 cosθt=cos0θt,…, cosN-1θt, 0<t≤Ds/v cosθt modDsv, t>Ds/v Proposal 2: If Option 2(a) cannot be agreed, then RAN4 to consider a new generalized bi-directional channel model with N=2 defined above (Option 2(f)). Proposal 3: RAN4 to consider the following options when introducing propagation channel models for HST FR2 deployment: - Option 1: Define only bi-directional model with Scenario-B parameters as a worst-case scenario. - Option 2: Define three models: uni-directional with Scenario-A and Scenario-B parameters, and bi-directional with Scenario-B parameters. |

Open issues summary

Last RAN4 meeting agreements in the WF R4-2115725 and R4-2115726

List of open issues

- Sub-topic 1-1 Channel Model

  - Issue 1-1-1: Channel model for Uni-directional RRH deployment

  - Issue 1-1-2: Channel model for Bi-directional RRH deployment

  - Issue 1-1-3: Channel model for performance requirements

  - Issue 1-1-4: Whether to incorporate propagation delay in DL demodulation channel model

  - Issue 1-1-5: Channel model with Delay Jumping for PDSCH demodulation test

  - Issue 1-1-6: Channel model for UL timing adjustment requirement

- Sub-Topic 2-2: PUSCH requirement Sub-topic 1-2 OTA test for UE and BS

  - Issue 1-2-1: Test feasibility with two panels for UE and BS

Sub-topic 1-1 Channel Model

Issue 1-1-1: Channel model for Uni-directional RRH deployment

- Observations

  - Observation 1(Nokia):

    - The behaviour of Doppler shift trajectories in uni-directional deployment of HST FR2 scenarios is similar: In Scenario-A, the drops in Doppler offset are sharper; In Scenario-B, they are smoother but deeper. Therefore, no meaningful difference in performance can be expected between Scenario-A and Scenario-B.

    - Uni-directional Scenario-A where the train is moving towards the serving beam demonstrates worse system performance and may have mobility problems.

- Proposals

  - Option 1(Nokia):

    - RAN4 to consider the configuration where the train is moving from the serving beam (i.e., in the same direction with RRH antenna panel orientation) as a basis for HST FR2 requirements in uni-directional deployments.

    - If uni-directional deployment in Scenario-A where the train moves towards serving beam is found to be impractical, change the uni-directional channel mode to match the scenario where the train is moving from the serving beam.

    - cosθt=-Ds_offset+vtDmin2+Ds_offset+vt2,  0<t≤Dsv    (eq. 1)

    - cosθt=cosθt modDsv,  t>Ds/v  ,   (eq. 2)

    - where 0 ≤Ds_offset<Ds   (eq. 3)

- Recommended WF

  - Agreement in previous meeting

    - For Uni-directional deployment, one channel model (either toward to serving beam or away from serving beam) is applied for demodulation requirement even if UE can travel in two directions in practice.

    - Use the HST-DPS channel model below as a starting point for FR2 HST Uni-Directional RRH Deployment

- UE is moving towards serving beam

- The cosine of angle θ(t)  used in Doppler shift fst=fd cosθt is provided as

cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2,  0<t≤Dsv

cosθt=cosθt modDsv,  t>Ds/v

where 0 ≤Ds_offset<Ds

  - From demodulation performance perspective, there is no difference either UE moving toward or away from serving beam, suggest to following the previous agreement.

Issue 1-1-2: Channel model for Bi-directional RRH deployment

- [Moderator] In last  meeting, option 2a is agreed to be considered as baseline:

- Agreement (achieved in this WF): - Channel modeling for FR2 HST bi-directional deployment - Option 2(a): To match Bi-directional deployment Scheme-1: UE connect to 2nd-nearest RRH. - cosθt=Ds-vtDmin2+Ds-vt2, 0<t≤(0.5*Ds)/v - cosθt=-vtDmin2+vt2, (0.5*Ds)/v<t≤Ds/v - cosθt modDsv, t>Ds/v - Agreement (GTW Aug 24th): - Companies are encouraged to draw conclusion in this meeting for RAN4 demodulation aspect. - All feasible transmission schemes with assioated channel modelling can be included into TR. - The baseline assumption was to consider option 2a for demodulation if introducing test cases pending on further checking by Nov 2021 RAN4 meeting. - Note: From frequency jump performance verification aspect, option 2a is more simple option.

- Observations

  - Observation 1(Nokia):

    - More frequent beam/RRH change/HOs are typical for bi-directional deployments.

  - Observation 2(Ericsson):

    - Different channel model options do not show significant PDSCH performance difference with regard to the SNR to achieve 70% of the maximum throughput

  - Observation 3 (ZTE):

    - If more than one beam per RRH panel and per CPE panel is used for Bi-directional deployment scenario-B, the CPE will experience more Doppler hopping and beam/panel switching than that of option 2a for bi-directional deployment.

  - Observation 4 (Huawei):

    - For Bi-directional Scenario-B deployment for FR2 HST, Scheme-3 provide the largest link budget remaining while it requires more switching which leads to smaller minimum beam dwelling time; link budget remaining may be un-sufficient for Scheme-1 considering the uplink.

- Proposals

  - Option 1(Nokia): If Option 2(a) cannot be agreed, then RAN4 to consider a new generalized bi-directional channel model with N=2 defined above (Option 2(f)).

cosnθt=Ds-vtDmin2+Ds-vt2,     Ds*nv*N<t≤ Ds*n+12*v*N -vtDmin2+vt2,     Ds*n+12*v*N<t≤ Ds*n+1v*N,    n=0,…,N-1

cosθt=cos0θt,…, cosN-1θt,   0<t≤Ds/v

cosθt modDsv,   t>Ds/v

    - Doppler shift trajectories for the new generalized model N=1 corresponds for the Option 2(a)

    - Doppler shift trajectories for the new generalized model N=1 corresponds for the Option 2(f)

  - Option 2(Ericsson): Apply option 2a for channel  model of Bi-directional scenario

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

  - Considering the group have faced the difficulties to align on a single scheme for bi-directional deployment for Scenario-B, as discussed during last meeting’s GTW, suggest to discussion the impact on channel modelling only.

Issue 1-1-3: Channel model for performance requirements

- Proposals

  - Option 1 (Nokia):

    - Option 1a: Define only bi-directional model with Scenario-B parameters as a worst-case scenario.

    - Option 1b: Define three models: uni-directional with Scenario-A and Scenario-B parameters, and bi-directional with Scenario-B parameters.

    - Option 1c: Define two models: uni-directional with Scenario-A parameters, and bi-directional with Scenario-B parameters.

- Recommended WF

  - Option 1b?

Issue 1-1-4:  Whether to incorporate propagation delay in DL demodulation channel model?

- [Moderator] Although this issue is related to Scenario-A, still suggest to be also discussed in demod email thread.

- Proposals

  - Proposal 1: To consider propagation delay for DL demodulation.

  - Proposal 2: The following model can be considered for propagation delay for DL demodulation.

- Recommended WF

  - Suggest to only discuss over demodulation email thread ([321 NR_HST_FR2_Demod_Part]).

  - Encourage feedback from companies

Issue 1-1-5: Channel model with Delay Jumping for PDSCH demodulation test

- Proposals

  - Option 1 (Ericsson): Not model delay jump for FR2 HST channel model

- Recommended WF

  - Option 1?

Issue 1-1-6: Channel model for UL timing adjustment requirement

- Observations

  - Observation 1 (Samsung):

    - The propagation delay from two neighbouring RRHs in Uni-directional scenario is larger than the CP length.

    - With larger TA command, the maximum adjustment can be up to 2us, which is still smaller than the maximum sudden change as 2.5us. The remaining propagation delay is about 0.5us, which is close to the length of CP.

  - Observation 2 (Intel):

    - If one-slot UE autonomous uplink timing adjustment will be considered to address timing issues in uni-directional deployments, there is no need to revise UL TA channel model

  - Observation 3 (Huawei):

    - For the current channel model for the UL timing case, the transient timing jump caused by beam switching from one RRH to another RRH under HST FR2 scenario still cannot be simulated.

  - Observation 4 (Nokia):

    - In the current (e.g., HST FR1) PUSCH timing adjustment test it is assumed that the UEs are in CONNECTED state. The range of timing adjustment that can be signaled with TAC MAC CE is ±2.1μs, and proposed test parameters for HST FR2 (A= 1.25 us, Δω = 1.04s-1) keeps Δτ within this range

    - The jump in RTT of -4.8us that happens at RRH change does not match the ±2.1μs range of timing adjustments that can be signalled with TAC MAC CE. Whereases, with the current model the change in propagation delay Δτ happen continuously. Therefore

- increasing the value of parameter A will not bring any additional value in the current test

- if the jump in RTT is added into the moving propagation condition then the feedbacked TAC will not be able to compensate for it and the test cannot be considered applicable.

    - Regardless of the type of solution that is agreed in RRM, we do not expect any changes in the situation when the CPE stays connected to the same RRH. Therefore, the current test should be still applicable for HST FR2 deployment.

    - Depending on RRM agreement, the scope of the solution to the problem of propagation delay difference between source and targets RRH at beam change might go beyond the scope of current UL TA PUSCH demodulation performance requirements. Therefore, proper testing of the solution might require a new testing setup

- Proposals

  - Option 1 (CATT, Samsung):

    - Reuse existing timing model for UL timing adjustment requirement

  - Option 2 (Ericsson):

Δt = (4.6*t/7.2) mod 4.6

    - If the TA test includes the timing jump associated with Scenario A uni-directional, enable that optionally the BS is informed when the timing jump occurs.

    - If there is a TA model including the timing jump change, then BS should declare whether they support Scenario A uni-directional and the associated requirement; for BS that do not support then the previously agreed TA requirement should be applicable.

  - Option 3 (Huawei):

    - Discuss timing jump issue later when conclusions are made in RRM session. If large timing jump cannot be verified in RRM session, the channel model for the UL timing case should be changed to involve the timing jump, e.g.

  - Option 4 (Nokia):

    - Discuss if a new requirement and setup is needed for testing of UL timing at beam change between non-collocated RRHs based on the results of RRM discussion.

- Recommended WF

  - Encourage feedback from companies

Sub-topic 1-2 OTA test for UE and BS

Issue 1-2-1: Test feasibility with two panels for UE and BS

- Proposals

  - Option 1 (Ericsson, Intel, Samsung): Bi-directional testing for both BS and UE is based on a single panel

- Recommended WF

  - Option 1?

Companies views’ collection for 1st round

Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Sub topic 1-1

| Company | Comments |
| --- | --- |
| XXX | Issue 1-1-1 Issue 1-1-2 Issue 1-1-3 Issue 1-1-4 Issue 1-1-5 Issue 1-1-6 |
| Intel | Issue 1-1-1: Channel model for Uni-directional RRH deployment From demodulation perspective there will be no difference between models when train moves towards the serving beam and when train moves from the serving beam. It also does not impact provided demodulation performance. We have not a strong preference and fine to update channel model considering system level observations obtained by Nokia. Issue 1-1-2: Channel model for Bi-directional RRH deployment Option 2 may not reflect accurately real propagation conditions in bi-directional deployment. Same time Doppler trajectory with double frequency jump is enough to verify baseband performance. Support baseline option 2a. Issue 1-1-3: Channel model for performance requirements Option 1 looks reasonable for BS performance verification. Same time for UE side there is an on-going discussion on UE frequency tracking approach that may impact channel model selections. Issue 1-1-4: Whether to incorporate propagation delay in DL demodulation channel model? Support recommended WF. Issue 1-1-6: Channel model for UL timing adjustment requirement Support option 4. It is immature to discuss changes of UL TA channel model without assumption on which side and how this time jump is handled. |
| Ericsson | Issue 1-1-1 Since the purpose pf UE/BS demodulation requirement is to verify the baseband performance with a certain condition. As far as the test purpose is satisfied, we don’t think the channel model need to model exactly same as the deployment scenario like other demodulation tests. We think RAN4 should discuss the purpose of the FR2 HST demodulation test cases. For example, the maximum Doppler shift, the Doppler shift changes between -fmax and +fmax, considering the bi-directional channel model. As far as the purpose is satisfied, we think any channel model is fine since we don’t see any performance difference. RAN4 should consider to minimize the number of test cases. Regarding the HST-DPS channel model for Uni-directional deployment, RAN4 has already agreed the channel model (R4-2108661/R4-2115725). We should keep the agreement. Issue 1-1-2 Option 2a is fine. Moreover RAN4 define the single test case using this model. Issue 1-1-3 Since there is no performance difference, we don’t see any reason to define the different requirements. If the baseband performance can be validated using one of the scenarios, then we can establish the test so that the baseband performance is sufficient for all of the scenarios Issue 1-1-4 Support Option 1. Issue 1-1-5 Firstly, the RRM session should discuss whether the TA jump can be handled by the one shot timing adjustment or some other RRM mechanism. If this is the case, then there is no need to model the delay jump as part of demodulation. In case the TA is not adjusted by an RRM mechanism and we need to consider it in the demodulation requirements, then we need to consider that the timing jump only occurs in scenario A and also that it only occurs when the serving BS changes (and hence the BS will always be aware that a timing jump has occurred). |
| Huawei | Issue 1-1-1: Channel model for Uni-directional RRH deployment From demodulation performance perspective, there is no difference either UE moving toward or away from serving beam. So we prefer to keep previous agreeements. Issue 1-1-2: Channel model for Bi-directional RRH deployment We are OK with Option 2. Issue 1-1-3: Channel model for performance requirements Option 1c. We prefer to define Uni-ScenarioA case and Bi-ScenarioB case and only define channel model for the cases to be defined. Issue 1-1-4: Whether to incorporate propagation delay in DL demodulation channel model? We propose to discuss this issue later when conclusions are made in RRM session. If large timing jump cannot be verified in RRM session, it should be considered in channel model since large demodulation performance degradation is observed if UE cannot correctly handle the large timing jump. Issue 1-1-5: Channel model with Delay Jumping for PDSCH demodulation test We propose to discuss this issue later when conclusions are made in RRM session. If large timing jump cannot be verified in RRM session, it should be considered in channel model since large demodulation performance degradation is observed if UE cannot correctly handle the large timing jump. Issue 1-1-6: Channel model for UL timing adjustment requirement Option 3. We propose to discuss this issue later when conclusions are made in RRM session. |
| Qualcomm | Issue 1-1-1: Channel model for Uni-directional RRH deployment; There is no expected difference in demod performances between different orientations, keep previous agreements; Issue 1-1-2: Channel model for Bi-directional RRH deployment We support Option 2; Issue 1-1-3: Channel model for performance requirements Define models according to test scope discussion, see Issue (2-1-1); Issue 1-1-4: Whether to incorporate propagation delay in DL demodulation channel model? Issue 1-1-5: Channel model with Delay Jumping for PDSCH demodulation test From the view of demodulation performances, what’s the difference between these 2 issues? It is our view that Delay Jump should be modelled in the PDSCH test because it can have an impact on performance, but since this has not been considered until now in RAN4 Demodulation tests we should check with TE vendors whether this implementation is feasible on their side; |
| Nokia, Nokia Shanghai Bell | Issue 1-1-1: Channel model for Uni-directional RRH deployment On the one hand we agree that no meaningful performance difference is expected in between the models. On the other hand, we think that the tested model, if possible, shall represent the typical scenario that will be used in practice. In this case, it is straightforward to update the model. However, we still would like to hear the opinion of other companies about the feasibility of unidirectional scenario A, where the train is moving to the opposite direction, Issue 1-2-2 in the Deployment email discussion thread [320]. Issue 1-1-2: Channel model for Bi-directional RRH deployment We are OK to use model Option 2(a). In our opinion, even though more frequent Doppler shift sign alternations can be expected in Scenario-B (as in our new channel model proposal), Model 2a is typical enough and we do not expect meaningful pefromance difference between these two models. Issue 1-1-3: Channel model for performance requirements Form the BS demod point of view, we think that Option 1a is sufficient. Thus, we prefer it more. However, Option 1b is also acceptable for us considering that applicability rules can be defined for PDSCH testing. Issue 1-1-4: Whether to incorporate propagation delay in DL demodulation channel model? Agree with Option 1. Issue 1-1-6: Channel model for UL timing adjustment requirement If the test stays in its current form, we support Option 1 because it follows the approach from LTE and HST in FR1. Otherwise, further discussion is very dependent on the concussions of RRM session on UL timing adjustment. |
| ZTE | Issue 1-1-1: Channel model for Uni-directional RRH deployment Keep the previous agreement. Issue 1-1-2: Channel model for Bi-directional RRH deployment We are OK with option 2. Issue 1-1-3: Channel model for performance requirements We think bi-directional model with Scenario-B is not the worst-case scenario, this scenario experiences Doppler jump but it does not experience delay jump. If the worst-case can not be identified option 1c is more preferred. Issue 1-1-4: Whether to incorporate propagation delay in DL demodulation channel model? We propose to wait for the conclusion from RRM session. Issue 1-1-5: Channel model with Delay Jumping for PDSCH demodulation test We propose to wait for the conclusion from RRM session. |
| Samsung | Issue 1-1-1: The purpose of defining channel model is to verify the receiver processing, either train moving towards the RRH beam direction or far away the RRH beam direction, there is no difference for demodulation requirement. As agreed in RAN4, one channel model (either toward to serving beam or away from serving beam) is applied for demodulation requirement even if UE can travel in two directions in practice, there is no restriction for the practical FR2 HST deployment, and UE is moving towards serving beam as staring point. Therefore, we still prefer to keep previous agreement. Meanwhile, in practice deployment, UE can travel in two directions, that means UE will experience the Doppler value between –fmax and +fmax, even change the channel model, the problem mentioned by company is still existed. Issue 1-1-2 Based on the agreement in the last meeting, option 2a is more simple, which is including at one Doppler jumping. Thus, the TCI state switching and tracking Doppler jumping processing can be verified. As agreed, all the transmission schemes with associated channel modelling are feasible, there is no restriction for practical FR2 HST deployment Meanwhile, from demodulation requirement perspective and baseband processing there is no difference with option 1 and option 2 Issue 1-1-3 We are fine with option 1c for UE side, regarding BS side, we propose option 1d as - Option 1d : Define only Uni-directional model with Scenario-A parameters as a worst-case scenario. Based on agreed channel model for Uni-directional and Bi-directional scenario, the following is the maximum Doppler experienced by RRH for different scenario. Scenario; Maximum Doppler. Uni-directional with scenario A(Ds_offset=10); 19442Hz. Uni-directional with scenario B(Ds_offset=100); 19111Hz. Bi-directional with scenario B; 19012Hz. Among three scenarios, the Doppler observed by RRH in Uni-directional with scenario A is the largest. While from the demodulation perspective, whether to define requirement is to identify whether different baseband processing will be foreseen. We suggest to discuss UE and BS separately For BS side, there is no different receiver processing foreseen for BS side. It is reasonable to define the requirement with one of them to verify the BS processing, Therefore, we prefer to define the requirement with one of them, the most challenge scenario as A can be considered. Regarding UE side, for Uni-directional RRH deployment scenario, there is no baseband processing different, similar results for can be achieved. So, there is no necessary to introduce requirement for both scenario A and scenario B with same transmission scheme. So, one set requirement for Uni-directional should be enough. While for Bi-directional scenario, huge Doppler frequency will be experienced by UE when UE switches one RRH from another RRH. The Doppler value will be double compared with the value in Uni-directional, which may exceed the frequency Doppler estimation range by TRS. UE may has different FOE strategies. Therefore, to verify the proper time/frequency offset tracking, it is necessary to define PDSCH requirement for Bi-directional Issue 1-1-4 We prefer to not incorporate propagation delay, Different with SFN, the propagation delay is incorporated into channel model, since UE can receive PDSCH from multiple RRH the received signal will have a delay due to the distance between RRH For FR2 HST, only DPS schemes is considered, at one time, UE can only receive the PDSCH from one RRH, the UE timing adjustment belong to RRM scope, it is not practical to design a joint test cases across demod and RRM Issue 1-1-5 Similar comment as Issue 1-1-4, not model delay jump for FR2 HST channel model Issue 1-1-6: Option 1 Where the maximum adjustment can be changed from +32*2 Ts (Ts= 64*Tc) to -31*2*Ts, which is about 2us, after the lager TA adjustment, the remaining propagation delay is about 0.5us, which is close to the length of CP, existing TA model can keep this keeps Δτ within this range. After switching, the speed of the timing change in the gNB time adjustment test is relatively slow, tracking the roundtrip propagation delay changes is not difficult and proper uplink timing can be maintained with small incremental adjustments |

Sub topic 1-2

| Company | Comments |
| --- | --- |
| XXX | Issue 1-2-1 |
| Intel | Issue 1-2-1: Test feasibility with two panels for UE and BS Support Option 1 which is aligned with WID. |
| Ericsson | Issue 1-2-1 Support Option 1. |
| Huawei | Issue 1-2-1: Test feasibility with two panels for UE and BS Option 1. |
| Qualcomm | Issue 1-2-1: Test feasibility with two panels for UE and BS Ok with option 1; |
| Nokia, Nokia Shanghai Bell | Issue 1-2-1: Test feasibility with two panels for UE and BS Option 1 is OK for us. |
| ZTE | Issue 1-2-1: Test feasibility with two panels for UE and BS OK with option 1. |
| Samsung | Issue 1-2-1: Test feasibility with two panels for UE and BS Option 1, For FR2, although two panels are considered, while only one panel is activated at each time. Therefore, for demodulation test perspective, there is no difference at all compared with one panel |

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Candidate options: Recommendations for 2nd round: Issue 1-1-1: Channel model for Uni-directional RRH deployment Candidate options: - Option 1(Nokia, Intel): - RAN4 to consider the configuration where the train is moving from the serving beam (i.e., in the same direction with RRH antenna panel orientation) as a basis for HST FR2 requirements in uni-directional deployments. - If uni-directional deployment in Scenario-A where the train moves towards serving beam is found to be impractical, change the uni-directional channel mode to match the scenario where the train is moving from the serving beam. cosθt=-Ds_offset+vtDmin2+Ds_offset+vt2, 0<t≤Dsv (eq. 1) cosθt=cosθt modDsv, t>Ds/v , (eq. 2) where 0 ≤Ds_offset<Ds (eq. 3) - Option 2(Samsung, Ericsson, Huawei, Qualcomm): Following previous agreement Recommendations for 2nd round: - As indicated in previous meeting - For Uni-directional deployment, one channel model (either toward to serving beam or away from serving beam) is applied for demodulation requirement even if UE can travel in two directions in practice. - Use the HST-DPS channel model below as a starting point for FR2 HST Uni-Directional RRH Deployment - UE is moving towards serving beam - The cosine of angle θ(t) used in Doppler shift fst=fd cosθt is provided as cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2, 0<t≤Dsv cosθt=cosθt modDsv, t>Ds/v where 0 ≤Ds_offset<Ds - From demodulation performance perspective, there is no different either the train moving toward to the server beam or away from serving beam) - The channel model defined is to verify the baseband processing, there is no restriction of practical FR2 HST deployment. - For moving forward, moderator suggests to follow the previous agreement - Further check in GTW session GTW discussion on 5th Nov GTW Agreement: Following previous agreement. - For Uni-directional deployment, one channel model (either toward to serving beam or away from serving beam) is applied for demodulation requirement even if UE can travel in two directions in practice. - Use the HST-DPS channel model below as a starting point for FR2 HST Uni-Directional RRH Deployment - UE is moving towards serving beam - The cosine of angle θ(t) used in Doppler shift fst=fd cosθt is provided as cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2, 0<t≤Dsv cosθt=cosθt modDsv, t>Ds/v where 0 ≤Ds_offset<Ds Issue 1-1-2: Channel model for Bi-directional RRH deployment Tentative agreements: GTW Agreement: Option 2(a) for for FR2 HST Bi-directional RRH deployment - Apply channel modelling as option 2a for FR2 HST Bi-directional RRH deployment - Option 2(a): To match Bi-directional deployment Scheme-1: UE connect to 2nd-nearest RRH. - cosθt=Ds-vtDmin2+Ds-vt2, 0<t≤(0.5*Ds)/v - cosθt=-vtDmin2+vt2, (0.5*Ds)/v<t≤Ds/v - cosθt modDsv, t>Ds/v Issue 1-1-3: Whether to incorporate propagation delay in DL demodulation channel model? Candidate options: - Option 1(ZTE): To consider propagation delay for DL demodulation, the following model can be considered for propagation delay for DL demodulation - Option 2(Ericsson, Samsung, Nokia): Not considering propagation delay in DL demodulation channel model - Option 3(Huawei): Pending on the RRM session discussion Recommendations for 2nd round: - For DL Rx timing, based on RRM conclusion in the last meeting, PSS/SSS detection is robust enough to handle the ISI and time difference - For FR2 HST, only DPS scheme is considered. UE can only receive the PDSCH signal from RRH at given time. UE timing adjustment belongs to RRM scope, can be verified based on RRM requirement. It is not proper to define a test case to cover demodulation test and RRM test - Delay jump only happen during RRH switching stage, the performance impact is minor compared with the whole transmissions. During the RRH switching, there is no PDSCH scheduling expected - The main concern to model delay or delay jump, it will has impact performance. To address the concern, additional marginal can be considered during requirement definition stage. - For moving forward, moderator suggest the following compromise solution - Not incorporate propagation delay in DL demodulation channel model - Note: additional marginal can be considered during requirement definition - Further check in GTW session GTW Agreement: -TCI switching belongs RRM scope, no need to be modelling and considering in demodulation requirements -Demodulation requirements will not verify the PDSCH performance during TCI switching period. Issue 1-1-4: Channel model with Delay Jumping for PDSCH demodulation test Candidate options: - Option 1 (Qualcomm): To model delay jump for FR2 HST channel model - Option 2 (Samsung, Ericsson): Not model delay jump - Option 3 (Huawei): Pending on RRM session discussion Recommendations for 2nd round: - For DL Rx timing, based on RRM conclusion in the last meeting, PSS/SSS detection is robust enough to handle the ISI and time difference - For FR2 HST, only DPS scheme is considered. UE can only receive the PDSCH signal from RRH at given time. UE timing adjustment belongs to RRM scope, can be verified based on RRM requirement. It is not proper to define a test case to cover demodulation test and RRM test - Delay jump only happen during RRH switching stage, the performance impact is minor compared with the whole transmissions. During the RRH switching, there is no PDSCH scheduling expected - The main concern to model delay or delay jump, it will has impact performance. To address the concern, additional marginal can be considered during requirement definition stage. - For moving forward, moderator suggest the following compromise solution - Not model delay jumping for PDSCH requirement - Note: additional marginal can be considered during requirement definition - Further check in GTW session Issue 1-1-5: Channel model for UL timing adjustment requirement Candidate options: - Option 1 (CATT, Samsung): - Option 2 (Ericsson): Δt = (4.6*t/7.2) mod 4.6 - If the TA test includes the timing jump associated with Scenario A uni-directional, enable that optionally the BS is informed when the timing jump occurs. - If there is a TA model including the timing jump change, then BS should declare whether they support Scenario A uni-directional and the associated requirement; for BS that do not support then the previously agreed TA requirement should be applicable. - Option 3 (Huawei): - Discuss timing jump issue later when conclusions are made in RRM session. If large timing jump cannot be verified in RRM session, the channel model for the UL timing case should be changed to involve the timing jump, e.g. - Option 4 (Nokia, Intel, Huawei): - Discuss if a new requirement and setup is needed for testing of UL timing at beam change between non-collocated RRHs based on the results of RRM discussion. Recommendations for 2nd round: - UE UL TA adjustment belongs to RRM scope, the TA jump issue is under discussion in RRM session. The requirement can be verified based on RRM requirement. It is not proper to define a test case to cover demodulation and RRM test - Based on discussion, majority companies think “one shot UE autonomous large uplink timing adjustment” can solve the TA jump issue for Uplink - With maximum TA adjustment based on RAN1, the remaining propagation delay is close to the length of CP, existing TA model can keep Δτ within this range. - Delay jump only happen during RRH switching stage, the performance impact is minor compared with the whole transmissions. After switching, the speed of the timing change in the gNB time adjustment test is relatively slow, tracking the roundtrip propagation delay changes is not difficult and proper uplink timing can be maintained with small incremental adjustments - The main concern to Uplink timing, it will has impact performance. To address the concern, additional marginal can be considered during requirement definition stage. - For moving forward, moderator suggest the following compromise solution - Apply existing UL TA model for UL timing adjustment requirement - Note: additional marginal can be considered during requirement definition - Note: The conclusion of RRM discussion on Uplink timing solution can be considered if necessary - Further check in GTW session GTW Agreements: The baseline assumption: following same approach as existing PUSCH UL timing adjustment requirement (option 1) - Further checking RRM conclusion on UL timing at beam change between non-collocated RRHs Issue 1-1-5: Channel model for performance requirements [moderator]: Combination Issue 2-1-1: Scope of PDSCH requirements with Uni-directional RRH scenario A/B and Bi-directional RRH scenario B and Issue 3-1-1: Scope of PUSCH requirements with Uni-directional RRH scenario A/B and Bi-directional RRH scenario B BS and UE has different RS tracking method, moderator suggests to discuss the scope of BS and UE demodulation requirement for different scenarios separately For BS side Candidate options: - Option 1 (Nokia, Intel, Samsung, Ericsson): Define single requirement with cover all scenarios - Option1a(Samsung): only based on Uni-directional scenario A - Option1b(Nokia): only based on Bi-directional scenario B - Option 2 (Nokia): Define three models: uni-directional with Scenario-A and Scenario-B parameters, and bi-directional with Scenario-B parameters with test applicability - Option 3 (Huawei, ZTE): Define two models: uni-directional with Scenario-A parameters, and bi-directional with Scenario-B parameters. Recommendations for 2nd round: - 4 companies have provided the simulation comparison among all scenarios. Minor difference is foreseen based on 3 companies’ results. - For moving forward, moderator suggest the following compromise solution - Define one set requirement for PUSCH to cover Uni-directional scenario A/B , and Bi-directional B - FFS on which scenario will be chose to requirement definition - Option 1: Uni-directional scenario A - Option 2: Bi-directional scenario B - Encourage company to bring simulation results for both scenarios. Based on collected results decide requirement definition - If time allowed, Further check in GTW session For UE side Candidate options: - Option 1 (Huawei, Intel, Samsung): Only define PDSCH requirement with Uni-directional scenario A and Bi-directional scenario B - Option 2 (Ericsson): Define a single PDSCH demodulation test case covering all the deployment scenarios [Scenarios A/B with Uni-directional and Scenario-B with Bi-directional deployment ] - Option 3 (Intel, CMCC): Define separate test cases for Uni-directional deployment scenario A and B, and bi-directional scenario B. - Further discuss and define applicability rules between defined test cases for different scenarios - Option 4 (ZTE) - We prefer to define test case for Uni-directional and bi-directional deployment separately if the test effort of a composite scheme is greater than the sum of the test effort of Uni-directional and bi-directional deployment. - Option 5 (Qualcomm) - Define only 1 Requirement (based on Scenario A OR B) for each Deployment Type - postpone the Scenario choice to after simulation results for both Scenarios are collected, based on the strictest requirement; Recommendations for 2nd round: - For moving forward, moderator suggest the following compromise solution - Define one requirement for PDSCH per each RRH deployment scenario (Uni-directional and Bi-directional scenario) - FFS on which scenario will be chose to requirement definition for Uni-directional A or B - Encourage company to bring simulation results for both scenarios, decide requirement definition based collected results. - If time allowed, Further check in GTW session |
| Sub-topic | Issue 1-2-1: Test feasibility with two panels for UE and BS Tentative agreements: - Assuming single panel for UE and BS FR2 HST Bi-directional RRH deployment scenario testing |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Topic #2: UE demodulation requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117592 | Samsung | Proposal 1: Define PDSCH requirement only with Uni-directional scenario in scenario A. If the requirements were introduced for both scenario A and scenario B, define test applicability rule as if UE passes the test of scenario A, it can skip the test of scenario B. Define PDSCH requirement with Bi-directional scenario with scenario B. Introduce UE capability to differentiate the requirements of Uni-directional and Bi-directional scenario. Proposal 2: Define PUSCH with one requirement as Uni-directional RRH deployment scenario in scenario A for FR2 HST. If both scenarios A and B are introduced, the test is performed based on BS manufacture declaration. If BS declared to support both scenario A and scenario B and BS can pass the test of Uni-directional for scenario A, it can skip the test of Uni-directional scenario for scenario B. If BS declared to support both Uni-directional and Bi-directional operation, and BS can pass the test of Uni-directional for scenario A, it can skip the test of Bi-directional scenario for scenario B. Proposal 3: Assume single probe and static UE for HST UE demodulation |
| R4-2117593 | Samsung | Observation 1: Similar performance can be achieved for Uni-directional scenario with A and B Observation 2: About 1 dB performance loss for Bi-directional compared with Uni-directional scenario Proposal 1: Define two sets of PDSCH requirement for Bi-directional scenario with two Doppler values, - 9722Hz - 7000Hz Proposal 2: Define PDSCH requirement with DPS scheme 1a and DPS scheme 1b to cover one scenario of Uni-directional RRH deployment - DPS scheme 1a with scenario A - DPS scheme 1b with scenario A Or - DPS scheme 1a with scenario A - DPS scheme 1b with scenario B Reuse Rel-16 applicability rule for UE test depending on UE capability, i.e, if a UE can support >1 TCI state, then UE will pass scheme 1b and skipped schem1a test cases, and if a UE only support 1 TCI state, the UE need to pass scheme 1a and ship scheme 1b test cases Proposal 3: Only Define PDSCH requirement with DPS scheme 1a for Bi-directional RRH deployment in scenario B. Proposal 4: Configure 2 SSBs association 2 CSI-RS resource sets for PDSCH requirement, where SSB # (k mod 2), CSI-RS (for tracking) resource set # ((k mod 2) + 1), CSI-RS (for CSI acquisition) resource set # ((k mod 2) + 3) and CSI-RS (for beam refinement) resource set # ((k mod 2)+5) are transmitted by kth RRH, Configure the same period for SSB and TRS as 20ms - SSB position in burst: 1 - CRS-RS offset: 2 for CSI-RS resource 1 and 2, 3 for CSI-RS resource 3 and 4 Proposal 5: Choose MCS 17 with rank2 PDSCH requirement as starting point. Proposal 6: Scheduled PDSCH in TDD special slots as baseline |
| R4-2117694 | CMCC | Proposal 1: For Bi-directional deployment scenario, the maximum Doppler shift is 9722Hz, and the assumption of RS for frequency tracking is up to UE implementation. Proposal 2: it is proposed to define PDSCH requirements for both DPS transmission scheme 1a and 1b for Bi-directional scenario of scenario B. Proposal 3: the applicability rule between DPS 1a and DPS 1b can be reused. |
| R4-2118008 | Intel | Proposal 1: Adopt 9722 Hz Doppler frequency for PDSCH demodulation requirements with Bi-directional deployment. Proposal 2: If found to be needed, define UE capability to support operation in HST FR2 bidirectional deployments with higher than 250 km/h speed. Define corresponding performance requirements with up to UE capability. Proposal 3: Define PDSCH requirement for Uni-directional deployment scenario A with DPS scheme 1a and Uni-directional deployment scenario B with DPS scheme 1b. Proposal 4: Define PDSCH requirements only with DPS scheme 1a for bi-directional deployment scenario. Proposal 5: Define PDSCH requirements with DPS scheme 1a scenario A and 1b for scenario B in uni-directional deployments. Define same applicability rule between 1a and 1b schemes as in HST FR1 Proposal 6: Define PDSCH requirements according to Table 1 or 2 depending on conclusion for bi-directional scenario with different speeds. Proposal 7: If bi-directional requirements will be defined for two different speeds (according to Table 2) define applicability rule that if UE has passed requirement with higher speed it does not need to pass requirement with lower speed. |
| R4-2118234 | Ericsson | Proposal 1: Bi-directional testing for both BS and UE is based on a single panel Proposal 2: Confirm model 2a for bi-directional modelling. Proposal 3: Set a single PDSCH and single PUSCH requirement for all scenarios (A, B Uni-directional, B Bi-directional). Passing the requirement demonstrates minimum performance can be achieved by the baseband in any of the scenarios. |
| R4-2118434 | ZTE | Observation 1: The transmission scheme is highly related to the number of beam which is still under discussion in RRM session. Proposal 1: To define scheme 1b for scenario-B uni-directional deployment and scenario-B bi-directional deployment. Proposal 2: Scheme 1a can be defined for scenario-A uni-directional deployment if the number of beam for scenario-A uni-directional deployment is one, otherwise scheme 1b is to be defined. |
| R4-2118684 | Ericsson | Observation 1: Different channel model options do not show significant PDSCH performance difference with regard to the SNR to achieve 70% of the maximum throughput. Proposal 1: Confirm Option 2a for bi-directional deployment scenario. Proposal 2: Channel model for HST FR2 should not model the delay jump. Proposal 3: Set 9722Hz to the maximum Doppler frequency offset for PDSCH requirement in Bi-directional deployment scenario. Proposal 4: RAN4 should choose one of scenarios for DPS transmission schemes for Uni-directional scenario. Proposal 5: RAN4 defines both DPS scheme 1a and 1b for PDSCH requirement in Uni-directional scenario. Test applicability depends on UE capability of active TCI states. Proposal 6: RAN4 defines both DPS scheme 1a and 1b for PDSCH requirement in Bi-directional scenario. Test applicability depends on UE capability of active TCI states. Proposal 7: Bi-directional testing for UE demodulation requirements is based on a single panel. This means the OTA test environment has one transmitter and it changes the Doppler shift according to the channel model. Proposal 8: Assume the following parameters for PDSCH demodulation requirements for FR2 HST. - Antenna configuration 2x2 - MCS 17, Rank 1 - SCS=120kHz, CBW=200MHz - DMRS type 1 with 2 additional DMRS symbols (i.e., DMRS configuration with 1+1+1) - TRS transmitted every 10ms Proposal 9: RAN4 should define a single PDSCH demodulation test case covering all the deployment scenarios (Scenarios A/B with Uni-directional and Scenario B with Bi-directional deployment). |
| R4-2119020 | Huawei | Observation 1: There is negligible demodulation performance difference for different FR2 HST scenarios for downlink. Observation 2: It is feasible to use SSB+TRS for tracking frequency offset for downlink to support 350km/h, using SSB for coarse frequency offset estimation and TRS for precise frequency offset estimation. Proposal 1: Only define Uni-Scenario A case and Bi-Scenario B case Proposal 2: Define 350km/h requirements, i.e. Doppler of 9722Hz targeting 350km/h at 30GHz for both Bi-directional and Uni-directional RRH deployment. Proposal 3: Do not introduce the UE capability to differentiate requirement for Bi/Uni-directional Proposal 4: Define DPS scheme 1a for Bi-directional deployment and define both DPS scheme 1a and scheme 1b for Uni-directional deployment. Same applicability rule as Rel-16 HST between DPS 1a and DPS 1b should be used. |
| R4-2119320 | Qualcomm | Observation 1: To actively track ≥2 TCI States in FR2 HST Bidirectional Deployment, the UE can be required to simultaneously receive from 2 different panels (ie. when RRHs transmitting Active TCI states are placed in opposite relative directions with respect to the UE). Observation 2: In order to avoid performance degradation, before receiving PDSCH after a TCI state switch across different RRHs with large Doppler and/or Delay jumps, the UE needs to be allowed additional time to select SSB as FO tracking RS, receive it from the RRH transmitting the target TCI and process it, before selecting back TRS as FO tracking RS, receive it and process it; Observation 3: The UE will have to switch to SSB-based tracking to estimate the doppler jump in bidirectional deployment or the delay jump in unidirectional deployment when switching TCI across different RRHs; Observation 4: For PDSCH Demodulation Tests, assume that UE is informed through network signaling whether the TCI State Switch happens across RRH according to the proposals included in [6]. Proposal 1: For PDSCH Demodulation Tests with bidirectional deployment, assume that the FR2 HST UE is connected to the 2nd nearest RRH according to the deployment model depicted in the picture above; Proposal 2: Do not define PDSCH Demodulation requirements for HST DPS Scheme 1b for Bidirectional deployment. Proposal 3: During the PDSCH demodulation tests for FR2 HST DPS, after the UE switches to a new TCI do not consider throughput statistics until the UE has received and processed at least one instance of SSB, followed by one instance of TRS and including respective RS processing time. Proposal 4: According to the previous proposal, for FR2 HST DPS extend the FR1 HST DPS PDSCH allocation timeline to include TfirstSSB, TSSB proc and replace TfirstTRS with TfirstTRSafterSSB. |
| R4-2119409 | Intel | Proposal 1: Assume that using of single panel is sufficient to verify demodulation processing in HTS FR2 bi-directional deployment scenario. Proposal 2: Define separate test cases for uni-directional deployment scenario A and B, and bi-directional scenario B. Proposal 3: Further discuss and define applicability rules between defined test cases for different scenarios. |

Open issues summary

Last RAN4 meeting agreements in the WF R4-2115725 and R4-2115726

List of open issues

- Sub-Topic 2-1: PDSCH requirement

  - Issue 2-1-1: Scope of PDSCH requirements with Uni-directional RRH scenario A/B and Bi-directional RRH scenario B

  - Issue 2-1-2: Doppler Frequency for Bi-directional scenario

  - Issue 2-1-3: UE capability

  - Issue 2-1-4: Test applicability rule

  - Issue 2-1-5: DPS transmission schemes for Uni-directional RRH scenario

  - Issue 2-1-6: DPS transmission schemes for Bi-directional RRH scenario

  - Issue 2-1-7: Number of Sets of requirement for Uni-directional and Bi-directional scenario

  - Issue 2-1-8: Deployment scenario for demodulation test for Bi-directional scenario

  - Issue 2-1-9: Test procedure of DPS transmission schemes

  - Issue 2-1-10: Throughput statistics during TCI state switching

  - Issue 2-1-11: Test setup for PDSCH requirement

Sub-topic 2-1 PDSCH requirements

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-1-1: Scope of PDSCH requirements with Uni-directional RRH scenario A/B and Bi-directional RRH scenario B

- Observations

  - Option 1 (Huawei):

    - There is negligible demodulation performance difference for different FR2 HST scenarios for downlink

  - Option 2 (Samsung):

    - Similar performance can be achieved for Uni-directional scenario with A and B

    - About 1 dB performance loss for Bi-directional compared with Uni-directional scenario

  - Option 3 (Ericsson):

    - Different channel model options do not show significant PDSCH performance difference with regard to the SNR to achieve 70% of the maximum throughput.

- Proposals

  - Option 1 (Huawei): Only define PDSCH requirement with Uni-directional scenario  A and Bi-directional scenario B

  - Option 2 (Ericsson): Define a single PDSCH demodulation test case covering all the deployment scenarios [Scenarios A/B with Uni-directional and Scenario-B with Bi-directional deployment ]

  - Option 3 (Intel): Define separate test cases for uni-directional deployment scenario A and B, and bi-directional scenario B.

    - Further discuss and define applicability rules between defined test cases for different scenarios

  - Option 4 (Samsung)

    - Define PDSCH requirement only with Uni-directional scenario in scenario A. If the requirement was introduced for both scenario A and scenario B, define test applicability rule as if UE passes the test of scenario A, it can skip the test of scenario B.

    - Define PDSCH requirement with Bi-directional scenario with scenario B

  - Option 5 (ZTE)

    - We prefer to define test case for uni-directional and bi-directional deployment separately if the test effort of a composite scheme is greater than the sum of the test effort of uni-directional and bi-directional deployment.

- Recommended WF

  - Encourage feedback from companies

Issue 2-1-2: Test applicability rule for requirements of different scenarios

- Proposals

  - Option 1 (Samsung): If the requirement was introduced for both scenario A and scenario B, define test applicability rule as if UE passes the test of scenario A, it can skip the test of scenario B.

- Recommended WF

  - Encourage feedback from companies

Issue 2-1-3: Doppler Frequency for Bi-directional scenario

- Observations:

  - Observation 1(Huawei):  It is feasible to use SSB+TRS for tracking frequency offset for downlink to support 350km/h, using SSB for coarse frequency offset estimation and TRS for precise frequency offset estimation.

  - Observation 2(Intel):

    - Option 1 contradicts with previous agreement to not take into account any extra UE frequency error margins to derive max supported Doppler frequency.

    - Conventional UE implementation assumes implementation of SSB based time/frequency estimation.

    - UEs without SSB based time/frequency re-synchronization after Tx beam (SSB) switching can operate in certain scenarios.

- Proposals

  - Option 1 (CMCC, Ericsson, Intel, Huawei, Samsung): 9722Hz,  and the assumption of RS for tracking is up to UE implementation

  - Option 2 (Samsung): Define two sets of PDSCH requirement for Bi-directional scenario with two Doppler values, up to UE capability

    - 9722Hz

    - 7000Hz

- Recommended WF

  - Encourage feedback from companies

Issue 2-1-4: UE capability

- Proposals

  - Option 1 (Huawei):

    - Do not introduce the UE capability to differentiate requirement for Bi/Uni-directional.

  - Option 2 (Intel, Samsung): if found to be needed, define UE capability to support operation in HST FR2 bidirectional deployments with higher than 250 km/h speed. Define corresponding performance requirements with up to UE capability

    - Define PDSCH requirements according to Table 1 or 2 depending on conclusion for bi-directional scenario with different speeds.

- Recommended WF

  - Encourage feedback from companies

Issue 2-1-5: Test applicability rule for two sets requirement for Bi-directional

- Proposals

  - Option 1(Intel): If bi-directional requirements will be defined for two different speeds (according to Table 2 define applicability rule that if UE has passed requirement with higher speed it does not need to pass requirement with lower speed.

- Recommended WF

  - Encourage feedback from companies

Issue 2-1-6: DPS transmission schemes for Uni-directional RRH scenario

- Observations

  - Observation  1 (ZTE):

    - The transmission scheme is highly related to the number of beam which is still under discussion in RRM session

  - Observation  2 (Intel):

    - There is no difference between deployment scenarios A and B from the baseband receive processing perspective and provided demodulation performance.

- Proposals

  - Option 1 (Intel, Samsung, Ericsson, Huawei, CMCC): Both DPS scheme 1a and DPS scheme 1b with same applicability rule as Rel-16 FR1 HST

    - Option 1a (Intel, Samsung): DPS scheme 1a for Uni-directional scenario A, DPS scheme 1b for Bi-directional scenario B, with the same applicability rule as Rel-16 FR1 HST

    - Option 2a (Ericsson): RAN4 should choose one of scenarios for DPS transmission schemes for Uni-directional scenario.

  - Option 2 (ZTE): Scheme 1a can be defined for scenario-A Uni-directional deployment if the number of beam for scenario-A Uni-directional deployment is one, otherwise scheme 1b is to be defined.

- Recommended WF

  - Introduce PDSCH requirement with  both DPS scheme 1a and DPS scheme 1b  for Uni-directional scenario with same applicability rule as Rel-16 FR1 HST

    - Option 1:  DPS scheme 1a and scheme 1b for both scenario A and scenario B with 4 test cases

    - Option 2:  DPS scheme 1a and scheme 1b for scenario A only with 2 test cases

    - Option 3:  DPS scheme 1a and scheme 1b for scenario B only with 2 test cases

    - Option 4: DPS scheme 1a for scenario A, and DPS scheme 1b for scenario B, with 2 test cases

  - Encourage feedback from companies whether option 4 can be acceptable with considering the test effort and test purpose?

Issue 2-1-7: DPS transmission schemes for Bi-directional RRH scenario

- Observations

  - Observation 1 (Qualcomm):

    - To actively track ≥2 TCI States in FR2 HST Bidirectional Deployment, the UE can be required to simultaneously receive from 2 different panels (ie. when RRHs transmitting Active TCI states are placed in opposite relative directions with respect to the UE).

  - Observation 2 (Intel):

    - There are no performance benefits of using DPS scheme 1b in bidirectional deployments. On the contrary, this scheme leads to the reduction of the max achievable throughput due to scheduling restriction of the PDSCH associated with active TCI state.

Wasting of useful resource due to active panel switching and processing of TRS resources associated with non-serving RRH

- Proposals

  - Option 1 (Intel, Huawei, Samsung, Qualcomm): Only DPS scheme 1a

  - Option 2 (CMCC, Ericsson, ZTE): Both DPS scheme 1a and scheme 1b with same applicability rule as Rel-16 FR1 HST

- Recommended WF

  - Encourage feedback from companies whether option 1 can be acceptable?

Issue 2-1-8: Number of Sets of requirement for Uni-directional and Bi-directional scenario

- Proposals

  - Option 1 (Intel):

    - Define PDSCH requirements according to Table 1 or 2 depending on conclusion for bi-directional scenario with different speeds.

    - If bi-directional requirements will be defined for two different speeds (according to Table 2) define applicability rule that if UE has passed requirement with higher speed it does not need to pass requirement with lower speed.

Table 1. First possible set of requirements for definition

| Deployment |  | DPS Tx scheme | Speed | Comment | Test # |
| --- | --- | --- | --- | --- | --- |
| Uni-directional | Scenario A | 1a | 350 km/h | Mandatory | 1 |
|  | Scenario B | 1b | 350 km/h | Up to UE capability on number of active TCI states | 2 |
| Bi-directional | Scenario B | 1a | 350 km/h | Mandatory | 3 |

Table 2. First possible set of requirements for definition

| Deployment |  | DPS Tx scheme | Speed | Comment | Test # |
| --- | --- | --- | --- | --- | --- |
| Uni-directional | Scenario A | 1a | 350 km/h | Mandatory | 1 |
|  | Scenario B | 1b | 350 km/h | Up to UE capability on number of active TCI states | 2 |
| Bi-directional | Scenario B | 1a | 250 km/h | Mandatory | 3 |
|  | Scenario B | 1a | 350 km/h | If is introduced, will be up to UE capability | 4 |

- Recommended WF

  - Pending on issue 2-1-1, 2-2-1, 2-2-4 and 2-2-5

Issue 2-1-9: Deployment scenario for demodulation test for Bi-directional scenario

- Proposals

  - Option 1 (Qualcomm):

    - For PDSCH Demodulation Tests with bidirectional deployment, assume that the FR2 HST UE is connected to the 2nd nearest RRH according to the deployment model depicted in the picture as

Assume that FR2 HST UEs: - Are always connected to the 2nd-nearest RRH, to avoid coverage holes when the train is located close or right below the RRH location; - According to the principle listed below, the switching points between connecting to one RRH to another RRH are located at: - At the midpoint between 2 RRH location [(2k+1)*Ds/2] - where the UE switches between adjacent RRHs, ie. from RRH#(2k+1) to RRH#(2k) or from RRH#(2k-1) to RRH#(2k), and; - Exactly at the RRH location (ie, (2k)*Ds), - where the UE switches between non-adjacent RRH, ie. from RRH#(2k+1) to RRH#(2k-1) or viceversa;

- Recommended WF

  - Encourage feedback from companies

Issue 2-1-10: Test procedure of DPS transmission schemes

- Observations

  - Observation  1:

    - In order to avoid performance degradation, before receiving PDSCH after a TCI state switch across different RRHs with large Doppler and/or Delay jumps, the UE needs to be allowed additional time to select SSB as FO tracking RS, receive it from the RRH transmitting the target TCI and process it, before selecting back TRS as FO tracking RS, receive it and process it;

    - The UE will have to switch to SSB-based tracking to estimate the doppler jump in bidirectional deployment or the delay jump in unidirectional deployment when switching TCI across different RRHs;

    - For PDSCH Demodulation Tests, assume that UE is informed through network signaling whether the TCI State Switch happens across RRH according to the proposals in R4-2119321

- Proposals

  - Option 1 (Qualcomm):

    - During the PDSCH demodulation tests for FR2 HST DPS, after the UE switches to a new TCI do not consider throughput statistics until the UE has received and processed at least one instance of SSB, followed by one instance of TRS and including respective RS processing time.

    - According to the previous proposal, for FR2 HST DPS extend the FR1 HST DPS PDSCH allocation timeline to include TfirstSSB, TSSB proc and replace TfirstTRS with TfirstTRSafterSSB.

- Adding the following two terms to the PDSCH allocation timeline: TfirstSSB + TSSB proc - TfirstSSB is the number of slots to the first SSB transmission occasion after MAC CE command is decoded by the UE; - TSSB proc is the number of slots for SSB processing; - While TfirstTRS should be updated to TfirstTRSafterSSB to reflect this new definition: - TfirstTRSafterSSB is the number of slot to the first TRS transmission occasion available after (TfirstSSB + TSSB proc) after MAC CE command is decoded by the UE;

- Recommended WF

  - Encourage feedback from companies

Issue 2-1-11: Throughput statistics during TCI state switching

- Proposals

  - Option 1 (Qualcomm): During the PDSCH demodulation tests for FR2 HST DPS, after the UE switches to a new TCI do not consider throughput statistics until the UE has received and processed at least one instance of SSB, followed by one instance of TRS and including respective RS processing time.

- Recommended WF

  - Encourage feedback from companies

Issue 2-1-12: Test setup for PDSCH requirement

- Proposals

  - Option 1 (Ericsson):

    - Antenna configuration : 2x2

    - MCS 17, Rank 1

    - SCS =120kHz, CBW=200MHz

    - DMRS type 1 with additional DMRS symbols (i,e., DMRS configuration with 1+1+1)

    - TRS transmitted every 10ms

  - Option 2 (Samsung):

    - MCS 17 with rank 2 at start point

    - Scheduled PDSCH in TDD special slots as baseline

    - TDD pattern : 3D1S1U, S=10D:2G:2U

    - Configure the same transmission period for SSB and TRS as 20ms with following

- SSB position in burst: 1

- CRS-RS offset: 2 for CSI-RS resource 1 and 2, 3 for CSI-RS resource 3 and 4

    - Configure 2 SSBs associated with 2 CSI-RS resources sets for PDSCH requirement,  where SSB # (k mod 2), CSI-RS (for tracking) resource set # ((k mod 2) + 1), CSI-RS (for CSI acquisition) resource set # ((k mod 2) + 3) and CSI-RS (for beam refinement) resource set # ((k mod 2)+5) are transmitted by kth RRH

- Recommended WF

  - Encourage feedback from companies

Companies views’ collection for 1st round

Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Sub topic 2-1

| Company | Comments |
| --- | --- |
| XXX | Issue 2-1-1 Issue 2-1-2 Issue 2-1-3 Issue 2-1-4 Issue 2-1-5 Issue 2-1-6 Issue 2-1-7 Issue 2-1-8 Issue 2-1-9 Issue 2-1-10 Issue 2-1-11 Issue 2-1-12 |
| Intel | Issue 2-1-1: Scope of PDSCH requirements with Uni-directional RRH scenario A/B and Bi-directional RRH scenario B We do not observe performance difference between different channel models. In this case our priority is to reduce UE test efforts. In this case option 1, 4 and option 3 are fine for us. For option 3 we propose to define different DPS Tx schemes for different unidirectional scenarios like Scenario A with 1a scheme and Scenario B with 1b. In this case only one unidirectional test will be performed depending on UE capability. Issue 2-1-2: Test applicability rule for requirements of different scenarios We support proposed approach to define applicability rule between scenarios A and B. We propose to define it based on UE capability for number of active TCI states and differentiation of schemes 1a and 1b between scenario A and B. Issue 2-1-6: DPS transmission schemes for Uni-directional RRH scenario We support Option 4. It has reasonable test load compared to Option 1 and better scenario coverage compared to Option 2 and 3. Issue 2-1-7: DPS transmission schemes for Bi-directional RRH scenario There is no value to configure DPS scheme 1b under assumption of a single panel reception at UE side. UE cannot efficiently make pre-tracking of the second TCI state without long interruptions during the data reception. We can address 1b scheme in future releases when two simultaneously active panels will be considered. Issue 2-1-9: Deployment scenario for demodulation test for Bi-directional scenario The current unidirectional channel model is fully aligned with the proposed approach. Issue 2-1-10: Test procedure of DPS transmission schemes We agree that in order to address big time/frequency jump UE needs to receive and process SSB and TRS. For scheme 1a with one active TCI state UE cannot do pre-tracking so additional delay to the current HST FR1 DPS test setup should be added. For 1b scheme UE can do pre-tracking in advance and there is no need to wait new SSB/TRS resources. This is fully aligned with TCI state It is aligned with MAC CE based TCI state switching delay requirements defined in TS 38.133. Issue 2-1-11: Throughput statistics during TCI state switching Support this for scheme 1a only. Otherwise, there are no benefits of scheme 1b if the interruption time will be same regardless of the number of active TCI states. Issue 2-1-12: Test setup for PDSCH requirement We should merge Option 1 and Option 2. Our suggestion is the following: - Antenna configuration: 2x2 - MCS 17, Rank 2 - SCS =120kHz, CBW=200MHz - DMRS type 1 with additional DMRS symbols (i,e., DMRS configuration with 1+1+1) - TRS transmitted every 10ms - Scheduled PDSCH in TDD special slots as baseline - TDD pattern: 3D1S1U, S=10D:2G:2U - SSB position in burst: 1 - CRS-RS offset: 2 for CSI-RS resource 1 and 2, 3 for CSI-RS resource 3 and 4 - Configure 2 SSBs associated with 2 CSI-RS resources sets for PDSCH requirement, where SSB # (k mod 2), CSI-RS (for tracking) resource set # ((k mod 2) + 1), CSI-RS (for CSI acquisition) resource set # ((k mod 2) + 3) and CSI-RS (for beam refinement) resource set # ((k mod 2)+5) are transmitted by kth RRH So we support to configure Rank 2 and we do not need to align SSB and TRS periodicities if we define proper TCI state switching time during the test (as discussed on issues 2-1-10 and 2-1-11) |
| Ericsson | Issue 2-1-1 Option 2. As commented in Issue 1-1-1, RAN4 should discuss the purpose of the FR2 HST demodulation test cases from the assumed deployment scenarios. In our understanding, from the demodulation requirements point of view, the purpose of HST FR2 test can be verified if the channel model including: - Max Doppler shift of 9722Hz (considering 350km/h at 30GHz) - Doppler shift jump between -fmax and +fmax (considering the bi-directional deployment) - Single panel, i.e., only one active antenna panel at a time As far as the purpose is satisfied, we think any channel model is fine since we don’t see any performance difference. Issue 2-1-2 We first need to agree with the number of test cases. Issue 2-1-3 Option 1. We don’t think RAN4 need to define two requirements since there is no performance difference. Considering the minimum requirements, it is enough to define one test with fmax=9722Hz. Issue 2-1-4 Option 1. It also depends on the conclusion on the number of test case in Issue 2-1-1. Issue 2-1-5 Depends on the conclusion on Issue 2-1-3. Issue 2-1-6 We first need to agree with the channel model in Issue 2-1-1. If we agree on the unified channel model, we can define DPS 1a and 1b for this unified channel model. Since the difference between DPS 1a and 1b is only the number of active TCI states, we prefer to define 1a and 1b with the same channel model. Issue 2-1-7 We agree this WI assumes the single Rx panel and DPS 1b with bi-directional deployment scenario is not applicable. However, it depends on the test setup. If we agree with the unified channel model and it verifies the Doppler shift changes only, the transmitter in OTA test system transmits the signals only from one direction. It can be applicable for both 1a/1b. Issue 2-1-9 It depends on the conclusion on the channel model (Issue 2-1-1). Issue 2-1-10 In our understanding it is up to UE implementation whether it uses SSB and/or TRS for frequency tracking. Considering the UE using TRS, we can consider both SSB and TRS. - TfirstRS + TRS_proc - TfirstRS is the larger number of slots to the first SSB transmission and the first TRS transmission after MAC CE command is decoded by the UE - TRS_proc is the larger number of slots for SSB processing and TRS processing. Issue 2-1-11 Option 1 is fine. Issue 2-1-12 The main difference between option 1 and 2 is the rank and TRS configuration. We are open to consider MCS 17 with rank 2 according to the required SNR level. We are also fine to schedule TRS every 20ms. (Assuming TDD configuration with DDDSU) |
| Huawei | Issue 2-1-1: Scope of PDSCH requirements with Uni-directional RRH scenario A/B and Bi-directional RRH scenario B Option 1. From the Doppler trajectory we can see that the Uni-ScenarioA case can provide largest frequency offset among all cases and the Bi-ScenarioB case provide largest frequency jump among all cases. The baseband processing verification under Uni-ScenarioA can be covered by the other two cases. To ensure the test coverage and reduce the test load at the same time, we propose to only define Uni-ScenarioA case and Bi-ScenarioB case. We are also open to discuss the new channel model to combine Uni-ScenarioA and Bi-ScenarioB. Issue 2-1-2: Test applicability rule for requirements of different scenarios As discussed in Issue 2-1-1, we prefer to define Uni-ScenarioA case and Bi-ScenarioB case. In this situation, we don’t support define applicability rule between above two scenarios but support to define applicability rule for DPS 1a and DPS 1b scheme for the certain scenario. If companies have strong concern about the test effort and the applicability rule must be defined for different scenarios, then we prefer to skip Uni-ScenarioA case if Bi-ScenarioB case is passed as larger Doppler jump is observed in Bi-ScenarioB case, considering that Doppler jump is the most important point to be verified. Issue 2-1-3: Doppler Frequency for Bi-directional scenario Option 1. It is feasible to use SSB+TRS for tracking frequency offset for downlink to support 350km/h, using SSB for coarse frequency offset estimation and TRS for precise frequency offset estimation. So we prefer to only define 350km/h requirements, i.e. Doppler of 9722Hz targeting 350km/h at 30GHz for both Bi-directional and Uni-directional RRH deployment. Issue 2-1-4: UE capability This issue should be discussed later when conclusion is made in Issue 2-1-3. Issue 2-1-5: Test applicability rule for two sets requirement for Bi-directional This issue should be discussed later when conclusion is made in Issue 2-1-3. Issue 2-1-6: DPS transmission schemes for Uni-directional RRH scenario We support Option 1. Only one requirement of DPS 1a and DPS 1b for the certain scenario is tested. Issue 2-1-7: DPS transmission schemes for Bi-directional RRH scenario We are OK to not consider DPS 1b for Bi-directional deployment and define requirement in future release. Issue 2-1-9: Deployment scenario for demodulation test for Bi-directional scenario We are OK with Option 1. Issue 2-1-10: Test procedure of DPS transmission schemes For Bi-directional DPS 1a, as per TS 38.331, the new TCI state should be applied after SSB has been processed. So the PDSCH allocation timeline should only include TfirstSSB and TSSB proc. TfirstTRS or TfirstTRSafterSSB should not be considered. For Uni-directional DPS 1a and 1b, we think the same test procedure of PDSCH allocation timeline as HST FR1 should be applied. Issue 2-1-11: Throughput statistics during TCI state switching We can use similar method as HST FR1, i.e. only for DPS 1a, PDCCH and PDSCH are DTXed in these slots in which throughput statistics are not considered. As we discussed in Issue 2-1-10, for DPS 1a, we prefer to not consider throughput statistics until the UE has received and processed at least one instance of SSB. Issue 2-1-12: Test setup for PDSCH requirement We prefer to select rank 2 that is same as HST FR1. We don’t think it necessary to configure the same transmission period for SSB and TRS as 20ms. It is benefit to configure 10ms TRS in HST scenario for more precise channel estimation. |
| Qualcomm | Issue 2-1-1: Scope of PDSCH requirements with Uni-directional RRH scenario A/B and Bi-directional RRH scenario B; From the point of view of demodulation performances we agree that we do not expect large difference between Scenario A or B. However, we propose to agree on: - Define only 1 Requirement (based on Scenario A OR B) for each Deployment Type; - postpone the Scenario choice to after simulation results for both Scenarios are collected, based on the strictest requirement; Issue 2-1-2: Test applicability rule for requirements of different scenarios Based on our proposal for Issue 2-1-1, we do not need this applicability rule; Issue 2-1-3: Doppler Frequency for Bi-directional scenario Regarding the observations proposed by Intel: We did have an agreement on not considering UE frequency error but in our understanding that applied to UE FO Error impact on PDSCH performances, for which it can be considered simply as a degradation. But, if we do not considered UE error in this case there is no guarantee that the UE can track FO using TRS, with potential unbounded performance impact and not a simple degradation. So it is our view that we cannot apply that agreement to Max Doppler computation, and 5652Hz with 0.1 PPM FOE error and 10% safety margin should still be an option; We agree with observation 3 and it is our view that the reference to a 'conventional procedure for UE to use SSB to obtain rough time/frequency synchronization' is limited to the case of initial access, and so it should not be used as argument to derive an expectation on UE behaviour during connected mode measurements. According to this, we believe that the implementation of SSB re-synchronization and as such: - support of speed >250km/h should be based on UE capability, and - the according performance requirement should be defined with maximum doppler 5652Hz with 0.1 PPM FOE error and 10% safety margin (see above); Issue 2-1-4: UE capability Support Option 2, see also Issue 2-1-3 Issue 2-1-5: Test applicability rule for two sets requirement for Bi-directional Agree with Option 1; Issue 2-1-7: DPS transmission schemes for Bi-directional RRH scenario Agree with Option 1 Issue 2-1-10/11: Test procedure of DPS transmission schemes @Huawei: It is our understanding that we do not have Demodulation requirements in RAN4 for SSB-based TO/FO tracking. It is our view that we should allow time for the UE to receive and process TRS before resuming evaluating throughput performances; Issue 2-1-12: We are fine starting from a baseline assumption of MCS17/Rank2. Whether to consider PDSCH in Special Slot should be first evaluated on the base on simulation results. We support TRS periodicity of 10ms and SSB periodicity of 20 ms, since they do not need to have the same periodicity for the test; |
| ZTE | Issue 2-1-1: Scope of PDSCH requirements with Uni-directional RRH scenario A/B and Bi-directional RRH scenario B As discussed in Issue 1-1-1, if a single PDSCH demodulation test is defined it should be the worst-case test which can cover both Doppler jump and delay jump. Maybe a composite test case is considered. If it’s hard to define the composite test case we prefer to define test cases separately. Issue 2-1-2: Test applicability rule for requirements of different scenarios We can wait for the conclusion of Issue 2-1-1 Issue 2-1-3: Doppler Frequency for Bi-directional scenario With SSB + TRS, 9722 Hz can be tracked for 350km/h HST, this issue is also related to Issue 2-1-4. Issue 2-1-4: UE capability We are open for this issue, if the need of different speeds is confirmed maybe we need to define a UE capability to support different speed. Issue 2-1-5: Test applicability rule for two sets requirement for Bi-directional This issue can be discussed when Issue 2-1-3 and 2-1-4 are concluded. Issue 2-1-6: DPS transmission schemes for Uni-directional RRH scenario This issue is related to Issue 2-1-1, if channel model is defined separately, we prefer option 4 which covers both scenarios with less test cases. Issue 2-1-7: DPS transmission schemes for Bi-directional RRH scenario One question to be clarified: Should multiple receive beams be excluded for a single active panel case? Issue 2-1-9: Deployment scenario for demodulation test for Bi-directional scenario We are OK with option 1 since 2a is the baseline reached in RAN4# 100-e meeting. |
| CMCC | Issue 2-1-1: Scope of PDSCH requirements with Uni-directional RRH scenario A/B and Bi-directional RRH scenario B Our preference is to define test cases for uni-directional scenario A and B, and bi-directional scenario B, but we are open to discussion. And the applicability rule for certain cases can be considered. Issue 2-1-3: Doppler Frequency for Bi-directional scenario Option 1. For the target carrier frequency of 30GHz and target speed of 350km/h, the maximum doppler shift is 9722Hz. the assumption of RS for frequency offset tracking can be up to UE implementation. Issue 2-1-6: DPS transmission schemes for Uni-directional RRH scenario We are OK with the recommended WF to introduce PDSCH requirement with both DPS scheme 1a and DPS scheme 1b for Uni-directional scenario with same applicability rule as Rel-16 FR1 HST. As for the details, our preference is option 1, but we are fine with option 4. Issue 2-1-7: DPS transmission schemes for Bi-directional RRH scenario Option 2. Option 2 provide better test coverage and with the applicability rule, the number of test cases is not increased compared with option 1. |
| Samsung | Issue 2-1-1: Scope of PDSCH requirements with Uni-directional RRH scenario A/B and Bi-directional RRH scenario B For Uni-directional RRH deployment scenario, compared with scenario A and scenario B, although the number of beam per RRH is different, where 1 beam per RRH panel, and 2 beams per RRH panel. From the UE receiver perspective, the different is minor. Based on our initial result, similar results can be achieved. So, there is no necessary to introduce requirement for both scenario A and scenario B with same transmission scheme. At least, for Uni-directional scenario, single set requirement should be enough to verify the baseband processing. We are fine with QC proposal for Uni-directional scenario As for Bi-direction scenario, huge Doppler frequency will be experienced by UE when UE switches one RRH from another RRH. The Doppler value will be double compared with the value in Uni-directional, which may exceed the frequency Doppler estimation range by TRS. UE may has different FOE strategies. Therefore, to verify the proper time/frequency offset tracking, it is necessary to define PDSCH requirement for Bi-directional Issue 2-1-3: Doppler Frequency for Bi-directional scenario We are fine with option 1 if no companies only apply TRS for frequency/timing offset tracking. Issue 2-1-4: UE capability If only one set requirement of Bi-directional, there is no need to define the UE capability Issue 2-1-5: DPS transmission schemes for Uni-directional RRH scenario Either option 2 or option 4 are fine us. As mentioned, for Uni-directional scenario, the Doppler observed by UE in scenario A is larger than that in scenario B. While from the demodulation perspective, there is no different receiver processing foreseen. Based on our initial simulation, similar performance can be achieved for Uni-directional in scenario A and scenario B. Therefore, we prefer to define the requirement with one of them, no need to cover both scenario A and scenario B for both DPS transmission schemes with considering the test effort. From our side, we prefer to only cover scenario A with two test cases, If companies have consensus about scenario B, another solution is to define PDSCH requirement with DPS scheme 1a and DPS scheme 1b for scenario A and scenario B separately, i.e., DPS scheme 1a with scenario A and DPS scheme1b with scenario B Issue 2-1-6: DPS transmission schemes for Bi-directional RRH scenario Option 1 From test ability perspective, existing OTA test with single panel can be applied for DPS scheme 1a and DPS scheme1b, From the demodulation requirement, there is no difference for them. While as mentioned, with DPS scheme 1b, there will be performance degradation. There is no benefit compared with DPS scheme 1a. We agree Intel proposal that DPS scheme 1b can be considered in future release if two panels are simultaneously activated Issue 2-1-7: Number of Sets of requirement for Unidirectional and Bi-directional scenario Pending on previous issues Issue 2-1-8: Deployment scenario for demodulation test for Bi-directional scenario Fine with option 1 Issue 2-1-9: Test procedure of DPS transmission schemes We suggest it is better to discussion with DPS scheme 1a and scheme 1b, Uni-directional and Bi-directional scenario separately For DPS scheme 1a and 1b under Uni-directional scenario, same Tx beam are transmitted by RRH, in our view, there is no need to include T_firstSSB and Tssb_proc, For DPS scheme 1a, since UE cannot do pre-tracking for new TCI state, it seems that additional delay for SSB processing Issue 2-1-10: Throughput statistics during TCI state switching Generally, we are ok that there is no PDSCH scheduling during the TCI switching, there is to align with FR1 DPS scheme. While detail condition should be discussed based on different DPS schemes and scenario Issue 2-1-11 Test setup for PDSCH requirement In FR2 HST, each RRH will send at least one SSB with configuring one CSI-RS resource, which has the QCL property, especially for QCL-D From beam switching procedure, UE should wait the next available SSB occasion for timing/frequency tracking. After beam switching successfully, UE can receive PDCCH/PDSCH with new TCI-state. With TRS period is 10ms, during beam switching procedure , it may result in the first resource is TRS and not SSB, if not Rx beam information based on SSB , how can UE know the Rx beam for TRS In existed FR2 PDSCH demodulation test, the default SSB period configuration is 20ms, TRS period configuration is 20ms, we can use as a starting point. |

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Issue 2-1-2: Doppler Frequency for Bi-directional scenario Candidate options: - Option 1 (CMCC, Ericsson, Intel, Huawei, Samsung, ZTE): 9722Hz, and the assumption of RS for tracking is up to UE implementation - Option 2 (Samsung): Two sets of requirement pending on UE capability - 9722Hz - 7000Hz - Option 3 (Qualcomm): Two sets of requirement pending on UE capability - 9722Hz - 5652Hz with 0.1ppm FOE error and 10% safety margin Recommendations for 2nd round: - If time allowed Further check in GTW session Issue 2-1-3: UE capability Candidate options: - Option 1 (Huawei): - Do not introduce the UE capability to differentiate requirement for Bi/Uni-directional. - Option 2 (Intel, Samsung, Qualcomm): if found to be needed, define UE capability to support operation in HST FR2 bidirectional deployments with higher than 250 km/h speed. Define corresponding performance requirements with up to UE capability - Define PDSCH requirements according to Table 1 or 2 depending on conclusion for bi-directional scenario with different speeds. Recommendations for 2nd round: - Pending on whether to define two sets of requirement for Bi-directional scenario Issue 2-1-4: Test applicability rule for two sets requirement for Bi-directional rule Candidate options: - Option 1(Intel): If bi-directional requirements will be defined for two different speeds (according to Table 2 define applicability rule that if UE has passed requirement with higher speed it does not need to pass requirement with lower speed. Recommendations for 2nd round: - Pending on whether to define two sets requirement for Bi-directional scenario Issue 2-1-5: DPS transmission schemes for Uni-directional RRH scenario Candidate options: - Introduce PDSCH requirement with both DPS scheme 1a and DPS scheme 1b for Uni-directional scenario with same applicability rule as Rel-16 FR1 HST - Option 1: DPS scheme 1a and scheme 1b for both scenario A and scenario B with 4 test cases - Option 2: DPS scheme 1a and scheme 1b for scenario A only with 2 test cases - Option 3: DPS scheme 1a and scheme 1b for scenario B only with 2 test cases - Option 4: DPS scheme 1a for scenario A, and DPS scheme 1b for scenario B, with 2 test cases Recommendations for 2nd round: - Further discussion in 2nd round, Pending on the test scope discussion of UE demod Issue 2-1-6: DPS transmission schemes for Bi-directional RRH scenario Candidate options: - Option 1 (Intel, Huawei, Samsung, Qualcomm): Only DPS scheme 1a - Option 2 (CMCC, Ericsson, ZTE): Both DPS scheme 1a and scheme 1b with same applicability rule as Rel-16 FR1 HST Recommendations for 2nd round: - Based on WID, only single Rx panel activate at one time, DPS scheme 1b with bi-directional show less benefit compared with DPS scheme 1a - For moving forward, moderator suggest the following compromise solution - Introduce PDSCH requirement only with DPS scheme 1a for Bi-directional scenario - Introduce PDSCH requirement with DPS scheme 1b in future release for Bi-directional scenario when two simultaneously enabled. - If time allowed, further check in GTW session Issue 2-1-7: Number of Sets of requirement for Uni-directional and Bi-directional scenario Candidate options: - Option 1 (Intel): - Define PDSCH requirements according to Table 1 or 2 depending on conclusion for bi-directional scenario with different speeds. - If bi-directional requirements will be defined for two different speeds (according to Table 2) define applicability rule that if UE has passed requirement with higher speed it does not need to pass requirement with lower speed. Table 1. First possible set of requirements for definition Deployment; DPS Tx scheme; Speed; Comment; Test #. Uni-directional; Scenario A; 1a; 350 km/h; Mandatory; 1. Scenario B; 1b; 350 km/h; Up to UE capability on number of active TCI states; 2. Bi-directional; Scenario B; 1a; 350 km/h; Mandatory; 3. Table 2. First possible set of requirements for definition Deployment; DPS Tx scheme; Speed; Comment; Test #. Uni-directional; Scenario A; 1a; 350 km/h; Mandatory; 1. Scenario B; 1b; 350 km/h; Up to UE capability on number of active TCI states; 2. Bi-directional; Scenario B; 1a; 250 km/h; Mandatory; 3. Scenario B; 1a; 350 km/h; If is introduced, will be up to UE capability; 4. Recommendations for 2nd round: - Pending on the test scope discussion of UE demod Issue 2-1-8: Deployment scenario for demodulation test for Bi-directional scenario Candidate options: - Option 1 (Intel, Qualcomm, Samsung, ZTE, Huawei): - For PDSCH Demodulation Tests with bidirectional deployment, assume that the FR2 HST UE is connected to the 2nd nearest RRH according to the deployment model depicted in the picture as Assume that FR2 HST UEs: - Are always connected to the 2nd-nearest RRH, to avoid coverage holes when the train is located close or right below the RRH location; - According to the principle listed below, the switching points between connecting to one RRH to another RRH are located at: - At the midpoint between 2 RRH location [(2k+1)*Ds/2] - where the UE switches between adjacent RRHs, ie. from RRH#(2k+1) to RRH#(2k) or from RRH#(2k-1) to RRH#(2k), and; - Exactly at the RRH location (ie, (2k)*Ds), - where the UE switches between non-adjacent RRH, ie. from RRH#(2k+1) to RRH#(2k-1) or viceversa;. Recommendations for 2nd round: - Further discussion in 2nd round, pending on the channel model for requirement Issue 2-1-9: Test procedure of DPS transmission schemes Candidate options: - Option 1 (Qualcomm): - During the PDSCH demodulation tests for FR2 HST DPS, after the UE switches to a new TCI do not consider throughput statistics until the UE has received and processed at least one instance of SSB, followed by one instance of TRS and including respective RS processing time. - According to the previous proposal, for FR2 HST DPS extend the FR1 HST DPS PDSCH allocation timeline to include TfirstSSB, TSSB proc and replace TfirstTRS with TfirstTRSafterSSB. - Adding the following two terms to the PDSCH allocation timeline: TfirstSSB + TSSB proc - TfirstSSB is the number of slots to the first SSB transmission occasion after MAC CE command is decoded by the UE; - TSSB proc is the number of slots for SSB processing; - While TfirstTRS should be updated to TfirstTRSafterSSB to reflect this new definition: - TfirstTRSafterSSB is the number of slot to the first TRS transmission occasion available after (TfirstSSB + TSSB proc) after MAC CE command is decoded by the UE;. - Option 2 (Ericsson): Considering UE using TRS , - TfirstRS + TRS_proc - TfirstRS is the larger number of slots to the first SSB transmission and the first TRS transmission after MAC CE command is decoded by the UE - TRS_proc is the larger number of slots for SSB processing and TRS processing - Option 3 (Huawei): - Bi-directional DPS scheme 1a, PDSCH allocation timeline should only include TfirstSSB and TSSB proc - Uni-directional DPS scheme 1a and 1b, reuse the same test procedure as FR1 HST - Option 4(Intel, Samsung) - DPS Scheme 1a, additional delay should be considered - DPS Scheme 1b, PDSCH allocation timeline should only include TfirstSSB and TSSB proc Recommendations for 2nd round: - Suggest to discussion PDSCH allocation time line based on DPS sheme1a, scheme1b for Unidirectional and Bi-directional scenario separately. - If time allowed, further check in GTW session Issue 2-1-10: Throughput statistics during TCI state switching Candidate options: - Option 1 (Qualcomm, Ericsson): During the PDSCH demodulation tests for FR2 HST DPS, after the UE switches to a new TCI do not consider throughput statistics until the UE has received and processed at least one instance of SSB, followed by one instance of TRS and including respective RS processing time. - Option 2 (Huawei, Intel): reuse the similar method as FR1 HST, only for DPS1a, PDCCH and PDSCH are DTXed in these slots in which throughput statistics are not considered. Not consider throughput statistics until the UE has received and processed at least one instance of SSB Recommendations for 2nd round: - Suggest to discussion PDSCH allocation time line based on DPS sheme1a, scheme1b for Unidirectional and Bi-directional scenario separately - Further discussion in the 2nd round Issue 2-1-11: Test setup for PDSCH requirement Candidate options: - Antenna configuration: 2x2 - DMRS type 1 with additional DMRS symbols (i,e., DMRS configuration with 1+1+1) - SCS =120kHz, CBW=200MHz - SSB position in burst: 1 - CRS-RS offset: 2 for CSI-RS resource 1 and 2, 3 for CSI-RS resource 3 and 4 - Configure 2 SSBs associated with 2 CSI-RS resources sets for PDSCH requirement, where SSB # (k mod 2), CSI-RS (for tracking) resource set # ((k mod 2) + 1), CSI-RS (for CSI acquisition) resource set # ((k mod 2) + 3) and CSI-RS (for beam refinement) resource set # ((k mod 2)+5) are transmitted by kth RRH - TDD pattern: 3D1S1U, S=10D:2G:2U - Scheduled PDSCH in TDD special slots as baseline - MCS and Rank - MCS 17, Rank 2 as starting point - SSB and TRS period configuration : - Option 1(Huawei, Qualcomm, Intel): 20ms for SSB, 10ms for TRS - Option 2(Samsung, Ericsson): 20ms for both SSB and TRS - Other options are not precluded Recommendations for 2nd round: - Further discussion in the 2nd round |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Topic #3: BS demodulation requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117409 | CATT | Proposal 1: To adopt 2 DMRS+PT-RS (L=1, K=2) and 3 DMRS+PTRS (L=1, K=2) with test applicability rule based on BS manufacturer declaration (Option 2). Proposal 2: To adopt MCS16 (Option 1). Proposal 3: To adopt FOE method is up to BS implementation (Option 2). |
| R4-2117410 | CATT | Proposal 1: To adopt A= 1.25 us, Δω = 1.04s-1 corresponding to 120KHz SCS for HST FR2 UL timing adjustment requirements (Option 1). |
| R4-2117411 | CATT | Proposal 1: To reuse Rel-15 timing offset configuration for PRACH, i.e., 0.8 us timing offset, and starting from 0us to 0.8us with 0.1 us step (Option 1). |
| R4-2117591 | Samsung | Simulation results summary |
| R4-2117594 | Samsung | Observation 1: The overhead of 1DMRS +PTRS (L=1, K=2) configuration is the smallest compared with other RS configuration schemes. Observation 2: similar performance can be achieved with 2 DMRS and 3 DMRS configuration Observation 3: Better performance can be achieved for bi-directional scenario in scenario B compared with Uni-directional scenario Proposal 1: Define PUSCH demodulation requirement with both 1 DMRS +PT-RS (L=1, K=2) and 2 DMRS +PT-RS (L=1, K=2) configurations. Apply the same test applicability rule for BS conformance test, based on BS manufacturer declaration. Proposal 2: FOE method is up to BS implementation. Choose the worst case for requirement definition. Proposal 3: Define PUSCH requirement with MCS 17, if no OTA testability issue. Proposal 4: Both MCS 16 and MCS 17 can be considered for FR2 HST PUSCH requirement. |
| R4-2117595 | Samsung | Observation 1: No Doppler Shift is taken into account for scenario Y and scenario Z for HST Proposal 1: Align RS configuration with PUSCH and UL timing adjustment requirement, and define demodulation requirement with configured both 1 DMRS and 2 DMRS. And apply the Rel-15 test applicability rule for BS conformance test, based on BS manufacturer declaration. Proposal 2: The SRS transmission location is the last symbol in slot#3 in radio frame. Observation 2: The propagation delay from two neighboring RRHs in Uni-directional scenario is larger than the CP length. Observation 3: With larger TA command, the maximum adjustment can be up to 2us, which is still smaller than the maximum sudden change as 2.5us. The remaining propagation delay is about 0.5us, which is close to the length of CP. Proposal 3: Use test parameter A=1.25us, , Δw= 1.04s-1 for UL timing adjustment requirement Proposal 4: The timing different between moving UE and stationary UE as Δt-(TA-31)x16*8Tc can be considered for UL timing adjustment requirement Observation 5: Similar performance can be achieved with different RS configurations |
| R4-2117596 | Samsung | Observation 1: Minor performance different between maximum timing offset as 0.8 and 4.6us Proposal 1: Reuse Rel-15 FR2 timing offset configuration for PRACH, i.e., 0.8us. |
| R4-2118009 | Intel | Proposal 1: Define PUSCH requirements with MCS 20 and pre-FFT FOC method. Define corresponding manufacturer declaration on supporting high MCS values for HST FR2 and apply requirements on MCS 20 only based on this declaration. Proposal 2: Define PUSCH requirements with MCS 16 and post-FFT FOC method. Proposal 3: Consider 1 DMRS+PT-RS (L=1, K=2) and 2 DMRS+PTRS (L=1, K=2) configurations for requirements definition with test applicability rule based on BS manufacturer declaration. Proposal 4: Introduce BS manufacturer declaration for applicable test case. Proposal 5: Introduce applicability rule that allows BS to pass only one scenario and other scenarios will be passed without explicit testing. |
| R4-2118010 | Intel | Observation 1: If one-shot UE autonomous uplink timing adjustment will be considered to address timing issues in uni-directional deployments, there is no need to revise UL TA channel model |
| R4-2118011 | Intel | Observation 1: Performance difference between scenario with 0.8us TO and 4.78us TO is about 0.1 dB at 1% of miss detection rate. Proposal 1: Define HST FR2 PRACH requirements with 4.78us max TO value. |
| R4-2118235 | Ericsson | Proposal 1: Performance requirements are based on MCS20 Proposal 2: Assume post-FFT compensation when developing the requirement Proposal 3: For DM-RS, either option 1 or option 3, preferably option 1. Proposal 4: Test bi-directional using a single BS panel if needed Proposal 5: Create a single PUSCH demodulation requirement for all scenarios |
| R4-2118236 | Ericsson | Proposal 1: If the TA test includes the timing jump associated with Scenario A uni-directional, enable that optionally the BS is informed when the timing jump occurs. Proposal 2: IF there is a TA model including the timing jump change, then BS should declare whether they support Scenario A uni-directional and the associated requirement; for BS that do not support then the previously agreed TA requirement should be applicable. |
| R4-2118237 | Ericsson | Simulation results |
| R4-2119017 | Huawei | - Observation 1: When the BS select pre FOC implementation, - There is negligible difference between different RS configuration. - Observation 2: When the BS select post FOC implementation, - There is negligible difference between RS configuration of DMRS 1+1 and DMRS 1+1+1. - There is a large difference between RS configuration with additional DMRS and without additional DMRS. - Maximum throughput cannot be achieved for MCS 20. - Observation 3: For the residual frequency offset for FOE, - There is negligible difference between RS configuration of DMRS 1+1 and DMRS 1+1+1. - There is a large difference between RS configuration with additional DMRS and without additional DMRS - Proposal 1: Only define Uni-ScenarioA case and Bi-ScenarioB case. - Proposal 2: Use post-FOE as the worst case for PUSCH demodulation requirements for FR2 HST. - Proposal 3: Use MCS 16 for PUSCH demodulation requirements for FR2 HST. - Proposal 4: Define one set of performance requirements with either DMR-RS 1+1 or 1+1+1 based on BS declaration, like Rel-16 2-step RACH WI did. |
| R4-2119018 | Huawei | Observation 1: For the current channel model for the UL timing case, the transient timing jump caused by beam switching from one RRH to another RRH under HST FR2 scenario still cannot be simulated. Proposal 1: Discuss timing jump issue later when conclusions are made in RRM session. If large timing jump cannot be verified in RRM session, the channel model for the UL timing case should be changed to involve the timing jump, e.g.: |
| R4-2119019 | Huawei | Proposal 1: Reuse Rel-15 FR2 timing offset configuration for PRACH, i.e., [0, 0.1...0.8] us. |
| R4-2119389 | Nokia | On PUSCH performance in HST FR2 deployments: - Observation 1: The PUSCH performance results with the same MCS and DM-RS pattern are almost the same in all propagation models/scenarios. The performance difference between MCS16 and MCS17 is not very significant, i.e., less than 1dB for SNR at 70% of maximum throughput. Observation 2: The impact of phase noise at higher MCS in HST FR2 scenario can be noticeable only if no phase noise compensation algorithm is used at the receiver, especially if only one DM-RS symbol per slot is transmitted. If a phase noise compensation algorithm is enabled, then no significant degradation in PUSCH performance is observed for MCS20. Proposal 1: Keep the former agreement that the phase noise impact can be included in the impairment results, but it is left up to companies. No explicit phase noise modelling in the alignment results is needed. On PUSCH requirement parameters: Proposal 2: It is beneficial to introduce PUSCH requirements for 1 DMRS+PT-RS (L=1, K=2) and 3 DMRS+PTRS (L=1, K=2) with test applicability rule based on BS manufacturer declaration (Option 3). Proposal 3: RAN4 to introduce PUSCH requirements with MCS16 and MCS20 (Option 1 and Option 3). On Test setup for PUSCH requirements: Observation 3: Even though it was agreed that no dedicated PUSCH requirement is needed for bi-directional deployment for Scenario-A, it is beneficial to equip RRHs with two panels oriented in the opposite directions to provide reliable mobility and higher system performance for the trains moving in both directions. Such deployment can be still tested as uni-directional, however, currently assumed scenario where the train is moving in the direction opposite to the RRH panel orientation is not needed any more. Proposal 4: RAN4 to consider the configuration where the train is moving from the serving beam (i.e., in the same direction with RRH antenna panel orientation) as a basis for HST FR2 requirements in uni-directional deployments. Observation 4: In our opinion, there is no much value in introduction of the unified channel model because the behaviour of uni-directional and bi-directional models are fundamentally different. Observation 5: The use of simple but characteristic single-tap channel model with Doppler shift sign alternation was proposed by us (Option 2e at the RAN4#99-e [R4-2108661]), however, it didn’t get enough support in the discussions. Observation 6: Bi-directional channel model can be considered as the worst-case for the formulation of PUSCH performance requirements. - Observation 7: Since PUSCH performance in uni-directional model with - Scenario-A and Scenario-B parameters are practically the same, there is no - much value in performing both test. However, both requirements might be still - needed if only one of the deployments is supported by the manufacturer. Proposal 5: RAN4 to consider the following options defining PUSCH performance requirements in HST FR2 deployment: - Option 1: Define only requirements based on bi-directional model with Scenario-B parameters as a worst-case scenario. - Option 2: Define requirements for all three channel models: uni-directional with Scenario-A and Scenario-B parameters, and bi-directional with Scenario-B parameters. Proposal 6: Define applicability rules in such a way that if bi-directional Scenario-B is tested then no other tests are needed. Proposal 7: Define applicability rules in such a way that it is sufficient to test only one of the uni-directional deployments by manufacturer choice. |
| R4-2119390 | Nokia | - Observation 1: In the current (e.g., HST FR1) PUSCH timing adjustment test it - is assumed that the UEs are in CONNECTED state. The range of timing - adjustment that can be signaled with TAC MAC CE is ±2.1μs, and proposed test parameters for HST FR2 (A= 1.25 us, Δω = 1.04s-1) keeps Δτ within this range. Observation 2: The jump in RTT of -4.8us that happens at RRH change does not match the ±2.1μs range of timing adjustments that can be signalled with TAC MAC CE. Whereases, with the current model the change in propagation delay Δτ happen continuously. Therefore, - increasing the value of parameter A will not bring any additional value in the current test - if the jump in RTT is added into the moving propagation condition then the feedbacked TAC will not be able to compensate for it and the test cannot be considered applicable. Observation 3: Regardless of the type of solution that is agreed in RRM, we do not expect any changes in the situation when the CPE stays connected to the same RRH. Therefore, the current test should be still applicable for HST FR2 deployment. Proposal 1: If the test setup (inc. moving propagation conditions) for PUSCH timing adjustment requirements is left as it is then test parameters for timing offset (A= 1.25 us, Δω = 1.04s-1) should be selected (Option 1). Observation 4: Depending on RRM agreement, the scope of the solution to the problem of propagation delay difference between source and targets RRH at beam change might go beyond the scope of current UL TA PUSCH demodulation performance requirements. Therefore, proper testing of the solution might require a new testing setup. Proposal 2: Discuss if a new requirement and setup is needed for testing of UL timing at beam change between non-collocated RRHs based on the results of RRM discussion. |
| R4-2119391 | Nokia | Observation 1: The maximum time offset of the received PRACH preamble can be up to the maximum roundtrip from RRH to the CPE and equals 4.6us or 4.8us depending on the scenario. These values are within the maximum timing offset that can be tolerated with the agreed HST FR2 PRACH configuration (C2 format, Ncs=0, 120kHz SCS). Observation 2: There is not meaningful difference in the SNR levels corresponding to the 99% probability of PRACH preamble misdetection with 0.8 us, 4.6 us and 4.8 us maximum timing offset. Proposal 1: RAN4 define PRACH requirements with maximum timing offset matching the HST FR2 deployment, i.e., Option 2. Proposal 2: Timing offset configuration can be selected to reflect the maximum timing offset in Scenario-B, i.e., 4.8 us, with TO range [0:0.48:4.8]. |

Open issues summary

Last RAN4 meeting agreements in the WF R4-2115725 and R4-2115726

List of open issues

- Sub-Topic 3-1: PUSCH requirements

  - Issue 3-1-1: Scope of PUSCH requirement with Uni-directional RRH scenario A/B and Bi-directional RRH scenario B

  - Issue 3-1-2: Test Applicability rule

  - Issue 3-1-3: RS configuration

  - Issue 3-1-4: Frequency offset Estimation implementation

  - Issue 3-1-5: MCS

  - Issue 3-1-6: Phase noise modelling

- Sub-Topic 3-2: UL timing adjustment requirements

  - Issue 3-2-1: RS configuration

  - Issue 3-2-2: Test Parameters for timing offset

  - Issue 3-2-3: Timing different between moving UE and stationary UE

  - Issue 3-2-4: SRS transmission configuration

- Sub-Topic 3-3: PRACH requirements

  - Issue 3-3-1: Timing offset configuration

Sub-topic 3-1 PUSCH requirements

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 3-1-1: Scope of PUSCH requirement with Uni-directional RRH scenario A/B and Bi-directional RRH scenario B

- Observations

  - Observation 1 (Nokia)

    - Even though it was agreed that no dedicated PUSCH requirement is needed for bi-directional deployment for Scenario-A, it is beneficial to equip RRHs with two panels oriented in the opposite directions to provide reliable mobility and higher system performance for the trains moving in both directions. Such deployment can be still tested as uni-directional, however, currently assumed scenario where the train is moving in the direction opposite to the RRH panel orientation is not needed any more

    - Bi-directional channel model can be considered as the worst-case for the formulation of PUSCH performance requirements.

    - Since PUSCH performance in Uni-directional model with Scenario-A and Scenario-B parameters are practically the same, there is no much value in performing both test. However, both requirements might be still needed if only one of the deployments is supported by the manufacturer.

- Proposals

  - Option 1 (Huawei):

    - Only define PUSCH requirements with Uni-directional RRH scenario-A and Bi-directional RRH scenario-B

  - Option 2:

    - Option 2a (Nokia): Define only requirements based on bi-directional model with Scenario-B parameters as a worst-case scenario.

    - Option 2b (Ericsson): Set a single PUSCH requirement for all scenarios (A, B Uni-directional, B Bi-directional). Passing the requirement demonstrates minimum performance can be achieved by the baseband in any of the scenarios.

    - Option 2c (Samsung): Define PUSCH with one requirement as Uni-directional RRH deployment scenario in scenario A for FR2 HST

  - Option 3 (Intel, Nokia): Define requirements for all three channel models: Uni-directional with Scenario-A and Scenario-B parameters, and bi-directional with Scenario-B parameters with test applicability rule based on BS manufacturer declaration

- Recommended WF

  - 4 companies have provided the simulation comparison among all scenarios. Minor difference is foreseen based on 3 companies’ results.

    - Intel

    - Huawei

| Case number | Scenario | SNR@70%maximum throughput (dB) |
| --- | --- | --- |
| 1 | Uni-ScenarioA | 12.76 |
| 2 | Uni-ScenarioB | 11.19 |
| 3 | Bi-ScenarioB | 11.73 |

    - Nokia

| BW | MCS | RS | Uni-directional A | Uni-directional B | Bi-directional B |
| --- | --- | --- | --- | --- | --- |
| 50 | MCS16 | 1 | 6.64 | 6.69 | 6.71 |
|  | MCS17 |  | 7.24 | 7.35 | 7.41 |
|  | MCS20 |  | 9.78 | 10.08 | 10.15 |
|  | MCS16 | 1+1 | 6.30 | 6.51 | 6.57 |
|  | MCS17 |  | 7.07 | 7.16 | 7.18 |
|  | MCS20 |  | 9.79 | 10.09 | 10.16 |
|  | MCS16 | 1+1+1 | 6.26 | 6.40 | 6.47 |
|  | MCS17 |  | 7.14 | 7.19 | 7.21 |
|  | MCS20 |  | 9.81 | 10.10 | 10.17 |

    - Samsung

| BW | MCS | RS | Uni-directional A | Uni-directional B | Bi-directional B |
| --- | --- | --- | --- | --- | --- |
| 200 | MCS16 | 1 | 15.3 | 13.1 | 9.7 |
|  | MCS17 |  | 23.7 | 16.2 | 11.1 |
|  | MCS20 |  | N.A | N.A | 17.7 |
|  | MCS16 | 1+1 | 14.7 | 12.7 | 9.5 |
|  | MCS17 |  | 19.5 | 15.1 | 10.7 |
|  | MCS20 |  | 9.79 | 10.09 | 17.2 |
|  | MCS16 | 1+1+1 | 14.6 | 12.7 | 9.5 |
|  | MCS17 |  | 21.7 | 15.7 | 10.9 |
|  | MCS20 |  | N.A | N.A | 17.5 |

Issue 3-1-2: Test Applicability rule

- Proposals

  - Option 1 (Intel):

    - Introduce BS manufacturer declaration for applicable test case

    - Introduce applicability rule that allows BS to pass only one scenario and other scenarios will be passed without explicit testing.

  - Option 2 (Nokia):

    - Option 1a: Define applicability rules in such a way that if bi-directional Scenario-B is tested then no other tests are needed.

    - Option 1b: Define applicability rules in such a way that it is sufficient to test only one of the Uni-directional deployments by manufacturer choice.

  - Option 3 (Samsung):

    - If both scenarios A and B are introduced, the test is performed based on BS manufacture declaration. If BS declared to support both scenario A and scenario B and BS can pass the test of Uni-directional for scenario A, it can skip the test of Uni-directional scenario for scenario B. If BS declared to support both Uni-directional and Bi-directional operation, and BS can pass the test of Uni-directional for scenario A, it can skip the test of Bi-directional scenario for scenario B.

- Recommended WF

  - Encourage feedback from companies

Issue 3-1-3: RS configuration

- Observations

  - Observation 1 (Huawei):

    - For the residual frequency offset for FOE

- There is negligible difference between RS configuration of DMRS 1+1 and DMRS 1+1+1.

- There is a large difference between RS configuration with additional DMRS and without additional DMRS

  - Observation 2 (Samsung):

    - The overhead of 1DMRS +PTRS (L=1, K=2) configuration is the smallest compared with other RS configuration schemes.

Overhead of RS with different RS configurations

| RS configuration | BW (MHz) | Position of DMRS | Overhead of DMRS (length of data is 10, number of CDM group =2 Mapping type B | Overhead of PTRS (length of data is 10, number of CDM group=2, mapping type B) | Total |
| --- | --- | --- | --- | --- | --- |
| 1 DMRS +PTRS (L=1,K=2) | 200 | {0} | 792/15840 | 594/15840 | 1386/15840 |
| 2DMRS + PTRS (L=1, K=2) | 200 | {0,8} | 1584/15840 | 528/15840 | 2112/15840 |
| 3DMRS+ PTRS (L=1, K=2) | 200 | {0,4,8} | 2376/15840 | 462/15840 | 2838/15840 |
| 1 DMRS +PTRS (L=1,K=2) | 50 | {0} | 192/3840 | 144/3840 | 336/3840 |
| 2DMRS + PTRS (L=1, K=2) | 50 | {0,8} | 384/3840 | 128/3840 | 512/3840 |
| 3DMRS+ PTRS (L=1, K=2) | 50 | {0,4,8} | 576/3840 | 112/3840 | 688/3840 |

    - Similar performance can be achieved with 2 DMRS and 3 DMRS configuration

  - Observation 3 (Ericsson)

    - Higher absolute throughput can be obtained with 1 DM-RS compared to 2 or 3 DM-RS

- Proposals

  - Option 1 (Samsung, Intel, Ericsson): 1 DMRS + PT-RS (L=1,K=2) and 2 DMRS+PT-RS (L=1,K=2) with test applicability rule based  on BS manufacturer declaration

  - Option 2 (CATT): 2 DMRS + PT-RS (L=1, K=2) and 3 DMRS +PT-RS (L=1,K=2) with test applicability rule based on BS manufacturer declaration

    - Option 2a (Huawei): Define one set of requirements with either DMRS 1+1 or DMRS 1+1+1 based on BS declaration, like Rel-16 2-step RACH WI did

  - Option 3 (Nokia): 1 DMRS + PT-RS(L=1, K=2) and 3 DMRS +PT-RS (L=1,K=2) with test applicability rule based on BS manufacturer declaration

- Recommended WF

  - 5 companies have provided the initial simulation results as following. The performance difference between different RS configuration is very minor, similar performance can achieved with 2 DMRS and 3 DMRS configurations no matter Pre-FFT FOE operation or Post-FFT FOE operation for each companies result. From overhead of RS perspective, 1 or 2 DMRS has less overhead compared with 3 DMRS, which can further improve the system performance.  Encourage companies to check whether option 1 is acceptable?

  - Large performance is existed between companies, even for Post- FFT FOE operation. Meanwhile, the details simulation assumption for BW and channel condition is indicated in company’s contributions. Encourage companies to further align the simulation assumption and check the simulation results

Table   PUSCH results with Bi-directional Scenario B

| FOE | BW | MCS | RS | Samsung | Huawei | Nokia ? | Intel | Ericsson ? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Post-FFT | 200 | MCS16 | 1 | 9.7 | 13.25 |  | 17.9 | 8.2 |
|  |  | MCS17 |  | 11.1 | 16.55 |  | 23.3 | 9.5 |
|  |  | MCS20 |  | 17.7 | N/A |  | N.A | 15.0 |
|  |  | MCS16 | 1+1 | 9.5 | 11.92 |  | 17 | 8.0 |
|  |  | MCS17 |  | 10.7 | 13.65 |  | 19.9 | 8.3 |
|  |  | MCS20 |  | 17.2 | N/A |  | N.A | 15.0 |
|  |  | MCS16 | 1+1+1 | 9.5 | 11.73 |  | 16.9 |  |
|  |  | MCS17 |  | 10.9 | 14.04 |  | 21 |  |
|  |  | MCS20 |  | 17.5 | N/A |  | N.A |  |
| Pre-FFT | 50 | MCS16 | 1 |  |  | 6.71 |  |  |
|  |  | MCS17 |  |  |  | 7.41 |  |  |
|  |  | MCS20 |  |  |  | 10.15 |  |  |
|  |  | MCS16 | 1+1 |  |  | 6.57 |  |  |
|  |  | MCS17 |  |  |  | 7.18 |  |  |
|  |  | MCS20 |  |  |  | 10.16 |  |  |
|  |  | MCS16 | 1+1+1 |  |  | 6.47 |  |  |
|  |  | MCS17 |  |  |  | 7.21 |  |  |
|  |  | MCS20 |  |  |  | 10.17 |  |  |
|  | 200 | MCS16 | 1 |  | 6.71 |  | 8.9 |  |
|  |  | MCS17 |  |  | 7.43 |  | 10.8 |  |
|  |  | MCS20 |  |  | 10.14 |  | 12.8 |  |
|  |  | MCS16 | 1+1 |  | 6.40 |  | 8.8 |  |
|  |  | MCS17 |  |  | 6.65 |  | 10.3 |  |
|  |  | MCS20 |  |  | 9.76 |  | 12.8 |  |
|  |  | MCS16 | 1+1+1 |  | 6.39 |  | 8.8 |  |
|  |  | MCS17 |  |  | 6.71 |  | 10.7 |  |
|  |  | MCS20 |  |  | 9.75 |  | 12.8 |  |

Table   PUSCH results with Uni-directional Scenario A

| FOE | BW | MCS | RS | Samsung | Huawei | Nokia? | Intel | Ericsson? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Post-FFT | 200 | MCS16 | 1 | 15.3 |  |  | 17.9 | 8.2 |
|  |  | MCS17 |  | 23.7 |  |  | 23.3 | 9.5 |
|  |  | MCS20 |  | N.A |  |  | N.A | 15.0 |
|  |  | MCS16 | 1+1 | 14.7 |  |  | 17 | 8.0 |
|  |  | MCS17 |  | 19.5 |  |  | 19.9 | 8.3 |
|  |  | MCS20 |  | N.A |  |  | N.A | 15.0 |
|  |  | MCS16 | 1+1+1 | 14.7 |  |  | 16.9 |  |
|  |  | MCS17 |  | 21.7 |  |  | 21 |  |
|  |  | MCS20 |  | N.A |  |  | N.A |  |
| Pre-FFT | 50 | MCS16 | 1 |  |  | 6.64 |  |  |
|  |  | MCS17 |  |  |  | 7.24 |  |  |
|  |  | MCS20 |  |  |  | 9.78 |  |  |
|  |  | MCS16 | 1+1 |  |  | 6.30 |  |  |
|  |  | MCS17 |  |  |  | 7.07 |  |  |
|  |  | MCS20 |  |  |  | 9.79 |  |  |
|  |  | MCS16 | 1+1+1 |  |  | 6.26 |  |  |
|  |  | MCS17 |  |  |  | 7.14 |  |  |
|  |  | MCS20 |  |  |  | 9.81 |  |  |
|  | 200 | MCS16 | 1 |  |  | 6.29 | 8.9 |  |
|  |  | MCS17 |  |  |  | 7.20 | 10.8 |  |
|  |  | MCS20 |  |  |  | 9.70 | 12.8 |  |
|  |  | MCS16 | 1+1 |  |  | 6.24 | 8.8 |  |
|  |  | MCS17 |  |  |  | 7.12 | 10.3 |  |
|  |  | MCS20 |  |  |  | 9.69 | 12.8 |  |
|  |  | MCS16 | 1+1+1 |  |  | 6.26 | 8.8 |  |
|  |  | MCS17 |  |  |  | 7.20 | 10.7 |  |
|  |  | MCS20 |  |  |  | 9.70 | 12.8 |  |

Table   PUSCH results with Uni-directional Scenario B

| FOE | BW | MCS | RS | Samsung | Huawei | Nokia ? | Intel | Ericsson? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Post-FFT | 200 | MCS16 | 1 | 13.1 |  |  | 17.9 | 8.2 |
|  |  | MCS17 |  | 16.2 |  |  | 23.3 | 9.5 |
|  |  | MCS20 |  | N.A |  |  | N.A | 15.0 |
|  |  | MCS16 | 1+1 | 12.7 |  |  | 17 | 8.0 |
|  |  | MCS17 |  | 15.1 |  |  | 19.9 | 8.3 |
|  |  | MCS20 |  | N.A |  |  | N.A | 15.0 |
|  |  | MCS16 | 1+1+1 | 12.7 |  |  | 16.9 |  |
|  |  | MCS17 |  | 15.7 |  |  | 21 |  |
|  |  | MCS20 |  | N.A |  |  | N.A |  |
| Pre-FFT | 50 | MCS16 | 1 |  |  | 6.69 |  |  |
|  |  | MCS17 |  |  |  | 7.35 |  |  |
|  |  | MCS20 |  |  |  | 10.08 |  |  |
|  |  | MCS16 | 1+1 |  |  | 6.51 |  |  |
|  |  | MCS17 |  |  |  | 7.16 |  |  |
|  |  | MCS20 |  |  |  | 10.09 |  |  |
|  |  | MCS16 | 1+1+1 |  |  | 6.40 |  |  |
|  |  | MCS17 |  |  |  | 7.19 |  |  |
|  |  | MCS20 |  |  |  | 10.10 |  |  |
|  | 200 | MCS16 | 1 |  |  | 6.58 | 8.9 |  |
|  |  | MCS17 |  |  |  | 7.21 | 10.8 |  |
|  |  | MCS20 |  |  |  | 9.80 | 12.8 |  |
|  |  | MCS16 | 1+1 |  |  | 6.49 | 8.8 |  |
|  |  | MCS17 |  |  |  | 7.18 | 10.3 |  |
|  |  | MCS20 |  |  |  | 9.75 | 12.8 |  |
|  |  | MCS16 | 1+1+1 |  |  | 6.53 | 8.8 |  |
|  |  | MCS17 |  |  |  | 7.21 | 10.7 |  |
|  |  | MCS20 |  |  |  | 9.89 | 12.8 |  |

Issue 3-1-4: Frequency offset Estimation implementation

- Observations

  - Observation 1 (Huawei):

    - When the BS select pre FOC implementation

- There is negligible difference between different RS configuration

    - When the BS select post FOC implementation

- There is negligible difference between RS configuration of DMRS 1+1 and DMRS 1+1+1.

- There is a large difference between RS configuration with additional DMRS and without additional DMRS

- Maximum throughput cannot be achieved for MCS 20.

  - Observation 2 (Intel):

    - There is a higher than 8 dB difference between post-FFT and pre-FFT FOC strategies for MCS values 16 and 17.

    - MCS 20 cannot reach 70% throughput with post-FFT FOC

    - Both pre-FFT and post-FFT FOC strategies has its own benefits and RAN4 should not preclude one of them.

    - Requirement definition based on the worst case (Option 2 from WF) can mask bed implementations due to huge difference between two FOC strategies

- Proposals

  - Option 1 (Intel):

    - Define PUSCH requirements with MCS 20 and pre-FFT FOC method. Define corresponding manufacturer declaration on supporting high MCS values for HST FR2 and apply requirements on MCS 20 only based on this declaration.

    - Define PUSCH requirements with MCS 16 and post-FFT FOC method

  - Option 2 (Huawei, Ericsson, Samsung, CATT): FOE method is up to BS implementation. Use Post-FOE as the worst case for PUSCH demodulation requirements for FR2 HST

- Recommended WF

  - 2 companies provided the simulation result comparison with two kinds of FOE implementation. The Large performance difference is existed, even for Post-FFT operation

  - Encourage companies further align the simulation assumption and check results.

| Company | FOC | RS | MCS 16 | MCS 17 | MCS 20 |
| --- | --- | --- | --- | --- | --- |
| Huawei (Bi-directional B) | Post-FFT | 1+0 | 13.25 | 16.55 | N.A |
|  |  | 1+1 | 11.92 | 13.65 | N.A |
|  |  | 1+1+1 | 11.73 | 14.04 | N.A |
|  | Pre-FFT | 1+0 | 6.71 | 7.43 | 10.14 |
|  |  | 1+1 | 6.40 | 6.65 | 9.76 |
|  |  | 1+1+1 | 6.39 | 6.71 | 9.75 |
| Intel (Uni-directional A/B and Bi-directional-B) | Post FFT | 1+0 | 17.9 | 23.5 | N.A |
|  |  | 1+1 | 17 | 19.9 | N.A |
|  |  | 1+1+1 | 16.9 | 21 | N.A |
|  | Pre-FFT | 1+0 | 8.9 | 10.8 | 12.8 |
|  |  | 1+1 | 8.8 | 10.3 | 12.8 |
|  |  | 1+1 | 8.8 | 10.7 | 12.8 |

Issue 3-1-5: MCS

- Observations

  - Observation 1 (Huawei, Intel)

    - For BS with Post-FFT FOC method, Maximum throughput cannot be achieved for MCS 20

  - Observation  2 (Nokia):

    - The performance difference between MCS16 and MCS17 is not very significant, i.e., less than 1dB for SNR at 70% of maximum throughput.

- Proposals

  - Option 1 (Huawei, CATT, Samsung): MCS16

  - Option 2 (Ericsson): MCS 20

  - Option 3 (Nokia,  Intel): Both MCS16 and MCS20

    - Option 3a (Intel):  MCS 16 based on Post-FFT FOC method, MCS 20 based on Pre-FFT FOC method with defining corresponding manufacturer declaration supporting high MCS values for HST FR2 and apply requirements on MCS 20 only based on this declaration

- Recommended WF

  - Encourage feedback from companies

Issue 3-1-6: Phase noise modelling

- Observations

  - Observation1 (Nokia):

    - The impact of phase noise at higher MCS in HST FR2 scenario can be noticeable only if no phase noise compensation algorithm is used at the receiver, especially if only one DM-RS symbol per slot is transmitted. If a phase noise compensation algorithm is enabled, then no significant degradation in PUSCH performance is observed for MCS20.

  - Observation2 (Intel):

    - Phase noise has negligible impact on BS demodulation performance on considered HST FR2 scenarios

- Proposals

  - Option 1 (Nokia): Keep the former agreement that the phase noise impact can be included in the impairment results, but it is left up to companies. No explicit phase noise modelling in the alignment results is needed

- Recommended WF

  - Follow the former agreement about phase noise modelling

    - Phase noise impact can be included in the impairment results, but it is left up to companies

    - No explicit phase noise modelling in the alignment results

Sub-topic 3-2 UL timing ajustment requirements

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 3-2-1: RS configuration

- Observations

  - Observation 1 (Samsung):

    - No Doppler Shift is taken into account for scenario Y and scenario Z in FR1 HST

    - Similar performance can be achieved with different RS configurations

  - Observation 2 (Huawei):

    - For the residual frequency offset for FOE, there is s negligible difference between RS configuration of DMRS 1+1 and DMRS 1+1+1

    - For the residual frequency offset for FOE, there is a large difference between RS configuration with additional DMRS and without additional DMRS

- Proposals

  - Option 1(Samsung, Intel, Ericsson): 1 DMRS + PT-RS (L=1,K=2) and 2 DMRS+PT-RS (L=1,K=2) with test applicability rule based  on BS manufacturer declaration

  - Option 2 (CATT): 2 DMRS + PT-RS (L=1, K=2) and 3 DMRS +PT-RS (L=1,K=2) with test applicability rule based on BS manufacturer declaration

    - Option 2a (Huawei): Define one set of requirements with either DMRS 1+1 or DMRS 1+1+1 based on BS declaration, like Rel-16 2-step RACH WI did

  - Option 3 (Nokia): 1 DMRS + PT-RS(L=1, K=2) and 3 DMRS +PT-RS (L=1,K=2) with test applicability rule based on BS manufacturer declaration

- Recommended WF

  - 3 companies have provided the initial simulation results as following. The performance difference between different RS configuration is very minor, similar performance can achieved with 2 DMRS and 3 DMRS configurations.

  - Align the RS assumption for PUSCH requirement. Encourage companies to check whether option 1 is acceptable?

| RS configuration | BW | Samsung | Huawei | Intel |
| --- | --- | --- | --- | --- |
| 1+0 | 50MHz | 7.5 | 6.56 | 5.5 |
| 1+1 |  | 7.1 | 6.41 | 5.8 |
| 1+1+1 |  | 7.3 | 6.40 | 5.6 |
| 1+0 | 200MHz | 7.2 | 6.40 | 6.1 |
| 1+1 |  | 7.2 | 6.40 | 5.6 |
| 1+1+1 |  | 7.2 | 6,.40 | 5.5 |

Issue 3-2-2: Test Parameters for timing offset

- Observations

  - Observation 1(Samsung):

    - The propagation delay from two neighbouring RRHs in Uni-directional scenario is larger than the CP length.

    - With larger TA command, the maximum adjustment can be up to 2us, which is still smaller than the maximum sudden change as 2.5us. The remaining propagation delay is about 0.5us, which is close to the length of CP.

  - Observation 2(Nokia):

    - In the current (e.g., HST FR1) PUSCH timing adjustment test it is assumed that the UEs are in CONNECTED state. The range of timing adjustment that can be signalled with TAC MAC CE is ±2.1μs, and proposed test parameters for HST FR2 (A= 1.25 us, Δω = 1.04s-1) keeps Δτ within this range.

    - The jump in RTT of -4.8us that happens at RRH change does not match the ±2.1μs range of timing adjustments that can be signalled with TAC MAC CE. Whereas, with the current model the change in propagation delay Δτ happen continuously. Therefore,

- There is increasing the value of parameter A will not bring any additional value in the current test

- if the jump in RTT is added into the moving propagation condition then the feed backed TAC will not be able to compensate for it and the test cannot be considered applicable

- Proposals

  - Option 1 (Samsung, CATT, Nokia): Use A=1.25us, , Δw= 1.04s-1 corresponding to 120KHz SCS for HST FR2 UL timing adjustment requirements

- Recommended WF

  - Encourage feedback from companies

Issue 3-2-3: Timing different between moving UE and stationary UE

- Observations

  - Observation 1: The timing different between moving UE and stationary UE as Δt-(TA-31)x16*8Tc can be considered for UL timing adjustment requirement

- Proposals

  - Option 1: Δt-(TA-31)x16*8Tc  as baseline

- Recommended WF

  - Encourage feedback from companies

Issue 3-2-4: SRS transmission configuration

- Proposals

  - Option 1 (Samsung): The SRS transmission location is the last symbol in slot#3 in radio frame with TDD pattern as DDDSU, S=10:2:2

- Recommended WF

  - Encourage feedback from companies

Sub-topic 3-3 PRACH requirements

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 3-3-1: Timing offset configuration

- Observations

  - Observation 1 (Nokia):

    - The maximum time offset of the received PRACH preamble can be up to the maximum roundtrip from RRH to the CPE and equals 4.6us or 4.8us depending on the scenario. These values are within the maximum timing offset that can be tolerated with the agreed HST FR2 PRACH configuration (C2 format, Ncs=0, 120 kHz SCS).

    - There is not meaningful difference in the SNR levels corresponding to the 99% probability of PRACH preamble misdetection with 0.8 us, 4.6 us and 4.8 us maximum timing offset.

  - Observation 2 (Intel):

    - Performance difference between scenario with 0.8us TO and 4.78us TO is about 0.1 dB at 1% of miss detection rate.

  - Observation 3 (Samsung):

    - Minor performance different between maximum timing offset as 0.8 and 4.6us

- Proposals

  - Option 1 (Huawei, CATT, Samsung): Reuse Rel-15 timing offset configuration for PRACH, i.e., 0.8 us

    - Value of Timing offset start: 0

    - Step of Timing offset increase : 0.1us

  - Option 2 (Nokia, Intel, Ericsson): Configure the maximum timing offset (i.e. the end of the tested range) in FR2 HST testing setup equal to [4.8]us

    - Value of Timing offset: 0

    - Step of Timing offset increase : [0.48]us

  - Note:

    - Scenario A (Ds =700m, Dmin =10m), cell radius =700m

    - Scenario B (Ds=700m, Dmin=150m), cell radius =716m

| Scenario | Cell radius (m) | Maximum timing offset (us) (RTT) | Coverage of format C2 | Maximum timing offset for C2 (us) |
| --- | --- | --- | --- | --- |
| Scenario A (Ds =700m, Dmin =10m) | 700 | 4.6us | 1.15km | 3.84 |
| Scenario B (Ds=700m, Dmin=150m) | 716 | 4.78us | 1.15km | 3.84 |

- Recommended WF

  - 5 companies has provided the simulation results as following. Similar performance can be achieved for this two options for each companies. Either option 1 or option 2 can meet test purpose of PRACH receiver verification. Opion1 can reduce the effort for specification change and reusing Rel-15 test setup, Option 2 is to align the FR2 HST deployment scenario. Encourage companies to compromise one of them for moving forward.

  - Meanwhile, based on existing results between companies, one company has large gap compared with other companies, encourage companies can further check their results during meeting or next meeting if there is any issue identified.

| SNR@1% Missing detection | Samsung | Huawei | Ericsson | Nokia | Intel |
| --- | --- | --- | --- | --- | --- |
| Option 1 | -12.49 | -12.30 | -10.8 | -6.92 | -13.1 |
| Option 2 (4.6us) | -12.56 | -12.36 | -10.8 | -6.87 | -13.2 |
| Option 2(4.8us) |  |  |  | -6.91 |  |

Companies views’ collection for 1st round

Open issues

Sub topic 3-1

| Company | Comments |
| --- | --- |
| CATT | Issue 3-1-1 Issue 3-1-2 Issue 3-1-3: RS configuration We think at least 2 DMRS configuration is needed for FR2 HST. Issue 3-1-4: Frequency offset Estimation implementation We support option 2, i.e. use Post-FOE as the worst case for PUSCH demodulation requirements for FR2 HST. Issue 3-1-5: MCS From the perspective of OTA testing, we prefer MCS 16. Issue 3-1-6 |
| Intel | Issue 3-1-1: Scope of PUSCH requirement with Uni-directional RRH scenario A/B and Bi-directional RRH scenario B Based on our results there is no meaningful performance difference between different scenarios. Therefore, we propose to reduce BS test load. We can define single test based on the one model or define all of them but with applicability rule and corresponding manufacturer declaration. As for a single channel model we slightly prefer bidirectional deployment since Doppler frequency jump is higher in this model compared to unidirectional. So, Options 2a and 3 are fine for us. Issue 3-1-3: RS configuration We support Option 1. It covers scenario with single symbol DMRS and scenario with additional DMRS symbols. Issue 3-1-4: Frequency offset Estimation implementation Both pre-FFT and post-FFT FOC strategies has its own benefits. Pre-FFT implementation provides huge performance benefits but it can be used only in dedicated deployments without many UEs. In order to have good test coverage we suggest defining requirements for both schemes. If requirements will be defined only for the worst case as post-FFT there will be huge margin for products with pre-FFT compensation scheme (higher than 8 dB based on our evaluations). In this case it will mask bad implementations. We suggest definition of MCS 16 requirements with post-FFT and MCS20 with pre-FFT. From specification perspective it is better not to mention specific compensation scheme. Just for requirements definition RAN4 should assume certain scheme to align SNR values. Also considering that post-FFT scheme is a baseline solution we suggest definition MCS16 as mandatory and MCS20 requirements up to manufacturer declaration on support of 64QAM in HST FR2. To summarize our proposal is: - FOE method is up to BS implementation - Define PUSCH requirements with MCS 16 assuming post-FFT FOC method for requirements definition. - Define PUSCH requirements with MCS 20 assuming pre-FFT FOC method for requirements definition. - Define manufacturer declaration on supporting 64QAM operation in HST FR2 and apply requirements with MCS 20 only based on this declaration. - Do not capture assumptions on FOC scheme in specification. Issue 3-1-5: MCS Same as in issue 3-1-4 Issue 3-1-6: Phase noise modelling Support Option 1 and recommended WF based on the obtained link-level results. |
| Ericsson | Issue 3-1-1 We also see only very minor difference between the scenarios. We prefer to set a single PUSCH requirement based on one of the scenarios. Since the baseband algorithm needed for frequency offset estimation is the same in each case and the performance is the same, we do not have a strong view as to which scenario is used. What is important is that we do not think of it as ”testing one scenario X”. What we would be saying is that the test is defined such that the basestation baseband is proven to meet a single minimum requirement that is sufficient to operate in all of the scenarios (even if the minimum requirement is based on one specific scenario). Issue 3-1-2 We prefer to define a single requirement rather than 3 requirements and a test applicability rule, because if we have 3 requirements and a test applicability rule we may create a perceived ambiguity as to whether the BS has really been fully tested. Issue 3-1-3 We are OK with option 1 Issue 3-1-4 We think that a single requirement should be defined, and that post FFT FOC should be assumed in the simulations. Issue 3-1-5 There is some divergence between companies results and a need for checking. We see MCS 20 is possible. After checking, our view is that the requirement should be based on the highest MCS that is achievable and for which the SNR is within the testable limit (20dB). Issue 3-1-6 Agree with the proposed WF |
| Huawei | Issue 3-1-1: Scope of PUSCH requirement with Uni-directional RRH scenario A/B and Bi-directional RRH scenario B Option 1. From the Doppler trajectory we can see that the Uni-ScenarioA case can provide largest frequency offset among all cases and the Bi-ScenarioB case provide largest frequency jump among all cases. The baseband processing verification under Uni-ScenarioA can be covered by the other two cases. To ensure the test coverage and reduce the test load at the same time, we propose to only define Uni-ScenarioA case and Bi-ScenarioB case. Issue 3-1-2: Test Applicability rule As discussed in Issue 3-1-1, we prefer to define Uni-ScenarioA case and Bi-ScenarioB case. In this situation, we don’t support define applicability rule between above two scenarios. If companies have strong concern about the test effort and the applicability rule must be defined for different scenarios, then we prefer to skip Uni-ScenarioA case if Bi-ScenarioB case is passed as larger Doppler jump is observed in Bi-ScenarioB case, considering that Doppler jump is the most important point to be verified. Issue 3-1-3: RS configuration Option 2a. In our simulation results, performance degradation is observed without additional DMRS. Also DMRS 1+1+1 is benefit for some implementation that perform phase noise tracking by DMRS, we don’t think such implementation should be prevented. Notice that the demodulation performance for DMRS 1+1 and DMRS 1+1+1 is very close, we propose to define one set of performance requirements with either DMR-RS 1+1 or 1+1+1 based on BS declaration, like Rel-16 2-step RACH WI did. Issue 3-1-4: Frequency offset Estimation implementation Option 2. At most of cases, post-FOE is used at the BS side, it should not be expected that the BS used in HST scenario must be special designed. In our view, FOE should be up to BS implementation and should not be limited since the requirements we defined are minimum requirements, the worst case should be selected. Issue 3-1-5: MCS Option 1. As per our simulation results, MCS 20 cannot achieve maximum throughput for post FFT processing, so we prefer to select MCS 16. We are also open to perform further evaluation to find the maximum achieved MCS within the testable limit. However, there is very large difference for the simulation results proposed by companies currently. Issue 3-1-6: Phase noise modelling Option 1. |
| Nokia, Nokia Shanghai Bell | Issue 3-1-1: Scope of PUSCH requirement with Uni-directional RRH scenario A/B and Bi-directional RRH scenario B In our opinion, Option 2a, i.e., testing of bi-directional models provides sufficient test coverage. However, we are also open to other options, e.g., Option 3 with corresponding applicability rules/manufacturer declarations is also acceptable to us. Issue 3-1-2: Test Applicability Rule If requirements are defined for bi-directional Scenario-B then no applicability rules are needed. Otherwise, the applicability rules should be designed in such a way that only one test is performed depending on the declaration of supported scenarios. Issue 3-1-3: RS configuration Basing FOE on PT-RS is feasible at these speeds. However, if the implementation from FR1 is to be re-used (i.e., based on DM-RS and not on PT-RS) then the speed requires DM-RS 1+1+1. We don’t see why implementation should be forced to use PT-RS. When a PT-RS less configuration is potentially feasible. Therefore, as an alternative we propose a new Option: Option 4: 1 DMRS + PT-RS (L=1,K=2), 2 DMRS+PT-RS (L=1,K=2), and 3 DMRS with test applicability rule based on BS manufacturer declaration. Issue 3-1-4: Frequency offset Estimation implementation Firstly, we would like to clarify if the discussion is about FO compensation (FOC) or FO estimation (FOE) because these terms looks to be used interchangeably in this and previous issues whereases they are two different operations. Secondly, we still would like to understand better what is the source of such a large difference between pre-FFT and post-FFT FOC? In our opinion, if only one UE is present in the system (which is the case in demod testing) the energy of the symbols can be recovered regardless of approach. Hence, the performance of both methods should theoretically be rather close. Finally, we agree with Option 2, that FOE/FOC is the BS implementation choice and might change over time, thus only one set of requirements shall be supported per MCS without specification of the type of FOC/FOE algorithm. Issue 3-1-5: MCS In our opinion, it is beneficial to test both MCS 16 and MCS 20. However, MCS 20 can be still up to manufacturer declaration if with some implementations it cannot be supported. Issue 3-1-6: Phase noise modelling We support the recommended WF. |
| ZTE | Issue 3-1-4: Frequency offset Estimation implementation As pre-FOC depends on specific scenarios, we support option 2. Issue 3-1-6: Phase noise modelling We support the recommended WF. |
| Samsung | Issue 3-1-1: Scope of PUSCH requirement with Uni-directional RRH scenario A/B and Bi-directional RRH scenario B Among three scenarios, the Doppler observed by RRH in Uni-directional with scenario A is the largest. While from the demodulation perspective, there is no different receiver processing foreseen. Therefore, we prefer to define the requirement with one of them, so, either option 2a, 2b or 2c are fine with us. Considering the Doppler in A is more challenge, we prefer option 2c, If companies are willing to define requirement for different scenario, we can accept with option 1 with defined test applicability Issue 3-1-2 : Test applicability rule This issue related with Issue 3-1-1, if the requirements for different scenarios are considered, as mentioned, from baseband processing, there is no different, In that sense, we prefer to define test applicability rule as - If both scenarios A and B are introduced, the test is performed based on BS manufacture declaration. If BS declared to support both scenario A and scenario B and BS can pass the test of Uni-directional for scenario A, it can skip the test of Uni-directional scenario for scenario B. - If BS declared to support both Uni-directional and Bi-directional operation, and BS can pass the test of Uni-directional for scenario A, it can skip the test of Bi-directional scenario for scenario B.” Issue 3-1-3: RS configuration Option 1. We are fine to align RS configuration with UL timing adjustment and PUSCH. Firstly, based on previous feasibility to enable 350km/h maximum speed, the PTRS should be configured. Based on maximum frequency offset tracking capacity, PTRS has small distance between adjacent RS compared with DMRS. Therefore, the number of DMRS is not the bottleneck of frequency tracking for FR2, As analysis, only with 3 DMRS, where the gap between neighbour DMRS is 4, where only up to 252km/h maximum speed can be supported. That is reason that PTRS should be configured. Based on the agreement, only single tap channel is considered for FR2 HST WI. Different with fading channel, there is no necessary to configure more DMRS to overcome the fading impact. Secondly, from companies’ results for PUSCH and UL timing adjustment, similar SNR can be achieved for 2 DMRS configuration and 3 DMRS configuration. Since there is no limitation of Uplink budget, with same SNR, with 2 DMRS configuration can achieve the throughput. We think it is not reasonable scheduling for BS to configure 3 DMRS, we wonder what is the benefit to configure 3 DMRS, if 2 DMRS can achieve similar performance? Could proponent of 3 DMRS clarify? Regarding the alternative options proposed by Nokia, we do not think it is necessary to cover all the possible DMRS configurations for requirement with considering the test effort and simulation effort. Based on company’s results, 2 DMRS and 3DMRS have the similar performance, so, there is no need to configure 3 DMRS. If companies have strong preference with 3 DMRS, Maybe one solution is that - Case 1: Define PUSCH requirement with 1DMRS+PTRS (L=1, K=1) configuration - Case 2: Define PUSCH requirement with 2 DMRS+PTRS (L=1, K=2) configuration - Note: The requirement can be applied for 3 DMRS +PTRS (L=1, K=2) configuration - Define test applicable rule, either test case 1 or case 2 based on BS manufacturer declaration Issue 3-1-4: Frequency offset As for FOE/FOC implementation, both Pre-FFT and Post-FFT operation are feasible, while it is up to BS implementation. In terms of RAN4 requirement, in our understanding, there is no restriction for them, similar as channel estimation, and TO/FO estimation, only the practical estimation is indicated in the simulation assumption for requirement. Therefore, we propose not to mandate the FOE/FOC implementation method, which is up to BS implementation. RAN4 will not any specific implementation into spec. Issue 3-1-5 MCS Regarding MCS, we think at least MCS 16 can be considered and can be tested, not matter Pre-FFT or Post-FFT FOC/FOE implementation. So, we can conclude that define MCS 16 for PUSCH requirement.. While, based on companies results, even with Post-FFT FOC/FOE implementation, there is a large gap existing. Further alignment is needed. If there is lager gap, additional margin can be considered if needed, As for MCS 20, based on some company’s results, it is not proper to define requirement with Post-FFT FOC/FOE operation. Even the maximum throughput can be achieved, the achieved SNR may be higher than 20dB, which is up to test limitation of existing OTA method. Also, some companies results MCS 20 is feasible, Lager gap is still existed. To allow two possible implementation, we are open to define two sets MCS requirements, while detail FOE/FOC is up to BS implementation. To address the potential issues of MCS 20 with Post –FFT/FOC operation, lower than MCS 20 can be considered Similar with Rel-16 LTE, two sets requirement for 500km are defined, one is for 1944Hz, and another is for 1750Hz. The test applicability rule for different requirement with Doppler value are defined .So, if two sets of requirement are defined, similar test applicability rule can be defined In summary, we are open to define two sets MCS requirements - FOE method is up to BS implementation - MCS 16 - [MCS 20] or lower than MCS20 with testable limit (20dB) for 64QAM - Define manufacturer declaration on supporting 64QAM operation in HST FR2 and apply requirements only based on this declaration. - MCS 16 is not applicable if BS manufacturer declares supported [MCS20] - [MCS 20] is not applicable if BS manufacturer declares support MCS16 Issue 3-1-6: Phase noise modelling Based on companies result and observations, phase noise has negligible impact on BS demodulation performance. So, we prefer to follow the former agreement about phase noise modelling as recommend WF |

Sub topic 3-2

| Company | Comments |
| --- | --- |
| XXX | Issue 3-2-1: RS configuration To align with RS configuration for PUSCH requirement. Issue 3-2-2: Test Parameters for timing offset Support option 1. Issue 3-2-3: Timing different between moving UE and stationary UE Support option 1 Issue 3-2-4: SRS transmission configuration Support option 1 |
| Intel | Issue 3-2-1: RS configuration Support alignment of UL TA RS configuration with PUSCH RS configuration. Issue 3-2-2: Test Parameters for timing offset Support Option 1 that is aligned with UL TA channel model for 120 kHz SCS. Issue 3-2-3: Timing different between moving UE and stationary UE Support Option 1. Issue 3-2-4: SRS transmission configuration Option 1 is fine for us. |
| Ericsson | Issue 3-2-1 The RS configuration for TA should be aligned with the RS configuration options for the PUSCH requirement. Issue 3-2-2 This is fine if there is no need to include any timing jump in the demodulation requirements |
| Huawei | Issue 3-2-1: RS configuration Option 2a. We prefer to not discuss this issue here and align the RS assumption for UL TA requirements with that for PUSCH requirement. Issue 3-2-2: Test Parameters for timing offset Option 1. Issue 3-2-3: Timing different between moving UE and stationary UE Option 1. Issue 3-2-4: SRS transmission configuration Option 1. |
| Nokia, Nokia Shanghai Bell | Issue 3-2-1: RS configuration We also think that the parameters shall be aligned with PUSCH requirements. Issue 3-2-2: Test Parameters for timing offset Option 1 is OK for us. Issue 3-2-3: Timing different between moving UE and stationary UE Agree with Option 1, except that Δτ should be used in the equation instead of Δt. Issue 3-2-4: SRS transmission configuration Option 1 is OK. |
| ZTE | Issue 3-2-2: Test Parameters for timing offset We agree with Option 1. Issue 3-2-3: Timing different between moving UE and stationary UE We agree with Option 1. |
| Samsung | Issue 3-2-1: RS configuration Option 1. We are fine to align RS configuration with UL timing adjustment and PUSCH. Generally, PUSCH with high Doppler frequency and UL timing adjustment requirement belong two different test purpose for Baseband receiver processing. One is to handle the Doppler impact, where Doppler tracking capability is pending on the gap of neighbour RS. Another is to handle the timing offset impact, which is based on the correlation operation. From companies’ results for PUSCH and UL timing adjustment, similar SNR can be achieved for 2 DMRS configuration and 3 DMRS configuration. Since there is limitation of Uplink budget, with same SNR, with 2 DMRS configuration can achieve the throughput. Meanwhile, as for UL timing adjustment, although it is targeting for high speed, there is no Doppler considering for timing offset model, thus, with 2 DMRS can still overcome the Doppler impact. We think it is not reasonable scheduling for BS to configure 3 DMRS, we wonder what is the benefit to configure 3 DMRS, if 2 DMRS can achieve similar performance? Could proponent of 3 DMRS clarify? Issue 3-2-2: Test Parameters for timing offset Ok with option 1 Issue 3-2-3: Timing different between moving UE and stationary UE Ok with option 1 Issue 3-2-4: SRS transmission configuration We are fine with option 1 to align the TDD pattern for FR2, which is same with Rel-15 |

Sub topic 3-3

| Company | Comments |
| --- | --- |
| XXX | Issue 3-3-1: Option 1. Prefer to reuse the Rel-15 timing offset configuration for PRACH, i.e. 0.8us timing offset, and starting from 0us to 0.8us with 0.1 us step. |
| Intel | Issue 3-3-1: Timing offset configuration We agree that either option 1 or option 2 meet test purpose of PRACH receiver verification. Choosing between work efforts minimizations and addressing HST FR2 deployments we support Option 2. Also, it will be clearer value for readers since it has technical background. Option 1 for HST FR1 also can be explained even it does not align with HST FR1 channel models. As we explained HST FR1 PRACH requirements cover many PRACH preambles with different cell size applicability. |
| Huawei | Issue 3-3-1: Timing offset configuration Option 1. As per simulation results provided by companies, there is very small difference between two options, and we think both options are feasible to verify the baseband demodulation function since the algorithm is same. However, in Rel-16 FR1 HST, 0.8 us is reused from the non-HST test setup although the maximum cell radius can be about 716m that is same as HST FR2 Scenario B, so we prefer to reuse Rel-15 FR2 timing offset configuration for PRACH, i.e., 0.8us. |
| Nokia, Nokia Shanghai Bell | Issue 3-3-1: Timing offset configuration We also have the same thinking as already described by Intel above. Option 2 is our preference. Additionally, we got updated PRACH simulations results: -12.49 for Option 1, -12.37 for Option 2 (4.6), and -12.48 for Option 2 (4.8). |
| Samsung | Issue 3-3-1: Timing offset configuration Either option 1 or option 2 are fine for us. From receiver processing perspective, both option 1 and option 2 are same. Meanwhile, the performance is similar. Option1 is reused from Rel-15, to reduce the work effort for spec modification. To address the HST FR2 deployment, option 2 is more proper. Since only one format is available for FR2 HST, then the timing offset can be set to min (maximum timing offset of C2 (RTTI), maximum cell radius of scenario 2)=4.8us. So, If companies have strong preference with option 2, we are fine to go option 2 for moving forward. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Issue 3-1-2 : Test applicability rule Candidate options: - Option 1 (Intel): - Introduce BS manufacturer declaration for applicable test case - Introduce applicability rule that allows BS to pass only one scenario and other scenarios will be passed without explicit testing. - Option 2 (Nokia): - Option 1a: Define applicability rules in such a way that if bi-directional Scenario-B is tested then no other tests are needed. - Option 1b: Define applicability rules in such a way that it is sufficient to test only one of the Uni-directional deployments by manufacturer choice. - Option 3 (Samsung): - If both scenarios A and B are introduced, the test is performed based on BS manufacture declaration. If BS declared to support both scenario A and scenario B and BS can pass the test of Uni-directional for scenario A, it can skip the test of Uni-directional scenario for scenario B. If BS declared to support both Uni-directional and Bi-directional operation, and BS can pass the test of Uni-directional for scenario A, it can skip the test of Bi-directional scenario for scenario B. - Option 4 (Ericsson): - Define a single requirement without test applicability rule - Option 5 (Huawei): - If needed, define applicability rule as skip Uni-directional with Scenario A case if Bi-directional with scenario B case is passed Recommendations for 2nd round: - Pending on the discussion scope for BS demodulation requirement Issue 3-1-3: RS configurations for PUSCH and UL timing adjustment Candidate options: - Proposals - Option 1 (Samsung, Intel, Ericsson, CATT): 1 DMRS + PT-RS (L=1,K=2) and 2 DMRS+PT-RS (L=1,K=2) with test applicability rule based on BS manufacturer declaration - Option 1 (CATT): 2 DMRS + PT-RS (L=1, K=2) and 3 DMRS +PT-RS (L=1,K=2) with test applicability rule based on BS manufacturer declaration - Option 2a (Huawei): Define one set of requirements with either DMRS 1+1 or DMRS 1+1+1 based on BS declaration, like Rel-16 2-step RACH WI did - Option 3 (Nokia, CATT): 1 DMRS + PT-RS(L=1, K=2) and 3 DMRS +PT-RS (L=1,K=2) with test applicability rule based on BS manufacturer declaration - Option 4 (Nokia): 1 DMRS + PT-RS(L=1, K=2) , 2 DMRS +PT-RS (L=1,K=2) and 3 DMRS with test applicability rule based on BS manufacturer declaration Recommendations for 2nd round: - 3 companies have provided the initial result. - From performance perspective, similar performance can be achieved with 2 DMRS and 3 DMRS configuration - From Doppler tracking capability to enable 350km/h velocity, the number of DMRS is not the bottleneck of UL - From overhead of RS perspective, 1 or 2 DMRS has less overhead compared with 3 DMRS, which can further improve the system performance - Rel-15 has already defined requirement with 1 DMRS or 2 DMRS with test applicability to allow infra vendors different implementation - For moving forward, moderator suggest the following compromise solution - Align RS configuration for PUSCH requirement and UL timing adjustment requirement - Case 1: Define requirement with 1 DMRS + PT_RS (L=1,K=2) configuration - Case 2: Define requirement with 2 DMRS+ PT_RS (L=1,K=2) configuration - Note : The requirement can be applied for 3 DMRS++PT_RS (L=1,K=2) configuration - Define test applicability for case 1 and case 2 testing based on BS manufacturer declaration - If time allowed , further check in GTW session Issue 3-1-4 and Issue 3-1-5: FOC implementation and MCS Candidate options: - Option 1 (Intel): - FOE method is up to BS implementation - Define PUSCH requirements with MCS 16 assuming post-FFT FOC method for requirements definition. - Define PUSCH requirements with MCS 20 assuming pre-FFT FOC method for requirements definition. - Define manufacturer declaration on supporting 64QAM operation in HST FR2 and apply requirements with MCS 20 only based on this declaration. - Do not capture assumptions on FOC scheme in specification. - Option 2 (Huawei, Ericsson, Samsung, CATT, ZTE, Nokia): - FOE method is up to BS implementation. Use Post-FOE as the worst case for PUSCH demodulation requirements for FR2 HST - Option 2a(Ericsson) - Single requirement with highest MCS within the testable limit (20dB) assuming Post-FFT FOC method for requirement definition - Option 2 - Option 2 (Ericsson, Huawei, Nokia, ): - Option 3 (Huawei): - MCS 16 and highest MCS within the testable limit (20dB) - Option 4 (Nokia): - FOC method is up to BS implementation - Both MCS16 and MCS 20, the requirement of MCS20 based on BS manufacture declaration if some implementation cannot be supported Recommendations for 2nd round: - 5 companies have provided the initial simulation results. Large performance is existed between companies, even for Post- FFT FOE operation. - Encourage companies to further align the simulation assumption and check the simulation results - FOC implementation, both Pre-FFT and Post-FFT operation are feasible, while it is up to BS implementation. RAN4 will not specify any specific implementation into spec. - MCS 20 requirement cannot be achieved by some companies’ results for Post-FFT FOC method. Even the maximum throughput can be achieved, the achieved SNR may be higher than 20dB, which is up to test limitation of existing OTA. - Large performance difference between Pre-FFT and Post-FFT FOC, even with lower MCS - For moving forward, moderator suggest the following compromise solution - FOC method is up to BS implementation - Do not capture assumptions on FOC scheme in specification - Define PUSCH requirements with MCS 16 - Define PUSCH requirements with [MCS 20] or lower than MCS20 with testable limit (20dB) for 64QAM - Define manufacturer declaration for FR2 HST and apply requirements only based on this declaration - MCS 16 requirement is not applicable if BS manufacturer declares supported [MCS20] for FR2 HST - [MCS20] requirement is not applicable if BS manufacturer declares only supported MCS16 for FR2 HST - Encourage companies to further check and align the results. Based on simulation collections, decide requirement definition. - Encourage companies to check the source of large difference between Pre-FFT and Post-FFT FOC, especially for MCS16 - If time allowed , further check in GTW session Issue 3-1-6: Phase noise modelling Tentative agreements: - Phase noise impact can be included in the impairment results, but it is left up to companies - No explicit phase noise modelling in the alignment result |
| Sub-topic#3-2 | Issue 3-2-2: Test Parameters for timing offset Tentative agreements: - Use A=1.25us, , Δw= 1.04s-1 corresponding to 120KHz SCS for HST FR2 UL timing adjustment requirements Issue 3-2-3: Timing different between moving UE and stationary UE Tentative agreements: - Apply the timing different between moving UE and stationary UE as Δτ-(TA-31)x16*8Tc for UL timing adjustment requirement Issue 3-2-4: SRS transmission configuration Tentative agreements: The SRS transmission location is the last symbol in slot#3 in radio frame with TDD pattern as DDDSU, S=10:2:2 |
| Sub-topic#3-3 | Issue 3-3-1: Timing offset configuration Candidate options: - Option 1 (Huawei, CATT, Samsung): Reuse Rel-15 timing offset configuration for PRACH, i.e., 0.8 us - Value of Timing offset start: 0 - Step of Timing offset increase : 0.1us - Option 2 (Nokia, Intel, Ericsson, Samsung): Configure the maximum timing offset (i.e. the end of the tested range) in FR2 HST testing setup equal to [4.8]us - Value of Timing offset: 0 - Step of Timing offset increase : [0.48]us Recommendations for 2nd round: - 5 companies has provided the simulation results as following. Similar performance can be achieved for this two options for each companies. Either option 1 or option 2 can meet test purpose of PRACH receiver verification. Either option 1 or option 2 can meet test purpose of PRACH receiver verification. Opion1 can reduce the effort for specification change and reusing Rel-15 test setup, Option 2 is to align the FR2 HST deployment scenario. Different with FR1 HST, three different formats are specified for requirement. Since only one format is defined for FR2 HST requirement, then the timing offset can be set to up the RTT of scenario 2 as 4.8us - For moving forward, moderator suggest to agree option 2? |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #4 CR work split for FR2 HST demod

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2119472 | Nokia | Proposal 1: Define PUST FR2 BS Demod CR work split based on the table above. Companies are encouraged to sign up for desired items. |

Open issues summary

List of open issues

- Sub-Topic 4-1: FR2 HST Demod CR work spilt

  - Issue 4-1-1: BS demodulation requirements CR

  - Issue 4-1-2: UE demodulation requirements CR

Sub-topic 4-1 FR2 HST Demod CR work split

Issue 4-1-1: BS demodulation requirement CR

- Proposals

  - Option 1 (Nokia)

| Section number | Section title | Responsible company |
| --- | --- | --- |
| TS 38.104 |  |  |
|  | Big CR | Huawei |
| 11 | Radiated performance requirements |  |
| 11.2 | Performance requirements for PUSCH |  |
| 11.2.2 | Requirements for BS type 2-O |  |
| 11.2.2.x | Requirements for PUSCH for high speed train | Intel |
| 11.2.2.y | Requirements for UL timing adjustment | CATT |
| 11.4 | Performance requirements for PRACH |  |
| 11.4.2 | Requirements for BS type 2-O |  |
| 11.4.2.2 | PRACH detection requirements |  |
| 11.4.2.2.x | Minimum requirements for high speed train | Huawei |
| Annex A | Reference measurement channels | Intel |
| Annex G.3 | High speed train condition | Nokia |
| Annex G.4 | Moving propagation conditions | CATT |
| TS 38.141-2 |  |  |
|  | Big CR | Nokia |
| 4.6 | Manufacturer's declarations | [Nokia] |
| 8 | Radiated performance requirements |  |
| 8.1.2 | Applicability rule |  |
| 8.1.2.4 | Applicability of PUSCH for high speed train performance requirements | [Nokia] |
| 8.2 | OTA performance requirements for PUSCH |  |
| 8.2.4 | Performance requirements for PUSCH for high speed train |  |
| 8.2.5 | Performance requirements for UL timing adjustment | CATT |
| 8.4 | OTA performance requirements for PRACH |  |
| 8.4.1 | PRACH false alarm probability and missed detection |  |
| 8.4.1.6 | Test requirement for high speed train | Huawei |
| Annex A | Reference measurement channels | Intel |
| Annex E | OTA measurement system set-up |  |
| Annex J.3 | High speed train condition | Nokia |
| Annex J.4 | Moving propagation conditions | CATT |

Issue 2-4-2: UE demodulation requirement CR

- Proposals

  - Option 1:

| Section number | Section title | Responsible company |
| --- | --- | --- |
| TS 38.101-4 |  |  |
|  | Big CR |  |
| 7.11 | Applicability of requirements | Intel |
| 7.2.2.2.x | Minimum requirements for PDSCH HST-DPS | Huawei |
| A.3.2 | Reference measurement channels for PDSCH performance requirement |  |
| B.3 | High speed Train Scenarios | Qualcomm |

- Recommended WF

RAN4#101-e E-meeting Arrangements and Guidelines presentation shared by the RAN4 chair contains instructions on CR handling for Rel-17 non-spectrum related WI:

- RRM CR handling - Draft CRs with are allowed for WIs of NR FR1 RF Enhancements [NR_RF_FR1_enh], NR FR2 RF Enhancements [NR_RF_FR2_req_enh2], NR FR1 HST Enhancements [NR_HST_FR1_enh], NR FR2 HST [NR_HST_FR2], NR FeRRM [NR_RRM_enh2] , NR MG Enhancements [NR_MG_enh] - For other items the discussion is focused on technical issues and initial CRs are planned for Q1’2022. - Note that the discussions will focus on technical issues first and Draft CRs will be handled with the 2nd priority.

It can be noticed that NR FR2 HST WI is already listed as allowed for the Draft CRs for RRM.

Regarding demodulation part, only TPs are allowed for Demod Rel-17 WIs.

- Demodulation CR handling - Draft TPs/TPs are allowed for performance enhancement WI.

For other demodulation topics, there is no guidelines yet. Although there is a recommendation on Work Plans as

- SI/WI RAN4 RF/RRM/demodulation Work Plans - Rapporteurs are encouraged to provide updated SI/WI RRM work plans to decide on - CR/TP work split among contributing companies to avoid duplicate efforts and to encourage sharing the workload and cross-checking CRs - CR work split should at least include Big CR split. It is also allowed if companies want to further split the work under each Big CR.

Based on current discussion, the core requirement is still under discussion, some of open issues have the impact on test setup for performance requirement. The test scope and test setup for FR2 HST demodulation requirement is not stable yet, it is early to discuss the related CR work split work.

As rapporteur and moderator, my suggestion is to focus on the technical discussion about the detail test scope and test setup for this meeting, postpone the related CR work split discussion.

Companies views’ collection for 1st round

Open issues

Sub topic 4-1

| Company | Comments |
| --- | --- |
| CATT | Issue 4-1-1: BS demodulation requirement CR We would like to take care of “Requirements for UL timing adjustment” and “Moving propagation conditions”. |
| Intel | Issue 4-1-1: BS demodulation requirement CR Issue 2-4-2: UE demodulation requirement CR We are fine with moderator suggestion to postpone CR work split discussion. Meantime if we will discuss it this meeting, we would like to take care of the following CRs: - “Requirements for PUSCH for high speed train” in TS 38.104 - “Reference measurement channels” in TS 38.104 - “Reference measurement channels” in TS 38.141-2. - “Applicability of requirements” in TS 38.101-4 |
| Ericsson | Issue 4-1-1/4-1-2: We agree with the moderator’s suggestion. We need to conclude the discussion the scope of UE/BS demodulation requirements. We also agree with Intel’s suggestion. |
| Huawei | We would like to take care of the following CRs: - Big CR of TS 38.104 - 11.4.2.2.x Minimum requirements for high speed train in TS 38.104 - 8.4.1.6 Test requirement for high speed train in TS 38.141-2 - 7.2.2.2.x Minimum requirements for PDSCH HST-DPS in TS 38.101-4 |
| Nokia, Nokia Shanghai Bell | In our opinion, on the level of detail presented in the table, the scope of the demodulation requirements is rather clear. We marked our preferences in the table. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: - Issue 4-1-1: BS demodulation requirements CR - Issue 4-1-2: UE demodulation requirements CR Recommendations for 2nd round: - As rapporteur and moderator, my suggestion is to focus on the technical discussion about the detail test scope and test setup for this meeting, postpone the related CR work split discussion. - Highly appreciated companies contributed FR2 HST demodulation requirement. About 22 CR works split was expected. Suggest companies to balance the CR work load. If other companies are willing to contribute, the CR assignment will be minor adjustment. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on general and UE demodulation requirement for FR2 HST | Samsung |  |
| WF on BS demodulation requirement for FR2 HST | Nokia, Samsung |  |
| Simulation assumption for PDSCH requirement for FR2 HST | Intel |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Moderator (Samsung) | Yunchuan Yang | yc0301.yang@samsung.com |
| CATT | Yuan Gao | gaoyuan@catt.cn |
| Intel | Artyom Putilin | artyom.putilin@intel.com |
| Ericsson | Kazuyoshi Uesaka | kazuyoshi.uesaka@ericsson.com |
| Qualcomm | Pierpaolo Vallese | pvallese@qti.qualcomm.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)