3GPP TSG-RAN WG4 Meeting #101-e                	R4-2120708

Electronic Meeting, Nov 1-12, 2021

Agenda Item:	8.12.2.1

Source:	Ericsson

Title:	WF on CSI requirements for inter-cell interference MMSE-IRC

Document for:	Approval

# Introduction

This is the WF to capture all agreements and open issues in [101-e][323] for WF on CSI requirements for inter-cell interference MMSE-IRC in RAN4#101e meeting.

# CSI requirements

Issue 2-1-1: Whether to define PMI reporting requirements

- Agreement:

  - Not to define PMI reporting requirements in the Rel-17 Further enhancement on NR demodulation performance WI

Issue 2-1-1a: whether to add the following note: ‘RAN4 will continue the discussion on the PMI reporting performance with inter-cell interference condition in Rel-17 FeMIMO WI performance part’

- Agreement:

  - Not to add such note

# Interference Model

Issue 2-2-1: CSI-IM on target cell

- Agreement:

  - Only consider overlapping with PDSCH from interference

Issue 2-2-2: NZP CSI-RS on target cell

- Agreement:

  - Overlaps with NZP CSI-RS from interference

# Simulation assumption for CQI requirements

Issue 2-3-1: Antenna configuration for serving cell

- Option 1: 2T2R, 2T4R, ULA Low

- Option 2: 1x2/1x4 ULA Low

Issue 2-3-2: Channel model for serving cell

- Agreement:

  - TDLA30-5

Issue 2-3-3: Antenna configuration for interference cell (s)

- Agreement:

  - 1Tx

Issue 2-3-4: Channel model for interference cell (s)

- Agreement:

  - Static

Issue 2-3-5: INR value

- Agreement:

  - One cell with 10.04 dB (DIP -0.41dB)

Issue 2-3-6: Other parameters

- Agreement:

  - Reuse from Rel-15 CQI test with fading channel

Issue 2-3-7: Test metric

- Agreement:

  - Re-use the test metric as LTE CQI reporting in ICI

    - the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ γ;

    - when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP, the average BLER for the indicated transport formats shall be greater than or equal to X%.

Issue 2-3-8: Requirements defination

- Agreement:

  - SINR, γ and X values should be decided based on the simulation results

  - Criteria will be further decided in the next meeting based on simulation results

    - Test metric for CQI reporting in ICI is chosen such that it cannot be met with MMSE processing (example).

# Reference

[1] R4-2115732, “WF on CSI requirements for inter-cell interference MMSE-IRC”, Ericsson, RAN4#100e, Aug,2021