3GPP TSG-RAN WG4 Meeting # 101-e 												R4-2120719

Electronic Meeting, November 01-12, 2021

Agenda item:			4.1.4

Source:	Moderator (Huawei)

Title:	Email discussion summary for [101-e][302] NR_Conformance_Maintenance

Document for:	Information

0 Introduction

The scope of this email discussion is to discuss the contributions submitted at agenda 4.1.4 on NR BS conformance maintenance.

- Topic #1: OTA measurement system set-up

- Topic #2: Sweep time for unwanted emission testing

- Topic #3: Test configuration for NC operation

- Topic #4: TC2 correction

- Topic #5: OTA BS with Luant modem testing

- Topic #6: Other Maintenance CR

The following contributions are moving to agenda 4.1.8	BS demodulation requirements.

| R4-2117414 | draft CR for TS 38.141-1 (Rel-15) correction on AWGN power level for PUCCH format 3 |
| --- | --- |
| R4-2117415 | draft CR for TS 38.141-1 (Rel-16) editorial correction on AWGN power level for PUCCH format 3 |
| R4-2117416 | draft CR for TS 38.141-1 (Rel-17) editorial correction on AWGN power level for PUCCH format 3 |

The following contributions are moving to thread 301.

| R4-2117212 | Draft CR to TS 38.141-1: Correction on tables for Band 23 co-location requirements |
| --- | --- |
| R4-2117213 | Draft CR to TS 38.141-1: Correction on tables for Band 23 co-location requirements |
| R4-2117214 | Draft CR to TS 38.141-1: Correction on tables for Band 23 co-location requirements |
| R4-2117215 | Draft CR to TS 38.141-2: Correction on tables for Band 23 co-location requirements |
| R4-2117216 | Draft CR to TS 38.141-2: Correction on tables for Band 23 co-location requirements |
| R4-2117217 | Draft CR to TS 38.141-2: Correction on tables for Band 23 co-location requirements |
| R4-2117218 | Draft CR to TS 37.105: Correction on tables for Band 23 co-location requirements |
| R4-2117219 | Draft CR to TS 37.105: Correction on tables for Band 23 co-location requirements |
| R4-2117220 | Draft CR to TS 37.105: Correction on tables for Band 23 co-location requirements |
| R4-2117221 | Draft CR to TS 37.105: Correction on tables for Band 23 co-location requirements |
| R4-2117222 | Draft CR to TS 37.145-1: Correction on tables for Band 23 coexistence and co-location requirements |
| R4-2117223 | Draft CR to TS 37.145-1: Correction on tables for Band 23 coexistence and co-location requirements |
| R4-2117224 | Draft CR to TS 37.145-1: Correction on tables for Band 23 coexistence and co-location requirements |
| R4-2117225 | Draft CR to TS 37.145-1: Correction on tables for Band 23 coexistence and co-location requirements |
| R4-2117226 | Draft CR to TS 37.145-2: Correction on tables for Band 23 co-location requirements |
| R4-2117227 | Draft CR to TS 37.145-2: Correction on tables for Band 23 co-location requirements |
| R4-2117228 | Draft CR to TS 37.145-2: Correction on tables for Band 23 co-location requirements |
| R4-2118089 | Draft CR to 38.141-1: BS OBUE Cat B requirements clarification |
| R4-2118090 | Draft CR to 38.141-1: BS OBUE Cat B requirements clarification |
| R4-2118091 | Draft CR to 38.141-1: BS OBUE Cat B requirements clarification |
| R4-2118092 | Draft CR to 38.141-2: BS OBUE Cat B requirements clarification |
| R4-2118093 | Draft CR to 38.141-2: BS OBUE Cat B requirements clarification |
| R4-2118094 | Draft CR to 38.141-2: BS OBUE Cat B requirements clarification |
| R4-2119278 | Maintenance CR for TS 37.105 Section 9.7.6.4.3.2 |
| R4-2119279 | Maintenance CR for TS 37.105 Section 9.7.6.4.3.2 |
| R4-2119280 | Maintenance CR for TS 37.105 Section 9.7.6.4.3.2 |
| R4-2119281 | Maintenance CR for TS 37.145-1 section 6.6.6.5.2.5 and 6.6.6.5.2.6 |
| R4-2119282 | Maintenance CR for TS 37.145-1 section 6.6.6.5.2.5 and 6.6.6.5.2.6 |
| R4-2119283 | Maintenance CR for TS 37.145-1 section 6.6.6.5.2.5 and 6.6.6.5.2.6 |
| R4-2119284 | Maintenance CR for TS 37.145-2 section 6.7.6.4.5.1.1 |
| R4-2119285 | Maintenance CR for TS 37.145-2 section 6.7.6.4.5.1.1 |
| R4-2119286 | Maintenance CR for TS 37.145-2 section 6.7.6.4.5.1.1 |

- Topic #1: OTA measurement system set-up

On OTA PWS measurement system, it is the followed-up discussion from previous meetings. One discussion paper and corresponding draft CRs are submitted for the topic.

  - Companies’ contributions summary

(Cat A CRs are not listed)

| T-doc number | Company | Proposal summary |
| --- | --- | --- |
| R4-2118538 | CAICT | The contribution provides the analysis to confirm that PWS is available for co-location measurement cases. Proposal 1: Endorse the draft CRs on the addition of PWS in co-location related annex E clauses in 38.141-2 r15, r16 and r17 latest versions. |
| R4-2118458 | CAICT | draftCR to 38.141-2: Addition of Plane Wave Synthesizer in OTA measurement system set-up |

1.2 Companies views’ collection for 1st round

1.2.1 CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2118458 | Ericsson: we are in favor of introducing PWS in the list of methods, but we would like to see a note stating the limitations for this method in terms of maximum supported power in the room Also, the existing figure does not reflect the geometry and architecture of PWS. |
|  | CAICT: 1. We agree to add a note followed by each “PWS” in this draft CR to state the limitation as follows: “Note: The maximum rated Power Density (PD) per section of the PWS area (e.g. dBm/cm2) might be restricted depending on the implementation. This Power Density at a specific reference plane can be calculated for each transmitter as a function of the total radiated power, the test distance and the radiation pattern of the transmitter.” 2. Regarding the figure issue, the OTA Chamber figures shown in Annex E of 38.141-2 are all intended to be generic, while the specific structure of each system can be found in TR 37.941. |

1.3 Summary for 1st round

1.3.1CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2118538 | To be Noted |
| R4-2118458 | To be revised, more clarification is needed |

1.4 Discussion on 2nd round

| CR/TP number | Comments collection |
| --- | --- |
| R4-2120634 (Revision of R4-2118458) | Moderator: No comment received on 2nd round, agreeable |

2 Topic #2: Sweep time for unwanted emission testing

On sweep time for unwanted emission testing, one discussion paper and corresponding draft CRs are submitted for the topic.

2.1 Companies’ contributions summary

(Cat A CRs are not listed)

| T-doc number | Company | Proposal summary |
| --- | --- | --- |
| R4-2117395 | CATT | Observation 1: Sweep time has critical impact for some emission test. However there is no definition on the sweep time for the concerning test in the current specification. Proposal 1: it is proposed to include the clarification on sweep time for true RMS detection mode in the general part of unwanted emission requirements. Unless otherwise stated, for true RMS detection mode the sweep time shall be set at least as (sweep points)*(symbol length) to improve the measurement accuracy. |
| R4-2117396 | CATT | draft CR for TS 38.141-1 On sweep time for unwanted emission testing (Rel-15) |
| R4-2117399 | CATT | draft CR for TS 38.141-2 On sweep time for unwanted emission testing (Rel-15) |

2.2 Companies views’ collection for 1st round

2.2.1 CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117396 | Nokia: We do not have issue with the proposed changes, but we do not see them necessary. The proposed changes, if agreed on, could also be considered for other RATs, e.g., E-UTRA. |
|  | Huawei: We support to add the clarification on the sweep time setting. Without such a clarification, the test people may use auto mode and will result a peak value rather than an average value. We are also supportive to make the change for other RATs. |
|  | Keysight: We don’t support this proposal and analysis. - Adding proposed sweep time text as mandated condition makes measurement time longer. - Proposed change possibly changes variation of result but not about accuracy. In order to improve result variation, there are other method rather setting sweep time as proposed. This choice is left for test implementer to choose. It’s good to be left for test implementer to decide what method to do (when it’s needed) as of current text. - Description regarding with relation with OFDM symbol length and sweep time is irrelevant for unwanted emission measurement. - For Huawei, if measured result is peak or RMS is determined by detector type, not relevant to proposed sweep time. Use of RMS detector is already specified. |
|  | CATT: As clarified in R4-2117395, different signals have different characteristics which may needs different seep time. We think it’s better to clarify it in 3GPP specifications. Otherwise it may increase the risk of failing a good equipment depending on the choice of the tester. The intention is not to mandate anything but give a minimum reasonable sweeping time setting considering the signal characteristic. |
|  | Qualcomm: We have no problem with the proposed change and the CR’s objective. We do agree with CATT that it is important to include a minimum reference value within the spec for the sweeping time. Huawei: to Keysight, yes, the result is determined by detector mode, i.e. peak or RMS, with correct setting on other parameters such as sweep time. Our concern is that we specify unwanted emission as RMS detector, but it will result a value as using peak detector in the test when sweep time Auto+ max hold is used. I believe it is not intended but it always happen if we do not have some clarification in the specification. |
|  | Ericsson: it is a good approach to make some clarifications about sweep time, but this requires more discussion and agreements, as the settings might be dependent on measurement equipment characteristics and particularities. We support further discussion on this topic |
|  | Keysight2: additional comment; It’s good to mention again, this proposed text makes measurement time longer. Considering unwanted emission measurement is TRP for OTA, total time for measurement becomes significantly longer. For those who worries about the risk described here, it’s still possible to makes sweep time longer as proposed without having such text in conformance spec. With current text, this trade off, risk vs measurement time is left for those who do test (tester or vender, or operator). However, adding such text forces this test to be longer measurement time. As CATT described, the intention is not to mandate, then proposed text is not necessary. And once again, I would emphasize, Unwanted emission measurement has no relation with OFDM symbol length. For Huawei, your example, use of max hold is simply a mis-setting with RMS detector specified. |
|  | NEC: We do not support to add the clarification on the sweep time setting. We have the same view as Keysight. Why do we need to double the sweeping time for the unwanted emission measurement in general when SCS of the wanted signal is changed from 30 kHz to 15 kHz, for example? NOTE 45 referred in the contribution is only applicable to the spurious frequency range of 5815-5855 MHz for band 47. In other word, the sweep time of at least (sweep points)*(symbol length) is not applied to other frequency range or other band. |
|  | CATT: it’s not appropriate to mandate something. But there should be a minimum requirement in the specification. The current spec just mentions RMS, which is not enough for conducting correct test. This is not an issue of test freedom. It may cause incorrect test setting. We agree with Nokia that, other specs (e,g E-UTTA and MSR) also need similar modifications. We think companies might need time to understand this issue. A WF on calling paper for the next meeting would be preferred in this meeting. |
|  | Anritsu: While we understand the concern of CATT and we agree that the correct sweep time should be chosen with the RMS detection mode, we should collect more materials before deciding the appropriate sweep time since this parameter may become a big factor of the total test time especially for the OTA measurement. Since at this moment we cannot judge whether the 100% symbol length is conservative (a bit too long) or necessary, it is appreciated if companies can bring analysis with regards to the necessary length per one sampling point compared to the symbol length to get a stable result. (e.g. whether 100% symbol length is necessary, 50%, or 10%?) There is one more thing that we need to remember that this requirement shall not be a burden for another measurement method, i.e. FFT sampling method by the signal analyzer. |
| R4-2117399 | Nokia: Same comments as R4-2117396. |
|  | Huawei: same comments as above |
|  | Keysight: same comments as above |
|  | CATT: same comments as above. |

2.3 Summary for 1st round

2.3.1CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2117395 R4-2117396 R4-2117399 | Based on 1st round of comments collection, some companies are supportive to the draft CR while some other companies think the proposed text is not necessary. Most of companies support to further discussion on the topic. Moderator suggest to have a WF to identify the issue and discuss the plan for future meeting. R4-2117395: to be noted R4-2117396, R4-2117399: Not Pursued |

2.4 Discussion on 2nd round

| WF number | comments collection |
| --- | --- |
| R4-2120636 | WF on sweep time for unwanted emission testing Huawei: We support the WF Nokia: We are ok with the Keysight version. Ericsson: the sweep time should not be tied to symbol rate or modulation. From regulatory perspective, what matters is RMS, measurement bandwidth. To add more information will just make it take more time to measure. We are for Option 4 in the draft WF Moderator: agreeable, companies are ok with version provided by Keysight. |

3 Topic #3: Test configuration for NC operation

The CR provides corrections to the test conguration for NC operation which are constructed with fixed number of carriers.

3.1 Companies’ contributions summary

(Cat A CRs are not listed)

| T-doc number | Company | Proposal summary |
| --- | --- | --- |
| R4-2117885 | Huawei, HiSilicon | Correction on the test configuration for NC operation 37.141 R15 Reason/Summary of change: Existing NTC3 is constructed with fixed two carriers to reflect high PSD scenarios. The test with total number of supported carriers may not be required, but wider CBW and/or more carrier may be placed to reach the rated total output power. |
| R4-2117886 | Huawei, HiSilicon | Correction on the test configuration for NC operation 37.141 R16 Reason/Summary of change: Existing NTC3 is constructed with fixed two carriers to reflect high PSD scenarios. The test with total number of supported carriers may not be required, but wider CBW and/or more carrier may be placed to reach the rated total output power. NTC21 has the similar issue |
| R4-2117888 | Huawei, HiSilicon | Correction on the test configuration for NC operation 37.145-1 R15 Reason/Summary of change: Existing ANTC3 and ANTC6 are constructed with fixed two carriers to reflect high PSD scenarios. The test with total number of supported carriers may not be required, but wider CBW and/or more carrier may be placed to reach the rated total output power. |
| R4-2117889 | Huawei, HiSilicon | Correction on the test configuration for NC operation 37.145-1 R16 Reason/Summary of change: Existing ANTC3 and ANTC6 are constructed with fixed two carriers to reflect high PSD scenarios. The test with total number of supported carriers may not be required, but wider CBW and/or more carrier may be placed to reach the rated total output power. ANTC8 has similar issue |
| R4-2117891 | Huawei, HiSilicon | Correction on the test configuration for NC operation 37.145-2 R15 Reason/Summary of change: Existing ANTC3, and ANTC7 are constructed with fixed two carriers to reflect high PSD scenarios. The test with total number of supported carriers may not be required, but wider CBW and/or more carrier may be placed to reach the rated total output power. |
| R4-2117892 | Huawei, HiSilicon | Correction on the test configuration for NC operation 37.145-2 R16 Reason/Summary of change: ANTC3, ANTC7 and ANTC9 are constructed with fixed two carriers to reflect high PSD scenarios. The test with total number of supported carriers is not required which need to corrected. |

3.2 Companies views’ collection for 1st round

3.2.1 CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117885 | Nokia: We agree in principle with the proposed changes, but we should still ensure the number of supported carriers will not be exceeded when placing more carriers within the BS RF Bandwidth. |
|  | Huawei: to Nokia, that is the reason we propose to use wider channel bandwidth firstly, then add more carrier if still there are some output power room. We are open on the further change and text proposal for it would be appreciated. |
|  | Ericsson: we think it is too late to make these changes in MSR now, we cannot simply delete those statements from NTC 21 |
|  | Huawei: response to Ericsson, the statement doesn’t help for this test configuration which is not intended to test at max number of carriers. And also we add some text to address the comment received in last meeting. As said, we are open on the further change and your text proposal would be appreciated. If Ericson think it is too late for the change to the earlier releases, we are ok to change Rel-17 only. |
| R4-2117886 | Nokia: Same comments as R4-2117885. |
|  | Ericsson: same comments as for R4-2117885. |
| R4-2117888 | Nokia: Same comments as R4-2117885. |
|  | Ericsson: same comments as for R4-2117885. |
| R4-2117889 | Nokia: Same comments as R4-2117885. |
|  | Ericsson: same comments as for R4-2117885. |
| R4-2117891 | Nokia: Same comments as R4-2117885. |
|  | Ericsson: same comments as for R4-2117885. |
| R4-2117892 | Nokia: Same comments as R4-2117885. |
|  | Ericsson: same comments as for R4-2117885. |

3.3 Summary for 1st round

3.3.1CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2117885 ~ R4-2117892 | To return to, more discussion is needed |

3.4 Discussion on 2nd round

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117885 | Ericsson: we appreciate that we will discuss this offline until next meeting and find together a proper text for this TC. Also, changes are to be made from rel 17. Huawei: ok, let’s come back next meeting. |
|  | Moderator: Not Pursued, more discussion is needed |
| R4-2117886 | Moderator: Not Pursued,, more discussion is needed |
| R4-2117888 | Moderator: Not Pursued,, more discussion is needed |
| R4-2117889 | Moderator: Not Pursued,, more discussion is needed |
| R4-2117891 | Moderator: Not Pursued,, more discussion is needed |
| R4-2117892 | Moderator: Not Pursued,, more discussion is needed |

4 Topic #4: TC2 correction

In RAN4#99-e corrections to 36.141 and 38.141-1 on ETC2 and NRTC2 were agreed.

4.1 Companies’ contributions summary

(Cat A CRs are not listed)

| T-doc number | Company | Proposal summary |
| --- | --- | --- |
| R4-2118727 | Nokia, Nokia Shanghai Bell | Draft CR to TS 37.145-1: ATC2b correction (rel-15) Reason/Summary of change: Instead of testing all carrier bandwidth combinations with different sum of channel bandwidth, only smallest and largest sum of channel bandwidth is tested. |
| R4-2118730 | Nokia, Nokia Shanghai Bell | Draft CR to TS 37.145-2: ATCR2b correction (rel-15) Reason/Summary of change: Instead of testing all carrier bandwidth combinations with different sum of channel bandwidth, only smallest and largest sum of channel bandwidth is tested. |
| R4-2118733 | Nokia, Nokia Shanghai Bell | Draft CR to TS 38.141-2: NRTC2 correction (rel-15) Reason/Summary of change: Instead of testing all carrier bandwidth combinations with different sum of channel bandwidth, only smallest and largest sum of channel bandwidth is tested. |

4.2 Companies views’ collection for 1st round

4.2.1 CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2118727 | Huawei: agree |
|  | CATT: Agree |
|  | Ericsson: agree |
| R4-2118730 | Huawei: agree |
|  | CATT: Agree |
|  | Ericsson: agree |
| R4-2118733 | Huawei: agree |
|  | CATT: Agree |
|  | Ericsson: agree |

4.3 Summary for 1st round

4.3.1CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2118727 | agreeable |
| R4-2118730 | agreeable |
| R4-2118733 | agreeable |

4.4 Discussion on 2nd round (if applicable)

5 Topic #5: OTA BS with Luant modem testing

5.1 Companies’ contributions summary

(Cat A CRs are not listed)

| T-doc number | Company | Proposal summary |
| --- | --- | --- |
| R4-2119000 | Ericsson | OTA BS with Luant modem testing Observation #1: The luant model specification is specified with reference point of BS antenna connector which is BS type 1-C Proposal-1: Luant modem testing aspect should be removed from the BS OTA test specification. Proposal-2: Make clarification on the TS 37.461 |
| R4-2119001 | Ericsson | CR on removal of Luant modem in radiated performance specification Reason/Summary of change: Luant modem only support BS type 1-C |
| R4-2119004 | Ericsson | CR on clarification scope on Luant modem Reason/Summary of change: Unclear about the Luant modem specification applicability on type 1-H |

5.2 Companies views’ collection for 1st round

5.2.1 CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2119001 | Nokia: Whether RET can be used with BS Type 1-O or 2-O could be considered as a functional modification, so this may not be a Category F change. |
|  | ZTE: we would like to have more time to check whether RET is not necessary for BS type 1-O and BS type 2-O |
|  | Ericsson: thanks Nokia comments, the CR type is ok to change to functional modification. To ZTE: Ok to wait confirmation from ZTE, do you mean feedback during the 2nd round or till next meeting? |
|  | Huawei: it may also not necessary for 1-H, we would like to check if other companies share the same view. |
| R4-2119004 | Nokia: Same comments as R4-2119001. |
|  | Samsung: It seems TS37.461 is RAN3 specification. Not sure whether RAN4 could submit CR directly to this TS. |
|  | ZTE: the same comment as R4-2119001. In addition this will have impacts on IAB spec. |
|  | Ericsson: To Samsung, RAN4 may endorse it first and then submit to RAN3? As this CR related to the RAN4 conformance specification, so RAN4 endorsement is needed. We can submit it to RAN3 after endorsement from RAN4. |

5.3 Summary for 1st round

5.3.1CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2119000 | To be Noted |
| R4-2119001 | To be revised, modification is needed and companies request more time to check. |
| R4-2119004 | To be return to, more discussion is needed. |

5.4 Discussion on 2nd round

| CR/TP number | Comments collection |
| --- | --- |
| R4-2120635 (Revision of R4-2119001) | Nokia: According to 3GPP Working Procedure: C; Functional modification of feature; Any functional modification shall correspond to an identified Work Item. However backward compatibility shall be ensured when the issue has an impact on the UE. This category shall not be used for a frozen Release, except for alignment CRs as described in clause 4.7.. |
|  | This type of CR to remove an optional feature should be Category C, and thus be considered only for open release. From technical perspective, Nokia currently does not see the necessity of the CR, as the clause 4.5.4 is only applicable for ‘BS with integrated Iuant BS modem’, thus it is not applicable for ‘BS without integrated Iuant BS modem’. If the issue is in TS 37.461, then there is no need to change TS 38.141-2, your proposed change may be interpreted that Iuant cannot be used in AAS or OTA BS (even as an internal interface). |
|  | Huawei: we agree that it may be no need to change 38.141, if there is not Iuant BS modem, then just do nothing. |
|  | Moderator: Not Pursued |
| R4-2120758 (Revisions of R4-2119004) | Nokia: Same comment as Revision of R4-2119001. |
|  | Huawei: We share similar view as Samsung. It will be RAN3 scope. CR can be submitted to RAN3 directly. |
|  | Moderator: Technically endorsed |

6 Topic #6: Other Maintenance CR

6.1 Companies’ contributions summary

(Cat A CRs are not listed)

| T-doc number | Company | Proposal summary |
| --- | --- | --- |
| R4-2117894 | Huawei, HiSilicon | General rule for resolution bandwidth 38.141-2 R15 Reason/Summary of change: The resolution bandwidth can be smaller than the measurement bandwidth is not clarified in TS 38.141-2, while it does in other specifications such as TS 38.141-1. |

6.2 Companies views’ collection for 1st round

6.2.1 CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117894 | CATT: Agree |

6.3 Summary for 1st round

6.3.1CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2117894 | agreeable |

6.4 Discussion on 2nd round (if applicable)

7Recommendations for Tdocs

7.1 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on sweep time for unwanted emission testing | CATT |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2117395 | Discussion on sweep time for unwanted emission testing | CATT | Noted |  |
| R4-2117396 | draft CR for TS 38.141-1 On sweep time for unwanted emission testing (Rel-15) | CATT | Not Pursued |  |
| R4-2117399 | draft CR for TS 38.141-2 On sweep time for unwanted emission testing (Rel-15) | CATT | Not Pursued |  |
| R4-2117885 | Correction on the test configuration for NC operation 37.141 R15 | Huawei, HiSilicon | Return to |  |
| R4-2117886 | Correction on the test configuration for NC operation 37.141 R16 | Huawei, HiSilicon | Return to |  |
| R4-2117888 | Correction on the test configuration for NC operation 37.145-1 R15 | Huawei, HiSilicon | Return to |  |
| R4-2117889 | Correction on the test configuration for NC operation 37.145-1 R16 | Huawei, HiSilicon | Return to |  |
| R4-2117891 | Correction on the test configuration for NC operation 37.145-2 R15 | Huawei, HiSilicon | Return to |  |
| R4-2117892 | Correction on the test configuration for NC operation 37.145-2 R16 | Huawei, HiSilicon | Return to |  |
| R4-2117894 | General rule for resolution bandwidth 38.141-2 R15 | Huawei, HiSilicon | Agreeable |  |
| R4-2118458 | draftCR to 38.141-2: Addition of Plane Wave Synthesizer in OTA measurement system set-up | CAICT | Revised |  |
| R4-2118538 | Discussion on Verification of Co-Location Measurement by using PWS | CAICT | Noted |  |
| R4-2118727 | Draft CR to TS 37.145-1: ATC2b correction (rel-15) | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2118730 | Draft CR to TS 37.145-2: ATCR2b correction (rel-15) | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2118733 | Draft CR to TS 38.141-2: NRTC2 correction (rel-15) | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2119000 | OTA BS with Luant modem testing | Ericsson | Noted |  |
| R4-2119001 | CR on removal of Luant modem in radiated performance specification | Ericsson | Revised |  |
| R4-2119004 | CR on clarification scope on Luant modem | Ericsson | Return to |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

7.2 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2117885 | Correction on the test configuration for NC operation 37.141 R15 | Huawei, HiSilicon | Not Pursued |  |
| R4-2117886 | Correction on the test configuration for NC operation 37.141 R16 | Huawei, HiSilicon | Not Pursued |  |
| R4-2117888 | Correction on the test configuration for NC operation 37.145-1 R15 | Huawei, HiSilicon | Not Pursued |  |
| R4-2117889 | Correction on the test configuration for NC operation 37.145-1 R16 | Huawei, HiSilicon | Not Pursued |  |
| R4-2117891 | Correction on the test configuration for NC operation 37.145-2 R15 | Huawei, HiSilicon | Not Pursued |  |
| R4-2117892 | Correction on the test configuration for NC operation 37.145-2 R16 | Huawei, HiSilicon | Not Pursued |  |
| R4-2120634 | draftCR to 38.141-2: Addition of Plane Wave Synthesizer in OTA measurement system set-up | CAICT | Agreeable |  |
| R4-2120635 | CR on removal of Luant modem in radiated performance specification | Ericsson | Not Pursued |  |
| R4-2120758 | CR on clarification scope on Luant modem | Ericsson | endorsed |  |
| R4-2120636 | WF on sweep time for unwanted emission testing | CATT | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Keysight | Takao Miyake | takao_miyake@keysight.com |
| NEC | Tetsu Ikeda | tetsu ikeda@nec.com |
| Anritsu | Osamu Yamashita | Osamu.Yamashita@anritsu.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)