3GPP TSG-RAN WG4 Meeting # 101-e 											      	R4-2120742

Electronic Meeting, 1st - 12th Nov., 2021

Agenda item:			8.13.3&8.13.4

Source:	Moderator (CATT)

Title:						Email discussion summary for [101-e][311] NTN_Solutions_Part3

Document for:	Information

Introduction

This E-mail thread will address the following issues for NTN BS and UE

- BS RF requirements

  - General aspects

  - TX requirements

  - RX requirements

- UE RF requirements

  - TX requirements

  - RX requirements

Topic #1: BS aspects

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2119556 | THALES | Observation 1: Depending on the scenario, the equation used to compute ACIR is different, and is summarized in the Table for evaluation methodology. In some cases, we need to evaluate ACIR of NTN UE, in some other cases the ACIR of NTN Sat, or the ACIR of TN UE, or the ACIR of TN gNB. The simulation assumptions are therefore changing from scenario to scenario, the ACIR is computed differently based on different equation (the equation is different each time) and the parameters to be varied are also different. Observation 2: From the scenario 2, it can be seen that increasing the ACS of the NTN Satellite node beyond 35 dB will not have a noticeable impact on the ACIR of the NTN Satellite node. Proposal 1: RAN4 shall not consider the ACS of the NTN Satellite node higher than 35 dB. Observation 3: From the scenario 3, it can be seen that increasing the ACLR of the NTN Satellite node beyond 25 dB will not have a noticeable impact on the ACIR of the TN UE. Proposal 2: RAN4 shall not consider the ACLR of the NTN Satellite node higher than 25 dB. Observation 4: From the scenario 5, it can be seen that increasing the ACLR of the NTN UE beyond 25 dB will not have a noticeable impact on the ACIR of the TN UE. For this reason it makes sense to use the same ACLR requirements for both TN UE and NTN UE. Proposal 3: RAN4 shall consider the ACLR of the NTN UE same as for TN UE. Proposal 4: RAN4 shall consider the ACS of the NTN UE same as for TN UE. |
| R4-2117381 | CATT | Proposal 1: it is proposed to replace “SATELLITE NODE” with “Sat-BS” in the box. Proposal 2: it is proposed to introduce NTN BS classes for LEO1200 and LEO600 scenarios in Rel-17 Proposal 3: it is proposed that Output power dynamics requirement is reused for NTN. Proposal 4: it is proposed to adopt 0.05 ppm for NTN BS. Proposal 5: it is proposed to consider lower modulation QPSK and 16QAM and exclude 64QAM and 256QAM in Rel-17. Proposal 6: It is proposed that only single carrier transmission is considered for NTN in Rel-17. TAE requirement is not needed in Rel-17. Proposal 7: RAN4 to make a decision whether ACLR/UEM requirement should be based on NTN-NTN co-existence or not. - If yes, NTN-NTN simulation needs to be moved to Phase 1. Proposal 8: Tx intermodulation requirement is not needed for NTN BS in Rel-17. |
| R4-2118160 | Ericsson | Observation 1: RAN4 will set requirements for the “satellite access node” as a whole. RAN4 will not specify requirement for any sub-component. Observation 2: The reference point for satellite access node type 1-H would most likely be located in the NTN payload. Proposal : Specify satellite access node’ spurious emissions based on ERC 74-01 Space stations’ limits, i.e. ITU Radio Regulations Annex 3. |
| R4-2119205 | ZTE Corporation | Proposal 1: to support both BS type 1-H and BS type 1-O in Rel-17. Proposal 2: not to define BS station output limitation with WID update that co-channel coexistence between different NTN BSs are not included in Rel-17 scope. Proposal 3: not to define the RE power dynamic range requirement. Proposal 4: only to define QPSK and 16QAM for NTN based and reuse TN BS EVM requirements; Proposal 5: to define MIMO related TAE requirement as 65ns and FFS for CA related TAE requirements; Proposal 6: to seek NTN vendors’ feedback on MCL between Tx antenna and Rx antenna of own BS and other surrounding BS. Proposal 7: propose to define Tx intermodulation requirements for NTN BS; Proposal 8: to support the NTN BS type 1-O with following OTA peak directions set and OTA coverage range declaration. |
| R4-2117382 | CATT | Proposal 1: it is proposed to introduce REFENSE requirement for NTN BS as shown in Table 2.1-1. - SNR operating point, NF and IM can be reused for S band. FFS on noise figure. Proposal 2: it is proposed that RAN4 decide which this requirement should be based on evaluations on NTN-NTN co-existence. - If yes, NTN-NTN scenario needs to move to Phase 1 and simulation should be done ASAP. Proposal 3: out-of-band blocking requirement is not needed for satellite BS. Proposal 4: receiver intermodulation requirement is not needed for satellite BS. Proposal 5: RAN4 discuss whether to keep ICS requirement for NTN BS. - If yes, the interference level should be changed for NTN BS. |
| R4-2118161 | Ericsson | Proposal1: Consider the satellite access node’s noise figure as a whole, starting from the values in Table 1. Observation1: Inputs are expected from satellite companies to better evaluate any impact of the feeder link + NTN Gateway sub-components on the noise figure (if any impact). Observation2: As agreed in RAN4#100-e, NR FRCs (as described above) should be re-used for REFSENS. Satellite; GEO; LEO 600; LEO 1200. G/T (dB K-1); 19; 1.1; 1.1. G_Rx (dBi); 51; 30; 30. NF (dB); 7.4; 4.3; 4.3. Table 1: Noise figure for GEO, LEO600 and LEO1200 |
| R4-2119142 | Huawei | TP on adding Rx requirement for TR 38.863. |
| R4-2119206 | ZTE Corporation | Proposal 1: to adopt the following noise figure for NTN BS Rx requirement definition; Proposal 2: not to define Rx dynamic range requirements for GEO NTN BS; Proposal 3: to define Rx dynamic range requirements with IoT level as18dBc for LEO600KM NTN BS; Proposal 4: to define Rx dynamic range requirements with IoT level as12dBc for LEO1200KM NTN BS; Proposal 5: to propose maximum in-channel selectivity as [IoT level+9dB] dB for NTN BS; Proposal 6: to postpone the Rx intermodulation requirement discussion until we could have clear and confirmed agreement on spectral utilization and stable coexistence simulation assumption and outcome; Proposal 7: to support the NTN BS type 1-O with following OSDD declaration and OTA REFSENS RoAoA declaration. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1 General aspects

Sub-topic description:

In RAN4#100e meeting, it has been decided to use the following figure as starting point for NTN BS type 1-H requirement development. How to name the “NTN payload RF + Gateway + Non-NTN infrastructure gNB functions” in the box of figure 1.2.1-1 remains FFS.

Figure 1.2.1-1 reference point for BS type 1-H

Issue 1-1-1: How to name “NTN payload RF + Gateway + Non-NTN infrastructure gNB functions” in the box of Figure 1.2.1-1?

- Proposals

  - Option 1: replace “Satellite Node” with “Sat-BS”

  - Option 2: replace “Satellite Node” with “satellite access node”

  - Option 3: Others, please specify

- Recommended WF

  - TBA

Option 2 is already concluded in Tuesday’s GTW.

Sub-topic 1-2 Tx requirements for Satellite BS

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-2-1: Base station output power

- Proposals

  - Option 1: Define different power classes for GEO/LEO/MEO/HEO etc. the following framework is proposed.

| NTN BS class | Prated,c,AC |
| --- | --- |
| NTN BS class A (TBD) | (Note) |
| NTN BS class B (LEO1200) | ≤ TBD dBm |
| NTN BS class C (LEO600) | ≤ TBD dBm |
| NOTE: FFS |  |

  - Option 2: Other, please specify.

- Recommended WF

  - TBA

Conclusion in Tuesday’s GTW

Agreement:

The power limitation on “satellite access node” are manufacture declaration basis, no limitation in RAN4 specification.

Some background information from regulatory can be considered to be included in the TR for information.

Issue 1-2-2: RE power dynamic range

- Proposal

  - Option 1: The current RE power dynamic range can be reused for Satellite BS

  - Option 2: not to define the RE power dynamic range requirement.

    - If this option, please also provide reason for removal of this requirement.

- Recommended WF

  - TBA

Issue 1-2-3: total power dynamic range

- Proposal

  - Option 1: The current total power dynamic range requirement can be reused.

    - The co-existence simulations indicate that there is no need to change the ACLR, SU can be reused from NR.

  - Option 2: Other, please specify.

- Recommended WF

  - TBA

Conclusion on Tuesday’s GTW

Option 2 is concluded.

The current total power dynamic range requirement can be reused.

- If the co-existence simulations indicate that there is no need to change the ACLR, SU can be reused from NR.

Issue 1-2-4: Frequency error

- Proposal

  - Option 1: The current requirement for WA BS can be reused, i.e. 0.05ppm.

  - Other, please specify.

- Recommended WF

  - TBA

Conclusion on Tuesday’s GTW:

Option 1:  The current requirement for WA BS can be reused, i.e. 0.05ppm.

Issue 1-2-5: Modulation quality (EVM)

- Proposal

  - Option 1: The current requirement can be reused for QPSK and 16 QAM for satellite BS.

    - 64QAM and 256QAM is not considered in Rel-17.

  - Option 2: others, please specify

- Recommended WF

  - TBA

Conclusion on Tuesday’s GTW:

For Satellite Access Node DL transmission:

- The current requirement can be reused for QPSK and 16 QAM

- FFS whether 64QAM can be supported in FR1.

- 256QAM is not supported by Satellite Access node in Rel-17.

Issue 1-2-5a: Companies are invited to further check whether to support 64QAM for not for FR1,

- Proposal

  - Option 1: Yes

  - Option 2: No.

- Recommended WF

  - TBA

Issue 1-2-6: Time alignment error

- Proposal

  - Option 1: The TAE requirement is not needed for satellite BS in Rel-17.

    - CA feature is not considered at this stage

  - Option 2: TAE requirement for MIMO is reused for satellite BS and FFS on TAE for CA.

- Recommended WF

  - TBA

Conclusion on Tuesday’s GTW:

CA is out of Rel-17 NTN WI scope for RAN4 requirements

FFS whether MIMO cases supported in Rel-17 NTN WI scope.

Issue 1-2-6a: Whether to include TAE requirement for MIMO?

- Proposal

  - Option 1: Yes.

Please clarify in which condition MIMO can be supported for NTN

  - Option 2: No. propagation condition does not support MIMO transmission.

- Recommended WF

  - TBA

Issue 1-2-7: ACLR

- Proposal

  - Option 1: RAN4 decides which scenario the ACLR should be based on, NTN-NTN or NTN-TN.

  - Option 2: The ACLR of the NTN Satellite node shall be no higher than 25 dB according to NTN – TN scenario simulation.

  - Option 3: Others, please specify

- Recommended WF

  - TBA

Issue 1-2-8: Transmitter spurious emissions

- Proposal

  - Option 1: To follow the recommendation of ITU-R SM.329 and reuse current definition in 38.104.

  - Option 2: Specify satellite access node’ spurious emissions based on ERC 74-01 Space stations’ limits, i.e. ITU Radio Regulations Annex 3.

- Recommended WF

  - TBA

Issue 1-2-9: Protection of the BS receiver of own or different BS

- Proposal

  - Option 1: Seek NTN vendors’ feedback on MCL between Tx antenna and Rx antenna of own BS and other surrounding BS.

  - Option 2: Other, please specify.

- Recommended WF

  - TBA

Issue 1-2-10: Co-location with other BS

- Proposal

  - Option 1: This requirement is not needed since there is no co-location scenario foreseen for satellite.

  - Option 2: Other, please specify.

- Recommended WF

  - TBA

Issue 1-2-11: Transmitter intermodulation

- Proposal

  - Option 1: This requirement is not needed since there is no nearby interfering signal reaching the transmitter unit via the antenna, RDN and antenna array.

  - Option 2: Define Tx intermodulation requirements for NTN BS.

    - Pending further input from satellite operators on the exact scenario and MCL value.

- Recommended WF

  - TBA

Issue 1-2-12: Transmitter requirement for NTN BS type 1-O

Only 1 company has input for BS type 1-O as shown in the following table 1.2.2-1. Companies please provide comments on how to proceed with BS type 1-O?

  - Option 1: Use the analysis in Table 1.2.2-1 as starting point and discuss in the next RAN4 meeting based on more inputs.

  - Option 2: Other, please specify.

Table 1.2.2-1 Tx requirement for NTN BS type 1-O

| Tx requirement | Classification | Coverage range |  | Number of |
| --- | --- | --- | --- | --- |
|  |  | FR1 | FR2 | conformance directions |
| Radiated transmit power | Directional | OTA peak directions set | OTA peak directions set | 5 |
| OTA BS output power | TRP | See annex I |  |  |
| OTA output power dynamics | Directional | OTA peak directions set | OTA peak directions set | 1 |
| OTA transmitter OFF power | Co-location | See clause 4.12 | N/A | See clause 4.12 |
|  | Directional | N/A | OTA peak directions set (Note 2) | 1 |
| OTA transient period | Co-location | See clause 4.12 | N/A | See clause 4.12 |
|  | Directional | N/A | OTA peak directions set (Note 2) | 1 |
| OTA modulation quality | Directional | OTA coverage range | OTA coverage range | 5 |
| OTA frequency error | Directional | OTA coverage range | OTA coverage range | 1 |
| OTA time alignment error | Directional | OTA coverage range | OTA coverage range | 1 |
| OTA occupied bandwidth | Directional | OTA coverage range | OTA coverage range | 1 |
| OTA ACLR | TRP | N/A | N/A | See annex I |

| OTA operating band unwanted emission | TRP | N/A | N/A | See annex I |
| --- | --- | --- | --- | --- |

| OTA transmitter spurious emission | General requirement | TRP | N/A | N/A | See annex I |
| --- | --- | --- | --- | --- | --- |
|  | Protection of the BS receiver of own or different BS | Co-location | See clause 4.12 | N/A | See clause 4.12 |
|  | Additional spurious emissions | TRP | N/A | N/A | See annex I |
|  | Co-location with other base stations | Co-location | See clause 4.12 | N/A | See clause 4.12 |
| OTA transmitter intermodulation |  | Co-location | See clause 4.12 | N/A | See clause 4.12 |
| NOTE 1: Directional requirement does not imply one compliance direction only. The directional requirement applies to a single direction at a time. NOTE 2: For FR2, RF Core requirements are defined on TRP levels. Conformance requirements are verified by EIRP measurements in the reference direction. |  |  |  |  |  |

Sub-topic 1-3 Rx requirements for Satellite BS

Sub-topic description

There should be no disagreement to use the following equation for satellite BS REFSENS requirement development. The open issue is how to determine each item in the formula.

Open issues and candidate options before e-meeting:

Issue 1-3-1: FRC

- Proposals

  - Option 1: The FRCs specified in TS 38.104 shall be re-used to specify the satellite node Rx requirements. Specifically the following FRCs and DMRS pattern is re-used.

|  | BS Channel BW | Subcarrier spacing | Number of PRBS |
| --- | --- | --- | --- |
| G-FR1-A1-1 | 5 MHz | 15 kHz | 25 |
| G-FR1-A1-2 | 5 MHz | 30 kHz | 11 |
| G-FR1-A1-3 | 10 MHz | 60 kHz | 11 |
| G-FR1-A1-4 | 20 MHz | 15 kHz | 106 |
| G-FR1-A1-5 | 20 MHz | 30 kHz | 51 |
| G-FR1-A1-6 | 20 MHz | 60 kHz | 24 |

  - Option 2: Other FRC’s are defined for satellite BS to address the low SNR operating point.

- Recommended WF

  - TBA

Conclusion on Tuesday’s GTW:

Agreement: taking option 1 as starting point.

Issue 1-3-2: Noise figure

- Proposals

  - Option 1: Consider the satellite access node’s noise figure as a whole and use the following as starting point.

    - Satellite companies are encouraged to input on this.

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 19 | 1.1 | 1.1 |
| G_Rx (dBi) | 51 | 30 | 30 |
| NF (dB) | 7.4 | 4.3 | 4.3 |

  - Option 2: Other, please specify.

- Recommended WF

  - TBA

Conclusion on Tuesday’s GTW:

Agreement: Option 1

Issue 1-3-3: Implementation margin

- Proposals

  - Option 1: reuse the implementation margin for TN in the corresponding frequency. E.g. 2GHz IM for S band and 1.5GHz IM for L band.

  - Option 2: Other, please specify.

- Recommended WF

  - TBA

Conclusion on Tuesday’s GTW:

Agreement: Reusing margin from TN as [2dB].

Issue 1-3-4: Dynamic range

- Proposals

  - Option 1: agree to use the following proposals,

    - define IoT level as 18dBc for LEO600KM NTN BS;

    - define IoT level as 12dBc for LEO1200KM NTN BS;

    - not to define Rx dynamic range requirements for GEO NTN BS;

  - Option 2: companies to confirm the IoT value by system-level simulation for co-channel interference.

  - Other, please specify

- Recommended WF

  - TBA

Issue 1-3-5: ACS

- Proposals

  - Option 1: RAN4 decides which scenario ACS should be based on. NTN-NTN or TN-NTN?

  - Option 2: The ACS of the NTN Satellite node shall not be higher than 35 dB

  - Option 3, other, please specify

- Recommended WF

  - TBA

Issue 1-3-6: In-band blocking/out-of-band blocking

- Proposals

  - Option 1: to decide by system level simulations on CDF of blocking signal.

  - Option 2: Other, please specify.

- Recommended WF

  - TBA

Issue 1-3-7: co-location requirement

- Proposals

  - Option 1: This requirement is not needed as there is no co-location scenario for satellite BS

  - Option 2: Other, please specify.

- Recommended WF

  - TBA

Issue 1-3-8: Receiver intermodulation

- Proposals

  - Option 1: This requirement is not needed considering the scenario of satellite BS.

  - Option 2: Postpone the Rx intermodulation requirement discussion until we could have clear and confirmed agreement on spectral utilization and stable coexistence simulation assumption and outcome.

- Recommended WF

  - TBA

Issue 1-3-9: In-channel selectivity

- Proposals

  - Option 1: RAN4 discuss whether to keep ICS requirement for NTN BS considering the calibration results, where the CL CDF is within 5dB difference.

    - If, yes, the interference level should be changed for NTN BS.

  - Option 2: Other, please specify.

- Recommended WF

  - TBA

Issue 1-3-10: Receiver requirement for NTN BS type 1-O

Only 1 company has input for BS type 1-O as shown in the following table 1.2.3-1. Companies please provide comments on how to proceed with BS type 1-O?

  - Option 1: Use the analysis in Table 1.2.3-1 as starting point and discuss in the next RAN4 meeting based on more inputs.

  - Option 2: Other, please specify.

Table 1.2.3-1 NTN BS Rx requirement for type 1-O

| Rx requirement | Classification | Applicability levels |  | Coverage range |  | Number of |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | FR1 | FR2 | FR1 | FR2 | conformance directions |
| OTA sensitivity | Directional | Minimum EIS | N/A | OSDD | N/A | 5 |
| OTA reference sensitivity | Directional | OTA REFSENS | OTA REFSENS | OTA REFSENS RoAoA |  | 5 |
| OTA Dynamic range | Directional | OTA REFSENS | N/A | OTA REFSENS RoAoA | N/A | 1 |
| OTA adjacent channel selectivity | Directional | minSENS | OTA REFSENS | minSENS RoAoA | OTA REFSENS RoAoA | 1 |
| OTA in-band blocking | Directional | OTA REFSENS and minSENS | OTA REFSENS | OTA REFSENS RoAoA and minSENS RoAoA | OTA REFSENS RoAoA | 5 |

| OTA out-of-band blocking | General requirement | Directional | minSENS | OTA REFSENS | minSENS RoAoA | OTA REFSENS RoAoA | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | Co-location with other base stations | Co-location (Note 2) | minSENS | N/A | minSENS RoAoA | N/A | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OTA receiver spurious emissions |  | TRP | See clause 7.7 | See clause 7.7 | N/A | N/A | See annex I |
| OTA receiver intermodulation |  | Directional | OTA REFSENS and minSENS | OTA REFSENS | OTA REFSENS RoAoA and minSENS RoAoA | OTA REFSENS RoAoA | 1 |
| OTA in-channel selectivity |  | Directional | minSENS | OTA REFSENS | minSENS RoAoA | OTA REFSENS RoAoA | 1 |
| NOTE 1: Directional requirement does not imply one compliance direction only. The directional requirement applies to a single direction at a time. NOTE 2: The compliance direction for co-location blocking is applicable for the wanted signal only but not the interfering signal. |  |  |  |  |  |  |  |

Companies views’ collection for 1st round

Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Example 1

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |

Example 2

Sub topic 1-1

| Company | Comments |
| --- | --- |
| ZTE | We also propose NTN BS type 1-H d definition to accommodate all architectures in Figure 2/3/4 which is not captured under this topic. Figure 2. reflector antenna architecture with beam port/[RF connector] Figure 3. Lens antenna architecture with beam port/[RF connector] Figure 4. antenna array architecture with beam port/[RF connector] |
| Ericsson | See RAN4#101-e GTW agreement |
| Nokia | Issue 1-1-1: See GTW agreements |
| CATT | Issue 1-1-1: See GTW agreements |
| THALES | See RAN4#101-e GTW agreements. Note: It is true that the issue mentioned by ZTE has not been discussed in [311], but it will be discussed in [309]. However, the current representation seems to be sufficiently general. |

Sub topic 1-2

| Company | Comments |
| --- | --- |
| ZTE | Issue 1-2-1: Base station output power MEO an HEO should be removed from option 1. In addition, as we mentioned at least GEO and LEOU should be defined, regarding whether it’s necessary to further split LEO into LTEO600KM and LEO120KM, this could be further discussed later on. In addition, our proposal 2 is missing and not captured in this summary. Proposal 2: not to define BS station output limitation with WID update that co-channel coexistence between different NTN BSs are not included in Rel-17 scope Issue 1-2-2: RE power dynamic range We don’t see any necessity to define RE power dynamic range requirements for NTN. Issue 1-2-3: total power dynamic range We need to wait for the coexistence study in coexistence study agenda. Issue 1-2-4: Frequency error More clarifications for option 1 might be needed whether this is applicable for all NTN BS class or only GEO or LEO here; Issue 1-2-6: Time alignment error We prefer to go with option 2 and also seek for the clarifications from satellite vendors. Issue 1-2-7: ACLR We need to wait for the coexistence study in coexistence study agenda. Issue 1-2-8: Transmitter spurious emissions We prefer to start with option 2 Issue 1-2-9: Protection of the BS receiver of own or different BS We prefer to go with option 1; Issue 1-2-10: Co-location with other BS It should be same story as issue 1-2-9 and the current summary is not aligned with proposals in R4-2119205. Issue 1-2-11: Transmitter intermodulation Prefer to go with option 2. Issue 1-2-12: Transmitter requirement for NTN BS type 1-O We would like any discussion on the proposed table in this meeting instead of postpone the discussion to next meeting. |
| Ericsson | Issue 1-2-1: See RAN4#101-e GTW agreement Issue 1-2-2: Option1. Issue 1-2-3: See RAN4#101-e GTW agreement Issue 1-2-4: See RAN4#101-e GTW agreement Issue 1-2-5: See RAN4#101-e GTW agreement Issue 1-2-5a: Yes, we think 64QAM might still be possible with some receivers that mitigate the issue. Issue 1-2-6: See RAN4#101-e GTW agreement. Issue 1-2-7: option 3, ACLR value will be decided once we agreed on ACIR value from coex studies, there is no other technical justification. Issue 1-2-8: option 2 Issue 1-2-9: need further discussion Issue 1-2-10: option 1 most likely Issue 1-2-11: most likely option 1: what could be co-located, what would be the interefer? Further clarification would be needed from ZTE here. Issue 1-2-12: Not sure wha to agree on here…. With option 1, some of the requirements should not be applicable here (e.g. OTA OFF power, OTA transient period, … which are applicable for TDD only) |
| Nokia | Issue 1-2-1/3/4/5/6: See GTW agreement Issue 1-2-2: Option1. Issue 1-2-5a: We would like to still consider 64QAM at least in DL. Issue 1-2-6a: No strong view. Issue 1-2-7: We need to conclude the co-ex study which will facilitate agreeing these values. Issue 1-2-8: Option 2 Issue 1-2-9: Option 1 Issue 1-2-10: Pending feedback but would suspect option 1. If no co-location requirement is defined this deployment scenario shall not be supported by 3GPP specification. Issue 1-2-11: Pending feedback but would suspect option 1. If no co-location requirement is defined this deployment scenario shall not be supported by 3GPP specification. Issue 1-2-12: RAN4 is contribution driven. No need to agree anything here. |
| Huawei | Additional comments on top of GTW agreements: Issue 1-2-5a: 64QAM Option 1: Yes, consider 64QAM. EVM requirement details may be further discussed next meeting. Issue 1-2-6a: TAE, MIMO After checking the WID in RP-211784, it seems that MIMO is not explicitly mentioned. Considering we will use AAS architecture as baseline (1-H, 1-O), removal of the MIMO TAE requirement may be premature. It is suggested to further discuss if the time alignment for the beamforming may be achieved indirectly by using other RF requirements. Issue 1-2-7: ACLR In our view, ACLR may be better than the 25 dB, i.e in range of [20-22] dB. Therefore option 2 as baseline is ok. Issue 1-2-8: Transmitter spurious emissions Option 2 as baseline. Both recommendations seems to be required to be further analysed (ERC 74-01 refers to SM.329), as to where derive the globally applicable spur limits. Issue 1-2-9: Protection of the BS receiver of own or different BS So far the need for this requirement is still unclear. If sufficient MCL can be assumed, there is no need for this requirement. Issue 1-2-10: Co-location with other BS Option 1: deployment specifics seems to be sufficient to reassure no need for such RAN4-driven requirements, especially considering low number of NTN bands so far. Still, we may need to clarify what to we mean by “NTN co-location scenario” Issue 1-2-11: Transmitter intermodulation Option 1: no good motivation for the IMD requirements was seen so far. Issue 1-2-12: Transmitter requirement for NTN BS type 1-O Option 1 as baseline. We would like to provide more inputs next meeting. |
| CATT | Issue 1-2-1/3/4/5/6: See GTW agreement Issue 1-2-2: Option1. We think RE power boosting/de-boosting is still useful for NTN. This requirement should be remained. Issue 1-2-5a: we are fine to keep 64QAM for DL. Issue 1-2-6a: Option 2. MIMO should not be out of Rel-17 scope. RAN1’s evaluation and discussion was based on single antenna port for NTN. The operating scenario and feasibility for MIMO has never been discussed in RAN1. With this background, we think it’s not appropriate to introduce requirement for MIMO in RAN4. Issue 1-2-7: Wait for the co-existence study. Issue 1-2-8: Fine with Option 2 Issue 1-2-9: Option 1 Issue 1-2-10: Option 1 Issue 1-2-11: Option 1 Issue 1-2-12: RAN4 is contribution driven. No need to agree anything here. |
| THALES | Issue 1-2-1: See RAN4#101-e GTW agreement Issue 1-2-2: Option 2. We don’t see any necessity to define RE power dynamic range requirements for NTN. Issue 1-2-3: See RAN4#101-e GTW agreement Issue 1-2-4: See RAN4#101-e GTW agreement Issue 1-2-5: See RAN4#101-e GTW agreement Issue 1-2-5a: We do not think 64QAM is possible in FR1 with current UE (handheld) antenna gains. We will not oppose to majority, but 64QAM does not make too much sense in FR1, even for DL. We encourage companies to check once more the simulation results from e.g. the coexistence studies in 310. Note: For FR2 is maybe easier since VSAT/ESIM is used and the antenna gain and power are higher. Issue 1-2-6: See RAN4#101-e GTW agreement. Issue 1-2-6a: As mentioned during RAN4#101-e, MIMO is discussed at RAN-P level for future Release Package. Is too early to discuss it in RAN4. Current WID (RP-211784) is not mentioning MIMO and current discussion in RAN-P are obvious enough. We therefore suggest a new option: - Option 3: Yes, in a feature release, according to RAN-P schedule. Issue 1-2-7: Option 3, or Option 4: - ACLR is in the range of 15-20 dBs according to simulations results in the coexistence studies. - ACS is in the range of 15-20 dBs (if TN FDD) or in the range of max 30-35dBs (if TN TDD) Issue 1-2-8: We could use option 2 as baseline. Issue 1-2-9: Option 1, but which are the other surrounding BS? Issue 1-2-10: Option 1. Issue 1-2-11: Option 1. Issue 1-2-12: Wait for contributions on 1-O. The table is mentioning FR2, is this a mistake? |

Sub topic 1-3

| Company | Comments |
| --- | --- |
| ZTE | Issue 1-3-1: FRC Fine with option 1, however companies are willing to check its lowest level for REFSENS, we are also open for further discuss it since Low SNR is expected in NTN system. Issue 1-3-2: Noise figure We are fine with option 1. This should be decided as soon as possible, otherwise the coexistence study results might be impacted. Issue 1-3-3: Implementation margin We would like to see more clarifications on option 1 on its proposed values. Issue 1-3-4: Dynamic range Both option 1and option 2 is fine for us. Issue 1-3-5: ACS We need to wait for the coexistence study in coexistence study agenda. Issue 1-3-6: In-band blocking/out-of-band blocking Fine with option 1 Issue 1-3-7: co-location requirement Similar as Tx co-location requirement,we would like to seem more inputs from NTN vendors. Issue 1-3-8: Receiver intermodulation Prefer to go with option 2. Issue 1-3-9: In-channel selectivity Our proposal is not captured in this summary. Proposal 5: to propose maximum in-channel selectivity as [IoT level+9dB] dB for NTN BS; Issue 1-3-9: Receiver requirement for NTN BS type 1-O Similar as Tx side,we would like any discussion on the proposed table in this meeting instead of postpone the discussion to next meeting. |
| Ericsson | Issue 1-3-1: option 1, See RAN4#101-e GTW agreement To ZTE: What we are targeting to verify here is the noise figure of the receiver, the SNR doesn’t have to reflect the intended conditions of operation as long as it could be tested in a lab. Issue 1-3-2: See RAN4#101-e GTW agreement Issue 1-3-3: See RAN4#101-e GTW agreement Issue 1-3-4: option 2, but option 1 is good input to start with. Issue 1-3-5: option 3, ACS value will be decided once we agreed on ACIR value from coex studies, there is no other technical justification. Issue 1-3-6: option 1 Issue 1-3-7: most likely option 1, need further input from satellite manufacturer to conclude. Issue 1-3-8: most likley option 1, why should we postpone dsicussion on this requirement? Moderator: Option 2 where it mentions to postpone this requirement comes from ZTE’s paper. Issue 1-3-9 (ICS): Option 1, some adjustment of TN BS requirement could be further discussed. Issue 1-3-10 (1-O): It would be good if ZTE could clarify what it their intention with this table. It seems like it captures current TN status, but how it applies to NTN? |
| Nokia | Issue 1-3-1/2/3: See GTW agreements Issue 1-3-4: Option 1 could be the assumption but shall be confirmed so added in [] Issue 1-3-5: We need to conclude the co-ex study which will facilitate agreeing these values. Issue 1-3-6: Option 1 Issue 1-3-7/8: Pending feedback. If no co-location requirement is defined this deployment scenario shall not be supported by 3GPP specification. |
| CATT | Issue 1-3-1/2/3: See GTW agreements Issue 1-3-4: Option 2. But option 1 is good input to start with. Issue 1-3-5: Wait for the co-existence study. Issue 1-3-6: Option 1 Issue 1-3-7: Option 1 Issue 1-3-8: Option 1 Issue 1-3-9: Option 1, some adjustment of TN BS requirement could be further discussed. Issue 1-3-10: it’s hard to comment since these table only lists 1-O requirements but without analysis. |
| Huawei | Issue 1-3-4: Dynamic range The dynamic range requirement assumes certain absolute level wanted signal values, as well as nearby interferer. Due to high pathloss and high isolation for NTN scenario, such interferer may be unrealistic to assume. More analysis needed to confirm IoT values. Issue 1-3-5: ACS Align with the ACLR decision. Issue 1-3-6: In-band blocking/out-of-band blocking Issue 1-3-7: co-location requirement Option 1: not needed, as discussed for TX Issue 1-3-8: Receiver intermodulation Option 1: not needed, as discussed for TX Issue 1-3-9: In-channel selectivity Similar to the dynamic range, we are ok with Option 1 to further discuss whether to keep the ICS requirement. Issue 1-3-9: Receiver requirement for NTN BS type 1-O. Option 1 as baseline. We would like to provide more inputs next meeting. |
| THALES | Issue 1-3-1: Option 1, See RAN4#101-e GTW agreement. However, do we need all the mentioned configurations? For example, for RRM we do not consider 60kHz configuration. We can consider it of course, but for some reason is left for FFS.. We want also to mention that given NF values (see also TR 38.821) should cover all cases. Issue 1-3-2: See RAN4#101-e GTW agreement Issue 1-3-3: See RAN4#101-e GTW agreement Issue 1-3-4: Both option 2 and option 1 are fine Issue 1-3-5: According to our results in the coexistence studies in R4-2120628, ACS is in the range of 15-20 dBs (if TN FDD) and should be maximum 30-35dBs (if TN TDD). So Option 3, with the following justification for the worst case (TN TDD & LEO@1200): If TN FDD, the ACS requirement is much lower. The observations in R4-2119556 were purely empirical, and also to show how and which parameters are varied, and also the asymptotic regions of the variation. Issue 1-3-6: Option 1; Issue 1-3-7: Option 1; Issue 1-3-8: Option 1; Issue 1-3-9: Option 1; Issue 1-3-10: Same discussion as previously for 1-O. We should wait for contributions on 1-O, and how is this applied for NTN? The table is mentioning FR2, is this a mistake? |

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2119142 | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Issue 1-1-1: How to name “NTN payload RF + Gateway + Non-NTN infrastructure gNB functions” in the box of Figure 1.2.1-1? - Proposals - Option 1: replace “Satellite Node” with “Sat-BS” - Option 2: replace “Satellite Node” with “satellite access node” - Option 3: Others, please specify Tentative agreements: According to RAN4#101e GTW session, Option 2 is agreed. Recommendations for 2nd round: No. |
| Sub-topic #1-2 | Issue 1-2-1: Base station output power - Proposals - Option 1: Define different power classes for GEO/LEO/MEO/HEO etc. the following framework is proposed. NTN BS class; Prated,c,AC. NTN BS class A (TBD); (Note). NTN BS class B (LEO1200); ≤ TBD dBm. NTN BS class C (LEO600); ≤ TBD dBm. NOTE: FFS. - Option 2: Other, please specify. Tentative agreements: According to RAN4#101e GTW session, The power limitation on “satellite access node” are manufacture declaration basis, no limitation in RAN4 specification. Some background information from regulatory can be considered to be included in the TR for information. Recommendations for 2nd round: No. |
|  | Issue 1-2-2: RE power dynamic range - Proposal - Option 1: The current RE power dynamic range can be reused for Satellite BS - Option 2: not to define the RE power dynamic range requirement. - If this option, please also provide reason for removal of this requirement. - Recommended WF - TBA Tentative agreements: Option 1 Recommendations for 2nd round: No |
|  | Issue 1-2-3: total power dynamic range - Proposal - Option 1: The current total power dynamic range requirement can be reused. - The co-existence simulations indicate that there is no need to change the ACLR, SU can be reused from NR. - Option 2: Other, please specify. Tentative agreements: According to RAN4#101e GTW session, Option 2 is agreed. The current total power dynamic range requirement can be reused. If the co-existence simulations indicate that there is no need to change the ACLR, SU can be reused from NR. Recommendations for 2nd round: |
|  | Issue 1-2-4: Frequency error - Proposal - Option 1: The current requirement for WA BS can be reused, i.e. 0.05ppm. - Other, please specify. - Recommended WF - TBA Tentative agreements: Conclusion on Tuesday’s GTW: Option 1: The current requirement for WA BS can be reused, i.e. 0.05ppm. Recommendations for 2nd round: No |
|  | Issue 1-2-5: Modulation quality (EVM) - Proposal - Option 1: The current requirement can be reused for QPSK and 16 QAM for satellite BS. - 64QAM and 256QAM is not considered in Rel-17. - Option 2: others, please specify - Recommended WF - TBA Tentative agreements: For Satellite Access Node DL transmission: - The current requirement can be reused for QPSK, 16 QAM and 64QAM in FR1. - 256QAM is not supported by Satellite Access node in Rel-17. Recommendations for 2nd round: No |
|  | Issue 1-2-6: Time alignment error - Proposal - Option 1: The TAE requirement is not needed for satellite BS in Rel-17. - CA feature is not considered at this stage - Option 2: TAE requirement for MIMO is reused for satellite BS and FFS on TAE for CA. - Recommended WF - TBA Tentative agreements: CA is out of Rel-17 NTN WI scope for RAN4 requirements Recommendations for 2nd round: Further discuss whether MIMO cases supported in Rel-17 NTN WI scope. (GTW discussion preferred.) |
|  | Issue 1-2-7: ACLR - Proposal - Option 1: RAN4 decides which scenario the ACLR should be based on, NTN-NTN or NTN-TN. - Option 2: The ACLR of the NTN Satellite node shall be no higher than 25 dB according to NTN – TN scenario simulation. - Option 3: Others, please specify Tentative agreements: Wait the conclusion from co-existence E-mail thread. Recommendations for 2nd round: No |
|  | Issue 1-2-8: Transmitter spurious emissions - Proposal - Option 1: To follow the recommendation of ITU-R SM.329 and reuse current definition in 38.104. - Option 2: Specify satellite access node’ spurious emissions based on ERC 74-01 Space stations’ limits, i.e. ITU Radio Regulations Annex 3. Tentative agreements: Option 2 Recommendations for 2nd round: |
|  | Issue 1-2-9: Protection of the BS receiver of own or different BS - Proposal - Option 1: Seek NTN vendors’ feedback on MCL between Tx antenna and Rx antenna of own BS and other surrounding BS. - Option 2: Other, please specify. Tentative agreements: Option 1 Recommendations for 2nd round: Propose to go for GTW discussion on the scenarios. |
|  | Issue 1-2-10: Co-location with other BS - Proposal - Option 1: This requirement is not needed since there is no co-location scenario foreseen for satellite. - Option 2: Other, please specify. Tentative agreements: Option 1 Recommendations for 2nd round: No. |
|  | Issue 1-2-11: Transmitter intermodulation - Proposal - Option 1: This requirement is not needed since there is no nearby interfering signal reaching the transmitter unit via the antenna, RDN and antenna array. - Option 2: Define Tx intermodulation requirements for NTN BS. - Pending further input from satellite operators on the exact scenario and MCL value. Tentative agreements: Option 1 Recommendations for 2nd round: |
|  | Issue 1-2-12: Transmitter requirement for NTN BS type 1-O Only 1 company has input for BS type 1-O as shown in the following table 1.2.2-1. Companies please provide comments on how to proceed with BS type 1-O? - Option 1: Use the analysis in Table 1.2.2-1 as starting point and discuss in the next RAN4 meeting based on more inputs. - Option 2: Other, please specify. Tentative agreements: Option 1 Recommendations for 2nd round: |
| Sub-topic #1-3 | Issue 1-3-1: FRC - Proposals - Option 1: The FRCs specified in TS 38.104 shall be re-used to specify the satellite node Rx requirements. Specifically the following FRCs and DMRS pattern is re-used. BS Channel BW; Subcarrier spacing; Number of PRBS. G-FR1-A1-1; 5 MHz; 15 kHz; 25. G-FR1-A1-2; 5 MHz; 30 kHz; 11. G-FR1-A1-3; 10 MHz; 60 kHz; 11. G-FR1-A1-4; 20 MHz; 15 kHz; 106. G-FR1-A1-5; 20 MHz; 30 kHz; 51. G-FR1-A1-6; 20 MHz; 60 kHz; 24. - Option 2: Other FRC’s are defined for satellite BS to address the low SNR operating point. Tentative agreements: Option 1 Recommendations for 2nd round: No |
|  | Issue 1-3-2: Noise figure - Proposals - Option 1: Consider the satellite access node’s noise figure as a whole and use the following as starting point. - Satellite companies are encouraged to input on this. Satellite; GEO; LEO 600; LEO 1200. G/T (dB K-1); 19; 1.1; 1.1. G_Rx (dBi); 51; 30; 30. NF (dB); 7.4; 4.3; 4.3. - Option 2: Other, please specify. Tentative agreements: Option 1 Recommendations for 2nd round: No |
|  | Issue 1-3-3: Implementation margin - Proposals - Option 1: reuse the implementation margin for TN in the corresponding frequency. E.g. 2GHz IM for S band and 1.5GHz IM for L band. - Option 2: Other, please specify. Tentative agreements: Reusing margin from TN as [2dB]. Recommendations for 2nd round: No |
|  | Issue 1-3-4: Dynamic range - Proposals - Option 1: agree to use the following proposals, - define IoT level as 18dBc for LEO600KM NTN BS; - define IoT level as 12dBc for LEO1200KM NTN BS; - not to define Rx dynamic range requirements for GEO NTN BS; - Option 2: companies to confirm the IoT value by system-level simulation for co-channel interference. Tentative agreements: Companies are encouraged to input the IoT values by system simulation and confirm whether the following proposal is feasible, - define IoT level as 18dBc for LEO600KM NTN BS; - define IoT level as 12dBc for LEO1200KM NTN BS; - not to define Rx dynamic range requirements for GEO NTN BS; Recommendations for 2nd round: No, further discuss in the next meeting. |
|  | Issue 1-3-5: ACS - Proposals - Option 1: RAN4 decides which scenario ACS should be based on. NTN-NTN or TN-NTN? - Option 2: The ACS of the NTN Satellite node shall not be higher than 35 dB - Option 3, other, please specify Tentative agreements: Wait for conclusion in co-existence E-mail thread. Recommendations for 2nd round: No, further discuss in the next meeting. |
|  | Issue 1-3-6: In-band blocking/out-of-band blocking - Proposals - Option 1: to decide by system level simulations on CDF of blocking signal. - Option 2: Other, please specify. Tentative agreements: Option 1: Companies are invited to provide system level simulations on CDF of blocking signal. Recommendations for 2nd round: No, further discuss in the next meeting. |
|  | Issue 1-3-7: co-location requirement - Proposals - Option 1: This requirement is not needed as there is no co-location scenario for satellite BS - Option 2: Other, please specify. Tentative agreements: Option 1 Recommendations for 2nd round: No. |
|  | Issue 1-3-8: Receiver intermodulation - Proposals - Option 1: This requirement is not needed considering the scenario of satellite BS. - Option 2: Postpone the Rx intermodulation requirement discussion until we could have clear and confirmed agreement on spectral utilization and stable coexistence simulation assumption and outcome. Tentative agreements: Option 1 Recommendations for 2nd round: No |
|  | Issue 1-3-9: In-channel selectivity - Proposals - Option 1: RAN4 discuss whether to keep ICS requirement for NTN BS considering the calibration results, where the CL CDF is within 5dB difference. - If, yes, the interference level should be changed for NTN BS. - Option 2: Other, please specify. Tentative agreements: Option1. Companies are invited to input on ICS interference level and decide whether this requirement is needed in the next meeting. Recommendations for 2nd round: No. |
|  | Issue 1-3-10: Receiver requirement for NTN BS type 1-O Only 1 company has input for BS type 1-O as shown in the following table 1.2.3-1. Companies please provide comments on how to proceed with BS type 1-O? - Option 1: Use the analysis in Table 1.2.3-1 as starting point and discuss in the next RAN4 meeting based on more inputs. - Option 2: Other, please specify. Tentative agreements: Option 1. Recommendations for 2nd round: No |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Open issue

It is proposed to further discuss the following topics in the 2nd round.

Issue 1-5-1: Whether to include TAE requirement for MIMO? Please also provide concrete reasons why you think it should be supported or not supported.

- Proposal

  - Option 1: Yes.

  - Option 2: No.

- Recommended WF

  - TBA

Issue 1-5-2: Protection of the BS receiver of own BS requirement is needed?

- Proposal

  - Option 1: Yes, this requirement is needed.

  - Option 2: No

- Recommended WF

  - TBA

Issue 1-5-3: Protection of the BS receiver of different BS requirement is needed?`

- Proposal

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - TBA

Companies views’ collection for 2nd round

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |
| Ericsson | Issue 1-5-1: no strong opinion, we were initially thinking option 2. Issue 1-5-2: FFS, we’ll come back on this in next meeting. Issue 1-5-3: option 2. |
| ZTE | Issue 1-5-1: we would like to come back next meeting and it’s not so urgent issues. Issue 1-5-2: still needed to ensure the coexistence, however the exact value might need more discussion. Issue 1-5-3: we think that on satellite, it will have multiple wireless system as mentioned in our contribution, in future single satellite might be equipped with different bands NTN BS, then we need to ensure coexistence with other receiver. For Issue 1-2-11: Transmitter intermodulation and Issue 1-3-8: Receiver intermodulation , we need to check with NTN vendors and it’s too early to conclude it. As I mentioned per satellite node, there will be multiple wireless system instead of only one. |
| THALES | Issue 1-5-1: it should be Option 2, for the reasons explained above. Issue 1-5-2: Option 2. Issue 1-5-3: Option 2. All these issues may be part of future releases, if required. For the time being is not needed. |
| CATT | Issue 1-5-1: Option 2. There is no reason to support MIMO before RAN1 confirms this feature is feasible. RAN1 discussion in Rel-17 is based on single antenna port. Issue 1-5-2: Could be option 1. . Issue 1-5-3: Option 2. No operating scenarios. |
| Nokia | Issue 1-5-1: Option 2 unless RAN1 confirms design within Rel-17 Issue 1-5-2/3: FFS |
| Huawei | Issue 1-5-1: it looks that the MIMO feature itself is not applicable (while no consensus on this in the group). Still, based on discussion it seems that the TAE may be seen as required due to the beamforming of AAS and the willingness to reassure that the signals forming a beam are coherent. As discussed in other WIs, some companies are of opinion that this can be reassured vie other RF requirements (if the beam is not formed correctly due to potential TAE misalignments, then it will not be detected and decoded properly, e.g. using thp metric). Let’s aim to conclude next meeting – views are still varying. Issue 1-5-2: initial thinking was Option 2, but it all depends on the scenario and deployment specific issues. ZTE concerns could be resolved first, and aim for decision next meeting. Issue 1-5-3: same as 1-5-2. |

Summary of 2nd round

|  | Status summary |
| --- | --- |
| Sub-topic #1-5 | Issue 1-5-1: Whether to include TAE requirement for MIMO? Please also provide concrete reasons why you think it should be supported or not supported. - Proposal - Option 1: Yes. - Option 2: No. - Recommended WF - TBA Per GTW agreement: The baseline assumption: MIMO TAE requirements not applicable pending further checking till Jan 2022 RAN4 meeting Issue 1-5-2: Protection of the BS receiver of own BS requirement is needed? - Proposal - Option 1: Yes, this requirement is needed. - Option 2: No - Recommended WF - TBA Per GTW agreement: Option 1 Issue 1-5-3: Protection of the BS receiver of different BS requirement is needed?` - Proposal - Option 1: Yes - Option 2: No - Recommended WF - TBA Per GTW agreement: Option 2 |

Topic #2: UE aspects

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117746 | Xiaomi | Observation 1: it is not possible the minimum output power for NTN can be defined lower than that for TN (e.g. -40dBm). Proposal 1: the minimum output power requirement in 38.101-1 can be reused for NTN FR1. Proposal 2: if the transmit intermodulation is needed, the requirement in 38.101-1 can be reused for NTN FR1. Proposal 3: if the receiver intermodulation is needed, the requirement in 38.101-1 can be reused for NTN FR1. |
| R4-2117332 | CATT | Observation: MPR/A-MPR requirements are pending further check ACLR/SEM requirement. Proposal 1: The current output power dynamics requirement can be reused for NTN UE. Proposal 2: Transmit signal quality for QPSK defined in clause 6.4 can be reused. 64QAM and 256QAM should be excluded for NTN UL in Rel-17. Proposal 3: RAN4 should decide which scenario should be used for determine the basic ACLR/ACS/SEM requirement. - If NTN-NTN case is used, RAN4 need to do the simulation as soon as possible. |
| R4-2118718 | Huawei, HiSilicon | TP for UE transmitter requirement for TR 38.863. |
| R4-2117333 | CATT | Proposal 1: The option 2 for reference sensitivity in WF [1] can be agreed as start point, i.e. Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain can be reused to specify the REFSENS. - Diversity gain, NF and IM can be reused for S band. FFS on FRC and SNR Proposal 2: The maximum input level requirements for NTN UE can be relaxed XdB based MCL increase, where X needs further discussion to decide. Proposal 3: Intermodulation characteristics requirements can be omitted for NTN. Proposal 4: The spurious emission requirement for NR UE should be reused for NTN FR1 UE. |
| R4-2118717 | Huawei, HiSilicon | Proposal 1: It’s proposed to reuse this sentence to clarify the Rel-17 handheld UE for satellite access. Observation 1: The expected SINR for UL 256QAM can’t be reached for PC3 satellite UE due to the lower UL SINR in the coexistence study. Proposal 2: The additional maximum output power reduction framework for TN UE can be reused for satellite UE. However, the band-specific AMPR requirements depend on the input about the additional spurious emission. Proposal 3: The requirements for Minimum output power, Transmit OFF power, Transmit ON/OFF time mask and Power control in clause 6.3 from 38.101-1 are applicable for the satellite UE. Proposal 4: The requirements for Frequency error and transmit modulation quality in clause 6.4 from 38.101-1 are applicable for the satellite UE. Proposal 5: The requirements for occupied bandwidth, SEM, general spurious emissions and transmit intermodulation are applicable for the satellite UE. Observation 3: If it’s assumed that the NR NTN is used in the area that there is no IMT BS coverage, there is no need to specify the spurious emissions for UE co-existence. Proposal 6: To specify the REFSENS as below. The FRC for receiver requirements for QPSK in clause A.3.2.2 from TS 38.101-1 are applicable for the satellite UE. Table 2: Two antenna port reference sensitivity QPSK PREFSENS Operating band / SCS / Channel bandwidth / Duplex-mode. Operating Band; SCS kHz; 5 MHz (dBm); 10 MHz (dBm); 15 MHz (dBm); 20 MHz (dBm); Duplex Mode. s/n256; 15; -100.0; -96.8; -95.0; -93.8; FDD. 30; -97.1; -95.1; -94.0. 60; -97.5; -95.4; -94.2. s/n255; 15; -100.0; -96.8; -95.0; -93.8; FDD. 30; -97.1; -95.1; -94.0. 60; -97.5; -95.4; -94.2. Table 3: Uplink configuration for reference sensitivity Operating band / SCS / Channel bandwidth / Duplex-mode. Operating Band; SCS kHz; 5 MHz; 10 MHz; 15 MHz; 20 MHz; Duplex Mode. s/n256; 15; 25; 50; 75; 100; FDD. 30; 24; 36; 50. 60; 10; 18; 24. s/n255; 15; 25; 50; 75; 100; FDD. 30; 24; 36; 50. 60; 10; 18; 24. Proposal 7: The requirements for Maximum input level in clause 7.4 from TS 38.101-1 are applicable for satellite UE. Proposal 8: The requirements for Blocking characteristics in clause 7.6 and spurious response in clause 7.7 from TS 38.101-1 are applicable for satellite UE. Proposal 9: The requirements for Intermodulation characteristics in clause 7.8 from TS 38.101-1 are applicable for satellite UE. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1 transmitter requirements

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1-1: clarification on Tx requirement

- Proposals

  - Proposal 1: It’s proposed to reuse this sentence to clarify the Rel-17 handheld UE for satellite access.

    - Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

- Recommended WF

  - It is proposed to agree proposal 1.

Issue 2-1-2: MPR/A-MPR

- Proposals

  - Option 1: MPR, A-MPR requirements should be further evaluated after ACLR/SEM is defined.

  - Option 2: The additional maximum output power reduction framework for TN UE can be reused for satellite UE. However, the band-specific AMPR requirements depend on the input about the additional spurious emission.

- Recommended WF

  - TBA

Issue 2-1-3: Output power dynamics

- Proposals

  - Option 1: The requirements for Minimum output power, Transmit OFF power, Transmit ON/OFF time mask and Power control in clause 6.3 from 38.101-1 are applicable for the satellite UE.

  - Option 2: other, please specify.

- Recommended WF

  - TBA

Issue 2-1-4: Transmit modulation quality

- Proposals

  - Option 1: this requirement can be reused for NTN UE for QPSK and 16QAM.

    - 64QAM and 256QAM should be excluded.

  - Option 2: this requirement can be reused for NTN UE for QPSK, 16QAM and 64QAM.

    - 256QAM should be excluded.

  - Option 3: other, please specify.

- Recommended WF

  - TBA

Issue 2-1-5: ACLR/ACS/SEM

- Proposals

  - Option 1: the TN ACLR/SEM from 38.101-1 shall be reused for NTN UE.

  - Option 2: RAN4 decides which scenario the NTN UE ACLR shall be based on at first.

  - Option 3: other, please specify.

- Recommended WF

  - TBA

Issue 2-1-6: spurious emission for UE co-existence

- Proposals

  - Option 1: confirm the NTN operating scenarios at first.

    - If it’s assumed that the NR NTN is used in the area that there is no IMT BS coverage, there is no need to specify the spurious emissions for UE co-existence.

  - Option 3: other, please specify.

- Recommended WF

  - TBA

Issue 2-1-7: Tx intermodulation

- Proposals

  - Option 1: The requirement is not needed for NTN UE.

  - Option 2: The requirement in 38.101-1 can be reused for NTN FR1.

  - Option 2: other, please specify.

- Recommended WF

  - TBA

Sub-topic 2-2 Receiver requirements

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-2-1: Reference sensitivity

- Proposals

  - Option 1: it is proposed to agree the following,

REFSENS, Sensitivity = -174dBm (kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain

Assuming NF=9dB, same FRC and -1dB SNR operating point, 2.5dB IM and 3dB diversity gain, the following requirement is proposed,

Table 2.2.2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
| s/n256 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | FDD |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |
| s/n255 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | FDD |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |

Table 2.2.2-2: Uplink configuration for reference sensitivity

| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
| s/n256 | 15 | 25 | 50 | 75 | 100 | FDD |
|  | 30 |  | 24 | 36 | 50 |  |
|  | 60 |  | 10 | 18 | 24 |  |
| s/n255 | 15 | 25 | 50 | 75 | 100 | FDD |
|  | 30 |  | 24 | 36 | 50 |  |
|  | 60 |  | 10 | 18 | 24 |  |

  - Option 2: Other, please specify

- Recommended WF

  - TBA

Issue 2-2-2: maximum input level

- Proposals

  - Option 1: Further evaluation is needed considering minimum CL between satellite and UE.

  - Option 2: reuse current requirement from 38.101-1.

  - Option 3: other, please specify.

- Recommended WF

  - TBA

Issue 2-2-3: ACS

- Proposals

  - Option 1: the NTN UE shall reuse the TN UE ACS requirement.

  - Option 2: RAN4 decides which scenarios NTN UE ACS should be based on. TN-NTN or NTN-NTN?

  - Option 3: other, please specify.

- Recommended WF

  - TBA

Issue 2-2-4: intermodulation characteristics

- Proposals

  - Option 1: This requirement is not needed considering the NTN UE operating scenario.

  - Option 2: The requirements for Intermodulation characteristics in clause 7.8 from TS 38.101-1 are applicable for satellite UE.

  - Option 3: Other, please specify.

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Example 1

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |
| Globalstar | Issue 2-1-5: ACLR/ACS/SEM: Use of existing TN requirements will not always be appropriate. 3GPP should consider existing regulatory requirements for L and S band satellite services. Specifically: ETSI EN 301 441 ETSI EN 301 444 ETSI EN 302 574 FCC 47 CFR 25.202, 25.216, 25.253 Issue 2-2-1: Reference Sensitivity: Same comment as above. Issue 2-2-3: ACS: Same comment as above. |
| Skyworks | Issue 2-1-2: MPR/A-MPR: If different ACLR or SEM UE requirement are needed for NTN, it may only affect outer and edge allocations. The A-MPR solution similar to UTRA ACLR could be re-used with a specific satellite NS. Issue 2-2-1: Reference sensitivity: Note: If SU is revisited it may impact REFSENS and UL configuration. |
| Ericsson | Issue 2-1-1: Ok with the proposal Issue 2-1-2: should be discussed once we agree on ACLR (and SEM).. Issue 2-1-3: option 2: there is no power control for UE NTN, right? [There is] Issue 2-1-5: option 3: ACLR value will be decided once we agreed on ACIR value from coex studies, there is no other technical justification. Issue 2-1-6: As we could still have NTN UE at TN cell edges, we should still need to consider ceoxistence with other TN bands here. Issue 2-2-1: option 2 Issue 2-2-3: option 3: ACS value will be decided once we agreed on ACIR value from coex studies, there is no other technical justification |
| Nokia | Issue 2-1-1: Ok with proposed WF Issue 2-1-2: Option 1. Issue 2-1-5: Option 3: Requirements from 38.101-1 shall be the baseline but we need to conclude the co-ex study which will facilitate agreeing these values. Issue 2-1-6: Option 3: If we are not to consider spurious emissions for UE co-existence. Then how to insure this is always the case that “there is no IMT BS coverage”, shall we mandate a NTN UE cannot transmit if TN is detected? |
| CATT | Issue 2-1-1: Ok with the proposal Issue 2-1-2: Option 1. Issue 2-1-3: Option 1. Issue 2-1-4: Either Option 1 or Option 2. Issue 2-1-5: Option 1. This is an urgent issue to be decided. Issue 2-1-6: UE co-existence with other system UE is also needed. Issue 2-1-7: Option 1. Tx intermodulation is in-band requirement. For NTN there is no such scenario. And it’s also not possible for NTN UE and TN UE to operate within the same band in the same region. Issue 2-2-1: No strong view. Option 1 could be starting point for further check. At least the formula can be agreed. Issue 2-2-2: Option 1 Issue 2-2-3: Option 2. This is an urgent issue to be decided. Issue 2-2-4: Option 1. |
| Qualcomm | Issue 2-1-1: Ok with the proposal Issue 2-1-2: Option 1 Issue 2-1-3: Option 1 Issue 2-1-4: Option 2. Per UL link budget, 64QAM is possible. Should not exclude 64QAM at this stage Issue 2-1-5: Option 3: ACLR value will be decided per co-ex studies. TN requirements can be as the baseline. Issue 2-1-6: Need to consider the UE-UE co-ex requirements with other TN bands. Issue 2-1-7: Option 2. Issue 2-2-1: For the REFSENS, n256 has the same frequency range as n24 and n255 has the same frequency range as n65. We can reuse the REFSENS from those two bands. Issue 2-2-2: Option 2 Issue 2-2-3: Option 2. ACS value should be decided from co-ex studies. ACS from TN can be as the baseline. Issue 2-2-4: Option 3. Need further discussion. |
| MediaTek | Issue 2-1-1: ok with proposal Issue 2-1-2: Option 1 Issue 2-1-3: Option 1 Issue 2-1-4: At least Option 2, we would be open to consider Option 1 further. Issue 2-1-5: We have not seen any reason why Option 1 could not be reused, but we should finalise the coexistence studies first. Issue 2-1-6: We would not like to restrict where NTN can be operated. So could take Option 2 as starting point and identify whether there is any value in some relaxation due to lower probability coupling loss with other UEs. Issue 2-1-7: Option 2 as starting point, but would like to further check. Issue 2-2-1: Option 2 – we would like a bit of time to analyse this properly for both bands. The current proposal seems to take the best case. Issue 2-2-2: Option 3 – Option 2 may be an ok starting point, but would like to further check. Issue 2-2-3: Option 2 – fine to wait for outcome of coexistence studies but would consider TN requirement as baseline. Issue 2-2-4: Option 3 – Option 2 may be an ok starting point, but would like a bit of time to further check. |
| Huawei | Issue 2-1-1: clarification on Tx requirement We are OK with proposal 1 to clarify the Rel-17 handheld UE for satellite access. Issue 2-1-2: MPR/A-MPR If no company provide the additional spurious emission for L/S bands before March 2022, we don’t have enough time to specify AMPR requirements. Issue 2-1-3: Output power dynamics Option 1 Issue 2-1-4: Transmit modulation quality Option 2, 64QAM can be kept for NTN UE UL based on the simulation results. Issue 2-1-5: ACLR/ACS/SEM Option 2. Based on our simulation results, 30dB ACLR may be overdesigned. If we relax this requirement, maybe UE don’t need much power reduction for some cases. Issue 2-1-6: spurious emission for UE co-existence ERC 74-01 also have the spurious emission requirements for UE. Companies can further check. Issue 2-1-7: Tx intermodulation Option 1. Issue 2-2-1: Reference sensitivity Option 1 Issue 2-2-2: maximum input level We are open to further evaluate the minimum couple loss between satellite and UE. Issue 2-2-4: intermodulation characteristics Option 1 |
| THALES | Issue 2-1-1: Ok with the proposal and with the way forward. However, please note that current simulations results clearly indicate that NTN UE can reuse TN UE specifications. Actually, NTN UE requirements seem to be lower, and therefore covered by TN UE. RAN4 can consider for NTN UE same ACLR and ACS parameter values as for TN UE, as seen in R4-2120628. Issue 2-1-2: Option 2 is possible (see previous comments), but we can also go for Option 1. Issue 2-1-3: Option 1, we can reuse current requirements from TN UE. Issue 2-1-4: Option 1, and we can reuse current requirements from TN UE. Issue 2-1-5: Option 1, as previously mentioned. Simulations are now available. Issue 2-1-6: Option 1 is correct. For QoS reasons, NTN UEs should operate in NTN only when IMT BS are not available. NTN UEs can also be located at the edge of IMT BS (e.g. below 0 or 5dB SINR in TN). Issue 2-1-7: Option 1 or Option 2 (since same terminal – NTN UE can also be TN UE) for handheld UE. Since NTN UE could be actually a TN UE operating in NTN, option 2 is also possible. Issue 2-2-1: Option 1 or Option 2 with modifications. Are we sure SCS 60kHz is considered? Please see RRM discussions. Issue 2-2-2: Option 1, but we could already use results from calibration simulations/coexistence simulations. However, since NTN UE could be actually a TN UE operating in NTN, option 2 is also possible. Maybe Option 1 can add more reference channels (as we already decided in RAN4 Part1 in the previous meetings). Issue 2-2-3: Option 1. As we mentioned TN UE ACS requirements can be re-used. The proof is here (for the worst case the NTN UE ACS is lower than TN UE ACS) – see R4-2120628: We want to be able to have UEs with double NTN and TN capacity => otherwise what is the purpose of integrating satellite with 5G NR? Issue 2-2-4 (intermodulation): Option 1. However, since NTN UE could be actually a TN UE operating in NTN, option 2 is also possible. |
| Xiaomi | Issue 2-1-2: MPR/A-MPR Whether can be reused or not pending on the conclusion on ACLR in coexistence study Issue 2-1-3: Output power dynamics Minimum output power could be reused considering the similar RF performance for TN and minimum output power is not necessary to defined a lower level than TN system. The same conclusion is also for transmit off power Issue 2-1-7: Tx intermodulation It depends on the operating scenario. Such as whether there is a scenario that UE is in close vicinity of each other with narrow bandwidth signal in adjacent carrier. Issue 2-2-5: intermodulation characteristics Same as above. |

Example 2

Sub topic 1-1

| Company | Comments |
| --- | --- |
| XXX |  |

Sub topic 1-2

| Company | Comments |
| --- | --- |
| XXX |  |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2118718 | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Issue 2-1-1: clarification on Tx requirement - Proposals - Proposal 1: It’s proposed to reuse this sentence to clarify the Rel-17 handheld UE for satellite access. - Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed. Tentative agreements: Agree proposal 1. Candidate options: Recommendations for 2nd round: |
|  | Issue 2-1-2: MPR/A-MPR - Proposals - Option 1: MPR, A-MPR requirements should be further evaluated after ACLR/SEM is defined. - Option 2: The additional maximum output power reduction framework for TN UE can be reused for satellite UE. However, the band-specific AMPR requirements depend on the input about the additional spurious emission. Tentative agreements: Option 1 Candidate options: Recommendations for 2nd round: |
|  | Issue 2-1-3: Output power dynamics - Proposals - Option 1: The requirements for Minimum output power, Transmit OFF power, Transmit ON/OFF time mask and Power control in clause 6.3 from 38.101-1 are applicable for the satellite UE. - Option 2: other, please specify. Tentative agreements: Option 1. Candidate options: Recommendations for 2nd round: |
|  | Issue 2-1-4: Transmit modulation quality - Proposals - Option 1: this requirement can be reused for NTN UE for QPSK and 16QAM. - 64QAM and 256QAM should be excluded. - Option 2: this requirement can be reused for NTN UE for QPSK, 16QAM and 64QAM. - 256QAM should be excluded. - Option 3: other, please specify. Tentative agreements: Option 2 as starting point and further check whether 64QAM needs to be removed later. Candidate options: Recommendations for 2nd round: |
|  | Issue 2-1-5: ACLR/ACS/SEM - Proposals - Option 1: the TN ACLR/SEM from 38.101-1 shall be reused for NTN UE. - Option 2: RAN4 decides which scenario the NTN UE ACLR shall be based on at first. - Option 3: other, please specify. - Recommended WF - TBA Tentative agreements: Wait for co-existence simulation conclusion and further check compliance to the following regulatory (others are also not precluded). ETSI EN 301 441 ETSI EN 301 444 ETSI EN 302 574 FCC 47 CFR 25.202, 25.216, 25.253 Candidate options: Recommendations for 2nd round: |
|  | Issue 2-1-6: spurious emission for UE co-existence - Proposals - Option 1: confirm the NTN operating scenarios at first. - If it’s assumed that the NR NTN is used in the area that there is no IMT BS coverage, there is no need to specify the spurious emissions for UE co-existence. - Option 3: other, please specify. Tentative agreements: UE co-existence with other system UE in other band is also needed. Could Thales compromise to this agreement? Candidate options: Recommendations for 2nd round: |
|  | Issue 2-1-7: Tx intermodulation - Proposals - Option 1: The requirement is not needed for NTN UE. - Option 2: The requirement in 38.101-1 can be reused for NTN FR1. - Option 2: other, please specify. Tentative agreements: There is no clear majority view on either of the above options. Further discuss in the 2nd round. Candidate options: Recommendations for 2nd round: further discuss whether this requirement is needed or not. (GTW is preferred) |
| Sub-topic#2-2 | Issue 2-2-1: Reference sensitivity - Proposals - Option 1: it is proposed to agree the following, REFSENS, Sensitivity = -174dBm (kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain Assuming NF=9dB, same FRC and -1dB SNR operating point, 2.5dB IM and 3dB diversity gain, the following requirement is proposed, Table 2.2.2-1: Two antenna port reference sensitivity QPSK PREFSENS Operating Band; SCS kHz; 5 MHz (dBm); 10 MHz (dBm); 15 MHz (dBm); 20 MHz (dBm); Duplex Mode. s/n256; 15; -100.0; -96.8; -95.0; -93.8; FDD. 30; -97.1; -95.1; -94.0. 60; -97.5; -95.4; -94.2. s/n255; 15; -100.0; -96.8; -95.0; -93.8; FDD. 30; -97.1; -95.1; -94.0. 60; -97.5; -95.4; -94.2. Table 2.2.2-2: Uplink configuration for reference sensitivity Operating Band; SCS kHz; 5 MHz; 10 MHz; 15 MHz; 20 MHz; Duplex Mode. s/n256; 15; 25; 50; 75; 100; FDD. 30; 24; 36; 50. 60; 10; 18; 24. s/n255; 15; 25; 50; 75; 100; FDD. 30; 24; 36; 50. 60; 10; 18; 24. - Option 2: Other, please specify Tentative agreements: The general formula could be agreeable? REFSENS, Sensitivity = -174dBm (kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain Further discuss the NF, SNR, IM, diversity gain and REFSENS requirements in next meeting. Candidate options: Recommendations for 2nd round: |
|  | Issue 2-2-2: maximum input level - Proposals - Option 1: Further evaluation is needed considering minimum CL between satellite and UE. - Option 2: reuse current requirement from 38.101-1. - Option 3: other, please specify. Tentative agreements: Further evaluation is needed considering minimum CL between satellite and UE. Companies are invited for input in the next meeting. Candidate options: Recommendations for 2nd round: |
|  | Issue 2-2-3: ACS - Proposals - Option 1: the NTN UE shall reuse the TN UE ACS requirement. - Option 2: RAN4 decides which scenarios NTN UE ACS should be based on. TN-NTN or NTN-NTN? - Option 3: other, please specify. Tentative agreements: Wait for the co-existence simulation conclusion. Candidate options: Recommendations for 2nd round: |
|  | Issue 2-2-4: intermodulation characteristics - Proposals - Option 1: This requirement is not needed considering the NTN UE operating scenario. - Option 2: The requirements for Intermodulation characteristics in clause 7.8 from TS 38.101-1 are applicable for satellite UE. - Option 3: Other, please specify. Tentative agreements: No clear majority view on either of the above options. Clarification on scenarios is needed at first. Candidate options: Recommendations for 2nd round: further discuss the operating scenarios (GTW discussion is preferred) |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Open issue

It is proposed to further discuss the following issue.

Issue 2-5-1: spurious emission for UE co-existence

- Proposals

  - Option 1: confirm the NTN operating scenarios at first.

    - If it’s assumed that the NR NTN is used in the area that there is no IMT BS coverage, there is no need to specify the spurious emissions for UE co-existence.

  - Option 3: this requirement is needed for co-existence with UE in other bands

- Recommended WF

  - TBA

Issue 2-5-2: Tx intermodulation

- Proposals

  - Option 1: The requirement is not needed for NTN UE.

  - Option 2: The requirement in 38.101-1 can be reused for NTN FR1.

  - Option 2: other, please specify.

- Recommended WF

  - TBA

Issue 2-5-3: Reference sensitivity

- Proposals

  - Option 1: it is proposed to agree the following general formula for NTN UE REFSENS

REFSENS, Sensitivity = -174dBm (kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain

FFS on NF, SNR, IM and diversity gain, and REFSENS and uplink configurations

  - Option 2: Other, please specify

- Recommended WF

  - TBA

Issue 2-5-4: intermodulation characteristics

- Proposals

  - Option 1: This requirement is not needed considering the NTN UE operating scenario.

  - Option 2: The requirements for Intermodulation characteristics in clause 7.8 from TS 38.101-1 are applicable for satellite UE.

  - Option 3: Other, please specify.

- Recommended WF

  - TBA

Companies views’ collection for 2nd round

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |
| Ericsson | Issue 2-5-1: Option 3. Option1 is not realistic, they might always be NTN UEs in the vicinity of TN, no mechanism prevents this. Issue 2-5-2: option 2 as starting point, might need further discussion on possible scenarios. Issue 2-5-3: option 1 is ok Issue 2-5-4: option 2 as starting point, might need further discussion on possible scenarios. |
| THALES | Issue 2-5-1: Option 3 is not clear which band, which UE, which configuration. We should discuss more in detail the exact scenario if UE to UE and which exact scenario. Are you referring to Scenario/Case 5 from the coexistence simulations? If this is the case, the impact of NTN UE is minimal to TN UE. As a matter of fact, under used simulation assumptions there is a low probability that UEs close enough to each other cause significant ACI. If UEs are close-by to each other they likely connect to the same cell. However, if the UEs are forced to different networks (NTN and TN) the NTN UE could pose a threat to the neighbour channel TN UE. We therefore still prefer Option 1, since our coexistence simulations are showing that NTN is not affecting TN performance. The TN protection might be sufficient. In any case, why not directly using TN UE requirements for NTN UE? The maximum transmission power of a handheld NTN UE is the same as for a TN UE, and therefore same requirements would apply.. Issue 2-5-2: Option 2 or Option 1. Option 2 is possible since the (NTN) UE could be actually a TN UE operating in NTN. Issue 2-5-3: Option 1 (at least as basic assumption), but we also agree with Qualcomm that we can use REFSENS from n65 for MSS S-band n256. Issue 2-5-4: Option 2 or Option 1. Option 2 is possible since the (NTN) UE could be actually a TN UE operating in NTN. |
| Qualcomm | Issue 2-5-1: Need to identify the protected TN bands first. Suggest to further discuss per the input from companies. Issue 2-5-2: Option 2 as starting point. Issue 2-5-3: Option 1 is ok. For the value, we prefer to reuse the REFSENS from TN bands n24 and n65 Issue 2-5-4: Option 2 as starting point. |
| MediaTek | Issue 2-5-1: Agree that Option 1 does not seem reasonable. But also agree we should focus on identifying the relevant bands first. Issue 2-5-2: Option 2 as a starting point. Issue 2-5-3: Option 1 as a starting point. But further assessment needed on the suitable REFSENS value. Issue 2-5-4: Option 2 as a starting point. |
| CATT | Issue 2-5-1: Option 3. Issue 2-5-2: Open for further discussion. Issue 2-5-3: option 1 is ok Issue 2-5-4: Open for further discussion. |
| Nokia | Issue 2-5-1: Option 3. We are fine to focus on identifying the protected TN bands first. However, protection of these is a perquisite for NTN operation in general. Issue 2-5-2: Option 2. Issue 2-5-3: Option 1. Issue 2-5-4: Option 2. |
| Huawei | Issue 2-5-1: Share the similar with QC. The protected bands should be identified if we need to specify this requirements. Issue 2-5-2: Option 2. Issue 2-5-3: Option 1. Issue 2-5-4: Option 2. |

Summary of 2nd round

|  | Status summary |
| --- | --- |
| Sub-topic #2-5 | Issue 2-5-1: spurious emission for UE co-existence - Proposals - Option 1: confirm the NTN operating scenarios at first. - If it’s assumed that the NR NTN is used in the area that there is no IMT BS coverage, there is no need to specify the spurious emissions for UE co-existence. - Option 3: this requirement is needed for co-existence with UE in other bands - Recommended WF - TBA Majority companies supporting option 3 except Huawei and QC who wants to clarify the co-existence band. It is proposed to further clarify this in the next RAN4 meeting. Issue 2-5-2: Tx intermodulation - Proposals - Option 1: The requirement is not needed for NTN UE. - Option 2: The requirement in 38.101-1 can be reused for NTN FR1. - Option 2: other, please specify. - Recommended WF - TBA Majority companies support Option 2. It is proposed to use option 2 as starting point. Issue 2-5-3: Reference sensitivity - Proposals - Option 1: it is proposed to agree the following general formula for NTN UE REFSENS REFSENS, Sensitivity = -174dBm (kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain FFS on NF, SNR, IM and diversity gain, and REFSENS and uplink configurations - Option 2: Other, please specify - Recommended WF - TBA It is proposed to use option 1. Issue 2-5-4: intermodulation characteristics - Proposals - Option 1: This requirement is not needed considering the NTN UE operating scenario. - Option 2: The requirements for Intermodulation characteristics in clause 7.8 from TS 38.101-1 are applicable for satellite UE. - Option 3: Other, please specify. - Recommended WF - TBA Option 2 to be used as the starting point. |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| R4-211xxxx | WF on Tx RF requirements for satellite access node | CATT |
| R4-211xxxx | WF on Rx RF requirements for satellite access node | Ericsson |
| R4-211xxxx | WF on NTN UE RF requirements | Huawei |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2120674 | WF on Tx RF requirements for satellite access node | CATT | Agreeable |  |
| R4-2120675 | WF on Rx RF requirements for satellite access node | Ericsson | Agreeable |  |
| R4-2120676 | WF on NTN UE RF requirements | Huawei | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Qualcomm | Bin Han | binhan@qti.qualcomm.com |
| THALES | Dorin Panaitopol |  |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)