3GPP TSG-RAN WG4 Meeting#102-e	                             R4- 2205650

# Electronic Meeting, February 21st – March 3rd, 2022

Title:	LS on UL gap in FR2 RF enhancement

Release:	Rel-17

Work Item:	NR_RF_FR2_req_enh2

Source:	RAN WG4

To:	RAN WG2, RAN WG1

Cc:

Contact Person:

#### Name:		Lars Dalsgaard

#### E-mail Address:	 lars.dalsgaard@nokia.com

1. Overall Description:

In RAN4#101bis-e, RAN4 informed RAN2 about the status of the UL gaps. In RAN4#102-e meeting RAN4 continued the discussion and would like to update RAN2 on UL gaps for Tx power management (P-MPR).

RAN4 has agreed that of the agreed UL gap patterns informed in R4-2202420 gap patter UL_MGP#0 is mandatory while other UL gaps remain optional.

On UL gap configuration:

The UL gap configuration is updated for UL MGP #3 based on SCS.

|  | UGL [ms] | UGRP [ms] | mandatory/optional |
| --- | --- | --- | --- |
| UL MGP #0 | 1.0 | 20 | mandatory |
| UL MGP #1 | 1.0 | 40 | optional |
| UL MGP #2 | 0.5 | 160 | optional |
| UL MGP #3 | 0.125 when SCS of active UL BWP =120kHz 0.25 when SCS of active UL BWP =60kHz | 5 | optional |

RAN4 would further like to inform concerning the prioritization of UL gaps. RAN4 has agreed that RACH procedure and the related signaling shall not be impacted by any allocated UL gaps. Also, PUCCH allocations for SR and LRR (link recovery request) or network triggered PUSCH and PUCCH transmissions shall be prioritized over UL gaps

RAN4 will inform RAN2 of any further updates in future meetings if needed.

2. To RAN WG1 and WG2 group.

ACTION: 	RAN4 respectfully asks RAN1 and RAN2 to take RAN4 conclusions in consideration for further work.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #103-e	May 16 - 27, 2022	Online

TSG-RAN4 Meeting #104		August 22 - 26, 2022	Toulouse, France