3GPP TSG-RAN WG4 Meeting #102-e              	R4-2203547

Electrical Meeting, 21th Feb-3rd March, 2022

Agenda item:	10.9.4.3.3

Source:	Samsung

Title:	Simulation results of PRACH requirement for Rel-17 FR2 HST

Document for:	Discussion

# 1	Introduction

During the last RAN4 meeting, the remaining issue on the test parameters on PRACH requirement for Rel-17 FR2 HST were further discussed and finalized.

The related agreement was captured in the WF [1] as

- Frequency offset - 19444Hz with 350km/h at 30GHz carrier frequency - Test Preamble Configuration for Ncs - Ncs = 0 - Time offset configuration - Configure the maximum timing offset (i.e., the end of the tested range) in HST FR2 testing setup equal to 4.8us - Value of Timing offset start: 0 - Value of Timing offset step: 0.48us - Note Scenario; Cell radius (m); Maximum timing offset (us) (RTT); Coverage of format C2; Maximum timing offset for C2 (us) (RTT). Scenario A (Ds =700m, Dmin =10m); 700; 4.6us; 1.15km; 7.6. Scenario B (Ds=700m, Dmin=150m); 716; 4.78us; 1.15km; 7.6.

In this contribution, we just re-submit our ideal and impairment simulation results to finalize the PRACH requirement

# 2	Simulation Results

In this contribution, both ideal and impairment results were provided for alignment. The simulation assumption can be referred as the following table

Table 1: Simulation assumption for PRACH

| Parameters | Value |
| --- | --- |
| Ncs | 0 |
| Logical sequence index | 0 |
| v | 0 |
| Channel | AWGN |
| Antenna configuration | 1T2R |
| SCS | 120KHz |
| Frequency offset | 19444Hz |
| Timing offset | 0:0.48:4.8 |

Table 1: Initial simulation results for PRACH with format C2

| Number of Tx antennas | Number of Rx antennas | Propagation conditions and correlation | Frequency offset | Timing offset range | SNR [dB] Ideal | SNR [dB] Impairment |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | AWGN | 19444Hz | 0: 0.48:4.8us | -12.55 | -10.55 |

# 4	Conclusion

In this contribution, the ideal and impairment simulation results are provided for PRACH requirement finalization.

# References

R4-2203006, WF on BS demodulation requirement for FR2 HST, Nokia, Samsung