TSG-RAN Working Group 4 (Radio) meeting #102-E	R4-2203578

Electronic Meeting, 21th February – 3rd March 2022

Source:	Ericsson

Title:	On BS RF receiver requirements for the frequency range 52 to 71 GHz

Agenda item:	10.16.4.2

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#101-bis-E) specific details related to BS RF core receiver requirements related to clause 10 in TS 38.104 applicable for the frequency range 52600 to 71000 MHz was discussed. The meeting outcome and guidance for this meeting was captured in a Way Forward (WF) contribution [1].

In this contribution we continue the discussion from [2] on BS RF receiver requirements relevant for the NR extension to support up to 71 GHz. The contribution presents an overview of base station receiver requirement changes compared to current version of TS 38.104 and some proposals to further progress the work.

# Discussion

Based on agreements in the WF from last meeting [1] and earlier agreements the impact on TS 38.104 clause 10 [3] is summarized in Table 2-1.

Table 2-1: Requirement overview

| Requirement | TS 38.104 subclause | Status |
| --- | --- | --- |
| OTA sensitivity | 10.2 | No impact since this requirement is not applicable for BS type 2-O. |
| OTA reference sensitivity | 10.3 | Requirement for FR2-2 is defined. FRC can be defined when SU is settled. |
| OTA dynamic range | 10.4 | No impact since this requirement is not applicable for BS type 2-O. |
| OTA in-band selectivity and blocking | 10.5 | Requirement for FR2-2 is defined. FFS depends on details related to SU. |
| OTA out-of-band blocking | 10.6 | Requirement for FR2-2 is defined. FFS on upper bound to set DfOOB. |
| OTA receiver spurious emissions | 10.7 | Requirement is defined for FR2-2. |
| OTA receiver intermodulation | 10.8 | Requirement is defined for FR2-2. |
| OTA in-channel selectivity | 10.9 | Requirement is defined for FR2-2. |

In the following sub-sections additional technical information is presented together with proposals moving the discussion towards a complete set of requirements required to extend NR up to 71 GHz.

## 2.1	Adjacent Channel Selectivity

Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

Last meeting RAN4 agreed to set the ACS requirement for FR2-2 according to Table 2.1-1

Table 2.1-1: ACS requirement for BS type 2-O

| Frequency range | BS channel bandwidth (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- | --- |
| FR2-1 (24.25 – 33.4 GHz) | 50, 100, 200, 400 | EISREFSENS + 6 dB | EISREFSENS_50M + 27.7 + ΔFR2_REFSENS |
| FR2-1 (37 – 52.6 GHz) | 50, 100, 200, 400 | EISREFSENS + 6 dB | EISREFSENS_50M + 26.7 + ΔFR2_REFSENS |
| FR2-2 (52.6 – 71 GHz) | 100, 400, 800, 1600, 2000 | EISREFSENS + 6 dB | EISREFSENS_50M + 25.7 + ΔFR2_REFSENS |

The interferer signal type used for the frequency range 52.6 to 71 GHz needs to be selected based on minimum supported sub-carrier spacing and carrier bandwidth supported within the frequency range 52.6 to 71 GHz. Hence, new table rows are required in the specification to describe the differences between 24.25 to 52.6 GHz and 52.6 to 71 GHz.

In Table 2.1-2, the interfering signal offset and type is documented.

Table 2.1-2: ACS interferer frequency offset for BS type 2-O

| Frequency range | BS channel bandwidth (MHz) | Interfering signal centre frequency offset (MHz) | Type of interfering signal |
| --- | --- | --- | --- |
| FR2-1 | 50 | ±24.29 |  |
|  | 100 | ±24.31 | 50 MHz DFT-s-OFDM NR |
|  | 200 | ±24.29 | signal, 60 kHz SCS, 64 RBs |
|  | 400 | ±24.31 |  |
| FR2-2 | 100 | ±48.58 | 100 MHz DFT-s-OFDM NR signal,120 kHz SCS, FFS RBs |
|  | 400 | ±48.62 |  |
|  | 800 | ±48.58 |  |
|  | 1600 | ±48.62 |  |
|  | 2000 | ±48.58 |  |

Proposal 1: For ACS define interferer signal offset as defined in Table 2.1-2.

## 2.2	In-band blocking

At last meeting RAN4 agreed to define the requirement for FR2-2 as described in Table 2.2-1.

Table 2.2-1: General blocking requirement

| Frequency Range | BS channel bandwidth of the lowest/highest carrier received (MHz) | OTA wanted signal mean power (dBm) | OTA interfering signal mean power (dBm) | OTA interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of OTA interfering signal |
| --- | --- | --- | --- | --- | --- |
| FR2-1 | 50, 100, 200, 400 | EISREFSENS + 6 dB | EISREFSENS_50M + 33 + ΔFR2_REFSENS | ±75 | 50 MHz DFT-s-OFDM NR signal, 60 kHz SCS, 64 RBs |
| FR2-2 | 100, 400, 800, 1600, 2000 | EISREFSENS + 6 dB | EISREFSENS_50M + 33 + ΔFR2_REFSENS | ±150 | 100 MHz DFT-s-OFDM NR signal, 120 kHz SCS, FFS RBs |

The details related to the interfering signal type is still FFS.

The size of the exclusion zone, DfOOB was defined as described in Table 2.2-2.

Table 2.2-2: ΔfOOB offset for NR operating bands in FR2

| BS type | Frequency Range | Operating band characteristics (MHz) | ΔfOOB (MHz) |
| --- | --- | --- | --- |
| BS type 2-O | FR2-1 | FUL_high – FUL_low ≤ 4000 | 1500 |
|  | FR2-2 | 4000 MHz < FUL_high – FUL_low ≤ FFS | 3500 |

, where the upper boundary of the operating band characteristics is FFS. It would be reasonable to set the upper boundary for DfOOB to the same value as for DfOBUE.

Proposal 2: For in-band blocking set the upper boundary for the operating band size equal for DfOOB and DfOBUE.

## 2.3	In-channel selectivity

In-channel selectivity (ICS) is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of an interfering signal received at a larger power spectral density. In this condition a throughput requirement shall be met for a specified reference measurement channel.

Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal.

At last, RAN4 meeting it was agreed to use 10 dB shift between wanted signal and interfering signal as before for FR2-1.

For FR2-2 the wanted signal and interfering signal needs to be scaled to 100 MHz and 400 MHz, according to Table 2.3-1.

Table 2.3-1: Relation between wanted signal power and interfering signal power

| BS channel bandwidth (MHz) | Wanted signal power (dBm) | Interfering signal power (dBm) |
| --- | --- | --- |
| 50 | EISREFSENS_50M + ΔFR2_REFSENS | EISREFSENS_50M + 10 + ΔFR2_REFSENS |
| 100 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS |
| 400 | EISREFSENS_50M + 9 + ΔFR2_REFSENS | EISREFSENS_50M + 19 + ΔFR2_REFSENS |

The interfering signal type is defined as an DFT-s-OFDM signal configured with minimum supported SCS. For FR2-2, the minimum supported SCS is 120 kHz.

In Table 2.3-2, the parameters related to FR2-2 is summarized.

Table 2.3-2: OTA in-channel selectivity requirement for FR2-2

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 100, 400 | 120 | G-FR2-A1-2 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 32 RB |
| 400, 800, 1600 | 480 | G-FR2-A1-6 | EISREFSENS_50M + 9 + ΔFR2_REFSENS | EISREFSENS_50M + 19 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 480 kHz SCS, FFS RB |
| 400, 800, 1600, 2000 | 960 | G-FR2-A1-7 | EISREFSENS_50M + 9 + ΔFR2_REFSENS | EISREFSENS_50M + 19 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 960 kHz SCS, FFS RB |

Based on the discussion following proposals are presented:

Proposal 3: For in-channel selectivity define wanted signal power level and interfering signal power level as described in Table 2.3-1.

## 2.4	Receiver spurious emissions

For the frequency range 30 MHz to 18 GHz the measurement bandwidth used for the requirement is defined in ITU-R SM.329 [7]. Close to the band limits and measurement bandwidth follows ERC recommendations 74-01 [8]. The frequency ranges close to the band is specified per band in Table 2.4-1, with additions of band n263 for unlicensed operation and n264 for licensed operation.

Table 2.4-1: Step frequencies for defining the spurious emission limits

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) | Fstep,4 (GHz) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 18 | 23.5 | 25 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n259 | 23.5 | 35.5 | 38 | 45 | 47.5 | 59.5 |
| n260 | 25 | 34 | 35.5 | 41.5 | 43 | 52 |
| n261 | 18 | 25.5 | 26.0 | 29.85 | 30.35 | 38.35 |
| n262 | 37.2 | 45.2 | 45.7 | 49.7 | 50.2 | 58.2 |
| n263 | 18 | 43 | 53.5 | 74.5 | 84 | 127 |
| n264 | 46 | 61 | 62.5 | 74.5 | 76 | 91 |

In addition to the requirements in Table 2.4-1, the requirement for protection of EESS for base station operating in frequency range 24.25 - 27.5 GHz defined for transmitter spurious emissions may be applied.

Proposal 4: For receiver spurious emission add row with values in Table 2.4-1 for band n264.

# Conclusion

In this contribution the impact on receiver requirements in TS 38.104 have been studied.

When the frequency range is extended up to 71 GHz, the supported carrier configuration will change which would result in some differences with respect to current FR2 requirements. In this contribution we show how requirements applicable for the frequency range 52.6 to 71 GHz can be integrated into TS 38.104.

Based on the technical analysis provided in this contribution following proposals are presented for approval:

Proposal 1: For ACS define interferer signal offset as defined in Table 2.1-2.

Proposal 2: For in-band blocking set the upper boundary for the operating band size equal for DfOOB and DfOBUE.

Proposal 3: For in-channel selectivity define wanted signal power level and interfering signal power level as described in Table 2.3-1.

Proposal 4: For receiver spurious emission add row with values in Table 2.4-1 for band n264.

# References

[1]	R4-2203017, “WF on BS RF Rx requirements”, Ericsson

[2]	R4-2200844, “On BS RF receiver requirements for the frequency range 52 to 71 GHz”, Ericsson

[3] 	TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP

[4]	TR 38.803, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”, 3GPP

[5]	TR 38.817-02, “General aspects for Base Station (BS) Radio Frequency (RF) for NR”, 3GPP

[6]	R4-2200843, “On BS RF transmitter requirements for the frequency range 52 to 71 GHz”, Ericsson

[7]	Recommendation ITU-T SM.329, “Unwanted emissions in the spurious domain”, ITU-R

[8]	ERC Recommendation 74-01, “Unwanted emissions in the spurious domain”, CEPT