3GPP TSG-RAN Working Group 4 (Radio) Meeting #102-E	R4-2203583

# Electronic Meeting, 21th February – 3rd March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.921 | CR | 0002 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.921: Update of information about interference management in subclause 6.1.4, 6.1.5, 8.1.2 and 9.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_IMT_param |  |  |  |  |  | Date: |  |  | 2022-02-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TR 38.921 spurious emission Category A limits for 10 to 10.5 GHz is missing. In Table 6.1.5-1, BS TRP values is listed. It seems that TRP levels are not aligned with parameters in Table 8.1.2-1 and 4.2.8. In TR 38.921, subclause 9.2 technical background information related to interference management is captured. The information is intended to described mitigation techniques to suppress radiation in unintended directions. TR 38.921 is an external TR which will be used as input for studies in other forumns, such as ITU-R and CEPT. In subclause 9.2 some general techniques on how to mitigate interference casued by AAS base stations is presented. This CR will add some new essential information currently missing in the TR and correct some editorial errors. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of Category A spurious emission limis in subclause 6.1.4. - Correction of values in Table 6.1.5-1 to reflect TRP - Addition of equations to relate basic parameters Ptx TX branch, TRP and EIRP in subclause 8.1.2. - Editoral corrections - Addition of general background related to side-lobe response to element separation. - Addition of relation between directivity and correlation relevant for unwanted emissions. - Addition of subclause for description of beam restrictions. - Addition of subclause for description of irregular array geometreis. - Addition of subclause for power back-off. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, subclause 9.2 will not present some of the most essential techniques relevant for AAS BS to suppress radiation in unintended directions and the TR technial information is not consistent with respect to BS power capabilities.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Subclause 6.1.4, 6.1.5, 8.1.2 and 9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CR to subclause 6.1:

## 6.1	Transmitter characteristics

### 6.1.1	Power dynamic range

There is no power control in downlink and fixed power per resource block is assumed in the co-existence simulation. Hence 0 dB power dynamic range was agreed for the LS reply.

### 6.1.2	Spectral mask

Existing FR1 operating band unwanted emission mask is the same as for LTE (5 MHz ~ 20 MHz CBW). For both 6.425 - 7.125 GHz and 10.0 - 10.5 GHz band, it is foreseen the smaller channel bandwidth such as less than 50 MHz CBW is less attractive. Hence the basic limits for OOB emission mask should be updated as below. It is agreed that it does not mean that 50 MHz will be the minimum channel bandwidth when RAN4 specified the channel bandwidth and emission requirements.

Table 6.1.2-1: Wide Area BS operating band unwanted emission limits for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -7dBm-750foffsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -13 dBm | 1MHz |

Table 6.1.2-2: Wide Area BS operating band unwanted emission limits for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -7dBm-750foffsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -15 dBm | 1MHz |

### 6.1.3	ACLR

According to the simulation results in clause 4.3, it is agreed to specify 38 dB ACLR for 6.425 - 7.125 GHz and 37 dB ACLR for 10.0 - 10.5 GHz.

For 6425 – 7125 MHz and 10 - 10.5 GHz, the ACLR should be higher than the value specified in table 6.1.3-1.

Table 6.1.3-1: Base station ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 20, 25, 30, 40, 50, 60, 70, 80, 90,100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 38 dB for 6425-7125 MHz 37 dB for 10-10.5 GHz |
|  | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 38 dB for 6425-7125 MHz 37 dB for 10-10.5 GHz |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |

### 6.1.4	Spurious emissions

From the Rel-15 discussion, the offset ΔfOBUE for the boundary between OBUE and spurious emissions is the most demanding parameters for OTA or Hybrid limits AAS BS. Based on the implementation evaluation and regulatory consideration, it is agreed to adopt ΔfOBUE = 100 MHz for both 6.425 - 7.125 GHz and 10.0 - 10.5 GHz.

Regarding spurious emission requirements for 6.425 - 7.125 GHz, as this frequency range is still within FR1 band definition, it is agreed to reuse the existing spurious emission requirements defined in TS 38.104 [3] and further update needed are to define upper frequency of Tx spurious emission to be 26 GHz instead of 5th harmonic of DL frequency according to ITU-R SM 329-10 [5] recommendation. The general spurious emissions for 6.425-7.125 GHz is defined in Table 6.1.4-1 and Table 6.1.4-3.

Regarding spurious emission requirements for 10-10.5 GHz, according to ERC 74-01 report [6], the lower frequency limit could be defined as 30 MHz. Without any specified spurious limit for AAS BS in between 6 and 24 GHz in ERC 74-01 [6], in relation to the existing frequency ranges specified, with observations of the foreseen implementation of AAS BS and how they are to be deployed and used, the spurious limits specified for above 24 GHz with upper frequency limited to 26 GHz will be used for 10.0-10.5 GHz as defined in Table 6.1.4-2 and Table 6.1.4-4.

Table 6.1.4-1: BS spurious emission limits for 6.425 - 7.125 GHz for Category B

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz – 150 kHz | -36 dBm | 1 kHz |
| 150 kHz – 30 MHz |  | 10 kHz |
| 30 MHz – 1 GHz |  | 100 kHz |
| 1 GHz – 26 GHz | -30 dBm | 1 MHz |

Table 6.1.4-2: BS spurious emission limits for 10 - 10.5 GHz for Category B

| Spurious frequency range | Limit | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz – 1 GHz | -36 dBm | 100 kHz |
| 1 GHz – 18 GHz | -30 dBm | 1 MHz |
| 18 GHz – 26 GHz | -20 dBm | 10 MHz |

Table 6.1.4-3: BS spurious emission limits for 6.425 - 7.125 GHz for Category A

| Spurious frequency range | Basic limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz – 150 kHz |  | 1 kHz | Note 1 |
| 150 kHz – 30 MHz |  | 10 kHz | Note 1 |
| 30 MHz – 1 GHz |  | 100 kHz | Note 1 |
| 1 GHz 12.75 GHz | -13 dBm | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz |  | 1 MHz | Note 1, Note 2, Note 3 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [5], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [5], s2.5 table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75 GHz. |  |  |  |

Table 6.1.4-4: BS spurious emission limits for 10 - 10.5 GHz for Category A

| Frequency range | Limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -13 dBm | 100 kHz | Note 1 |
| 1 GHz – 2nd harmonic of the upper frequency edge of the DL operating band |  | 1 MHz | Note 1, Note 2 |
| NOTE 1: Bandwidth as in ITU-R SM.329 [5], s4.1 NOTE 2: Upper frequency as in ITU-R SM.329 [5], s2.5 table 1. |  |  |  |

### 6.1.5	Maximum output power

The maximum output power will be provided in the antenna parameter table. It was agreed to be aligned with antenna characteristics.

The Total Radiated Power for two polarizations was agreed as shown in table 6.1.5-1 below.

Table 6.1.5-1: The Total Radiated Power

| Parameter | Macro Sub-urban | Macro Urban | Micro Urban |
| --- | --- | --- | --- |
| Total Radiated Power for two polarizations (dBm) | 44 | 44 | 35 |

### 6.1.6	Average output power

It was agreed the average output power won’t be mentioned in the reply LS.

CR to subclause 8.1.2:

### 8.1.2	Array antenna parameters

In Table 8.1.2-1, base station array antenna parameters for different deployment scenarios is listed. Element parameters have been selected to produce correct element peak gain determined by calculating the directivity from a given geometry including beam widths. The element directivity can be calculated based on the pattern described by Table 8.1.2-1 in dBi as:

GE,max=DE,max-LE	(8.1.2-1)

, where the peak directivity DE,max is calculated from given values on 3dB, 3dB, dh and dv as:

DE,max=10log104πAlinθ,φmax-ππ0πAlinθ,φsinθdθdφ	(8.1.2-2)

, where Alin(,) is defined in linear scale as:

Alinθ,φ=10Aθ,φ10	(8.1.2-3)

Table 8.1.2-1: BS array antenna parameters

| Parameter | Macro Sub-urban | Macro Urban | Micro Urban | Small cell indoor |
| --- | --- | --- | --- | --- |
| Am (dB) | 30 | 30 | 30 | 30 |
| SLAv (dB) | 30 | 30 | 30 | 30 |
| 3dB (deg.) | 90 | 90 | 90 | 90 |
| 3dB (deg.) | 65 | 90 | 90 | 90 |
| GE,max (dBi) | 6.4 | 5.5 | 5.5 | 5.5 |
| LE (dB) | 2.0 | 2.0 | 2.0 | 2.0 |
| (M, N) | (16, 8) | (16, 8) | (8,8) | (4, 4) |
| dh (m) | 0.5 | 0.5 | 0.5 | 0.5 |
| dv (m) | 0.7 | 0.5 | 0.5 | 0.5 |
| Horizontal coverage range (deg.) | +/- 60 | +/- 60 | +/- 60 | N/A |
| Vertical coverage range (deg.) | 90 to 100 | 90 to 120 | 90 to 120 | N/A |
| Conducted power (before ohmic loss) per antenna element, Ptx (dBm) | 22 (Note 4) | 22 (Note 4) | 16 (Note 5) | 9 (Note 6) |
| Mechanical downtilt (deg.) | 6 | 10 | N/A | N/A (Note 7) |
| Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max Note 3: The vertical coverage range includes the mechanical downtilt. Note 4: The conducted power per element assumes 16x8x2 elements (i.e. power per H/V polarized element). Note 5: The conducted power per element assumes 8x8x2 elements (i.e. power per H/V polarized element). Note 6: The conducted power per element assumes 4x4x2 elements (i.e. power per H/V polarized element). Note 7: Boresight direction is perpendicular to the ceiling. |  |  |  |  |

Based on the array parameters sets fundamental characteristics such as TRP and peak EIRP can be derived. For BS output power considerations, the total radiated power (TRP) is calculated in logarithmical scale as:

TRP=10log10PMN+Ptx-LE	(Eq. 8.1.2-4)

, where P, M, N, Ptx and LE is given by Table 8.1.2-1.

For radiated power considerations peak Equivalent Isotropic Radiated Power (EIRP) is calculated in logarithmical scale calculated as:

EIRP=Ptx+GE+20log10MN+10log10P 		(Eq. 8.1.2-5)

, where P, M, N, Ptx, GE and LE is given by Table 8.1.2-1.

CR to subclause 9.2:

## 9.2	Interference management

### 9.2.0	General

Given an antenna array with M multiplied by N identical elements, the radiation pattern of the array antenna can be described according to the pattern multiplication theorem as:

Rθ,φ=REθ,φRAθ,φ 	(Eq. 9.2.0-1)

, where RE is the radiation pattern for the array elements and RA is the radiation pattern associated to the array factor. The element patten, RE(, is based on a parameterized Gaussian shaped element (as described in Table 8.1.1-2) or an sub-array constituted of multiple Gaussian shaped elements, with floors to model for side-lobe levels and front-to-back ratio. The element peak gain is directivity normalized, hence the peak element gain GE,max, element loss LE and half power beam widths 3dB and 3dB should be selected carefully to maintain correct antenna gain, as described in subclause 8.1.2.

It can be noticed that both the element factor and the array factor can be used to shape the composite radiation pattern. The element pattern can be used to suppress side-lobe characteristics. For a limited steering range, a sub-array element can be used to suppress side-lobes better than a single element configuration. Typically for an AAS base station implementation the element radiation pattern and the array factor are customized to optimize the coverage within a specific coverage range for a given deployment scenario.

For a general array antenna, the array factor radiation pattern for transmitting array antenna with MN element per polarization can be expressed as:

RAθ,φ=n=1MNwnejkr2 	(Eq. 9.2.0-2)

, where

-	wn is the complex array excitation,

-	k is the wave vector of the transmitted wave, and

-	r is the element location matrix.

From Eq. 9.2.0-1 it can be noticed that both the element factor and the array factor can be used to shape the composite radiation pattern, which will be further described later. The element pattern can be used to suppress side-lobe characteristics. For a limited steering range, a sub-array element configuration can be used to suppress side-lobes better than a single element configuration. Typically for an AAS base station implementation the element radiation pattern and the array factor are customized to optimize the coverage within a specific coverage range and coexistence with other services for a given deployment scenario.

For an array antenna with element separation 0.5l or less the first side-lobe is the strongest. For uniform amplitude excitation the first side lobe power level is approximately 13 dB below the main beam power level. For array antenna with element separation larger than 0.5l folding effects will create grating lobes in the side lobe region. A common scenario with element separation larger than 0.5l is when sub-array is used in the vertical domain. The angular location of the grating lobe is determined by the element separation and steering angle and the power level of the grating lobes is determined by the element factor. As an example, in Figure 9.2.0-1, the impact of element separation is visualised for an 8x8 element Uniform Rectangular Array (URA) where the vertical beam steering directions is set to 130 degrees.

Figure 9.2.0-1: Impact of element separation on vertical radiation pattern

The array factor described by Eq. 9.2.0-2 will produce maximum directivity for the wanted carrier for which the system is calibrated minimizing the excitation error. For unwanted emission outside the carrier the directivity will roll-off gradually to a point where the array factor directivity is lost.

The relation between array excitation correlation and directivity can be modelled as described in TR 37.840, subclause 5.4.4.1.4. The average radiation pattern for different correlation values is plotted in Figure 9.2.0-2. The maximum directivity is achieved for correlation equal to 1 and the minimum directivity equal to the element directivity is achieved for correlation equal to 0.

Figure 9.2.0-2: Relation between excitation correlation and directivity

To be able to conduct accurate coexistence analysis the unwanted emission directivity roll-off characteristics is required. The decorrelation effect in an array antenna will together with traditional filters provide suppression of unwanted emission EIRP levels required to guarantee coexistence with other adjacent services.

There is no single general solution available to mitigate all possible interference situations. Depending on situation one or more mitigation techniques described in following section can be used to guarantee coexistence between AAS base stations and other services operating in adjacent spectrum.

There is no signle general solution solving interference issues for all situations. Depending on situation mitigation solutions described in following sections can be used to protect adjacent niegboring other services.

### 9.2.1	Beam nulling

Beam nulling technology is used to suppress the unwanted spatial emission by inserting nulls in the radiation pattern for the direction of the interference. One typical applicable scenario is multiple beams transmission. As shown in Figure 2 as one example, Beam 1 is the serving beam of UE 1 but the side lobe of beam 1 would interfere the UE2. The SINR for UE2 will be affected. In order to support high order modulation scheme such as DL 256 QAM, using beam nulling technology, a null can be placed at the direction to UE2 for beam 1. The weight of each antenna array of beam 1 can be obtained by specific algorithms so that SINR of beam1 is maximized in the direction to UE1 and the transmission power in the main lobe is maintained, and the side lobe are suppressed in the direction to UE2. Beam nulling can also be used in the inter-cell or inter system scenarios as long as the location or direction of the protected station is known.

Figure 9.2.1-1: Side lobe interference

### 9.2.2	Amplitude weighting/tapering

AAS offer a wide range of opportunities on optimizing the directivity patterns through amplitude and phase control. High directivity antenna array also has side lobes which are often undesirable since they may cause intra-cell or inter-cell interference. Side lobe levels can be reduced via tailoring the amplitude across the antenna array which is often referred as amplitude weighting technology. Whilst amplitude weighting/tapering reduces the side lobes it also makes the main lobe wider and hence reduces gain. It needs tradeoff between antenna gain and side lobe suppression. For example:

Figure 9.2.2-1: Examples for amplitude weighting/tapering: left: Kaiser (a=3), right: Chebwin (R=35dB)

Applying tapering electrically for the complete array will results in a main beam peak EIRP drop due to the amplitude window itself. In addition, the directivity will also reduce as an effect of a wider beam, as seen in Figure 9.2.2-1.

If tapering is applied at sub-array level the power can be be redistributed between elements in the sub-array, hence no power loss due to tapering will be introduced.

### 9.2.3	Asymmetric side lobe shaping

In addition to beam tapering, it is possible to manipulate the amplitude and the phase of the window (for example using modified Taylor series) to modify the side lobe levels asymmetrically. This technique is widely used with passive BS arrays to make the ground side lobes larger to fill in the angles between the main beam and the antenna. The same technique can be used to minimise radiation in specified unwanted directions. For example:

Figure 9.2.3-1: Example for modified Taylor window

### 9.2.4	Beam restrictions

Depending on base station implementation and intended deployment scenario different types of beamforming schemes can be considered. For AAS base stations operating within FR1 (410 to 7125 MHz) typically codebook or reciprocity-based beamforming will be used for transmitted down link beams, while for AAS base stations operating within FR2 (24250 to 52600 MHz) typically grid of beams beamforming concepts will be used for transmitted downlink beams. By restricting the angular range for which the beams can be transmitted, interference power in unintended directions can be limited. For codebook-based beamforming codes in the codebook associated to large beam steering angles can be disabled and prohibited from being used for transmission. The same principle can be adopted for grid of beams systems. For reciprocity-based beam forming, modification to the applied excitations needs to be carried out to avoid steering directions associated to generation of unintended interference in the side lobe region.

For systems using sparse arrays in the vertical domain by having using sub-arrays it is essential to control the grating lobe response. By limiting the vertical beam steering range within a certain interval, it is possible to control the level of the grating lobe to a level similar to other sidelobes. Not using specific beams with large down tilts angles does not affect the network performance since, other beams can be used by proving sufficient power using sidelobes close to the base station. The vertical range in which the side lobe level is guaranteed will depend on the array antenna geometry used be a specific base station. As an example, in Figure 9.2.4-1 the results of beam restrictions are visualised. In the left figure all beams are plotted, while in the right figure beams causing interference towards the sky have been disabled.

Figure 9.2.4-1: Controlling interference using beam restrictions

### 9.2.5	Irregular array geometries

Instead of using uniform array geometries where all the elements are assumed to be equal and the elements are located in a rectangular lattice with given vertical and horizontal element spacing, irregular array geometries can be used to suppress unintended radiation in the side lobe region. One example is when the grating lobe response is suppressed by shifting the columns with a vertical column offset. The offset can be selected in a way so that the grating lobe reduces. Instead of concentrating the power is one specific direction the grating lobe power is spread out in the sidelobe region. The penalty is that the array antenna aperture size slightly growth compared to the non-shifted case, as showed in Figure 9.2.5-1. Other examples on irregular array structures are where different element factor is used for different locations in the array.

Figure 9.2.5-1: Offset applied to columns

As an example, in Figure 9.2.5-2 the vertical radiation pattern for 0 degrees down tilt (reference direction) and 12 degrees down tilt (maximum down-tilt direction) is visualized for uniform structure and irregular structure. For the irregular structure elements columns have been shifted up and down by an offset vector equal to [0, -0.5, 0, -1, 0, 0.5, 0, 1] wavelengths.

Figure 9.2.5-2: Irregular array geometries

It can be noticed that by applying the offset vector the grating lobe is spread out spatially suppressing the interference in a specific direction.

### 9.2.6	Power back-off

To reduce the EIRP in a specific direction (e.g., towards the sky or in the horizontal plane to specific directions) the base station power can be backed off. As consequence the power in the main beam will reduce as well as the power towards a victim system. The penalty in terms of coverage will be quite significant.

To maintain coverage more advanced power back off schemes have been developed. Instead of reducing the power capability of the base station the average power is given specific directions can be controlled. Depending on the traffic situation the base station will select beams so that the unintended emission in specific directions can be controlled. This means that the base station scheduler algorithms have knowledge about the array antenna geometry and can calculate average power radiated in given angular regions. This approach be used to control average power levels both for wanted carrier power and unintended power radiating is the sidelobe region due to grating lobes (e.g., towards the sky).