3GPP TSG-RAN WG4 Meeting # 102-e	R4-2203585

Electronic Meeting, February 21 – March 3, 2022

Source:	ZTE Corporation

Title:	Timing requirements for RedCap UEs

Agenda item:	10.20.3.1.3

Document for:	Discussion

# 1	Introduction

In RAN4 101-e-bis, there were discussions on timing related issues for RedCap UEs.

In the WF [1] the options were captured, and we copy paste some of the issues here to further discuss.

Condition for meeting UE transmit timing requirements Redcap UE should meet the existing Te and Tq requirements provided that the SSB is available at the UE at least once every 160 ms is agreeable: - Option 1 (E///, HW, CMCC, NOkia ): SSB refers to CD-SSB or any of CS- and NCD-SSB - Option 2 (Apple, QC): SSB has to be in active BWP - Option 3 (MTK): SSB refers to both CD-SSB and NCD-SSB and SSB shall be in the active BWP. Whether CSI-RS/TRS can be used for meeting the timing requirements - Option 1 (Xiaomi, MTK, OPPO, Nokia, QC, vivo, Xiaomi, Apple, MTK, E///): CSI-RS/TRS is not needed to acquire the reference cell timing in Rel-17. - Option 2 (HW, CMCC): - CSI-RS can be used for RedCap UEs to acquire the reference cell timing depending on UE capability. - No CSI-RS based timing requirements are defined the CSI-RS in R17.

- This paper discusses the issues listed above and provide our views.

2	Discussions

Condition for meeting UE transmit timing requirements

With current discussions on NCD-SSB going on, we think that Option 1 is quite reasonable. What’s more, the RedCap UE can always re-tune to the BWP with a SSB to make timing related adjustments.

Similar requirements are also applicable for similar features like NR-U, where the availability of SSB can ensure the timing requirements of UE.

Redcap UE should meet the existing Te and Tq requirements provided that the SSB is available at the UE at least once every 160 ms, where SSB can be CD-SSB or NCD-SSB.

Whether CSI-RS/TRS can be used for meeting the timing requirements

Here what is being discussed is whether CSI-RS is feasible for the RedCap UEs to acquire timing. In our view, at least from RAN4 perspective, using CSI-RS or TRS to acquire timing is possible. Some companies may say that CSI-RS itself is an optional feature, though this is true for legacy UEs, it is still under discussion for RedCap UEs. Thus, from RAN4 perspective, we don’t see any issue to support CSI-RS or TRS for timing acquisition.

From RAN4 perspective, CSI-RS can be used for RedCap UEs to acquire the reference cell timing depending on UE capability.

3	Conclusion

- Proposal 1: Redcap UE should meet the existing Te and Tq requirements provided that the SSB is available at the UE at least once every 160 ms, where SSB can be CD-SSB or NCD-SSB.

Proposal 2: From RAN4 perspective, CSI-RS can be used for RedCap UEs to acquire the reference cell timing depending on UE capability.

References

- R4-2202670