3GPP TSG-RAN WG4 Meeting # 102-e	R4-2203586

Electronic Meeting, February 21 – March 3, 2022

Source:	ZTE Corporation

Title:	On Signalling characteristics of RedCap UEs

Agenda item:	10.20.3.1.4

Document for:	Discussion

# 1	Introduction

In RAN4 101-e-bis, there were discussions on signalling related issues for RedCap UEs.

In the WF [1] the options were captured, and we copy paste some of the issues here to further discuss.

New BWP switching delay when only center-frequency is changed in Rel-17 - Option 1 (CMCC, E///, HW, Nokia): Define new BWP switching delay involving only changing of the center-frequency of the BWP without changing its BW, SCS or any other parameter for RF retuning as follows: Frequency Range; Type 1 Delay (us); Type 2 Delay (us). 1; 200; 1050. 2; 200; 1050. - Option 2 (Xiaomi, vivo, Oppo, ZTE, MTK, QC, Apple): RAN4 to reuse the legacy BWP switching delay for RedCap UE in Rel-17.

- This paper discusses the issues listed above and provide our views.

2	Discussions

New BWP switching delay when only center-frequency is changed in Rel-17

Here the scenario is that since CD-SSBs may not be transmitted in the RedCap configured initial BWP, the UE may have to perform RF retuning in order to switch to different BWP (e.g. initial BWP) to receive those signals. And in this case, only the center-frequency is changed. We observe that BWP switching with only center frequency changed is also a possible case for legacy UEs, and no enhancement is made with that regard.

Observation 1: BWP switching with only center frequency changed is also a possible case for legacy UEs, and no enhancement is made with that regard.

Thus, we propose to re-use the legacy requirements and not to require RedCap UEs to meet stricter requirements than legacy UEs.

Reuse legacy BWP switching delay when only center frequency is changed.

3	Conclusion

Observation 1: BWP switching with only center frequency changed is also a possible case for legacy UEs, and no enhancement is made with that regard.

- Proposal 1: Reuse legacy BWP switching delay when only center frequency is changed.

References

- R4-2202670