3GPP TSG-RAN WG4 Meeting #102-e                                                    	R4-2203625

Online, 21st February – 3rd March, 2022

Source:	Vodafone

Title:	Discussion on UE RF requirements for DC_8-32_n78

Agenda item:			9.6.1

Document for:	Approval

# 1. Introduction

This contribution is a draft text proposal for TR 37.717-21-11 to include DC_8-32_n78. Since this combination has an IMD3 hit in B32 as highlighted in section 5.x.4, input is needed on a suitable MSD test point.

The proposed TIB,c and RIB values were derived from those present in 38.101-3 for DC_8_n78.

# 2. Reference

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	DC_8-32_n78

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-32A_n78A | DC_8A_n78A |

### 5.x.2	Co-existence studies

Table 5.x.2-1 lists the Band 8A + Band n78A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2-1: Band 8 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1760 – 1830 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2640 – 2745 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2385 – 2920 |  |  | 4180 – 4715 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1470 – 2040 |  |  | 5685 – 6720 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 5060 – 5630 |  |  | 7480 – 8515 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 780 – 1015 |  |  | 3290 – 3810 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 555 – 1160 |  |  | 8985 – 10520 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 4770 – 5840 |  |  | 8360 – 9430 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5940 – 6545 |  |  | 10780 – 12315 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12285 – 14320 |  |  | 280 – 360 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8070 – 9640 |  |  | 3855 – 4960 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14080 – 16115 |  |  | 6820 – 7460 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11660 – 13230 |  |  | 9240 – 10345 |  |  |

Based on Table 5.x.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 7, 41 and 90.

-	3rd order harmonics may fall into Rx frequencies of band 3.

-	2nd order IMD may fall into Rx frequencies of bands 7, 38, 40, 41, 53, 69, 77, 79 and 90.

-	3rd order IMD may fall into Rx frequencies of bands 2, 3, 9, 11, 21, 24, 25, 32, 33, 34, 35, 36, 37, 39, 46, 47, 50, 70, 74, 75, 92 and 94.

-	4th order IMD may fall into Rx frequencies of bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 44, 46, 67, 68, 71, 79 and 85.

-	5th order IMD may fall into Rx frequencies of bands 77 and 79.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.x.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.x.2-2: 2UL Band 8 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD2 |
|  | 2400 | - | 2494 | Yes | Asia | IMD2 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, IMD4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3, IMD4 |
|  | 5470 | - | 5725 | Yes |  | IMD3, IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, IMD4 |

The requirements for spurious emission band UE coexistence exist for DC_8_n78 in 38.101-3.

### 5.x.3	∆TIB and ∆RIB values

Table 5.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8A-32A_n78 | 8 | 0.6 |
|  | n78 | 0.8 |

Table 5.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8A-32A_n78 | 8 | 0.2 |
|  | 32 | 0 |
|  | n78 | 0.5 |

### 5.x.4	Reference sensitivity exceptions

Test point required for band 32 IMD3 hit from DC_8_n78 UL.

<End of Text Proposal>