3GPP RAN WG4 Meeting #102-e		R4-2203669

Electronic meeting, 21st February – 3rd March 2022

Agenda item:	11.2.2.1

Source:	Apple

Title:	Further input on performance when using the next larger channel

WI/SI:	FS_NR_eff_BW_util

Release:	Rel-17

Document for:	Approval

# 1	Introduction

During Rel-16 and Rel-17 discussions, several operators expressed an interest in enabling more efficient utilization of "non-standard" channel bandwidths, i.e. the ones which are not present now in TS 38.101 specifications. Referring to the corresponding operator requests, the following channel bandwidths were suggested by operators: 6, 7, 11, 12, 13, 33, 35, 45. As an outcome a new SI was agreed at the RAN#89 meeting aiming to study further which existing solutions can be used and whether new mechanism should be devised [1].

RAN WG4 has been considering several solutions, latest technical descriptions of which are captured in TR 38.844. One of the considered solutions is using next larger standard channel. In this TP we provide further technical information on the anticipated performance when the next larger channel is configured to the UE.

# 2	Text proposal

### 6.1.3	UE channel filters

A typical UE architecture utilises a number of filters of two major types – analogue and digital – and it is generally up to the UE implementation how they are combined. Nevertheless, it is often the case that a UE uses first the wideband analogue filter which typically covers a whole band. In addition to that, a UE may use another NR channel bandwidth specific analogue filter, premise function on which is to filter our non-adjacent blockers. However, since even the NR channel bandwidth specific analogue filter cannot ensure "brick-wall" like filtering, a UE also applies digital filter after ADC to eliminate adjacent blockers. Depending on the UE implementation, the digital filter is a combination of the hardware and software components that allow the UE to apply the corresponding filter coefficients to support a wide range of standard channels, e.g. from 5MHz to 100MHz in case of FR1.

As an example, consider Figure 6.1.3-1. In this example, RBs are scheduled close to the NR band edge.  The potential blocker below the band edge will be removed by the CBW filter with no degradation of ACS at the lower side of the irregular BW since the filters have the same capability as the regular CBW operation. However, for the scenario in the Figure 6.1.3-2 the wider channel filter cannot protect against blocker(s) when the irregular spectrum block is narrower than the channel filter and/or there are blockers on both sides.

Figure 6.1.3-1: Example of the NW utilizing the lower RBs near the NR band edge with BWP

NOTE: for the text above multi-operator scenario can be considered further in this example.

Current specifications do not define how a UE configures its digital filter within the configured channel bandwidth. So, in the provided example below, a UE implementation may configure the digital filter in accordance with the carrier bandwidth "ignoring" the actual smaller bandwidth part size. This is illustrated further in Figure 6.1.3-2. The wanted signal is smaller than 10MHz, but the UE filter is always set to 10MHz as signalled by the network. As can be seen, if there is an adjacent blocker, then it can "leak" into the wanted signal region.

Figure 6.1.3-2: Possible scenarios for the 10MHz channel filter.

To estimate anticipated performance when a UE configures its digital filter according to the channel bandwidth potentially allowing the blocker to "leak" into the wanted signal area, a series of measurements were conducted with the following common parameters:

-	Center frequency: 1850MHz

-	UE channel: 10MHz

-	Wanted signal level at UE antenna: -82dBm

-	Blocker signal level at UE antenna: from -120dBm to -50dBm in step of 1dBm

-	Blocker signal: 5MHz, 16QAM, SCS 15kHz

As also presented in Figure 6.1.3-2, two extreme irregular channel bandwidths are considered – 9MHz and 6MHz – and the blocker offset is set accordingly. As the main performance indicator, the resulting SNR is measured whereupon the SINR is estimated over the whole 10MHz region irrespective of the actual irregular channel size.

As can be seen from Figure 6.1.3-3 below, when the blocker level is low then the estimated SNR is around 17.5dB for all irregular channels. The 9MHz irregular channel bandwidth can sustain same SNR of 17.5dB even when the blocker level is same as the wanted signal. SNR degradation starts at the blocker level of approximately -72dBm, i.e. 10dB higher than the wanted signal. When the blocker level is 32dB higher than the wanted signal, the estimated SNR is around 5dB for the 9MHz irregular channel.

As for the 6MHz irregular channel, SNR degradation can be already observed at the blocker level of around -112dBm. When the blocker level becomes as high as the wanted signal, the estimated SNR drops down to 5dB. Finally, when the blocker is 32dB higher than the wanted signal, the SNR becomes as low as -27dB.

Figure 6.1.3-3: Estimated SNR depending on the blocker level.

To further reduce the risk of ACS reduction, as shown in this second example in Figure 6.1.3-2, it is possible for new UEs to align their digital filter to the actual smaller bandwidth part within the wider analogue CBW filter bandwidth.

NOTE:	It is FFS whether it will require new UE capability and/or configuration signalling for the NW to indicate the blocker location, left, right of the BWP.

# 3	Conclusions

In this discussion paper we have presented TP for TR 38.844 providing further technical input on the anticipated performance for a solution when the next larger channel is configured at the UE side.

# 4	References

RP-202103, "New SID: Study on Efficient utilization of licensed 	spectrum that is not aligned with existing NR channel bandwidths", T-Mobile USA, Ericsson