3GPP TSG RAN WG4 Meeting #102-e	                          R4-2203696

Electronic Meeting, 21th February – 3rd March  2022

Agenda item:	10.1.3.3

Source:	Apple Hungary Kft., MVG

Title:                         FR1 MIMO OTA Lab Alignment, Channel Model Validation update

Document for:	Discussion

# 1	Introduction

As defined on previous contributions [1,2], the FR1 MIMO OTA Lab Alignment activity is currently under way. Based on the initial timeline defined on [3] the volunteer test labs were defined prior to RAN4 #101-e as follows:

|  | Lab volunteer | City (Lab address) | Contact Info. |
| --- | --- | --- | --- |
| 1 | CAICT | Beijing, China | Xuan Yi, yixuan@caict.ac.cn Siting Zhu, zhusiting@caict.ac.cn |
| 2 | Huawei | Shanghai, China | Li jin xing, lijinxing3@huawei.com Lin hui, linhui20@huawei.com |
| 3 | Xiaomi | Beijing, China | Rui Zhou, zhourui1@xiaomi.com Baojun Feng, fengbaojun@xiaomi.com |
| 4 | MediaTek | Beijing, China | ting-wei.kang@mediatek.com |
| 5 | Apple | Cupertino, USA | Istvan Szini, iszini@apple.com |
| 6 | BUPT & CMCC Joint Lab | Beijing, China | Yuxiang Zhang, zhangyx@bupt.edu.cn Yichen Zhao, zhaoyichen@cmdc.chinamobile.com |
| 7 | SGS | New Taipei City, Taiwan | Peter Liao, Peter.liao@sgs.com |

PADs will be provided by vivo, Xiaomi, OPPO and Samsung

As reiterated on [4] this paper provides updated results from [5] channel model validation evidences, fulfilling the requirement to finalize the process on considering Apple’s FR1 MIMO OTA lab as a participant on this lab alignment effort.

# 2	Discussion

In this document, results of channel model validations are provided. The setup is composed by MVG StarMIMO – 1.2m with 16 dual polarized probes equipped with F64 for a total of 32 channels setup (3GPP standard probe layout). The following channel model CDL-C Uma parameters have been validated:

- 2.1, PDP 2450MHz Beam 1 (1+2) (updated)

- 2.2, PDP 2450MHz Beam 2 (3+4) (updated)

- 2.3, PDP 3600MHz Beam 1 (1+2) (updated)

- 2.4, PDP 3600MHz Beam 2 (3+4) (updated)

- 2.5, TCF (Doppler) 2450MHz Beam 1 (1+2)

- 2.6, TCF (Doppler) 2450MHz Beam 2 (3+4)

- 2.7, TCF (Doppler) 3600MHz Beam 1 (1+2)

- 2.8, TCF (Doppler) 3600MHz Beam 2 (3+4)

- 2.9, Spatial Correlation 2450MHz Combined Beams (updated)

- 2.10, Spatial Correlation 3600MHz Combined Beams (updated)

- 2.11, Cross-polarization 2450MHz Beam 1

- 2.12, Cross-polarization 2450MHz Beam 2

- 2.13, Cross-Polarization 3600MHz Beam 1

- 2.14, Cross-Polarization 3600MHz Beam2

Test procedures and instrument settings are according to TR 38.827 clause 7.4.1. Two frequencies have been tested 2450MHz (representing n41) and 3600MHz (representing n78). CE inputs have also been configured properly for specific beam, namely Beam 1 (Input 1 + Input 2) and Beam 2 (Input 3 + Input 4)

2.1, PDP 2450MHz -> Beam 1 (Input 1 + Input 2)

2.2, PDP 2450MHz -> Beam 2 (Input 3 + Input 4)

2.3, PDP 3600MHz -> Beam 1 (Input 1 + Input 2)

2.4, PDP 3600MHz -> Beam 2 (Input 3 + Input 4)

Observation1: Looking at the comparison between original model and agreed reference seems the agreed references are targeted for 40MHz CE BW

Observation 2: PDPs are within the agreed tolerances

2.5, TCF 2450MHz -> Beam 1 (Input 1 + Input 2)

2.6, TCF 2450MHz -> Beam 2 (Input 3 + Input 4)

2.7, TCF 3600MHz -> Beam 1 (Input 1 + Input 2)

2.8, TCF 3600MHz -> Beam 2 (Input 3 + Input 4)

Observation 1: Doppler has been measured for both Beam1 and Beam2 configuration by using the frequency domain technique which is the only method of testing agreed in TS38.151 Annex C.3.3.

2.9, Spatial Correlation 2450MHz combined beams

2450MHz -> Beam 1 + Beam2 (Input 1 + Input 2+Input3 + Input4)

2.10, Spatial Correlation 3600MHz combined beams

3600MHz -> Beam 1 + Beam2 (Input 1 + Input 2+Input3 + Input4)

Observation 1: The az points from 20 to 24 which are the furthest az points from the reference (270deg). cannot be modelled with 16 probes layout (more probes are needed).

Proposal: The following options are proposed to access the tolerance limit for the mentioned az points:

Option1: Set 0.2 as the maximum error for measured correlation below 0.65

Option2: make a distance dependent’s limits

2.11, Cross-Polarization, 2450MHz Beam 1

2.12, Cross-Polarization, 2450MHz Beam 2

2.13, Cross-Polarization, 3600MHz Beam 1

2.14, Cross-Polarization, 3600MHz Beam 2

# 3	Conclusion

This contribution provide the channel model validation factual evidences to fulfil the 3GPP FR1 MIMO OTA lab alignment requirements.

# 4	References

R4-2108613, WF on NR MIMO OTA, RAN4 #99-e

R4-2108617, Framework on FR1 MIMO OTA requirements development, RAN4 #99-e

R4-2113312, Time plan for FR1 lab alignment and requirement development, RAN4 #100-e

R4-2120683, Updated time plan for FR1 lab alignment and requirement, RAN4 #101-e

R4-2203066, FR1 MIMO OTA Lab Alignment, Channel Model Validation, RAN4 #101-bis-e