3GPP RAN WG4 Meeting #102-e		R4-2203700

Online, Feb 21st – Mar 3rd, 2022

Agenda item:	9.39.3

Source:	Apple

Title:	MSD analysis for FR2 inter-band CA with simultaneous Rx/Tx

WI/SI:	NR_RF_FR2_req_enh-Core

Release:	Rel-17

Document for:	Approval

# 1	Introduction

For FR2 inter-band CA, due to the anticipated severe MSD under simultaneous Rx/Tx operation caused by substantially low front-end cross-band isolation, it has been proposed that the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between inter-band NR carriers [1-2]. The consent was reached in RAN4 #101-e meeting with two draft CRs endorsed for CA_n260-n261 [3-4] which however were not implemented in the latest FR2 technical specifications [5]. In last RAN4 meeting, the draft CRs for CA_n260-n261 were resubmitted [6-7] in conjunction with a separate draft CR to apply the same non-simultaneous Rx/Tx footnote for CA_n257-n259 and CA_n258-n260 [8]. Though the resubmitted draft CRs for CA_n260-n261 were endorsed, the draft CR for CA_n257-n259 and CA_n258-n260 was postponed for the reason that one company would like to see the technical analysis and justification on why simultaneous Rx/Tx cannot be supported.

In our view, the RF characteristics among CA_n260-n261, CA_n257-n259 and CA_n258-n260 are quite similar where the minimum frequency separation between the two bands are 8.65GHz, 10GHz, and 9.5GHz respectively which should not result in much difference in cross-band isolation. The draft CR for CA_n257-n259 and CA_n258-n260 should have been endorsed to maintain the consistency in requirements. Nevertheless, in this contribution, we provide the MSD analysis for inter-band CA between 28GHz and 39GHz band groups to support the proposal that the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between inter-band NR carriers for all three specified inter-band CA combinations.

# Discussion

The MSD analysis for inter-band CA between 28GHz and 39GHz band groups was based on the RF parameters as summarized in table 2-1, which were characterized either by measurements or simulations.

| Parameter | Value | Unit | Comment |
| --- | --- | --- | --- |
| Rx NF | 12 | dB | w/o blocker |
| PA output noise floor | -125 | dBm/Hz | at Rx band |
| 28GHz <-> 39GHz ANT isolation | 5 | dB | Lumped |
| Normalized PA output power | 13 | dBm | at PCMAX |
| LO phase noise at Tx blocker offset | -142 | dBc/Hz |  |

Table 2-1 RF parameters for 28GHz+39GHz inter-band CA MSD analysis

Table 2-2 presents the link analysis for MSD calculation.

| PA output noise in Rx band | -130 | dBm/Hz |
| --- | --- | --- |
| Tx blocker level at Rx LNA input | 8 | dBm |
| Rx LO reciprocal mixing noise | -134 | dBm/Hz |
| Total Tx induced noise | -128.5 | dBm/Hz |
| Thermal noise (kT) | -174 | dBm/Hz |
| Rx noise floor | -162 | dBm/Hz |
| MSD | 34.5 | dB |
| Note: Blocker induced NF degradation was not accounted. |  |  |

Table 2-2 Link analysis for MSD calculation

Notice that with 8dBm Tx blocker at the receiver LNA input, the receiver noise figure (NF) can be substantially degraded. However, even without taking into account the Tx blocker induced NF degradation, the Tx output noise floor in Rx band and Rx LO reciprocal mixing noise alone could already contribute up to 34.5dB MSD.

Observation 1: For 28GHz+39GHz inter-band CA, the Tx output noise floor in Rx band and Rx LO reciprocal mixing alone could already contribute up to 34.5dB MSD.

As the Tx output noise floor and Rx LO reciprocal mixing noise would only decrease dB by dB with Tx output power, it would require more than 30dB aggressor output power reduction to avoid MSD. However, based on the FR2 field data as we presented in [9] where it was observed that more than 80% of UE output power was within top 10dB range from PCMAX for 28GHz band and more than 90% of UE output power was within top 10dB range from PCMAX for 39GHz band. As a result, the chance for FR2 inter-band CA to stay away from MSD impact under simultaneous Rx/Tx operation in typical field operating condition would be rather slim.

Observation 2: The chance for FR2 inter-band CA to stay away from MSD impact under simultaneous Rx/Tx operation in typical field operating condition is rather slim.

Based on the above analysis and observations, we think the proposal to clarify that the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between inter-band NR carriers for all three specified inter-band CA combinations is well justified.

Proposal: To endorse draft CR R4-2201343 postponed from last RAN4 meeting for the clarification that the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between inter-band NR carriers for CA_n257-n259 and CA_n258-n260 as shown below.

Table 5.2A.2-1: Inter-band CA operating bands in FR2

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n257-n2591 | n257, n259 |
| CA_n258-n2601 | n258, n260 |
| CA_n260-n2611 | n260, n261 |
| NOTE 1: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between inter-band NR carriers in the current version of this specification. |  |

# 3	Conclusion

In this contribution, we provide the MSD analysis for inter-band CA between 28GHz and 39GHz band groups to support the proposal that the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between inter-band NR carriers for all three specified inter-band CA combinations.

Observation 1: For 28GHz+39GHz inter-band CA, the Tx output noise floor in Rx band and Rx LO reciprocal mixing alone could already contribute 33.5dB MSD.

Observation 2: The chance for FR2 inter-band CA to stay away from MSD impact under simultaneous Rx/Tx operation in typical field operating condition is rather slim.

Proposal: To endorse draft CR R4-2201343 postponed from last RAN4 meeting for the clarification that the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between inter-band NR carriers for CA_n257-n259 and CA_n258-n260 as shown below.

Table 5.2A.2-1: Inter-band CA operating bands in FR2

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n257-n2591 | n257, n259 |
| CA_n258-n2601 | n258, n260 |
| CA_n260-n261 | n260, n261 |
| NOTE 1: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between inter-band NR carriers in the current version of this specification. |  |

# 4	References

R4-2117991 “Draft CR for TS 38.101-2: FR2 inter-band DL CA requirements clarification”, Apple, 3GPP TSG-RAN WG4 Meeting #101-e, November 1st – 12th, 2021

R4-2118179 “DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA”, ZTE Corporation, 3GPP TSG-RAN WG4 Meeting #101-e, November 1st – 12th, 2021

R4-2120052 “DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA – R16”, ZTE Corporation, 3GPP TSG-RAN WG4 Meeting #101-e, November 1st – 12th, 2021

R4-2119950 “DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA – R17”, ZTE Corporation, 3GPP TSG-RAN WG4 Meeting #101-e, November 1st – 12th, 2021

3GPP TS 38.101-2 V17.4.0(2021-12)

R4-2201341 “DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA – R16”, ZTE Corporation, 3GPP TSG-RAN WG4 Meeting #101bis-e, January 17th – 25th, 2022

R4-2201342 “DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA – R17”, ZTE Corporation, 3GPP TSG-RAN WG4 Meeting #101bis-e, January 17th – 25th, 2022

R4-2201343 “Draft CR to TS38.101-2[R17] On Simultaneous RxTx capability for FR2 inter-band CA_n257-n259 and CA_n258-n260”, ZTE Corporation, 3GPP TSG-RAN WG4 Meeting #101bis-e, January 17th – 25th, 2022

R4-2112384 “FR2 UL field data for NR 60GHz TPC consideration”, Apple, 3GPP TSG-RAN WG4 Meeting #100-e, August 16th – 27th, 2021