3GPP TSG-RAN WG4 Meeting #102e	R4-2203822

# Electronic Meeting, February 21 - March 3, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0992 | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.101-1 CR: UE RF requirements for DMRS bundling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support DMRS bundling and joint channel estimation for PUSCH and PUCCH in Rel-17 coverage enhancements WI, phase continuity and power consistency need to be maintained with certain tolerances. For power consistency, the existing requirement for aggregate power tolerance applies and no dedicated new requirements are to be introduced. For phase continuity, new requirements should be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the phase continuity requirements for DMRS bundling for PUSCH and PUCCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DMRS bundling cannot be supported in Rel-17. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sub-clauses: 6.4.2.5, 6.4D.2.5, 6.4G.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of first change >

6.4.2.5	Phase continuity requirements for DMRS bundling

For bands that UE declares the support for DMRS bundling, the maximum allowable difference between the measured phase value in any slot k and in [the first slot or its previous slot k - 1] for one DMRS port is listed in Table 6.4.2.5-1 for the parameters defined in Table 6.4.2.5-2, within a measurement time window determined by the UE’s capability of maximum duration for DMRS bundling [maxDMRS-BundlingDuration]. The phase value for each slot is measured [after the channel estimation and before the equalization], as shown in Annex F.x.

Table 6.4.2.5-1: Maximum allowable phase difference for DMRS bundling

| UL channel | Modulation order | Phase difference between any slot k and [the first slot or its previous slot k - 1] | Measurement time window |
| --- | --- | --- | --- |
| PUSCH | Pi/2 BPSK, QPSK | [30 or 15] degrees | Maximum duration reported by UE |
| PUCCH | Pi/2 BPSK, BPSK, QPSK |  |  |
| NOTE: For one freuqncy band, the same length of maximum duration applies for PUSCH and PUCCH, and for DFT-s-OFDM and CP-OFDM. |  |  |  |

Table 6.4.2.5-2: Parameters for the maximum allowable phase difference

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | PCMAX,f,c in clause 6.2.4, P-MPR = 0 |
| UE downlink received power |  | Not change |
| Operating conditions |  | Normal conditions |

The above requirements are applicable when all the following conditions are met within the specified time window:

-	RB allocation in terms of length and frequency position does not change, and intra-slot and inter-slot frequency hopping does not apply.

-	Modulation order does not change.

-	No network commanded TA takes effect.

-	The TPMI precoder does not change.

-	UE is not scheduled with uplink transmission of other physical channel/signal in-between the PUSCH or PUCCH transmissions.

-	In the case of non-back-to-back transmission, the un-scheduled gap in-between PUSCH or PUCCH transmission is no more than 13 symbols and [11] sybmols for different SCSs with normal CP and extended CP respectively.

-	For TDD, no downlink slot or downlink reception or downlink monitoring in-between the PUSCH or PUCCH transmissions.

< End of first change >

< Start of second change >

6.4D.2.5	Phase continuity requirements for DMRS bundling

For bands that UE declares the support for UL MIMO and DMRS bundling, the phase continuity requirements for DMRS bundling specified in clause 6.4.2.5 apply at [each transmit antenna connector], with the exception on UE output power configuration for the requirements, as in Table 6.4D.2.5-1.

Table 6.4D.2.5-1: Parameters for the maximum allowable phase difference

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | PCMAX, c in clause 6.2D.4, P-MPR = 0 |

< End of second change >

< Start of third change >

#### 6.4G.2.5	Phase continuity requirements for DMRS bundling

For bands that UE declares the support for Tx diversity and DMRS bundling, the phase continuity requirements for DMRS bundling specified in clause 6.4.2.5 apply, with the exception on UE output power configuration for the requirements, as in Table 6.4G.2.5-1.  The phase contunity requirement is derived based on the measurement at each antenna connector according to Annex F.y.

Table 6.4G.2.5-1: Parameters for the maximum allowable phase difference

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | PCMAX, c in clause 6.2G.4, P-MPR = 0 |

< End of third change >