3GPP TSG-RAN WG4 Meeting # 102-e		R4-2203908

Electronic Meeting, February 21 – March 3, 2022

Agenda item:	10.15.6

Source: 	CATT

Title: 	Discussion on RRM test cases for sidelink operation

Document for:	Discussion

Introduction

In RAN4#101-bis-e meeting, the core requirements of Rel-17 NR sidelink enhancement are endorsed in [1]. Although there are some remaining issues, the framework of core requirements is finalized. This contribution provides discussion on the RRM test case list.

Discussion

According to the work plan [2] which approved in RAN4# meeting, the list of test cases should be discussed in RAN4#102-e meeting.

- 3GPP RAN4 #102(e) meeting (February, 2022, 1TU/0.25TU, Core part/Performance part) - Discussions on : - Remaining issues on RRM requirements related to new features introduced in Rel-17 - Draft CRs - List of Test Cases - Agreement on : - Finalization on RRM core requirements - Big CR - List of Test Cases

The RRM requirements for Rel-17 NR sidelink enhancements are summarized as below:

- UE Transmit Timing when NR Cell is configured as synchronization reference source

- Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source with SL-DRX

- Selection / Reselection of V2X Synchronization Reference Source with SL-DRX

- L1 SL-RSRP measurements when partial sensing mechanism is enabled

- Interruptions to WAN at transitions between active and non-active during SL-DRX

- Interruptions to V2X sidelink at transitions between active and non-active during DRX

- Interruptions to V2X sidelink due to Active BWP switching Requirement

- Scheduling availability of UE switching between uplink transmission and V2X sidelink transmission

RAN4 should specify test cases to verify the enhanced RRM requirements for sidelink enhancement.

Based on the RRM core requirements, it is proposed to define test cases for Rel-17 NR sidelink enhancements in list as below:

Proposal 1: The following list of test cases in table are specified for NR sidelink enhancement

|  | Test cases |
| --- | --- |
| Initiation/Cease of SLSS Transmissions | Test for SyncRef UE as synchronization reference source with SL-DRX |
| Selection / Reselection of V2X Synchronization Reference Source | The current two test cases can be used as reference to be updated with SL-DRX |
| L1-RSRP | Test for sidelink when partial sensing mechanism is enabled |
| Interruptions | Test for Interruptions to WAN at transitions between active and non-active during SL-DRX(test cases for Sync and Async) Test for Interruptions to V2X sidelink due to Active BWP switching Requirement |
| UE Transmit Timing when NR Cell is configured as synchronization reference source And Scheduling availability of UE switching between uplink transmission and V2X sidelink transmission | In Rel-16, no test case is introduced to test for scheduling availability of UE switching between E-UTRA sidelink and NR sidelink In Rel-17, one test case can be introduced to test for scheduling availability of UE switching between uplink transmission and V2X sidelink transmission. It also verified the update UE transmit timing requirements. |

Conclusion

In this contribution, the test case list for Rel-17 NR sidelink enhancements is proposed as:

Proposal 1: The following list of test cases in table are specified for NR sidelink enhancement

|  | Test cases |
| --- | --- |
| Initiation/Cease of SLSS Transmissions | Test for SyncRef UE as synchronization reference source with SL-DRX |
| Selection / Reselection of V2X Synchronization Reference Source | The current two test cases can be used as reference to be updated with SL-DRX |
| L1-RSRP | Test for sidelink when partial sensing mechanism is enabled |
| Interruptions | Test for Interruptions to WAN at transitions between active and non-active during SL-DRX(test cases for Sync and Async) Test for Interruptions to V2X sidelink due to Active BWP switching Requirement |
| UE Transmit Timing when NR Cell is configured as synchronization reference source And Scheduling availability of UE switching between uplink transmission and V2X sidelink transmission | In Rel-16, no test case is introduced to test for scheduling availability of UE switching between E-UTRA sidelink and NR sidelink In Rel-17, one test case can be introduced to test for scheduling availability of UE switching between uplink transmission and V2X sidelink transmission. It also verified the update UE transmit timing requirements. |

Reference

[1] R4-2202747, Draft Big CR: RRM requirements for Rel-17 NR SL enhancement, LG Electronics

[2] R4-2109704, Work Plan of RRM requirements for Rel-17 SL enhancement, LG Electronics