3GPP TSG-RAN WG4 Meeting #102-e	R4-2203922

# Electronic Meeting, February 21 – March 3, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | <CR#> | rev | <Rev#> | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interruption requirement to LTE serving cell, and impacts to other LTE RRM requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-2-8 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The requirements for NR SRS antenna port switching should be introduced in LTE RRM specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing the requirements for NR SRS antenna port switching in LTE RRM specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for NR SRS antenna port switching are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.32.1, new 7.32.2.17, 7.36.1, new 7.36.2.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS36.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

========================= First Change Request ===========================

7.32	Interruptions with EN-DC

7.32.1	Introduction

This section contains the requirements related to the interruptions on PCell, and MCG SCell when

NR PSCell is added or released, or

transitions between active and non-active during NR PSCell DRX, or

transitions from NR PSCell non-DRX to DRX, or

SCell in either E-UTRA MCG or NR SCG is added or released, or

SCell(s) in either E-UTRA MCG or NR SCG is activated or deactivated, or

measurements on SCC with deactivated SCell in either E-UTRA MCG or NR SCG, or

a downlink bandwidth part (BWP) and/or an uplink BWP is switched in NR PSCell or in any NR SCell, or

UE dynamic Tx switches between two uplink carriers, or

NR SRS carrier based switching is performed, or

NR SRS transmission with antenna port switching.

The requirements shall apply for EN-DC.

This section contains interruption requirements when the victim cell is PCell or SCell belonging to MCG. Requirements for interruptions where victim cell is the NR PSCell or an NR SCell belonging to SCG are specified in [50].

For a UE which does not support per-FR measurement gaps, interruptions to the PCell or active MCG SCells may be caused by NR PSCell or NR SCells on any frequency range. For UE which support per-FR gaps, interruptions to the PCell or active MCG SCells may be caused by NR PSCell or NR SCells on FR1 only.

========================= Second Change Request ===========================

7.32.2.17	 Interruptions due to NR SRS antenna port switching

SRS transmission with antenna port switching can be configured on a NR SCell. When a UE needs to transmit SRS with antenna port switching on a NR serving cell, the UE can perform SRS transmitssion with antenn port switching, provided that:

-	the SRS is not colliding with any other transmission with higher priority defined in TS 38.214 [51].

-	the SRS is not colliding with any L3 measurements and RLM/BFD/CBD are scheduled in the same OFDM symbol in cells in the same carrier or same CG.

-	semi-persistent and periodic the SRS is not colliding with L1 measurements are scheduled in the same OFDM symbol in cells in the same carrier or same CG.

-	for UE, which does not support simultaneous reception and transmission for inter-band TDD CA specified in TS 38.331 [38], and is compliant to the requirements for inter-band CA with uplink in one NR band and without simultaneous Rx/Tx specified in TS 38.101-3 [54].

-	the SRS transmission are not simultaneously scheduled with DL SSB/CSI-RS for L3 measurements and  RLM/BFD/CBD on the carriers belong to the IE txSwitchImpactToRx indiction.

-	semi-persistent and periodic the SRS is not colliding with L1 measurements are scheduled in the same OFDM symbol in cells on the carriers belong to the IE txSwitchImpactToRx indiction.

The UE shall not perform SRS transmission with antenna port switching if the above conditions cannot be met.

When SRS transmission with antenna port switching is performed in a slot, the UE is allowed interruptions on any active serving cell recieving if the cell frequency is belong to the IE txSwitchImpactToRx indication, or interruptions on any active serving cell transmttion if the cell frequency is belong to the IE txSwitchWithAnotherBand indication overlapped with the SRS transmission, regardless of per-FR MG capability.

The interruption on PCC and each of the activated E-UTRA SCCs during UE transmt SRS with antenna port switching shall not exceed 2 subframes.

========================= Third Change Request ===========================

7.36.1	Introduction

This clause contains the requirements related to the interruptions on PSCell and SCG SCells when

transitions between active and non-active during NR PCell DRX, or

transitions from NR PCell non-DRX to DRX, or

SCell in either NR MCG or E-UTRA SCG is added or released, or

SCell(s) in either NR MCG or E-UTRA SCG is activated or deactivated, or

measurements on SCC with deactivated SCell in either NR MCG or E-UTRA SCG, or

a downlink bandwidth part (BWP) and/or an uplink BWP is switched in NR PCell or in any NR SCell, or

NR SRS carrier based switching is performed, or

NR SRS transmission with antenna port switching.

The requirements shall apply for NE-DC.

This clause contains interruption requirements when the victim cell is PSCell or SCell belonging to SCG. Requirements for interruptions where victim cell is the NR PCell or an NR SCell belonging to MCG are specified in TS 38.133 [50].

For a UE which does not support per-FR measurement gaps, interruptions to the PSCell or active SCG SCells may be caused by NR PCell or NR SCells on any frequency range. For UE which support per-FR gaps, interruptions to the PSCell or active SCG SCells may be caused by NR PCell or NR SCells on FR1 only.

========================= Fourth Change Request ===========================

7.36.2.16	 Interruptions due to NR SRS antenna port switching

SRS transmission with antenna port switching can be configured on a NR Cell. When a UE needs to transmit SRS with antenna port switching on a NR serving cell, the UE can perform SRS transmitssion with antenn port switching, provided that:

-	the SRS is not colliding with any other transmission with higher priority defined in TS 38.214 [51].

-	the SRS is not colliding with any L3 measurements and RLM/BFD/CBD are scheduled in the same OFDM symbol in cells in the same carrier or same CG.

-	semi-persistent and periodic the SRS is not colliding with L1 measurements are scheduled in the same OFDM symbol in cells in the same carrier or same CG.

-	for UE, which does not support simultaneous reception and transmission for inter-band TDD CA specified in TS 38.331 [38], and is compliant to the requirements for inter-band CA with uplink in one NR band and without simultaneous Rx/Tx specified in TS 38.101-3 [54].

-	the SRS transmission are not simultaneously scheduled with DL SSB/CSI-RS for L3 measurements and  RLM/BFD/CBD on the carriers belong to the IE txSwitchImpactToRx indiction.

-	semi-persistent and periodic the SRS is not colliding with L1 measurements are scheduled in the same OFDM symbol in cells on the carriers belong to the IE txSwitchImpactToRx indiction.

The UE shall not perform SRS transmission with antenna port switching if the above conditions cannot be met.

When SRS transmission with antenna port switching is performed in a slot, the UE is allowed interruptions on any active serving cell recieving if the cell frequency is belong to the IE txSwitchImpactToRx indication, or interruptions on any active serving cell transmttion if the cell frequency is belong to the IE txSwitchWithAnotherBand indication overlapped with the SRS transmission, regardless of per-FR MG capability.

The interruption on each of the activated E-UTRA SCCs during UE transmt SRS with antenna port switching shall not exceed 2 subframes.