3GPP TSG-RAN WG4 Meeting #102-e	R4-2203926

Electronic Meeting, February 21 – March 3, 2022

Title: 	Further discussion on UE Tx RF requirements for NTN

Source: 	CATT

Agenda item:	10.13.4.1

Document for:	Discussion

- Introduction

In last RAN4 meeting, the RF requirements for NTN UE were discussed and a way forward was approved [1]. The minimum output power requirement needs further discuss for NTN UE Tx RF requirements.

- Discussion

In WF [1], the decision for minimum output power for NTN UE Tx RF requirements is as following:

Introduce Minimum output power requirements, further discuss whether relaxed value needed or not based on existing TN UE requirements

The minimum output power is used for network to control UE transmit power to get expected received signal level. When a UE moves nearby the gNB, the UE transmit power can be controlled to minimum output power due to the propagation loss may be as low as MCL. In TN system, the MCL is assumed as 70dB. According Path loss defined in clause 6.6 in TS38.811 [2], the minimum path loss can be calculated as 154dB for LEO 600km for 2GHz. But the antenna gain will be greater than TN system, and may be 40dB for Tx antenna of UE fixed or mounted on moving platforms, and about 30dB for satellite Rx antenna reference parameters in TS38.821 [3]. The MCL may be about 84dB. So 15dB relax may be appropriate. For handheld UE, the antenna gain is assumed 0dB, more relax can be defined. But we think it may not be necessary to further relax and can be defined unified minimum output power requirement for NTN UE, i.e. from -40dBm relaxed to -25dBm.

Proposal: The minimum output power for NTN UE can be relaxed by ~ 15dB compared to TN UE, e.g.  -25 dBm.

- Conclusion

This document discussed minimum output power requirement for Tx RF requirements for NTN UE, and give following proposal.

Proposal: The minimum output power for NTN UE can be relaxed by ~ 15dB compared to TN UE, e.g.  -25 dBm.

- Reference

[1]	R4-2203036, WF on UE RF requirement for NTN UE, Huawei

[2]	3GPP TS38.811, Study on New Radio (NR) to support non-terrestrial networks

[3]	3GPP TS38.821, Solutions for NR to support non-terrestrial networks (NTN)