3GPP TSG-RAN WG4 Meeting #102-e	R4-2203928

Electronic Meeting, February 21 – March 3, 2022

Title: 	Further discussion on general RRM requirements for NTN

Source: 	CATT

Agenda item:	10.13.5.1

Document for:	Discussion

- Introduction

In last RAN4 meeting, a way forward on NR NTN RRM requirements was approved [1]. About general requirements, more agreements have reached as captured in section 5. There are some open issues need further discussion. This document will further discuss these open issues and present our understanding and proposals.

- Discussion

Here we will discuss open issues and present our understandings one by one.

4).  Issue 1-5: Cell Service Time

Issue 1-5-1: Measurement based on Cell Service Time

We have following Agreements in last meeting [1]:

- UE in RRC Idle/Inactive mode shall be able to detect, measure, and evaluate neighbour cells before a serving cell stops serving the area, if Serving cell service time information is broadcasted and applicable, regardless of whether the distance condition based on serving cell reference location or the legacy Srxlev/Squal condition are met. When to start detection, measurement, and evaluation is up to UE implementation.

  - FFS whether UE shall start the detection, measurement and evaluation on neighbour cells at the time when the legacy S/R criteria are met, e.g. serving cell RSRP is worse than threshold.

- The above does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of ‘serving cell stop time’ is started to the first slot when the cell is scheduled to stop serving the area according to the broadcasted information is less than Tdetect,NR_Intra and K*Tdetect,NR_Inter. Tdetect,NR_Intra, K, and Tdetect,NR_Inter are FFS.

Here, it is FFS whether UE shall start the detection, measurement and evaluation on neighbour cells at the time when the legacy S/R criteria are met, e.g. serving cell RSRP is worse than threshold. Before the moment to start detection, measurement and evaluation for serving cell stop, we think that UE should start the detection, measurement and evaluation on neighbour cells if the legacy S/R criteria are met, e.g. serving cell RSRP is worse than threshold for getting better network serving even it doesn’t need to start the detect, measure, and evaluate neighbour cells based on the time of serving cell stops serving the area.

Proposal 1: Before the moment to start detection, measurement and evaluation for serving cell stop, UE should start the detection, measurement and evaluation on neighbour cells if the legacy S/R criteria are met, e.g. serving cell RSRP is worse than threshold for getting better network serving.

5).  Issue 1-6: Neighbour/Target Cell/Satellite Information Acquisition

Issue 1-6-1: If valid neighbour/target cell’s timing information in terms of validity or accuracy is not provided to UE,

In last meeting, following agreements and conclusions were reached in WF [1]:

Agreement:

- RAN4 to discuss whether and how to define “availability of valid target satellite information as side condition” in RAN4#102 e-meeting if RAN2 reply LS is received.

- It is recommended companies provide details on the following aspects as well:

  - How to use the availability information, if defined, e.g. for differentiating requirements depending on the availability or for defining requirement applicability rule

  - Whether the above is only for measurement or both measurement and mobility

  - Whether the above is only for RRC Connected mode or irrespective of RRC state

Conclusion: The following can be discussed based on contributions in RAN4 #102 e-meeitng.

- Whether specify UE behavior whether the UE stops RRM measurements and reporting once the validity timer is expired (i.e., new or additional ephemeris information is not available within the associated validity duration).

A LS is received from RAN2 [2] and said that RAN2 has not decided on whether validity timer for SMTC configuration is defined. RAN2 will further provide update, if needed. So the discussion for this issue should be postponed until RAN2 have decision.

Proposal 2: The discussion for valid neighbour/target cell’s timing information should be postponed until RAN2 have decision.

7).  Issue 1-8: Signalling characteristics

Issue 1-8-1: Requirements related to Signalling Characteristics

Some agreements have reached as list in section 5 for this issue. But it is FFS on the following candidates to enhance RLM and Link Recovery requirements.

- Modification of hypothetical PDCCH formats for SSB and/or CSI-RS based RLM and BFD

- Modification of BLER_out/in

- Introducing an intermediate BLER between BLER_out and BLER_in

- Modification of Evaluation period

For RLM and BFD, we think the main difference between TN and NTN is propagation channel model. More link simulation study should be done if we make above modification. Before having sufficient data require above modification, we propose to reuse TN RLM BFD requirements for NTN.

Proposal 3: Reuse TN RLM and BFD requirements for NTN before having sufficient link level simulation for above modification.

Proposal 4: More link simulation study is needed for NTN RLM and BFD.

- Conclusion

This document discussed general RRM requirements for NTN. The following proposals are presented.

Proposal 1: Before the moment to start detection, measurement and evaluation for serving cell stop, UE should start the detection, measurement and evaluation on neighbour cells if the legacy S/R criteria are met, e.g. serving cell RSRP is worse than threshold for getting better network serving.

Proposal 2: The discussion for valid neighbour/target cell’s timing information should be postponed until RAN2 have decision.

Proposal 3: Reuse TN RLM and BFD requirements for NTN before having sufficient link level simulation for above modification.

Proposal 4: More link simulation study is needed for NTN RLM and BFD.

- Reference

[1]	R4-2202637, WF on NR NTN RRM requirements, Qualcomm

[2]	R4-2203512, Reply LS on Multiple SMTCs for NR NTN, RAN2 to RAN4

- Annex: Agreements for general requirements

1).  Issue 1-2: TN-NTN

Issue 1-2-1: TN-NTN RRM requirements for RRC Connected mode

Conclusion: It is recommended to skip this high-level discussion unless any critical issues are identified.

2).  Issue 1-3: Side condition and Simulations

Issue 1-3-2: Elevation angle

Agreement: Elevation angle is not defined as a side condition for NTN RRM requirements.

Issue 1-3-3: The number of neighbour cells/beams for measurement

Agreement:

- Number of neighbour cells/beams is defined as NTN UE capability requirements, and the number of neighbour cells/beams for GEO cell measurements is up to 4 per frequency layer.

3).  Issue 1-4: DRX Cycle

Issue 1-4-2: Applicability of Legacy DRX Cycles for Earth-moving Cell

Agreements

- Define RRM requirements for all legacy DRX cycles

- Regarding whether 2.56s DRX cycle is applicable for earth-moving LEO deployment.

  - Whether and what DRX cycle length to configure is up to NW, but UE is not required to fulfil the requirements for 2.56s DRX cycle length for earth-moving LEO deployment.

6).  Issue 1-7: RRM Spec Documentation

Issue 1-7-1: A spec structure of NTN UE RRM requirements

Session chair: Rapporteur (Thales) and moderator are asked to prepare a draft of CR split and provide initial version of the list the affected specification clauses. It will be discussed in the 2nd round.

Agreement:

- Section numbers for NTN always end with B, i.e. x.y.zB

- For those requirements applicable to but not affected by NTN, the following options will be further discussed:

  - Option 1) RAN4 will copy and paste them in separate sections for NTN

  - Option 2) RAN4 will introduce the applicability of the existing requirements in respective requirements

  - Depending on the conclusion of ‘Issue 1-7-1: A spec structure of NTN UE RRM requirements’, the owner of big-draft CR will implement all individual draft CRs in one big CR accordingly.

- Applicability of the following requirements are as below:

  - NR FR2 – NR FR1 HO

    - FFS

  - NR FR1 – NR FR2 HO

    - FFS

  - NR FR2 – NR FR2 HO

    - FFS

  - L1-SINR measurements for Reporting

    - Not applicable in Rel-17

  - CSI-RS based L3 measurements

    - Not applicable in Rel-17

  - NR measurements with autonomous gaps

    - Not applicable in Rel-17

  - Requirements indicated as FFS will be included in CRs with ‘Void’ until any conclusion is made.

- Idle/Inactive mode mobility Requirements

| Category | Sub-category | Requirement | (applicable and will be affected) | Company |
| --- | --- | --- | --- | --- |
| 4. RRC_ IDLE state mobility | Cell Re-selection | UE measurement capability | (Y, Y) | LGE |
|  |  | Measurement and evaluation of serving cell | (Y, FFS) | Intel |
|  |  | Measurements of intra-frequency NR cells | (Y, FFS) | Intel |
|  |  | Measurements of inter-frequency NR cells | (Y, FFS) | Huawei |
|  |  | Maximum interruption in paging reception | (Y, FFS) | Xiaomi |
|  |  | Minimum requirement at transitions | (FFS, FFS) | Huawei |
|  |  | Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion | (FFS, FFS) | Huawei |
|  | Minimization of Drive Tests (MDT) | Measurement Requirements | (FFS, FFS) | Qualcomm |
|  |  | Requirements for Relative Time Stamp Accuracy | (FFS, FFS) | Qualcomm |
|  |  | Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting | (FFS, FFS) | Qualcomm |
|  |  | Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting | (FFS, FFS) | Qualcomm |
| 5 RRC_ INACTIVE state mobility | Cell Re-selection | UE measurement capability | Same as the ones for IDLE. | Intel |
|  |  | Measurement and evaluation of serving cell | Same as the ones for IDLE. | Intel |
|  |  | Measurements of intra-frequency NR cells | Same as the ones for IDLE. | Intel |
|  |  | Measurements of inter-frequency NR cells | Same as the ones for IDLE. | Intel |
|  |  | Maximum interruption in paging reception | Same as the ones for IDLE. | Intel |
|  | Minimization of Drive Tests (MDT) | Measurement Requirements | Same as the ones for IDLE. | Qualcomm |
|  |  | Requirements for Relative Time Stamp Accuracy | Same as the ones for IDLE. | Qualcomm |
|  |  | Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting | Same as the ones for IDLE. | Qualcomm |
|  |  | Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting | Same as the ones for IDLE. | Qualcomm |
|  |  | Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting | Same as the ones for IDLE. | Qualcomm |

- Connected Mode Mobility Requirements

| Category | Sub-category | Requirement | (applicable and will be affected) | Company |
| --- | --- | --- | --- | --- |
| 6. RRC_ CONNECTED state mobility | Handover Parameters - NR Handover | NR FR1 - NR FR1 Handover | (Y, FFS) | CATT |
|  |  | NR FR2- NR FR1 Handover | (FFS, FFS) |  |
|  |  | NR FR2- NR FR2 Handover | (FFS, FFS) |  |
|  |  | NR FR1- NR FR2 Handover | (FFS, FFS) |  |
|  | Handover Parameters - NR DAPS (Dual-Active Protocol Stack) Handover | NR FR1 - NR FR1 DAPS Handover | (N, N) |  |
|  |  | NR FR2- NR FR1 DAPS Handover | (N, N) |  |
|  |  | NR FR1- NR FR2 DAPS Handover | (N, N) |  |
|  | Handover Parameters - NR Conditional Handover | NR FR1 – NR FR1 conditional handover | (Y, Y) |  |
|  |  | NR FR2 – NR FR1 conditional handover | (FFS, FFS) |  |
|  |  | NR FR2 – NR FR2 conditional handover | (FFS, FFS) |  |
|  |  | NR FR1 – NR FR2 conditional handover | (FFS, FFS) |  |
|  | RRC Connection Mobility Control | SA: RRC Re-establishment | (Y, FFS) |  |
|  |  | Random access | (Y, FFS) |  |
|  |  | SA: RRC Connection Release with Redirection | (Y, FFS) |  |

- Timing Requirements

| Category | Requirements | (applicable and will be affected) | Company |
| --- | --- | --- | --- |
| 7. Timing | UE transmit timing | (Y, Y) | HW |
|  | UE timer accuracy | (Y, FFS) | Xiaomi |
|  | Timing advance | (Y, Y) | MTK |
|  | Cell phase synchronization accuracy | (N, N) |  |
|  | Maximum Transmission Timing Difference | (N, N) |  |
|  | Maximum Receive Timing Difference | (N, N) |  |
|  | deriveSSB-IndexFromCell tolerance | (FFS, FFS) | THALES if applicable and affected. |

- Signaling Characteristics Requirements

| Category | Requirements | (applicable and will be affected) | Company |
| --- | --- | --- | --- |
| 8. Signalling characteristics Parameter | Radio Link Monitoring | (Y, FFS) | Ericsson |
|  | Interruption | (N, N) |  |
|  | SCell Activation and Deactivation Delay | (N, N) |  |
|  | UE UL carrier RRC reconfiguration delay | (N, N) |  |
|  | Link Recovery Procedures | (Y, FFS) |  |
|  | Active BWP switch delay | (Y, FFS) |  |
|  | PSCell Change | (N, N) |  |
|  | Uplink spatial relation switch delay | (FFS, FFS) |  |
|  | UE-specific CBW change | (Y, FFS) |  |

- Measurement Procedures Requirements

| Category | Requirements | (applicable and will be affected) | Company |
| --- | --- | --- | --- |
| 9. Measurement procedure | General measurement requirement | (Y, Y) | OPPO |
|  | NR intra-frequency measurements | (Y, Y) | HW |
|  | NR inter-frequency measurements | (Y, Y) | Xiaomi |
|  | L1-RSRP measurements for Reporting | (Y, Y) | Apple |
|  | Cross Link Interference measurements | (N, N) |  |
|  | L1-SINR measurements for Reporting | (N, N) |  |
|  | NR measurements for positioning | (N, N) |  |
|  | CSI-RS based L3 measurements | (N, N) |  |
|  | NR measurements with autonomous gaps | (N, N) |  |

7).  Issue 1-8: Signalling characteristics

Issue 1-8-1: Requirements related to Signalling Characteristics

Agreement:

- The following requirements in Signalling Characteristics are not relevant to NTN.

  - Interruption: no other active cells that can be interrupted by the other cell in NTN

  - SCell activation

  - UL carrier re-configuration

  - NE-DC/NR-DC PSCell addition

  - PSCell change

  - UL spatial relation switch

- For the following requirements, the legacy requirements are reused in NTN.

  - Active BWP switch

  - UE-specific CBW change

  - (Note) Detailed text can be modified to reflect NTN specific updates made by RAN1/2.