3GPP TSG-RAN WG4 Meeting # 102-e 		                       R4-2203964

Electronic Meeting, February 21 – March 3, 2022

Title: 	UE feature for NTN

Source: 	CATT

Agenda item:	10.13.1

Document for:	Discussion

- Introduction

In the last RAN4 meeting, it has been agreed that UE will support 64QAM with UE capability signalling [1]. This paper will provide the UE feature list for 64QAM for NTN.

- Discussion

It is proposed the following is included in the Rel-17 UE feature list for NTN 64QAM.

- NR_NTN_solutions

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory /Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [22. NR_NTN_solutions | [22-1] | 64QAM for PDSCH | 64QAM for PDSCH | [N/A] | [Yes] | [No] | [UE does not support higher modulation 64QAM for PDSCH for NTN. The network cannot schedule 64QAM for PDSCH when DL propagation condition is good enough.] | [Per Band] | [FDD only] | [FR1 only] | [N/A] | [N/A] | [Optional with capability signalling] |
| [22. NR_NTN_solutions | [22-2] | 64QAM for PUSCH | 64QAM for PUSCH | [N/A] | [Yes] | [No] | [UE does not support higher modulation 64QAM for PUSCH for NTN. The network cannot schedule 64QAM for PUSCH when DL propagation condition is good enough.] | [Per Band] | [FDD only] | [FR1 only] | [N/A] | [N/A] | [Optional with capability signalling] |

- Reference

[1]	R4-2203036, WF on UE RF requirement for NTN UE, Huawei, HiSilicon